GB10007 PCjr Function Key Usage

07/17/85

IBM PCjr System Unit/keyboard                      (4860067)

KEYWORDS: INFORMATION PROBLEM SOLUTION

PCjr function keys may appear not to function if the correct

procedure is  not  followed.  The proper use is to press and

RELEASE the  Fn  key  before pressing a numeric key.  In the

case of extended codes the shift or the alt key is held down

while pressing  and  RELEASING  the  Fn key then while still

holding the shift or alt key depressed, pressing the numeric

key. The keyboard definition chart in the PCjr GTO states to

press and  hold  certain  keys  but  does not say to release

others.

*** END OF DOCUMENT  ***

Note :  This modification or recommendation has not been subjected to

exhaustive testing and may not, in all circumstances, provide

a complete solution.

#sting and may not, in all circumstances, provide

a complete solution.