JC20007 Use of ALT key to enter extended codes on the PCjr.

07/17/85

UNIT-SYSTEM JR                 (4860067)

MULTIPLAN  1.0/1.1             (6024108)

KEYWORDS: INFORMATION PROBLEM

This  document  is  to clarify the use of the ALT key on the

PCjr  keyboard when entering the extended codes.  When using

the  ALT  key to enter the extended codes, the keyboard must

be  in Numeric Lock first.  To put the keyboard into Numeric

Lock,  hold  ALT and press FN, then N. (This is a toggle, to

take  the  keyboard  out  of  Numeric  Lock, repeat the same

procedure.)  Once  the  keyboard is in Numeric Lock, all 256

extended  codes  may be entered by  holding down the ALT key

and  then  typing in the numeric value of the extended code.

See  page 4-9 in the Guide to Operations Personal Computer -

PCjr manual, PN:1592292.

*** END OF DOCUMENT  ***

Note :  This modification or recommendation has not been subjected to

exhaustive testing and may not, in all circumstances, provide

a complete solution.

#sting and may n