EF10144 Screen intensity

09/05/85

IBM PCjr BASIC (with Cartridge)  Version 1.0       (6024101)

IBM PCjr System Unit/keyboard                      (4860067)

KEYWORDS: INFORMATION

ThePCjr  provides  for full contrast/brightness on all types

of monitors  when  it  is running with DOS 2.1 but, when the

machine goes  into  BASIC  either directly or via a program,

the  screen  intensity  drops to approximately half.  On the

display,  full gain on contrast and brightness controls does

not allow  you  to regain a normal brightness.  This is true

in Cassette or cartridge BASIC.  The intensity is  purposely

dimmed so  that  it  does  not overdrive an RGB or composite

monitor.  To  get  full  brightness on the display enter the

following BASIC statement:

Screen 1,0

This statement will set your screen  to high intensity.

*** END OF DOCUMENT  ***

Note :  This modification or recommendation has not been subjected to

exhaustive testing and may not, in all circumstances, provide

a complete solution.

#sting and may not, in all circumstances, provide

a complete solution.