UPGRADE YOUR IBM PCjr from 256K to 640K

If you have upgraded your IBM PCjr to 256K with IBM's 128K memory sidecar you

are in luck.  IBM designed the 128K memory board to support both 64K DRAMs and

256K DRAMs although this fact is undocumented.  The upgrade requires replacing

the 64K DRAM chips with 256K DRAM chips, cutting a jumper trace and adding a

jumper wire.  We, Turning Point Logic Corporation, have upgraded two IBM

PCjr's from 256K to 640K via this method with excellent results.  If you are

interested in upgrading the directions are given below.

WARNING: THIS UPGRADE IS FAIRLY DIFFICULT AND SHOULD ONLY BE ATTEMPTED BY

THOSE WITH SUBSTANTIAL SOLDERING AND CHIP REMOVAL EXPERIENCE.

IN ADDITION THIS IS NOT AN IBM SUPPORTED OR ACKNOWLEDGED UPGRADE

AND THEREFORE THIS UPGRADE IS PERFORMED AT THE USER'S OWN RISK.

UPGRADE DIRECTIONS

1.  Tools needed (if you do not have these tools then please do not proceed,

this procedure is very delicate and it is VERY EASY to DAMAGE the board).

a.  Small screwdriver

b.  A good pair of side cutters

c.  A good soldering iron

d.  Solder

e.  A good solder extractor

f.  An X-acto knife

g.  A small pair of needle nose pliers or tweezers.

2.  Parts needed.

a.  16 - 16 pin low profile solder tail sockets (sockets are not

necessary but are advantageous if a memory chip is bad or

found bad).

b.  16 - 16 256K DRAMs 150 ns

c.       A small piece of 28 or 30 AGW wire (1" is more than adequate).

3.  Power down the PCjr and disconnect the power cable.

4.  Remove the IBM 128K memory option sidecar.

5.  Completely unscrew the four mounting screws.

6.  Remove the interior sidecar panel with a small screwdriver.  The panel

is held in by eight tongs; four on the top, four on the bottom.

7.  Remove the printed circuit board from the sidecar case.

8.  Using a pair of side cutters (we highly recommend purchasing a good

pair of side cutters, if you do not have a pair, before attempting this

step. They cost about $10 and can make the difference between success and

failure of this project) completely cut legs of all of the installed 64K

DRAM chips.  Do not attempt to salvage the 64K chips.  Cut the legs at

the edge closest to the DRAM chip, NOT by the board.

9.  With good electrical solder, re-tin the solder pad at each cut leg.

This step may seem a waste but makes the overall job of removing the cut

legs and solder much, much easier.  Please take the time of performing

this step - we did not with our first upgrade and it took eight hours

to complete compared to two hours for the second upgrade.

10.  With a pair of needle nose pliers or tweezers and a soldering iron,

remove each cut leg.  Apply the heat of the soldering iron from the

component side.  Grip the leg to be removed with as little contact of

the pliers as possible.  The pliers become a heat sink and the less

surface contact of the pliers with the leg the less heat drawn off by

the pliers.

11.  Again re-tin all of the solder pads with a fair amount of clean solder.

Taking this step make take a longer amount of time in the short run but

can save a significant amount of time and headache in the long run.

12.  Remove all of the solder from the holes with a solder iron and solder

extractor.

13.  With a lightly tinned soldering iron apply enough heat to each hole

so the remaining solder melts around the edge of plated through

hole and solder pad.

14.  Scan the bare solder holes and surrounding area for lose pieces of

solder.  This lose solder can be a killer for it can cause intermittent

shorts which are near impossible to locate once the sockets have been

soldered in.  Use the soldering iron or an X-acto knife to remove the

excess solder.

15.  Solder in the 16 low profile solder tail sockets.

16.  With the component side of the board facing you there is a set of

three plated though holes to the right of the big chip (SN74S409N DRAM

Controller) with a white rectangle drawn around them.  This is a jumper

which determines whether 64K or 256K DRAMs are used.  Currently there is

a foil on the component side which connects the left and center holes

together.  Completely cut the foil which connects the two holes with an

X-acto knife.  Then solder a wire between the center hole and the right

hole.  The diagram below should give a better idea where the jumper is

located.

__________  __________  ________

_________________________   o-o o  <--- Jumper

|                         |     ___

|  SN74S409N              |   -[___]-

|_________________________|

17.  Install the 16 256K DRAM chips into their sockets.  Be sure each chip

is oriented properly (the notch indicating pin 1 should point upward).

18.  Set all four DIP switches on the rear board to their on position (OPEN).

19.  Reinstall the PCB back into the sidecar case.  Note that in one of our

upgrades we found that the pins of the DIP switch were shorted with the

strip of gold "fingers" located in the sidecar case.  The purpose of

the gold fingers is to ensure contact between the PCB and the sidecar

case.  It may be advisable to place a small piece of electrical tape

over the 8 pins of the DIP switch on the foil side of the board.

20.  Reinstall the sidecar onto the PCjr.

21.  Reconnect the power cable.

22.  Turn on the PCjr and verify that power-on diagnostics find 640K of

memory with no errors.

23.  It is then advisable to run the special diagnostics (CTL ALT INS) for

several hours.  If the special diagnostics do not find any errors then

the memory should be usable.

##########################################