Here is yet more additional info on unslushv.sys.  This message

appeared in comp.sys.zenith, and may be relevant for those with newer

Zenith machines.

-------------------------------------------------------------------

Article 699 in comp.sys.zenith:

From: easton@zds-oem.zds.com (Jeff Easton)

Subject: Re: Shadow ROM.

Message-ID: <1991Sep13.190759.28320@zds-oem.zds.com>

Date: 13 Sep 91 19:07:59 GMT

References: <18813@mentor.cc.purdue.edu>

Organization: Zenith Data Systems, OEM Systems Engineering

Lines: 45

In article <18813@mentor.cc.purdue.edu> tim@mentor.cc.purdue.edu (Timothy Lange)

writes:

>This is the documentation file that comes with UNSLUSHV.SYS, a device

>driver that will disable the video ROM from being shadowed in RAM.

>

>All current Zenith 286 and 386 desktop systems (except for the

>Z-286/25) are equipped with a system whereby slow ROMs are copied to

^^^^^^^^

Never heard of that one.  :-)

>high speed RAM to increase performance. The system is refered to as

>the slushware system, since it is a hybrid of software (RAM) and

>firmware (ROM). Other manufacturers use a similar scheme and refer to

>it as "shadow RAM."

[...]

>The area of memory from E0000-E7FFFH is reserved for video slush.

>Regardless of the type of video card used, this block of memory is

>used by write-protected RAM which cannot be disabled. As a direct

>result of this, even when video slushware is not used, those

>addresses are not available to any hardware card.

[...]

>To summarize: BIOS slushware cannot be disabled. Doing so would

>prevent the CPU from accessing the ROM code it requires to operate.

>Video slushware can be disabled to the point where it is no longer

>used, but will still be a block of write-protected RAM from

>E0000-E7FFFH. This block will conflict with any other hardware

>devices using those addresses. But if the block is unused, software

>packages which use 386 memory mapping tricks can use E0000-EFFFFH for

>EMS page frames and whatnot.

Note that this is no longer true.  The video code in the Z-LS and

Z-286LP+ is slushed at C000 - C7FF.  The actual ROM code is located at

the top of the 16Mb space along with the actual copy of the system

BIOS code.  This free's up the E000-E7FF range to be used for other

things like UMB's.  Thanks to the NEAT chipset, all memory from C800-

EFFF can be used as upper memory blocks (assuming EMS is disabled).

Jeff Easton               Zenith Data Systems

// Systems Engineer

\X/  easton%zds-oem@caspian.cs.andrews.edu

easton@andrews.edu