----==================================----

----====< Audio Port Programming >====----

----==================================----

TABLE OF CONTENTS

1. Audio Port overview

2. Programming Considerations

A. Hardware I/O Addresses

B. Audio Port Interface Controller

C. Command/Data Handshaking

3. Digital Audio programming

A. Sending Micro Commands/Data

B. Reading Micro Data

C. Reading FIFO data

D. Writing FIFO data

E. Micro Commands

4. FM programming.

5. Interrupt processing.

Appendix A - Hardware Ports & States

Appendix B - Example routines

AUDIO PORT OVERVIEW

The Audio Port is a portable audio device designed for

use with laptop or desktop computers. This device, which

attaches to the standard IBM PC parallel port, can record

and playback 22khz mono digital audio. Included with the

Audio Port is a built-in FM synthesis chip, the Yamaha 3812.

The Audio Port supports ADPCM hardware compression, and

decompression, in real-time, at rates up to 22khz. Speci-

fically, the device support 2-to-1 and 4-to-1 compression

ratios.

The programming interface is based upon the standard

parallel port I/O scheme, with a few bits redefined. The

parallel port on the IBM PC uses 3 I/O addresses; one output

data, one input status, and one output control. On certain

IBM AT compatible machines, the data port has been made into

a bi-directional data path for use with network adapters,

and other peripherals. The Audio Port supports this

bi-directional 8 bit data path.

The Audio Port interfaces the FM and microcontroller

chip to the PC via the Audio Port Interface Controller; aka,

the APIC. The following diagram illustrates the data/control

path.

������Ŀ                                     �Ŀ/�

�      �      ������������������������Ŀ     � � �

�      �Ŀ    ��������Ŀ    �����Ŀ    � ���Ĵ � �

�      �p�����Ĵ       ����͵ FM  ��Ŀ � �   � � �

� IBM  �a�����Ĵ       �    �������  � � �   ���\�

�      �r�����ĴAudio  �    �������Ŀ� � �

� PC/  �a�����ĴPort   ����͵DAC/   ������

�      �l�����ĴInterfc�    �ADC    ����Ŀ

� AT   �l�����ĴCtlr   �    ���������  � �     �Ŀ

�      �e�����Ĵ       �    �����Ŀ    � �����Ĵ��

�      �l�����Ĵ       ����͵Micro�    � �    ���

�      ���    ����������    �������    �  ����Ĵ��

�      �      ��������������������������       ���

��������

The APIC handles the actual clocking of data, and

determines the target device, either FM or the micro-

controller. In certain DAC/ADC modes, the APIC will drive

the DAC/ADC without intervention from the microcontroller

through the use of an internal 1k FIFO.

Data sent to the different devices require some

handshaking to guarantee that commands and data do not

overrun. This control is a single bit that is multiplexed

between the FM and Micro.

----====< PROGRAMMING CONSIDERATIONS >====----

H A R D W A R E   I / O   A D D R E S S E S

On the IBM PC/AT, three parallel ports can be found at

several addresess. For each of the parallel port there are

three I/O ports for communications to the external parallel

devices. The following map defines the three I/O ports,

across the three ranges of addresses.

NAME             LPT1  LPT2  MONO  DESCRIPTION

����������������������������������������������������Ŀ

�Data Output�R/W� 378 � 278 � 3BC � write output data�

����������������������������������������������������Ĵ

�Status     � R � 379 � 279 � 3BD � read status/data �

����������������������������������������������������Ĵ

�Control    � W � 37A � 27A � 3BE � control lines    �

������������������������������������������������������

The following illustrations show the actual bit definitions

of the three parallel port I/O addresses:

Data Register Definitions:

D7   D6   D5   D4   D3   D2   D1   D0

�����������������������������������������������Ŀ

�Data   �bit7�bit6�bit5�bit4�bit3�bit2�bit1�bit0�

�������������������������������������������������

|    |    |    |    |    |    |    |

+----+----+----+----+----+----+----+-- Data lines

Status Register Definitions:

D7   D6   D5   D4   D3   D2   D1   D0

�����������������������������������������������Ŀ

�Status � W  �IRQ �PAR �SRQ � DA � X  � X  � X  �

�������������������������������������������������

|    |    |    |    |    |    |    |

|    |    |    |    |    +----+----+-- unused

|    |    |    |    +----------------- Data Available

|    |    |    +----------------------  SRQ/D4/D0

|    |    +---------------------------  PAR/D5/D1

|    +--------------------------------  IRQ/D6/D2

+------------------------------------- WAIT/D7/D3

D7 - Wait status bit. It presents the

command/data register full for either

the FM or Micro registers, depending

on the control register state.

D6 - IRQ is pending bit. Same as the

parallel port definition.

D5 - Input Data Enabled.

D4 - Service ReQuest. This bit is used

by the digital audio FIFO to indicate

if the FIFO can take more data.

D3 - Data is available from the Micro or

FIFO. FM never sends data to the PC.

When the control register is in a read

state, D4-D7 become input lines presenting

each nibble of the 8 bit byte. During the

control port read idle state, the low nibble

is presented. During a read data state, the

high nibble is presented.

Control Register Definitions:

D7   D6   D5   D4   D3   D2   D1   D0

�����������������������������������������������Ŀ

�Control�PAR �PAR �PAR �IRQ � AS �MFM � DS � RW �

�������������������������������������������������

|    |    |    |    |    |    |    |

|    |    |    |    |    |    |    +-- Read(0)/Write(1)

|    |    |    |    |    |    +------- Data Strobe

|    |    |    |    |    +------------ Micro/FM select

|    |    |    |    +----------------- Address Strobe

|    |    |    +---------------------- IRQ Enable

+----+----+--------------------------- Parallel Input

D0-D3. These lines control the clocking to

the API . It is best to view these

four lines as control words with 16

possible states. A list of the control

states follows.

D4     When this bit is set to 1, interrupts

are allowed to be pass to the 8259

interrupt controller on the mother-

board. Using interrupts from the

parallel port is never advised. It is

best to a faster system timer to

perform interrupt duties.

D5-D7  With these bits high, some bi-

directional parallel ports become

active, that is, allow for 8 bit data

to be read from the data port.

The DATA port is used for sending data/commands to the

Audio Port. In cases where the computer supports bi-

directional data transfers, data can be read from the Audio

Port via this register. IBM did not design the parallel port

to be bi-directional, so in most cases, this feature will

not be used.

A U D I O   P O R T    I N T E R F A C E   C O N T R O L L E R

It is important to understand the concepts of talking to

the APIC. This device is the front-end to the separate

pieces of the Audio Port. The PC software is responsible for

creating the proper handshaking with the APIC. As implied by

it's name, there are specific hardware states for driving

either FM, or digital audio. The following is a list of all

the possible states for the Audio Port:

Value    State Name   Function

00h     RdFMIdle     Read FM Idle

01h     WrFMIdle     Write FM Idle

02h     invalid      invalid state

03h     WrFMData     Write FM Data

04h     RdIdle       Read Sound Idle

05h     WrIdle       Write Sound Idle

06h     RdSndData    Read Sound Data

07h     WrSndData    Write Sound Data

08h     RdFMStat     Get FM Status

09h     WrFMAddr     Write FM Address

0Ah     invalid      invalid state

0Bh     invalid      And another Reset

0Ch     RdSndStat    Read Sound Status

0Dh     WrSndCmd     Write Sound Command

0Eh     invalid      invalid state

0Fh     WrReset2     Write Reset

The four low order bits in the Control register

determine the state of the interface. Notice that some of

the states are invalid. Never assume that setting an invalid

state is harmless! An invalid state may present certain

clocking signals that may, for instance, load PCM data into

the FM index register. Other transitions from or to an

invalid state may have undetermined effects on the Audio

Port.

The actual APIC logic requires the PC to program the

Audio Port in a concise manner. Transitions from idle to

active, then back to idle states, must be strictly adhered

to, or unpredictable results may occur. The basic programm-

ing convention is to leave the control port in Write Idle

mode when idle. This way, another state can be transistioned

in and out. Another benefit of leaving the control port in

Write Idle is that the status register is switched to status

mode, not read nibble mode; therefore, all status bits are

available for reading. For more info, see the above

description of the Status Port.

The following are simple state flow diagrams. Reading

from left to right, these diagrams show the various legal

states for writing commands or data, or just reading data.

The I/O to the data port and status ports are not shown.

TO RESET THE AUDIO PORT:

WrIdle__>___ WrReset2  __>________________________ WrIdle

TO WRITE COMMANDS/DATA TO THE MICRO:

_ WrSndCmd  __>_____________________

WrIdle__>_/                                    \__ WrIdle

\_ WrSndData __>_____________________/

TO READ MICRO DATA:

WrIdle__>_ RdIdle__>_ RdSndData__>_ RdIdle__>_____ WrIdle

TO WRITE INDEX/DATA TO THE FM CHIP:

_ WrFMAddr _

WrIdle__>_ WrFMIdle__/            \__ WrFMIdle__>_ WrIdle

\_ WrFMData _/

TO READ FM STATUS:

WrIdle__>_ RdFMIdle__>_ RdFMStat__>_ RdFMIdle__>__ WrIdle

A typical sequence to send a PCM data byte would be:

mov     dx,[PortAddress]  ; get the base address.

add     dl,_CONTROL       ; move to the control port.

mov     al,WrIdle         ; set to Write Idle mode.

out     dx,al

Idledly

sub     dl,_CONTROL       ; move to the data port.

mov     al,[DataByte]     ; port, then send the

out     dx,al             ; data byte.

add     dl,_CONTROL       ; move to the control port.

mov     al,WrSndData      ; strobe the data out.

out     dx,al

mov     al,WrIdle         ; complete the strobe by

out     dx,al             ; returning to write idle.

Idledly

Two things to note from the above example; to strobe

data out to the Audio Port, the Audio port must make state

transitions from Idle, to an active send, then back to idle.

The second thing to note, is the required I/O delay when

returning to idle mode. By IBM's design, the parallel port

hardware signal timings make transitions to active states

very quickly. When transitioning into idle modes, the

signals take almost twice as long. The above example shows

the use of a simple text macro called "Idledly". This macro

It could be defined as:

Idledly   equ   <out dx,al>

All it does, is perform an addition I/O write instruction,

which on the IBM ISA bus, is guaranteed to take a minimum of

850ns of time.

The other equates used in the above code segment have

the following values:

_DATA     equ   0

_STATUS   equ   1

_CONTROL  equ   2

C O M M A N D / D A T A   H A N D S H A K I N G

When the PC clocks data to the Micro or FM, the APIC

latches the data, watches the signals, then sends the data

to the appropriate device. In order to help the PC know when

to send the index, commands, or data, a status bit in the

Status register is presented for the PC to poll. This status

bit, bit D7, is used for both the FM and Micro WAIT

status's. The Control register state, WrIdle or WrFMIdle,

determine which WAIT state is being presented for either

the FM or Micro.

For sending index/data to the 3812 FM chip, Yamaha has

defined certain timing delays between the writes. Index

writes take a minimum of 5 micro seconds to complete. Data

writes take a minimum of 35 micro seconds to complete. The

APIC starts clocking the index/data out to the FM chip once

the PC finishes clocking out the data byte to the APIC.

During the time the APIC is clocking the index/data out to

the 3812, the WAIT bit will be held high. The APIC holds the

wait line high for the entire duration of both index and

data write times. This means the PC has to poll the wait

line before writing the index, but can write both the index,

then data byte with no intervening waits.

For Micro reads and writes, the WAIT line is derived

from the micro. The APIC just passes this signal through

unaltered. During Write idle, the WAIT line indicates

whether the Micro can receive more commands or data. The

signal goes high (1) when WAIT is asserted.

For queued DAC output, the WAIT line goes active (1)

when the queue cannot receive more data. For ADC input, the

WAIT line goes active (1) when the queue is emptied.

DIGITAL AUDIO PROGRAMMING

The Digial Audio section is controlled by both the APIC

and Microcontroller. All commands issued are processed by

the microcontroller. The APIC provides the clocking control,

and FIFO management. To issue commands to the micro, the

status port must be polled to check the WAIT bit. The Micro

WAIT is presented during WrIdle mode. When the bit is low

(0), commands or data may be written.

Micro Command Set:

�������������������������������������������������������������������Ŀ

�    Command     �D7-D4 Code�   D3    �   D2    �   D1    �   D0    �

�������������������������������������������������������������������Ĵ

�   reserved     �    0     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�System Control  �    1     �    X    �    X    �    X    �    X    �

�������������������������������������������������������������������Ĵ

�8 Bit PCM Direct�    2     �Play/Rcd �AGC Kill �  Timer  �    0    �

�������������������������������������������������������������������Ĵ

�   reserved     �    3     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�Play PCM  Queue �    4     �Compress � 4/2 Bit �  Timer  �   Ref   �

�������������������������������������������������������������������Ĵ

�Rcd 8 Bit Queue �    5     �   VOX   �AGC Kill �  Timer  �    0    �

�������������������������������������������������������������������Ĵ

�Rcd 4 Bit Queue �    6     �   VOX   �AGC Kill �  Timer  �    1    �

�������������������������������������������������������������������Ĵ

�Rcd 2 Bit Queue �    7     �   VOX   �AGC Kill �  Timer  �    1    �

�������������������������������������������������������������������Ĵ

�   reserved     �    8     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    9     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    A     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    B     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    C     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    D     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    E     �    .    �    .    �    .    �    .    �

�������������������������������������������������������������������Ĵ

�   reserved     �    F     �    .    �    .    �    .    �    .    �

���������������������������������������������������������������������

System Control Commands - 0001xxxx

10h SYSCMDIDLE    - Terminate DAC/ADC modes, return to Idle.

This is guaranteed to return the Audio

Port Micro to an Idle state

Parameters: No Parameters

Returns:    No Data Returned

11h SYSCMDGETID   - Get ESP version

Parameters: No parameters

Returns:    Returns 2 bytes

13h SYSCMDENAPARR - Enable Parallel Input

Parameters: No Parameters

Returns:    No Data Returned

14h SYSCMDDISPARR - Disable Parallel Input

Parameters: No Parameters

Returns:    No Data Returned

15h SYSCMDTSTPARR - Test Parallel Input

Parameters: One data byte following (any value)

Returns:    Returns the same data byte

in parallel mode.

Direct DAC/ADC Control Commands - 0010xxxx

NOTE: The AGC kill bit disables the internal Auto Gain

Control. This bit is normally set to 0, thus allowing

the AGC to function. Setting this bit to a 1 turns

off the AGC for the duration of the recording.

20h SYSDIRPLAY    - 8 bit direct play.

This places the Audio Port Micro into DAC

output mode. The queue is not enabled.

Parameters: Sound Data follows.

Returns:    No Data Returned

22h SYSDIRNA2     - 8 bit direct play w/time constant following.

This places the Audio Port Micro into

DAC output mode. The queue is not enabled.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Sound Data follows.

Returns:    No Data Returned

28h SYSDIRRECD    - 8 bit direct record.

Sampled data will be made available until

command 10H is issued.

Parameters: No Parameters

Returns:    Sampled data will be made available until

command 10H is issued.

2Ah SYSDIRNAA     - 8 bit direct record w/time constant following.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Returns:    Sampled data will be made available until

command 10H is issued.

FIFO DAC/ADC Commands - 0100xxxx

40h SYSDIRQPLAY   - 8 bit FIFO DAC out.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: Sound Data follows.

Returns:    No Data Returned

42h SYSDIRQNA2    - 8 bit FIFO DAC out w/time constant following.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Parameters: Sound Data follows.

Returns:    No Data Returned

48h SYSDIRQRECD   - 4 bit FIFO DAC out decompression

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: Sound Data follows.

Returns:    No Data Returned

49h SYSDIRQRECD   - 4 bit FIFO DAC out decompression with

reference byte.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: One 8 bit sample follows. This is the seed

value for decompression.

Sound Data follows.

Returns:    No Data Returned

4Ah SYSDIRQRECD   - 4 bit FIFO DAC out decompression with

time constant following.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Sound Data follows.

Returns:    No Data Returned

4Bh SYSDIRQRECD   - 4 bit FIFO DAC out decompression with

time constant following.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: Two parameter bytes follows. The first is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

The second byte is the seed value for

decompression.

Sound Data follows.

Returns:    No Data Returned

4Ch SYSDIRQRECD   - 2 bit FIFO DAC out decompression.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: Sound Data follows.

Returns:    No Data Returned

4Dh SYSDIRQRECD   - 2 bit FIFO DAC out decompression with

reference byte.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: One 8 bit sample follows. This is the seed

value for decompression.

Sound Data follows.

Returns:    No Data Returned

4Eh SYSDIRQRECD   - 2 bit FIFO DAC out decompression with

time constant following.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Sound Data follows.

Returns:    No Data Returned

4Fh SYSDIRQRECD   - 2 bit FIFO DAC out w/time constant following.

This function enables the 1k FIFO output

to the DAC. Watch the SRQ line to send out

256 byte blocks.

Parameters: Two parameter bytes follows. The first is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

The second byte is the seed value for

decompression.

Sound Data follows.

Returns:    No Data Returned

FIFO 8 Bit Recording Commands - 0101xxxx

NOTE: The AGC kill bit disables the internal Auto Gain

Control. This bit is normally set to 0, thus allowing

the AGC to function. Setting this bit to a 1 turns

off the AGC for the duration of the recording.

The VOX bit allows voice activated recording. When

this bit is set (1), the Audio Port will only begin

recording when the mic signal reaches certain volumes.

When in VOX mode, the Audio Port will stop recording

1 to 2 seconds after the mic signal becomes quiet.

50h SYSDIRPLAY    - 8 bit FIFO record.

Sampled data will be made available until

command 10H is issued.

Parameters: No Parameters

Returns:    Sampled data will be made available until

command 10H is issued.

52h SYSDIRNA2     - 8 bit FIFO w/time constant following.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Returns:    Sampled data will be made available until

command 10H is issued.

FIFO 4 Bit Recording Commands - 0110xxxx

NOTE: The AGC kill bit disables the internal Auto Gain

Control. This bit is normally set to 0, thus allowing

the AGC to function. Setting this bit to a 1 turns

off the AGC for the duration of the recording.

The VOX bit allows voice activated recording. When

this bit is set (1), the Audio Port will only begin

recording when the mic signal reaches certain volumes.

When in VOX mode, the Audio Port will stop recording

1 to 2 seconds after the mic signal becomes quiet.

60h SYSDIRPLAY    - 4 bit FIFO record compression.

Sampled data will be made available until

command 10H is issued.

Parameters: No Parameters

Returns:    Sampled data will be made available until

command 10H is issued.

63h SYSDIRNA2     - 4 bit FIFO w/time constant byte parameter.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Returns:    Sampled data will be made available until

command 10H is issued. The first sample

returned is the full 8 bit sample used

as the reference byte.

FIFO 2 Bit Recording Commands - 0110xxxx

NOTE: The AGC kill bit disables the internal Auto Gain

Control. This bit is normally set to 0, thus allowing

the AGC to function. Setting this bit to a 1 turns

off the AGC for the duration of the recording.

The VOX bit allows voice activated recording. When

this bit is set (1), the Audio Port will only begin

recording when the mic signal reaches certain volumes.

When in VOX mode, the Audio Port will stop recording

1 to 2 seconds after the mic signal becomes quiet.

70h SYSDIRPLAY    - 2 bit FIFO record.

Sampled data will be made available until

command 10H is issued.

Parameters: No Parameters

Returns:    Sampled data will be made available until

command 10H is issued.

73h SYSDIRNA2     - 2 bit FIFO w/time constant following.

Parameters: One byte parameter follows. This is the

sample rate speed. The following function

calculates the sample rate:

TimeConst = 256-(1000000/SampleRate)

Returns:    Sampled data will be made available until

command 10H is issued. The first sample

returned is the full 8 bit sample used

as the reference byte.

FM PROGRAMMING.

For sending index/data to the 3812 FM chip, Yamaha has

defined certain timing delays between the writes. Index

writes take a minimum of 5 micro seconds to complete. Data

writes take a minimum of 35 micro seconds to complete. The

APIC starts clocking the index/data out to the FM chip once

the PC finishes clocking out the data byte to the APIC.

During the time the APIC is clocking the index/data out to

the 3812, the WAIT bit will be held high. The APIC holds the

wait line high for the entire duration of both index and

data write times plus delay times. This means the PC has to

poll the wait line before writing the index, but can write

both the index, then data byte with no intervening waits.

INTERRUPT PROCESSING.

The recommended approach for interrupt processing is to use

a fast system timer interrrupt. The system timer should be re-

programmed to generate an interrupt every 30 milliseconds to

check the SRQ line. If the SRQ is high, another 256 byte block

of PCM data can be read or written. At interrupt time, upon

completing the first 256 byte block, check SRQ again, and

repeat the process until the SRQ goes low. Once all blocks are

finished, the system timer can be reprogrammed to interrupt at

it's normal frequency of 18.2 times per second.

On DAC output, The SRQ line goes high when the 1k buffer

has 256 bytes, or less remaining. It goes low when it has

768 bytes or more stored.

On ADC input, the SRQ line goes high if the FIFO has

768 or more bytes. It goes low when it has 256 or fewer

bytes stored.

APPENDIX A - HARDWARE PORTS & STATES

NAME             LPT1  LPT2  MONO  DESCRIPTION

���������������������������������������������������Ŀ

�Data Output� R � 378 � 278 � 3BC �write to TP      �

���������������������������������������������������Ĵ

�Status     � R � 379 � 279 � 3BD �read status/data �

���������������������������������������������������Ĵ

�Control    � W � 37A � 27A � 3BE �set I/O control  �

�����������������������������������������������������

Base Address + 0 offset -  Data Register Definitions:

D7   D6   D5   D4   D3   D2   D1   D0

�����������������������������������������������Ŀ

�Data   �bit7�bit6�bit5�bit4�bit3�bit2�bit1�bit0�

�������������������������������������������������

|    |    |    |    |    |    |    |

+----+----+----+----+----+----+----+-- Data lines

Base Address + 1 offset -  Status Register Definitions

D7   D6   D5   D4   D3   D2   D1   D0

�������������������������������������������������Ŀ

�Status   � W  �IRQ �PAR �SRQ � DA � X  � X  � X  �

���������������������������������������������������

|    |    |    |    |    |    |    |

|    |    |    |    |    +----+----+- unused

|    |    |    |    +---------------- Data Available

|    |    |    +---------------------  SRQ/D4/D0

|    |    +--------------------------  PAR/D5/D1

|    +-------------------------------  IRQ/D6/D2

+------------------------------------ WAIT/D7/D3

Base Address + 2 offset -  Control Register Definitions

D7   D6   D5   D4   D3   D2   D1   D0

�������������������������������������������������Ŀ

�Control  �PAR �PAR �PAR �IRQ � AS �MFM � DS � RW �

���������������������������������������������������

|    |    |    |    |    |    |    |

|    |    |    |    |    |    |    +- Read(0)/Write(1)

|    |    |    |    |    |    +------ Data Strobe

|    |    |    |    |    +----------- Micro/FM select

|    |    |    |    +---------------- Address Strobe

|    |    |    +--------------------- IRQ Enable

+----+----+-------------------------- Par. input select

Control Register Clock State:

00h     RdFMIdle     Read FM Idle

01h     WrFMIdle     Write FM Idle

02h     invalid      invalid state

03h     WrFMData     Write FM Data

04h     RdIdle       Read Sound Idle

05h     WrIdle       Write Sound Idle

06h     RdSndData    Read Sound Data

07h     WrSndData    Write Sound Data

08h     RdFMStat     Get FM Status

09h     WrFMAddr     Write FM Address

0Ah     invalid      invalid state

0Bh     invalid      And another Reset

0Ch     RdSndStat    Read Sound Status

0Dh     WrSndCmd     Write Sound Command

0Eh     invalid      invalid state

0Fh     WrReset2     Write Reset

Control Register Additional bit definitions

10h     Interrupt    Enable interrupt

E0h     EnableInput  Enable Parallel Input

APPENDIX B - EXAMPLE ROUTINES

The following code assumes the parallel port base I/O

address for the Audio Port is stored in the variable,

"PortAddress".

The following macros and equates are used in the code

examples:

Idledly   equ   <out dx,al>

_DATA     equ   0      ; Parallel port data port offset

_STATUS   equ   1      ; Parallel port status port offset

_CONTROL  equ   2      ; Parallel port control port offset

RdFMIdle  equ   00h    ; Read FM Idle

WrFMIdle  equ   01h    ; Write FM Idle

invalid   equ   02h    ; invalid state

WrFMData  equ   03h    ; Write FM Data

RdIdle    equ   04h    ; Read Sound Idle

WrIdle    equ   05h    ; Write Sound Idle

RdSndData equ   06h    ; Read Sound Data

WrSndData equ   07h    ; Write Sound Data

RdFMStat  equ   08h    ; Get FM Status

WrFMAddr  equ   09h    ; Write FM Address

invalid   equ   0Ah    ; invalid state

invalid   equ   0Bh    ; And another Reset

RdSndStat equ   0Ch    ; Read Sound Status

WrSndCmd  equ   0Dh    ; Write Sound Command

invalid   equ   0Eh    ; invalid state

WrReset2  equ   0Fh    ; Write Reset

;

;   /*\

;---|*|----====< AP_ADC_input >====----

;---|*|

;---|*| Input 256 bytes from the AP FIFO. This routine

;---|*| assumes that the AP is already in ADC input mode.

;---|*|

;---|*| Entry Conditions:

;---|*|   ES:DI point to the buffer.

;---|*|   CX holds a count of 1 to 256

;---|*|

;---|*| Exit Conditions:

;---|*|   ES:DI is updated. CX is zero

;---|*|   AX,BX,CX,DX,ES:DI modified

;---|*|

;   \*/

;

public  AP_ADC_input

AP_ADC_input    proc    near

;

; load the FIFO quarter size for use with the SRQ handshaking.

;

mov     dx,[PortAddress]        ; get the port address

add     dl,_CONTROL

cld                             ; STOSBs are used

;

apadc05:

mov     al,RdIdle               ; go into Read Idle to pick up the nibble

out     dx,al                   ; Idle w/o interrupts

Idledly

dec     dx                      ; move to STATUS

in      al,dx

mov     ah,al                   ; save the high nibble

inc     dx

mov     al,RdSndData            ; ah = RdIdle, al = RdSndData

out     dx,al                   ; this delivers the second nibble

Idledly

dec     dx

in      al,dx                   ; fetch it

shr     ah,4                    ; merge the nibbles

shr     ax,4

stosb                           ; save the byte

or      di,di                   ; segment wrap?

jz      apadcsegwrap            ; yes, go adjust...

;

apadc10:

inc     dx                      ; place the board into idle mode again

loop    apadc05                 ; do all 256 or less bytes

mov     al,WrIdle               ; return back to write idle

out     dx,al

Idledly

ret                             ; all done recording this block!

;

apadcsegwrap:

mov     di,es                   ; wrap the segment for huge model

add     di,1000h                ; support.

mov     es,di

sub     di,di

jmp     short  apadc10

AP_ADC_input    endp

;

;   /*\

;---|*|----====< AP_DAC_output >====----

;---|*|

;---|*| Output 256 bytes to the AP FIFO. This routine

;---|*| assumes that the AP is already in DAC output mode.

;---|*|

;---|*| Entry Conditions:

;---|*|   ES:DI point to the buffer.

;---|*|   CX holds a count of 1 to 256

;---|*|

;---|*| Exit Conditions:

;---|*|   ES:DI is updated. CX is zero

;---|*|   AX,CX,DX,ES:DI modified

;---|*|

;   \*/

;

public  AP_DAC_output

AP_DAC_output   proc    near

;

; make sure the port is in write idle mode...

;

mov     dx,[PortAddress]        ; get the port address

add     dl,_CONTROL

mov     al,WrIdle               ; make sure its idle

out     dx,al

sub     dl,_CONTROL             ; move back to the data port

;

apdac05:

;

; send 256 bytes of data out to the FIFO

;

mov     al,es:[di]              ; fetch the byte

inc     di

jz      apdacsegwrap            ; odd case jumps out...

;

apdac10:

out     dx,al                   ; write it to the audio port

add     dl,_CONTROL

mov     al,WrSndData            ; start the strobe...

out     dx,al

mov     al,WrIdle               ; end the strobe....

out     dx,al

Idledly

sub     dl,_CONTROL

loop    apdac05                 ; do all 256 or less bytes

ret

;

apdacsegwrap:

mov     di,es                   ; move to the next segment

add     di,1000h

mov     es,di

sub     di,di

jmp     short apdac10           ; then get back into the playback

AP_DAC_output   endp

/*\

|*|----====< AP_Reset >====----

|*|

|*| Reset the Audio Port

|*|

|*| Entry Conditions:

|*|   None.

|*|

|*| Exit Conditions:

|*|   AL holds the reset byte - 05A if successful

|*|   AX,BX,CX,DX modified

|*|

\*/

public  AP_Reset

AP_Reset proc

mov     dx,[PortAddress]

mov     al,-1           ; load FF into the data port to drive the

out     dx,al           ; data lines.

add     dl,_CONTROL

mov     cx,16

mov     al,WrReset2     ; output reset mode

;

@@:

out     dx,al

loop    @B

mov     cx,16

;

@@:

mov     al,WrIdle       ; clear reset

out     dx,al

loop    @B

mov     al,RdIdle       ; get sound data read status

out     dx,al

Idledly

dec     dx              ; move back to the status port

sub     cx,cx

;

aprst05:

in      al,dx

test    al,stIDav       ; wait for data to become available

loopz   aprst05

jz      aprstbad

mov     ah,al           ; save the result (should not be 0xFF)

inc     dx              ; move to the control port

mov     al,RdSndData    ; Clock in the data

out     dx,al

dec     dx

in      al,dx

mov     bx,ax

inc     dx

mov     al,RdIdle       ; go back to idle

out     dx,al

Idledly

mov     al,WrIdle       ; go back to idle

out     dx,al

Idledly

mov     cl,4            ; build the reset byte.

shr     bh,cl

shr     bx,cl

mov     al,bl

ret

;

aprstbad:

mov     al,-1

ret

AP_reset endp

/*\

|*|----====< AP_MDR_stat >====----

|*|

|*| Returns the status of data available

|*|

|*| Entry Conditions:

|*|   None.

|*|

|*| Exit Conditions:

|*|   Carry clear & AX = 0,  no data available

|*|   Carry set   & AX = -1, data available

|*|   DX,AX modified

|*|

\*/

public  AP_MDR_stat

AP_MDR_stat     proc

mov     dx,[PortAddress]

add     dl,_CONTROL

mov     al,RdIdle       ; Micro read idle

out     dx,al

Idledly

dec     dx              ; mov to the status port

in      al,dx           ; data rdy is high for data available

mov     ah,stIDav

and     ah,al           ; save the status bit

mov     al,WrIdle       ; send the port back to

out     dx,al           ; idle mode

Idledly

neg     ah              ; set carry if non-zero

sbb     ax,ax           ; if data is available, return -1 & Cy

ret                     ; else return 0 & NC

AP_MDR_stat     endp

/*\

|*|----====< AP_MDR_wait >====----

|*|

|*| Wait for Micro Data Read to be ready.

|*|

|*| Entry Conditions:

|*|   None.

|*|

|*| Exit Conditions:

|*|   Carry clear & AX = 0,  no data available

|*|   Carry set   & AX = -1, data available

|*|   DX,AX modified

|*|

\*/

public  AP_MDR_wait

AP_MDR_wait     proc

push    cx

sub     cx,cx

;

@@:

call    AP_MDR_stat

or      ax,ax

loopz   @B

pop     cx

ret

AP_MDR_wait     endp

/*\

|*|----====< AP_MCW >====----

|*|

|*| Send a command byte out to the Micro.

|*|

|*| Entry Conditions:

|*|    AL holds the command

|*|

|*| Exit Conditions:

|*|    AX = -1 if data is available, or 0

|*|    DX,AX modified

|*|

\*/

public  AP_MCW

AP_MCW  proc

push    cx

mov     ah,al           ; save the command/data

;

apmcw_00:

mov     dx,[PortAddress]

add     dl,_CONTROL

mov     al,WrIdle

out     dx,al

Idledly

mov     al,ah           ; get the byte and write it out

sub     dl,_CONTROL     ; move to the data port

out     dx,al           ; load the data

inc     dx              ; move to the status port

sub     cx,cx

;

apmcw_05:

in      al,dx           ; wait=1 when cmd/data cannot be sent

test    al,stWait

loopnz  apmcw_05

jz      apmcw_10        ; its available

push    ax              ; The device is probably powered off, so

call    AP_reset        ; we just have to kick it...

pop     ax              ; AH holds the command/data byte

jmp     short apmcw_00

;

apmcw_10:

inc     dx              ; move to the control port

mov     al,WrSndCmd     ; start the strobe

out     dx,al

mov     al,WrIdle       ; end the strobe

out     dx,al

Idledly

pop     cx

ret

AP_MCW  endp

/*\

|*|----====< AP_MDR >====----

|*|

|*| Read a data byte from the Micro of FIFO

|*|

|*| Entry Conditions:

|*|    Data Available must have been checked before calling

|*|    this routine. It is assumed to be high.

|*|

|*| Exit Conditions:

|*|    AL holds something...

|*|    DX,AX modified

|*|

\*/

AP_MDR  proc

push    cx

mov     dx,[PortAddress]

add     dl,_CONTROL

mov     al,RdIdle       ; go to read IDLE

out     dx,al

Idledly

mov     al,RdSndData    ; read the high nibble

out     dx,al           ; start the strobe

dec     dx              ; grab one nibble

in      al,dx           ; from the status port

mov     ah,al           ; save in AH!

inc     dx

mov     al,RdIdle       ; swap nibbles

out     dx,al           ; end the strobe

Idledly

dec     dx

in      al,dx           ; read the next nibble

mov     cl,WrIdle

xchg    ax,cx           ; save the bytes, load WrIdle

inc     dx              ; send H/W into write idle mode

out     dx,al

xchg    ax,cx           ; get the bytes

mov     cl,4

shr     ah,cl           ; this order makes for a

shr     ax,cl           ; fast conversion

pop     cx

ret

AP_MDR  endp

/*\

|*|----====< AP_MDW >====----

|*|

|*| Send a data byte out to the FIFO.

|*|

|*| Entry Conditions:

|*|    AL holds the data

|*|

|*| Exit Conditions:

|*|    AX = -1 if data is available, or 0

|*|    DX,AX modified

|*|

\*/

public  AP_MDW

AP_MDW  proc

push    cx

mov     ah,al           ; save the command/data

;

apmdw_00:

mov     dx,[PortAddress]

add     dl,_CONTROL

mov     al,WrIdle       ; make sure its' idle

out     dx,al

Idledly

mov     al,ah           ; get the byte and write it out

sub     dl,_CONTROL     ; move to the data port

out     dx,al           ; load the data

inc     dx              ; move to the status port

sub     cx,cx

;

apmdw_05:

in      al,dx           ; wait=1 when cmd/data cannot be sent

test    al,stWait

loopnz  apmdw_05

jz      apmdw_10        ; its available

push    ax              ; The device is probably powered off, so

call    AP_reset        ; we just have to kick it...

pop     ax              ; AH holds the command/data byte

jmp     short apmdw_00

;

apmdw_10:

inc     dx              ; move to the control port

mov     al,WrSndData    ; start the strobe

out     dx,al

mov     al,WrIdle       ; end the strobe

out     dx,al

Idledly

pop     cx

ret

AP_MDW  endp

/*\

|*|----====< AP_SRQ_stat >====----

|*|

|*| Returns the status of Service Request

|*|

|*| Entry Conditions:

|*|   None.

|*|

|*| Exit Conditions:

|*|   Carry clear & AX = 0,  wait

|*|   Carry set   & AX = -1, service request is posted

|*|   DX,AX modified

\*/

public  AP_SRQ_stat

AP_SRQ_stat     proc

mov     dx,[PortAddress]

add     dl,_CONTROL

mov     al,WrIdle       ; Write Idle mode...

out     dx,al

Idledly

dec     dx              ; mov to the status port

in      al,dx           ; data rdy is high for data available

and     al,stSRQ

neg     al              ; set carry if non-zero

sbb     ax,ax           ; if data is available, return -1 & Cy

ret                     ; else return 0 & NC

AP_SRQ_stat     endp

/*\

|*|----====< AP_FM_out >====----

|*|

|*| Send index/data out to the parallel FM chip

|*|

|*| Entry Conditions:

|*|    AL holds the index

|*|    AH holds the data

|*|

|*| Exit Conditions:

|*|    None

|*|    DX,AX modified

\*/

public  AP_FM_out

AP_FM_out       proc

push    cx

push    ax              ; save the index/data

;

apfmo00:

mov     dx,[PortAddress]

add     dl,_CONTROL     ; move to the status port

mov     al,WrFMIdle     ; go into FM Mode

out     dx,al

Idledly

dec     dx              ; move back to status

sub     cx,cx           ; loop control

;

apfmo05:

in      al,dx

test    al,stWait       ; FM wait high?

loopnz  apfmo05         ; yes, dont write it

jz      apfmo10         ; its available

call    AP_reset        ; The device is probably powered off, so

jmp     short apfmo00   ; we just have to kick it...

;

apfmo10:

pop     ax              ; get the index and data

jnz     apfmobad        ; bomb out if we cant do it

dec     dx              ; move to the data port

out     dx,al           ; load the data port

inc     dx              ; move to the control register

inc     dx

mov     al,WrFMAddr

out     dx,al           ; start the strobe

mov     al,WrFMIdle

out     dx,al           ; end the strobe

Idledly

mov     al,ah

sub     dl,_CONTROL     ; move to the data port

out     dx,al           ; load the data port

add     dl,_CONTROL

mov     al,WrFMData

out     dx,al           ; start the strobe

mov     al,WrFMIdle

out     dx,al           ; end the strobe

Idledly

mov     al,WrIdle

out     dx,al           ; back to write idle...

;

apfmobad:

pop     cx

ret

AP_FM_out       endp

/*\

|*|----====< AP_FM_in >====----

|*|

|*| Read the FM status register

|*|

|*| Entry Conditions:

|*|    None

|*|

|*| Exit Conditions:

|*|    AL holds the status byte

|*|    DX,AX modified

\*/

AP_FM_in        proc

mov     dx,[PortAddress]

add     dl,_CONTROL     ; point to control port

mov     al,WrIdle       ; make sure we're idle

out     dx,al

Idledly

mov     al,RdFMIdle     ; go into FM Mode

out     dx,al

Idledly

mov     al,RdFMStat     ; clock in the data

out     dx,al

dec     dx              ; move to the status port

in      al,dx

mov     bl,al           ; save the high nibble

inc     dx

mov     al,RdFMIdle     ; all set

out     dx,al           ; strobe address

Idledly

dec     dx

in      al,dx           ; clock in the data

mov     bh,al

inc     dx

mov     al,WrIdle       ; go back to idle mode

out     dx,al

Idledly

mov     cl,4

shr     bh,cl

shr     bx,cl

mov     al,bl

ret

AP_FM_in        endp