TABLE OF CONTENTS

MapROM

MapROM                                                                 MapROM

NAME

MapROM - A Developer Tool for testing Kickstarts with the 3640 board

SYNOPSIS

MapROM <kickstartfile> [FORCE]

FUNCTION

MapROM is a tool that uses a feature of the A3640 (the Commodore

68040 CPU board in the A4000 and A3000/040 systems) to let developers

load disk-based kickstarts.

MapROM will check to see if you are already running the kickstart

that is contained in the file and if you have the memory required

for the kickstart.  If the kickstart file is different than the

current kickstart, it will replace it and cause a system reboot.

Kickstarts loaded with MapROM do not go away until a full power down.

The FORCE option can be used to force a reload of a kickstart image

even if it is the same one.

MapROM does *NOT* use the MMU of the 68040 CPU but some special

hardware on the A3640 card.

INPUTS

FILE/A	- Required file name.  It should be the name of the

developer kickstart file for your machine.  The A4000

and A3000 kickstarts are *different* and can not be

interchanged.

FORCE/S	- An option to FORCE the reloading of the kickstart even

if it matches.

RESULTS

If the kickstart is new and has loaded successfully, the system will

reboot and use that kickstart.  If not, MapROM quietly exits.

NOTES

In order for this tool to work correctly with other system tools

you should place it in the main C: directory.  Various developer

tools will look for it there.  (This is SYS:C)

Also, in order to use this tool, it *MUST* be run *ABSOLUTELY FIRST*

in the Startup-Sequence.  It *MUST* come before SetPatch as it

must not have the MMU turned on.  Since MapROM will either work

or quietly exit, there is no problem with doing this.

The standard setup is:

IF EXISTS C:MapROM

MapROM DEVS:Kickstart

ENDIF

SEE ALSO

Your developer non-disclosure agreement.  This is a *developer only*

tool and is not to be released to the public in any way.

BUGS

None?