ToneLoc v0.98

Reference Manual

by

Minor Threat & Mucho Maas

This is the reference manual.  You should read this AFTER reading

TL-USER.DOC unless:

1. You are already familiar with Toneloc, or

2. You want to get started quickly

------------------------

COMMAND LINE OPTIONS

------------------------

Here are the command line options for ToneLoc:

Toneloc  [DataFile] /M[Mask] /X[ExMask] /R[Range] /D[ExRange] /C[Config]

/S[StartTime] /E[EndTime] /H[Hours] /Q /T[-] /K[-]

ToneLoc must ALWAYS be run with at least one parameter, and if you

only use one parameter it MUST be the dialing mask to use.  If you only

give ToneLoc one parameter, the first 8 characters of the dialing mask

will also be the data file name.  So if you run "TONELOC 555-XXXX", the

mask will be 555-XXXX and the data file will be 555-XXXX.DAT.

/M - Dial Mask.  Use this parameter if you want to use a data file-

name other than your dialing mask.

/X - Exclude dial mask.  This tells ToneLoc to -NOT- dial certain

numbers.  For example, if your local Police department has

phone numbers ranging from 555-5000 to 555-5099, and you were

using the dial mask 555-XXXX, use the exclude mask 50XX. This

mask applies to the active part of the dial mask (in this case

XXXX) and therefore should be no more than 4 characters long.

If an exclusion mask is longer than 4 characters, ToneLoc will

ignore it.  Up to 9 exclude masks are allowed at once, but all

exclusions must be sub-sets of the dialing mask.

/C - Config File.  This tells toneloc to use a config file other

than the default, TONELOC.CFG.  You may change this for different

types of scans, but you'll probably want to make different

config files for different kinds of scanning or for different

kinds of modems.  For example, you could have a config file,

TONES.CFG, that is configured to scan for tones, and another

called CARRIERS.CFG configured to scan for carriers.

/S - StartTime.  Tells ToneLoc to wait until a certain time to begin.

You may either use military time or end the time with a "p" for

P.M.  Example:  5:30p and 17:30 are the same.

/E - EndTime.  Tells ToneLoc to STOP dialing at a certain time.  Use

the same notation as the /S parameter above.

/H - Hours.  Tells ToneLoc to dial for a certain number of hours

after starting, and then exit.  "/H5:30" would mean 5 hours,

30 minutes.

/Q - Quiet.  Tells Toneloc to suppress ALL sound.  The speaker

and all sounds will be turned off.

/T - Sets the scan mode to look for tones, overriding whatever is

specified in the config file.  /T- will look for everything

_except_ tones.

/K - Sets the scan mode to look for carriers, overriding whatever is

specified in the config file.  /K- will look for everything

_except_ carriers.

These parameters may appear in any order, EXCEPT the first parameter

MUST always be the datafile, and if you do not specify a dial mask with

/M it MUST ALSO be the mask.

-----------------

WHILE DIALING

-----------------

While ToneLoc is scanning, there are several keys that do various

things.  Here they are:

A - Blanks the screen totally black, and turns off the cursor.

Press 'A' again to unblank it.

B - Blanks the screen, but instead of making it black, it loads

the optional Alternate Screen file set in the config file.

Preess 'B' again to unblank it.

C - Logs the current number as a "Carrier".  Say you're scanning

for tones, but you hear a carrier, you can press 'C'.

F - Logs the current number as a "Fax".  Useful for when you

hear a fax machine while scanning.  A fax machine sounds like

a 300 baud modem underwater.

G - Logs the current number as a "Girl".  Useful if you hear

a really cute-sounding babe saying "Hello? HELLO? <click>".

J - Pressing 'J' will Jump to a DOS shell.  Upon returning,

ToneLoc will redial the last number.

K - Allows you to enter a custom note for the current number.

N - This will "Note" the current number by putting a special entry

in the logfile.  It looks like this: "* Noted *".  Use this if

you want to remember this number for later.

P - Pauses scanning until another key is hit.

Q - Quits scanning and exits AFTER the current dial is finished.

R - Redials the current number.

S - This toggles the modem speaker ON or OFF while dialing, but

ToneLoc will wait until the current dial is completed before

toggling the speaker.  If you press 'S' again before the

speaker is toggled, the speaker won't be toggled.

T - Logs the current number as a "Tone".  Say you're scanning for

carriers, but you hear a PBX dialtone, you can press 'T'.

V - Logs the current number as a "VMB", in case you hear a VMB

system during a scan.

X - This will extend the current dial time by 5 seconds.  The

meter will reflect the time addition.

Y - Logs the current number as a "Yelling Asshole".  Useful if you

hear some dick yelling "I KNOW ABOUT YOU HACKERS AND YOUR

2500 BAUD MODEMS!"

0-9 - These are the modem volume changing keys.  They will send

the corresponding string in the config file to the modem.  They

work like <S>peaker toggle.

[Spacebar] - Aborts the current number and goes on to the next one.

[Escape] - Pressing Escape will abort the current dial and

exit ToneLoc.

----------------------

CONFIGURATION FILE

----------------------

ToneLoc uses a configuration file.  Before running ToneLoc, you

need to edit this config file for your setup.  The config file has a

little bit of help in it, so its mostly self-explanatory.  But just

in case, here's an explanation of each keyword in the configuration file.

Fossil - This tells toneloc to use the FOSSIL modem routines, which

require an external FOSSIL driver, like BNU, to be loaded.

You should only use this if the built-in SERIAL routines don't

work.

Serial - This tells toneloc to use the fast, built-in modem routines.

If you use these, no external FOSSIL is requied.

Port - This is which COM port to use.  ToneLoc has been tested

on COM1 - COM4 successfully.  It should, theoretically,

work up to COM8.  If you use COM5 or above, you need to

specify an IRQ and base port address (below).

IRQ -  If you're using COM5 or above, or if your comport's IRQ

is non-standard, you need to specify this.  (see below)

IOAddx - If your comport's base I/O address is non-standard, or

if you are using COM5 or above, need to specify the base

I/O address.  Usually they are as follows:

PORT    ADDRESS     IRQ

----    -------     ---

COM1      3F8        4

COM2      2F8        3

COM3      3E8        4

COM4      2E8        3

Baud - This is the baud rate at which ToneLoc communicates with the

modem.  For a USRobotics HST or Dual Standard, it should be

set as high as the modem can handle (38400 or 57600).  If you

are losing characters on a high speed modem, try lowering

this to 19200 or 9600.

Speaker - This tells if the modem speaker should start ON or OFF.

The number 0 (off) or 1 (on) should follow "Speaker".  You may

toggle the speaker while dialing by pressing 'S'.

FoundSound - ToneLoc can make a noise when it finds a tone or carrier,

if you want this enabled set it to 1, otherwise set it to 0.

Sound - ToneLoc can makes noises for speaker toggles and volume

changes. If you want to hear these sounds, set this to 1.

InitString - This is the string that is sent to the modem when

ToneLoc is first run.  Usually this is ATZ|.  You may need to

send other parameters like X7, &N1, etc.  The following special

characters are used:

~ - pause for 1/2 second

| - send [Enter] to modem

< - drop DTR (don't forget to raise it)

> - raise DTR

! - drop DTR for 1/4 second, and raise it

These special characters are used with all modem commands.

InitResponse - ToneLoc checks for this after initializing the modem.

If the modem doesn't send this back, toneloc will try to

initialize the modem up to 3 times.  If it doesn't work,

ToneLoc exits.

DialString - This is the string to dial the modem.  Usually "ATDT".

If you need to dial *70 to disable call waiting, for example,

add it here like "ATDT*70".  Or if you need to dial "9" first,

put it here.

DialSuffix - This is the string to append to the DialString AFTER

the phone number.  Sometimes, adding a comma after the number

will help to avoid false VOICE detection on a 1AESS switch.

(The post-dial click may be detected as a VOICE).

SpeakOn - String to send to the modem that turns the speaker ON.

Usually "ATM1|" or "ATM3|".

SpeakOff - Same as above, but to turn the speaker off.  Usually "ATM0|"

Volume0-Volume9 - Strings that adjust the volume of the modem while

dialing.  Press '0' - '9' while dialing to adjust the modem

volume.  You could also use these as other macros besides

volume.

HangUp - String that tells the modem to hang up during a dial

attempt.  Usually "ATH0|"

CarrierHangup - String to hang up the modem when connected to another

modem.  Usually a quick DTR drop '!' will work.  If not, try

adding "&C1&D2" to your init string.  If that doesn't work,

try a longer DTR drop, like "<~~~>".  Finally, if that doesn't

work, try "~~~+++~~~ATH0|", but beware -- this is slow.

ToneHangup - String to hang up the modem after it finds a tone.

Some modems require "ATH0|" rather than just a simple DTR

drop.

LogFile - File that toneloc logs it's activity to.  Usually TONE.LOG

CommandDelay - Number of milliseconds to pause after sending a

command to the modem.  Usually about 150.

BetweenDelay - Number of milliseconds to pause between dial attempts.

If you have 3-way calling, or a slow switch, you'll need to

set this to a higher number.

BetweenWipe - How to 'wipe' the meter between calls.  Cosmetic.

1 = Ends to Middle

2 = Middle to Ends

3 = Backwards

4 = Double right

5 = Triple right

6 = No wipe

WaitDelay - How many milliseconds to wait for a carrier or tone

before hanging up and trying the next number.  Usually, for

tones you'll want 15000-20000 (15-20 seconds).  For carriers,

20000-30000 (20-30 seconds).

NoDupes - This tells toneloc to keep a .DAT file to avoid dialing

the same number twice, in case you abort in the middle of a

scan and continue later. Recommended.

MaxRing - If your modem reports RINGING, you can set this to abort

the call after X number of rings.  Since most PBX's answer

after 1 ring, and most carriers after 2 or 3 rings max, this

can speed up the scan.  Set this to 0 to disable it.

ScanType - Tells ToneLoc what to scan for:

0 = Tones

1 = Carriers

2 = Everything except tones

3 = Everything except carriers

If you're wondering why the hell you'd want to scan for

everything EXCEPT tones or carriers, see the section in

TL-USER.DOC explaining this.

IgnoreCD - If your modem always sets Carrier Detect (CD) as TRUE,

or if your modem is reporting false carriers to toneloc,

turn this on.  Otherwise, turn it off.

IgnoreCTS - ToneLoc checks for CTS at startup to make sure the

modem is on and ready.  If ToneLoc always says the CTS

signal isn't there, but your modem is actually on and ready,

turn IgnoreCTS on.  This will also disable periodic checks

toneloc performs to make sure the modem hasn't been turned

off during the scan.  (what a lame thing to do)

IgnoreUnknown - IgnoreUnknown tells toneloc to ignore modem

responses that are not recognized by toneloc.  If your modem

sends strange messages like "DIALING ...", turn this on.

AutoSave - This is how often ToneLoc should do a backup of the .DAT

file and the LOG file.  Set this to 0 to disable it.

BlackList - This tells ToneLoc the name of your BlackList file.  The

BlackList file is a file of phone numbers to never dial.  For

an example, see the file BLACK.LST.

Random/Forward/Backward - ToneLoc's default mode is to dial numbers

randomly without duplication, but it can also dial forwards

and backwards sequentially. Set this to the keyword you want.

Logging - This tells ToneLoc if it should log activity to a file.

NoToneAbort - If your line dies, and you get "NO DIALTONE" too many

times in a row, ToneLoc will abort.  This sets the limit

of how many "NO DIALTONE"s toneloc will allow before quitting.

GetDelay - This tells ToneLoc how many milliseconds to allow for the

modem send its responses.  Usually this is about 250, however,

if you are using a slow modem (300 or 1200 baud), it may need

to be raised to 400 or so.  If you're using a high speed modem

(38400+ bps), it may be set lower, but setting it lower doesn't

increase performance significantly.  If your modem's responses

are getting cut off, like "RIN" instead of "RINGING", or

"CONNE" instead of "CONNECT", you probably need to raise this

number a bit.

SlowModem - SlowModem - Some modems MAY require a delay between

characters sent by ToneLoc.  This specifies how many

milliseconds to pause between each character sent.  This

should be small (0 - 25 milliseconds).

FoundFile - This is the name of the file where ToneLoc should store

the found numbers (tones or carriers.)

AltScreen - When you press 'B' while scanning, ToneLoc will blank

the screen and load this binary file onto the screen.  There

are a few sample .BIN files that come with ToneLoc.  This is

useful for a "parent" screen.  If you want to create your own,

use TheDraw, and save the file in <B>inary mode.

FlogString - When ToneLoc finds a tone or carrier, it is logged

to the Found File (usually FOUND.LOG).  With FlogString, you

can customize the string that is written to the Found File.

Here are the symbols:

%d = Date

%t = Time

%n = Number

%b = Brief result (T, C, NT, NC)

%l = Long result (Tone, Carrier, No Tone, No Carrier)

%r = Response from modem (i.e: "CONNECT 9600/ARQ")

For example, "FlogString %n: %b, %r" would be logged like this:

486-1993: C, CONNECT 2400/NONE

-----------------

DESQview INFO

-----------------

ToneLoc runs most excellently under DESQview.  It will detect

DESQview, and if DESQview is found, screen writes are directed to the

screen buffer so your screen isn't trashed.  ToneLoc also releases

unneeded timeslices to DESQview.  Because of this, ToneLoc uses VERY

LITTLE CPU time, so you can run it in background with minimal slowdown.

I tested ToneLoc with two windows open.  One was a DOS window sitting

at the prompt idle (doing nothing).  The other was ToneLoc scanning.

ToneLoc used about 10% of the CPU time, and the idle DOS window used

about 88% (with about 2% DESQview overhead).

The file "TL-PIF.DVP" is included with ToneLoc.  To add ToneLoc

to your DESQview setup, do the following:

1. Run DESQview

2. Choose "Add a Program"

3. Type 'O' for other

4. Enter the path where ToneLoc is found

5. Move the cursor bar to "ToneLoc" and hit [Space Bar]

6. Press [Enter].

ToneLoc will be added to the list of applications on your DESQview menu.

To run ToneLoc, choose "TL" from the application menu.  You will be placed

at the DOS prompt with "ToneLoc " already typed.  Just finish the command

line and press enter.

---------------

FOSSIL INFO

---------------

ToneLoc no longer requires a FOSSIL driver.  However, if the built-in

SERIAL routines don't work, or you are using a very strange COMport, like

COM16 or something, you may need to use the FOSSIL routines.  The FOSSIL

routines aren't any less reliable or slower, but the FOSSIL routines

require an external FOSSIL driver program to be loaded.  The two most

popular FOSSIL drivers are X00 and BNU.  Just call your local PD board

and search for "FOSSIL", "BNU", or "X00".  ToneLoc works with both.

--------------------------------------------------------------------------

If this doc file sucks, please us know how to improve it.

-- Minor Threat

-- Mucho Maas