THE DESIGN CENTER 5.1

1.0)   Introduction

This file describes the new features and changes for the 5.1

release of the Design Center programs: PSpice, Probe,

Stimulus Editor, and Parts.  The platforms covered in this

file include: non-windows IBM-PC and NEC-PC with DOS/16M,

Windows 3.0 IBM-PC and NEC-PC, Macintosh, Sun, DECstation,

and VAX/VMS.  Most of the material in this file will direct

you to the location in the analysis manuals where detailed

information on each subject can be found.

The analysis documentation referenced in this README.DOC

consists of the following manuals:

"Analysis System Setup Manual"

"Analysis User's Guide"

"Analysis Reference Manual"

2.0)   PSpice

2.1)   DOS Only / Install Program for PSpice

The installation program used to install PSpice, INSTALL, is

licensed software provided by Knowledge Dynamics Corporation,

P.O. Box 1558, Canyon Lake, Texas 78130-1558 (USA).  INSTALL

is Copyright (c) 1987-1992 by Knowledge Dynamics Corporation

which reserves all copyright protection worldwide.  INSTALL

is provided to you for the exclusive purpose of installing

PSpice.  MicroSim Corporation is exclusively responsible for

the support of PSpice, including support during the

installation phase.  In no event will Knowledge Dynamics

Corporation be able to provide any technical support for

PSpice.

2.2)   Table 4 on page 52 of the Analysis Reference Manual

lists two new intrinsic functions for PSpice expressions.

Please refer to the manual for detailed information regarding

the results of each expression.

The new functions are:

TABLE(x,x1,y1,x2,y2,...xn,yn)

LIMIT(x,min,max)

2.3)   To link the DOS/16M version, you must use the Microsoft

segment-executable linker that comes with the Microsoft C

compiler.  To link the DOS version, we recommend that you

use the Plink86plus linker (version 2.24) from Intersolve.

For more information contact:

Life Boat Distribution Services

1163 Shrewsbury Avenue

Shrewsbury, New Jersey 07702

(800) 445-7899

2.4)   Fourier Analysis  (.FOUR command)

The .FOUR command for Fourier analysis now allows an

argument to specify the number of harmonics you wish

to have calculated.

The general format is:

.FOUR <frequency value> [no. harmonics value] <output variable>*

For example:

.FOUR 60Hz 20 V(17)

If the number of harmonics is not specified, the DC

component, fundamental, and 2nd through 9th harmonics

are calculated by default.

Refer to Chapter Three, page 62 of the Analysis Reference

Manual for information on the .FOUR command.

2.5)   Lossy Transmission Line Model

A Lossy Transmission Line model has been implemented in

version 5.1.  Using the distributed model approach rather

than the the traditional lumped model approach allows you

to simulate lossy lines in a significantly reduced amount

of time.

The general format is:

T<name>        <A port (+) node>  <A port (-) node>

+              <B port (+) node>  <B port (-) node>

+              LEN=<value> R=<value> L=<value>

+              G=<value> C=<value>

For examples refer to Chapter Four, page 127 "Transmission

Line" (T device), of the Analysis Reference Manual.

2.6)   Device Libraries

2.6.1) Digital Library Additions

The new library files added to the Digital Library are:

dig_pal.lib   -  Programmable Array Logic devices

Note: PAL20RA10 is listed in the library list,

but is not currently available in this library.

dig_gal.lib   -  Generic Array Logic devices

dig_ecl.lib   -  Emitter Coupled Logic 10K and 100K devices

Note: 100181 is listed in the library list, but is not

currently available in this library.

The devices and their pin lists are provided in tables in the

Analysis Reference Manual.  These tables can be found at the

end of Chapter Five on pages 288-292.

2.6.2) Analog Library Additions

The following libraries have been added to the 5.1 version

of PSpice:

europe.lib    -  European-manufactured semiconductor devices

harris.lib    -  RCA/GE/Harris power-transistor devices

swit_rav.lib  -  Averaging power supply devices

Note:  The Average Power Supply Model VMSSDCM is listed in

the library list, but is not currently available in this

library.

2.6.2.1) EUROPE.LIB

Some of the devices in EUROPE.LIB and their counterparts in

the regular libraries (eg. DIODE.LIB) have identical names.

In order for PSpice to find the correct model/subcircuit

definition, you must ensure that the appropriate library is

specified first in NOM.LIB.

If EUROPE.LIB is specified before another model library

containing duplicate names, none of the duplicate devices

in the second library will be accessed.  Therefore, if

D1N4148 and D1N4149 exist in both EUROPE.LIB and DIODE.LIB,

PSpice would take both the D1N4148 and D1N4149 definitions

from EUROPE.LIB which is listed before DIODE.LIB in NOM.LIB.

To avoid this from happening you can extract the specific

devices from the desired libraries and put them into a library

that is listed before any other library containing the device

with the duplicate name.

If you are using Schematics, the order of symbol libraries

specified in the [SCHEMATICS LIBS] section of msim.ini should

reflect the order of model libraries specified in NOM.LIB.

2.7)   Inertial and Transport Delay

PSpice uses two different types of internal delay functions when

simulating the digital portion of the circuit: "inertial delay"

and "transport delay".  The application of these concepts is

embodied within the implementation of the digital primitives

within PSpice.  Therefore, they are not user-selectable.

To obtain detailed information on Inertial and Transport Delay,

please refer to the Analysis Reference Manual, Chapter Four,

Section 4.8.4.4 (pages 100-101).

2.8)   Digital Worst-case Timing Simulation

With "digital worst-case timing simulation" (new for the

5.1 release), you will be able to use PSpice to evaluate

the timing behavior of your digital and mixed analog/digital

designs using a full range of component propagation delay

tolerances.  In Chapter Five of the Analysis User's Guide

you will find information on digital worst-case timing

simulation, starting with Section 5.5

on page 135.  The remaining sections and their subjects are:

5.5.1    Simulator Representation of Timing Ambiguity

5.5.2    Simulator Propagation of Timing Ambiguity

5.5.3    Identification of Timing Hazards

5.5.3.1  Convergence Hazard

5.5.3.2  Critical Hazard

5.5.3.3  Cumulative Ambiguity Hazard

5.5.3.4  Reconvergence Hazard

5.5.4    Glitch Suppression Due to Inertial Delay

5.5.5    Methodology

2.9)   PSpice as a Native Windows 3.0 Program on the IBM-PC and NEC-PC

The general operating instructions for Windows PSpice as a

native Windows 3.0 program can be found in Chapter Two,

Section 2.3.1.2, (pages 16-20) of the Analysis Reference Manual.

Windows PSpice will run on any IBM 386 or 486 based PC, IBM-PC

compatible computers, or NEC-PC running the Design Center

package with schematic capture.  This program requires a

minimum of one megabyte of memory available in the Windows

environment, plus the amount needed to run your particular

circuit files.

2.9.1) Windows PSpice Menus

To assist you in navigating through the Windows PSpice menus

here is a list of the new menus and the commands within each.

(a)  HELP MENU

About PSpice...

(b)  FILE MENU

Open

Terminate Simulation

Save Window Configuration

Exit to Windows

(c)  FONT MENU

System

ANSI Fixed

ANSI Variable

OEM

Other Fonts...

(d)  COLOR MENU

Text Color

Background Color

Error Text Color

Error Background Color

(e)  DISPLAY MENU

Immediate

Fast

Slow

3.0)   Probe

3.1)   Analog and Digital Plots

For the 5.0 and earlier releases of Probe, you could have

0, 1, or more analog plots, and 0 or 1 digital plots.

For the 5.1 release of Probe, you can have 0, 1, or several

plots.  Each plot can be analog only, digital only, or both

analog and digital.  When a plot is both analog and digital,

the analog traces are displayed in the bottom part of the

plot with a dashed border, and the digital traces are

displayed in the top part of the plot with a solid border.

When you start Probe with a .dat file that has both analog and

digital data, an empty analog plot is shown.  After adding

traces, the plot will have both analog and digital parts if

both analog and digital traces have been displayed.

Digital plot size is now specified as a percentage of the

overall plot size.  In Probe 5.0, there was only one digital

plot and the digital plot size was specified as the number of

digital traces the digital plot could display.

3.2)   Menus

The menus have been modified to improve the command flow for

displaying both analog and digital waveforms on the same plot.

The Analog Plot Menu and the Digital Plot Menu are now combined.

This new menu structure includes the digital_Trace_commands

which allow the user to move, insert, change, and scroll

digital traces.

3.3)   Remove_trace

Changes have been made to allow one or more traces to be

selected for removal, before actually removing the traces

(and doing the sometimes time consuming redraw of the

display).

The traces to be removed may be selected by using either the

mouse or the arrow keys and space bar.  Selected traces are

marked with a '<' to the right of the trace name

e.g. V(4)< ).

Using the mouse:

Clicking on the trace name the first time selects

the trace.

Clicking on the trace again deselects it.

Using the arrow keys and space bar:

When select is chosen, the first trace is

highlighted.  The right and left arrows will

move the highlight to other trace names.

The space bar selects/deselects the trace.

Also, an Undelete command has been added.  It will redisplay

the last trace(s) deleted since entering the Remove_trace menu.

Undelete only appears in the menu if traces have been deleted

since entering the Remove_trace menu.

NOTE: because of these changes, Probe command files created by

Probe 5.0 or earlier, will no longer work for deleting traces.

3.4)   Add Trace

Add_trace now allows you to specify analog traces, digital

traces, or both at the same time.  You cannot create an

expression that combines analog and digital nodes to create

one trace.

Digital expressions have been added, and the syntax for

specifying a bus has changed.

A digital signal can be any digital node in the data file or

a boolean expression involving digital nodes.  Please see

page 312 of the Analysis Reference Manual for information

on digital signals and digital expressions.

3.5)   Goal function search commands:

Limited expressions have been added for LEVEL(val),

and XVALUE(val).

In 5.0, val could be:

a floating number  (e.g., 1e5, 100n, 1, ...)

a percent (eg 50%)

a marked point (e.g., x1, y1, ...)

In 5.1, val can also be:

a db value relative to max or min

max-3db or -3db => 3db below maximum range

min+3db or  3db => 3db above minimum range

a value relative to max or min

max-3 => maximum range - 3

min+3 => minimum range + 3

a relative db value

.-3db => 3db below lastvalue

.+3db => 3db above lastvalue

a relative value

.-3   => lastvalue - 3

.+3   => lastvalue + 3

where lastvalue is the x or y value found by the last search

command.  For the first search command, lastvalue is the

starting point for the first search command.

Refer to page 320 of the Analysis Reference Manual for

more information on Search_commands.

3.6)   Probe as a Native Windows 3.0 Program on the IBM-PC and NEC-PC

Probe running as a Windows 3.0 application, is only available

in the Design Center package with schematic capture.  Windows

Probe provides similar functionality included in non-windows

Probe, implemented by utilizing pull-down menus and dialog

boxes.

To help you locate Probe functions, the following Windows

Probe main menus and their command items are provided.   For

details on the command defintions see Chapter Six, Section

6.3.2.1 (page 342) of the Analysis Reference Manual.

FILE MENU          ANALYSIS MENU           TRACE MENU

Load                 AC                    Add

Print                DC                    Remove

Printer Setup        Transient             Macro

Log Commands                               Symbols

Run Commands                               Mark Data Points

Exit

About Probe

AXIS MENU          ZOOM MENU                CURSOR MENU

Set Range          Display Pointer          Display

Add Y              Normal                   Peak

Delete Y           In                       Trough

Color              Out                      Slope

Title              Area                     Min

Y Log Scale                                 Max

X Log Scale                                 Search Commands

X Variable                                  Next Transition

Restrict Data                               Previous Transition

Fourier

Performance Analysis

DISPLAY MENU

Digital Plot Size

Save/Restore

Copy to Clipboard

Redraw

In general, all commands are available at all times, provided

that the command makes sense.  For instance, Remove in the

trace menu is enabled only when there are traces to remove.

Some options, such as Fourier, are toggles which can be

either on or off.  On is denoted by a check mark next to the

command, and off is denoted by the absence of the check mark.

Some commands require some prior action before they will work.

Trace Remove:  You must first select the traces to remove.

This is done by pointing at the name of the trace in the

legend and clicking the left mouse button.  You may

select multiple traces by holding down the shift key while

selecting the trace names.

Zoom commands: You must enable the zoom pointer before you

zoom in or out.  You may move the zoom pointer with the

mouse by clicking over a point of interest.  Zooming in

and out works by a factor of two around the zoom pointer.

If you "click and drag" (press and hold the mouse button,

move the mouse, release the button), you can define an

area to zoom into using the Zoom Area command.  Zoom Normal

is always available.

Cursor commands: You must display the cursors before you can

use any cursor commands.  If the command has implied

direction, such as Peak, then the direction moved is the

same as the last direction moved, or right if there was no

last direction.  Likewise, the cursor moved will be the same

as the last cursor moved.  Next Transition and Previous

Transition are not available in the beta version.

Features not available through the menus:

Digital Scrolling:  To scroll the digital plot, use the mouse

to click on the "+" signs in the digital legend.  They will

only appear if there are additional digital traces not

currently displayed.

Compatibility with non-windows Probe:

Log/Command Files:  These are specific to the windows version.

Macro Files (.mac): Compatible

Display Files (.dsp): Compatible

Data Files (.dat): Compatible

Data Files (.txt): CSDF data files are not available in the

beta version.

Goal Function Files (.gf): Compatible

4.0)   Parts

If you are running on an NEC-PC or IBM-PC (or compatible)

then you will need 510 kilobytes of conventional memory free

to run Parts.

4.1)   Non-linear Magnetic Core Model

A non-linear magnetic core model has been added and uses

a derivative of the Jiles-Atherton formulation to provide

a closed-form, analytic solution.   For detailed information

please refer to Chapter Eight, page 414, of the Analysis

Reference Manual.