Orchid Designer - ProDesigner VGA

Windows 3.0 Driver

Readme file

��������������������������ĴIMPORTANT���������������������������������Ŀ

�                                                                      �

� The instructions which follow are applicable to users of Microsoft   �

� Windows 3.0 and the Orchid Designer and ProDesigner VGA display      �

� adapters. Please note that the Orchid Display Adapters with 256 KB   �

� will support the 640x480 (16 color and monochrome) drivers on this   �

� disk, as well as the 800x600, 16 color driver. The Orchid Display    �

� Adapters configured with 512 KB RAM will support all of the          �

� drivers on this disk. Attempting to install and run the 512 KB       �

� drivers on a 256 KB display adapter will cause display problems      �

� with Windows 3.0. Please use the appropriate drivers for your        �

� display memory configuration.                                        �

�                                                                      �

� Please be sure your monitor supports the drivers you wish to run.    �

�                                                                      �

������������������������������������������������������������������������

Please read this information carefully.

The Designer and ProDesigner VGA drivers on this disk contain drivers for

several popular modes, including support for 800 x 600 with 16 colors.

Also included is a replacement Windows 3.0 SETUP.INF file that will permit

you to change your display options from within Windows 3.0.

Follow the instructions listed below to install the drivers:

�����������������������������������������������������������������������

Quick Install (These instructions assume you are an experienced user)

1. Install Windows 3.0 in the usual way, selecting the appropriate

standard IBM display mode as your video display adapter.

2. Copy the contents of the Designer - ProDesigner VGA Windows 3.0

Driver Disk to your Windows\System directory.

3. When you are running Windows 3.0 you may change your display type

by using the Windows Setup program residing in the Main desktop

window.

�����������������������������������������������������������������������

Standard Installation:

1. Use DISKCOPY to make a copy of the Windows 3.0 Setup disk (Disk 1).

2. Return the original Windows Setup Disk to its disk sleeve.

3. Using the copy of the Windows Setup Disk, copy the file SETUP.INF

from the Designer - ProDesigner VGA Windows Drivers disk to the

newly-created copy of the Windows Setup Disk.

4. Still using the copy of the Windows Setup Disk, run the Windows 3.0

Setup program to begin installation of Windows 3.0.

5. When the configuration list is shown, review it for any changes you

wish to make. If you wish to change the display type, you may select

an appropriate Designer - ProDesigner VGA driver from the Display

Driver list.

6. Windows will prompt you to insert the Designer - ProDesigner VGA

Windows 3.0 Driver Disk into your floppy drive to install the

requested driver. Once the appropriate files have been copied, the

Windows Setup installation program will continue normally.

7. After you complete the above steps you can begin using Windows 3.0.

Once Windows 3.0 is installed you may change your display type by

using the Windows Setup program residing in the Main desktop

window.

�����������������������������������������������������������������������

Please note the following:

When you run Windows Setup to change your display driver, Windows

may prompt you to insert other Windows Installation diskettes into

your disk drive. This is because the Designer - ProDesigner VGA

Windows display drivers use display fonts which are contained on the

standard Windows 3.0 distribution diskettes. Have your Windows 3.0

disks handy.

�����������������������������

When selecting your Windows 3.0 driver please be aware of the

Designer - ProDesigner VGA memory required for each mode. The list

below lists the drivers supplied on this disk, as well as the amount

of display memory required for each mode (graphics modes are given

in hexadecimal):

Resolution  Colors    Mode      Display Memory     Driver Name

����������������������������������������������������������������

640x480       2      11h           256K             VGAMONO.DRV *

640x480      16      12h           256K             VGA.DRV     *

800x600      16      29h           256K             VGA364.DRV

1024x768      16      37h           512K             VGA374.DRV

640x480     256      2Eh           512K             VGA348.DRV

* Uses standard VGA drivers supplied on Windows 3.0 diskettes.

�����������������������������

IMPORTANT!!!

Please make sure your monitor is able to support the resolution

you select.

�����������������������������

IMPORTANT!!!

It is important that the drivers be installed properly,

replacement grabbers (GR2 extension) and Virtual-Display-Device

handler (386 extension) are included to properly support the

extended feature set of the Orchid Designer and ProDesigner VGA

Plus display adapters.

�����������������������������