This Disk contains a Printer Driver for use with the Canon BJ-10e

Bubble Jet Printer and Commodore Amiga Computers.

The following files are included on the disk:

CanonBJ10           -    Printer Driver revision 1.3

Readme              -    This file

Notes

Setup and connect your BJ-10e as described in the BJ-10e

Users Manual.

Set the printers DIP Switches as follows:

Switch         Function                 Setting

1       Auto Sheet Feeder            As Appropriate

2       Graphics image density       As Appropriate

3       Automatic Line Feed          OFF

4       Page length                  On Europe/Australia OFF USA

5       Character Set                ON

6       Automatic carriage return    OFF

7       Alternat graphics mode       OFF

8       buffer/download memory       OFF

9       Code page                    OFF

10       Printer control mode         ON

General Printer Driver Information

1.   Bubble Jet black/white and grey scale printer.

2.   The driver supports the following graphics dump densities.

Density   X-DPI     Y-DPI     X*Y-DPI

1    360       360       1296000

2    180       180         32400

3    120       360         43200

4    180       360         64800

5    240       360         86400

6    360       180         64800

7    360       360       1296000

3.   A PaperSize of 'Wide Tractor' selects a maximum print width

of 13.6" this is applicable to wide carriage printers only

and not to the BJ-10e

4.   A PaperType of "Single" ejects a sheet with a Form Feed

after every print.

5.   This driver automatically checks for the processor in use

and optimizes the print routines accordingly.

Hints and Tips

1.   If using Pagestream 2.1 select density 0 to obtain 360x360

DPI resolution and not density 6.

2.   If your output is striped or misses print lines then check

your printers DIP switch settings and software setup

values. Check especially page settings such as margins.

3.   Data should only be sent to the printer either via

parallel.device (par:) or printer.device (prt:) and not

both together as this may confuse the printer.

The driver is available via modem:

WildCat

FIDO 2:243/43

++49-6173-2544 (Germany)

1200-14400 bps HST/V32/V32.BIS

SETTING: 8N1