This Disk contains a Printer Driver for use with the Canon BJ-

300/330 Bubble Jet Printer Series and Commodore Amiga Computers.

The driver is designed for use when the BJ-300/330 is configured

for IBM Proprinter mode. ie SW1-1 and SW1-2 are set to on.

Contents

The following files are included on the disk:

Canon BJ300         -    Printer Driver revision 1.3

Readme              -    This file

Notes

Setup and connect your BJ-300/330 as described in the BJ-

300/330 Users Manual.

Set the printers DIP Switches SW1-1 and SW1-2 to the ON

position all other switches may be left at default or

customised appropriately.

General Printer Driver Information

1.   Bubble Jet black/white and grey scale printer.

2.   The driver supports the following graphics dump densities.

Density   X-DPI     Y-DPI     X*Y-DPI

1    360       360        129600

2    180       180         32400

3    120       360         43200

4    180       360         64800

5    240       360         86400

6    360       180         64800

7    360       360       1296000

3.   A PaperSize of 'Wide Tractor' selects a maximum print width

of 13.6" this is applicable to the wide carriage BJ-330

printer only.

4.   A PaperType of "Single" ejects a sheet with a Form Feed

after every print.

5.   This driver automatically checks for the processor in use

and optimizes the print routines accordingly.

Hints and Tips

1.   If using Pagestream 2.1 select density 0 to obtain 360x360

DPI resolution and not density 6.

2.   If your output is striped or misses print lines then check

your printers DIP switch settings and software setup

values. Check especially page settings such as margins.

3.   Data should only be sent to the printer either via

parallel.device (par:) or printer.device (prt:) and not

both together as this may confuse the printer.

The driver is available via modem:

WildCat

FIDO 2:243/43

++49-6173-2544 (Germany)

1200-14400 bps HST/V32/V32.bis

SETTING: 8N1