(c) Wolf Faust

Am Dorfgarten 10

W-6000 Frankfurt 50

Germany

Fido 2:243/43.5

UUCP:cbmvax.commodore.com!cbmehq!cbmger!venus!wfaust

Files you must get with this readme:

Canon48.020  -  68020/68030 Driver

Canon48      -  68000/68010 Driver

Readme       -  This textfile

26.10.90  Version 35 Rev 1

10.12.91  Version 35 Rev 1.5

- Elite Bug (Setting Elite in Preferences) removed.

This bug causes a "MW" while initialising the

printer, also Autoline was setting on, which

causes a LF with every CR (so you got to much

lines in your printing...)

- known bug left: transfer.a is not

set right for 360YPI

- reverse P in char table fixed

10.12.91  Version 35 Rev 2.0a

- Bug in transfer.a removed

new transfer.c should now work with 360YDPI

10.12.91  Version 35 Rev 2.0b

- transfer.c translated to assembler, so graphicdumps

should be faster...

Description:

------------

This  driver  is  intended  for  all those Canon compatible inkjet

printers (BJ-130...) out there.  Because of improved routines this

driver  is  very fast compared to the commodore driver.  Also this

driver  supports  more  graphic  and  text  modes  (see  below for

additional features).

For  the latest  version call the above given FIDO/UUCP address or

send  me  a  selfaddressed  letter  and  $15  (this  money is only

intended as donation for paying my postage and printer handbooks).

Further donations won't be refused ;-)

Improved   EpsonLQ, Oki 39x and  Star24  (... ?)  drivers  are  also

available (see note below)!!!!!

Canon Capsule 48/XL (BJ-130 and compatible 48 inkjet printers)

--------------------------------------------------------------

1. 48-inkjet printer b&w/greyscale printer (text and graphics).

2. Density	XDPI	YDPI	XYDPI	Comments

1	180	180	 16200

2	180	180	 21600

3	180	180	 32400

4	180	180	 43200

5	180	360	 64800

6	360	180	 64800

7	360	360	129600

3. A PaperSize of 'Wide Tractor' selects a maximum print width of

13.6 inches (for wide carriage printers).

4. A papertype of 'SINGLE' ejects the sheet with a formfeed after every

dump.  Formfeed after a graphic dump is send according to the

SPECIAL_NOFORMFEED flag.

Improvements added (compared to epsonx)

---------------------------------------

1.  Support of more graphic resolutions (360 DPI) !

2.  The driver has been speeded up. Now it's much faster than

a standard WB driver.

3.  Extra driver with support for 68020/30 should be included with

this package.

4.  Escape sequence for shadow print, fonts... added.

5.  This driver supports alphabetic accents and other special

characters as far as possible. Please note

that this driver has some limitations: some alphabetic Accents are

not supported.

6.  There is now support for selecting different typefaces.  Due

to the extended character translation table, the character set

selection commands (aFNT0 - aFNT10) are no longer really needed.

The following fonts are yet supported (some only with additional

cartridges):

0 - Internal ROM CG

1 - RAM CG

2 - External ROM CG 1 (font cartridge 1)

3 - External ROM CG 2 (font cartridge 2)

4 - Character set 2

5 - Character set 1

6 - not supported

7 - not supported

8 - not supported

9 - not supported

10 - not supported

Hints & Bug Report & Wishes

---------------------------

1.   If  possible  don't use the command ESC-'A'.  If you have to,

don't  forget  to reset the printer to 1/6inch linefeed!  There is

no extra 1/6inch linefeed command, so the driver can't do this for

you!

2.   Set DIP switch 2-6 ON. Character Set 2 should be default!

3.   The printer does no perforation skip with graphic dumps!

So watch out!

4.   Set DIP switch 1-3 (CR only) and 1-5 OFF!!!!!!!

5.   In case of a real bug make sure you have the latest release

and if so, drop me a note and it will be fixed with the next

release.

6.   In case of stripes and empty lines check your printer

margins (DIP switches, preferences setting, settings within

the program you use)!

Notes

-----

After my first release of an EpsonLQ compatible printer driver I got

many letters from people asking for an unsupported  printer driver.

Please:

1. include some money if you wan't to get answered. Postage takes a

major part of my income (as student I have no real income :-(  )

2. in case you found a bug: please give me a full description. In

most cases I can't verify your statements!

3. if you wan't some improvements or you have wishes: drop me a note,

4. if you don't have a driver for your printer or your driver

doesn't fully support your printer: drop me a note with a

description of your printer and maybe I'll do this for you...

(in case you have some bucks to much: don't wait for an answer

and send me a copy of your manual. But remember: there is no

guarantee I'll do the job!)

There will be no single typestroke without a complete copy of

the printer manual!!

5. Because this is the first version of the driver you can expect

more to come (I'm working on RLE and spooler support).

If you use this driver, look out for a new version...