The Complete Video Test System

Dave Muse

Version 1.0

March, 1994

INTRODUCTION

The Complete Video Test System is a collection of twenty test

patterns that you can use to evaluate, troubleshoot, or compare video

equipment.  Many of our tests are unique, and can be found nowhere

else.  This text will explain the tests in clear, non-technical

language, and show you how to use your Complete Video Test System to

make purchasing decisions, adjust your equipment for optimum

performance, or just demonstrate the quality of the system you have.

It is important to remember that, just as the strength of a chain

is limited by its weakest link, the results of tests performed on your

video system can only be as good as the poorest piece of equipment

that the video signal must travel through. I will point out, as we go

along, what types of problems with your system could result in low

scores on the various tests.

In addition to helping you evaluate your system, I will be

telling you how to make adjustments to your TV or monitor to improve

it. These adjustments are optional, as you may have to remove the back

of your set to get some of them done.  Many sets have an interlock to

disconnect the power cord when the back is removed. You can defeat

this by removing the end of the cord that is attached to the set's

back and plugging it directly into the chassis (if you observe how the

back goes onto the set, you will see how it connects the power cord to

the chassis).  THIS INTERLOCK IS DESIGNED INTO YOUR SET FOR A REASON!

Working in back of your set with the power on can be dangerous!  Be

careful not to touch anything except the controls we are talking

about, stay away from high-voltage areas (the high voltage cage and

the picture tube), work with one hand only on the set (and keep the

other one away from anything metal), and sit or stand on a dry,

insulated surface while you work (like a cushion or carpet). Last and

most important, don't sue me for your electrocution!  I've just warned

you, and you're on your own!

CVTS is available as software that runs directly on an AMIGA

computer system, or as a videotape playable on any of several types of

VCRs.  If you have the option, better results will be obtained by

using CVTS directly from your AMIGA system rather than from tape.

Most of the tests, however, will work just fine from tape with no

compromise in function.

USING YOUR SOFTWARE

When using the software version of CVTS, remember that these

tests are designed for NTSC television systems.  This means you will

need a genlock or NTSC encoder if one is not built into your system.

The black-and-white video output on the AMIGA 500/2000 will work

with a number of the tests (the ones that do not require color), and

in fact I recommend using this output for as many tests as possible,

if your genlock or encoder is not a high quality unit.  The tests you

will need color composite output for are:  COLORBARS, COLOR PURITY,

CHROMA RESOLUTION, CHROMA NOISE, COLOR CRAWL, and the MULTIPLE TEST.

Black-and white output can be used for all the remaining tests, even

when you are looking for the color symptoms mentioned and making the

requested color adjustments.  Users with monochrome only video output

that do not wish to obtain an encoder can purchase a tape to run the

color test patterns.

If you want to try some of the CVTS tests on your RGB computer

monitor, be my guest.  All you'll prove is how incredibly superior RGB

is to NTSC!

The tests consist of pictures and animations displayable with any

IFF picture/animation viewer.  An easy way to tell whether a given

test is designed for a TV or a VCR is to look at the icon.  A drawing

of a monitor is represented on some of the icons, and a video tape on

others.  A test that is intended for both will have a drawing of a

monitor with a VCR on top.  One test, the camera resolution chart, has

a camera lens represented on its icon.  This video camera test is the

only one that needs to be printed before use.  The easiest way to

print it is to run Graphicdump (it came with your Workbench, in the

utilities drawer) and display the test.  It could also be loaded into

a program like Deluxe Paint and printed from there.  This test is

specifically designed to work well regardless of the quality of the

printer you have.

GUARANTEE?

The video tests that make up CVTS are designed for informal use

by both video professionals and non-professionals alike.  CVTS is not

and cannot be designed to the same rigorous technical specifications

as professional test equipment, and results from the former should not

be given the same weight as results from the latter.  I can't

guarantee that you will make correct video purchasing decisions just

because you own CVTS - as much as I hope and think you will.  I can't

guarantee that you won't damage yourself or your video equipment in

the course of using CVTS (please be careful)!  You'll have to assume

the full risks for these things.  So what do I guarantee?  I want you

to be happy with CVTS, so if you've purchased this package or anything

else from me and you're not pleased with it, you may return it for a

refund at any time.

DISTRIBUTION NOTICE

CVTS is copyrighted.  This package is freely distributable, but

not public domain.  Distribute it as you wish - just keep everything

together, this document included.  Since this version of CVTS is

shareware, you would (hopefully!) feel an obligation to contribute

money to use it.  Instead of a guilt-inducing diatribe from the author

about $$, how about this:  If you like this product and plan to use

it, I hope you will consider purchasing a colorbar viewing filter,

printed manual, videotape, or other product from me.  They will

enhance your copy of CVTS and make it more useful to you.  Purchasing

any product (see the included order form)  will register you and put

you on my mailing list for updates to CVTS.  Yes, new tests and

improvements to existing ones are in the works.  I am also quite

receptive to feedback.  If you spot something wrong, don't curse me -

write me a letter instead!

THE TESTS

Here is a brief description of each test and instructions for its

use.

COLOR BARS

Everybody has seen the color bar test pattern before. It's a

standard test signal, used throughout the television industry, for

adjusting video signals. Professional television monitors frequently

have a switch, labelled "blue only" , which is meant to be used with

the color bars for precise adjustment. By using the blue viewing

filter supplied with your kit, you can adjust your TV or monitor the

same way professionals do. Here's how it works:

(1)  Find the control labelled "Color" on your set (it can also be

called "Chroma" or "Color Level").

(2)  Display the Color Bars on your monitor or TV, then look at the

screen through your filter.  CONTINUE TO USE YOUR VIEWER for steps two

through five. The odd-numbered bars should appear blue, while the

even-numbered ones are very dark, almost black. Observe that each blue

bar has a smaller blue box just underneath it.  While adjusting the

COLOR control, compare the smaller boxes of the FIRST and LAST bars to

the larger bars above them. When they are exactly the same brightness,

the COLOR control is set correctly.

(3)  Find the control labelled "Tint" on your set (this control is

sometimes called "Hue" or "Phase").

(4)  While observing the two MIDDLE blue bars, adjust the TINT control

so that the smaller boxes underneath them are the same brightness as

the larger bars.

(5)  If the smaller boxes underneath the FIRST and LAST bars no longer

match the bars above them, then go back to step one and repeat these

adjustments.

(6)  Put down the blue viewer.  Look for three small black bars in the

bottom right quadrant of your screen.  If you can't find them, turn

your brightness up.  When your TV is properly adjusted, the third

(rightmost) of these boxes will be just barely visible - and the other

two, invisible.  Adjust your contrast and brightness controls until

this happens.  The reason you don't want to see the first black box is

that it's actually "blacker" than black.  Normal video material will

never get blacker than the middle box, which is official television

"black".

Your TV or monitor is now perfectly adjusted to display color video.

GREYSCALE

Providing that you have just set up your monitor using the color

bars, you should be able to distinguish all of the shades of black,

grey, and white in this test pattern.  If not, try adjusting your

CONTRAST and BRIGHTNESS controls (some sets have a PICTURE control

instead of BRIGHTNESS).  Remember that the darkest shade of grey

should be just barely visible apart from the black background.

Observe, as well, whether inappropriate color tints come and go as

these controls are rotated through their ranges.

If some or all of the shades appear to be tinted with a color, it

may be a sign that the "grey-scale tracking" or "color balance" of

your set is out of adjustment. You can adjust this yourself, but you

will need to look around for the controls in the back of your set. It

may be necessary to remove the back cover to find them.  What you need

is a set of adjustments labelled "RED SCREEN","BLUE SCREEN", and

"GREEN SCREEN". Alternatively, there may be adjustments called "RED

DRIVE (the word GAIN may be used in place of DRIVE),"BLUE DRIVE", and

"GREEN DRIVE".  If BOTH sets of controls are present, remember that

the SCREEN controls should be adjusted while observing the DARKER

shades, or black, and the DRIVE controls adjusted while looking at

LIGHT shades, or white.  If you haven't done something like this

before, be careful, especially if the back of the set is removed.

Don't touch anything inside the set besides the controls you need

(high voltages are present). Use a mirror so you can see the screen,

and adjust the controls slowly.  If turning a knob doesn't produce the

effect you want, turn it back to where it was.  A minor adjustment is

probably all you need, so try to find the RIGHT knob to turn instead

of turning ALL of them!  Obviously, if your picture has a red cast,

you will want to turn down the red control a little bit. If it has a

yellow cast, though, you would need to add some blue. That's because

of the way the three light primary colors mix to make white. Here's a

quick guide:

SINCE THE                       YOU NEED        OR YOU NEED

PICTURE LOOKS TOO:              TO ADD:         TO REMOVE:

RED                         BLUE AND GREEN         RED

BLUE                         RED AND GREEN        BLUE

GREEN                       BLUE AND RED         GREEN

YELLOW                           BLUE          RED AND GREEN

CYAN (BLUISH GREEN)              RED          BLUE AND GREEN

MAGENTA (REDDISH PURPLE)         GREEN         RED AND BLUE

The above adjustments are for tinted shades of grey, white or black -

usually affecting the entire screen at once - not a "splotch" of color

that stays in one spot on the screen. If you have this symptom, or if

a grey square appears colored BUT its identical partner on the other

row is NOT colored or different colored, you have a COLOR PURITY

problem.

COLOR PURITY

In this test, we are determining whether the electron guns are

hitting the right phosphors on the face of the picture tube.  What you

are looking for on each of the colored screens is a small patch of

"wrong" color - a green spot on the white screen, for example. Do not

be alarmed at seeing "streaks" on the colored screens that come and go

(if you are playing back this test from tape).  These are caused by

"noise" and are a normal part of the way tape reproduces colors.  Look

instead for a small-to-medium sized, fuzzy, stationary blotch of

color, probably near an edge or corner of the screen.

The test consists of several colored screens.  Each screen should

be a solid color from edge to edge.  Is each shade of color consistent

over the entire screen, or does it vary at any spot?  Normally, color

purity has been checked and set at the factory - its adjustment is

located on the neck of the picture tube - and you should not need to

readjust it.  What a color purity problem usually means is that your

picture tube has been subjected to a magnetic field, and may need to

be de-magnetized, or degaussed.  Most newer sets have a "degaussing

coil" built in to take care of this automatically - yours could be

malfunctioning or not be adequate to demagnetize your tube.  See your

local TV repair shop to have your tube degaussed - or borrow a

degaussing coil to do it yourself.  In addition to using this test,

sometimes the GREY SCALE test works better for spotting subtle color

purity problems.

It is important to keep sources of magnetism away from your TV

screen. Your picture tube can pick up magnetism from a telephone on

top of the set (the bell generates magnetism when it rings), a floppy

drive next to your monitor, a large loudspeaker closer than about two

feet, or even a vacuum cleaner running too close to your TV.

LINEARITY

This test pattern will show you how accurately your monitor or TV

set can display things like lines, boxes, and circles. The first thing

you should look at is how well centered the picture seems to be. Can

you see about the same amount of each of the four circles in the

corners?  Next, note how level the picture is. If it appears tilted,

the YOKE (the large coil mounted on the neck of the picture tube) may

have been incorrectly installed at the factory, or else has become

loose and has twisted.  Now, look at the large circle. Does it look

perfectly round? If it's squashed or misshapen, that's a linearity

problem.  Observe that the lines appear straight and evenly spaced,

too.  The moving dashed lines will help you spot any sweep

irregularities.  Let your eye follow one of the dashes across the

screen.  Does it's length change as it moves?  Does it appear to

travel up and down a "hill" anywhere?  If you'd like to improve the

situation, look for two controls in the back of your set called VERT

LINEARITY and VERT SIZE. Use your mirror to see the screen while you

adjust them. You will find that these two controls interact with each

other - that is, they both affect the vertical height of the picture -

so you will need to adjust both controls simultaneously.  Use the

VERTICAL LINEARITY adjustment to make the large circle look as round

as possible while turning the VERTICAL SIZE knob to keep the picture

the right height.

Some older TV sets and monitors have a PINCUSHION control.  Its

purpose is to compensate for the curved screens of some older picture

tubes by "pushing in" the scan lines at the top of the screen.  If the

lines at the top or bottom of your screen ONLY look bowed or wavy,

look for a PINCUSHION control in back of your set.

CONVERGENCE

Since it takes a mixture of the three primary colors to create

all the colors we know, it probably comes as no surprise to know that

there are three "electron guns" inside your picture tube, one for each

of the primaries. The tricky part of all this is getting the beams

from those three guns to strike the screen in exactly the right

places.  If they don't line up just right, colored fringes will appear

around objects on the screen.

Display the dot screen and look right in the center of the

screen. Do you see white dots or multi-colored dots? Most sets are

pretty good at getting the dots white in the middle of the screen. Now

look in one of the corners. If you look closely at one of the dots, it

will probably appear to be colored around the edges, or it may have

split completely into two or three colored dots.

If the convergence of your set is particularly bad (that is, the

dots don't appear white even at a distance) you may wish to adjust the

convergence of your set. There are typically two sets of convergence

adjustments, called STATIC and DYNAMIC. The STATIC adjustments are

little tabs connected to magnets on the neck of the picture tube, and

do not usually need adjustment outside of the factory. They are used

to get the center of the screen lined up.  The DYNAMIC controls are

needed to get the screen converged around the edges, and may need to

be touched up every few years.  These controls may or may not be

clearly marked, so look carefully for a small group of electronic

controls, possibly on a circuit board of their own, near the back of

the set or even on the picture tube neck.  We're talking screwdriver

type adjustments here, so find a small insulated screwdriver and/or

coil adjuster to use. I have no ideal strategy for adjusting these, as

every TV or monitor is a little bit different, and the convergence

adjustments can be difficult. If you would like to experiment, then

use your mirror to see the screen, proceed carefully, and turn each

control slowly to see what effect it has on the picture. If It doesn't

do what you want, turn it back to where it was!

FOCUS

I have provided a black-on-white dot screen for checking and

adjusting FOCUS, but the same dot screen used in the convergence test

above is also useful - try them both.  Do the dots appear as tiny,

well-defined little rectangles, or as fuzzy round blobs? Many sets

have a FOCUS control which is accessible without removing the back. If

you have such a control, try adjusting it while observing the dot

screen.  You want to make the dots look as small and well-defined as

possible.  If you find it impossible to get both the center of the

screen and the edges to look sharp at the same time, a compromise

setting of the control may be necessary.  Picture brightness often has

an effect on image sharpness as well.  To check this, see whether both

of the dot screens play with equal focus - and try turning the

BRIGHTNESS control up and down as well.  A well-designed and built set

will not lose very much focus as the brightness control is turned up,

but some sets lose quite a bit of their image quality. If yours is in

this category, you may have to choose between a bright picture or a

sharp one!

BLOOM

Blooming refers to the tendency of the size of the RASTER (the tv

image projected on your picture tube) to increase with brightness.

When you run the BLOOM test, you will see a cross made of two

graduated lines, alternately black on a white background, then white

on black.  To see how much size variation your TV has, you will need a

tape measure.  Measure from point A to B on the black screen, then

again on the white screen.  Divide the larger number by the smaller.

Do the same thing again, measuring from C to D this time.  A good

score on this test will be as close to 1.00 as possible.

WINDOW

The WINDOW test, despite its simple appearance, presents a

challenge to your video system.  Turn your BRIGHTNESS and CONTRAST (or

PICTURE) control up and down with the window test displayed and look

for problems.  Some things to look out for:

- Are all the sides of the large white square straight and even, or

are there waves and kinks?

- Are there streaks or smears running horizontally across the

picture?

- Is the square white, or tinted with a color?

- Do you see "ghost" images of the sides of the square (possibly

multiple ghosts, especially with the brightness turned up -- and with

the peaking or sharpness control turned up).

- Are the square edges sharp or blurry?

Good results on this test require a solid, flat response from the

electronics in all your video equipment.  A problem could be caused by

several things.  If there is a peaking or sharpness control on your TV

or VCR, make certain it isn't turned up too high.  If you are using a

monitor rather than a TV, make sure that it is terminated properly.

Check for a termination switch on your monitor.  If video "loops

through" your monitor on it's way to another piece of video equipment,

the last video connection in the chain needs to be terminated.

CENTERING

A quick look at the concentric boxes on your screen should tell

you how well centered your monitor is.  Can you read the same letter

on both the right and left sides of the screen?  Follow the outermost

box all the way around the screen to judge your top/bottom centering.

You can quickly gauge the amount of overscan your set has, as

well.  The outermost box is labelled "A".  Which letter can you read

at the edge of your screen?  For a more precise overscan measurement,

see the next test.

OVERSCAN

It's almost impossible to frame a picture without covering a

small part of it behind the frame.  The same thing happens with the

video image displayed on your set.  The image is purposely made just a

little larger than your screen, so that you won't see the edges.

While a small amount of lost image, called overscan, is acceptable, a

good monitor or TV will crop very little off the sides of your

picture.

To measure overscan in your set, run the test until the expanding

box begins to leave the screen. If you pause the tape (or stop the

animation, if playing from computer), you will be able to read the

percentage of overscan, or lost image in your monitor or TV. A reading

in single digits is acceptable.  This is a good time to check your

centering, as well. If all four sides of the box seem to be leaving

the screen at the same time, then your picture is centered perfectly.

If you would like to adjust the overscan or centering of your

set, you may be out of luck.  Many TVs or monitors don't have

adjustments for horizontal size or centering. Check for yourself and

see.  Vertical size and centering controls are more common - but

you'll want to be careful about changing the vertical size of the

picture if you can't change the horizontal - squashed or stretched

pictures could result.  If your monitor does have a full complement of

controls (horizontal size, vertical size, horizontal centering,

vertical centering, and vertical linearity), then call up the

LINEARITY test screen and use the centering controls first, then the

size controls. Don't forget that you might need to adjust the vertical

linearity control as well, to keep the circles as round as possible.

Once you get everything right, call up the CENTERING test to fine-tune

the positioning controls.

Suppose you've adjusted all the overscan out of your set, so that

all the edges of the raster are visible on the screen, and you still

can't get to zero on the overscan test?  This problem has to do with

the BLANKING in your set.  Blanking refers to the period of time the

scanning beam is turned off, so that it can be positioned for the next

line or frame of video.  In many sets, this BLANKING INTERVAL is

longer than absolutely necessary, and so may contribute to the

overscan in your set.

HORIZONTAL RESOLUTION

This is an important test, and it applies to both your VCR and TV

or monitor. RESOLUTION means sharpness, and what we're measuring is

exactly how sharp an image your video system can display.  There are

separate tests for HORIZONTAL and VERTICAL resolution, as well as one

for CHROMA, or color resolution. We need all these separate tests

because different technical characteristics are responsible for

resolution in each case.

HORIZONTAL RESOLUTION is typically measured as "Bandwidth" by

engineers. The concept of "Lines" is more easily grasped by consumers,

however, so camcorders, VCRs and monitors usually have their

resolution expressed in lines, that is, the number of vertical lines

that can be fit on the screen while still being able to distinguish

them.  This measurement is sometimes made in more than one way, so I

have included three sets of numbers on the test.  The top set of

numbers represent the traditional way of measuring LINES OF

RESOLUTION, which is the number of black and white lines that would

fit on your screen IF IT WERE SQUARE.  Since it is not square, but

rather rectangular, the bottom set of numbers indicates the number of

lines that will ACTUALLY fit on your screen.  From time to time,

measurements made this way will be quoted in specifications - which is

hardly fair if everyone else uses the traditional measurement.  The

middle set of numbers are for BANDWIDTH, expressed in megahertz.  This

method of measuring resolution is the most popular with engineers.

The test is for the luminance part of the picture (the

black-and-white portion of the TV signal) SO TURN YOUR COLOR CONTROL

ALL THE WAY DOWN.  How far to the right can you look before the lines

blur together?  The highest number you can get while still being able

to see individual lines is your horizontal resolution.  If the lines

are dim, but you can still see them, it still counts (this means that

your response is "falling off" at these frequencies). Increase your

contrast control and back off the brightness a little bit to improve

your view of the lines.

A good VHS VCR will only score in the low 200s on this test.  Some

formats (like S-VHS) are capable of scores of up to 400 lines, but

you'll need a good monitor to see it.  Most TV sets are only good for

250 lines or so.  If you have some good quality equipment, you might

be curious why you can see the 315 line grid, but not the 275!  The

reason is complicated, but it has to do with the fact that the part of

the TV signal that carries the color information has a frequency very

close to that of the 275 line grid.  This is why, if you turn up your

color control now, you will see crawling colors all over this part of

the screen.  Your TV is mistaking this grid for color information! If

your TV has a "comb filter", this grid may be a little clearer.  If

your TV or monitor has a special S-VIDEO connection to your VCR, it is

much more likely to be visible. This is why a special monitor is

recommended for S-VHS, HI-8 and similar formats.  If you are running

CVTS from the AMIGA software, you may notice a quality difference on

this test between the color composite output of your genlock or

encoder, and the monochrome output (AMIGA 500/2000) which contains

no color subcarrier (this will cause it to look better).

VERTICAL RESOLUTION

This is a little tricky to explain, so bear with me. All TV

equipment has exactly the same vertical resolution. This is

necessarily so because this parameter is a part of the standard that

our TV system (the north american TV standard is called NTSC) is based

on. So what is the standard vertical resolution?  Well, this is the

tricky part. There are 525 scan lines making up one frame of video.

Some of these aren't used for video, so there are a little less than

500 left. This would imply a vertical resolution of almost 500 lines.

The catch is that it takes the electron beams in your picture tube two

passes to paint all these lines on the screen.  First the odd-numbered

lines are drawn, then the even-numbered ones.  This even-odd business

(called INTERLACING) makes it very hard to distinguish lines right

next to each other, because very thin lines will appear to flicker

like crazy!

If you look very closely at the VERTICAL RESOLUTION test pattern,

you probably can see all the lines in all the grids, all the way up to

500. However, it would give you a headache to count them. This is why

the APPARENT vertical resolution of NTSC television is usually quoted

at 300 or so.  Lots of flicker means your equipment is working

perfectly.  Little flicker, or unusual difficulty seeing the lines

could mean that your TV is not interlacing properly, or your VCR is

not playing back both "fields" of the TV picture (mis-tracking, or a

dirty or damaged head are some possible causes).  When using this test

to compare TVs, record the highest number of lines you can EASILY see

(the APPARENT vertical resolution).

INTERLACE

If you think about the the process of INTERLACING, where the

lines of the second field of video are drawn in between the lines of

the first video field, you might start to wonder if anything could go

wrong.  The answer is yes, things can go wrong, and lines of video can

be scanned in wrong places.  In particular, if there are brightness

variations from field to field, the raster can expand or contract -

putting the lines in the wrong place.  The result - fuzzy video.

When you run the INTERLACE test, you'll see a screen that

flickers like crazy.  Start with the brightness and contrast on your

set turned down as far as possible, while still being able to see the

video.  Superimposed on this flickering pattern are the words "How

well can you read this?", repeated in five places on the screen.

You will notice that this message is white, with a black drop shadow

that just barely falls off to the lower right.  Now turn up the

BRIGHTNESS and CONTRAST (or PICTURE) controls to normal levels, and

observe both the legibility of the five messages, and the positions of

the black drop shadows.  Ideally, all five messages should look the

same, with the drop shadows in the same places - just barely falling

off to the lower right.  Otherwise, your monitor has some trouble

interlacing under the difficult circumstances presented by this test.

CHROMA RESOLUTION

This test is just like the HORIZONTAL RESOLUTION test except that

we are looking at the resolution in the color portion of the TV

signal.  Make sure that the CHROMA (or COLOR) knob on your TV or

monitor is turned UP to it's normal level.  To score your equipment,

find the closest together set of bars that you can make out, and read

its corresponding number.  Don't expect a high score on this test,

unless you have professional equipment.  This is a good test to use

when trying out VCRs and monitors, as there can often be substantial

differences.  To test a monitor, I recommend playing the test direct

from computer, as your monitor probably has better resolution than

your VCR.

Why is your chroma resolution so much poorer than your luminance

resolution?  The answer has a lot to do with the designers of our NTSC

TV system (and PAL, as well, a related TV system).  They knew, from

vision research, that humans see much more detail in luminance than

they do in color.  This is why there is much more bandwidth allocated

for brightness information than for color in the TV specifications.

Unfortunately, your TV or VCR will truncate still more of this chroma

information.

LUMINANCE NOISE

NOISE is what you're looking at when you watch a TV station that

doesn't "come in" very well. Most of us are used to calling this

"snow" because that's what it looks like - a snowstorm in front of the

TV show we want to watch.  All TV signals contain noise, but usually

not in objectionable amounts.  To measure noise objectively,

professional test equipment is required.  However, we have developed

tests that work quite well for comparative measurements of both luma

and chroma noise.  Two different tests are needed because the video

signal has two parts: one carries the brightness, or luminance

information, and the other carries the color, or chrominance

information.  These tests are best used for subjective, comparative

purposes and will not produce meaningful numbers for professional use!

To use the LUMINANCE NOISE test, first turn the CHROMA (or COLOR)

knob on your set all the way down.  If your VCR or TV has a sharpness

or peaking control, set it at the middle position.  If you have

digital noise reduction, or a "detail enhancer" on your VCR, turn

these gadgets off (they will hurt your score, not help).  Run the

test.  You will see a noise bar, with a significant amount of noise on

the right end.  Now, take note of the amount of noise visible on the

left side of the bar.  Next, look slowly across to the right until you

find the point where the noise seems to be just noticeably worse.  The

number above this spot is your score.

CHROMA NOISE

The CHROMA NOISE test is just like the LUMINANCE NOISE test

above, except that you should now turn the CHROMA (or COLOR) knob on

your TV back up to its normal position.  Look across the red bar from

left to right until you find the spot where the noise in the color

seems to begin worsening.  Read your score just above this spot.

COLOR CRAWL

The color TV signal, as we have mentioned before, is a mixture of

luminance (brightness) information and chrominance (color)

information.  An unfortunate side-effect of mixing these two signals

is that they can interfere with each other.  At the places on the

screen where this is happening, an artifact known as COLOR CRAWL or

DOT CRAWL appears.  Run the COLOR CRAWL test pattern, and look at the

left and right sides of the various colored boxes.  You might see a

pattern of dots, "crawling" up the screen just at the borders where

colors change, or, alternatively, it may look more like an ill-defined

crosshatch pattern.  If you see this effect on the left and right

sides of the colored boxes, and nowhere else, your set uses the "trap

filter" method of separating the chrominance and luminance.  If you

see little-to-no effect on the left/right sides, but some chroma crawl

on the tops/bottoms of the colored boxes, your set almost certainly

uses a "comb filter", a vastly better method of separating the

signals.  If you are watching on a set connected by an S-Video

connector to an S-VHS or HI-8 deck, you should be treated to a display

with NO chroma crawl at all.  Now you know why you spent all that

money on your video system!  You could find, as I did, that playing

your test tape on different VCRs, even through the same cheap TV set,

can produce widely differing results on this test - owing to the

quality of the VCR's encoding (the process of recreating the TV signal

from tape).  Don't be dismayed to discover that an expensive monitor

has more chroma crawl than a cheap one - this is probably a trade-off

that comes with the higher resolution of the better monitor.

CAMERA RESOLUTION

If you have a video camera, you can perform horizontal and

vertical resolution tests on it using this chart.  First, you'll need

to have it printed.  To print from Workbench, open the printer

graphics section of preferences and set your ASPECT to VERTICAL and

SHADE to BLACK AND WHITE.  Now run GRAPHICDUMP (this program is in the

utilities drawer of your workbench) and immediately display the CAMERA

RESOLUTION chart using the IFF viewer of your choice.  Alternatively,

you may print from a program such as Deluxe Paint.  When your printer

is done, compare the finished chart to the one on the screen.  It

doesn't need to look perfect - as long as all the lines seem to be

present, it's fine.

To measure the horizontal resolution of your camera, set your

camera on a tripod and tack the chart on the wall as straight and

level as you can.  Get the camera as close and perfectly centered to

the chart as you can, while still being able to focus.  Then, zoom all

the way in on the exact center of the pattern and focus there.  Now,

slowly zoom out, watching the pattern of lines.  When they have

blurred together so that you can no longer make out individual lines,

stop zooming out.  Look to either edge of the frame.  The number you

see there is your resolution.  Measuring vertical resolution is simply

a matter of turning the chart sideways, and repeating the above

instructions.

It's possible that your camera won't be able to focus close

enough to the chart.  One solution is to print the chart larger, if

possible.  Otherwise, use the MACRO setting on your camera lens to

focus closer.  Since this will preclude the use of the zoom lens,

simply focus on the resolution brackets one at a time, until you find

your camera's limit.

DROPOUTS

This test is designed for comparing video tapes.  If you were to

try to choose the best brand of video tape by running the CVTS tests

on them (the appropriate tests are: LUMINANCE NOISE, CHROMA NOISE,

CHROMINANCE RESOLUTION, and LUMINANCE RESOLUTION), it isn't likely

that you'll spot any differences at all.  Video tape is made to such

high specifications that any performance differences are going to be

extremely minor - unless you're comparing ultra-cheap made in China

dollar-store special tape to extra-high-grade super-VHS stock.

The one area where you can often find substantial differences

between tapes is in DROPOUT COUNT.  A dropout is a small area of tape

where a signal won't record.  This could be caused by a speck of dust,

a small wrinkle, or an uneven distribution of oxide.  A dropout, on

playback, looks like a small - or occasionally large - horizontal

white streak on the picture.

What this extremely simple test does is give you a chance to

count dropouts.  To use it, record it on the tape of your choice in

two spots - at the very beginning of the tape, and again in the middle

of the same tape.  Next, play back the sections of tape you have just

recorded, and get ready to count the number of white streaks you see.

The test consists of a countdown, followed by exactly one minute of

black.  When you see the black, begin counting dropouts.  When the

screen says stop, write down the number of dropouts you saw.

Typically, there will be many more dropouts at the beginning of the

tape than in the middle.  To get the best idea of the quality of the

brand you are testing, test several of their tapes bought at different

times, including some that have been used for a year or two.

If you can't spot ANY dropouts at all, your VCR may have a DROPOUT

COMPENSATOR, a circuit that "fills in" dropouts so you won't see them.

Use a different VCR for your tape tests.

THE MULTIPLE TEST PATTERN

You will recognize elements of many of the tests discussed above

in this test.  When using this pattern to evaluate a TV set or

monitor, first set the COLOR and TINT controls using the color bars,

then use the grey scale to set the BRIGHTNESS (or PICTURE control) and

CONTRAST.  Now, look at the grey scale... are the shades of grey

accurate, or tinted with a color?  Look at the dots around the edges

of the picture... how sharp are they?  Does the sharpness, color tint,

or picture size change when you turn the BRIGHTNESS or PICTURE control

up and down?  How many dots look mis-converged?  Examine the

transitions between colors on the color bars for dot crawl.  Look at

the large circle... is it round, or is it oval or misshapen?  Does the

picture appear level, and centered?  How much overscan does this set

have (count the number of dot rows you see - there are two columns on

each side and four rows on the top and bottom).  You can use the two

resolution tests if you like, but to be fair, if you can't run the

test directly from your computer, you'll have to use a SUPER-VHS or

HI-8 version of this tape to test monitors.  A plain old VHS machine

will always have poorer resolution than a TV or monitor.  I don't

recommend using the noise tests at all when comparing TVs, for the

same reason.  Any non-professional format VCR will create more noise

than a typical TV set.  If the score on one of these tests looks way

out of line, though, the TV could have a problem.

NOTES ON USING THE TESTS

When comparing VCRs, the important tests are HORIZONTAL

RESOLUTION, CHROMA RESOLUTION, LUMINANCE NOISE, CHROMA NOISE, WINDOW,

and COLOR CRAWL.  Using the same good-quality TV or monitor each time

will give you the fairest result.

When comparing TVs or monitors, all the tests are valid, though

the NOISE tests have been designed principally for VCRs - a typical TV

by itself (running the tests directly from computer) will score at the

top of the scale.

Don't be led to believe that just because you've set your monitor

up to the CVTS colorbars, all TV programming will now appear in

perfectly adjusted color.  This may turn out to be the case, by the

way, but if it's not, it's because you don't have the colorbars

associated with each and every tape or TV show, as in the professional

video production world.  Some programmers (your local cable access

channel, for example) may be lazy about implementing good technical

standards.  Having your personal equipment properly set up does

nothing to assure that the signal entering it has been processed with

care.

HINTS ON SHOPPING FOR A TV

You know what it's like.  Fifty-seven television sets, all

displaying the same TV show.  Should you buy the set with the picture

that looks best to your eyes?

Believe it or not, sometimes the set with the worst-looking

picture is the best one.  If the program displayed on all those TV

sets contains NOISE (a VHS movie or a snowy cable channel, for

instance), then the set with the best resolution will be displaying

the most noise!  There's a relationship between noise and sharpness -

they go together.  The situation is analogous to playing a noisy old

78 RPM record through a set of expensive stereo speakers.  This record

might sound better played through an old, low-fidelity speaker that

doesn't reproduce every scratch and pop with annoying clarity.  You

wouldn't use an old record like this to buy a stereo system, would

you?  Don't forget that most sets have a SHARPNESS control.  You can

turn down the sharpness when watching those fuzzy VHS tapes, to get

rid of the noise.  If your set has the extra resolution, however,

you'll really appreciate it on high-quality noise-free program

material.

If you can record the CVTS tests on tape and take it along, you

will find out a lot about the set you want to buy (you will awe the

sales staff, too!).  You may want to narrow down the field a bit

first, though, so here are some ways to analyze the sets quickly.

Watch the picture as the program fades to and from black between

commercials.  Does it seem to change size, focus or color? (Bad

signs.)  Compare the pictures of several adjacent TVs to find and

eliminate the ones with too much overscan.  Look for dot crawl on the

sharp edges of strongly colored objects.  If you see it on the TOP and

BOTTOM edges more than on the SIDE edges, this is a sign of a COMB

FILTER (this is good).  Check for straight, level lines and round

circles whenever these things appear on the screen.  Next, look around

the edges of sharp bright objects (white lettering, for example)

whenever you can.  Are these edges clean, or are there "wrinkles",

"ghosts", double edges or unnatural looking white borders?  Look in

the corners of the screen for colored fringes around objects - a sign

of poor convergence.  Finally, try not to overlook a set that may have

been simply misadjusted by a salesperson.  You may even want to help

him/her out by properly setting it up.

ORDER FORM

The following items are available from Dave Muse Video.

You may print this form and simply circle what you want - or write on

a separate sheet of paper.

I am working on a list of benchmarks for various pieces of video

equipment - I will include this with every order.

COLORBARS VIEWING FILTER

Used in conjunction with the colorbars test signal for adjusting

monitors and TVs. - $5.00

COMPLETE VIDEO TEST SYSTEM - TAPE, DISKS AND MANUAL

The CVTS tests recorded on tape and ready to use for evaluating

monitors, TVs, and VCRs.  Includes printed manual, colorbars viewing

filter, camera resolution chart and the latest Amiga computer version

on disk as well.  Additional help screens explaining each test are

recorded on the tape.

VHS - $28.00

SUPER VHS - $34.00

3/4" - $36.00

Other formats - Write first

COMPLETE VIDEO TEST SYSTEM - DISKS AND MANUAL

The latest version of CVTS on disk plus a printed manual, camera

resolution chart and colorbars viewing filter. The manual contains

additional instructions and diagrams to make using the tests even

easier. - $16.00

Please circle the prices of the items you want above, and total here:

$________

Shipping: Add $2.00 per product ordered

(except for the viewing filter - no addl. charge.)$________

MI residents have to include sales tax:           $________

Add everything up here:                           $________

Mail this and any other correspondence to:

Dave Muse

22443 Leewright

Southfield, MI

48034