8    PARALLEL DEVICE

8.1  INTRODUCTION

All Amiga models have a parallel port to which you can connect

external devices like a printer, a video digitizer or a sound

sampler. The most common external device for the parallel port

is undoubtedly a printer, although some printers are connected

to the serial device.

The parallel port can send and receive eight bits

simultaneously. This can be compared with the serial port which

only can send/receive a stream of bits. The parallel port

is because of this much faster and is therefore often used for

video digitizers or sound samplers.

The parallel device helps you to work with the printer at a

very low level, but is still easy to handle. The parallel

device can as the serial device be locked for exclusive access

or you can allow other programs to use the device

simultaneously.

It is important to note that the parallel device should only be

used when you want to handle the port directly at a low level.

This can be useful when you want to collect data from a video

digitizer, or send untranslated printer codes. However, if you

simply want to use the printer you should use the printer

device instead. The printer device will automatically take care

of all printer handling, and is using the preference settings.

See next chapter (29) "Printer Device".

8.2  PARALLEL PORT

A parallel port sends a whole byte each time as explained above,

and is therefore very fast. Data that is sent to or received

from the parallel port does not need to be translated in any

way, it is immediately usable.

The Amiga's parallel (Centronics) port is a 25-pin D-female-

type connector. (On the old A1000s the parallel port have a

male connector.) Below is an almost complete list of the pin

assignment, together with a short description. (See

illustration "Centronics".)

Pin  Name    Direction  Description

-----------------------------------------------------

1  STROBE  Out        Used to coordinate the events

2  Data 0  In/Out     Bit 0

3  Data 1  In/Out     Bit 1

4  Data 2  In/Out     Bit 2

5  Data 3  In/Out     Bit 3

6  Data 4  In/Out     Bit 4

7  Data 5  In/Out     Bit 5

8  Data 6  In/Out     Bit 6

9  Data 7  In/Out     Bit 7

10  ACK     In         Data acknowledge

11  BUSY    In/Out     General Input/Output pin

12  POUT    In/Out     General Input/Output pin

13  SEL     In/Out     General Input/Output pin

14  +5V     -          +5 Volt

15  NC      In/Out     No connection pin

16  RESET   Out        The system resets

17  GND     -          Signal ground

18  GND     -          Signal ground

19  GND     -          Signal ground

20  GND     -          Signal ground

21  GND     -          Signal ground

22  GND     -          Signal ground

23  GND     -          Signal ground

24  GND     -          Signal ground

25  GND     -          Signal ground

8.3  PARALLEL DEVICE

The parallel device is very similar to the serial device. It

can either be used in exclusive mode, or several programs may

use the port simultaneously, shared access. When you want that

the parallel device to do something you simply send an already

initialized request block (struct IOExtPar), and the device

will send a message to the reply port when the request has been

done. Exactly as all other devices.

8.3.1  THE PARALLEL REQUESTBLOCK

The request block you should use with the parallel device look

like this: (defined in header file "devices/parallel.h")

struct IOExtPar

{

struct IOStdReq IOPar;

ULONG io_PExtFlags;

UBYTE io_Status;

UBYTE io_ParFlags;

struct IOPArray io_PTermArray;

};

IOPar:         This is the standard request block. The IOStdReq

structure is defined in header file "exec/io.h",

and is fully documented in chapter 17 "Devices".

io_PExtFlags:  This is currently not used, but will maybe be

used in the future when more parallel flags are

needed.

io_Status:     The status of the parallel port and parallel

device. There exist for the moment four

status flags:

IOPTF_RWDIR    If this flag is set the device

is currently writing to the

parallel device. On the other

hand, if the flag is not set

the device is collecting data

at the parallel port.

IOPTF_PARSEL   Printer selected.

IOPTF_PAPEROUT The printer ran out of paper.

Inform the user!

IOPTF_PARBUSY  The parallel port is currently

busy.

Use the command "PDCMD_QUERY" before you look at

these fields to make sure everything is up to

date.

io_ParFlags:   This field contains all special parallel flags.

There exist only three flags for the moment, and

one of these is still not usable. Here is the

complete list:

PARF_SHARED     Set this flag if you want to

share the parallel port with

other programs. Note that this

flag should only be altered

before you have opened the

parallel device, and should

NOT be changed later on.

If you want to change status

you should close the parallel

device, alter the status and

then try to open the device

again.

PARF_RAD_BOOGIE This flag is currently not

used. It is supposed to be

set when you want to send/

receive data at a very high

speed.

PARF_EOFMODE    This is actually the only

flag you may alter after you

have opened the device. If

the flag is set the parallel

device will immediately

stop the transmission of data

when it finds one of the

specified end-of-file

characters.

io_PTermArray: This field contains eight characters which will

be treated as the end-of-file characters if the

PARF_EOFMODE flag is set.

8.3.2  OPEN THE PARALLEL DEVICE

As with all devices you have to open a message port through

which the parallel device can communicate with you, and

allocate a request block (a IOExtPar structure), before you

may open the device itself.

1. Open a message port: (Since it is only our task and the

device that will use the message port, we do not need

to make it "public", hence no name. Priority should as

usual be set to 0, normal priority.)

struct MsgPort *replymp;

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

2. Allocate a request block of type IOExtPar structure.

(The IOExtPar structure is an extended version of the

normal request block, and should therefore be allocated

with help of the CreateExtIO() function with the size

set to sizeof( struct IOExtPar ).

struct IOExtPar *parallel_req;

parallel_req = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req )

clean_up( "Not enough memory!" );

Once the message port and the request block have successfully

been created, you need to decide if you want exclusive or

shared access. If you want shared access, other programs may

also use the parallel port, should you set the flag

"PARF_SHARED" in the io_ParFlags" field. If you want exclusive

access you do not need to set any flags, since it is the

default mode.

3. Either set shared or exclusive access, and open the

device.

UBYTE error;

/* We want shared access: */

parallel_req->io_ParFlags = PARF_SHARED;

/* Open the parallel device: */

error = OpenDevice( PARALLELNAME, 0, parallel_req, 0 );

if( error )

clean_up( "Could not open the Parallel Device!" );

8.3.3  SET PARALLEL PARAMETERS

Once you have successfully opened the parallel device you

may set some parameters that tells the device how it should

work. Luckily, the parallel device is much simpler than the

serial device.

For the moment there exist only one parallel flag you may use,

and that is PARF_EOFMODE. It should be set if you want the

device to immediately stop the current communication if one of

the eight specified end-of-file characters appears. To set the

flag do like this:

/* Look for end-of-file characters: */

parallel_req->io_ParFlags += PARF_EOFMODE;

Note that we used the sign "+=". This is very useful if you

want to set one flag (bit), but keep all others unchanged. If

we simply used the sign "=" the "PARF_SHARED" flag we

previously had set would be erased. We could of course have

written it like this:

/* Set both the end-of-file and shared mode: */

parallel_req->io_ParFlags = PARF_EOFMODE | PARF_SHARED;

This is however not equally good, since it is easy to forget

one flag, and then it would be erased. So if you want to

add flags use the command "+=". To erase one flag you only

have to do the opposite and subtract it ("-=").

If you want to use the end-of-file mode, and have set the

"PARF_EOFMODE" flag, you must also tell the device which

characters should be treated as end-of-file characters. The

list of these characters is stored in the "IOPArra" array

which is placed at the bottom of the request structure, and

looks like this:

struct IOPArray

{

ULONG PTermArray0;

ULONG PTermArray1;

};

Each ULONG data consists of four bytes, and in total you can

store eight bytes (end-of-file characters) in the array. To

make the checking routine efficient you must store the

characters in descending order! To copy the desired

characters into the array do like this:

/* Here is an array with all EOF characters: */

/* NOTE! They MUST be in descending order!   */

UBYTE eof_char[8]={ 0x54, 0x32, 0x16, 0x15, 0x12, 0x03, 0x00, 0x00 };

/* Declare a unsigned byte pointer: */

UBYTE *ptr;

/* Simple loop variable: */

int loop;

...

/* Get the address of the IOTArray: */

ptr = (UBYTE *) &(ioreq->io_PTermArray);

/* Set all eight end of file characters: */

for( loop=0; loop < 8; loop++ )

{

/* Copy character after character: */

*ptr = eof_chars[ loop ];

/* Step one byte forward: */

ptr++;

}

Once you have set all desired parameters in the request block

(IOExtPar structure) you set the IO command to PDCMD_SETPARAMS

and tell the parallel device to do your request by calling the

DoIO() function. If something went wrong DoIO() will return

with an error number, else 0 is returned which means that your

request have successfully been executed. (See below for a

complete list of error messages.) Here is an example:

/* We want to set the parallel device's parameters: */

ioreq->IOPar.io_Command = PDCMD_SETPARAMS;

/* Do our request, and return when done: */

error = DoIO( ioreq );

8.3.4  WRITE DATA

To send data to the parallel device you have to:

1. Set the command flag "CMD_WRITE" in the "io_Command"

field.

2. Tell the parallel device how many bytes you want to send

by setting the "io_Length" field as desired. If you set it

to -1 the device will continuously send data and stop

first when an end-of-file character is found. (The end-of-

file character will also be sent.) Note that the parallel

flag "PARF_EOFMODE" must have been set, or the device will

never stop sending data.

3. Give the field "io_Data" the address of the first byte of

data that should be sent.

4. Finally you send the request to the parallel device. If

you want to wait for the request to be completed

(synchronous) you should use the DoIO() function. On the

other hand, if you want that your program continues to

work while your request is completed, you should use the

SendIO() function.

Here is an example on how to write data to the parallel port:

(In this example the program will be put to sleep while the data

is sent to the parallel port.)

error: (UBYTE) is a simple unsigned byte variable.

ioreq: (struct IOExtPar *) is a pointer to an IOExtPar

structure.

data:  (BYTE) is a pointer to a block of memory where all

data you want to send is located.

/* We want to send (write) some data: */

ioreq->IOPar.io_Command = CMD_WRITE;

/* Give the start address of our data buffer: */

ioreq->IOPar.io_Data = data;

/* We want to send 150 bytes/characters: */

ioreq->IOPar.io_Length = 150;

/* Do our request: */

error = DoIO( ioreq );

/* Everything OK? */

if( error )

printf( "Problems while writing!\n" );

If you do not want to wait for the request to be completed you

should use the SendIO() function instead. To check later if the

request have been completed or not you can either use the

function CheckIO(), or wait for a message to arrive at the

request block's reply port. (Since other request may also send

messages to this port, it is usually easiest to use CheckIO().)

Once the request have been completed you can look at the

io_Error field of the request block to check if everything

was OK. If the field is zero everything was executed without

any problems, but if it is non zero the request failed.

Here is an example:

/* Declare a pointer and set it to NULL: */

struct IOExtPar *ptr = NULL;

...

/* We want to send (write) some data: */

ioreq->IOPar.io_Command = CMD_WRITE;

/* Give the start address of our data buffer: */

ioreq->IOPar.io_Data = data;

/* We want to send 280 bytes/characters: */

ioreq->IOPar.io_Length = 280;

/* Do our request and return immediately: */

SendIO( ioreq );

/* As long as the pointer is not pointing to */

/* the request we should stay in the loop:   */

while( ptr == NULL )

{

... do something ...

/* Check if the request has been completed: (If the  */

/* request has been completed CheckIO() will return  */

/* a pointer to the request, else NULL is returned.) */

ptr = CheckIO( ioreq );

}

/* Remove the requst block's message. (The ptr and ioreq */

/* are in this example identical, so it does not matter  */

/* whichever you will use. The parenthesis around the    */

/* expression is actually unnecessary, but this looks    */

/* better.)                                              */

Remove( &(ptr->IOPar.io_Message.mn_Node) );

/* Everything OK? Check the io_Error filed: */

if( ioreq->IOPar.io_Error )

printf( "Problems while writing!\n" );

8.3.5  READ DATA

Although most people use the parallel port to send data to their

printer, more and more users connect other external devices

like video digitzers and sound samplers. With these devices

you do not only need to send data, you also have to collect

data from the parallel port.

The process of collecting (reading) is very similar to reading.

Here is what you have to do:

1. Set the command flag "CMD_READ" in the "io_Command"

field.

2. Tell the parallel device how many bytes you want to read

by setting the "io_Length" field as desired. If you set it

to -1 the device will continuously read data and stop

first when an end-of-file character is received. (The end-

of-file character will also be collected.) Note that the

parallel flag "PARF_EOFMODE" must have been set, or the

device will never stop collecting data.

3. Give the field "io_Data" a pointer to your data buffer

where all collected data should be placed. Note that the

buffer must be big enough so all data will fit!

4. Finally you send the request to the parallel device. If

you want to wait for the request to be completed

(synchronous) you should as usual use the DoIO() function.

On the other hand, if you want that your program

continues to work while your request is completed, you

should use the SendIO() function.

Here is an example on how to read data: (Your program will be

put to sleep while the data is collected.)

error: (UBYTE) is a simple unsigned byte variable.

ioreq: (struct IOExtPar *) is a pointer to an IOExtPar

structure.

data:  (BYTE) is a pointer to a block of memory where all

data which is collected will be stored. Note that the

data buffer must be big enough so all data will fit.

/* We want to read some data: */

ioreq->IOPar.io_Command = CMD_READ;

/* Give the start address of our data buffer: */

ioreq->IOPar.io_Data = data;

/* We want to read 400 bytes/characters: (The buffer must */

/* then be at least 400 bytes.)                           */

ioreq->IOPar.io_Length = 400;

/* Do our request: */

error = DoIO( ioreq );

/* Everything OK? */

if( error )

printf( "Problems while reading!\n" );

The program above will go to sleep while the data is collected

from the parallel port. If you want to do something while the

data is fetched and not go to sleep you should use the

asynchronous function SendIO() function as explained above.

Here is an example:

/* Declare a pointer and set it to NULL: */

struct IOExtPar *ptr = NULL;

...

/* We want to read some data: */

ioreq->IOPar.io_Command = CMD_READ;

/* Give the start address of our data buffer: */

ioreq->IOPar.io_Data = data;

/* We want to read 400 bytes/characters: (The buffer must */

/* then be at least 400 bytes.)                           */

ioreq->IOPar.io_Length = 400;

/* Do our request and return immediately: */

SendIO( ioreq );

/* As long as the pointer is not pointing to */

/* the request we should stay in the loop:   */

while( ptr == NULL )

{

... do something ...

/* Check if the request has been completed: (If the  */

/* request has been completed CheckIO() will return  */

/* a pointer to the request, else NULL is returned.) */

ptr = CheckIO( ioreq );

}

/* Remove the requst block's message. (The ptr and ioreq */

/* are in this example identical, so it does not matter  */

/* whichever you will use. The parenthesis around the    */

/* expression is actually unnecessary, but this looks    */

/* better.)                                              */

Remove( &(ptr->IOPar.io_Message.mn_Node) );

/* Everything OK? Check the io_Error filed: */

if( ioreq->IOPar.io_Error )

printf( "Problems while reading!\n" );

8.3.6  HOW TO HANDLE SEVERAL REQUESTS SIMULTANIOUSLY

Since you usually will want to read and write data at the same

time, you have to use two separate request blocks. If you are

using shared access mode you simply create two request blocks

and open the parallel device twice. (Do not forget to close

both requests later on.) Here is an example:

struct MsgPort *replymp;

struct IOExtPar *parallel_req_read;

struct IOExtPar *parallel_req_write;

UBYTE error;

/* We use only one reply message port: */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create the request block "read": */

parallel_req_read = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req_read )

clean_up( "Not enough memory!" );

parallel_req_read->io_ParFlags = PARF_EOFMODE | PARF_SHARED;

/* Create the request block "write": */

parallel_req_write = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req_write )

clean_up( "Not enough memory!" );

parallel_req_write->io_ParFlags = PARF_EOFMODE | PARF_SHARED;

/* Open the parallel device for the read request: */

error = OpenDevice( PARALLELNAME, 0, parallel_req_read, 0 );

if( error )

clean_up( "Could not open the parallel device (Read)!" );

/* Open the parallel device for the write request: */

error = OpenDevice( PARALLELNAME, 0, parallel_req_write, 0 );

if( error )

clean_up( "Could not open the parallel device (Write)!" );

If you are using exclusive access mode you can of course not

open the parallel device twice. Instead we have to copy the

first request block to the other request block, byte for byte.

Here is an example:

struct MsgPort *replymp;

struct IOExtPar *parallel_req_read;

struct IOExtPar *parallel_req_write;

BYTE *r_ptr;

BYTE *w_ptr;

UBYTE error;

int loop;

/* We use only one reply message port: */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create the request block "read": */

parallel_req_read = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req_read )

clean_up( "Not enough memory!" );

parallel_req_read->io_ParFlags = PARF_EOFMODE | PARF_SHARED;

/* Create the request block "write": */

parallel_req_write = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req_write )

clean_up( "Not enough memory!" );

parallel_req_write->io_ParFlags = PARF_EOFMODE | PARF_SHARED;

/* Open the parallel device for the read request: */

error = OpenDevice( PARALLELNAME, 0, parallel_req_read, 0 );

if( error )

clean_up( "Could not open the Parallel Device (Read)!" );

/* Since we can not open the parallel device once again */

/* for the write request, we have to copy the whole     */

/* read request block into the write request block.     */

/* Get the start address of both request blocks: */

r_ptr = (BYTE *) parallel_req_read;

w_ptr = (BYTE *) parallel_req_write;

/* Copy the request block, byte by byte: */

for( loop=0; loop < sizeof( struct IOExtPar ); loop++ )

{

/* Copy one byte: */

*w_ptr = *r_ptr;

/* Step one byte forward: */

w_ptr++;

r_ptr++;

}

Now we have two request blocks, and can therefore use both

read and write requests at the same time. If you also want to

use the special extra functions described further down, you

may want to create even one more request block (or maybe

even more...). The procedure is the same as described above.

8.3.7  ERRORS

While you are using the parallel device you may sometimes

encounter an error message. Usually it is easy to guess what

went wrong, but it is always good to check what really happened.

There exit for the moment seven different types of parallel

port errors, all defined in the header file "devices/

parallel.h".

You will either receive the error message from the function you

just called (for example, DoIO() returns 0 or an error number),

or you can check the request block to see if there were any

problems. (The io_Error filed of the request block either

contains 0, which means everything is OK, or an error number.)

Here is a complete list of the parallel error messages:

ParErr_DevBusy        Some other task/request is already using

the parallel device.

ParErr_BufTooBig:     Buffer too big. Hey, what is this?

ParErr_InvParam:      The request block's parameters were not

properly initialized.

ParErr_LineErr:       There were some problems with the

communication. The parallel cable is

faulty or the other device is not

properly initialized or not connected.

Tell the user to check the cables!

ParErr_NotOpen:       The parallel device was not open!

ParErr_PortReset:     The parallel device have just been

resetted. Someone just sent a CMD_Reset

request. Default parameters are

set.

ParErr_InitErr:       The parallel device could not be

initialized with your requirements.

(Probably forgot to clear all unused

flags.)

While you are using the parallel device it may happen that you

also receive error messages from Exec. (Exec is handling all

stuff like messages, requests, tasks and so on.) Here is

a complete list of exec error messages: (defined in the header

file "exec/errors.h")

IOERR_OPENFAIL  The device (unit) could not be opened. (If you

are denied access to the parallel device you

should receive the ParErr_DevBusy flag instead

of this exec message, but internally this flag

is used.)

IOERR_ABORTED   When you abort a previously started request by

calling the AbortIO() function, the io_Error

filed of that request is set to IOERR_ABORTED.

If you find a request block with this flag set,

you know that it has been aborted.

IOERR_NOCMD     You tried to use a command that is not

supported by the parallel device.

IOERR_BADLENGTH The length of the request was not valid.

8.3.8  CLEAN UP

As usual on the Amiga you must remember to close and return

everything you have opened or allocated. If you do not close

the parallel device after you, other programs will then not

be able to use the it. PLEASE be very careful about this!

The routine is very similar to how you close the serial device

which was described in the previous chapter. However this is

so important that it can not be said too often.

Here is a list of what you have to do:

1. All requests you have started with SendIO() or BeginIO()

(asynchronous commands) must either have been completed

or aborted before you may close the device. It is a very

common error to forget this, and it can be hard to find

this bug. Usually the program will work fine (the command

was completed in time), but now and then your program will

crash (the command was completed after the device have

been closed).

A simple way is to abort all commands that have not

reported that they have been completed, but this is not

always good way to do it. (The last commands may be

important and should therefore not be aborted.)

If you do not want to abort the command, you should

instead wait for it to be completed. The WaitIO()

function is simple to use, and will put your program

to sleep while waiting, so no computer time is wasted.

If the request has already been completed, the function

will return immediately. WaitIO() will also remove the

message from the reply port. It is a very useful and

simple function to use, but do NOT try to wait for

a request that has not been started!

Here is an example on how to wait for a request to be

completed: (If the request already has been completed

it does not matter, WaitIO() will then simply return

immediately. Note that we do not have to remove any

messages from the reply port if we use WaitIO().)

/* Store possible error numbers here: */

UBYTE error;

/* ... */

/* Wait for the request to be completed: */

error = WaitIO( ioreq );

/* Everything OK? */

if( error )

printf( "Something went wrong!" );

/* Well, successful or not, we may now */

/* close the device!                   */

To abort a request, simply use the AbortIO() function:

/* Try to abort a previously started request:  */

/* (Do not try to abort a request that has not */

/* been started.)                              */

AbortIO( ioreq );

2. When all requests have been completed or aborted you may

close the parallel device. (Requests that have been

executed by calling the DoIO() function have already been

completed before your program wakes up, and thus you do

not need to wait for these.)

The parallel device is closed as all other devices, by

calling the CloseDevice() function. Here is an example:

/* Close the Parallel Device: */

if( !parallel_dever )

CloseDevice( ioreq );

3. You should now return all request blocks you have

allocated. If you allocated a standard sized request block

by calling the CreateStdIO() function, you should free

it by calling the DeleteStdIO() function. However, if

you have allocated an extended request block (like the

parallel device's request blocks) by calling the

CreateExtIO() function, you MUST call the DeleteExtIO(),

and of course remove the same amount of data as you

allocated.

How to delete a standard request block: (Since it is of

the standard size, you do not need to specify any size.)

/* Deallocate a standard sized request block: */

DeleteStdIO( ioreq );

With extended request block you have to specify the size

as you did when you allocated it:

/* Deallocate an extended request block: */

/* (The size may vary depending on what  */

/* device it was used for.)              */

DeleteExtIO( ioreq, sizeof( struct IOExtSer ) );

Note that ALL request blocks that have been allocated,

must be removed!

4. Finally you should close all message ports you have

previously opened. Simply use the DeletePort() function

as this example demonstrates:

/* Remove the replyport: */

DeletePort( replymp);

Please be careful with how your program terminates! Your

program should not only run fine, but it should also allow

other programs to run after and simultaneously. Remember that

your program must also be able to quit nice and neatly even if

it had to terminate too early because of some fatal error. The

cleaning up should only be done where it is needed, and if you

have not allocated the memory or opened the device before your

program quits, you should of course NOT try to free these

resources! If you do the Amiga will most certainly crash! Too

many programs contain this very annoying error. Make sure yours

will not be the same.

The best way to manage this cleaning up routine is to write a

separate function which checks each thing before it frees it.

The idea is that you may call this routine at any time, and it

will still manage to clear everything properly. Here is an

example: (Note how we check each thing to see if it should be

removed!)

/* Close and return everything that has been */

/* opened and allocated before we quit:      */

void clean_up()

{

/* 1. Close the Parallel Device: */

if( !parallel_dever )

CloseDevice( parallel_req );

/* 2. Deallocate the parallel request block: */

if( parallel_req )

DeleteExtIO( parallel_req, sizeof( struct IOExtPar ) );

/* 3. Remove the replyport: */

if( replymp )

DeletePort( replymp);

/* 4. Quit: */

exit( 0 );

}

Comments: 1. The parallel_dever is a variable that we have

declared ourself, and is used only to determine

if the device has been opened or not. (The

variable is set to TRUE before the program

starts, and is only changed to FALSE after

a successful opening of the device. See the

included examples for more information.) We

should of course only close the device if

the parallel_dever is FALSE. ("parallel_dever"

stands for "parallel device error".)

2. The parallel_req is a pointer which is either

pointing to a request block, or NULL if no

request block has been allocated. We should

of course only free the request block if there

exist one.

3. The replymp is a pointer which is either

pointing to a message port, or NULL if no message

port has been opened. And of course, we should

only close the message port if it has been

opened.

4. Finally our program may terminate. The exit()

function is placed here so we are sure the

program will quit. (If we did not have this

exit(), our function would then return the

control to our program again, and then we would

be in deep s*#^%.)

8.4  A COMPLETE EXAMPLE

Here is a complete example that opens the parallel device, sets

all necessary parameters, sends some data, collects some data

(well actually only 0 bytes) and finally cleans up and quits.

Since we send the read and write requests after each other (the

first is completed before the other is started) we only need to

use one request block.

#include <exec/types.h>

#include <devices/parallel.h>

/* Declare a pointer to our reply port: */

struct MsgPort *replymp = NULL;

/* Declare a pointer to our parallel request block: */

struct IOExtPar *parallel_req = NULL;

/* Store the parallel device error here: */

UWORD parallel_dever = TRUE;

/* Declare our own data buffer: */

/* (Last byte is a NULL sign.)  */

BYTE buffer[] = "Anders Bjerin was here...";

/* Declare our functions: */

void main();

void clean_up( STRPTR text );

void main()

{

int loop;

UBYTE *ptr;

UBYTE error;

/* The eight end-of-file characters: */

UBYTE eof_char[8]={ 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00 };

/* OPEN THE PARALLEL DEVICE: */

/* Get a reply port: (No name, priority 0) */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create a parallel request block: */

parallel_req = (struct IOExtPar *)

CreateExtIO( replymp, sizeof( struct IOExtPar ) );

if( !parallel_req )

clean_up( "Not enough memory for the parallel request block!" );

/* Since we want exclusive access mode, we do not set the */

/* parallel flag "PARF_SHARED", which otherwise must have */

/* been set before the device is opened. To make sure the */

/* field is empty we set it to 0:                         */

parallel_req->io_ParFlags = 0;

/* Open the Parallel Device: */

parallel_dever = OpenDevice( PARALLELNAME, 0, parallel_req, 0 );

if( parallel_dever )

clean_up( "Could not open the Parallel Device!" );

/* SET THE REST OF THE PARALLEL PARAMETERS (not many): */

/* Check for end-of-file characters: */

parallel_req->io_ParFlags = PARF_EOFMODE;

/* No additional flags: */

parallel_req->io_PExtFlags = NULL;

/* Set all eight end of file characters: */

ptr = (UBYTE *) &(parallel_req->io_PTermArray);

for( loop=0; loop < 8; loop++ )

{

/* Copy character after character: */

*ptr = eof_char[ loop ];

/* Step one byte forward: */

ptr++;

}

/* All values have now been set, lets */

/* do a PDCMD_SETPARAMS request:      */

parallel_req->IOPar.io_Command = PDCMD_SETPARAMS;

/* Do our request: */

error = DoIO( parallel_req );

if( error )

clean_up( "Could not set the parallel parameters!" );

/* SEND DATA TO THE PARALLEL PORT: */

/* We want to send (write) some data: */

parallel_req->IOPar.io_Command = CMD_WRITE;

/* Give the start address of our data: */

parallel_req->IOPar.io_Data = (APTR) buffer;

/* We want to send data until we find a NULL sign: */

/* (The NULL sign was specified in the EOF-char.)  */

parallel_req->IOPar.io_Length = -1;

/* Do our request: */

error = DoIO( parallel_req );

if( error )

clean_up( "Could not send data to the parallel port!" );

/* READ DATA FROM THE PARALLEL DEVICE: */

/* We want to read some data: */

parallel_req->IOPar.io_Command = CMD_READ;

/* Give the start address of our buffer: */

parallel_req->IOPar.io_Data = (APTR) buffer;

/* We want to read 0 bytes: */

/* (Since most of you do not have anything which sends data */

/* to the parallel device, I do not want to wait for more   */

/* than 0 bytes to arrive.)                                 */

parallel_req->IOPar.io_Length = 0;

/* Do our request: */

error = DoIO( parallel_req );

if( error )

clean_up( "Could not read data from the parallel port!" );

/* THE END: */

clean_up( "The End" );

}

/* Close and return everything that has been */

/* opened and allocated before we quit:      */

void clean_up( STRPTR text )

{

/* Close the Parallel Device: */

if( !parallel_dever )

CloseDevice( parallel_req );

/* Deallocate the parallel request block: */

if( parallel_req )

DeleteExtIO( parallel_req, sizeof( struct IOExtPar ) );

/* Remove the replyport: */

if( replymp )

DeletePort( replymp);

/* Print the message: */

printf( "\n%s\n", text );

/* Quit: */

exit( 0 );

}

8.5  OTHER USEFUL COMMANDS

Although reading and writing are most commonly used commands,

there exist some other functions that can sometimes be needed.

Here is a complete list of commands that can be sent with help

of a request block:

1. Flush, removes all queued requests.

2. Query, get some information from the parallel device.

3. Reset, reinitializes the parallel device.

4. Start, restarts the parallel communication.

5. Stop, temporary stops the parallel communication.

8.5.1  FLUSH

If several requests are sent to the parallel device they are

all queued on a FIFO (First In First Out) basis. The command

"CMD_FLUSH" can then be used to remove all these queued

commands. Here is an example:

/* We want to remove all queued requests: */

ioreq->IOPar.io_Command = CMD_FLUSH;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not remove the queued requests!\n" );

8.5.2  QUERY

The command "PDCMD_QUERY" can be used to get some information

from the parallel device. It is useful if you want to see if

the paper is out, printer is busy or if it is currently writing

or reading. All this can be found in the "io_Status" field of

the request structure, as this table shows:

Name           Bit  Hex   Active  Description

------------------------------------------------------------

IOPTF_PARBUSY    0  0x01  Low     Printer is selected

IOPTF_PAPEROUT   1  0x02  Low     Paper out

IOPTF_PARSEL     2  0x04  Low     Printer is busy

IOPTF_RWDIR      3  0x08  -       Reading (0) or Writing (1)

-                4  0x10  -       Reserved

-                5  0x20  -       Reserved

-                6  0x40  -       Reserved

-                7  0x80  -       Reserved

------------------------------------------------------------

Here is an example:

/* Check the parallel device: */

ioreq->IOPar.io_Command = PDCMD_QUERY;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not get any information from the device!\n" );

else

{

/* Check the "io_Status" field: */

if( ioreq->io_Status & IOPTF_PARBUSY )

printf( "Printer is busy.\n" );

if( ioreq->io_Status & IOPTF_PAPEROUT )

printf( "Paper out!\n" );

if( ioreq->io_Status & IOPTF_PARSEL )

printf( "Printer selected!\n" );

printf( "Device is %s\n",

ioreq->io_Status & IOPTF_RWDIR ? "Writing" : "Reading" );

}

8.5.3  RESET

Send the command "CMD_RESET" to reset the parallel device. All

commands that are queued to the device will be removed, commands

that are currently executed will be aborted all parallel flags

are resetted. Here is an example:

/* We want to reset the parallel device: */

ioreq->IOPar.io_Command = CMD_RESET;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not reset the parallel device!\n" );

8.5.4  START

After you have stopped the parallel communication by sending an

CMD_STOP command, you may want to start the communication again.

It is done by sending a CMD_START command. Here is an example:

/* We want to start parallel communication again: */

ioreq->IOPar.io_Command = CMD_START;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not start the parallel communication!\n" );

8.5.5  STOP

To temporary stop all parallel communication you send a CMD_STOP

command. The communication will then first start again when a

CMD_START command is broadcasted. Here is an example:

/* We want to temporary stop all parallel communication: */

ioreq->IOPar.io_Command = CMD_STOP;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not stop the parallel communication!\n" );

8.6  FUNCTIONS

DoIO()

DoIO() is used to send requests to a device, and waits for it

to be completed. While the program is waiting it is put to

sleep so it will not waste any computer time. DoIO() will

return first when the request have been completed or failed,

and no message is therefore sent to the reply port.

Synopsis: error = DoIO( req );

error:    (long) DoIO() will return first when the request has

been completed or something has failed. If the

request was successfully completed zero is returned,

else an error number is returned. What error number

depends on which device was used.

req:      (struct IORequest *) Pointer to the request you

want to have executed.

SendIO()

SendIO() is used to send requests to a device, but will

return immediately without any delay. To check if the request

have been completed use the CheckIO() function, or look

at the request's reply port for any messages. Once the

request has been completed you must remove the message at

the reply port. (CheckIO() will not do it.) To remove a

message use the function Remove(). Note that you may NOT

close the device before all requests have been completed or

aborted!

Synopsis: SendIO( req )

req:      (struct IORequest *) Pointer to the request you

want to have executed.

CheckIO()

CheckIO() is used to check if a previously started request

has been completed. Note that this function will not remove

the message at the reply port. This must be done with the

Remove() function.

Synopsis: ptr = CheckIO( req );

ptr:      (long) CheckIO() will either return NULL if the

request have not been completed or it will return a

pointer to the request block.

req:      (struct IORequest *) Pointer to the request you

want to check.

WaitIO()

WaitIO() will wait for the request to be completed, and while

the program is waiting it is put to sleep so no computer time

is wasted.

Synopsis: error = WaitIO( req );

error:    (long) WaitIO() will return first when the request,

that has previously been sent, has been completed or

something has failed. If the request was successfully

completed zero is returned, else an error number is

returned. What error number depends on which device

was used.

req:      (struct IORequest *) Pointer to the request you

want to wait for to be completed. Note that the

request must have already been sent to the device

by either a SendIO() or BeginIO() function call.

BeginIO()

BeginIO() is a low level form of the SendIO() function. The

advantage with BeginIO() is that no fields of the request

block will be altered as which is the case with SendIO().

With the parallel device you should only use SendIO().

Synopsis: BeginIO( req )

req:      (struct IORequest *) Pointer to the request you

want to have executed.

AbortIO()

AbortIO() will try to abort a previously started request. This

function should be used sparsely since it does not look so

good if you start a request and the try to stop it. (Better

not start it at all.) However, it is easy, and can sometimes

be very useful.

A request that is aborted will have its io_Error field set

to IOERR_ABORTED (defined in header file "exec/errors.h").

Synopsis: AbortIO( req )

req:      (struct IORequest *) Pointer to the request you

want to abort.

CloseDevice()

CloseDevice() will (surprise!) close a device. Note that you

should NOT close the device before all started asynchronous

requests have either been completed or aborted.

Synopsis: CloseDevice( ioreq );

ioreg:    (struct IORequest *) Pointer to the device's

request block.

OpenDevice()

OpenDevice() will try to open the specified device.

Synopsis: error = OpenDevice( name, unit, req, flags );

error:    (long) If OpenDevice() managed to open the device

it returns 0, else an error number is returned.

If you try to open the parallel device, and there is

already a program that is using it, the error

message "ParErr_DevBusy" is returned.

name:     (char *) Name of the device you want to open.

The name of the parallel device is defined as

PARALLELNAME in header file "devices/parallel.h".

unit:     (long) Which unit you want to open. Since there

exist only one parallel port, this field is ignored.

req:      (struct IORequest *) Pointer to a request block.

For the parallel device it must be a pointer to an

extender parallel request block (struct IOExtPar).

flags:    (long) Any special mode is set here. Ignored by

the parallel device.

8.7  COMMANDS

Here is a complete list of commands you may send to the

parallel device. For full documentation se examples above.

The special parallel device commands: (Defined in header file

"devices/parallel.h")

PDCMD_QUERY      Check the status of the parallel device.

PDCMD_SETPARAMS  Set the parameters of the parallel device.

The rest of the commands you may use are normal exec commands,

and are defined in header file "exec/io.h".

CMD_RESET  Resets all parameters of the parallel device.

CMD_READ   Read data from the parallel port.

CMD_WRITE  Write data to the parallel port.

CMD_STOP   Temporary stops all parallel communication.

CMD_START  Restarts parallel communication.

CMD_FLUSH  Removes all queued requests.

8.8  EXAMPLES

Example 1

This program demonstrates how you can use the Parallel Device.

It does not do very much since I do not know what you have

connected to your parallel port, but with small modifications

you should be able to write your own parallel communication

packages.

Example 2

This example is rather similar to Example 1, but this time

we do not wait for the parallel port to complete our request.

Instead we do somethings (well not very much) and now and

then checks if the request has been completed.

Example 3

This example is rather similar to Example 1, but this time

we do not wait for the parallel port to complete our request.

We are also trying to read and write at the same time. To be

able to do several requests simultaneously we need one request

block for each command. In this example we use three separate

request blocks.

Example4

This example does not do anything, but it consists of

several useful functions that you can use yourself after

small modifications. The functions demonstrates all

commands there exist for the parallel device, so if you

had problems in understanding how a command was used you

can look here.