1    AMIGA HARDWARE

1.1  INTRODUCTION

This chapter is intended to give you a brief introduction

of the Amiga and its hardware.

1.2  AMIGA MODELS

The first Amiga model was released in the middle of 1985 and

caused much publicity. It was the first "home" computer with

a so called "multi media architecture" (it can combine raw

data with astonishing graphics and stereo sound) and it was

using a new operating system with true multi-tasking (several

programs/tasks can run at the same time).

Although it was far superior to other available computers on

the marked, it did not sell as good as could be expected. The

main problem was, as with all new computers, that there did

not exist any software that used the special features which

the Amiga offered. The Amiga was expensive (although just a

fraction of a similar computer would cost), and most software

was simply "ported" from other computers and were therefore

extremely bad. (Who wants to work with an Amiga which feels

like an Atari or PC?)

However, slowly more and more programs were written specially

for the Amiga and people started to realize what the Amiga

really could do (and what other computers could not do). Two

new models (A2000 and A500) were released with even more

features and with a more attractive price tag.

Now everything started to accelerate. More people bought an

Amiga and therefore more programmers started to write software

for it, which encouraged even more people to buy it and so

on... During 1989 and 1990 the Amiga community grew so fast

that it became the best selling home computer.

The success has continued since then, despite what all so called

"professional" programmers have said. The Amiga has constantly

been looked down on by the rest of the computer world, accusing

it for being a game machine. The problem is that they do not

understand that a computer that can play stunning games with

stereo sound and fantastic graphics must have a computer power

far beyond other so called "professional" computers which hardly

could handle graphics not to mention sound.

The attitude towards sound and graphics have luckily slowly

changed, and now these so called "professional" people start

to dream about operating system which offers good graphics, and

maybe even some sound. I do not think we shall tell them that

this already existed 1985, with true multi-tasking, on a machine

which in their opinion "only" could play games...

1.2.1  AMIGA 1000

The Amiga 1000 was the first Amiga model, and was released

in the middle of 1985. The very first models were sold with

only 256 KB of memory, but this was soon changed to 512 KB.

At that time, this amount of memory was considered to be a

lot, but most people using the Amiga realized that more,

actually a lot more, could sometimes be needed.

The Amiga 1000 consists of a main box (a bit smaller that

the box of the A200) with a separate keyboard. It uses

a 7 MHz Motorola 68000 processor (see below for more

information), and came with DOS V1.0 and V1.1 (some are

even using V1.2 and V1.3). This model is still used by some

people, but more and more are switching over to newer models,

or changing the mother board so it becomes very similar to an

Amiga 2000. Most new programs does not support this model any

more.

1.2.2  AMIGA 2000

Two new models of the Amiga were released 1987/88 - Amiga 2000

and Amiga 500. The A2000 was intended for the more professional

part of the Amiga community. The main box contains five large

slots (100 pins) and two smaller ones (intended for IBM PC

cards), in total seven slots. To each slot you may connect

external devices like hard disks, extra memory, internal

modems, etc...

Both the A2000 and A500 were released together with DOS V1.2,

and was later upgraded to V1.3. It is now also possible to buy

the new chips for V2.0.

Both computers are sold with 512 KB of memory (so called "chip

memory"). When upgraded the next 512 KB of memory is called

"slow memory" and the rest (up to 8 MB) is called "fast".

The Amiga's special chips (see below) can only reach the "chip

memory", hence the name, while the main processor (the 68000)

can reach all 9 MB. The 512 KB above the "chip memory" is called

"slow memory" since the main processor may be interrupted by

the custom chips while working with this memory type, and this

memory is therefore not always as fast as the rest.

The later models of the Amiga 2000 are sometimes called

"2000C". This is because they have 1 MB of chip memory instead

of only 512 KB as previous models.

1.2.3  AMIGA 500

The smaller model is almost identical to its big brother A2000,

but does not have any internal expansion slots. The keyboard

and the computer are built together. It has only one (external)

expansion slot on the left side and a small internal expansion

slot for 512 KB of extra memory. This model is much cheaper than

the A2000 and was intended to become a home computer for the

family, which it one year later became.

Many people thinks that the A2000 is far superior to the small

A500, but this is actually not true. As said above, the only

difference between these two models are that it is easier to

expand (add on extra devices) an A2000, and that it has a

separate keyboard. The rest is identical.

1.2.4  AMIGA 3000

The Amiga 3000 is currently the most powerful model. It comes

with a Motorola 68030 processor which is boasted with a 16 or

25 MHz mathematical coprocessor. All Amiga 3000s are sold with

an internal hard disk of various sizes. This model comes with

the latest version of the operating system V2.0 (explained

below).

This model comes with the new "Enhanced Chip Set" (ECS) and

can address up to 2 MB of chip memory. On the mother board

can up to 16 MB be installed, and if using one of the slots

up to 1 Gigabyte can be used. This is a mean machine!

1.2.5  AMIGA 500+

This is a new version of the Amiga 500. It comes also with the

new "Enhanced Chip Set" (ECS) and can address up to 2 MB of

chip memory. This model is also sold with the new V2.0 operating

system.

It still uses the old and trusted 68000 processor, but is still

a nice update from the Amiga 500.

1.2.6  AMIGA 600

The latest model is the Amiga 600. This machine comes in a much

smaller box than the Amiga 500, and does not have any numeric

key pad. This will definitely cause some problems with programs

that are using these keys. However, this model is easier to

take to your friends, and will probably be cheaper in the end.

The Amiga 600 does not have the famous expansion slot on the

left side as the Amiga 500 and Amiga 500+ have. Instead there

is a new type of slot which is made for "electronic credit

cards". These small cards can contain programs, games, extra

memory up to 8 MB, and a lot of other useful stuff like sound

samplers and video digitizers.

Since there is no expansion bus on this model you can not

connect existing hardwares like hard disk, and memory

expansion slots to it. This is sad, but since the Amiga 600

can use an internal hard disk I do not think this cause any

problems.

This model will definitely be called a "game machine", and will

will undoubtedly appeal to many families who previously used

game consoles.

1.2.7  FUTURE MODELS

Everything is changing these days, and so is the Amiga. It is

impossible to say what is coming next, but I would not mind

an Amiga "4000" with 68040 processor, 1 Gigabyte hard disk, 16-

bit stereo sound channels and a 24-bit graphics cards with

built in genlock... One thing is sure at least, the Amiga will

continue to live, more than ever.

1.3  PROCESSORS AND SPECIAL CHIPS

The main processor on the Amiga is not specially strong or

powerful, but together with several coprocessors (extra

processors) which are specialized in doing certain things very

fast, the Amiga can do things other computers can not.

The main processor on the Amiga is a Motorola 680x0. The famous

coprocessors are called ("Fat") Agnus, Paula, and Denise. These

chips are close linked to each other and can only work with

"chip memory" (explained below). These coprocessors are often

referred as the "custom chips".

1.3.1  MOTOROLA MC680X0 PROCESSORS

The main processor on all Amiga models except the Amiga 3000,

is a simple Motorola 68000 with only 7 MHz. This is a very

good processor although it is not very fast. This processor can

physically address up to 16 MB, although the "maximum" on the

Amiga is 8 MB.

There exist newer models which are faster, and the nice thing

is that all these new versions can be used by the Amiga. The

first step up is the 68010, which is a bit faster than the

previous model. However, it costs more than it gives in speed,

and a faster 68000 in 14 MHz gives more value for money.

The 68020 is a fast processor which is fitted on many so called

turbo cards. If you give it 32 bit memory it can be very fast.

The Amiga 3000 comes with a 68030 which is very fast. It can

address up to 2 Gigabytes although the "maximum" on the Amiga is

1 Gigabyte. This processor is used together with either 16 MHz

or a 25 MHz mathematical coprocessor to speed up the floating

calculations.

The latest Motorola processor is the 68040, which is (surprise)

even faster.

If you intend to do a lot of calculations a fast processor is

essential. It should be noted that a faster processor does not

necessarily increase the overall speed that much. To take

advantage of the new versions you also need to use 32-bit

memory instead of the slower 16-bit memory. Most new turbo

cards comes both with a faster processor and 32-bit memory.

Mathematical coprocessors like the 68881 or 68882 are great

if you have to do a lot of floating calculations. Although all

programs does not make use of these extra coprocessors most

"serious" programs do.

1.3.2  BLITTER

The "Blitter" is a part of the Agnus chip. It is specialized

in moving data, and can even do logically calculations and bit-

shifting while the data is moved. It is the Blitter which is

used when BOBs are drawn, data from and to the disks are

transferred, when windows are moved etc... The Blitter can also

be used to draw lines with patterns and fill areas with a

colour or pattern.

1.3.3  COPPER

The Copper is a part of the coprocessors on the Amiga which

main task is to change colours and display modes while the

picture is "drawn" on the monitor/TV. It is because of the

Copper you can use several screens on the same display and drag

them up or down.

The Copper is also used to increase the number of colours on

the display by making colourful backgrounds with smoothly

coloured skies. Furthermore, the Copper can be used to

synchronize other parts of the Amiga like the Blitter, and

can be used as a timer.

1.3.4  AUDIO CHANNELS

There exist four audio channels on the which each can play

sounds independently of the other processes. They convert 8-bit

data into analog waveforms which are sent to the two audio

ports. Since two of the channels are dedicated for the left

port and the other two channels for the right port can stereo

sound be played.

You only have to give the audio channels some digital waveforms

or sampled data, tell them which rate and volume should be used,

and they can play the sound without using any other processing

time. No other home computer have yet matched the quality nor

the efficiency with which the Amiga plays sound.

1.3.5  HARDWARE SPRITES

The sprite hardware is a part of the custom chips. It can

handle eight hardware sprites which can independently of the

display below be moved around and animated. Each sprite can

only use four colours where one is transparent, but if they

are paired up to 16 colours can be used.

Since these sprites are handled by the hardware are they

extremely fast but also simple to use. With some clever

software routines can the eight hardware sprites be used

several times on the same display thus greatly increase the

number of available sprites.

The sprite hardware can also be used to detect collisions

between two sprites, something which can be very useful in

games.

1.3.6  COMPLEX INTERFACE ADAPTOR (CIA)

To tie all processors and other hardware parts there exist two

so called "Complex Interface Adaptors" (CIA - 8520) chips.

These two chips several timers, status bits etc... See header

file "hardware/cia.h" for a full list.

Normally should a C programmer not go this deep into the

system, but it is acceptable as long as you do it with help of

the macros and constants defined in the header files. A lot of

fun and useful stuff like flashing with the power led can be

done.

1.3.7  DMA

DMA stands for "Direct Memory Access" which means that the

hardware can directly get and store data in the memory without

going through the main processor. For example; each audio

channel has a DMA port and thus can each channel play waveforms

continuously without disturbing other processes.

Most parts of the Amiga (the blitter, sprite hardware, copper,

audio channels etc..) have separate DMA channels/ports. Each

process can then very efficiently execute their tasks without

interfering or disturbing other parts.

1.3.8  THE ENHANCED CHIP SET

The custom chips have several times been updated. The latest

version is called the "Enhanced Chip Set", and the chips can

address up to 2 MB of chip memory. These new chips also offers

new display modes and other nice things. I will cover this

subject in the next version of the manual. (Hopefully very

soon...)

1.4  AMIGA DOS V2.0

The new operating system V2.0 (2.05 and 2.05) has been out for

quite a while now. With this new version comes a new ROM chip

on 512 KB which replaces the old "Kick Start" of only 256 KB.

A lot of things have been added which will be covered in the

next version. I am sad that I could not manage to include it in

this edition, but do not worry, it is coming soon!

1.5  MEMORY

The Amiga is using different types of memory, and it can

sometimes be rather confusing. The problem is that some of

the coprocessors can only use one type of memory while the

main processor can use all types. It is best to give a short

explanation.

1.5.1  CHIP

The so called "chip" memory is the only type of memory that can

be used by the custom chips on the Amiga. Anything that should

be used by the Blitter, Copper, Sprite Hardware, Audio Channels

etc... must be in chip memory.

The first Amiga models had 512 KB of chip memory. Later the

Amiga 2000C was released with 1 MB of chip memory. The newest

versions, Amiga 3000, Amiga 500+, Amiga 600, can all access up

to 2 MB of chip memory.

The more chip memory you have the more graphics, sounds,

sprites etc can be used. If your program does not necessarily

have to use chip memory should you instead first try to use

other types of memory. This is explained in the first

introduction chapter of the manual.

1.5.2  SLOW

The so called "slow" memory exists only on old Amiga 500 and

Amiga 2000 models. These models can only access 512 KB of chip

memory. The following 512 KB can not be used by the custom

chips, but the memory can still be interrupted by the custom

chips. If the Amiga is using a lot of graphics or sound this

memory is often interrupted and therefore will all operations

with this type of memory be a bit slower.

1.5.3  FAST

The following 8 MB (or more if you have an Amiga 3000) are of

the type "fast memory". This memory can only be accessed by the

main processor, and will not be interrupted by the custom

chips.

1.6  HIT THE HARDWARE

Normally should all operations be made by the special functions

which are included in the libraries. You can then be sure that

your program will run without problems on even future models.

However, sometimes can it be necessary to directly modify the

hardware registers. If speed is essential or you want to write

a small and simple program you may use parts of the hardware.

The problem when you are using the hardware directly is that if

a new Amiga model is released it may happen that the resources

have been moved to a new location. The functions in the

libraries are always updated when the system is changed so any

program that is using these functions will continue to run as

before. However, programs that hit the hardware may crash on

the new system!

To use the hardware and still be relative safe you should use

two special global structures:

1. The Custom structure.

2. The CIA structure.

1.6.1  GET ACCESS TO THE CUSTOM CHIPS

The Custom structure is defined in header file "hardware/

custom.h". It is a very large structure that contains many of

the custom chips' hardware registers. This structure is

automatically linked to the custom chips, and any modifications

in the structure will directly affect the hardware registers.

To get access to the Custom structure you should write like

this: (The name must be "custom"!)

extern struct Custom far custom;

You can now start to modify the values in the custom structure,

but still be relative safe that your program will continue to

run on later Amiga models. See the header files and the

included examples for more information about how to use these

registers.

Note that some of these registers should be "reserved" and

"locked" before you may start to alter them. See chapter

"AudioiDevice" for more information. You can of course directly

modify the values, but you may end up modifying other programs'

values. This will normally not be any problems for the system,

but the other program will definitely not like it.

1.6.2  GET ACCESS TO THE CIA CHIPS

The CIA structure is automatically linked to the CIA chips.

There exist two CIA chips and therefore it also exist two

CIA structure (defined in the header file "hardware/cia.h"). To

get access to these structures you should write like this: (The

names must be "ciaa" and "ciab"!)

extern struct CIA far ciaa;

extern struct CIA far ciab;

You can now start to use the CIA registers. See header files

and included examples for more information about how eo use

these registers.

1.7  EXAMPLES

Example 1

This fantastic useful program does what all true hackers have

dreamt of. Enjoy your Amiga's fantastic ability to flash one

LED!

Example 2

This is a very bad program that plays a tune with the first

audio channel. The reason why it is bad is because the

program does not "lock" the channel before it uses the

hardware. It simply steals the sound channel without asking

nor notifying anyone.

If you lock the channel as described in the Audio Device

chapter you may use the hardware registers. However, this

program does not lock it nor tries to reserve the channels

as we should. The program is included here only as a

demonstration how the hardware registers are working, but

should not be used in your own programs!!!

See also programs in the chapter "DirtyInput".