Article 40640 of comp.sys.cbm:

Xref: undergrad.math.uwaterloo.ca comp.sys.cbm:40640

Path: undergrad.math.uwaterloo.ca!watserv2.uwaterloo.ca!torn!howland.reston.ans.net!nntp.crl.com!crl2.crl.com!not-for-mail

From: ceejack@crl.com (Jack Vander White)

Newsgroups: comp.sys.cbm

Subject: VIC-20 Info

Date: 31 Jul 1995 11:11:36 -0700

Organization: Humanity Against Late-Night Infomercials

Lines: 557

Message-ID: <3vj6co$7d4@crl2.crl.com>

NNTP-Posting-Host: crl2.crl.com

X-Newsreader: TIN [version 1.2 PL2]

Posted to comp.sys.cbm by request of the author.

======================

=                      =

=   Inside your Vic 20   =

=     By Ward Shrake     =

=                      =

======================

This document is a collection of pinout diagrams and technical notes, meant

to assist anyone who wishes to experiment electronically with the "obsolete"

Commodore Vic20 computer. Specifically, this document addresses the cartridge

or Memory Expansion port and related items, such as ROM & EPROM memory chips.

This is a very preliminary version of a document which could be expanded, if

I decide to go even more ballistic, later on. All this text was written out in

one day, with no prior planning, so if its a little jumpy, that's why. Later

versions should be better. Right now, probably anything helps, so here it is.

(Additions, corrections or whatever can be sent via email to the author.)

Ward Shrake, July 1995

email: wardshrake@aol.com

----------------------------------------------------------------------------

Pinout diagram #1: Memory Expansion port connector of the Vic20 computer

----------------------------------------------------------------------------

Below is the pinout diagram of the Vic20 cartridge port, or Memory Expansion

Connector. Please note that this is NOT the pinout for a cartridge that might

fit into any port, nor is it a pinout of the User Port, which is something

completely different. This diagram shows the cartridge port's pinout, as you

face the rear of the Vic 20 computer. (The port is on the Vic20's left side.)

Bottom side of expansion port.       Top side of expansion port.

(This half is on bottom.)            (This half is towards the keyboard.)

(This edge of the connector faces left,

where the power switch and the LED are.)

_________

|  ___  |

GND         A  |  | |  |   1       GND

CA0         B  |  | |  |   2       CD0

CA1         C  |  | |  |   3       CD1

CA2         D  |  | |  |   4       CD2

CA3         E  |  | |  |   5       CD3

CA4         F  |  | |  |   6       CD4

CA5         H  |  | |  |   7       CD5

CA6         J  |  | |  |   8       CD6

CA7         K  |  | |  |   9       CD7       (Memory Location:)

CA8         L  |  | |  |   10      BLK 1       ($2000 - $3fff)

CA9         M  |  | |  |   11      BLK 2       ($4000 - $5fff)

CA10        N  |  | |  |   12      BLK 3       ($6000 - $7fff)

CA11        P  |  | |  |   13      BLK 5       ($a000 - $bfff)

CA12        R  |  | |  |   14      RAM 1       ($0400 - $07ff)

CA13        S  |  | |  |   15      RAM 2       ($0800 - $0bff)

I/O 2       T  |  | |  |   16      RAM 3       ($0c00 - $0fff)

I/O 3       U  |  | |  |   17      V R/W

S02         V  |  | |  |   18      C R/W

NMI         W  |  | |  |   19      IRQ

RESET       X  |  | |  |   20      NC

NC          Y  |  | |  |   21      +5 Volts

GND         Z  |  | |  |   22      GND

|  ---  |

---------

(This edge of the connector is on the

right, where all the other ports are.)

Pinout Notes:

-------------

BLKxx =  8K decoded RAM/ROM block xx, active low. See chart above for area.

CAxx  =  Address bus line xx

CDxx  =  Data bus line xx

C R/W =  Read/Write line from CPU. (Read = high, Write = low)

GND   =  System ground

I/O 2 =  Decoded I/O block 2, starting at $9130

I/O 3 =  Decoded I/O block 3, starting at $9140

IRQ   =  6502 Interrupt Request line (active low)

NC    =  No connection

NMI   =  6502 Non-Maskable Interrupt line (active low)

RAMxx =  1K decoded RAM blockxx, active low. See chart above for memory area.

RESET =  6502 reset line (active low)

S02   =  Phase 2 system clock

V R/W =  Read/Write line from Vic chip. (Read = high, Write = low)

-----------------------------------------------------------------------

Pinout diagram #2: A standard Vic20 cartridge's card edge connector

-----------------------------------------------------------------------

Below is a pinout diagram of a standard Vic20 cartridge, seen facing its card

edge connector. Please note that this is NOT the pinout of the port it plugs

into, which is shown above. The two pinouts are exact opposites, because you

are facing the port head-on as is, but you have to flip a cartridge around

180 degrees to be able to look at it head-on, facing its card edge connector.

Bottom side of cartridge      Top (label or component) side of cartridge

_-_

GND         Z   | |   22    GND

NC          Y   | |   21    +5 Volts

RESET       X   | |   20    NC

NMI         W   | |   19    IRQ

S02         V   | |   18    C R/W

I/O 3       U   | |   17    V R/W     (Memory location:)

I/O 2       T   | |   16    RAM 3       ($0c00 - $0fff)

CA13        S   | |   15    RAM 2       ($0800 - $0bff)

CA12        R   | |   14    RAM 1       ($0400 - $07ff)

CA11        P   | |   13    BLK 5       ($a000 - $bfff)

CA10        N   | |   12    BLK 3       ($6000 - $7fff)

CA9         M   | |   11    BLK 2       ($4000 - $5fff)

CA8         L   | |   10    BLK 1       ($2000 - $3fff)

CA7         K   | |   9     CD7

CA6         J   | |   8     CD6

CA5         H   | |   7     CD5

CA4         F   | |   6     CD4

CA3         E   | |   5     CD3

CA2         D   | |   4     CD2

CA1         C   | |   3     CD1

CA0         B   | |   2     CD0

GND         A   | |   1     GND

-_-

--------------------------------------------------

Pinout diagram #3: EPROM # 2764A

(This is a standard, 8K x 8 bit memory chip)

--------------------------------------------------

____    ____

|   !__!   |

Vpp | 1     28 | Vcc  (+5 Volts)

A12 | 2     27 | PGM  (Active low)

A7  | 3     26 | N.C. (No connection)

A6  | 4     25 | A8

A5  | 5     24 | A9

A4  | 6     23 | A11

A3  | 7     22 | OE  (Output Enable; Active low)

A2  | 8     21 | A10

A1  | 9     20 | CE  (Chip Enable; Active low)

A0  | 10    19 | D7

D0  | 11    18 | D6

D1  | 12    17 | D5

D2  | 13    16 | D4

GND | 14    15 | D3

|__________|

See Note 6 below, for some tips on using this as a replacement memory

chip, on a modified cartridge body, for experimentation purposes. Note

that there are differences between it and the standard Vic20 chip below,

that will have to be accounted for before it can be wired to the boards.

--------------------------------------------------------

Pinout diagram #4: "MPS 2364" ROM chip

(Commodore-standard, 24-pin ROM chip, 8K x 8 bit.)

--------------------------------------------------------

____    ____

|   !__!   |

CA7  | 1     24 | +5 Volts

CA6  | 2     23 | CA8

CA5  | 3     22 | CA9

CA4  | 4     21 | CA12

CA3  | 5     20 | CS (Chip select, active low)

CA2  | 6     19 | CA10

CA1  | 7     18 | CA11

CA0  | 8     17 | CD7

CD0  | 9     16 | CD6

CD1  | 10    15 | CD5

CD2  | 11    14 | CD4

GND  | 12    13 | CD3

|__________|

This pinout was derived from a Vic20 schematic, found in the book the

"Vic20 Programmer's Reference Guide". (Great book!) Please note that while

this is an 8k-by-8-bit Commodore memory chip (its actually the Kernal chip,

located at $E000-$FFFF), and that while it could normally be assumed safely

that a company would standardize and use the same chips in their cartridges

that they used in their computer's motherboards, this is Commodore we're

talking about. A bit of paranoia might be in order. Having said that, I'll

note that as of this writing, the author has not compared this diagram and

an actual memory chip from a Commodore-produced Vic20 cartridge. 'Nuff said!

---------------------------------------------------------------------------

Assorted notes on the diagrams above, and related subjects. (No order.)

---------------------------------------------------------------------------

Note 1: Information for the diagrams above was taken from the Vic20 reference

book "The Vic Revealed" by Nick Hampshire, 1982, Hayden Book Co, Inc.

That info was verified by checking it against information found in

the "Vic20 Programmer's Reference Guide," 1982, by Commodore Business

Machines, Inc. and Howard W. Sams & Company, Inc.

Note 2: You are looking at the cart pinout as if you were holding an unopened

cartridge, label-side up (or right, in these diagrams) with the gold

fingers pointing at you. If you are looking at a bare circuit board

once it has been taken out of its outer plastic case, the chip is on

top (or right, as shown here) again with the gold fingers facing you.

(The diagrams were drawn sideways, as the ASCII drawings are somewhat

clearer this way, believe it or not. There is less confusion this way

over which pin is which. Horizontally, each name takes up more space.)

Note 3: Radio Shack currently (as of July 1995) carries a part that works as

a Memory Expansion Port connector for the Vic20. However, they are

discontinuing it, and are selling off all remaining stock. Their part

number is 276-1551 for their "44-position card-edge connector." Once

these are gone, you'll have to find another source for them. ($1.27)

This part is useful for experimenting with this port on the Vic20, as

it is a perfect fit, as is, to accept standard plug-in Vic cartridges.

Note 4: A perhaps useful experimenter's tip: If you plan to take the circuit

board out of the cartridge case, then remove the ROM chip(s) from the

circuit board, mark the two sides of the board first, or you may no

longer be able to tell which is the top, and which is the bottom!

This is especially true if one is removing chips to install sockets.

(Any magic marker will do; just write "Bottom" on the "green stuff".

Do not write on top of any exposed metal surfaces or the gold pins!)

Note 5: For experimenting purposes, it might be handy to find a cartridge you

no longer care about, desolder the existing ROM chip from the circuit

board inside the cartridge and install an IC socket to make it easier

to change chips, later. However, note two things about this. One, the

socket makes the whole thing taller, and it may no longer fit into a

standard case. You can either (carefully!) use the new circuit board

without using its outer case, at least for in-house testing, or you

may be able to cut a "window" in the top of the case to clear it all.

Second, some cartridges don't use standard IC ROM's, so try another

cartridge, until you find one that does use a standard DIP package.

Note 6: On all the Vic20 carts the author has opened and seen so far, the ROM

chip inside has 24-pins. Standard replacement EPROM's have 28-pins.

This creates problems, but not insurmountable ones. It is a bit of a

nuisance, but the dedicated experimenter can modify a 24-pin circuit

board, to accept any 28-pin standard EPROM, such as the 2764 package.

If one did not want the hassle of doing this, one other method exists

to be able to put your own 8K eprom chips inside; however, it is not

cheap. Motorola makes an 8K EPROM that is completely compatible with

Commodore's "standard" 24-pin ROM chips. It even works as a direct

pin-for-pin replacement for the 8K Kernal and Basic ROM's inside your

Vic20 or your C64; the author has replaced both types in the past.

The part number for this EPROM is MCM 68764. It is about $18.00 each.

(Yes, it is programmable via the C64's "Promenade" eprom burner too!)

I think I got my 68764 years ago, from Jameco Electronics; try them.

(A 2764 adapter circuit board can be made to adapt the differences in

24-to-28 pin sockets, but it will likely be too tall to fit inside.)

Note 7: A reset button can be wired into the Vic20 fairly simply. This might

be a good idea, if one plans to experiment much. The Vic20, or any

computer, might lock up when you've told it to do something odd while

experimenting. Simply use a momentary-contact, normally-open switch,

wired between the RESET line and any port GND line. When you press

the new button for a second and let go, the computer will reset. This

is because the RESET line is active low, and your switch grounds that

line. If you decide to do this *inside* your Vic20, remember you are

now looking at the *back* of the port, NOT the front. Therefore, use

the pinout diagram for a cartridge, not the port, to find the lines.

Radio Shack switch part number 275-1547 or equivalent will

work fine.

Note 8: The memory area located at $A000-BFFF ("Block 5") is normally

considered to be a ROM-only block of memory. RAM can be mapped into

that space if you modify the cartridge's internal wires or jumpers,

to redirect a standard RAM cartridge to load higher up in memory.

However, BASIC cannot normally access that area, as it is meant to

be for ROM's only. This is important, if one wishes to archive an

8K autostart ROM cartridge (normally located in block 5), and run the

resulting ROM image from within RAM. It can be done, but you have to

modify a standard 8K RAM cartridge to do it. (Ground BLK 5, and cut

any trace or jumper that tells the cart to load in a different area.)

Note also that some carts may be copy-protected, and will not run in

RAM without modification or "training," to alter the copy protection.

It may be possible to get around this copy-protection (but the author

hasn't tried it yet) by modifying the read/write lines, so that after

loading the program's ROM image into RAM memory, you flip a switch

first, then type in the SYS command to start the program up. If this

idea works, it would emulate ROM by not being able to be written to,

when the copy protection tries to determine if the copy is in RAM or

ROM. Of course, when reprogrammed into an EPROM, it will act as if it

were ROM anyway, so this may no longer matter in that case.

Note 9: Trying to relocate a machine language program from the memory area

where it was written in and intended to be, and to have it actually

work afterwards, is not advised. It won't work, without doing so

many modifications to the machine language object code that you may

as well just start over and reprogram it all. Don't bother trying.

It seems the idea is to get the real, actual code as written, anyway.

Similar problems confront the user who wishes to archive a tape or a

diskette-based original. Trying to make a cartridge out of it, isn't

worth the huge headaches. You are better off, in this case, trying to

archive tape-based programs to floppy disk (which will still require

a little modification, but not as much), and floppies to floppies.

Note 10: Some FTP archives exist on the Internet for the Vic20 enthusiast.

("ftp.funet.fi" in Finland, and "ftp.hrz.uni-kassel.de" in Denmark.)

When downloading archived ROM images from these places, one should

be aware that they are missing two bytes that the Commodore Vic20 and

the Commodore 64 computers consider absolutely critical to load and

use files with. These are the first two bytes of any Commodore file,

which tells the computer a program's "Starting address," or where to

load it into memory. If its not in the right place, it will not work.

Period. Even one byte off, and it will not work. These raw image

files include only the ROM's internal codes, and no starting address.

The FTP site in Denmark has some instructions on how to add these two

bytes to a file, using a Commodore Vic20 compatible disk drive. The

author made an executable program that does this on the IBM, but it

isn't very user-friendly at this point. It does work, however, and as

soon as it will pass muster publicly, I plan to upload it to both FTP

sites. This program, when available to the public, is meant to be used

after a ROM image file has been downloaded onto an IBM computer, but

before the user transfers it to a Commodore-compatible diskette, using

whatever process the user likes most. (The "X-1541" user-built cable

which connects the IBM printer port to the CBM serial port, is the

author's favorite way of transferring files from IBM to CBM and back.)

Note 11: On all the NTSC Vic20 motherboards I looked at (4 or 5), the BASIC

memory chip is labeled "UE11", while the Kernal chip is labeled as

"UE12". My PAL schematic lists these two as UD5 and UD6, respectively.

The Basic memory map area is at $C000-DFFF. Kernal is at $E000-FFFF.

The only other 24-pin chip is the Character Generator; its 4K, not 8K.

Ward Shrake

email: wardshrake@aol.com

July 1995

-------------------------------------------------------------------------

Short technical tutorial on Vic20 ROM cartridges, including notes

on cartridge archiving methods, and the Vic autostart process.

-------------------------------------------------------------------------

A Vic20 cartridge is approximately 5.5 inches wide, 3 and 3/8ths inches deep,

and 5/8ths inches tall, when viewed as if ready to be installed. Standard

configurations are 8K and 16K of 8-bit ROM memory, using one or two chips of

ROM memory onboard. If other standard cartridge memory configurations exist,

the author is currently unaware of them. (But would like to hear of any, if

they do/did exist, and especially if they were once made commercially.)

Inside a typical Vic20 cartridge is one double-sided, etched circuit board,

with some form of memory chip installed. This may be a "standard" IC chip as

we are used to seeing (24 or 28 pin ROM, in a DIP package), or it could be

a blank circuit board, with a tiny blob of black epoxy material on it, under

which are presumably the internal components of a typical, normal ROM chip.

The standard circuit board size is approximately 3 and 9/16ths inches wide,

and 1 and 3/4 inches deep. A cart fits into a 44-position, double-sided card

edge connector, which is a permanent part of the Vic20 itself, in the back.

On the cart circuit board itself may (or may not) be several wire traces or

jumpers, which if there, are meant to configure the cartridge to a certain

memory arrangement. These jumpers are meant to connect the BLK or RAM lines

to the system ground. These lines, when connected, tell the Vic20 where to

place the cartridge within the Vic20's internal memory map scheme.

While all "normal," autostart game cartridge are located in one fixed area

of memory, this is not true with all cartridges, game or not. Some of the

earliest cartridges did not use the autostart procedure for whatever reason,

(presumably because they did not know it existed, at that early stage?) and

those are located in "non-standard" memory locations. The Vic20 was built

and designed when memory was expensive, and computers were designed to be

bare-bones memory-wise, but with the capability to expand on later. This may

take some getting used to at first, but understanding it gets easier in time.

16K cartridges, for example, generally have one 8K ROM block located from

$A000-$BFFF, and another 8K ROM block located at $6000-7FFF. If a cartridge

was factory-made not to autostart; in other words, if it needs you to type in

a "SYS" number to start it, its memory could be located in any block.

The "normal" spot for an 8K cartridge to be located in memory is in "Block 5"

(which is located at $A000 to $BFFF). Again, this is not the only possible

spot in memory for a cartridge to be located, but it is the only spot where

the Vic20 will look for a cartridge that automatically starts on power-up.

This is because (at power up) the Vic20 looks for a precise code in a precise

spot, to see if it should autostart a cartridge or not. If the Vic20 finds

this *exact* five-byte code, *exactly* where it is supposed to be located in

memory, the Vic20 turns control over to the cartridge. If not, it gives over

control to the user, via the normal, power-up Basic READY prompt screen.

This 8K autostart sequence code is shown below:

Address______Hex value_____Decimal value____ASCII values

$A004        $41           65               Capitol "A"

$A005        $30           48               Digit "zero"

$A006        $C3           195              Reverse "C" character

$A007        $C2           194              Reverse "B" character

$A008        $CD           205              Reverse "M" character

If the computer finds this five-byte sequence exactly as shown, it

turns control over to the machine language program in the cartridge.

To do so, it needs to know where the program begins. There are four

bytes which determine this, as shown below: (Also within the cart.)

Address______What this byte of information contains__________

$A000        Low-byte of a 16-bit "hard start" address (to begin at)

$A001        High-byte of the 16-bit "hard start" address

$A002        Low-byte of 16-bit "warm start" address (restore key)

$A003        High-byte of 16-bit "warm start" address

This is all very important when trying to archive an image of the information

contained on a ROM cartridge. Altering either of the two required parts of

this sequence, causes the cartridge to not autostart, but to be mapped into

memory properly, anyway. Assuming a certain level of technical expertise, a

user can do one of two (or more?) things to achieve this.

One is to move the cartridge's starting location temporarily, so that the

code is present, but not in the right place. This can be done by altering the

physical traces on each cartridge's circuit board. This is undesirable,

except perhaps for self-training purposes, as every single cartridge would

have to be individually modified, archived, then reassembled for normal use.

Permanently modifying the Vic20's internal wiring is also possible, but

hardly desirable, as permanent damage to the Vic20 might also result.

The best way I could recommend for this would be to have a qualified tech

build or wire a cartridge-port pass-through device, where nearly all the

traces re-connected where they are supposed to, except for a critical few.

Using a 44-position card edge connector, some wire and a junk cartridge (or

etched circuit board like a cartridge's pins), this could be done. Most wires

would start at the card edge, where you'd plug the device into the Vic20, and

pass-through harmlessly to the card edge connector, where carts would plug in.

This device reroutes the "BLK 5" trace, preferably by use a "normal/modified"

switch, so that even though the cartridge intends to load at $A000, it loads

to another 8K block of memory instead. Whichever one you wire it to; perhaps

Block 1 or 3. Make the switch(es) act as a simple pass-through, normally, but

disconnect the "Block 5 to ground line" while at the same time connecting the

"Block X line to ground", to remap where the cartridge will temporarily go.

This device would ideally be plug-in-able and removable, like a cartridge, so

as to require no modifications to either the Vic20 or cartridges. A reset

button could also be installed in the device, as well, while you're at it. If

you use an old cartridge as your plug-in on the Vic20 end, cut all traces that

still go to the existing chip(s). Solder wire ends onto the portions that exit

the Vic20, and route them to your external plug-in connector box. If you use

ribbon cable as your wiring, you can probably even sneak the wire out the back

of the existing cartridge's case, as there is a wide slot in most Vic cases!

This method will allow you to temporarily re-map where the cartridge image

will appear in memory, to defeat the autostart feature. Once this is defeated

the technically-inclined user who is familiar enough with Commodore computers

can copy the information to tape or diskette for storage. This takes some

knowledge and expertise, but can be learned. Some help is offered; see below.

The user should understand that if this method (memory relocation) is used to

defeat the autostart feature, and to thereby attempt to archive a cart's ROM,

the work is not over. Once archived to disk, the hard part is done, true, but

when you saved it to diskette, it was saved with a now-incorrect starting

address. Before the image can be used further, this has to be corrected.

This can be changed, fairly easily, by someone who knows how, but it requires

knowledge of using a software tool called a "disk sector editor" to change

the now-incorrect high-byte of the relocated starting address, to the correct

unrelocated high byte: where it would have been, before you moved it. To do

so, you find the first disk sector of the ROM image file, and change the

fourth byte in (not byte $04!) to be $A0, resave the sector, and you're done.

After this modification, your ROM image files will now load where they should,

(into $A000 RAM, if you have it), and are ready for further processing.

Even the common C64 "Fastload" utility cart has a very servicable sector

editor in it, so it isn't as bad as it may have sounded. Still, you need to

know how to use it, and what to change, so it is best left to a professional.

To actually save the block of memory to tape/disk, you either use the memory

save feature of a machine language monitor program (not built in) to save the

8K block of ROM memory temporarily, or you can change certain POKE's in the

Vic20's memory, to tell it your cartridge is located in the Basic program

memory area, so that the normal SAVE command works. The beauty of the latter

is that no additional programs are needed to make this work; just type in four

POKE commands, then tell it to SAVE as it would normally save a Basic program.

For those of you already familiar enough with Commodore's style and memory

arrangement schemes to make sense of this, bytes 43 and 44 (decimal, not hex)

are the pointers to the Start of Basic memory area, and bytes 45 and 46 are

the pointers to the Start of Variables, or in other words, the End of Basic.

Follow this chart, to save a block of memory from these areas via pokes:

Block #___Hex Address____Poke 43,x___Poke 44, x___Poke 45, x__Poke 46, x

1       $2000-3fff      x = 0       x = 32       x = 255     x = 63

2       $4000-5fff      x = 0       x = 64       x = 255     x = 95

3       $6000-7fff      x = 0       x = 96       x = 255     x = 127

5       $a000-bfff      x = 0       x = 160      x = 255     x = 191

The above is only one possible way to archive an 8K block of Vic20 memory.

However, a second method exists, requiring some Vic20 modifications. A truly

hard-core Vic user could alter the copy of the startup code that is stored in

the Kernal ROM chip, so that no normally-correct autostart sequence code ever

matches it as long as your new Kernal is in place.

Remember, there must always be *two* copies of the start-up code; one in the

cartridge itself and one stored in the computer for comparison. If either of

these two seperate codes are altered, even slightly, it will not autostart!

The cartridge gets mapped into memory at its correct location, but the Basic

"READY" screen comes up instead, giving the user full control. Once you've

achieved this, you simply do four pokes (to change the start and end of BASIC

memory to match where the cart code is), or save the info with an ML monitor.

With some work and an "MCM 68764" 8K EPROM, this can be done. It is not just

a vague theory; the author has done it. It works, as does a similar technique

for doing the same general thing with the Commodore 64 computer. (With the

appropriate changes, of course, to reflect the C64's different architecture

and memory map; the C64 codes are "CBM80" instead of "A0CBM", and the Kernal

expects that code to be in a cartridge at locations $8004-$8008 instead of

$A004-$A008. Starting address is in the same exact format as the Vic's.)

This a much more involved modification, and not for the squeemish or

those who only own one Vic20 (or C64) without any spares to fall back on!

To do it, you need a heck of a lot of computer knowledge specific to the

computer you're working on, an eprom burner, an eprom eraser, and patience.

You have to search in the Kernal's memory image, for the start-up code, and

modify those bytes to be something different than they are, while leaving all

the other bytes just as they are. This alone takes some serious skills, no?

You also need skills and tools in desoldering and removing IC chips, as well

as soldering in IC sockets, and not destroying anything in the process.

I won't go into this much, because I don't imagine many people will bother to

actually attempt this method, since the other has some serious pro's to it,

and since this should be about enough info to go on; if you can't figure out

how to do this modification with this info, you probably should not try it,

until you've progressed enough at a later time. Or, have someone else do it

for you, with these instructions, if you really want it done badly enough.

This way may appeal to some people, but I recommend the other for most folks.

The overly-technical types might be tempted to go ballistic however, and more

power to you! A thought, in parting; one could put 2 copies of the Kernal ROM

chip, one stock and one modified, into one 16K eprom, with an external switch

to flip between the two versions. (Or just go lower tech, and just manually

swap out Kernal chips as needed; most Vic20's seem to have sockets anyway.)

Whatever you do, I hope you've enjoyed this article! Later, all!

Ward Shrake

wardshrake@aol.com

July 1995