Is WP-ARC.TI Compatible with my hardware?

Hardware requirements:

Console, TV or monitor, 32K, XB, 2 disk drives, either CorComp disk controler &

CorComp Disk Manager program or TI disk controler and EA module,

7 bit daisy wheel printer.

If you have the TI disk controller, you need the EA module.

If you have the CorComp disk controller, you do NOT need the EA module.

IMPORTANT! Printer MUST IGNORE 8th bit! A daisy wheel printer which uses the 8th

bit to access special characters is INCOMPATIBLE with WP-ARC.TI as the 8th bit

is used as a flag by the EDITOR to tell the formatter to precede the 7 bit

character with CHR$(27). [Send an escape sequence to the printer].

Printer MUST recognize the following control codes and escape sequences and the

stated meaning for each:

Control H = Backspace

Control I = Horizontal Tabulator

Control J = Line Feed

Control L = Form Feed

Control M = CR

Control S = Wait for operator to press SEL button on front panel before resuming

printing

CHR$(27)  = Escape

Escape sequences:

Esc+BS    = 1/120" backspace in Proportional spacing mode

Esc+HT+n  = Absolute Htab. Example: Esc+HT+CHR$(6) = HTAB to 6th whole character

position or 50/120" from left margin.

Esc+LF    = Reverse line feed

Esc+FF+n  = Page length set (in lines). Example: Esc+FF+CHR$(66) = set page

length to 66 lines (11 inches).

Esc+CHR$(31)+n = set character horizontal pitch to (n-1)/120"

Example: Esc+CHR$(31)+CHR$(11) = set pitch to 10/120" per

character. or 12 characters per inch.

Esc+"     = Auto LF on. When sent, CHR$(13) causes both CR & LF

Esc+&     = Clear Shadow printing, double strike or (if the printer has it)

boldface.

Esc+1     = Set an HTAB stop at current position

Esc+A     = Switch to spool ribbon (usually a red ribbon where the correction

ribbon would be on an electronic typewriter).

Esc+B     = Return to normal black cartridge ribbon

Esc+C     = Clear top & bottom margins

Esc+D     = Feed paper back 1/2 line for superscript or return from subscript

Esc+E     = Start auto underline

Esc+P     = Switch to PROPORTIONAL SPACING mode regardless of previous

horizontal pitch setting

Esc+R     = Stop auto underline

Esc+U     = Feed paper 1/2 line for subscript or return from superscript

Esc+W     = Start SHADOW printing (print character then advance printhead 1/120"

then restrike same character)

Esc+Y     = Print "cents" character on an ASCII or American English typewriter

daisy wheel, or the Greek letter "mu" (as in microfarads)

on a German/DIN 2137) daisy wheel

Esc+Z     = Print a "reverse grave" with an ASCII daisy wheel (character

printed may vary with other character sets)

If your printer acceps ALL of the above control codes and escape sequences, and

also accepts other escape sequences while IGNORING the 8th bit, then your

printer is 100% compatible with WP-ARC.TI

The Brother printers (HR-15XL, HR-25, & HR-35) [parallel interface]

and the Comrex ComRiter CR-IIE [parallel interface] are known to be 100%

compatible, and there may also be other 100% compatible printers I am not

aware of.

Try FASTERM to download. 4A/TALK is notorious for turning a Dis/Fix 128 file

(the compressed ---ARC.TI files) into Dis/Var 80 files on download.

[unreadable by the UNARC function of ARCHIVER.TI ]