Hidden Chapter Field Code.  Do Not Delete

## CSTARetrieveEventXE "CSTARetrieveEvent"§

This event report identifies a call which was previously on hold and has been retrieved at a device. This is equivalent to taking the call off the hold state and into the active state.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTARetrievedEvent_t  retrieved;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			retrievedConnection;

SubjectDeviceID_t		retrivingDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTARetrievedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_RETRIEVED, which  identifies  this message as an CSTARetrievedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

retrievedConnection

This parameter specifies the Connection for which the call has been taken off the hold state.

retrievingDevice

This specifies the device which de-activated the call from the hold state.

localConnectionInfo

This parameter defines the local connection state of the call after the call has been retrieved from the hold state. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event informs the application that a call is no longer on hold. This can occur if the end-user physically takes the call off the hold state or in response to the cstaRetrieveCall( ) function request.

Figure 6-12<bookmark retrieve_event>

Retrieved Event Reporttc "Retrieved Event Report" \f f \l3§

µ §

## CSTAServiceInitiatedEventXE "CSTAServiceInitiatedEvent"§

This event report indicates to the application that telephony service was requested at a device. This is equivalent to getting dial tone on a standard analog telephone.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{	   struct

{	      CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAServiceInitiatedEvent_t  serviceInitiated;

}u ;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			initiatedConnection;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAServiceInitiatedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_SERVICE_INITIATED, which  identifies  this message as an CSTAServiceInitiatedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

initiatedConnection

This parameter indicates the Connection for which service (dial tone) has been established or a feature is invoked. The same Connection identifier will continue to be used if a call is eventually established by the device.

localConnectionInfo

This parameter defines the local connection state of the call after the service has been initiated. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event will not occur every time a call is established or launched from a device. For example, the event will not occur with functional type devices (e.g. ISDN BRI devices) when services is being requested by taking the device off-hook (dial tone state). The event will also not occur when a call is established using the cstaMakeCall( ) function.

Figure 6-13<bookmark sinit_event>

Service Initiated Event Reporttc "Service Initiated Event Report" \f f \l3§

µ §

## CSTATransferredEventXE "CSTATransferredEvent"§

This event report indicates that an existing call was transferred to another device and that the device which transferred the call is no longer part of the call, i.e. the transferring device has dropped from the call.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTATransferEvent_t  transferred;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			primaryOldCall;

ConnectionID_t			secondaryOldCall;

SubjectDeviceID_t		transferringDevice;

SubjectDeviceID_t		transferredDevice;

ConnectionList_t		transferredConnections;

LocalConnectionState_t 	localConnectionInfo;

CSTAEventCause_t		cause;

} CSTATransferredEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_TRANSFERRED, which  identifies  this message as an CSTATransferredEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

primaryOldCall

This parameter identifies the primary known call that was transferred.

secondaryOldCall

This parameter identifies the secondary call that was transferred. This would identify the consultative call used to make the transfer, after the primary call was placed on hold.

transferringDevice

This indicates which device transferred the call. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

transferredDevice

This indicates to which device the call was transferred. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

transferredConnections

This is a list of connections (parties) on the call which resulted from the transfer.  The call ID may be different from either the primary or secondary old call (or both)..

localConnectionInfo

This parameter defines the local connection state of the call after the calls have been transferred from the device which performed the transfer. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event provides the application with all the information it needs regarding a call which was transferred from one device to another.

Figure 6-14<bookmark trans_event>

Transferred Event Reporttc "Transferred Event Report" \f f \l3§

µ §

# Feature Event Reports (Unsolicited)XE "Feature:Event:Unsolicited"§

TSAPI feature event reports indicate a change in the state of a specific feature operating on a call or a device on the switch. Each feature eventXE "Feature event"§ gives the current stateXE "Feature state"§ of the feature regardless of what the state of the feature was before an application receives a feature event.

## CSTACallInfoEventXE "CSTACallInfoEvent"§

This event report is provided when a user account code feature has collected data for a party on the call. The event includes the account code and authorization information which was collected by the switch feature.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and  CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTACallInfoEvent_t  callInformation;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t		connection;

SubjectDeviceID_t	device;

AccountInfo_t		accountInfo;

AuthCode_t		authorizationCode;

} CSTACallInforEvent_t;

typedef char            AccountInfo_t[32];

typedef char            AuthCode_t[32];

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_CALL_INFORMATION, which  identifies  this message as an CSTACallInfoEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

connection

This parameter identifies the party that has entered the account code.

device

Indicates from which device was the account code information entered. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

accountInfo

Specifies the account code which was entered at the device.

authorizationCode

Specifies the authorization code which was entered at the device.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event informs the application when an account code feature has been activated and what information was collected by the switch as a result of the feature being activated.

## CSTADoNotDisturbEventXE "CSTADoNotDisturbEvent"§

This event report indicates a change in the status of the Do Not Disturb feature for a specific device. The Do Not Disturb event will result in all calls to a device to be automatically forwarded to the device coverage path.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

CSTAEventCategory_t	eventCategory;

union

{

CSTADoNotDisturbEvent_t  doNotDisturb,

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t	device;

Boolean			doNotDisturbOn;

} CSTADoNotDisturbEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_DO_NOT_DISTURB, which  identifies  this message as an CSTADoNotDisturbEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

device

Specifies the device for which the DO Not Disturb feature has been activated/deactivated. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

doNotDisturbON

Specifies whether the DO Not Disturb feature is on (1)  or off (0).

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

## CSTAForwardingEventXE "CSTAForwardingEvent"§

This event report will indicate a change in the state of the Forwarding feature for a specific device. The event will also indicate the type of forwarding being invoked when the feature is activated.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct 
{
	ACSHandle_t	acsHandle;
	EventClass_t	eventClass;
	EventType_t	eventType;
} ACSEventHeader_t;

typedef struct 
{
	ACSEventHeader_t	eventHeader;
	union 
	{
		struct 
		{
		   CSTAMonitorCrossRefID_t	monitorCrossRefID;
			union
			{
				CSTAForwardingEvent_t  forwarding;
			} u;
		} cstaUnsolicited;
	} event;
} CSTAEvent_t;

typedef struct 
{
	SubjectDeviceID_t	device;
	ForwardingInfo_t	forwardingInformation;
} CSTAForwardingEvent_t;

typedef enum ForwardingType_t {
	FWD_IMMEDIATE = 0,
	FWD_BUSY = 1,
	FWD_NO_ANS = 2,
	 FWD_BUSY_INT = 3,
	FWD_BUSY_EXT = 4,
	 FWD_NO_ANS_INT = 5,
	 FWD_NO_ANS_EXT = 6
} ForwardingType_t;
typedef struct ForwardingInfo_t {
	ForwardingType_t forwardingType;
	Boolean	forwardingOn;
	DeviceID_t 	 forwardDN;	/* NULL for not present */
} ForwardingInfo_t;
Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_FORWARDING which  identifies  this message as an CSTAForwardingEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

device

Specifies the device for which the Forwarding feature has been activated/deactivated. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

forwardingType

Specifies the type of forwarding being invoked for the specific device. This may include one of the following:

| Immediate | Forwarding all calls |
| --- | --- |
| Busy | Forwarding when busy |
| No Answer | Forwarding after no answer |
| Busy Internal | Forwarding when busy for an internal call |
| Busy External | Forwarding when busy for an external call |
| No Answer Internal | Forwarding after no answer for an internal call |
| No Answer External | Forwarding after no answer for an external call. |

forwardingON

Specifies whether the Forward feature is on (1)  or off (0).

forwardDN

Specifies the destination device to which the calls are being forwarded. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

The application should be aware that the forwardingInfo paramater can indicate any of the defined values depending on the switch implementation of the forwarding feature.

## CSTAMessageWaitingEventXE "CSTAMessageWaitingEvent"§

This event report is used to indicate whether the Message Waiting feature has been activated/deactivated.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAMessageWaitingEvent_t  messageWaiting;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

CalledDeviceID_t	deviceForMessage;

SubjectDeviceID_t	invokingDevice;

Boolean			messageWaitingOn;

} CSTAMessageWaitingEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_MESSAGE_WAITING which  identifies  this message as an CSTAMessageWaitingEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

deviceForMessage

Indicates the device where the message is waiting (i.e. address of device where the message waiting feature was activated). If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

invokingDevice

Specifies which device invoked the message waiting feature (i.e. address of the device who activated the message waiting feature). If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

messageWaitingOn

Specifies whether the Message Waiting feature is on (1)  or off (0).

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event can occur for both a device or a call association.

# Agent Status Event Reports (Unsolicited)

XE "Agent Feature Event Reports:See Feature Event (Agent)"§XE "Feature:Event:Agent"§This section covers event reports which pertain to the use of ACD agent features. The agent feature event reportsXE "Agent feature event reports"§XE "Feature:Event:Agent"§ indicate a change in the state of a specific agent. Each event defines the current state of the agent feature regardless of the state of the feature before the event. Typically, applications in the call center or message center environment use agent status event reports.

## CSTALoggedOnEventXE "CSTALoggedOnEvent"§

This event report informs the application that an agent has logged into a device (usually an ACD Split).

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTALoggedOnEvent_t  loggedOn,

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t	agentDevice;

AgentID_t		agentID;

AgentGroup_t		agentGroup;

AgentPassword_t	password;

} CSTALoggedOnEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_LOGGED_ON which  identifies  this message as an CSTALoggedOnEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

agentDevice

Specifies the device from which the agent is logged on to the system. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the agent identifier of the agent who logged 	into the system.

agentGroup

Specifies the group or ACD Split to which the agent is logging into.

password

This paramater specifies the agent's password used to log into the system.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

In most cases, when an agent logs into a device it usually means that the agent is ready to start receiving calls at the device. This may not be true for some implementations.

## CSTALoggedOffEventXE "CSTALoggedOffEvent"§

This event report indicates that an agent has logged out of the device/ACD Split for which the agent had previously logged in and was providing service.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTALoggedOffEvent_t  loggedOff;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t		agentDevice;

AgentID_t			agentID;

AgentGroup_t			agentGroup;

} CSTALoggedOffEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_LOGGED_OFF which  identifies  this message as an CSTALoggedOffEvent.

agentDevice

Specifies the device from which the agent is logged off the system. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the agent identifier of the agent who logged 	off the system.

agentGroup

Specifies the group or ACD Split from which the agent is logging out.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

## CSTANotReadyEventXE "CSTANotReadyEvent"§

This event report indicates that an agent is busy with tasks other than servicing a call at the device. In most cases this will imply that the agent is not ready to receive a call or that the agent is taking a break.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTANotReadyEvent_t  notReady;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t		agentDevice;

AgentID_t			agentID;

} CSTANotReadyEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_NOT_READY which  identifies  this message as an CSTANotReadyEvent.

agentDevice

Specifies the device from which the agent is logged on to the system. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the identifier of the agent who in not ready to 	receive calls.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

## CSTAReadyEventXE "CSTAReadyEvent"§

This event report indicates that an agent is ready to receive calls at the device. This event can occur even if the agent is busy on an active call at the device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See   Data Types and  CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAReadyEvent_t  ready;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t		agentDevice;

AgentID_t			agentID;

} CSTAReadyEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_READY which  identifies  this message as an CSTAReadyEvent.

agentDevice

Specifies the device which is ready to receive calls from the ACD. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the identifier of the agent who in ready to receive calls.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

## CSTAWorkNotReadyEventXE "CSTAWorkNotReadyEvent"§

This event report indicates that the agent is in after call work mode completing the tasks involved in servicing a call after the connection has been disconnected. This will implies that the agents is no longer on the call but is completing the servicing of the last call and the agent should not receive any additional calls.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct 
{
	ACSHandle_t	acsHandle;
	EventClass_t	eventClass;
	EventType_t	eventType;
} ACSEventHeader_t;

typedef struct 
{
	ACSEventHeader_t	eventHeader;
	union 
	{
		struct 
		{
		     CSTAMonitorCrossRefID_t	monitorCrossRefID;
			union
			{
			    CSTAWorkNotReadyEvent_t  workNotReady;
			} u;
		} cstaUnsolicited;
	} event;
} CSTAEvent_t;

typedef struct 
{
	SubjectDeviceID_t		agentDevice;
	AgentID_t			agentID;
} CSTAWorkNotReadyEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_WORK_NOT_READY which  identifies  this message as an CSTAWorkNotReadyEvent.

agentDevice

Specifies the device which has invoked the Work Not Ready mode. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the identifier of the agent who is in the Work Not Ready mode.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

In the case of this event the agent is still working on completing the after call work for the last call. The difference between this event and the CSTAWorkReadyEvent is that the agent has indicated that he/she is not ready to receive additional calls.

## CSTAWorkReadyEventXE "CSTAWorkReadyEvent"§

This event report indicates that the agent is in "after call work mode" completing the tasks involved in servicing a call after the connection has been disconnected. This implies that the agents is no longer on the call but is completing the servicing of the last call and the agent may receive any additional calls.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAWorkReadyEvent_t  workReady;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

SubjectDeviceID_t		agentDevice;

AgentID_t			agentID;

} CSTAWorkReadyEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_WORK_READY which  identifies  this message as an CSTAWorkReadyEvent.

agentDevice

Specifies the device which has invoked the Work Ready mode. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

agentID

This paramater specifies the identifier of the agent who is in the Work Ready mode.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

In the case of this event the agent is still working on completing the after call work for the last call. The difference between this event and the CSTAWorkNotReadyEvent is that the agent has indicated that he/she is ready to receive additional calls.

# Event Report Data Types (Unsolicited)XE "Event:Data Types (Unsolicited)"§

This section defines the data structures associated with the CSTA Event Reports defined in the Status Reporting Services section of this document.

## CSTAMonitorFilter_tXE "CSTAMonitorFilter_t"§

This structure is used to identify the event type filters requested or available on a monitored CSTA association.

typedef unsigned short  CSTACallFilter_t;#define                     CF_CALL_CLEARED 0x8000#define                     CF_CONFERENCED 0x4000#define                     CF_CONNECTION_CLEARED 0x2000#define                     CF_DELIVERED 0x1000#define                     CF_DIVERTED 0x0800#define                     CF_ESTABLISHED 0x0400#define                     CF_FAILED 0x0200#define                     CF_HELD 0x0100#define                     CF_NETWORK_REACHED 0x0080#define                     CF_ORIGINATED 0x0040#define                     CF_QUEUED 0x0020#define                     CF_RETRIEVED 0x0010#define                     CF_SERVICE_INITIATED 0x0008#define                     CF_TRANSFERRED 0x0004typedef unsigned char   CSTAFeatureFilter_t;#define                     FF_CALL_INFORMATION 0x80#define                     FF_DO_NOT_DISTURB 0x40#define                     FF_FORWARDING 0x20#define                     FF_MESSAGE_WAITING 0x10typedef unsigned char   CSTAAgentFilter_t;#define                     AF_LOGGED_ON 0x80#define                     AF_LOGGED_OFF 0x40#define                     AF_NOT_READY 0x20#define                     AF_READY 0x10#define                     AF_WORK_NOT_READY 0x08#define                     AF_WORK_READY 0x04typedef unsigned char   CSTAMaintenanceFilter_t;#define                     MF_BACK_IN_SERVICE 0x80#define                     MF_OUT_OF_SERVICE 0x40typedef struct CSTAMonitorFilter_t {    CSTACallFilter_t call;    CSTAFeatureFilter_t feature;    CSTAAgentFilter_t agent;    CSTAMaintenanceFilter_t maintenance;    Boolean         private;} CSTAMonitorFilter_t;CALL_FILTERS

These values indicate that a call event filter should be used for processing events. The provided filter may be different than the one requested.

FEATURE_FILTERS

These values indicate that a feature event filter should be used for processing events. The provided filter may be different than the one requested.

AGENT_FILTERS

These values indicate that a agent event filter should be used for processing events. The provided filter may be different than the one requested.

MAINTENANCE_FILTERS

These values indicate that a maintenance event filter should be used for processing events. The provided filter may be different than the one requested.

PRIVATE_FILTER

This value indicates that a private filter should be used for processing events. The provided filter may be different than the one requested.

## CSTAEventCause_tXE "CSTAEventCause_t"§

This structure contains an enumerated list of all the possible event causes which can occur with different events. The definitions of these event cause codes are also provided.

typedef enum CSTAEventCause_t {

EC_NONE = -1,

EC_ACTIVE_MONITOR = 1,

EC_ALTERNATE = 2,

EC_BUSY = 3,

EC_CALL_BACK = 4,

EC_CALL_CANCELLED = 5,

EC_CALL_FORWARD_ALWAYS = 6,

EC_CALL_FORWARD_BUSY = 7,

EC_CALL_FORWARD_NO_ANSWER = 8,

EC_CALL_FORWARD = 9,

EC_CALL_NOT_ANSWERED = 10,

EC_CALL_PICKUP = 11,

EC_CAMP_ON = 12,

EC_DEST_NOT_OBTAINABLE = 13,

EC_DO_NOT_DISTURB = 14,

EC_INCOMPATIBLE_DESTINATION = 15,

EC_INVALID_ACCOUNT_CODE = 16,

EC_KEY_CONFERENCE = 17,

EC_LOCKOUT = 18,

EC_MAINTENANCE = 19,

EC_NETWORK_CONGESTION = 20,

EC_NETWORK_NOT_OBTAINABLE = 21,

EC_NEW_CALL = 22,

EC_NO_AVAILABLE_AGENTS = 23,

EC_OVERRIDE = 24,

EC_PARK = 25,

EC_OVERFLOW = 26,

EC_RECALL = 27,

EC_REDIRECTED = 28,

EC_REORDER_TONE = 29,

EC_RESOURCES_NOT_AVAILABLE = 30,

EC_SILENT_MONITOR = 31,

EC_TRANSFER = 32,

EC_TRUNKS_BUSY = 33,

EC_VOICE_UNIT_INITIATOR = 34

} CSTAEventCause_t;

Certain cause codes will appear in events only if they make sense. The Table 6-1 gives cause code definitionsXE "Cause code definitions"§.  Table 6-2 illustrates which cause codes are possible for the each of the call events.

Table 6-3<bookmark cause_code>

Cause Code Definitionstc "Cause Code Definitions" \f t \l3§

| Cause Code | Definition |
| --- | --- |
| Active Monitor | an Active Monitor Feature has occurred. This feature typically allows intrusion by a supervisor into an agent call with the ability to speak and listen. The resultant call can be considered as a conference so this cause code may be supplied with the Conferenced Event Report. |
| Alternate | the call is in the process of being exchanged. This feature is typically found on single-line telephones, where the human interface puts one call on hold and retrieves a held call or answers a waiting call in an atomic action. |
| Busy | the call encountered a busy tone or device |
| Call Back | Call Back is a feature invoked (by a user or via CSTA) in an attempt to complete a call that has encountered a busy or no answer condition. As a result of invoking the feature, the failed call is cleared and the call can be considered as queued. The switch may subsequently automatically retry the call (normally when the called party next becomes free). Consequently, this cause code may appear in Event Reports related to the feature invocation (Call Cleared, Connection Cleared and Queued) or related to the subsequent, retried call (Service Initiated, Originated, Delivered, and Established). |
| Call Canceled | the user has terminated a call without going on-hook. |
| Call Forward | the call has been redirected via a Call Forwarding feature set for general, unknown, or multiple conditions. |
| Call Fd. - Immediate | the call has been redirected via a Call Forwarding feature set for all conditions. |
| Call Fd. - Busy | the call has been redirected via a Call Forwarding feature set for a busy endpoint. |
| Call Fd. - No Answer | the call has been redirected via a Call Forwarding feature set for an endpoint that does not answer. |
| Call Not Answered | the call was not answered because a timer has elapsed. |
| Call Pickup | the call has been redirected via a Call Pickup feature. |
| Camp On | a Camp On feature has been invoked or has matured. |
| Dest. Not Obtainable | the call could not obtain the destination. |
| Do Not Disturb | the call encountered a Do Not Disturb condition. |

Table 6-3   continued

Cause Code Definitions

| Cause Code | Definition |
| --- | --- |
| Incompatible Destination | the call encountered an incompatible destination. |
| Invalid Account Code | the call has an invalid account code. |
| Key Operation | indicates that the Event Report occurred at a bridged or twin device. |
| Lockout | the call encountered inter-digit time-out while dialing. |
| Maintenance | the call encountered a facility or endpoint in a maintenance condition. |
| Net Congestion | the call encountered a congested network. In some circumstances this cause code indicates that the user is listening to a "No Circuit" Special Information Tone (SIT) from a network that is accompanied by a statement similar to "All circuits are busy..." |
| Net Not Obtainable | the call could not reach a destination network. |
| Do Not Disturb | the call encountered a Do Not Disturb condition. |
| Incompatible Destination | the call encountered an incompatible destination. |
| Invalid Account Code | the call has an invalid account code. |
| Key Operation | indicates that the Event Report occurred at a bridged or twin device. |
| Lockout | the call encountered inter-digit time-out while dialing. |
| Maintenance | the call encountered a facility or endpoint in a maintenance condition. |
| Net Congestion | the call encountered a congested network. In some circumstances this cause code indicates that the user is listening to a "No Circuit" Special Information Tone (SIT) from a network that is accompanied by a statement similar to "All circuits are busy..." |

Table 6-3   continued

Cause Code Definitions

| Cause Code | Definition |
| --- | --- |
| Net Not Obtainable | the call could not reach a destination network. |
| Resources not Available | resources were not available |
| Silent Monitor | the event was caused by the invocation of a feature that allows a third party, such as an ACD agent supervisor, to join the call. The joining party can hear the entire conversation, but cannot be heard by either original party. The feature, sometimes called silent intrusion, may provide a tone to one or both parties to indicate that they are being monitored. This feature is not the same as a CSTA Monitor request. This cause shall not indicate that a CSTA Monitor has been initiated. |
| Transfer | a Transfer is in progress or has occurred |
| Trunks Busy | the call encountered Trunks Busy |
| Voice Unit Initiator | indicates that the event was the result of action by automated equipment (voice mail device, voice response unit, announcement) rather than the result of action by a human user. |

Table 6-4<bookmark event_cause>

CSTA Event Report - Cause RelationshipsXE "Event:Cause Relationships"§tc "CSTA Event Report:Cause Relationships" \f t \l3§

| Cause | Call Clr. | Conf | Con.Clr. | Dlv. | Div. | Est. | Fail | Held | Net.Rch. | Orig. | Q-ed | Retr. | Svc. Init. | Tran |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Active Monitor |  | y |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternate |  |  |  |  |  | y | y | y |  |  |  | y |  |  |
| Busy |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Call Back | y |  | y | y |  |  |  |  |  | y | y |  | y |  |
| Call Canceled | y |  | y |  |  |  | y |  |  |  |  |  | y |  |
| Call Forward |  |  |  | y | y |  | y | y | y |  | y |  |  |  |
| Call Fd. - Immediate |  |  |  | y | y |  | y |  | y |  | y |  |  |  |
| Call Fd. - Busy |  |  |  | y | y |  | y |  | y |  | y |  |  |  |
| Call Fd. - No Answer |  |  |  | y | y |  | y | y | y |  | y |  |  |  |
| Call Not Answered | y |  | y |  | y |  | y |  |  |  |  |  |  |  |
| Call Pickup |  |  |  |  | y | y |  |  |  |  |  |  |  |  |
| Camp On |  |  |  | y |  |  | y |  |  |  | y |  |  |  |
| Dest. not Obtainable |  |  | y |  |  |  | y |  |  |  | y |  |  |  |
| Do Not Disturb |  |  | y |  | y |  | y |  |  |  | y |  |  |  |
| Incpt. Destination | y |  | y |  | y |  | y |  |  |  |  |  |  |  |
| Invalid Account Code | y |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Key Operation | y | y | y | y | y | y | y | y | y | y | y | y | y | y |
| Lockout |  |  |  |  |  |  | y |  |  |  |  |  |  |  |

Table 6-4   continued

CSTA Event Report - Cause Relationships

| Cause | Call Clr. | Conf | Con.Clr. | Dlv. | Div. | Est. | Fail | Held | Net.Rch. | Orig. | Q-ed | Retr. | Svc. Init. | Tran |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance | y |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Net Congestion |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Net Not Obtainable |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| New Call |  | y |  | y |  | y |  |  |  | y |  |  |  | y |
| No Available Agents |  |  |  | y | y |  | y |  |  |  | y |  |  |  |
| Overflow | y |  | y | y | y |  | y |  | y |  | y |  |  |  |
| Override | y | y | y | y |  | y | y |  |  | y |  |  | y |  |
| Park |  |  | y |  |  |  |  |  |  |  | y |  |  |  |
| Recall |  | y |  | y | y | y | y | y |  |  | y | y |  | y |
| Redirected |  |  |  | y | y |  | y |  | y |  | y |  |  | y |
| Reorder Tone |  |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Resrcs. not Available | y |  | y |  |  |  | y |  | y |  | y |  |  |  |
| Silent Monitor |  | y |  |  |  |  |  |  |  | y |  |  |  |  |
| Transfer |  |  |  | y |  | y | y | y | y |  | y | y |  | y |
| Trunks Busy |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Voice Unit Initiator |  |  |  |  | y |  |  |  |  |  |  |  |  | y |

Telephone numbers associated primarily with one device often appear also on a second device.  One example is a secretary who's phone has mirrored or bridged lines of a boss's phone.

Telephone numbers associated primarily with one device often appear also on a second device.  One example is a secretary who's phone has mirrored or bridged lines of a boss's phone.