Hidden Chapter Field Code.  Do Not DeleteChapter	7	Snapshot ServicesXE "Snapshot Services"§

An application uses CSTA Snapshot Services to query the current state of a CSTA Call or a Device object.  Snapshot services query the switch to provide an application with information about the object.  The information is a "snapshot" since the state of the Call or Device object changes over time.

The Call Snapshot Services return a list of the Devices and Connections associated with a given Call, and the Connection States for each of those Devices.  As Figure 7-1 illustrates, the union of the Connection States for the Call defines the overall Call State.  Also refer to the definition of Call State in Chapter 3.

Figure 7-1 shows a Call that has four associated devices.  Recall from Chapter 3 that the relationship between a CSTA Call and a CSTA Device is a CSTA Connection (C1, C2, C3, and C4 are Connections).  Each Connection has an associated Connection State.  The Call Snapshot Services inform an application of each Device that is on a given Call and the associated Connection State for those Devices.  The Call State is the union of all the Connection States associated with the Call.  The application can use snapshot information to control Connections.  For example, if Figure 7-1 shows a four-party conference call, then an application can use the Call Snapshot Services together with the cstaClearConnection( ) service to disconnect any party from the conference.  To disconnect connection C4, an application uses the Call Snapshot Services to obtain a Connection Identifier (for C4) that it then passes to the cstaClearConnection( ) service.

Figure 7-1<bookmark callsnap>

Call Snapshot Servicetc "Call Snapshot Service" \f f \l3§

µ §

Device Snapshot Services return information about Calls that are associated with a given CSTA Device object.  The information includes a list of Calls associated with the given Device and the Connection State of each Call (at that Device).  Note the duality here:  Call Snapshot Services return information about Connections at all Devices associated with a given Call, while Device Snapshot Services return information about all Connections at a given Device.  Applications use the Device Snapshot Services when they need to know what is happening at a specific Device.  As Figure 7-2 shows, Device Snapshot Services do not provide information about the other parties on those Calls connected to the given Device.

An application can use Device Snapshot information to manipulate any Connection, (C1, C2, or C3 in Figure 7-2) at the given Device.

Figure 7-2<bookmark devsnap>

Device Snapshot Servicetc "Device:Snapshot Service" \f f \l3§

µ §

Before an application requests the Call or Device Snapshot Services, it must have previously obtained a Call or Device identifier (that it will use as a parameter to request those services).  The identifier specifies a Call or Device in the switching domain.  Depending on the implementation, Snapshot Services may not provide information about devices or connections outside of that switching domain (devices not attached to that switch)  .

# Call Snapshot Services

XE "Call Snapshot Services"§This section defines the Call Snapshot Services that query the switch for the status of calls within the switching domain.  Call Snapshot Services return information about all Devices and Connections associated with a specified CSTA Call object.

## cstaSnapshotCallReq( )XE "cstaSnapshotCallReq( )"§

The cstaSnapshotCallReq( ) service provides information about a Call object in the switching domain. The service will return the Devices associated with a given Call and the Connection State for each Device. The Call State is the union of the Connection States.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t   cstaSnapshotCallReq (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

ConnectionID_t		*snapshotObj,

PrivateData_t		*privateData),

Parameters

acsHandle

This is the handle to an active ACS Stream over which the request will be made.

invokeID

This is an application provided handle that the application uses to match a specific instance of a service request with its confirmation event.  The application supplies this parameter only when the Invoke ID mechanism is set for Application-generated IDs in acsOpenStream( ).  The ACS Library ignores this parameter when the Stream is set for Library-generated invoke IDs.

snapshotObj

This is a pointer to the Connection Identifier identifying the Call object for which Snapshot information is requested.

privateData

This is an optional pointer to CSTA private data.

Return Values

cstaSnapshotCallReq( ) returns the following values depending on whether the application is using library or application-generated invoke identifiers:

Library-generated Identifiers - if the function call completes successfully it will return a positive value, the invoke identifier. If the call fails it will return a negative error (<0). For library-generated identifiers the return will never be zero (0).

Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails it will return a negative error (<0). For application-generated identifiers the return is never positive (>0).

An application should always check the CSTASnapshotCallConfEvent message to insure that the Telephony Server and switch have acknowledged and processed the cstaSnapshotCallReq( ) request.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

The application provided a bad or unknown acsHandle.

ACSERR_STREAM_FAILED

A previously active ACS Stream has been abnormally aborted.

Comments

A call to cstaSnapshotCallReq( ) results in a confirmation event, CSTASnapshotCallConfEvent, that returns the information about the call.  cstaSnapshotCallReq( ) provides information about calls that make further monitoring meaningful. For example, when an application requests cstaMonitorStart( ), there may already be active calls at the Device being monitored.  The application can use Call Snapshot information to obtain information about those existing calls process additional events about them in a reasonable way.

cstaSnapshotCallReq( ) is passive and does not affect the state of any object in the switching domain.

### CSTASnapshotCallConfEventXE "CSTASnapshotCallConfEvent"§

The Call Snapshot confirmation event returns call related information in response to the cstaSnapshotCallReq( ) service. The call information includes the static Device Identifiers, the Connection Identifiers, and Connection States for every endpoint in the specified call.

Syntax

The following structure shows only the relevant portions of the unions for this message.  See ACS Data Types and  CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t		acsHandle;	EventClass_t	eventClass;	EventType_t		eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			InvokeID_t	invokeID;

union

{

CSTASnapshotCallConfEvent_t	snapshotCall;

} u;		} cstaConfirmation

} event;} CSTAEvent_t;

typedef struct CSTASnapshotCallConfEvent_t {	CSTASnapshotCallData_t snapshotData;} CSTASnapshotCallConfEvent_t;

typedef struct CSTASnapshotCallData_t {	int			count;	struct		CSTASnapshotCallResponseInfo_t *info;} CSTASnapshotCallData_t;

typedef struct CSTASnapshotCallResponseInfo_t {	SubjectDeviceID_t			deviceOnCall;	ConnectionID_t				callIdentifier;	LocalConnectionState_t		localConnectionState;} CSTASnapshotCallResponseInfoEvent_t;Parameters

acsHandle

This is the handle for the ACS Stream over which the confirmation arrived.  This is the same as the ACS Stream over which the request was made.

eventClass

This is a tag with the value CSTACONFIRMATION, which identifies this message as an CSTA confirmation event.

eventType

This is a tag with the value CSTA_SNAPSHOT_CALL_CONF, which identifies  this message as an CSTASnapshotCallConfEvent.

invokeID

This parameter specifies the service request instance for the cstaSnapshotCallReq( )  The application uses this parameter to correlate responses with requests.

snapshotData

Contains all the snapshot information for the Call for which the query was made.

count: A count of the number of  CSTASnapshotCallResponseInfo_t structures.  Each structure contains information about one device on the call.

info: A pointer to an array of CSTASnapshotCallResponseInfo_t structures, each of which contains the following fields:

deviceOnCall - A pointer to the Device Identifier of a device that is a party on the call for which the query was made.

callIdentifier -The Connection Identifier for the Connection  between the deviceOnCall and the call for which the query was made.

localConnectionState - The Connection State for the local Connection in the callIdentifier parameter.

privateData

If private data accompanies this event, then the private data would be stored in the location that the application specified as the privateData parameter in the acsGetEventBlock( ) or acsGetEventPoll( ) request. If the privateData pointer is set to NULL in these requests, then CSTASnapshotCallConfEvent does not deliver private data to the application.

Comments

The CSTASnapshotCallConfEvent returns a linked list since the number of devices on a call can be greater than one. Each member of the list identifies a Device on the call, the Connection between the Device and the Call, and the Connection State (see Figure 7-1).  An application should be aware that the number of members on the list is not fixed.  The pointer, *next, will be NULL for the last member (device) on the list.

# Device Snapshot ServiceXE "Device Snapshot Service"§

This section defines the Device Snapshot Services that query the switch for the status of Devices within the switching domain.  Device Snapshot Services return information about Calls (Connections) associated with a specified Device.

## cstaSnapshotDeviceReq( )XE "cstaSnapshotDeviceReq( )"§

The cstaSnapshotDeviceReq( ) service returns information about a Device object in the switching domain. The service returns a list of calls associated with the given Device and the Connection State of each of those calls at that Device.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t   cstaSnapshotDeviceReq (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

DeviceID_t			*snapshotObj,

PrivateData_t		*privateData);

Parameters

acsHandle

This is the handle to an active ACS Stream over which the request will be made.

invokeID

This is an application provided handle that the application uses to match a specific instance of a service request with its confirmation event.  The application supplies this parameter only when the Invoke ID mechanism is set for Application-generated IDs in acsOpenStream( ).  The ACS Library ignores this parameter when the Stream is set for Library-generated invoke IDs.

snapshotObj

This parameter is a pointer to the Device Identifier for the Device object for which Snapshot information is being requested.

privateData

This is an optional pointer to CSTA private data.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

Library-generated Identifiers - if the function call completes successfully it will return a positive value, the invoke identifier. If the call fails it will return a negative error (<0). For library-generated identifiers the return will never be zero (0).

Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails it will return a negative error (<0). For application-generated identifiers the return is never positive (>0).

The application should always check the CSTASnapshotDeviceConfEvent message to insure that the Telephony Server and the switch have acknowledged and processed the cstaSnapshotDeviceReq( ) request.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

A call to cstaSnapshotDeviceReq( ) results in a confirmation event, CSTASnapshotDeviceConfEvent, which returns information about the Device.  cstaSnapshotDeviceReq( ) provides information about Devices that permit an application to synchronize state with the switching domain.  For example, an application can call cstaSnapshotDeviceReq( ) to find out about the Calls present at a Device, then call cstaMonitorStart( ) to monitor the Device.  The information from the Device query permits the application to process the monitoring events in a proper context.

The cstaSnapshotDeviceReq( ) is passive and does not affect the state of any object within the switching domain.

## CSTASnapshotDeviceConfEventXE "CSTASnapshotDeviceConfEvent"§

The Device Snapshot confirmation event returns Device related information in response to the cstaSnapshotDeviceReq( ) service. The Device information includes a Connection Identifier for each Call at the Device and the Connection State for each Call at the Device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and  CSTA Data Types in section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t		acsHandle;	EventClass_t	eventClass;	EventType_t		eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			InvokeID_t	invokeID;

union

{

CSTASnapshotDeviceConfEvent_t	snapshotDevice;

} u;		} cstaConfirmation;

} event;} CSTAEvent_t;

typedef struct CSTASnapshotDeviceConfEvent_t {	CSTASnapshotDeviceData_t snapshotData;} CSTASnapshotDeviceConfEvent_t;typedef struct CSTASnapshotDeviceData_t {	int		count;	struct	CSTASnapshotDeviceResponseInfo_t *info;} CSTASnapshotDeviceData_t;

typedef struct CSTASnapshotDeviceResponseInfo_t {	ConnectionID_t		callIdentifier;	CSTACallState_t	callState;} CSTASnapshotDeviceResponseInfo_t;typedef struct CSTACallState_t {	int							count;	LocalConnectionState_t		*state;} CSTACallState_t;typedef enum CSTASimpleCallState_t {	CALL_NULL = 0,	CALL_PENDING = 1,	CALL_ORIGINATED = 3,	CALL_DELIVERED = 35,	CALL_DELIVERED_HELD = 36,	CALL_RECEIVED = 50,	CALL_ESTABLISHED = 51,	CALL_ESTABLISHED_HELD = 52,	CALL_RECEIVED_ON_HOLD = 66,	CALL_ESTABLISHED_ON_HOLD = 67,	CALL_QUEUED = 83,	CALL_QUEUED_HELD = 84,	CALL_FAILED = 99,	CALL_FAILED_HELD = 100} CSTASimpleCallState_t;

/* Used to take a CSTACallState_t which contains only two

* LocalConnectionState_t and match them to the set of

*/	#define SIMPLE_CALL_STATE(ccs) (ccs.stat[0]+(ccs.state[1] << 4))

typedef struct CSTACallState_t {

int							count;

LocalConnectionState_t		*state;

} CSTACallState_t;

Parameters

acsHandle

This is the handle for the ACS Stream over which the confirmation arrived.  This is the same as the ACS Stream over which the request was made.

eventClass

This is a tag with the value CSTACONFIRMATION, which identifies this message as an CSTA confirmation event.

eventType

This is a tag with the value CSTA_SNAPSHOT_DEVICE_CONF, which identifies this message as an CSTASnapshotDeviceConfEvent.

invokeID

This parameter specifies the service request instance for the cstaSnapshotDeviceReq( )  The application uses this parameter to correlate responses with requests.

snapshotData

Contains all the snapshot information for the Device for which the query was made.

count:  A count of the number of  CSTASnapshotDeviceResponseInfo_t structures. Each contains information about one Device on the Call.

info:  A pointer to an array of  CSTASnapshotDeviceResponseInfo_t structures, each of which contains the following fields:

callIdentifier - A pointer to a Connection Identifier for each call at the device. For some implementations, this parameter points to the device's dynamic device identifier for the call object.

callState - The CSTA Call State.  The Call State is returned as a list of local Call States. If there are only two Call States, then count is two.  The application can use the macro SIMPLE_CALL_STATE( ) to determine if a local Call State is one of the CSTA Simple Call States (defined in Chapter 3 and enumerated in the CSTASimpleCallState_t structure).

privateData

If private data accompanies this event, then the private data would be stored in the location that the application specified as the privateData parameter in the acsGetEventBlock( ) or acsGetEventPoll( ) request. If the privateData pointer is set to NULL in these requests, then CSTASnapshotDeviceConfEvent does not deliver private data to the application.

Comments

The CSTASnapshotDeviceConfEvent returns a linked list since the number of calls on a device can be greater than one.  Each member of the list identifies a call at the device, and the local call state of the Connection for that call at the device (see Figure 7-2).  An application should be aware that the number of members on the list is not fixed.  The pointer, *next, will be NULL for the last member (call) on the list.