| Cause Code | Indicates that (definition): |
| --- | --- |
| Active Monitor | an Active Monitor Feature has occurred. This feature typically allows intrusion by a supervisor into an agent call with the ability to speak and listen. The resultant call can be considered as a conference so this cause code may be supplied with the Conferenced Event Report. |
| Alternate | the call is in the process of being exchanged. This feature is typically found on single-line telephones, where the human interface puts one call on hold and retrieves a held call or answers a waiting call in an atomic action. |
| Busy | the call encountered a busy tone or device |
| Call Back | Call Back is a feature invoked (by a user or via CSTA) in an attempt to complete a call that has encountered a busy or no answer condition. As a result of invoking the feature, the failed call is cleared and the call can be considered as queued. The switch may subsequently automatically retry the call (normally when the called party next becomes free). Consequently, this cause code may appear in Event Reports related to the feature invocation (Call Cleared, Connection Cleared and Queued) or related to the subsequent, retried call (Service Initiated, Originated, Delivered, and Established). |
| Call Cancelled | the user has terminated a call without going on-hook. |
| Call Forward | the call has been redirected via a Call Forwarding feature set for general, unknown, or multiple conditions. |
| Call Fd. - Immediate | the call has been redirected via a Call Forwarding feature set for all conditions. |
| Call Fd. - Busy | the call has been redirected via a Call Forwarding feature set for a busy endpoint. |
| Call Fd. - No Answer | the call has been redirected via a Call Forwarding feature set for an endpoint that does not answer. |
| Call Not Answered | the call was not answered because a timer has elapsed. |
| Call Pickup | the call has been redirected via a Call Pickup feature. |
| Camp On | a Camp On feature has been invoked or has matured. |
| Dest. Not Obtainable | the call could not obtain the destination. |
| Do Not Disturb | the call encountered a Do Not Disturb condition. |
| Incompatible Destination | the call encountered an incompatible destination. |
| Invalid Account Code | the call has an invalid account code. |
| Key Operation | indicates that the Event Report occurred at a bridged or twin device. |
| Lockout | the call encountered inter-digit timeout while dialing. |
| Maintenance | the call encountered a facility or endpoint in a maintenance condition. |
| Net Congestion | the call encountered a congested network. In some circumstances this cause code indicates that the user is listening to a "No Circuit" Special Information Tone (SIT) from a network that is accompanied by a statement similar to "All circuits are busy..." |
| Net Not Obtainable | the call could not reach a destination network. |
| New Call | the call has not yet been redirected. |
| No Available Agents | the call could not access any agent. |
| Overflow | the call overflowed a queue, group, or target. |
| Override | the call resulted because of an Override feature. |
| Park | indicate that the Event Report is associated with an action to place a call to or retrieve a call from a parked position. Placing a call in a park position releases the call from the parking device, but retains the call in the Switching Function so that it can be connected to another (or the same) device by invoking the un-parking feature there. |
| Recall | the call is alerting a device due to a time-out built into a feature that failed to complete or that anticipated further action from the user. |
| Redirected | the call has been redirected |
| Reorder Tone | the call encountered reorder - a tone provided by a network to indicate that the request (call, feature, or supplementary service) was not recognizable. This condition usually results when a user dials a number that is not valid or attempts to obtain a service that is not enabled for that user or device. In some circumstances this cause code indicates that the user is listening to a "Reorder" Special Information Tone (SIT) from a network that is accompanied by a statement similar to "The call did not go through as dialed..." |
| Resources not Available | resources were not available |
| Silent Monitor | the event was caused by the invocation of a feature that allows a third party, such as an ACD agent supervisor, to join the call. The joining party can hear the entire conversation, but cannot be heard by either original party. The feature, sometimes called silent intrusion, may provide a tone to one or both parties to indicate that they are being monitored. This feature is not the same as a CSTA Monitor request. This cause shall not indicate that a CSTA Monitor has been initiated. |
| Transfer | a Transfer is in progress or has occurred |
| Trunks Busy | the call encountered Trunks Busy |
| Voice Unit Initiator | indicates that the event was the result of action by automated equipment (voice mail device, voice response unit, announcement) rather than the result of action by a human user. |

The following table illustrates which cause codes values make sense for a specific call status event. The "y" indicates that the cause code is likely to appear in the specific event.

Table 6-2  CSTA Event Report - Cause Relationships

| Cause | Call Clr. | Conf | Con.Clr. | Dlv. | Div. | Est. | Fail | Held | Net. Rch. | Orig. | Q-ed | Retr. | Svc. Init. | Tran |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Active Monitor |  | y |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternate |  |  |  |  |  | y | y | y |  |  |  | y |  |  |
| Busy |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Call Back | y |  | y | y |  |  |  |  |  | y | y |  | y |  |
| Call Cancelled | y |  | y |  |  |  | y |  |  |  |  |  | y |  |
| Call Forward |  |  |  | y | y |  | y | y | y |  | y |  |  |  |
| Call Fd. - Immediate |  |  |  | y | y |  | y |  | y |  | y |  |  |  |
| Call Fd. - Busy |  |  |  | y | y |  | y |  | y |  | y |  |  |  |
| Call Fd. - No Answer |  |  |  | y | y |  | y | y | y |  | y |  |  |  |
| Call Not Answered | y |  | y |  | y |  | y |  |  |  |  |  |  |  |
| Call Pickup |  |  |  |  | y | y |  |  |  |  |  |  |  |  |
| Camp On |  |  |  | y |  |  | y |  |  |  | y |  |  |  |
| Dest. not Obtainable |  |  | y |  |  |  | y |  |  |  | y |  |  |  |
| Do Not Disturb |  |  | y |  | y |  | y |  |  |  | y |  |  |  |
| Incpt. Destination | y |  | y |  | y |  | y |  |  |  |  |  |  |  |
| Invalid Account Code | y |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Key Operation | y | y | y | y | y | y | y | y | y | y | y | y | y | y |
| Lockout |  |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Maintenance | y |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Net Congestion |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Net Not Obtainable |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| New Call |  | y |  | y |  | y |  |  |  | y |  |  |  | y |
| No Available Agents |  |  |  | y | y |  | y |  |  |  | y |  |  |  |
| Overflow | y |  | y | y | y |  | y |  | y |  | y |  |  |  |
| Override | y | y | y | y |  | y | y |  |  | y |  |  | y |  |
| Park |  |  | y |  |  |  |  |  |  |  | y |  |  |  |
| Recall |  | y |  | y | y | y | y | y |  |  | y | y |  | y |
| Redirected |  |  |  | y | y |  | y |  | y |  | y |  |  | y |
| Reorder Tone |  |  |  |  |  |  | y |  |  |  |  |  |  |  |
| Resrcs. not Available | y |  | y |  |  |  | y |  | y |  | y |  |  |  |
| Silent Monitor |  | y |  |  |  |  |  |  |  | y |  |  |  |  |
| Transfer |  |  |  | y |  | y | y | y | y |  | y | y |  | y |
| Trunks Busy |  |  |  |  |  |  | y |  |  |  | y |  |  |  |
| Voice Unit Initiator |  |  |  |  | y |  |  |  |  |  |  |  |  | y |

XE " CSTA Event Report - Cause Relationships"§

Telephone numbers associated primarily with one device often appear also on a second device.  One example is a secretary who's phone has mirrored or bridged lines of a boss's phone.