The Genlock utility features an ARexx interface to allow external Amiga

programs to control the video output of the Toaster.  The Genlock's ARexx

implementation uses both the command host AND the function host interface

paradigm.  In a command host interface, commands are sent to the program as

character strings after the program's ARexx port has been selected to

receive commands with the 'ADDRESS' directive.  The name of the Genlock's

port is "GENLOCK_REXX_PORT", so any commands meant for the genlock utility

should be preceded by the line "address GENLOCK_REXX_PORT".  The function

host interface requires that the genlock utility's port be added to a list

of function libraries with the 'ADDLIB()' function.  Once the function host

has been added, ARexx will recognize function calls made to the genlock

program. Again the name to use is "GENLOCK_REXX_PORT" and the proper systax

for adding the function host is "ADDLIB(GENLOCK_REXX_PORT,0)".  Since most

Amiga software uses the command host interface paradigm, we recommend that

you use this mode of controlling the genlock.  The following is a list of

supported Genlock commands, their function host equivalent, and a brief

description of their purpose.  There are also several example ARexx

programs in the ARexx_Examples directory, as well as an AmigaVision sample

which will let you use the genlock utility with your AmigaVision multimedia

presentations.

M001    M001()    Set Main (Program) Output to source 1

M002    M002()    Set Program Output to source 2

M003    M003()    Set Program Output to source 3

M004    M004()    Set Program Output to source 4

MDV1    MDV1()    Set Program Output to Still Store DV1

MDV2    MDV2()    Set Program Output to Still Store DV2

MGEN    MGEN()    Set Program Output to overlay Amiga RGB graphics

MRGB    MGEN()    Set Program Output to display Amiga RGB graphics

P001    P001()    Set Preview Output to source 1

P002    P002()    Set Preview Output to source 2

P003    P003()    Set Preview Output to source 3

P004    P004()    Set Preview Output to source 4

PDV1    PDV1()    Set Preview Output to Still Store DV1

PDV2    PDV2()    Set Preview Output to Still Store DV2

PGEN    PGEN()    Set Preview Output to overlay Amiga RGB graphics

PRGB    PGEN()    Set Preview Output to display Amiga RGB graphics

TAKE    TAKE()    Do a cut from Program to Preview out (Take)

AUTO    AUTO()    Do an automatic transition

SLOW    SLOW()    Set AUTO transition speed to Slow

MEDM    MEDM()    Set AUTO transition speed to Medium

FAST    FAST()    Set AUTO transition speed to Fast

TBAR n  TBAR(n)   Move T-Bar by n (relative, i.e. TBAR -50 moves up by 50)

RBUP    RBUP()    Release right mouse button (refresh preview) after

transition

QUIT    QUIT()    Exit genlock utility