PlayCDDA - Play audio CDs on your Amiga's audio.device

------------------------------------------------------

Version 1.1    04-Jan-94    (C) 1993 by Frank Munkert

(ln_fmu@pki-nbg.philips.de)

INTRODUCTION

PlayCDDA is a program for owners of Toshiba 3401 and Apple CD-300

CDROM drives. These drives are capable of transferring CD-DA

(digital audio) data over the SCSI bus. PlayCDDA reads this data and

reproduces the corresponding sounds on the Amiga's audio.device.

DISCLAIMER

This software is provided as-is, without warranty of any kind,

either expressed or implied. In no event will the author be liable

for direct, indirect, incidental or consequential damages or data

loss resulting from the use or application of this software. The

entire risk as to the results and performance of this software is

assumed by the user.

REQUIREMENTS

AmiCDROM should work with any Amiga model which has a "SCSI-direct"

compatible SCSI bus adapter. A Toshiba 3401 or Apple CD-300 CDROM

drive must be connected to the SCSI bus.

Your SCSI bus setup must have a bandwidth of at least 173 KB/sec in

order to transfer 44100 4-bit-samples per second. The higher the

bandwidth, the better. Also, while running PlayCDDA there shouldn't

be much other activity on the SCSI bus (e.g. hard disk accesses).

INSTALLATION

Copy the file 'PlayCDDA' to your favourite location for executable

programs.

USING PLAYCDDA

Invocation from CLI

You have to supply the name of your SCSI device and the

SCSI unit number of your CDROM drive as arguments to PlayCDDA,

e.g.

PlayCDDA scsi.device 1

invokes PlayCDDA with SCSI device name "scsi.device" and

unit number 1.

The complete command line template for PlayCDDA is

DEVICE/A,UNIT/A/N,CHIP/S,FAST/S,DMA/S,ANY/S,LOW/S,MEDIUM/S,HIGH/S

DEVICE:  scsi device name

UNIT:    scsi unit number

CHIP:    use chip memory for CDROM buffers (this is the default)

FAST:    use fast memory for CDROM buffers

DMA:     use 24 bit DMA memory for CDROM buffers

ANY:     use any available memory for CDROM buffers

The options CHIP, FAST, DMA and ANY are mutually exclusive.

LOW:     low volume

MEDIUM:  medium volume

HIGH:    high volume

The options LOW, MEDIUM and HIGH are mutually exclusive.

Invocation from WorkBench

Edit the tooltype entries "DEVICE" and "UNIT" of the PlayCDDA icon

such that they reflect your system setup.

Double-click on the PlayCDDA icon.

The optional tooltype "MEMORY" may have the values "CHIP", "FAST",

"DMA" or "ANY" (see above).

The optional tooltype "VOLUME" may have the values "LOW", "MEDIUM"

or "HIGH" (see above).

PlayCDDA communicates with the user over a simple graphical

interface. Basically, you have to click at the button for the

track you want to hear, and PlayCDDA starts playing this track

on your Amiga's loudspeaker.

The rest of the section explains the individual buttons of the

user interface.

"Start"

Click at this button to play the first track.

"Stop"

Stop playing.

"1" ... "20"

Click at on of these buttons to play the corresponding track.

"Next"

Plays the next track.

"Prev"

Plays the previous track.

"Volume"

Adjust the volume of the sound reproduction.

"Sampling rate"

The number of bytes per second that are sent to the audio.device.

The higher the sampling rate the better the sound quality.

"Buffers"

Number of buffers. The higher the number of buffers, the lesser is

the chance that the sound may be interrupted by other tasks with

higher priority.

You should adjust "Buffers" and "Sampling rate" to fit your system's

need. On an Amiga 3000T with built-in SCSI device, the default settings

should work. With an Apple CD-300 it might be necessary to set the

number of buffers to 64 for increased performance.

HISTORY

V1.1: 04-Jan-94

Added support for the Apple CD-300, implemented by Heiko Rath

(hr@brewhr.swb.de)

New options: CHIP, FAST, DMA, ANY, LOW, MEDIUM, HIGH.

Minor changes. (ln_fmu)

V1.0: 21-Nov-93

Original release with support for Toshiba 3401 CDROM drives.

CONTACTING THE AUTHOR

Please send bug reports and suggestions to ln_fmu@pki-nbg.philips.de.

If you want to report an error, please describe your system

configuration (Amiga model, SCSI device, PlayCDDA version number).