17 June 81

Documentation for

C User's Group

Transmittal Disk

Robert Pasky

36 Wiswall Rd.

Newton Centre, MA 02159

(617) 449-4600 x240	work (7-3 e.d.t.)

(617) 964-3641		home

Files contained on disk:

1.	symbug.c	- converts "clink" .SYM output file to "vbug" format.

2.	symbug.com	- object file for same.

3.	prolog.c	- EPROM programmer utility.

4.	prolog.com	- object file for same.

5.	chargen.c	- character generator utility

6.	chargen.com	- object file for same.

7.	charset		- character descriptions for chargen program.

8.	bin2hex.c	- binary to hex-ascii format converter.

9.	bin2hex.com	- object file for same.

10.	printf.c	- _spr function modified for leading 0's.

11.	getc.c		- getc function modified for proper eof.

12.	disk.doc	- this file.

1. & 2. symbug

This program was intended to convert the .SYM file produced by

CLINK (and MAC) to a format compatible with VBUG, a symbolic debugger

which I had previously written.

Although it may not be of immediate use to others as written, it

is a simple example of a file sorting routine.

3. & 4. prolog

A modification of Leor Zolman's earlier TELNET program, this

transmits hex-ascii files from disk to a PRO-LOG(tm) EPROM programmer

and verifies the contents of an EPROM against a hex-ascii file.

This program should be easily modified for other PROM programmers,

although I don't have any specific information on any of them.

You should note that I am using memory-mapped i/o for the serial

interface -- remove #define MMREFIO ... for inp/outp instead of peek/poke.

Secondly, the masks and default values for PROM addresses assumes

a 16k bit (2k byte) EPROM (2516, 2716 types). If you normally use other

sizes, you will want to change the masks and defaults. If you program PROMS

of 256 bytes or less, the program must be changed to send 2 digits

instead of 3 for the starting and ending addresses to the PRO-LOG.

You might add a "menu" allowing the user to select various size PROMs

and change the masks and defaults accordingly.

One thing this program doesn't do that perhaps it should is

verifying the sumcheck bytes in the hex-ascii file. I'm assuming that

if the disk file can be read sucessfully the sumcheck is probably correct.

5. & 6. chargen

This program was written when I purchased an Imsai VIO video

interface board. I decided I wanted to modify the character set which

was contained in three 2708 PROMs in an awkward to edit format.

The screen displays a large box within which a 7x10 matrix of

squares can be individually turned on or off. In this way a large-scale

version of the character can be constructed, edited and then stored.

When the entire character set is completed, the data are

converted to the format required by the VIO logic and stored as a disk

file. The disk file is a binary image of the three EPROMS used for

upper case (0-0x7f), lower case (0x80-0xff) and descenders, respectively.

If you will be using the "prolog" program, above, or something

similar, you will need a hex-ascii version of this file. And so was

born the "bin2hex" program.

7. charset

This file is the output of the "chargen" program. It can be

used as a basis for creating your own character set.

Use the 'g' function of "chargen" to get this file into memory,

then the 'o' function to unformat it for editing. Finally, don't forget

to re-format the data using the 'p' function before writing it back to

disk with 'f'. If you do forget and save the unformatted file and then

quit the program, most of it can be recovered. Get the file but don't

unformat it (no 'o'), the end of the data (i.e., characters somewhere

above 0x80) may be missing.

8. & 9. bin2hex

This is a simple program to convert a binary file (e.g. .COM files)

to a hex-ascii file.

It takes as input any file and produces a .HEX type file of the

same name. The user may specify any starting address, and all data are

assumed to be contiguous. The program produces blocks of 32 bytes of data

each, with a proper sumcheck byte at the end of the block.  The final

block contains a zero count and zero address as required by most hex

loaders, including the LOAD.COM program of CP/M.

Since the program cannot assume that a 0x1a byte means an

end-of-file, it may produce some extraneous data at the end of the

hex file. However, if you have set NSECTS to a number greater than 1

(as I have), the standard "getc" function may produce a lot more

garbage than necessary. The modification to "getc" supplied on this disk

attempts to minimize the problem. An alternative, of course, is to leave

NSECTS at 1.

10. printf

The _spr() function modified to optionally print leading zeros.

To specify leading 0's, use a leading 0 in the field length. See K&R,

section 7.3.

For example,

num = 123;

printf ("\nleading zeros %05d", num);

printf ("\nno zeros %4d", num);

printf ("\nleft justified %-04d", num);

printf ("\nleft justified %-4d", num);

will print:

leading zeros 00123

no zeros   123

left justified 00123

left justified 123

This is often used when printing hex or octal addresses, for

instance, to print a "split octal notation" address, use:

address = 512+10;

printf ("%03o.%03o",address);

to get:

002.012

11. getc

The getc() function modified to calculate "nleft" from the

actual number of sectors read. This change is useful only if "NSECTS"

is greater than 1, and you wish to read binary files as, for example,

the "bin2hex" program does.

The problem is that if you have set NSECTS to 8, for example,

and the file has 9 sectors, the "getc" function (if you call it repeatedly

without checking for the eof character, 0x1a) will hand you 16 sectors

worth of data, the last 7 of which are leftovers from the previous 8

sectors.

This modification will minimize the problem by returning the

ERROR flag after the last (9th in this example) sector has been read.

###########################################################################################################################