README.DOC for CC09 MSDOS C COMPILER

Brian Brown

Senior Lecturer, Software Engineering

Central Institute of Technology

School of Electronic Engineering

Private Bag, Trentham

Upper-Hutt, Wellingon

New Zealand

15th November 1989

The Small C Compiler for 6809 running on FLEX (CUG 221) has been ported to

MSDOS, and changed to allow creation of embedded target software. It supports

the ASxxx group of assemblers (CUG 292) available from the C users Group, as

well as the Motorola AS9 assembler.

A host of routines is supplied. These work with particular boards used by our

students. The system we have is,

6809 Processor card, 32k StaticRAM 0000 to 7FFF

Addresses 9000-93ff are port mapped to 000 to 3ff for PC

type boards

Addresses 8000 to 8fff are mapped to B0000 for PC

monochrome cards

On board ACIA, MC6850 at A000 (ControlReg) A001 (DataReg)

OnBoard EPROM or StaticRAM C000 to FFFF

The processor card drives an IBM-PC backplane with four expansion sockets,

thus can talk directly to standard PC cards. The routines for this board are

ACIA.H, MEMORY.H, STRINGS.H

Standard PC Serial Card (SERIAL.H)

Standard Hercs/Monochrome card (HERCS.H, PRINTER.H)

32bit Digital I/O card, plus 8 channel A/D (0808 chip). The mapping arrangment

of this board is,

Port 220h  A/D Channel register

Port 221h  A/D Data register

222h  A/D End Of Conversion signal

223h  Simple latched I/O

224h  8255 PPI Port A

225h  8255 PPI Port B

226h  8255 PPI Port C

227h  Intel 8255 PPI control register

In conjunction with this board, a panel comprising,

1 x 7 segment display

1 x 8 LEDS

1 x 8 Digital Switches

1 x 16 hexidecimal keypad

is used to allow students to write software routines. (DIOBOARD.H, MCRDRV.H)

A magnetic card swipe reader attaches to port C of the DIOBoard.

The DIOBoards address is configurable via DIP, and the A/D convertor can be

- strapped for auto restart, EOC signal restarts ADC

- software polled via EOC bit status

- Interrupt driven, by staps which connect EOC to IRQ2 to IRQ7

The POD SOFTWARE is a simple board MC6821 which has a 40PIN DIP which plugs across the 6809 target processor. By writing to the MC6821 (which connects to DIOBoard) and setting certain pins, it is possible to alter the processor state (ie, RESET, HALT) and perform READ/WRITE cycles (by emulating bus cycles asserting pins mapped to processors pins in correct sequence).

All the available items are available from us at reasonable cost, eg, US funds

6809 processor card, assembled and tested (DS PCB)     $ 80

DIO Card  32bit I/O, 8 channel A/D etc                 $ 80

DIO Panels, switches, leds, keymatrix, sevenseg        $ 80

POD System with POD Software                           $100

We are also currently working on MC68000 and i8051 processor boards.

IF YOU RE-COMPILE THE SOURCE, USE A MEDIUM MEMORY MODEL!! AND TURN OFF

WARNINGS