FUNCTIONAL TESTING OF RAM--IMPLEMENTATION OF THE ATS AND

THE ATS+ ALGORITHMS

by

Mohammad Khurrum

and

Dean Lance Smith

Dept. of Electrical Engineering

Memphis State University

Memphis, TN 38152

(901) 678-3253

(901) 678-2175

This program is an implementation of the ATS (Algorithmic Test

Sequence) algorithm developed by Knaizuk and Hartman and the

ATS+ algorithm developed by Nair. The program tests RAM for any

single or multiple stuck-at-0 or stuck-at-1 faults.  The inputs

to the program are the starting and ending addresses of any part

of the Random Access Memory.  The output is the address of the

location where the fault occurrs and the type of the fault which

occurrs (stuck-at-0 or stuck-at-1).  A message is displayed if

no stuck-at fault is detected.

Two functions, ats() and atst(), do the actual testing.  The

ats() function has eight steps.  These steps write patterns and

then read them back to check if they were written properly or if

there were any faults.  The atst() function has thirteen steps

which do the testing and indicate a fault if there is one.

The variables g0, g1, g2, end and start are huge pointers.  The

variables seg1, seg2, off1 and off2 are integers.  The RAM is

divided into three sections represented by g0, g1 and g2.  The

function hex() reads the starting and ending addresses (segment,

offset) from the keyboard in hexadecimal.