7    SERIAL DEVICE

7.1  INTRODUCTION

All Amiga models have a serial port to which you can connect

external devices like a modem, scanner or a printer with a

serial interface. Data can be sent in both directions, in

several different formats, like seven or eight bits, with

or without error checking etc.

On the Amiga the serial port consists of a 25 pin connector,

and can communicate with baud rate up to 32250. It is the

custom chip called "Paula" which contains a Universal

Asynchronous Receiver/Transmitter (UART) that takes care of

all serial data transmissions. The chip itself can manage

up to one million bits per second, but at that speed the data

buffer would be filled before the system had time to react,

and normal cables can not manage to transmit more than

100,000 - 200,000 bits per second.

Baud rates between 1200 and 9600 are in most cases more than

sufficient, and at that speed the computer will still be able

to multitask properly.

7.2  THE SERIAL PORT

The serial port sends and receives data in streams of bits.

In theory only one physical wire would therefore be required,

but there have been several extra cables added to send and

receive special information as well as to supply external

devices with power (+12V) and so on.

Since all data is sent bit by bit, and not as with the parallel

port which sends one byte (8 bits) each time, it is rather

slow. (Well at least not as fast as the parallel port, which

is fully documented in the next chapter.) However, since

some applications only works with one line, for example

the modems which are connected to the telephone system,

serial communication is easier to handle. Serial cables are

also much cheaper.

Although only one line is required there is a whole 25-pin

connector on the Amiga. Normal applications like a modem, or

a scanner only needs some of the lines, but since there are

so many possible lines to use, a lot of special equipment can

be connected to the serial port. (See illustration "RS-232".)

7.2.1  BYTE TO BITS AND VICE VERSA

Since the serial device only sends a stream of bits, a special

chip has to convert the data (bytes) to bits (8). The character

"A" would for example be transformed to 01000001 and "B" to

01000010 and so on. Of course the chip must also be able to

do the opposite, to convert an incoming stream of bits to

bytes.

--------

A (64) <- |      | <- 01000001  (Receiving data)

| UART |

D (68) -> |      | -> 01000100  (Transmitting data)

--------

The special chip that takes care of this is the "Universal

Asynchronous Receiver/Transmitter" usually referred to as

"UART". (See illustration "UART") Luckily we do not need to

bother too much about this. However, it is good to know what

is actually happening.

7.2.2  PIN ASSIGNMENT

The Amiga's serial port is a 25-pin D-female-type connector.

Below is an almost complete list of the pin assignment,

together with a short description. (See illustration "RS-232".)

Pin  Amiga   RS232  HAYES  Direction  Type      Description

-------------------------------------------------------------------

1  SHIELD  GND    GND    -          Standard  Ground

2  TXD     TXD    TXD    Out        Standard  Transmit data

3  RXD     RXD    RXD    In         Standard  Receive data

4  RTS     RTS x  -      Out        Standard  Request to send

5  CTS     CTS x  CTS    In         Standard  Clear to send

6  DSR     DSR x  DSR    In         Standard  Data set ready

7  GND     GND    GND    -          Standard  Signal ground

8  DCD     DCD    DCD    In         Standard  Carrier detect

9  +12V    -      -      -          Amiga     +12 Volt

10  -12V    -      -      -          Amiga     -12 Volt

11  AUDO    -      -      Out        Amiga     Audio output

12  -       S.DS   SI     +          -         Speed indicate

13          S.CTS  -      +          -         RS232 code

14  -       S.TXD  -      +          -         RS232 code

15  -       TXC    -      +          -         RS232 code

16  -       S.RXD  -      +          -         RS232 code

17  -       RXC    -      +          -         RS232 code

18  AUDI    -      -      In         Amiga     Audio in (not used)

19  -       S.RTS  -      +          -         RS

20  DTR     DTR x  DTR    Out        Standard  Data terminal ready

21  -       SQD    -      -          *         (A1000 +5V)

22  RI      RI     RI     In         Standard  Ring indicator

23  -       SS     -      -          *         (A1000 +12V)

24  -       TXC1   -      -          -         3.58 MHz clock

25  -       -      -      -          Amiga     Reset (A1000)

(x) Used only if you have set the "seven-wire flag". Se below

for more information.

WARNING! Pin 21 and 23 are connected to the power supply on

the old Amiga 1000s (+5V and +12V). On all other models the

power is sent through pin 9 and 10. Be careful with this if

you intend to make serial cables! We do not want to burn the

user's external devices, do we?

7.2.3  THE DATA STREAM

Since all data is sent and received in a stream of bits, the

UART chip needs to know when each new byte (set of 8 bits) is

coming in. To make this possible, following rules have been

stated: (See illustration "Serial Bits".)

1. When no data is sent, a mark bit (1) is sent over and over.

2. Just before a byte is sent, a start bit (0) is transmitted.

The receiver will now know that seven or eight bits (depends

on what type of device it is) of data will be received.

3. An optional "parity" bit will come directly after the

data bits. The parity bit is used for error checking. The

receiver may use "even", "odd" or no parity at all. If

the receiver is using even parity, the sender should set

the parity bit to 1 if the remainder of all data bits

divided by two is 0, else the parity bit should be set to

0. If the receiver is using odd parity, the sender should

set the parity bit to 0 if the remainder of all data bits

divided by two is 0, else the parity bit should be set to

1. If the receiver does not use parity checking, either

set the parity bit to 0 or do not send any parity bit at

all.

4. Finally we need to send one (or two) stop bit(s) to tell

the receiver that the last data bit has been sent.

...1 0 0 0 1 0 0 0 1 0 1 0 0 0 1... -> -> ->

..._       _       _   _       _...

\_____/ \_____/ \_/ \_____/

^                           ^

| ^ ^ ^ 8 7 6 5 4 3 2 1 0 ^ |

| | | | <-- Data bits --> | Mark bit(s)

| | | |   (7 or 8 bits)   |

| | | |                   Start bit

| | | |

| \ / Parity bit (We use even parity, hence 0)

|  |

|  One or two stop bits

|

Mark bit(s)

Even parity: The sum of the data bits is divided by two, and

if the remainder is 0, the parity bit should be

set to 1. If the remainder is 1, the parity bit

should be set to 0.

10100110 = 4 bits are set. 4/2  remainder = 0 -> parity = 1

01110110 = 5 bits are set. 5/2  remainder = 1 -> parity = 0

Odd parity: The sum of the data bits is divided by two, and

if the remainder is 0, the parity bit should be

set to 0. If the remainder is 1, the parity bit

should be set to 1.

10100110 = 4 bits are set. 4/2  remainder = 0 -> parity = 0

01110110 = 5 bits are set. 5/2  remainder = 1 -> parity = 1

The nice thing about the serial device is that we do not need

to bother about this. (So why did I write it?) You only have

to tell the device if you want to use parity, and if so if it

should use even (default) or odd parity. You should also tell

the serial device if you want to send seven or eight data bits,

and one or two stop bits. Once you have told the device what

you want, you do not need to think about it any more.

7.3  THE SERIAL DEVICE

The Serial Port is a limited resource it that sense that there

exist (normally) only one port. There may however be several

programs running at the same time which all want to use the

serial port. To make this possible they all have to coordinate

their work with the port. That is the serial device's main

task.

You can tell the device that you want a shared access, which

means that other programs may also use the serial port, or if

you want exclusive access, no other programs may use the

device. It is then up to the device if it will accept your

demands or else deny you the right to use the serial port.

(Another task may have exclusive access.)

Once you have been allowed to use the serial port, you should

send all your demands to the serial device, and it will take

care of everything. You only have to give it some information

like how it should work (eight or seven bits data, parity?

one or two stop bits, baud rate etc...) and where data should

be fetched/stored.

7.3.1  PREPARE THE SERIAL DEVICE

Before you should open the serial device you need to decide if

you want exclusive or shared access, and if you want to use the

special RTS/CTR, DTR/DSR ("seven-wire-mode").

1. If you want shared access, other programs may also use the

serial port, set the flag SERF_SHARED. (Se below.) If you

want exclusive access you do not need to set any flags,

since it is the default mode.

2. If you want to use the RTS/CTR, DTR/DSR handshaking

protocol ("seven-wire-mode") set the flag SERF_7WIRE.

7.3.2  OPEN THE SERIAL DEVICE

As with all devices you have to open a message port through

which the serial device can talk to you, and allocate a

request block (a IOExtSer structure), before you may open

the device itself.

1. Open a message port: (Since it is only our task and the

device that will use the message port, we do not need

to make it "public", hence no name. Priority should as

usual be set to 0, normal priority.)

struct MsgPort *replymp;

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

2. Allocate a request block of type IOExtSer structure.

(The IOExtSer structure is an extended version of the

normal request block, and should therefore be allocated

with help of the CreateExtIO() function with the size

set to sizeof( struct IOExtSer ).

struct IOExtSer *serial_req;

serial_req = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req )

clean_up( "Not enough memory!" );

Once the message port and the request block have successfully

been created, you should set the desired flags which were

explained above, and then open the serial device.

3. Set desired flags (only SERF_SHARED and SERF_7WIRE are

accepted), and open the device.

UBYTE error;

serial_req->io_SerFlags = SERF_SHARED|SERF_7WIRE;

error = OpenDevice( SERIALNAME, 0, serial_req, 0 );

if( error )

clean_up( "Could not open the Serial Device!" );

7.3.3  SET SERIAL PARAMETERS

Once the serial device has successfully been opened you should

set desired parameters like baud rate, seven or eight data bits

and so on. To set these parameters you initialize the request

block as described below, set the flag SDCMD_SETPARMS and

tells EXEC to execute your request with a DoIO() command.

Here is a list of parameters that should be initialized: (We

assume you have already opened the device as described above,

and have also created an IOExtSer structure with a pointer

named "ioreq" to it.

1. The serial device collects data and stores it in a

temporary buffer. Because of this it can read a lot of

data without any interruption, and once there is a break

the data in this temporary buffer is moved to the real

data buffer. The temporary buffer should be at least

512 bytes, but more is recommended, especially if you

want to collect a lot of data at high speed.

This temporary buffer is automatically created by the

serial device, and also automatically deallocated when

you close the device. If you have set one buffer size,

and later on changes the size, the old buffer will be

removed and a new buffer allocated. All data will then

be lost, so do not change the size of the buffer while

you are still collecting data.

The size is set in the io_RBufLen field of the request

structure. The size must be at least 512, but other

recommended sizes are 1024, 2048, 4096, 8192 or 16384.

To set the internal input buffer size to 8192 bytes do

like this:

ioreq->io_RBufLen = 8192;

2. You must tell the serial device at what speed it should

communicate (how many bits per second). It is important

that both the sender and receiver are using the same

speed.

You set the desired speed by initializing the io_Baud

field to one of the following baud rates (you may use

other speeds, but most applications expect you to use

on of the recommended speeds, and if it is to fast data

can be lost or corrupted): 110, 300, 1200, 2400, 4800,

9600, 19200 or 31250. To set the baud rate to 9600 do

like this:

ioreq->io_Baud = 9600;

3. Your program or the external device you are communicating

with can send a "break message", which will pause the

transmission for a specified time. This is useful if you

have received to many messages and you can not answer all

of them before more are arriving. A small break would in

this case be appreciated. A break signal simply stops the

communication for a specified number of microseconds.

You can set the number of microseconds the communication

should be down after a break signal by initializing the

io_BrkTime field. To set the break time to half a second

do like this:

ioreq->io_BrkTime = 500000;

4. If you want to read normal ASCII characters you only need

the first seven bits, and should therefore only try to

collect seven data bits instead of eight. However,

sometimes you want to receive whole bytes, and should then

collect all eight data bits.

To set the number of bits you want to read each time you

initialize the io_ReadLen field accordingly. To read eight

data bits do like this:

ioreq->io_ReadLen = 8;

5. Since you sometimes only want to send 7 data bits, but

other times want send all 8 data bits, you should

initialize the io_WriteLen field accordingly. To write

eight bits do like this:

ioreq->io_WriteLen = 8;

6. After each character you need to send one or two stop

bit(s), depending on what the other device expect.

Normally you only use one stop bit, but if you are using

seven data bits it happens that you should use two stop

bits. To tell the serial device to use one stop bit to

like this:

ioreq->io_StopBits = 1;

7. There exist some special modes that are selected by

setting some serial flags. Here is a complete list of

possible flags together with a short description:

SERF_XDISABLED  Set this flag if you want to turn off

the XON-XOFF feature. Default is on.

SERF_EOFMODE    Set this flag if you want the Serial

Device to send/collect data until it

finds one of the eight specified

"end-of-file" characters. (See below

for more information.)

SERF_SHARED     Set this flag if you want to share the

serial port with other programs. Note

that this flag should only be used

BEFORE you have opened the device, and

should NOT be changed later on. If you

want to change status you should close

the serial device, alter the status and

then try to open it again.

SERF_RAD_BOOGIE Set this flag if you want to use the

serial port at high speed. When this

flag is set no parity is used, xON/xOFF

handling is turned off, no break signals

are allowed, and finally only eight-bit

characters are used. When you need fast

communication set this flag, but

remember that the data will then not be

error checked, and break signals are

ignored.

SERF_QUEUEDBRK  Set this flag if you want break commands

to be queued along with all other

signals. The default is that a break

command interrupts the process

immediately.

SERF_7WIRE      If the flag is set, the seven-wire

"handshaking" mode will be used.

(Default is three-wire.) With this flag

set you can use the RS232 RTS/CTS,

DTR/DSR commands. This flag should only

be used BEFORE when you have opened the

serial device, and should NOT be changed

after that. To change this mode, you

must close the device, set the desired

mode and then try to open it again.

SERF_PARTY_ODD  Set this flag if you want to use odd

parity. (The default setting is even

parity.)

SERF_PARTY_ON   Parity checking/writing is turned on.

(The sum of all data bits are divided by

two, and the remainder is the parity

bit. If even parity is used the bit will

be set to 1 if the remainder is even. If

odd parity is used the parity bit will

be set to 0 if the remainder is even.

You set desired flags in the io_SerFlags filed of the

request structure. If you want to set several flags, put

a "|" ("binary-OR") between. To set the parity on, and

use the end of character mode, do like this:

ioreq->io_SerFlags = SERF_PARTY_ON | SERF_EOFMODE;

Note! If you do not want to set any flags, remember to set

the field to 0. If you do not make sure it is empty, some

flags might accidentally be set.

8. There exist a special field where more serial flags can

be used in the future. For the moment there exist only two

flags for this field.

SEXTF_MSPON Set this flag if you want to use mark-space

parity rather than odd-even parity.

SEXTF_MARK  If this and the SEXTF_MSPON flag is set, it

will Mark.

If you want to use the mark-space method, with mark, do

like this:

ioreq->io_ExtFlags = SEXTF_MSPON | SEXTF_MARK;

Note! If you do not want to set any of these flags, remember

to set the field to 0!

9. If you have set the flag "SERF_EOFMODE", the serial device

will send/collect data until it finds one of eight

specified "end-of-file" characters. These eight characters

are stored in a IOTArray structure which look like this:

struct IOTArray

{

ULONG TermArray0;

ULONG TermArray1;

};

Each ULONG data consists of four bytes, and in total you

can store eight bytes (end-of-file characters) in the

array. To make the checking routine efficient you must

store the characters in descending order! To copy the

desired characters into the array do like this:

/* Here is an array with all EOF characters: */

/* NOTE! They MUST be in descending order!   */

UBYTE eof_char[8]={ 0x54, 0x32, 0x16, 0x15,

0x12, 0x03, 0x00, 0x00 };

/* Declare a unsigned byte pointer: */

UBYTE *ptr;

/* Simple loop variable: */

int loop;

...

/* Get the address of the IOTArray: */

ptr = (UBYTE *) &(ioreq->io_TermArray);

/* Set all eight end of file characters: */

for( loop=0; loop < 8; loop++ )

{

/* Copy character after character: */

*ptr = eof_chars[ loop ];

/* Step one byte foreward: */

ptr++;

}

Once you have set all desired parameters in the request block

(IOExtSer structure) you set the IO command to SDCMD_SETPARAMS

and tell the serial device to do your request by calling the

DoIO() function. If something went wrong DoIO() will return

with an error number, else 0 is returned which means that your

request have successfully been executed. (See below for a

complete list of error messages.)

ioreq->IOSer.io_Command = SDCMD_SETPARAMS;

error = DoIO( ioreq );

7.3.4  READ DATA

When you want to read data from the serial port, you have to do

four things. First you have to give the serial device a pointer

to a buffer where all data which you read will be stored.

(This memory buffer is not the same as the serial device's own

temporary input buffer that was explained above.)

Secondly you have to tell the device how many bytes/characters

you want to read. You do it by either telling the device

exactly how many characters you want to read, or set the

length to -1 which means you want to read continuously and stop

first when an end-of-file characters have been found. (Remember

to set the flag "SERF_EOFMODE", or the serial device will not

check for termination characters.)

Thirdly you have to set the command flag "CMD_READ", which

tells the serial device that you want to read data from the

serial port.

Finally you tell the serial device to do your request. You can

either put your program to sleep while the serial device is

reading data with a DoIO() call, or you use the SendIO()

function and continue to do something while all data is

collected. There exist even a special "quick" mode which

should be used if very fast communication is needed.

Here is an example on how to read data: (Your program will be

put to sleep while the data is collected.)

error: (UBYTE) is a simple unsigned byte variable.

ioreq: (struct IOExtSer *) is a pointer to an IOExtSer

structure.

data:  (BYTE) is a pointer to a block of memory where all

data which is collected will be stored. Note that the

serial device will not check if it sends more data

than actually can be stored in the buffer. Innocent

memory can be corrupted if you do not make the buffer

big enough!

/* We want to read some data: */

ioreq->IOSer.io_Command = CMD_READ;

/* Give the start address of our data buffer: */

ioreq->IOSer.io_Data = data;

/* We want to read 400 bytes/characters: (The buffer must */

/* then be at least 400 bytes.)                           */

ioreq->IOSer.io_Length = 400;

/* Do our request: */

error = DoIO( ioreq );

/* Everything OK? */

if( error )

printf( "Problems while reading!\n" );

The program above will go to sleep while the data is collected

from the serial port. However, sometimes you may want to do

something while the data is fetched and not go to sleep.

Instead of using the function DoIO() you should then use the

asynchronous SendIO(). As explained in chapter 17 DEVICES, the

SendIO() will not wait for the request to be completed.

To check if the request have been completed or not you can

either use the function CheckIO(), or wait for a message to

arrive at the request block's reply port.

Once the request have been completed you can look at the

io_Error field of the request block to check if everything

was OK. If the field is zero everything was executed without

any problems, but if it is non zero the request failed.

Here is an example:

/* Declare a pointer and set it to NULL: */

struct IOExtSer *ptr = NULL;

...

/* We want to read some data: */

ioreq->IOSer.io_Command = CMD_READ;

/* Give the start address of our data buffer: */

ioreq->IOSer.io_Data = data;

/* We want to read 400 bytes/characters: (The buffer must */

/* then be at least 400 bytes.)                           */

ioreq->IOSer.io_Length = 400;

/* Do our request and return immediately: */

SendIO( ioreq );

/* As long as the pointer is not pointing to */

/* the request we should stay in the loop:   */

while( ptr == NULL )

{

... do something ...

/* Check if the request has been completed: (If the  */

/* request has been completed CheckIO() will return  */

/* a pointer to the request, else NULL is returned.) */

ptr = CheckIO( ioreq );

}

/* Remove the request block's message. (The ptr and ioreq */

/* are in this example identical, so it does not matter   */

/* whichever you will use. The parenthesis around the     */

/* expression is actually unnecessary, but this looks     */

/* better.)                                               */

Remove( &(ptr->IOSer.io_Message.mn_Node) );

/* Everything OK? Check the io_Error filed: */

if( ioreq->IOSer.io_Error )

printf( "Problems while reading!\n" );

Finally their exist a special "quick" mode. It can only

sometimes be used and only for reading. This special mode

can be used when a lot of data at a very (!) high speed

should be collected. The "quick" mode is not the same as the

"SERF_RAD_BOOGIE" mode explained above. The quick mode can

only be used for reading, and there are some special

restrictions, while the high speed rad boogie mode can be

used both for reading and writing.

To use the quick mode you set the IOF_QUICK flag, before

you send the request. You should then not use the normal DoIO()

or SendIO() functions, but instead the low level BeginIO()

function. The BiginIO() is synchronous (like DoIO()) if we

were allowed to use quick mode, else the request is

asynchronous (like SendIO()).

Although you want to use the quick mode, it is not sure you

are allowed to do it. For some requests it is OK, others

will automatically remove the IOF_QUICK flag. You must

therefore check the request after you have posted it to

see if the IOF_QUICK flag is still on or not. If the flag was

removed, the quick mode was turned off and the request should

now be treated as those started with a SendIO() call (remember

to remove the message). However, it the flag is still there

the quick mode was used and we do not need (nor should) remove

any message at the reply port.

/* Initialize the request block as normal. */

/* ...                                     */

/* Try to use the "quick" mode: */

ioreq->IOSer.io_Flags = IOF_QUICK;

/* Do the request: */

BeginIO( ioreq );

if( (ioreq->IOSer.io_Flags & IOFQUICK) )

{

/* OK! The request was using quick mode, which means  */

/* that our task was put to sleep while the data was  */

/* fetched, and the request have now been completed.  */

/* Since we are using quick mode there are no message */

/* that should be removed.                            */

}

else

{

/* Too bad, we were not allowed to use the quick mode. */

/* The request should now be treated as if it was      */

/* started with a SendIO() function call. This request */

/* is asynchronous - request returns immediately.      */

/* We can either use the routine above with CheckIO()  */

/* or put our task to sleep while we are waiting, by   */

/* calling the WaitIO() function. The WaitIO()         */

/* function will automatically remove the message, so  */

/* we do not need to do anything more. However, if we  */

/* used the function Wait(), we have to remove the     */

/* messages ourself.                                   */

WaitIO( ioreq );

}

The "quick" mode can only sometimes be used:

1. You may only read data. The "quick" mode is not allowed for

writing.

2. There is enough data in the internal input buffer so the

request will immediately be satisfied.

3. No other requests is using/waiting for the serial port.

7.3.5  WRITE DATA

When you want to write data to the serial port it is almost the

same procedure as when reading. The only difference is that you

set the command flag "CMD_WRITE".

Here is an example on how to write data to the serial port:

(Your program will be put to sleep while the data is moved to

the serial port.)

error: (UBYTE) is a simple unsigned byte variable.

ioreq: (struct IOExtSer *) is a pointer to an IOExtSer

structure.

data:  (BYTE) is a pointer to a block of memory where all

data you want to send is stored.

/* We want to send (write) some data: */

ioreq->IOSer.io_Command = CMD_WRITE;

/* Give the start address of our data buffer: */

ioreq->IOSer.io_Data = data;

/* We want to send 150 bytes/characters: */

ioreq->IOSer.io_Length = 150;

/* Do our request: */

error = DoIO( ioreq );

/* Everything OK? */

if( error )

printf( "Problems while writing!\n" );

The program above will go to sleep while the data is copied to

the serial port. If you do not want the program to wait for the

request to be completed you should use SendIO(). The procedure

is very similar with reading.

To check if the request have been completed or not you can

either use the function CheckIO(), or wait for a message to

arrive at the request block's reply port.

Once the request have been completed you can look at the

io_Error field of the request block to check if everything

was OK. If the field is zero everything was executed without

any problems, but if it is non zero the request failed.

Here is an example:

/* Declare a pointer and set it to NULL: */

struct IOExtSer *ptr = NULL;

...

/* We want to send (write) some data: */

ioreq->IOSer.io_Command = CMD_WRITE;

/* Give the start address of our data buffer: */

ioreq->IOSer.io_Data = data;

/* We want to send 280 bytes/characters: */

ioreq->IOSer.io_Length = 280;

/* Do our request and return immediately: */

SendIO( ioreq );

/* As long as the pointer is not pointing to */

/* the request we should stay in the loop:   */

while( ptr == NULL )

{

... do something ...

/* Check if the request has been completed: (If the  */

/* request has been completed CheckIO() will return  */

/* a pointer to the request, else NULL is returned.) */

ptr = CheckIO( ioreq );

}

/* Remove the requst block's message. (The ptr and ioreq */

/* are in this example identical, so it does not matter  */

/* whichever you will use. The parenthesis around the    */

/* expression is actually unnecessary, but this looks    */

/* better.)                                              */

Remove( &(ptr->IOSer.io_Message.mn_Node) );

/* Everything OK? Check the io_Error filed: */

if( ioreq->IOSer.io_Error )

printf( "Problems while writing!\n" );

You can not use the quick mode while you are sending data. If

you need fast communication you should set the serial flag

"SERB_RAD_BOOGIE". Note the restrictions mentioned above. (Only

eight-bit characters, and no parity checking etc...)

Since you usually will want to read and write data at the same

time, you have to use two separate request blocks. If you are

using shared access mode you simply create two request blocks

and open the serial device twice. (Do not forget to close both

requests later on.) Here is an example:

struct MsgPort *replymp;

struct IOExtSer *serial_req_read;

struct IOExtSer *serial_req_write;

UBYTE error;

/* We use only one reply message port: */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create the request block "read": */

serial_req_read = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req_read )

clean_up( "Not enough memory!" );

serial_req_read->io_SerFlags = SERF_SHARED|SERF_7WIRE;

/* Create the request block "write": */

serial_req_write = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req_write )

clean_up( "Not enough memory!" );

serial_req_write->io_SerFlags = SERF_SHARED|SERF_7WIRE;

/* Open the serial device for the read request: */

error = OpenDevice( SERIALNAME, 0, serial_req_read, 0 );

if( error )

clean_up( "Could not open the serial device (Read)!" );

/* Open the serial device for the write request: */

error = OpenDevice( SERIALNAME, 0, serial_req_write, 0 );

if( error )

clean_up( "Could not open the serial device (Write)!" );

If you are using exclusive access mode you can of course not

open the serial device twice. Instead we have to copy the

first request block to the other request block, byte for byte.

Here is an example:

struct MsgPort *replymp;

struct IOExtSer *serial_req_read;

struct IOExtSer *serial_req_write;

BYTE *r_ptr;

BYTE *w_ptr;

UBYTE error;

int loop;

/* We use only one reply message port: */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create the request block "read": */

serial_req_read = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req_read )

clean_up( "Not enough memory!" );

serial_req_read->io_SerFlags = SERF_SHARED|SERF_7WIRE;

/* Create the requestblock "write": */

serial_req_write = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req_write )

clean_up( "Not enough memory!" );

serial_req_write->io_SerFlags = SERF_SHARED|SERF_7WIRE;

/* Open the serial device for the read request: */

error = OpenDevice( SERIALNAME, 0, serial_req_read, 0 );

if( error )

clean_up( "Could not open the Serial Device (Read)!" );

/* Since we can not open the serial device once again */

/* for the write request, we have to copy the whole   */

/* read request block into the write request block.   */

/* Get the start address of both request blocks: */

r_ptr = (BYTE *) serial_req_read;

w_ptr = (BYTE *) serial_req_write;

/* Copy the request block, byte by byte: */

for( loop=0; loop < sizeof( struct IOExtSer ); loop++ )

{

/* Copy one byte: */

*w_ptr = *r_ptr;

/* Step one byte forward: */

w_ptr++;

r_ptr++;

}

Now we have two request blocks, and can therefore use both

read and write requests at the same time. If you also want to

use the special extra functions described further down, you

may want to create even one more request block (or maybe

even more...). The procedure is the same as described above.

7.3.6  ERRORS

While you are using the serial device several errors may

occur. The most common problem is that some other program is

already using the serial port, but it can happen that data has

been corrupted while reading/writing (the parity checking is

a simple way to find these errors).

There exit for the moment nine different types of serial port

errors, all defined in the header file "devices/serial.h".

You will receive the error message from the function you

just called, or you can check a request block to see if there

were any problems. (The io_Error filed of the request block

either contains 0, which means everything is OK, or an error

number.)

Here is a complete list of the serial error messages:

SerErr_DevBusy        Some other task/request is already

using the serial device.

SerErr_BufErr:        The serial device could not allocate

enough memory for the internal input

buffer.

SerErr_InvParam:      The request block's parameters were not

properly initialized.

SerErr_LineErr:       The serial cable is faulty or the other

device is not properly initialized or

not connected. Tell the user to check the

cables!

SerErr_ParityErr:     The data you just sent/received contained

at least one byte which was corrupted.

Note that the serial flag "SERF_PARITY_ON"

must be set, else the device will not

check for errors.

SerErr_TimerErr:      There was some problem with the timer.

SerErr_BufOverflow:   The serial input buffer has been filled.

You should empty it as fast as possible

so you do not lose valuable information.

SerErr_NoDSR:         No DSR.

SerErr_DetectedBreak: A break was detected.

While you are using the serial device it may happen that you

receive other error messages than described above. It is

then usually the Exec which sent them: (defined in the header

file "exec/errors.h")

IOERR_OPENFAIL  The device (unit) could not be opened. (If you

are denied access to the serial device you

should receive the SerErr_DevBusy flag instead

of this exec message, but internally this flag

is used.)

IOERR_ABORTED   When you abort a previously started request by

calling the AbortIO() function, the io_Error

filed of that request is set to IOERR_ABORTED.

If you find a request block with this flag set,

you know that it has been aborted.

IOERR_NOCMD     You tried to use a command that is not

supported by the serial device.

IOERR_BADLENGTH The length of the request was not valid.

7.3.7  CLEAN UP

As usual on the Amiga you must remember to close and return

everything you have opened or allocated. If you do not close

the serial device after you, other programs will then not

be able to use the serial port. PLEASE be very careful about

this!

Here is a list of what you have to do:

1. All requests you have started with SendIO() or BeginIO()

(asynchronous commands) must either have been completed

or aborted before you may close the device. It is a very

common error to forget this, and it can be hard to find

this bug. Usually the program will work fine (the command

was completed in time), but now and then your program will

crash (the command was completed after the device have

been closed).

A simple way is to abort all commands that have not

reported that they have been completed, but this is not

always good way to do it. (The last commands may be

important and should therefore not be aborted.)

If you do not want to abort the command, you should

instead wait for it to be completed. The WaitIO()

function is simple to use, and will put your program

to sleep while waiting, so no computer time is wasted.

If the request has already been completed, the function

will return immediately. WaitIO() will also remove the

message from the reply port. It is a very useful and

simple function to use, but do NOT try to wait for

a request that has not been started! (It will then be

a very long wait, probably some million years before

the user realizes that he/she has to hit the reset keys.)

Here is an example on how to wait for a request to be

completed: (If the request already has been completed

it does not matter, WaitIO() will then simply return

immediately. Note that we do not have to remove any

messages from the reply port if we use WaitIO().)

/* Store possible error numbers here: */

UBYTE error;

/* ... */

/* Wait for the request to be completed: */

error = WaitIO( ioreq );

/* Everything OK? */

if( error )

printf( "Something went wrong!" );

/* Well, successful or not, we may now */

/* close the device!                   */

To abort a request, simply use the AbortIO() function:

/* Try to abort a previously started request:  */

/* (Do not try to abort a request that has not */

/* been started.)                              */

AbortIO( ioreq );

2. When all requests have been completed or aborted you may

close the serial device. (Requests that have been executed

by calling the DoIO() function have already been completed

before your program wakes up, and thus you do not need to

wait for these.)

The serial device is closed as all other devices, by

calling the CloseDevice() function. Here is an example:

/* Close the Serial Device: */

if( !serial_dever )

CloseDevice( ioreq );

3. You should now return all request blocks you have

allocated. If you allocated a standard sized request block

by calling the CreateStdIO() function, you should free

it by calling the DeleteStdIO() function. However, if

you have allocated an extended request block (like the

serial device's request blocks) by calling the

CreateExtIO() function, you MUST call the DeleteExtIO(),

and of course remove the same amount of data as you

allocated.

How to delete a standard request block: (Since it is of

the standard size, you do not need to specify any size.)

/* Deallocate a standard sized request block: */

DeleteStdIO( ioreq );

With extended request block you have to specify the size

as you did when you allocated it:

/* Deallocate an extended request block: */

/* (The size may vary depending on what  */

/* device it was used for.)              */

DeleteExtIO( ioreq, sizeof( struct IOExtSer ) );

Note that ALL request blocks that have been allocated,

must be removed!

4. Finally you should close all message ports you have

previously opened. Simply use the DeletePort() function

as this example demonstrates:

/* Remove the replyport: */

DeletePort( replymp);

It can not be said to often. Please be careful with how your

program terminates! Your program should not only run fine,

but it should also allow other programs to run after and

simultaneously. Remember that your program must also be able

to quit nice and neatly even if it had to terminate too

early because of some fatale error. The cleaning up should

only be done where it is needed, and if you have not allocated

the memory or opened the device before your program quits, you

should of course NOT try to free these resources! If you do the

Amiga will most certainly crash! Too many programs contain this

very annoying error.

The best way to manage this cleaning up routine is to write a

separate function which checks each thing before it frees it.

The idea is that you may call this routine at any time, and it

will still manage to clear everything properly. Here is an

example: (Note how we check each thing to see if it should be

removed!)

/* Close and return everything that has been */

/* opened and allocated before we quit:      */

void clean_up()

{

/* 1. Close the Serial Device: */

if( !serial_dever )

CloseDevice( serial_req );

/* 2. Deallocate the serial request block: */

if( serial_req )

DeleteExtIO( serial_req, sizeof( struct IOExtSer ) );

/* 3. Remove the replyport: */

if( replymp )

DeletePort( replymp);

/* 4. Quit: */

exit( 0 );

}

Comments: 1. The serial_dever is a variable that we have

declared ourself, and is used only to determine

if the device has been opened or not. (The

variable is set to TRUE before the program

starts, and is only changed to FALSE after

a successful opening of the device. See the

included examples for more information.) We

should of course only close the device if

the serial_dever is FALSE. ("serial_dever"

stands for "serial device error".)

2. The serial_req is a pointer which is either

pointing to a request block, or NULL if no

request block has been allocated. We should

of course only free the request block if there

exist one.

3. The replymp is a pointer which is either

pointing to a message port, or NULL if no message

port has been opened. And of course, we should

only close the message port if it has been

opened.

4. Finally our program may terminate. The exit()

function is placed here so we are sure the

program will quit. (If we did not have this

exit(), our function would then return the

control to our program again, and then we would

have serial (sorry, serious) problems.

7.4  A COMPLETE EXAMPLE

Here is a complete example that opens the serial device, sets

all necessary parameters, sends some data, collects some data

and finally cleans up and quits. Since we send the read and

write requests after each other (the first is completed before

the other is started) we only need to use one request block.

#include <exec/types.h>

#include <devices/serial.h>

/* Declare a pointer to our reply port: */

struct MsgPort *replymp = NULL;

/* Declare a pointer to our serial request block: */

struct IOExtSer *serial_req = NULL;

/* Store the serial device error here: */

UWORD serial_dever = TRUE;

/* Declare our own data buffer: (Big enough to hold 300 bytes.) */

BYTE buffer[ 300 ];

/* Declare our functions: */

void main();

void clean_up( STRPTR text );

void main()

{

int loop;

UBYTE *ptr;

UBYTE error;

/* The eight end-of-file characters: */

UBYTE eof_char[8]={ 0x06, 0x05, 0x04, 0x03,

0x02, 0x01, 0x00, 0x00 };

/* OPEN THE SERIAL DEVICE: */

/* Get a reply port: (No name, priority 0) */

replymp = (struct MsgPort *)

CreatePort( NULL, 0 );

if( !replymp )

clean_up( "Could not create the reply port!" );

/* Create a serial request block: */

serial_req = (struct IOExtSer *)

CreateExtIO( replymp, sizeof( struct IOExtSer ) );

if( !serial_req )

clean_up( "Not enough memory for the serial request block!" );

/* Open the Serial Device: */

serial_dever = OpenDevice( SERIALNAME, 0, serial_req, 0 );

if( serial_dever )

clean_up( "Could not open the Serial Device!" );

/* SET THE SERIAL PARAMETERS: */

/* Set the Serial Device's own input buffer to 512 bytes: */

serial_req->io_RBufLen = 512;

/* Set baud rate to 9600 baud: */

serial_req->io_Baud = 9600;

/* Set break time to half a second: */

serial_req->io_BrkTime = 500000;

/* Read 8 bits per character: */

serial_req->io_ReadLen = 8;

/* Write 8 bits per character: */

serial_req->io_WriteLen = 8;

/* Use 1 stop bit: */

serial_req->io_StopBits = 1;

/* Use parity and end-of-file characters: */

serial_req->io_SerFlags = SERF_PARTY_ON | SERF_EOFMODE;

/* No additional flags: */

serial_req->io_ExtFlags = NULL;

/* Set all eight end of file characters: */

ptr = (UBYTE *) &(serial_req->io_TermArray);

for( loop=0; loop < 8; loop++ )

{

/* Copy character after character: */

*ptr = eof_char[ loop ];

/* Step one byte forward: */

ptr++;

}

/* All values have now been set, lets do a SDCMD_SETPARAMS request: */

serial_req->IOSer.io_Command = SDCMD_SETPARAMS;

/* Do our request: */

error = DoIO( serial_req );

if( error )

clean_up( "Could not set the serial parameters!" );

/* SEND DATA TO THE SERIAL PORT: */

/* Put the data we want to send to the serial port into */

/* our own data buffer: (Well in this example we only   */

/* send two bytes.)                                     */

buffer[ 0 ] = 0x4C;

buffer[ 1 ] = 0x31;

/* We want to send (write) some data: */

serial_req->IOSer.io_Command = CMD_WRITE;

/* Give the start address of our data: */

serial_req->IOSer.io_Data = (APTR) buffer;

/* We want to send two bytes: */

serial_req->IOSer.io_Length = 2;

/* Do our request: */

error = DoIO( serial_req );

if( error )

clean_up( "Could not send data to the serial port!" );

/* READ DATA FROM THE SERIAL DEVICE: */

/* We want to read some data: */

serial_req->IOSer.io_Command = CMD_READ;

/* Give the start address of our buffer: */

serial_req->IOSer.io_Data = (APTR) buffer;

/* We want to read 0 bytes: */

/* (If you do not have anything connected to your serial */

/* port, we better not wait for any signals, thus read   */

/* only 0 bytes.)                                        */

serial_req->IOSer.io_Length = 0;

/* Do our request: */

error = DoIO( serial_req );

if( error )

clean_up( "Could not read data from the serial port!" );

/* THE END: */

clean_up( "The End" );

}

/* Close and return everything that has been */

/* opened and allocated before we quit:      */

void clean_up( STRPTR text )

{

/* Close the Parallel Device: */

if( !serial_dever )

CloseDevice( serial_req );

/* Deallocate the serial request block: */

if( serial_req )

DeleteExtIO( serial_req, sizeof( struct IOExtSer ) );

/* Remove the replyport: */

if( replymp )

DeletePort( replymp);

/* Print the message: */

printf( "\n%s\n", text );

/* Quit: */

exit( 0 );

}

7.5  OTHER USEFUL COMMANDS

You can not only read and write from/to the serial port. There

exist several other functions that can sometimes be needed.

Here is a complete list:

1. Break, stop the serial device for a short time.

2. Clear, clear the input buffer.

3. Flush, removes all queued requests.

4. Query, get some information from the serial device.

5. Reset, reinitializes the serial device.

6. Start, restarts the serial communication.

7. Stop, temporary stops the serial communication.

7.5.1  BREAK

While you are sending or reading data you sometimes have to

take a small pause. Your program may for example need to empty

the data buffer, or coordinate its action with some other task.

To pause the serial device you simply send a break request,

and all communications is halted for a specified time.

You send a break signal by setting the io_Command field to

"SDCMD_BREAK". The default is that all serial requests are

immediately halted, but if the serial flag "SERF_QUEUEDBRK"

is set, the break command will be queued as all other requests

and the device will first take a break when all previous

requests have been completed.

The serial device will normally take a 250000 microseconds

(1/4 seconds) long break, but you may change this break time

by altering the serial device's "io_BrkTime" parameter.

Here is an example on how you can send a break command:

/* We want to take a pause: */

ioreq->IOSer.io_Command = SDCMD_BREAK;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Problems with the break!\n" );

7.5.2  CLEAR

While you are reading data from the serial device, it is

actually first stored in the internal input buffer, and then

moved to your own buffer when requested. If you want to clear

the buffer before you start to read you should set the

io_Command field to "CMD_CLEAR". Here is an example:

/* We want to clear the input buffer: */

ioreq->IOSer.io_Command = CMD_CLEAR;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not clear the input buffer!\n" );

7.5.3  FLUSH

If several requests are sent to the device they are all queued

on a FIFO (First In First Out) basis. The command "CMD_FLUSH"

can then be used to remove all these queued commands. Here

is an example:

/* We want to remove all queued requests: */

ioreq->IOSer.io_Command = CMD_FLUSH;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not remove the queued requests!\n" );

7.5.4  QUERY

The command "SDCMD_QUERY" can be used to get some information

from the serial device. It is useful if you want to see the

serial status and/or how many bytes there are already in the

internal input buffer. The "io_Status" field of the request

structure will be set as following table shows:

Bit  Hex     Active  Description

---------------------------------------------------------------

0  0x0001  -       Reserved

1  0x0002  -       Reserved

2  0x0004  High    Connected to par. "select" and ser. "ring"

3  0x0008  Low     DSR - Data Set Ready

4  0x0010  Low     CTS - Clear To Send

5  0x0020  Low     DCD - Carrier Detect

6  0x0040  Low     RTS - Ready To Send

7  0x0080  Low     DTR - Data Terminal Ready

8  0x0100  High    Read overrun

9  0x0200  High    Break sent

10  0x0400  High    Break received

11  0x0800  High    Transmit X-OFF

12  0x1000  High    Receive X-OFF

13  0x2000  -       Reserved

14  0x4000  -       Reserved

15  0x8000  -       Reserved

---------------------------------------------------------------

The field "io_Actual" of the request structure contains the

number of bytes still left in the internal input buffer. Here

is an example:

/* Check the serial device: */

ioreq->IOSer.io_Command = SDCMD_QUERY;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not get any information from the device!\n" );

else

{

/* Check the "io_Status" field: */

if( ioreq->io_Status & 0x0200 )

printf( "A break request have just been sent!\n" );

/* Check number of characters left in the input buffer: */

printf( "Characters left: %ld\n", ioreq->IOSer.io_Actual );

}

7.5.5  RESET

Send the command "CMD_RESET" to reset the serial device. All

commands that are queued to the device will be removed, commands

that are currently executed will be aborted, the internal input

buffer will be cleared and reallocated to the default size and

finally all serial flags are resetted. Here is an example:

/* We want to reset the serial device: */

ioreq->IOSer.io_Command = CMD_RESET;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not reset the serial device!\n" );

7.5.6  START

After you have stopped the serial communication by sending an

X-OFF message to the other device, you may want to start the

communication again. It is done by sending an X-ON message,

and the external device will then know that it may start to

send/receive data. X-ON messages are sent to the other device

(as well to our self) by issuing a "CMD_START" command. Here

is an example:

/* We want to start serial communication again: */

ioreq->IOSer.io_Command = CMD_START;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not start the serial communication!\n" );

7.5.7  STOP

To temporary stop all serial communication you send a

"CMD_STOP" command. If you try to stop the communication an

X-OFF message will be sent to the external device (as well as

to all other programs currently using the serial device). When

a X-OFF message has been received, communication will first

start again when an X-ON message is sent. (See CMD_START.) Here

is an example:

/* We want to temporary stop all serial communication: */

ioreq->IOSer.io_Command = CMD_STOP;

/* Do our request: */

error = DoIO( ioreq );

/* OK? */

if( error )

printf( "Could not stop the serial communication!\n" );

7.6  FUNCTIONS

DoIO()

DoIO() is used to send requests to a device, and waits for it

to be completed. While the program is waiting it is put to

sleep so it will not waste any computer time. DoIO() will

return first when the request have been completed or failed,

and no message is therefore sent to the reply port.

Synopsis: error = DoIO( req );

error:    (long) DoIO() will return first when the request has

been completed or something has failed. If the

request was successfully completed zero is returned,

else an error number is returned. What error number

depends on which device was used.

req:      (struct IORequest *) Pointer to the request you

want to have executed.

SendIO()

SendIO() is used to send requests to a device, but will

return immediately without any delay. To check if the request

have been completed use the CheckIO() function, or look

at the request's reply port for any messages. Once the

request has been completed you must remove the message at

the reply port. (CheckIO() will not do it.) To remove a

message use the function Remove(). Note that you may NOT

close the serial device before all requests have been

completed or aborted!

Synopsis: SendIO( req )

req:      (struct IORequest *) Pointer to the request you

want to have executed.

CheckIO()

CheckIO() is used to check if a previously started request

has been completed. Note that this function will not remove

the message at the reply port. This must be done with the

Remove() function.

Synopsis: ptr = CheckIO( req );

ptr:      (long) CheckIO() will either return NULL if the

request have not been completed or it will return a

pointer to the request block.

req:      (struct IORequest *) Pointer to the request you

want to check.

WaitIO()

WaitIO() will wait for the request to be completed, and while

the program is waiting it is put to sleep so no computer time

is wasted.

Synopsis: error = WaitIO( req );

error:    (long) WaitIO() will return first when the request,

that has previously been sent, has been completed or

something has failed. If the request was successfully

completed zero is returned, else an error number is

returned. What error number depends on which device

was used.

req:      (struct IORequest *) Pointer to the request you

want to wait for to be completed. Note that the

request must have already been sent to the device

by either a SendIO() or BeginIO() function call.

BeginIO()

BeginIO() is a low level form of the SendIO() function. The

advantage with BeginIO() is that no fields of the request

block will be altered as which is the case with SendIO(). If

you are for example using the "quick mode" you should not

use SendIO() since it will alter some of the data blocks.

BeginIO() is synchronous command if you use the quick mode,

but if you are not, the command will be asynchronous. Note

that you may NOT close the serial device before all requests

have been completed or aborted, so be careful with

asynchronous commands.

Synopsis: BeginIO( req )

req:      (struct IORequest *) Pointer to the request you

want to have executed. BeginIO() will not alter any

values in this request structure as which is the case

with SendIO() and DoIO(). Normally this is not any

problem, and thus the SendIO() and DoIO() functions

should be used. However, with the Audio Device for

example some of these fields which are altered should

not be changed, and you should therefore use this

low level function BeginIO().

AbortIO()

AbortIO() will try to abort a previously started request. This

function should be used sparsely since it does not look so

good if you start a request and the try to stop it. (Better

not start it at all.) However, it is easy, and can sometimes

be very useful.

A request that is aborted will have its io_Error field set

to IOERR_ABORTED (defined in header file "exec/errors.h").

Synopsis: AbortIO( req )

req:      (struct IORequest *) Pointer to the request you

want to abort.

CloseDevice()

CloseDevice() will (surprise!) close a device. If you close

the serial device the internal input buffer will

automatically be deallocated. Note that you should NOT close

the device before all started asynchronous requests have been

either completed or aborted.

Synopsis: CloseDevice( ioreq );

ioreg:    (struct IORequest *) Pointer to the device's

request block.

OpenDevice()

OpenDevice() will try to open the specified device.

Synopsis: error = OpenDevice( name, unit, req, flags );

error:    (long) If OpenDevice() managed to open the device

it returns 0, else an error number is returned.

If you try to open the serial device, and there is

already a program that is using it, the error

message "SerErr_DevBusy" is returned.

name:     (char *) Name of the device you want to open.

The name of the serial device is defined as

SERIALNAME in header file "devices/serial.h".

unit:     (long) Which unit you want to open. Since there

exist only one serial port, this field is ignored.

req:      (struct IORequest *) Pointer to a request block.

For the serial device it must be a pointer to an

extender serial request block (struct IOExtSer).

flags:    (long) Any special mode is set here. Ignored by

the serial device.

7.7  COMMANDS

Here is a complete list of commands you may send to the serial

device. For full documentation se examples above.

The special serial device commands: (Defined in header file

"devices/serial.h")

SDCMD_BREAK      Sends a break signal.

SDCMD_QUERY      Check the status of the serial device.

SDCMD_SETPARAMS  Set the parameters of the serial device.

The rest of the commands you may use are normal exec commands,

and are defined in header file "exec/io.h".

CMD_RESET  Resets all parameters of the serial device.

CMD_READ   Read data from the serial port.

CMD_WRITE  Write data to the serial port.

CMD_CLEAR  Clears the internal input buffer.

CMD_STOP   Temporary stops all serial communication. (X-OFF)

CMD_START  Restarts serial communication. (X-ON)

CMD_FLUSH  Removes all queued requests.

7.8  EXAMPLES

The included examples demonstrates how you can use the Serial

Device. It demonstrates most of the features described in this

chapter. Since I do not know what you have connected to your

serial port the examples will not do very much. However, the

examples are easy modify, so it should not be hard for you to

change them as desired.

Example 1

If you have a Sharp JX-100 scanner you can run this program

since it will try to turn the lamp on and then off again.

Very useful! (hmmm...) The program does not check if there

is any contact with the scanner, nor if the lamp really was

turned on or not. It simply demonstrates how to send data.

Example 2

This example is rather similar to Example 1, but this time

we do not wait for the serial port to complete our request.

Instead we do somethings (well not very much) and now and

then checks if the request has been completed. Using a busy

wait.

Example 3

This example is also rather similar to Example 1, but this

time we try to read and write at the same time. To be able

to do several requests simultaneously we need one request

block for each command. In this example we use three

separate request blocks. Using asynchronous commands but

puts the task to sleep just before we clear and return

everything.

Example 4

This example does not do anything, but it consists of

several useful functions that you can use yourself after

small modifications. The functions demonstrates all

commands there exist for the serial device, so if you

had problems in understanding how a command was used you

can look here.