Disk Enhancer v1.0   Copyright 1992

Disk Enhancer is a utility that can speed up the operation of your

floppy diskette drives.  This is accomplished by modifying values in the

Disk Base table, an 11 byte segment in low memory that is read directly

by the diskette controller.  The diskette controller uses the Disk Table

to determine timing values (how the long the controller gives the drive's

motor to complete various tasks) for the diskette drives connected to it.

Disk Enhancer enhances diskette drives only -- it does not effect hard drive

parameters or performance in any way.  Disk Enhancer is NOT A TSR, so it

does not use up memory.  Just run Disk Enhancer every time you boot/reboot,

and the Disk Base table will be updated.  The easiest way to accomplish

this is by calling Disk Enhancer from your AUTOEXEC.BAT.  This program

resets the following values:

Motor Off delay-      default: 2 seconds        new: user specified

This is the amount of time the diskette drive motor stays on after

completing a diskette operation.  The main feature of this program is

to allow you to specify up to a 14 second delay via a command line

parameter.  Increasing the delay allows you to enter DOS commands

without having to wait for the diskette drive motor to spin up between

every command.

Head Settle time-     default: 15 milliseconds  new: 2 milliseconds

This is the amount of time the diskette controller waits for the head

to become stabilized after seeking to a different track.

Motor Startup time-   default: 1 second         new: 0.25 second

This is the amount of time the diskette controller waits for the

diskette drive motor to spin up.

Performance increases will depend on your machine, but might be up to

5 seconds on copy operations over 1MB.  Also, older machines may not be

able to handle the faster speed settings.  It is a good idea to insure

that disk operations operate normally before permanently using this program

on your PC.  This can be accomplished by copying several text files to a

diskette, then verifying that the data was copied correctly by using a text

editor or the TYPE command.  The responsibility of insuring that this program

works on your machine is completely yours, so use at your own risk!

To run Disk Enhancer type:

DISKENH <delay (in seconds)>

where delay is how long you want the diskette drive motor to stay on after

completing a diskette read/write.  Valid values are 1 through 14 inclusive.

For example:

DISKENH 7

sets the delay to 7 seconds.

Disk Enhancer must be run every time you boot/reboot your PC.

This program is freeware and is for everyone's personal use.  However,

this program may not be used commercially, to produce any tangible profit,

nor can it be packaged with any other products/software without my written

consent.

Lee Greer

Marietta, GA

#