DNDBBS Documentation                                           Page 1

Table of contents                                              Page

-----------------                                              ----

1.0  Hardware requirements . . . . . . . . . . . . . . . . . . . 2

2.0  File distribution . . . . . . . . . . . . . . . . . . . . . 2

3.0  Modem settings  . . . . . . . . . . . . . . . . . . . . . . 2

4.0  Communications device driver  . . . . . . . . . . . . . . . 2

5.0  Console ansi support  . . . . . . . . . . . . . . . . . . . 3

6.0  Command line options  . . . . . . . . . . . . . . . . . . . 3

7.0  Dndbbs as a single user program . . . . . . . . . . . . . . 4

8.0  Problem determination . . . . . . . . . . . . . . . . . . . 4

9.0  Configuration problems  . . . . . . . . . . . . . . . . . . 5

10.0 Setup instructions  . . . . . . . . . . . . . . . . . . . . 5

11.0 Configuring instructions  . . . . . . . . . . . . . . . . . 5

12.0 Operating dndbbs  . . . . . . . . . . . . . . . . . . . . . 6

13.0 Modem communications  . . . . . . . . . . . . . . . . . . . 7

14.0 Configuring for communications ports  . . . . . . . . . . . 7

15.0 Door information and nodes  . . . . . . . . . . . . . . . . 7

16.0 Door batch files  . . . . . . . . . . . . . . . . . . . . . 8

17.0 System environment  . . . . . . . . . . . . . . . . . . . . 8

18.0 Files used  . . . . . . . . . . . . . . . . . . . . . . . . 9

19.0 Compiling instructions  . . . . . . . . . . . . . . . . . . 9

20.0 Rbbs door file structure information  . . . . . . . . . . . 10

DNDBBS Documentation                                           Page 2

1.0  Hardware requirements

--------------------------

The  adventure system  requires  an IBM (or compatible) PC, XT, or AT,

dos 4.0  or  higher,  serial port,  hayes  smartmodem (or compatbile),

minimum  of 640k memory,  an 80 column color monitor,  and 10 megabyte

fixed disk drive with at least one megabyte of free disk storage.

2.0  File distribution

----------------------

The adventure system is distributed on  Executive-PC Bbs,  data number

1-414-789-4210,  in file dnd29c4.zip containing all  executable files,

documentation, data, source, and  help files.  You must use the PKWare

utility to separate the programs from the compressed .zip files.

3.0  Modem settings

-------------------

Dndbbs  can autoanswer the  modem by  receiving  the response ring and

returning the command ata,  or by receiving  the response connect then

the modem speed (i.e. connect 2400). This way the modem can autoanswer

with s0=1 or  with s0=0  (modem register s0 sets the number of rings).

Dndbbs configures serial ports for 300 up to 38,400 baud at no parity,

eight data bits,  and one stop bit (n,8,1). The initial modem settings

should be one of these commands:

Modem initialization string 1: atm0e0q0v1x1

m0     --   speaker off

e0     --   do not echo commands

q0     --   enable result codes

v1     --   verbose result codes

x1     --   extended result codes

Modem initialization string 2: atm0e0q0v1x1&c1&d2

&c1    --   carrier detection

&d2    --   DTR detection

Modem initialization string 3: atm0e0q0v1x1&c1&d2s0=1Ss=255

s0=1   --   autoanswer on ring one

s2=255 --   disable escape codes

4.0  Communications device driver

---------------------------------

Dndbbs can access most communications  device drivers for modem input,

output and initializing. Normally all you need to do is put the device

driver  statement  in your  config.sys file and reboot  to install the

port driver.  The  fossil  (fido/opus/seadog standard interface layer)

driver distributed as x00.sys is required to  be placed  in config.sys

with  the statement  device=x00.sys,  or if  you are  using more ports

(up to a maximum of eight),  use the statement  device=x00.sys 2, etc.

DNDBBS Documentation                                           Page 3

5.0  Console ansi support

-------------------------

Dndbbs sends color code changes to the ansi.sys device driver. This is

done by calling the basic interrupt function. The ansi driver is found

on  your dos disk.  Install the  driver  by placing the program on the

boot directory, add the statement device=ansi.sys into the  config.sys

file then reboot. Desqview users  should  put the statement dvansi.com

in the current dndbbs  partition.  The  ansi.sys driver is distributed

with dndbbs.  The vdisk.sys device should be installed for a small ram

disk of 16k and selected in the dndcnfg files variables.  Vdisk.sys is

only required for the ram disk option if you are using  multinodes for

message transfers between nodes by using the message work files.

Example contents of config.sys:

files=20

buffers=8

device=ansi.sys

device=vdisk.sys s:16

Dndbbs can also utilize the enhanced ansi code as stated by the avatar

device driver specification. The driver included for using these codes

codes are zavt.sys and zavtnk.sys.  The zavtnk file has  the same ansi

codes  without  keyboard  redefinition code parsing.  When  the config

options Prompt For Avatar, and Detect Remote Avatar are selected, then

dndbbs will use local avatar code sequences.  Dndbbs  will also detect

remote avatar if these options are selected. Example Avatar driver is,

device=zavtnk.sys.

6.0  Command line options

-------------------------

The  command line options  are  parameters you type after  the command

dndbbs  at the dos prompt.  If there  are  any  parameters at all, the

first must  be the node number.  The node number  must be in the range

from 0 to 9 or A to Z. All command line options should be separated by

a space. There are six options: [node],  local,  bbs, bypass, disable,

and name. Bbs starts the bulletin board service. Local starts the game

without modem communications. Bypass skips the startup screen. Disable

prevents dm commands or dm logins from being entered.  Name starts the

dndbbs with the username in quotes without being required to enter the

user's password.  The last two optional parameters are for over-riding

the communications port number specified in the configure modem screen

variables and  over-riding  the amount of time the user can be online.

The port parameter is /p:n where n is replaced with the alternate port

number. The time parameter is /t:n where n is replaced with the amount

of time the user can be online in minutes.

Examples starting dndbbs:

dndbbs bbs

dndbbs local

dndbbs 1 bypass

dndbbs c local name="uzernaym"

dndbbs bbs /p:1 /t:30

DNDBBS Documentation                                           Page 4

7.0  Dndbbs as a single user program

------------------------------------

To start the adventure at your computer without a modem or serial port

type dndbbs local and you  will be able to enter the program from your

keyboard. When started locally you  are not required to operate dndbbs

as a bbs, need to  have a modem,  or need the fossil driver installed.

8.0  Problem determination

--------------------------

The  modem does not  answer the phone.  This is  probably because  the

modem  is not initializing correctly. Try rebooting and  then starting

the program.  Try initializing the  modem with  another communications

program.  If none of these work then  your modem might not support the

hayes  command set.  When you start dndbbs, the fossil is initialized,

then  the opening  screen  appears.  The  status indicator  will  show

Waiting for Calls if  the modem has initialized correctly.

If the status indicator shows Reset Timeout or Init Timeout this means

the modem has not sent ok back to dndbbs and  the modem might not have

initialized properly.  In this case dndbbs  continues normally anyway.

Most users  do not  increment the files= statement  in config.sys when

installing dndbbs for the  first time.  Dndbbs uses sixteen files  for

its own use. Dos uses four files. Adjust  the  files= statement in the

config.sys file to 20 or more. The increased files will go into effect

when you reboot.

The message Out Of Memory,  or Program To Large To Load during exiting

to edit, dndcnfg, or dndedit  from dndbbs opening screen,  or from the

dnd game indicates that the program cannot load the secondary ones. In

this case select option shell/chain from the configuration menu screen

for modem  variables and set the option to chain.  Shell has advantage

over chain  because the main program  still  resides in memory.  Chain

directly loads the secondary program and must restart dndbbs from disk

after returning.  Therefore chaining is slower yet  takes less memory.

DNDBBS Documentation                                           Page 5

9.0 Configuration problems

--------------------------

The program does not read the configuration file:

Delete all the configuration files (files with the extension .cfg) and

start over  with dndcnfg.  Dndcnfg loads zero  into all  configuration

values. Load the default configuration values with control-d. You will

be prompted for the node number.  Press enter for no node or enter the

node number from 0 to 9 or A to Z.

The program does not recognize the configuration file:

You  have not written a  new configuration file to disk. Make sure you

use the control-w command before exiting dndcnfg. Or the configuration

file is from an old version. You will prompted for the node number for

the configuration file. Press enter or type the node number.

Installing a new version of dndbbs:

The configuration file structure may have changed from an old version.

Delete  all  old configuration files and write  new files with dndcnfg

and the control-w command.

The program displays errors and does not start:

Make  sure the  statement files= in config.sys is set  for 20 or more.

Place the  statement  files=20 in config.sys and  reboot the computer.

10.0 Setup instructions

-----------------------

The remaining contents assumes you are installing dndbbs into a fixed

disk system and your  computer is an MS-Dos compatible,  or  100% IBM

compatible.  Create a subdirectory  on your  fixed disk  and copy the

dndbbs  .zip files.  Separate the  compressed  .zip  files  with  the

PKWare  utility.

11.0 Configuring instructions

-----------------------------

This  section is   primarily text on  creating the configure file for

dndbbs.  The dndbbs will look for this file in the  default directory

it is started from.  If the configuration file is not  found then the

program  exits with an error message to  create a configuration file.

Start the configure program by typing  dndcnfg  in the dnd directory.

You will be presented  with a color screen  containing eight options:

F1 Logon variables                            F5 Command variables

F2 Processing variables                       F6 Modem variables

F3 File variables                             F7 Custom equations

F4 Rooms variables                            F8 Quit configure

DNDBBS Documentation                                           Page 6

11.0 Configuring instructions (cont.)

-----------------------------

And a selection on the bottom line:

Highlight Menu Choice W/ Cursor Keypad, Press Return To Select Option.

Control-D Load Default, Control-W Write Config, Control-R Read Config.

Select the option for  loading the configuration defaults (control-d),

loading a configuration file (control-r), or writing the configuration

file (control-w).

At this point you can select one of the eight options  with the cursor

keypad and  the enter key,  or by pressing the function key selection,

or with the first letter of  the option.  The screen will display  the

parameters for  the selected option.

The critical operating parameters  are the datapath to the .dat (data)

files, the datapath to the .doc (documentation) files, the datapath to

the temporary work directory, and the datapath to the door information

file. They are in the file variable screen. The modem variables screen

contains  the option to operate as a rbbs door,  the door  information

file path, and communications port.

When you have  entered  the critical options,  write the configuration

out to the dndbbs.cfg file. If you are using dndbbs as a door you must

use a node number.  The node number is used for the configuration file

name.  For node one the file created is dndbbs1.cfg,  for node two the

file created is dndbbs2.cfg, etc.  You will be  prompted for  the node

number,  or to press enter for no node number. The  configuration file

dndbbs.cfg is read by local mode (when the sysop runs the program from

dos or starts  as a standalone bbs).  When dndbbs operates  in the bbs

door mode, the program will search for the door information  file with

the specified node number.

12.0 Operating dndbbs

---------------------

After writing the configuration  file to disk dndbbs can start locally

with the command  dndbbs local  or  dndbbs bbs  for bbs mode. While in

bbs mode the  program  controls the modem and answers  incoming calls.

The opening screen status indicator displays the activity of the modem

during answering and connection. Dndbbs answers at speeds up to 38,400

baud. Your modem  must be 100% hayes compatible and support the entire

hayes command set. The opening screen shows the modem status, recycle,

intializaltion, reset, and off hook commands.

DNDBBS Documentation                                           Page 7

13.0 Modem communications

-------------------------

Sample of dndbbs to modem conversation bbs mode:

From dndbbs to modem:

atz          (reset command, status indicator: Reset ok).

atm0e0q0v1x1 (initialization command, status indicator: Init ok).

Now the opening screen status indicator displays Waiting for call.

From modem to dndbbs:

ring         (for s0=0, incoming call, status indicator: Ring,

dndbbs responds with ata, or),

connect 1200 (for s0=1, modem autoanswers, returns connect speed

after autoanswer, 1200, 4800, 9600, 19200, or 38400).

For 300 baud, modem returns connect then dndbbs waits and assumes 300.

If dndbbs  does not  detected a carrier signal  within 30 seconds then

the modem  disconnects and the program recycles.  Dndbbs can configure

to disallow 300  baud access.

14.0 Configuring for communications ports

-----------------------------------------

For sysops using multinode systems, networks or multitask applications

you may need to  configure dndbbs for  a  specific communication  port

number  besides the default of port 1. Dndcnfg modem variables  screen

allows you to set the port to any other number from 1 to 8. The fossil

device driver can access up to eight ports  on an AT.  In  order to do

this  you  must  modify  the  command   line on the  device=x00.sys in

config.sys.  Read x00  documentation for further  information. Example

of the x00 command line for 2 ports would be device=x00.sys 2.

Example contents of config.sys:

files=20

buffers=8

device=x00.sys 2

15.0 Door information and nodes

-------------------------------

To start dndbbs as a door put the node number after the dndbbs command

(for example: dndbbs 1).  To specify the node  parameter  from a batch

file  use the  node number for the  first replacement  parameter  (for

example: dndbbs %1).  Select the  option in dndcnfg for the door type.

The  option for dndbbs  to be an  rbbs door should be selected.  Other

door types need to be converted to an rbbs door file with a conversion

program like DoorMaster or similar program. The door file searched for

will be dorinfox.def with x replaced with the node number.  Select the

dndcnfg option for the pathname where the dos directory containing the

door file is to be found.  Dndcnfg will add the trailing  slash at the

end of the pathname if you forget.

DNDBBS Documentation                                           Page 8

16.0 Door batch files

---------------------

Here is a  sample batch file to call dndbbs from rbbs. This batch file

is named dnd.bat  and is called from rctty.bat  from rbbs.bat with the

command dnd 1.

cd\dnd

dndbbs %1

cd\rbbs

For futher rbbs batch file  programming  read the  rbbs documentation.

There  are many involved batch routines for files rbbs.bat, rctty.bat,

and rbbstm.bat.  Rbbs uses the file menu5 to look  for the  keyword to

start the batch file.  The menu5 file must have the batch file name in

uppercase  surrounded  by  spaces so  rbbs can find it.  Then the rbbs

attaches  .bat to  this filename and checks the disk for the filename.

For example,  menu5  could look  like  this:

+--------------------------------------------------------------------+

|                      Rbbs doors menu selections                    |

|                                                                    |

| Type in one of the following door options:                         |

|                                                                    |

| DND - The adventure system v2.9c-4!                                |

+--------------------------------------------------------------------+

17.0 System environment

-----------------------

An environment  string is a  system variable locally  known to dos and

the program dos executes.  Environment strings are placed into the dos

system variable space with the set command.  Dndbbs will recognize the

strings and  use them for disk access.  There are  five  set variables

recognized by dndbbs.  You can also place the dos environment varables

into the autoexec.bat file. For example:

set dndbbs=c:\dnd\

set dnddoc=c:\dnd\doc\

set dnddat=c:\dnd\data\

set dndtemp=d:\

set dnddoor=c:\rbbs\

The path command can  specify the directory  containing the dndbbs.exe

program.  By combining the environment strings  with the path command,

Dndbbs can be started from any drive.  The dos environment strings are

used instead of the configuration file pathnames.  Dndbbs will add the

trailing slash at  the end of  the set  statements.  If the  directory

containing dndbbs is  c:\rbbs\dnd\ then  the path statement  would be:

path=c:\rbbs\dnd\

DNDBBS Documentation                                           Page 9

18.0 Files used

---------------

A well  organized  dndbbs  system would  make full use of the datapath

options.  The disk  drive structure  could  look something  like this:

c:\                     c:\dnd\data\          c:\dnd\doc\

config.sys              rooms.dat             readme.1st

autoexec.bat            spells.dat            catalog.doc

ansi.sys                objects.dat           dndbbs.doc

vdisk.sys               monsters.dat          edit.doc

x00.sys                 monclass.dat          hint.doc

montalk.dat           summary.doc

c:\dnd\                 nonplyrs.dat          sysop.doec

brt71efr.exe            treasure.dat

dndbbsx.cfg             users.dat             c:\dnd\source\

dndbbs.exe              prelog.dat            *.bas

dndcnfg.exe             notice.dat            *.bat

dndedit.exe             welcome.dat           *.inc

dndutil.exe             logoff.dat            *.lib

edit.exe                dndedit.dat           bc.exe

logon.dat             link.exe

d:\ (ram disk)          help.dat

messwrk1.dat            dmhelp.dat

messwrk2.dat            messbase.dat

messwrk3.dat            *.new *.tbl

19.0 Compiling instructions

---------------------------

The dndbbs system is compiled  into many separate executable  files by

linking  together individual submodules.  Each submodule  has the same

include file,  dndbbs.inc,  which also  loads in several other include

files.  If  any of the .inc files are  changed then  all the  programs

must  be recompiled.  The command  line compiler distributed  with the

Microsoft Professional Development System 7.1  and  most recent linker

are  required to make  the dndbbs .exe files.

The  bc.exe  compiler command line  switches /e, /s, and /fs are  used

when  compiling the submodules.  After each submodule is compiled then

the linker makes the .exe file.  For example:

bc dndbbs/e/s/fs;

bc dndsub1/e/s/fs;

bc dndsub2/e/s/fs;

bc dndsub3/e/s/fs;

bc dndsub4/e/s/fs;

bc dndsub5/e/s/fs;

link dndbbs+dndsub1+dndsub2+dndsub3+dndsub4+dndsub5,,,qbx+dtfmter/e;

The compiler switches are:

/e  -- local error trapping

/s  -- compress strings

/fs -- use far strings

The linker switches are:

/e  -- compress .exe size

The  dndbbs source is packaged  with many  batch files to  autocompile

executable  programs.  Compile1.bat  compiles  the  dnd  main program,

compile2.bat  compiles dndedit, etc.  There  are several linking batch

files,  linkmain.bat, etc.  For example,  you could type compile4,  or

linkall,  or make,  to compile, link, or make programs.

There  are three  library files used  by the linker to compile dndbbs.

The .lib files used are qbx.lib, dtfmter.lib, and brt71efr.lib.  These

files are  installed by  PDS 7.1 during setup.  Link the files in with

link  (brt71efr.lib links in for default)  as the  library part of the

linking command.

DNDBBS Documentation                                           Page 10

20.0 Rbbs door file structure information

-----------------------------------------

Rbbs v17.4 door option, 13 lines ascii text:

dndrbbs           (bbs name)

smedley           (bbs sysop first name)

nowon             (bbs sysop last name)

com1              (comm port used, com0 for rbbs local sysop)

2400 baud,n,8,1   (baud rate and parameters)

1                (network type)

schmuckle         (sysop for local access, otherwise the user's

newbody            first name, and last name)

new york, ny.     (users city, state)

2                (graphics preference)

10               (the user level)

60               (time remaining)

-1               (fossil used)

========== END OF DOCUMENTATION ==========