PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 1

DESCRIPTION

-----------

PREPS is a command-line utility which allows you to send ANY commands

to your Epson compatible printer. PREPS has over 70 built-in commands,

and it also gives you the ability to create and store your own, using

binary, octal, decimal, hex, and text codes. Commands can be stacked,

and they can be sent to any device or file.

REQUIREMENTS

------------

PREPS will run on any PC compatible running DOS 3.0 or better, with an

Epson compatible printer (which, while not actually required, would

make the program a bit more useful... ;^). If you wish to add new

commands, an ASCII text editor will also be needed.

DISCLAIMER

----------

PREPS is distributed on an "AS IS" basis without warranty, expressed

or implied. Considerable testing effort has been expended, but the

user is advised to check the program's suitability before relying on

it. The user assumes full risk as to the results of using this

program. In no event shall the author be liable for any incidental or

consequential damages arising from the use, or inability to use this

program.

All product names mentioned in this document are either Copyright or

Trademark of respective owners.

REGISTRATION

------------

PREPS is freely distributable and fully functional, and may be used

for a 30-day trial period. If you plan to continue using PREPS, you

are obligated to pay the modest registration fee of $10, which

entitles you to unlimited free upgrades (to be distributed through

standard Shareware channels). Upgrades obtained directly ($5 shipping)

will have an individual serial number and no registration reminders.

All registration information is contained in the PREPS.REG file. If

that file is missing, register by sending your name, address, and a

check or money order for $10 ($15 for the registered version) in US

dollars to:

Copy Con Incorporated

PO Box 23255

Baltimore MD 21203-5255

MD residents add 5% sales tax. If ordering the registered version,

please specify 3.5" or 5.25" diskette.

Thank you for supporting Shareware.

PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 2

INSTALLATION

------------

To install, copy both PREPS.EXE and PREPS.DAT into your path. To add

new commands to PREPS, you will also need an ASCII text editor to edit

PREPS.DAT (see CREATING NEW COMMANDS).

OPERATION

---------

PREPS is activated from the command line, using the format

PREPS [/device] command[-] [command[-]]...

By default, PREPS sends commands to the LPT1 device; to specify

another, use the /device switch, specifying a valid device or

filename; for example,

PREPS /lpt2 reset

sends the reset command to the device connected to LPT2.

PREPS has over 70 built-in commands, and they can be sent individually

or combined with other commands. A command directly followed by a -

(minus sign) disables the command, where applicable; for example,

PREPS italic underline-

enables italics and disables underlining.

Commands do not need to be typed out in their entirety; only as much

of the command that distinguishes it from other commands is needed;

for example, when sending subscript and superscript commands,

"PREPS su" would send subscript because it alphabetically precedes

superscript; "PREPS sup" would send superscript. When using PREPS

inside batch files, it is a good idea to use the full command, as

future versions of PREPS may have more built-in commands that may be

similar to existing commands.

BUILT-IN COMMANDS

-----------------

Following are the commands that are built into PREPS, along with their

descriptions. (Some commands are not supported on some printers.)

Command           Description

1/6               Sets paper feed amount to 1/6 inch

1/8[-]            Sets paper feed amount to 1/8 inch

7/72[-]           Sets paper feed amount to 7/72 inch

BackSpace         Prints buffer and backspaces one space

Bell              Sounds buzzer

Black             Selects Black color (color printers only)

Blue[-]           Selects Blue color (color printers only)

BoldPS[-]         Selects Bold PS font

PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 3

BUILT-IN COMMANDS (continued)

-----------------------------

Command           Description

Cancel            Clears all data in the buffer

Center[-]         Enables automatic centering of print lines

Compressed[-]     Sets printing to 17 CPI

Courier[-]        Selects Courier font

CR                Prints buffer and starts next line at left margin

Delete            Deletes the last character stored in the buffer

Denmark[-]        Selects Denmark character set

DeSelect[-]       Disables printer from receiving data

Double[-]         Sets double printing

Draft[-]          Selects Draft font

Elite             Sets printing to 12 CPI

Emphasized[-]     Sets printing to twice the original dot density

England[-]        Selects England character set

Escape            Sends the escape ($1B) code

FF                Prints buffer and feeds paper to next TOF position

Flush             Prints all data in buffer

France[-]         Selects France character set

Germany[-]        Selects Germany character set

Graphic           Selects Standard Graphic character set

Green[-]          Selects Green color (color printers only)

HalfLF[-]         Prints buffer and feeds paper 1/12 inch

HalfSpeed[-]      Sets printing to half speed

IBM1              Selects IBM 1 character set

IBM2              Selects IBM 2 character set

Incremental[-]    Prints each character after it is entered

International[-]  Defines undefined codes as international characters

Italic[-]         Selects Italic font

Italy[-]          Selects Italy character set

Japan[-]          Selects Japan character set

Justification[-]  Sets automatic justification of print lines

LatinAmerica[-]   Selects Latin America character set

LeftAlign         Sets print alignment at the left margin

Legal[-]          Selects Legal character set

LF[-]             Prints buffer and executes a single line feed

Micron            Sets printing to 15 CPI

NLQ[-]            Selects NLQ font

Norway[-]         Selects Norway character set

Null              Sends the null ($00) code

Orange[-]         Selects Orange color (color printers only)

PaperOut[-]       Enables paper out detector

Pica              Sets printing to 10 CPI

Prestige[-]       Selects Prestige font

Proportional[-]   Sets proportional spacing between characters

Red[-]            Selects Red color (color printers only)

Reset             Resets printer to initial state

RightAlign[-]     Sets print alignment at the right margin

Roman[-]          Selects Roman font

SansSerif[-]      Selects Sans Serif font

Script[-]         Selects Script font

PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 4

BUILT-IN COMMANDS (continued)

-----------------------------

Command           Description

Select[-]         Enables printer to receive data

SheetFeed[-]      Enables the sheet feeder (if installed)

Single[-]         Sets single direction printing mode

Single1[-]        Sets single direction printing mode for one line

Spain1[-]         Selects Spain 1 character set

Spain2[-]         Selects Spain 2 character set

SubScript[-]      Selects subscript font

SuperScript[-]    Selects superscript font

Sweden[-]         Selects Sweden character set

Tab               Sends the horizontal tab ($09) code

Tall[-]           Sets double height character printing

Underline[-]      Sets continuous underlining of characters

USA               Selects USA character set

Violet[-]         Selects Violet color (color printers only)

VTab              Sends the vertical tab ($11) code

Wide[-]           Sets double width character printing

Wide1[-]          Sets double width character printing for one line

Yellow[-]         Selects Yellow color (color printers only)

CREATING NEW COMMANDS

---------------------

If you want to send a code that is not already built-in, you can

create your own using binary, octal, decimal, hex, and text codes.

Binary codes must be preceded with the % symbol, octal with the &

symbol, + for decimal, $ for hex, and ' for text; for example,

PREPS $1b 'P +1

sends 1B in hex, 'P' in text, and 1 in ASCII decimal. To send an

entire text string, you still use the ' symbol, but you must also

enclose the entire string in quotes; for example,

PREPS bold "'one two three" bold-

prints "one two three" (without the quotes) in bold.

Strings of codes can be assigned a command name and stored in a

separate data file called PREPS.DAT, which must be kept in the same

directory as PREPS.EXE. PREPS.DAT will be searched first in case you

wish to redefine the internal commands. The format of each line of the

data file is as follows:

command_name code [code]... [ - code [code]... ]

PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 5

CREATING NEW COMMANDS (continued)

---------------------------------

The command name must be left-justified (no left margin), and it

cannot contain any spaces. Following the name are the codes that

enable the command. If the command can be disabled, the enabling codes

are followed by a minus sign and the codes that disable the command.

The name, codes, and minus sign must be separated by spaces and/or

tabs. There is no limit to the number of commands, but there can only

be one command per line.

For example, let's create a new command called SkipPerf. The codes to

skip six lines at the bottom of each page are $1b, 'N, and +6. Using

an ASCII text editor, create PREPS.DAT with the first line reading:

SkipPerf $1b 'N +6

Now, if you type "PREPS skip", $1b, 'N, and +6 will be sent. What if

you want to disable perforation skip? First, edit PREPS.DAT to read:

SkipPerf $1b 'N +6 - $1b 'O

Now, if you type "PREPS skip-", $1b and 'O will be sent, which are the

codes to disable perforation skip.

EXAMPLES

--------

PREPS

will show a help screen and a list of built-in commands.

PREPS ff

sends a formfeed to the printer attached to LPT1.

PREPS /lpt2 ff

sends a formfeed to the printer attached to LPT2.

PREPS bold under- italic-

enables bold text, and disables underlining and italics.

PREPS esc 'W +1

sends the escape code, the letter W, and ASCII character 1.

PREPS "'line one" cr lf wide "'line two" wide- cr lf

prints two lines, the second one being of double width.

PREPS /hi.txt tall wide bold under "'hello there" cr lf

sends "hello there" in tall, wide, bold, underlined characters, to

the file HI.TXT, which can be printed by typing "copy hi.txt LPTx",

where x is the number of the printer port to be used.

PREPS V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 6

SUPPORT

-------

If you have questions, comments, suggestions, bug reports, et cetera,

feel free to contact me by one of the following methods:

1: By modem at The Programmer's Corner (301-596-7692 or 410-995-6873).

Send a message to RD Thomas in the General conference, and check

back in a couple of days for a reply.

2: By modem in the RIME Shareware conference (RD Thomas ->1520). If

you do not recieve a reply within a week, it is because there are

still some kinks to be worked out at this node; try contacting me

via methods 1 or 3.

3: By US Mail at the address given on the first page. If you would

like a reply, please include a self-addressed, stamped envelope.

Looking forward to hearing from you.

This document can be printed by typing "copy preps.doc LPTx" at the

DOS prompt, where x is the number of the printer port to be used.