[WS 1.5 in]

SPECTRAL  ANALYSIS  ON  A  PC

[WS 3.5 in]

David E. Hess

Fluid Flow Group

Process Measurements Division

Chemical Science and Technology Laboratory

National Institute of Standards and Technology

Email: HESS@ENH.NIST.GOV

Phone: (301) 975-5937

[WS 0.5 in]

[December 1991]

Abstract

These notes are intended to serve as a brief instruction manual for

engineers engaged in the spectral analysis of experimentally-derived random

data.  This manual is intended to accompany a software routine (Spectrum

Calculation Routine, V 1.0, David E. Hess) designed to compute amplitude

density, autospectral (power) density and cross-spectral density functions for

discrete, stationary random time series.  This program will run on any

PC-compatible desktop computer and accepts input data files obtained from

experiments in which analog data has been digitized by an analog-to-digital

converter.

A collection of techniques are described which are necessary for the

proper software implementation of spectral analysis procedures: data

transformation, mean value and linear trend removal, digital filtering, time

history tapering and data overlapping.  The various spectral density functions

are then defined and explained along with a means for the calculation of the

error associated with a particular estimate.  Several detailed examples are

included which serve to illustrate the methods described.  The manual also

contains a brief description of the considerations necessary when sampling the

data to be analyzed.  References to more complete sources of information are

provided as required.

Notice to Users

This program was developed for use with the ongoing Compliant Surface

Program in the Fluid Flow Group; this group is a part of the Process

Measurements Division at the National Institute of Standards and Technology.

With the evolution of the SPECTRUM routine to its present form, the author felt

that the program may perhaps be of some utility to others.  Therefore, SPECTRUM

is being released as freeware.  This program, the source code and accompanying

utilities may be freely copied and distributed, but may not be sold.  There is

no warranty of SPECTRUM's suitability for any purpose, nor any acceptance of

liability, express or implied.

Originally, the SPECTRUM routine was designed to run on any IBM-PC or

compatible computer using the Intel 80x86 series of microprocessors.  However,

the code was compiled using the Microsoft Fortran Optimizing Compiler, V 5.00,

and this compiler allows the selection of a variety of options at compile-time

which enhance the speed of operation of the program but limit its suitability

for use with certain computers.  The version contained on the diskette requires

a computer equipped with an 80286 or better microprocessor and an accompanying

80287 or better coprocessor.  A version of the routine which will run on

computers with an 8086 or 8088 processor, or a version which does not require a

numeric coprocessor can be obtained by contacting the author.  The program uses

about 450K bytes of available RAM, and requires that a hard disk or RAM disk be

available for storage of temporary files.  The amount of space to be reserved

depends upon the size of the data file to be processed, but can easily be as

much as several megabytes.

Microsoft has announced the release of V 5.1 of their Fortran compiler.

To this version the capability of porting programs to the Windows environment

has been added.  This is significant only in that it allows the use of the DOS

extender which is a part of the Windows environment.  Fortran programs written

for Windows are not limited by the DOS memory restriction of 640K, but instead

can access substantially larger amounts of available RAM at run-time.  This can

greatly increase the utility and speed of the SPECTRUM routine, and a version

incorporating these improvements may be available at a later date.

This program has been subjected to extensive testing and appears to be

free of any bugs; however, it is possible that not all situations have been

anticipated.  The author encourages any modifications to or customization of

the source code as required by the user, and welcomes any suggested

improvements or bug-fixes.  Please direct all correspondence to the address

given below.

National Institute of Standards

and Technology

Building 230   Room 105

Gaithersburg, MD 20899

Attn   : David E. Hess

Phone  : (301) 975-5937

Fax    : (301) 258-9201

Email  : HESS@ENH.NIST.GOV

[PB Odd Page]

Table of Contents

1 Overview and Parameters [LN String:"."]                                1

2 Input Data Files [LN String:"."]                                       3

2.1 Naming Convention [LN String:"."]                                 3

2.2 File Structure [LN String:"."]                                    3

2.2.1 File Header [LN String:"."]                                  3

2.2.2 Remainder of Data File [LN String:"."]                       4

2.2.3 Writing and reading the Data File [LN String:"."]            5

2.3 Multiple File Processing [LN String:"."]                          6

2.3.1 Consecutively numbered files [LN String:"."]                 6

2.3.2 Arbitrarily numbered files [LN String:"."]                   7

2.3.3 Files entered on command line [LN String:"."]                7

2.4 Coefficient Data Files [LN String:"."]                            8

2.4.1 Naming Convention [LN String:"."]                            8

2.4.2 Unformatted File Structure [LN String:"."]                   8

2.4.3 Formatted Data Files [LN String:"."]                         10

3 Data Preparation [LN String:"."]                                       11

3.1 Transformation [LN String:"."]                                    11

3.2 Mean Removal [LN String:"."]                                      13

3.3 Removal of Linear Trend [LN String:"."]                           14

3.4 Filtering [LN String:"."]                                         14

3.5 Tapering [LN String:"."]                                          18

4 Spectrum Computation [LN String:"."]                                   25

4.1 Fast Fourier Transform [LN String:"."]                            25

4.2 Autospectral (Power) Density [LN String:"."]                      27

4.3 Cross-spectral Density [LN String:"."]                            29

4.3.1 Calculation Procedure [LN String:"."]                        29

4.3.2 Adjustment of Phase Angle [LN String:"."]                    30

4.3.3 Coherence Function [LN String:"."]                           35

4.3.4 Error Estimates [LN String:"."]                              35

4.4 Amplitude Spectral Density [LN String:"."]                        37

5 Output Data Files [LN String:"."]                                      41

5.1 Naming Convention [LN String:"."]                                 41

5.2 File Structure [LN String:"."]                                    41

5.2.1 Power Spectrum Output Files [LN String:"."]                  41

5.2.2 Cross Spectrum Output Files [LN String:"."]                  42

5.2.3 Amplitude Spectrum Output Files [LN String:"."]              43

5.2.4 Error Estimate Output Files [LN String:"."]                  44

6 Run-time Considerations [LN String:"."]                                45

6.1 Configuration File [LN String:"."]                                45

6.2 Temporary Files [LN String:"."]                                   46

7 Examples [LN String:"."]                                               49

7.1 Colored Noise [LN String:"."]                                     49

7.2 Sinusoidal Input [LN String:"."]                                  56

7.3 Experimental Noise Input [LN String:"."]                          61

7.4 Sampling Considerations [LN String:"."]                           64

8 Utility Routines [LN String:"."]                                       67

8.1 MAKDAT routine [LN String:"."]                                    67

8.2 MAKCOEF routine [LN String:"."]                                   69

9 References [LN String:"."]                                             73

^S1 Overview and Parameters

This program is designed to compute amplitude density, autospectral

(power) density and cross-spectral density functions for discrete, stationary

random time series in the form

[EQUATION "center x sub n #3#3 = #3#3 x #3 (t sub 0 + n Delta t) #4#4 #R[for]

#3

where  [EQUATION "N", Position:In Text]  is the number of data points per

record.  Typically,  [EQUATION "M", Position:In Text]  such records are stored

in a data file, and each record is a contiguous segment of the stationary

process.  Most often, this data is in the form of 2-byte integers after having

been digitized by an analog to digital converter.  This routine reads this data

file and then transforms the integer data back into voltages or optionally into

some other desired quantity.  This transformation into a dimensional quantity

is by means of a polynomial of up to fourth degree.  The coefficients necessary

to complete the transformation are placed in coefficient data files which are

read by the routine.  Alternatively, the routine can read 4-byte floating-point

data which has already been transformed by some other program.  The input data

may consist of one or two separate time series which are to be analyzed.

After transformation, various cosmetic operations may be performed on the

data prior to the spectrum computation.  The mean value, which is calculated

during the transformation process, may be subtracted from the data; or instead,

a linear trend in the data may be removed using a least squares fit technique.

Recursive digital filters including lowpass, highpass, bandpass and bandstop

have been incorporated and may be applied prior to the calculation of the

spectra.  Finally, to reduce the leakage of power from other frequencies due to

the fact that the input time series are of finite length and are then

truncated, a tapering operation has been included.  A choice of four tapering

functions is supplied, and the appropriate scale factors are computed

automatically.  The use of tapering may introduce some additional variability

in the resulting spectral estimate; to counteract this tendency, a means for

overlapping the input data records is provided.  This option employs 50%

overlap to reduce overall error at the expense of computation time (which is

doubled); however, the use of overlapping requires that the data be contiguous

in time from record to record.

After computation of the selected spectral density function, the data are

stored to an ASCII output file which may then be imported into a plotting

program.  The routine is designed for multiple file processing; many input data

files may be processed sequentially if certain naming conventions are

followed.  The maximum number of files is currently set to IFMAX = 100 in order

to allocate storage for data.  Entering data file names on the command line

upon invocation of the program is also permitted and the maximum is set to

JFMAX = 5.  Each record of the input data file may contain no more than NMAX =

16384 points; note that in the case of two channels of data per record, each

channel may contain no more than NMAX / 2 points.  As described below, the

order of the recursive digital filters is set by the number of cascaded stages

desired; the maximum number of stages is currently placed at NSMAX = 20.  For

convenience, the base 10 logarithms of certain data in the output are computed;

to avoid taking the logarithm of zero, any number in the output data less than

[EQUATION "epsilon #3 = #3 1.0 #3 x #3 10 super [-20]", Position:In Text]  is

set equal to  [EQUATION "epsilon", Position:In Text] .  Any of these maximum

values may be altered by making a change in the appropriate PARAMETER statement

near the beginning of the code and then recompiling.

The speed of the routine is limited by the large number of I/O operations

that are required; reduce the number of points per record and/or the number of

records per file for increased speed.  Alternatively, specify in the

configuration file that the routine use a RAM disk for storage of temporary

files.  The size of the executable file is largely governed by the setting of

NMAX; since this parameter must be a power of two, the largest possible value

is 16384 for a DOS-based computer which results in an executable file size of

430K bytes.  The CHKDSK command must report that at least 450K bytes of memory

are present for the routine to execute for this value of NMAX.  When compiling

the source code using Microsoft Fortran V 5.00, at least 603K of RAM must be

available for full optimization of the code.

When the data to be processed by the spectrum routine are samples of a

continuous function of time at a single spatial location, then the calculated

spectral density functions are a function of frequency.  The user should keep

in mind that if the input data are samples of a continuous function of one

spatial direction at an instant of time (such as intensity data for horizontal

or vertical rows of pixels on a photograph from a CCD camera), then this

routine can be used to determine one-dimensional estimates of wavenumber

spectra.

^S2 Input Data Files

For any routine in which the processing of large amounts of data is

required, a rather rigid structure is necessarily imposed on the manner in

which the data is transferred into the program for manipulation.  The spectrum

routine is certainly no exception to this rule.  Furthermore, since the

spectrum routine has the ability to process data files in batches, additional

rules are required, such as naming conventions, to allow this feature to be

implemented.  Although the proliferation of rules can be a bit wearying, later

sections discuss some situations in which the rules can be bent or discarded.

2.1 Naming Convention

A convention for naming the input data files was chosen to facilitate

multiple file processing.  The selected method resulted from the ease with

which the file names could be generated by a few lines of code.  Each file name

consists of eight characters: the first character must be alphabetic (A-Z), the

next three characters are digits (0-9) and the remaining four characters are a

period followed by the letters DAT.  Note that the routine is not

case-sensitive, lower-case as well as upper-case characters may be entered;

however, all lower-case inputs are converted to upper-case internally.  The

routine automatically appends the suffix [.DAT] and therefore it should never

be entered upon input.  The data file must reside in the same directory as the

executable file unless specified otherwise in the configuration file (discussed

later).  Examples of acceptable data file names are: A001.DAT, z087.dat and

M418.DAT.  The latter filename would require that IFMAX

[EQUATION ">=", Position:In Text] 418.  Every rule has an exception; data file

names entered on the command line when calling the routine need not be so

restrictive as outlined above.  They must still consist of eight characters and

the first character must still be alphabetic (A-Z); however, the next three

characters can be any of the alphanumeric characters.  The blank character is

used to delimit the names on the command line and is therefore not a valid

character for a file name.  The remaining four characters must be a period

followed by the letters DAT.  Examples of acceptable filenames for files

entered on the command line are: TEST.DAT, x_in.dat and S1$$.DAT.  When

actually entering the names on the command line, only the first four characters

are entered and the program appends [.DAT].

2.2 File Structure

All input data files must be FORTRAN data files opened as sequential,

unformatted files.  The minimal amount of internal formatting allows the files

to be more compact which is a consideration when dealing with large quantities

of data.

2.2.1 File Header

The first record of the data file consists of a header containing four

quantities which characterize the data which follows. The first quantity is a

2-byte integer which gives the number of channels of data per record (1 or 2).

The next number is the record size in bytes and may be either a 2-byte or a

4-byte integer.  The record size is twice the number of points per record if

the data consists of 2-byte integer values, or the record size is four times

the number of points per record if the data consists of 4-byte real data.  The

third quantity is a 2-byte integer which gives the number of records of data

which follow, and the last quantity is the time interval in microseconds

between data points of the same channel expressed as a 2-byte integer.  Recall

that the largest positive signed number that may be stored as a 2-byte integer

is 32,767 and, for a 4-byte integer, the value is 2,147,483,647.

For example, suppose the data file consists of 100 records of one channel

of data saved as 2-byte integers and sampled at 10000 samples per second with

2048 data points per record.  The file header would contain the four values:

[EQUATION "center 1 #4#4 4096 #4#4 100 #4#4 100 #4#4"]

Now suppose that two channels of data were sampled with a time interval of

0.016383 seconds between each successive data point resulting in a time

interval of 0.032766 seconds between each successive data point of the same

channel.  Furthermore, 40 records of data were sampled with 8192 data points

per record such that each channel of data consisted of 4096 data points.

Finally, the data were transformed into desired quantities and saved as 4-byte

floating-point numbers.  The header for this data file should consist of the

following four numbers:

[EQUATION "center 2 #4#4 32768 #4#4 40 #4#4 32766"]

2.2.2 Remainder of Data File

Following the first record containing the file header is the actual data.

When the data comes directly from an analog to digital converter (12-bit, say),

then the values range from -2048 to 2047 (or 0 to 4095) and are appropriately

stored as 2-byte integers.  Alternatively, if some preprocessing is typically

performed on the data, the data may be stored as 4-byte floating-point

numbers.  As a result of the Fast Fourier Transform (FFT) algorithm used in

this routine, the number of data points per channel per record must be a power

of 2.  One or two separate time series may be stored per record.  If only one

channel of data is sampled then an even number of records following the file

header should be stored in the data file.  This is necessary because in the

case of one channel of data, two records are submitted to the FFT algorithm to

be transformed simultaneously as a time-saving measure.  If an odd number of

records is detected, then the program automatically appends an additional

record of data containing a copy of the data values of the previous record.

When two channels of data are sampled, the number of records may be even or

odd.  The method of storing the data values in each record follows the standard

procedure used by analog to digital converters:

[EQUATION "center #R[one] #4 #R[channel] #^ #R[:] #4#4 x sub 0 #3 x sub 1 #3 x

s

where the notation is as defined above.  This convention must be followed even

if the data are processed and then stored as floating-point numbers.  When two

channels of data are sampled and then stored to a data file, the values for

each channel of data should be of the same data type: either 2-byte integers or

4-byte floating-point numbers.  If the user is unfamiliar with the task of

creating unformatted FORTRAN data files, then the user may instead create these

files in an ASCII file format.  A utility MAKDAT which is included on the

diskette can then be used to write the file header and transform the ASCII data

files into the unformatted FORTRAN data files required by the spectrum

routine.  See the section on utility files.

2.2.3 Writing and reading the Data File

This section gives a few lines of code which are representative of the

manner in which these data files are written and read.  When writing, the file

is first opened, the file header is written, the data is written and then the

file is closed.

INTEGER*2 ICHANS, IRSIZE, NUMREC, IDELTMS

INTEGER*2 NDATA (NMAX)

OPEN (2, FILE = 'A001.DAT', STATUS = 'UNKNOWN',

ACCESS = 'SEQUENTIAL', FORM = 'UNFORMATTED')

WRITE (2) ICHANS, IRSIZE, NUMREC, IDELTMS

DO J = 1, NUMREC

... put data record into NDATA array ...

WRITE (2) (NDATA (I), I = 1, N)

... get next record of data ...

ENDDO

CLOSE (2, STATUS = 'KEEP')

The same procedure is followed when the data file is opened and read by

the spectrum routine.  The file is opened, the file header is read, the data is

read and then the routine is closed.

INTEGER*2 ICHANS, IRSIZE, NUMREC, IDELTMS

INTEGER*2 NDATA (NMAX)

OPEN (2, FILE = 'A001.DAT', STATUS = 'OLD',

ACCESS = 'SEQUENTIAL', FORM = 'UNFORMATTED')

READ (2) ICHANS, IRSIZE, NUMREC, IDELTMS

N = IRSIZE/2

DELT = FLOAT (IDELTMS) / 1000000.0

DO J = 1, NUMREC

READ (2) (NDATA (I), I = 1, N)

... process this record of data ...

ENDDO

CLOSE (2, STATUS = 'KEEP')

2.3 Multiple File Processing

This routine is capable of sequentially processing many files in a batch.

The program will handle up to IFMAX files which are consecutively numbered or

which are not consecutively numbered but have an arbitrary numbering

arrangement.  All data files in the batch must start with the same first

letter.  The letter and number combination in the file name are used to

associate each data file to a corresponding set of coefficients (if needed) in

an accompanying coefficient data file; this is the reason for the rather rigid

naming convention.  Alternatively, up to JFMAX files may be entered on the

command line when calling the program, and these will be processed

sequentially.  These files need not have the same first letter; however, these

data files also cannot use coefficient data files.  This is further clarified

below.  All data files must conform to the naming convention described above.

The selection of options to perform various operations on the data will then be

applied to all files in the batch.

2.3.1 Consecutively numbered files

If it is desired to process several files in a consecutive order, then

this option is selected. The routine will ask for the beginning and ending file

numbers and then the first letter in the file names. As an example, suppose it

was desired to process files C031.DAT through C094.DAT, then the user would

respond as follows:

spectrum

Process files (C)onsecutively or in an (A)rbitrary order : c

Enter starting & ending data file # : 31 94  (or 31,94)

Enter first letter of data file : c

2.3.2 Arbitrarily numbered files

Perhaps it is not desirable to process all of the files indicated above,

but only selected ones from the set.  If it were necessary to process C031.DAT,

C042.DAT, C067.DAT and C085.DAT, then the user would respond as follows.

SPECTRUM

Process files (C)onsecutively or in an (A)rbitrary order : A

Enter number of files to process : 4

Enter last digits of file   1 : 31

Enter last digits of file   2 : 42

Enter last digits of file   3 : 67

Enter last digits of file   4 : 85

Files are :

31 42 67 85        (routine echoes file numbers selected)

Enter first letter of data file : C

2.3.3 Files entered on command line

This option was added as a means of processing a few files without having

to answer all of the prompts above and to allow a bit more freedom in the

selection of file names.  Suppose it was desired to process TSTX.DAT, X_IN.DAT

and WOW!.DAT, then the user would enter the following command line.

SPECTRUM TSTX X_IN WOW!

By relaxing the naming convention, it is no longer possible to easily associate

a coefficient data file with the correct input data file.  Therefore, if the

user desires to transform the integer data in each of the input files to

voltages only, or if the input data in each file already consists of

transformed floating-point numbers (as a result of some other program), then

coefficient data files are not necessary and this method for quickly processing

a small number of files may be used.  If the integer data in each input file

must be transformed into a desired floating-point quantity using a polynomial

transformation prior to the spectral calculation, then the program must be

initiated using the methods described in sections 2.3.1 or 2.3.2.  Further

discussion on the manner in which coefficient data files are associated with

the respective input data files appears below.

2.4 Coefficient Data Files

These data files may be sequential unformatted or sequential formatted

FORTRAN data files.  They contain the constants which are used as the

coefficients of the polynomial that transforms the input data from voltages to

some other desired quantity.

2.4.1 Naming Convention

The coefficient data file names consist of eight characters.  The first

letter must be alphabetic (A-Z) and must be the same letter as the input data

file (or series of input data files) with which it is associated.  The spectrum

routine assigns the remaining seven characters of the name as CON.DAT (if the

file is unformatted) or CON.PRN (if the file is formatted).  If the input data

file D061.DAT were to be transformed from voltages into velocities, then the

coefficients of the polynomial would be found in the coefficient data file

named DCON.DAT (if unformatted) or DCON.PRN (if formatted), and this file must

reside in the same directory as that of the executable routine and the input

data file unless specified otherwise in the configuration file (discussed

later).  If D061.DAT contained two channels of data, then the coefficients

needed to transform the data for the second channel would be found in the

coefficient data file named DCON2.DAT (if unformatted) or DCON2.PRN (if

formatted).  Again, the routine requests that the user enter the first letter

only and the remaining eight characters are automatically assigned by the

routine.

2.4.2 Unformatted File Structure

These coefficient data files are accessed only if the input data are

2-byte integers and the user desires to convert the integers first into

voltages and then into another quantity.  The voltage transformation factor is

not included in these files; instead, it is entered into the routine at the

beginning and therefore applies to all files processed.  If a series of files

are to be processed, the coefficients for the transformation from voltages into

the other quantity may not be the same for all files in the batch, and this

situation is addressed as follows.  The conversion from a voltage into a

velocity, say, for each data point is carried out using

[EQUATION "center u sub n #3#3 = #3#3 b sub 0 #3 + #3 b sub 1 v sub n #3 + #3 b

where  [EQUATION "v sub n", Position:In Text]  is the nth data point expressed

as a voltage,

[EQUATION "b sub 0 #1 , #3 b sub 1 #1 , ... , b sub 4", Position:In Text]  are

the coefficients of the velocity transformation and

[EQUATION "u sub n", Position:In Text]  is the nth data point which has been

converted to a velocity.  The five coefficients of the transformation remain

the same for each data point in each record of the file, but are permitted to

vary from file to file.  These constants are stored in a two-dimensional array

of 4-byte floating-point numbers.  The first index of the array is the number

of the file to which the coefficients are to apply (1 to IFMAX), and the second

index is the number of the coefficient (1=

[EQUATION "b sub 0", Position:In Text] to 5=

[EQUATION "b sub 4", Position:In Text] ).  The first record of the unformatted

coefficient data file is a file header containing two 2-byte integer

quantities: the first is the number of sets of coefficients to read, and the

second is the starting file number to which these coefficients should be

associated.  The remainder of the data file consists of one large record

containing all of the coefficients.  If the user wishes to first create these

coefficient data files in an ASCII file format (without the header), the

utility MAKCOEF has been included on the diskette which will convert these

ASCII files into the unformatted FORTRAN files needed by the spectrum routine.

See the section on utility routines.  An example of the code used to write and

read a sequential unformatted coefficient data file follows.

INTEGER*2 NUMSETS, NUMCON, NSTART

REAL*4 CONST (IFMAX, 5)

NUMCON = 5

OPEN (2, FILE = 'DCON.DAT', STATUS = 'UNKNOWN',

ACCESS = 'SEQUENTIAL', FORM = 'UNFORMATTED')

WRITE (2) NUMSETS, NSTART

WRITE (2) ((CONST (I,J), J = 1, NUMCON), I = 1, NUMSETS)

CLOSE (2, STATUS = 'KEEP')

The spectrum routine then reads the coefficient data file using code of the

following form.

INTEGER*2 NUMSETS, NUMCON, NSTART

REAL*4 CONST (IFMAX, 5)

OPEN (2, FILE = 'DCON.DAT', STATUS = 'OLD',

ACCESS = 'SEQUENTIAL', FORM = 'UNFORMATTED')

READ (2) NUMSETS, NSTART

READ (2) ((CONST (NSTART + I - 1, J), J = 1, NUMCON), I = 1, NUMSETS)

CLOSE (2, STATUS = 'KEEP')

The coefficients

[EQUATION "b sub 0, #1 b sub 1, #1 ... #1 , #1 b sub 4", Position:In Text]  for

the data file D061.DAT are then given by CONST (61,1), CONST (61,2), ... ,

CONST (61,5).

2.4.3 Formatted Data Files

It is not always convenient to store coefficients in sequential

unformatted data files.  This option was included to allow a quick way of

storing coefficients.  A sequential formatted file is exactly that created by

any simple text editor.  To create a sequential formatted coefficient data file

for the input data file D061.DAT, enter a text editor or word processor

(capable of creating ASCII files) and give the file the name DCON.PRN.  Next,

enter the five coefficients,

[EQUATION "b sub 0, #1 b sub 1, #1 ... #1 , #1 b sub 4", Position:In Text], one

per line and then save the file.  If you display the file to the screen, it

should look something like the following.  Note that the numbers should all be

floating-point numbers.

3.14159265359            (

[EQUATION "b sub 0", Position:In Text])

0.0                      (

[EQUATION "b sub 1", Position:In Text])

1.012345E-03             (

[EQUATION "b sub 2", Position:In Text])

0.1                      (

[EQUATION "b sub 3", Position:In Text])

2.3E-05                  (

[EQUATION "b sub 4", Position:In Text])

An example of the code that the spectrum routine will use to read in this data

file follows.

REAL*4 B (5), CONST (IFMAX, 5)

OPEN (2, FILE = 'DCON.PRN', STATUS = 'OLD')

READ (2, *) (B (I), I = 1, 5)

CLOSE (2, STATUS = 'KEEP')

DO I = 1, IFMAX

DO J = 1, 5

CONST (I, J) = B (J)

ENDDO

ENDDO

As is seen from the above code, the routine assigns the five constants to apply

to all of the IFMAX different input data files that might be read.  If a

different set of constants is required for each data file, then the files must

be processed one at a time, or sequential unformatted data files as described

above must be used.  If an input data file (D061.DAT, say) contains two

channels of information, then a second sequential formatted data file with the

name DCON2.PRN should be prepared as described above.

^S^^Data Preparation

This section deals with a sequence of cosmetic operations which may be

applied to the data prior to performing spectral calculations.  The first

section discusses the transformation of the input data from integer values into

voltages and then optionally into some other desired dimensional quantity (for

example, velocities).  If the input data consists of floating-point values,

then the transformation operation is skipped.  If desired, the mean value may

be removed from each record of the input data, or a linear trend resulting,

perhaps, from instrumentation drift may be removed from each record.  Unwanted

frequencies in the input data can be removed using the recursive digital

filtering routines discussed below.  Finally, a tapering operation which may be

coupled with a 50% overlap of the input data records is included to eliminate

the discontinuities found at the beginning and end of these records resulting

from the fact that only a finite amount of data can be sampled.  Currently, the

user has a choice of four different window functions.  If the input consists of

two channels of data, then the first three operations: transformation, mean or

trend removal and filtering may be different for each of the two input

channels.  The choice of whether or not to employ tapering as well as the

choice of window function, however, applies to both input channels.

3.1 Transformation

Experimental data sampled by means of an analog-to-digital (A/D) converter

typically results in data files containing an array of two-byte integer

values.  This is due to the fact that a two-byte integer is capable of

representing numbers in a range from -32768 to 32767.  This range is sufficient

for 12, 14 and 16-bit converters and is used to approximate the magnitude of an

analog voltage at a given instant of time.  This voltage is constrained to lie

within a specified input voltage range which may or may not be programmable and

which is particular to the A/D converter used.  Before proceeding to the

spectral calculation, the integer input data must be transformed back into

voltages using the scale factor appropriate for the input voltage range of the

A/D converter used.

During development of this routine, a 12-bit A/D converter was used which

was capable of digitizing three different bipolar voltage ranges: -1V to 1V,

-5V to 5V and -10V to 10V.  The use of 12 bits allows integer values ranging

from -2048 to 2047 for a total of 4096 digital levels to be used to quantize

each of the above input voltage ranges.  The appropriate scale factors for

conversion of the integer data back into voltages are given by

[EQUATION "center [2 #3 V] over [4096 #3 units] #4 , #4 [10 #3 V] over [4096 #3

respectively.  The user may choose from one of these scale factors or may enter

an alternative scale factor from the keyboard.  The routine first prompts the

user with:

Transform into (V)oltages or into (O)ther quantity

or read (R)eal data which is already transformed :

If the input data consists of floating-point values, then the letter R should

be entered, and the routine skips the rest of the transformation operation.

For integer input data, the user responds with V for transformation into

voltages, or O for voltage transformation followed by an additional

transformation into a second dimensional quantity; the following prompt is then

displayed:

Enter VOFST 1) 10/4096, 2) 20/4096, 3) 2/4096 or 4) other :

To this prompt, the user responds with a digit (1-4) indicating the desired

scale factor for voltage transformation.  Selection of item (4) leads to the

prompt:

Enter VOFST :

and, at this point, the user enters a floating-point number representing the

desired scale factor.

The input data will now be converted back into voltages using the scale

factor specified above.  Since the analog voltage sampled by the A/D converter

often comes from a transducer, which measures some dimensional quantity and

then converts it to a voltage signal, provision for transforming the digitized

input voltages back into the original transduced quantity is included.

Knowledge of calibration data for the particular transducer is required for

this transformation, and the calibration curve may be linear, quadratic,

exponential, etc.  It was impossible during development of the code to

anticipate all forms that the calibration curve might take; therefore, a

polynomial fit was decided upon in hopes that it would satisfy the calibration

requirements of most users.  The polynomial may consist of terms up to fourth

degree; however, lower degree polynomial representations may be used by setting

the coefficient for the appropriate term to zero.  The conversion from a

voltage into a velocity, say, for each data point is carried out using

[EQUATION "center u sub n #3#3 = #3#3 b sub 0 #3 + #3 b sub 1 v sub n #3 + #3 b

where  [EQUATION "b sub 0", Position:In Text]  through

[EQUATION "b sub 4", Position:In Text]  are the five calibration coefficients,

[EQUATION "v sub n", Position:In Text]  is the

[EQUATION "nth", Position:In Text]  data point of the current record which has

just been transformed into a velocity as described above and

[EQUATION "u sub n", Position:In Text]  is the resulting

[EQUATION "nth", Position:In Text]  velocity value.  The coefficients necessary

to complete the transformation must be included in a suitable format in

coefficient data files which are then read by the routine; instructions for the

creation of these coefficient data files are contained in section 2.4.

An alternative to the transformation operations described herein is also

available.  The user may transform the original integer data from the A/D

converter by means of his own program into floating-point data which can then

be read by this routine.  The floating-point input data files must conform to

the structure set forth in section 2.  The routine then skips the

transformations described above and proceeds to the calculation of the mean and

root-mean-square (rms) values of each data record and averaged values for the

file.

Summarizing then, the spectrum routine begins by reading in an input data

file.  If this file consists of integers, then these integers are transformed

to voltages using a voltage transformation factor (VOFST) selected by the

user.  Optionally, these voltages can be further transformed into some other

dimensional quantity.  If the input data file contains floating-point numbers,

then the routine assumes that any transformation of the input data has already

been applied by the user.  As the input data is read in by the spectrum

routine, record by record, the mean and rms values of each record are

calculated and displayed on the screen at run-time.  After all records of the

input file have been read, the mean and rms values for each record are averaged

to obtain values which are representative of the entire file.  These averaged

mean and rms quantities are also displayed on the screen at run-time.  The

spectrum routine then proceeds to the various data preparation operations

discussed in the next section.  Note that the averaged mean and rms values for

the file are calculated a second time later on in the program as a result of

the spectral computation and are also displayed on the screen.  Theoretically,

these two sets of values should be identical and serve as a check on the

accuracy of the spectral computation.  However, if any of the data preparation

operations to be discussed next are applied to the data to improve the accuracy

of the spectral computation, the two sets of mean and rms values may not

exactly match.  This is due to the fact that the data preparation operations

change the input data.  The first set of mean and rms values are calculated

prior to the data preparation operations, and the second set are computed from

spectral density calculations which occur subsequent to the data preparation

procedures.

3.2 Mean Removal

For calculation of spectral quantities, it is common to remove the mean

value from the input data.  Both this operation and the trend removal operation

described in the next section may be used for the removal of the mean from the

sample data.  Trend removal is not always desirable; for that reason a separate

operation to remove the mean is needed and is described here.  The mean value

for each record of data in the input file is calculated from

[EQUATION "center x bar #3#3 = #3#3 1 over N #3 sum below [n=1] above N #3 x

sub

where  [EQUATION "x sub n", Position:In Text]  is defined as in equation 1.1;

for convenience, and to reduce the algebra in the derivations in this section,

[EQUATION "n", Position:In Text]  is defined such that

[EQUATION "n #3#3 = #3#3 1,2, ... , N ", Position:In Text] , and the initial

point of the record  [EQUATION "t sub 0", Position:In Text]  is set to zero.

If  [EQUATION "y sub n", Position:In Text]  is used to represent the data with

zero mean, then this quantity is computed from

[EQUATION "center y sub n #3#3 = #3#3 x sub n #3#3 - #3#3 x bar right #R(3.4)"]

[PAGE BREAK]

3.3 Removal of Linear Trend

As recommended by Bendat and Piersol (1986), a linear trend removal

operation has been included in this routine.  This operation is designed to

remove low frequency components in the data with a period longer than the

record length  [EQUATION "T #3 = #3 N Delta t", Position:In Text] .  These low

frequency trends can significantly distort spectral data; however, trend

removal should only be performed if instrumentation drift or other sources of

trends are expected.  Use of trend removal when no trends are present can

introduce small contributions to the spectral density at frequencies on the

order of the reciprocal of the record length.

This operation is accomplished by fitting a straight line through the data

using the least squares fit method.  The values calculated from this fit are

then subtracted from the original data to yield data with zero mean and with no

frequency components smaller than  [EQUATION "1 #1 / #1 T", Position:In Text]

where  [EQUATION "T", Position:In Text]  is the record length.  Let

[EQUATION "x tilde", Position:In Text]  be a first degree polynomial fit to the

original data  [EQUATION "x sub n", Position:In Text]  (defined as in equation

1.1) where  [EQUATION "x tilde", Position:In Text]  is given by

[EQUATION "center x tilde sub n #3#3 = #3#3 b sub 0 #3#3 + #3#3 b sub 1 #1 (n

De

and with  [EQUATION "b sub 0", Position:In Text]  and

[EQUATION "b sub 1", Position:In Text]  defined as

[EQUATION "center #^ b sub 0 #3#3 = #3#3 [2 #1 (2N #3 + #3 1) #1 sum below

[n=1]

Let  [EQUATION "y sub n", Position:In Text]  represent the detrended data;

these data are obtained from the following difference

[EQUATION "center y sub n #3#3 = #3#3 x sub n #3#3 - #3#3 x tilde sub n right

#R

3.4 Filtering

The filtering operations incorporated in this routine were added to allow

the user to pass only desired frequencies in the input data on to the spectral

calculation.  Four recursive (infinite impulse response) Butterworth digital

filters for filtering in the time domain have been included for this purpose:

lowpass, highpass, bandpass and bandstop (notch).  A short discussion of

digital filtering in the time domain including a summary of the particular

filters used in this routine and instructions for their use follows.  Note

however that filtering may be applied not only in the time domain but also in

the frequency domain.  When filtering in the frequency domain, the input data

record is Fast Fourier Transformed, then the FFT output is multiplied by a

filter function, [EQUATION "H(f)", Position:In Text] ; if required, an inverse

FFT could be performed to get the filtered data back into the time domain.

Frequency domain filtering may perhaps be implemented in a future version of

this program.

Consider a set of discrete input values

[EQUATION "x sub n", Position:In Text]  defined as in equation 1.1.  Digital

filtering involves a computing algorithm in which a set of filtered values

[EQUATION "y sub n", Position:In Text]  is produced as a result of operating on

the inputs  [EQUATION "x sub n", Position:In Text] .  For the filters included

in this routine, this is a linear operation.  Digital filters can generally be

categorized as recursive or nonrecursive; if the formula for

[EQUATION "y sub n", Position:In Text]  contains only input terms

[EQUATION "x sub n", Position:In Text] , then the filter is termed

nonrecursive.  The general form of the linear nonrecursive algorithm is given

by

[EQUATION "center y sub n #3#3 = #3#3 sum below [m #3 = #3 - #1 M] above M b

sub

where  [EQUATION "b sub m", Position:In Text]  are the coefficients of the

transformation.  For values of  [EQUATION "m #3 < #3 0", Position:In Text] ,

the above algorithm requires future values of the signal

[EQUATION "x(t)", Position:In Text] ; this can cause a realizibility problem in

analog systems, but can be accomplished in digital systems where, in many

cases, the future values of  [EQUATION "x(t)", Position:In Text]  can be stored

in advance.  A recursive filter is one in which previously filtered values such

as  [EQUATION "y sub [n-1]", Position:In Text] ,

[EQUATION "y sub [n-2]", Position:In Text] , etc. appear explicitly in the

algorithm for  [EQUATION "y sub n", Position:In Text] .  For recursive filters,

the above equation is modified to include past values of the filtered output

[EQUATION "y(t)", Position:In Text]  as follows:

[EQUATION "center y sub n #3#3 = #3#3 sum below [m #3 = #3 - #1 M] above M b

sub

The transfer function  [EQUATION "H(f)", Position:In Text]  for the filter is

related to the coefficients  [EQUATION "a sub m", Position:In Text]  and

[EQUATION "b sub m", Position:In Text]  by

[EQUATION "center H(f) #3#3 = #3#3 [sum below [m #3 = #3 - #1 M] above M b sub

m

This equation tells how to determine  [EQUATION "H(f)", Position:In Text]  from

the coefficients  [EQUATION "a sub m", Position:In Text]  and

[EQUATION "b sub m", Position:In Text] .  However, filter design requires the

inverse problem: how to obtain a suitable set of coefficients based upon

knowledge of a desired filter transfer function.  A variety of techniques exist

to solve this problem, and the interested reader is referred to Stearns (1975)

for a lengthy discussion of the matter.  The transfer function discussed above

is more commonly expressed in terms of the variable

[EQUATION "z", Position:In Text]  where

[EQUATION "center z #3#3 == #3#3 e super [i 2 pi f Delta t] right #R(3.12)"]

For recursive filters,  [EQUATION "H(z)", Position:In Text]  is a rational

function of  [EQUATION "z", Position:In Text] .  It seems that rational

functions are particular good at fitting functions with sharp edges; most

filter functions are in this category.  The filters contained in this routine

are modified versions of FORTRAN subroutines which originally appeared in

Stearns (1975).  The order of the filter is determined by the number of

cascaded stages that are used; a filter of order

[EQUATION "P", Position:In Text]  will require

[EQUATION "P #1 / #1 2", Position:In Text]  cascaded stages.  The transfer

functions,  [EQUATION "H sub k #1 (z)", Position:In Text] , for the

[EQUATION "kth", Position:In Text]  stage for each filter, and the recursive

algorithms required to implement those transfer functions are given below.

[EQUATION "center #^ H sub k #1 (z) #3#3 = #3#3 [A sub k #1 (z super 2 #3 + #3

2

Note that the quantity  [EQUATION "K", Position:In Text]  in equation 3.16

above is a constant which is the same for all stages of the notch filter.  The

other coefficients,  [EQUATION "A sub k", Position:In Text]  through

[EQUATION "E sub k", Position:In Text] , are a more compact notation for the

[EQUATION "a sub m", Position:In Text]  and

[EQUATION "b sub m", Position:In Text]  values for each stage

[EQUATION "k", Position:In Text] .  In general,

[EQUATION "A sub k", Position:In Text]  through

[EQUATION "E sub k", Position:In Text]  change from stage to stage and are

evaluated from complicated expressions which the interested reader may find in

the source code.

To use these filters, the user selects the desired filter type and enters

the critical (-3 dB) frequency for the low and highpass filters.  For the

bandpass and bandstop filters, two critical frequencies must be specified.  The

number of filter sections to cascade must also be specified.  The order of the

filter is twice the number of cascaded sections.  As the order of the filter is

increased, the transfer function for the filter becomes sharper and more

closely approximates the ideal; however, the amount of computation time is also

significantly increased.  A good starting point may be to use five cascaded

sections (tenth order); this is a reasonable trade-off between accuracy and

computational efficiency.  These filters work well when the input data is not

deterministic; however, unlike nonrecursive filters, recursive filters can

become unstable and produce an output which grows exponentially.  This has been

observed in a few rare instances during testing of this program and, so far,

has been found to occur only when the input data has been a perfect

computer-generated sinusoid.  The user should be aware of this limitation.

[PAGE BREAK]

3.5 Tapering

The tapering option has been added to the routine as a grooming operation

in order to improve the resulting spectral density estimates.  A wide variety

of window functions may be found in the literature including extended

discussions concerning their performance characteristics.  See, for example,

Bendat and Piersol (1986) and Press, et al. (1986).  Rather than repeat these

long discussions here, a brief summary of the need for tapering will be

followed by a description of the window functions available for use with this

routine.

As discussed earlier in regard to equation 1.1, the data processed by this

routine is in the form of a finite set of samples

[EQUATION "x sub n", Position:In Text]  of the continuous signal

[EQUATION "x(t)", Position:In Text]  which exists for a time

[EQUATION "T #3 = #3 N #1 Delta t", Position:In Text] .  Now, suppose that

[EQUATION "x(t)", Position:In Text]  is a finite portion of a continuous

signal  [EQUATION "v(t)", Position:In Text]  which is of unlimited extent in

time.  Then,  [EQUATION "x(t)", Position:In Text]  can be considered to be the

product of  [EQUATION "v(t)", Position:In Text]  and a rectangular window

function  [EQUATION "w(t)", Position:In Text]

[EQUATION "center x(t) #3#3 = #3#3 w(t) #3 v(t) right #R(3.17)"]

where

[EQUATION "center w(t) #3#3 = #3#3 #( { stack left [[ 1 #4#4 0 <= t <= T] [ 0

#4

A plot of the rectangular (square) window function

[EQUATION "w(t)", Position:In Text]  is shown in figure 3.1 where, for

convenience,  [EQUATION "T", Position:In Text]  is chosen to equal one.  The

computation of spectral density estimates for

[EQUATION "x(t)", Position:In Text]  involves the calculation of the Fourier

transform of  [EQUATION "x(t)", Position:In Text]  which is equivalent to the

Fourier transform of the product of  [EQUATION "w(t)", Position:In Text]  and

[EQUATION "v(t)", Position:In Text] .  A theorem from Fourier analysis relates

the Fourier transform of a product of two functions in time to the convolution

of the transforms of these functions.  In other words, the following is a

Fourier transform pair:

[EQUATION "center w(t) #3 v(t) #3#3 <==> #3#3 W(f) #3 * #3 V(f) right

#R(3.19)"]

where

[EQUATION "center W(f) #3 * #3 V(f) #3#3 = #3#3 int sub [#3 - infinity] super

[i

The spectral density of interest would be exact if the transform

[EQUATION "V(f)", Position:In Text]  could be calculated.  Instead,

[EQUATION "X(f) #3 = #3 W(f) #3 * #3 V(f)", Position:In Text]  is the quantity

which is, in fact, calculated.  This problem then is seen to be directly

related to the finite length of the data to be transformed and to the fact that

the data is sharply truncated at each end of the time record.  A plot of the

modulus of the Fourier transform of the rectangular window function,

[EQUATION "W(f)", Position:In Text] , may be found in figure 3.2 and is defined

as[PAGE BREAK]

[WHITESPACE 0.25 in, Place All on Next Page]

[PICTURE FIG31.CGM, Scale:0.800]

Figure 3.1  Window functions available for tapering the data

[PICTURE FIG32.CGM, Scale:0.800]

Figure 3.2  Normalized amplitude of Fourier transform of window

functions[PAGE BREAK]

[EQUATION "center W(f) #3#3 = #3#3 [sin #1 pi f T] over [pi f] #3#3 e super [-i

Again, for plotting convenience,  [EQUATION "T", Position:In Text]  is chosen

to equal one.  This function is characterized by a large main lobe with smaller

side lobes which slowly decrease in size.  The ideal window function would be a

delta function; that is, an infinitely thin main lobe with no side lobes at

all.  As  [EQUATION "W(f)", Position:In Text]  is convolved with

[EQUATION "V(f)", Position:In Text] , information at frequencies well separated

from the main lobe can leak into the calculation and significantly distort the

resulting spectra.  The large amplitude sidelobes need to be reduced to

suppress this problem.

This is accomplished by multiplying the original data by a window function

which tapers the time history at the beginning and end of the record to remove

the discontinuities there.  The tapering functions included in this routine are

the most commonly used ones, and the choice of a particular function requires a

trade-off between the thickness of the main lobe (resolving power) and the size

of the side lobes.  The following window functions (the subscripts merely

indicate the name of the window) may be selected:

[EQUATION "center #^ w sub h (t) #3#3 = #3#3 #( { stack left [[ 1 over 2 #3 (1

#

[PAGE BREAK]

[EQUATION "center #^ w sub p (t) #3#3 = #3#3 #( { stack left [[ #4 1 #3 - #3 (|

These functions are plotted in figure 3.1.  The moduli of the Fourier

transforms for each of the above windows are plotted in figure 3.2 and are

normalized by their value at  [EQUATION "f #3 = #3 0", Position:In Text] .  The

fact that these transforms do not equal one for

[EQUATION "f #3 = #3 0", Position:In Text]  implies that the resulting spectral

estimates would be reduced in magnitude upon application of one of these

windows.  To avoid this problem, the appropriate scale factor is computed by

the routine when any of these windows are selected and automatically applied to

the data before computing the spectral estimate in order to preserve the

correct amplitudes.  The Fourier transform for each of the above windows is

given below.

[EQUATION "center W sub h (f,T) #3#3 = #3#3 {#3#3 1 over 2 ( [sin #3 pi f T]

ove

[EQUATION "center W sub t (f,T) #3#3 #^ = #3#3 {#3#3 1 over 2 ( [sin #3 pi f T]

[EQUATION "center W sub w (f,T) #3#3 = #3#3 2 over [pi f T] #3#3 ( 1 over [ pi

f

[EQUATION "center W sub p (f,T) #3#3 = #3#3 1 over [pi f T] #3#3 ( 1 over [ pi

f

In equations 3.26 and 3.27 above

[EQUATION "f sub 1 #3 = #3 1 #1 / #1 T", Position:In Text] .  Recall that if

none of these windows are selected (no tapering), then the square window

defined in equation 3.18 and with a Fourier transform as defined in equation

3.21 is implicitly used.  The most commonly used window function is the Hanning

window; the transform for this function has the smallest side lobes but also

has the widest main lobe.  Also shown is a window which provides a cosine taper

for the first and last tenths of the data record, is one for the remainder of

the data record and is denoted by the name Tukey (also known as a 10% cosine

taper).  This window has been used by many researchers in the hopes that it

will throw away less of the data.  Examination of the modulus of the Fourier

transform for this window shows that it manages to narrow the main lobe a small

amount relative to the other windows, but it does this at the expense of large

side lobes that are nearly as large as that of the square window (no tapering

at all).

A better way to throw away less of the data is to use any of the windows

available above and then to use overlapped processing techniques.  Before

describing overlapped processing, it is useful to review the procedure for the

calculation of spectra when no overlap is used.  The data are prepared

(detrended, filtered, etc.), and then the data are tapered.  The data are

tapered record by record and transformed to get the necessary spectra.  When

50% overlapped processing is used, the tapering operation proceeds a bit

differently.  First, the initial record of the data is tapered.  Then, the

second half of the first record is grouped together with the first half of the

second record to form a new second record, and then this new record of data is

tapered.  Then, the two halves of the old second record form the new third

record, and this is then tapered.  Then, the second half of old second record

is grouped together with the first half of the old third record to form a new

fourth record which is then tapered.  This proceeds throughout the file until

[EQUATION "M", Position:In Text]  old records are processed creating exactly

[EQUATION "2M", Position:In Text]  new records.  The last grouping pairs the

second half of the old last record with the first half of the old first record

to form the new  [EQUATION "2Mth", Position:In Text]  record.  This last record

was included in order to simplify the code for this procedure.  Because the

[EQUATION "2Mth", Position:In Text]  record has a discontinuity at the

midpoint, this record should not be transformed.  However, simply removing this

record would require that an odd number of records be submitted to the FFT

code.  As pointed out earlier, the FFT routine has been designed such that it

transforms two records of data in one pass as a time-saving measure; this

requires (in the one channel case) that an even number of records be processed

by the FFT routine.  Therefore, the overlapping operation has not been

modified;  [EQUATION "2M", Position:In Text]  records are created from

[EQUATION "M", Position:In Text]  original records for both the one and two

channel cases.  These  [EQUATION "2M", Position:In Text]  records are then

submitted to the FFT subroutine and are transformed; but the results of the

[EQUATION "2Mth", Position:In Text]  record are discarded after the

transformation.  Summarizing, when overlapping is employed,

[EQUATION "2M", Position:In Text]  records are created and are then

transformed, but only the results from

[EQUATION "2M #3 - #3 1", Position:In Text]  records are actually used in the

final calculations.  To avoid discontinuities resulting from the use of

overlapping in any of the other  [EQUATION "2M #3 - #3 1", Position:In Text]

records, the  [EQUATION "M", Position:In Text]  original records must be

contiguous in time.  The data need not be contiguous from record to record if

overlapping is not employed.  The use of 50% overlapped processing doubles the

number of tapering and FFT operations; however, this will retrieve about 90% of

the stability lost due to the tapering operation and will decrease the

variability of the resulting estimate.

Finally, tapering is best used when the input data are essentially random

data -- that is, not deterministic.  When this program was tested, input data

(both deterministic and random) with known spectral densities were used.  When

a sinewave was used for input, the power and amplitude spectra exhibited a

lower noise floor when no tapering was used.  On the other hand, when colored

noise (bandwidth limited white noise) was used for input, a substantial

improvement was evident in the output spectra when any of the four window

functions described above was used to taper the data.

^SSpectrum Computation

This section introduces the definitions of the discrete Fourier transform

and the various spectral density functions computed by this routine.  The means

by which these estimates are computed as well as the errors associated with the

calculation are discussed.  The need for additional code to adjust the value of

the computed phase angle of the cross-spectral density function is explained by

means of an example, and an explanation of the coherence function calculated by

this routine is given.

4.1 Fast Fourier Transform

Using the notation of Bendat and Piersol (1986), the Fourier transform of

a function  [EQUATION "x(t)", Position:In Text] , where

[EQUATION "x(t)", Position:In Text]  may be real or complex-valued, is given by

[EQUATION "center X(f) #3#3 = #3#3 int super [infinity] sub [- infinity] #3

x(t)

This definition is not particularly useful for experimentally derived data

which exist for only a finite time interval.  For this reason, one usually

employs a finite-range transform over the interval

[EQUATION "0 <= t <= T", Position:In Text] , and this transform is defined by

[EQUATION "center X (f,T) #3#3 = #3#3 int super T sub [#3 0] #3 x (t) #3 e

super

As in equation 1.1, assume that each record of the input data file consists of

[EQUATION "N", Position:In Text]  samples of

[EQUATION "x(t)", Position:In Text]  which have been obtained at equally spaced

points in time with spacing  [EQUATION "Delta t", Position:In Text] .  Note

that  [EQUATION "t sub 0", Position:In Text]  has been set equal to zero for

convenience.  Then, each record contains the values

[EQUATION "x sub n", Position:In Text]  where

[EQUATION "center x sub n #3#3 = #3#3 x(n Delta t) #4#4 #R[for] #4 n #3 = #3

0,1

Now, the finite-range transform defined in equation 4.2 can be written in

discrete form as

[EQUATION "center X(f,T) #3#3 = #3#3 Delta t #3#3 sum below [n=0] above [N-1]

#3

Since only  [EQUATION "N", Position:In Text]  input values of

[EQUATION "x sub n", Position:In Text]  are used in the computation of the

transform,  only  [EQUATION "N", Position:In Text]  output values of

[EQUATION "X(f,T)", Position:In Text]  may be specified.  These are typically

chosen at the following discrete frequency values

[EQUATION "center f sub k #3#3 = #3#3 k over T #3#3 = #3#3 k over [N Delta t]

#3

Thus, the discrete form of the finite-range Fourier transform becomes

[EQUATION "center X(f sub k #1,T) #3#3 = #3#3 Delta t #3#3 sum below [n=0]

above

Although  [EQUATION "N", Position:In Text]  output values have been specified,

results are unique only out to

[EQUATION "k #3 = #3 N #1 / #1 2", Position:In Text] ; this is a result of the

symmetry of the Fourier transform for real input values.  Thus, the highest

frequency that can be resolved for a sampling rate of

[EQUATION "1 #1 / #1 Delta t", Position:In Text]  samples per second is the

Nyquist frequency given by

[EQUATION "center f sub [N over 2] #3#3 = #3#3 f sub c #3#3 = #3#3 1 over [2 #1

which occurs for  [EQUATION "k #3 = #3 N #1 / #1 2", Position:In Text] .  To

compute the  [EQUATION "X sub k", Position:In Text]  values in equation 4.6

requires  [EQUATION "N super 2", Position:In Text]  complex multiply-add

operations for each record of the data file; note that 1 complex multiply-add

operation is equivalent to 4 real multiply-adds.  This can be quite a

formidable computational task as  [EQUATION "N", Position:In Text]  becomes

large.  Fortunately, several algorithms for reducing the complexity and

increasing the speed of the computation exist and are known collectively as

fast Fourier transforms (FFT).  Rather than discuss these algorithms in detail,

the reader is referred to the discussions that may be found in the references.

The FFT algorithm used here employs a technique known as time decomposition

with input bit reversal.  The kernel of this algorithm was taken from

Stearns (1975).  The simplest implementation of this algorithm calls for

[EQUATION "N", Position:In Text]  to be a power of 2.  This is required in this

routine, and if necessary, each record of the input data file should be padded

with zeros at the end to satisfy this requirement.

The FFT algorithm used in this routine is designed to handle the general

case where  [EQUATION "x(t)", Position:In Text]  is a complex-valued function;

that is, the input data which is passed to the FFT subroutine consists of two

arrays: one containing the real parts and the other containing the imaginary

parts.  However, experimentally sampled random time series data are

real-valued.  When the FFT of a purely real-valued function is computed,

certain symmetries in the resulting transform allow one to compute the FFTs for

two real-valued functions simultaneously by inserting one into the input array

of real parts and the other into the array of imaginary parts.  The trick is to

unscramble the individual transforms from the resulting computation.  This

speed improvement has been incorporated into this routine, but it places a

restriction on the input data files.  If the input data file to be processed by

SPECTRUM consists of one channel of sampled data, then the data file must

consist of an even number of records.  The reason for this is that input data

records are submitted to the FFT algorithm two at a time to obtain the speed

improvement discussed above.  If the input data file consists of two channels

of sampled data, then each record of the file is first decomposed into the two

separate channels, and these are then submitted to the FFT algorithm to be

simultaneously processed.  So, for the case of two channels of sampled data,

the input data file may have an even or odd number of records.

4.2 Autospectral (Power) Density

The one-sided autospectral density function, defined for positive

frequencies only, is usually expressed as twice the amplitude of the Fourier

transform of the autocorrelation function as follows:

[EQUATION "center G sub [xx] (f) #3#3 = #3#3 2 #3 int super infinity sub [#3 -

i

where  [EQUATION "R sub [xx] (tau)", Position:In Text]  is the autocorrelation

function defined as

[EQUATION "center R sub [xx] (tau) #3#3 = #3#3 E #[ #3 x(t) #3 x(t + tau) #3 #]

and the notation E[ ] denotes the expected value operation.

[EQUATION "G sub [xx] (f)", Position:In Text]  is a real function of

[EQUATION "f", Position:In Text]  regardless of whether

[EQUATION "x(t)", Position:In Text]  is real or complex.  This calculation is

implemented in this routine using digital computing techniques employing finite

fast Fourier transforms of the original data records.  Using this method, the

autospectral density may be obtained from

[EQUATION "center G sub [xx] (f) #3#3 = #3#3 2 #3#3 lim below [T -> infinity]

#3

[EQUATION "| #3#3 |", Position:In Text]  denotes the modulus of a complex

quantity, and  [EQUATION "X (f,T)", Position:In Text]  is the finite Fourier

transform which is valid for the interval

[EQUATION "0 <= t <= T", Position:In Text]  and is calculated as in equation

4.6.  If the input data file contains two channels of data, denoted as

[EQUATION "x(t)", Position:In Text]  and  [EQUATION "y(t)", Position:In Text] ,

then selection of the power spectral density option results in the computation

of  [EQUATION "G sub [xx] (f)", Position:In Text]  and

[EQUATION "G sub [yy] (f)", Position:In Text] .

Specifically, the calculation proceeds by computing the FFT for each

record of the input data file, determining the modulus of the complex result,

squaring it and then saving the results to an array.  This is repeated for each

record; upon completion, the results are ensemble averaged to approximate the

expected value operation required in the definition.  The computation of an

expected value is needed to reduce the random error of the estimated

autospectral density function.  The number of records in the input data file

determines the number of members in the ensemble which will be averaged.  If

the number of records in the data file (before tapering) is denoted by

[EQUATION "n sub r", Position:In Text] , then the normalized random error of

the autospectral density estimate is given by

[EQUATION "center G sub [xx] (f) #3 : #4#4 epsilon #3#3 = #3#3 1 over [root[n

su

where the normalized random error is defined as a fractional quantity by

dividing the standard deviation of the estimate by the actual value estimated.

If the tapering operation is used, it will increase the variability of the

resulting estimate by an amount which depends upon the particular window

function chosen.  This increase in the variance of the estimate is, in the

worst case (Hanning), about two; but if the 50% overlap technique is also

employed, it will counteract this increase and will recover about 90% of the

variability caused by tapering.  Thus, the normalized random error calculated

from equation 4.11 will serve as a reasonably accurate estimate for both the no

taper and taper with overlap cases.  If tapering is used but overlapping is

not, then the variance is doubled and the normalized random error defined in

equation 4.11 should be multiplied by the square root of two.  The routine

computes the appropriate value of the normalized random error and displays it

on the screen at run-time.  As an example, consider an autospectral density

estimate computed from a data file with 100 original records which are then

overlapped and tapered to produce 199 records; this estimate will have a

normalized random error of 10%.  To reduce this error to 5%, the input data

file would have to contain 400 records.  There is no specific numerical limit

to the number of records that an input data file may contain; the spectrum

routine has successfully computed spectral densities for input data files with

10,000 records.  However, data files with more than 100 records are usually

quite large. Furthermore, the spectrum routine must create temporary files

during computation, and the size of these files (depends upon the size of the

input data file) is often very large.  Reserving a sufficient amount of space

on the hard disk or on a RAM disk for these files is typically the limiting

factor on the number of records.

The computation of a fast Fourier transform and power spectral density

function allows the calculation of the mean and root-mean-square (rms) values

of each record of the input time series.  The equation for the mean value of

each data record can be obtained by setting

[EQUATION "k #3 = #3 0", Position:In Text]  in equation 4.6.  One then obtains

[EQUATION "center x bar #3#3 = #3#3 X sub 0 over N #3#3 = #3#3 1 over N #3 sum

b

The mean-square value of each record can be computed from

[EQUATION "center R sub [xx] (0) #3#3 = #3#3 E #[ #3 x super 2 (t) #3 #] #3#3 =

The rms value is just the square root of this number.  The spectrum routine

calculates the mean and rms values of the input time series using this

procedure and reports them on the screen at run-time along with the appropriate

value of the normalized random error.

4.3 Cross-spectral Density

4.3.1 Calculation Procedure

A one-sided cross-spectral density function

[EQUATION "G sub [xy] #1 (f)", Position:In Text]  may be defined in a manner

similar to equation 4.8; this quantity is, in general, complex and is given by

[EQUATION "center G sub [xy] (f) #3#3 = #3#3 2 #3 int super infinity sub [#3 -

i

The quantity  [EQUATION "R sub [xy] (tau)", Position:In Text]  is the cross

correlation function defined by

[EQUATION "center R sub [xy] (tau) #3#3 = #3#3 E #[ #3 x(t) #3 y(t + tau) #3 #]

The cross-spectral density may also be computed using fast Fourier transform

procedures and is the method adopted here.  The equivalent expression to

equation 4.10 for the one-sided cross-spectral density function is

[EQUATION "center G sub [xy] (f) #3#3 = #3#3 2 #3#3 lim below [T -> infinity]

#3

where the notation  [EQUATION "X super * (f,T)", Position:In Text]  is used to

represent the complex conjugate.  Alternatively, this can be expressed in polar

notation as

[EQUATION "center G sub [xy] #1 (f) #3#3 = #3#3 | G sub [xy] #1 (f) | #3 e

super

Note that this option may be selected only when the input data file contains

two channels of data, then, the magnitude and phase,

[EQUATION "| G sub [xy] #1 (f) |", Position:In Text]  and

[EQUATION "theta sub [xy] #1 (f)", Position:In Text]  respectively, are the

quantities which are calculated and then saved in the output file.

As described above in the section on power spectral density calculation,

it is possible to compute the value of the autocorrelation function for

[EQUATION "tau #3 = #3 0", Position:In Text]  (the mean-square value) by

calculating the integral of the autospectral density function.  In a similar

manner one may compute the cross correlation function evaluated at

[EQUATION "tau #3 = #3 0", Position:In Text]  from

[EQUATION "center R sub [xy] #1 (0) #3#3 = #3#3 E #[ #3 x(t) #3 y(t) #3 #] #3#3

The spectrum routine calculates this quantity and displays it on the screen at

run-time.

4.3.2 Adjustment of Phase Angle

In order to clarify the need for additional code to adjust the values of

the phase angle  [EQUATION "theta sub [xy] (f)", Position:In Text] , an example

taken from Hess (1990) will be given which also serves to illustrate the use of

the cross spectral density function in general.  Specifically, a point

measurement of the vertical displacement of a compliant surface, responding to

the unsteady forcing of a fluid flow above the surface, was acquired; call this

displacement signal  [EQUATION "x(t)", Position:In Text] .  As

[EQUATION "x(t)", Position:In Text]  was measured, a second displacement signal

was simultaneously measured; call this second signal

[EQUATION "y(t)", Position:In Text] .  The locations on the compliant surface

of the measurement of  [EQUATION "x(t)", Position:In Text]  and

[EQUATION "y(t)", Position:In Text]  were as follows:  both were obtained on

the centerline of the rectangular compliant slab, and

[EQUATION "y(t)", Position:In Text]  was located 11.4 cm downstream of

[EQUATION "x(t)", Position:In Text] .  The fluid flow was turbulent; the

instantaneous streamwise component of velocity above the compliant surface

consisted of a small fluctuating quantity superimposed upon a larger mean

value.  A disturbance in the fluid flow caused a displacement of the compliant

surface which was measured at the upstream location and was recorded at time

[EQUATION "t", Position:In Text]  as  [EQUATION "x(t)", Position:In Text] .  As

the flow disturbance was convected downstream by the mean flow, the

displacement of the compliant surface also travelled downstream and was

measured at the downstream location as  [EQUATION "y(t+tau)", Position:In Text]

where  [EQUATION "tau", Position:In Text]  was the time delay for the

disturbance to travel 11.4 cm.  Quantities of interest, then, are the

convection speed of the displacement fluctuation on the compliant surface as

well as the decrease in amplitude of this fluctuation as it decays.

For this particular situation, the decay of the displacement field may be

modeled by the time delay problem, see Bendat and Piersol (1986).  This model

provides a method for determining the convection speed of the displacement

fluctuation.  Consider two zero-mean stationary random signals

[EQUATION "x(t)", Position:In Text]  and  [EQUATION "y(t)", Position:In Text]

where  [EQUATION "y(t)", Position:In Text]  is defined by

[EQUATION "center y(t) #3#3 = #3#3 alpha #1 x(t-tau sub 0) #3#3 + #3#3 n(t)

#3#3

The value  [EQUATION "alpha", Position:In Text]  is a constant attenuation,

[EQUATION "tau sub 0", Position:In Text]  is a constant time delay and

[EQUATION "n(t)", Position:In Text]  is uncorrelated, zero mean value noise at

the output.  This process is illustrated in figure 4.1.

[PICTURE FIG41.CGM, Scale:0.800]

Figure 4.1  Schematic for time delay problem

The cross-correlation of these two signals can be computed to be

[EQUATION "center R sub [xy] #1 (tau) #3#3 = #3#3 alpha #3 R sub [xx] #1 (tau -

Thus, the cross-correlation is an attenuated replica of the autocorrelation,

which is displaced in time by the amount

[EQUATION "tau sub 0", Position:In Text] .  Without going through all of the

details, a one-sided cross-spectral density function

[EQUATION "G sub [xy] #1 (f)", Position:In Text]  may also be computed; this

quantity is, in general, complex and for this problem is given by

[EQUATION "center G sub [xy] #1 (f) #3#3 = #3#3 alpha #3 G sub [xx] #1 (f) #3 e

Since the time delay  [EQUATION "tau sub 0", Position:In Text]  appears only in

the phase angle,  [EQUATION "theta sub [xy] #1 (f)", Position:In Text] , it can

be determined from measurement of the phase angle and by noting that it is a

linear function of the frequency with slope

[EQUATION "2 pi tau sub 0", Position:In Text] .  Summarizing then, a

cross-spectral density is computed for  [EQUATION "x(t)", Position:In Text]

and  [EQUATION "y(t)", Position:In Text] .  If the process can be modelled by

the time delay problem, then the modulus will be an attenuated replica of the

autospectral density,  [EQUATION "G sub [xx] (f)", Position:In Text] , and the

phase angle will be a linear function of the frequency with a slope given by

[EQUATION "2 pi tau sub 0", Position:In Text] .  A convection speed may then be

calculated from

[EQUATION "U sub c #3#3 = #3#3 11.4 over [tau sub 0]", Position:In Text]

cm/sec.

An example of this procedure is shown in figure 4.2.  The top graph shows

the autospectral density curve  [EQUATION "G sub [xx] (f)", Position:In Text]

and below it the modulus  [EQUATION "| G sub [xy] (f) |", Position:In Text]

for the corresponding cross-correlation; indeed, the bottom curve does appear

to be an attenuated copy of the first.  Note the substantial additional scatter

in the cross-spectral density estimate relative to that for the autospectral

density estimate at the higher frequencies.  The normalized random error

[PICTURE FIG42.CGM, Scale:0.800]

Figure 4.2  Cross-spectral density modulus example

for the cross-spectral density estimate turns out to be a function of frequency

unlike that for the autospectral density estimate.  A discussion of the means

for estimating this error is given in a later section.

Figure 4.3 displays the computed phase angle expressed in radians.  With

the data presented in this form, it is difficult to fit the data with a least

squares fit in order to determine the slope, and therefore,

[EQUATION "tau sub 0", Position:In Text] .  This difficulty may be traced to

the fact that an inverse tangent must be computed in order to calculate the

value of the phase angle.  The FORTRAN intrinsic function ATAN2 returns a value

for the inverse tangent, but forces the result to lie in the range

[EQUATION "- pi #3 <= #3 result #3 <= #3 pi", Position:In Text] .  However, the

phase angle function is periodic with period

[EQUATION "2 pi", Position:In Text] .  Using this fact, additional code may be

used to straighten out the curve by adding integer multiples of

[EQUATION "2 pi", Position:In Text]  as required.  Figure 4.4 shows the

adjusted values of the phase angle and a least squares fit to the slope.  The

calculated values of  [EQUATION "tau sub 0", Position:In Text]  and

[EQUATION "[U sub c] #1 / #1 [U sub infinity]", Position:In Text] , where

[EQUATION "U sub [infinity]", Position:In Text]  is the mean flow speed far

above the compliant surface, are given on the plot.  The scatter in the phase

angle data becomes large at a frequency for which the energy in the signal is

about two decades below the peak value.  Computed values of the convection

speed using this method vary by less than 2% from those obtained using an

alternative method.

The phase angle data can be adjusted manually, but this is a tedious and

time-consuming procedure.  Instead, the spectrum routine automatically computes

both the original values of the phase angle as well as the adjusted values of

the phase angle when the cross spectral density and the coherence function are

calculated.  Note that the relationships derived in equations 4.21 apply only

when equation 4.19 may be used as a model for the data.  Therefore, both sets

of phase angle data, unadjusted and adjusted values, are stored to the output

file and made available to the user.  The task of adjusting these phase angle

values, while conceptually simple, is somewhat tricky to program.  A

description of the technique follows.

The standard deviation of the phase angle estimate as a function of

frequency is computed when the coherence function is computed (this will be

explained in the section on error estimates).  The average of a moving window

of  [EQUATION "NSD", Position:In Text]  values of this standard deviation is

computed and checked against a threshold value given by

[EQUATION "SDLIM", Position:In Text] .  If the average standard deviation is

too high, then the phase angle points are considered to be essentially random

and no further attempts to adjust the values are made.  Thus, when this

condition occurs, the remaining phase angle values are kept the same as the

original values (not adjusted).  When the moving average of the standard

deviation is less than  [EQUATION "SDLIM", Position:In Text] , then adjustment

may take place as follows.  A straight line is fit to a moving window of

[EQUATION "NPTS", Position:In Text]  values of the original phase angle data

using a least squares fit.  The coefficients of this fit are used to guess the

next value of the phase angle; if the actual value of the next phase angle data

point differs from the guessed value by an amount greater than

[EQUATION "pi", Position:In Text]  radians, then multiples of

[EQUATION "2 pi", Position:In Text]  radians are added to (or subtracted from)

the actual value until the difference between the actual and guessed values is

less than  [EQUATION "pi", Position:In Text]  radians.  The procedure continues

to adjust subsequent values of the phase angle until the moving average of the

standard deviation exceeds  [EQUATION "SDLIM", Position:In Text] .  This

portion of the code contains three adjustable parameters:

[EQUATION "NSD #3 = #3 7", Position:In Text] ,

[EQUATION "NPTS #3 = #3 7", Position:In Text]  and

[EQUATION "SDLIM =70.0 degree", Position:In Text] .  These values have been

selected by trial and error to yield the best response for the data most often

processed by the author.  Other values may be chosen by changing the numbers in

the appropriate PARAMETER statement at the beginning of the code.  Note that

[EQUATION "NSD", Position:In Text]  and [EQUATION "NPTS", Position:In Text]

must be chosen such that

[EQUATION "center NSD #3#3 <= #3#3 2 #3 NPTS #3#3 - #3#3 1 right #R(4.22)"]

is satisfied.  This adjustment procedure seems to work reasonably well for most

situations encountered by the author; however, some manual adjustment of values

may still be required in some circumstances.[PAGE BREAK]

[WHITESPACE 0.4 in, Place All on Next Page]

[PICTURE FIG43.CGM, Scale:0.800]

Figure 4.3  Phase angle example

[WHITESPACE 0.25 in, Place All on Next Page]

[PICTURE FIG44.CGM, Scale:0.800]

Figure 4.4  Phase angle - adjusted values[PAGE BREAK]

4.3.3 Coherence Function

The coherence function, also known as the coherency squared function is

defined by

[EQUATION "center gamma super 2 sub [xy] (f) #3#3 = #3#3 [ | G sub [xy] (f) |

su

where

[EQUATION "0 #3 <= #3 gamma super 2 sub [xy] (f) #3 <= #3 1", Position:In Text]

for all  [EQUATION "f", Position:In Text] .  This function is analogous to

the square of the more well known correlation coefficient function given by

[EQUATION "center rho sub [xy] (tau) #3#3 = #3#3 [R sub [xy] (tau)] over [ #[ R

where  [EQUATION "-1 #3 <= #3 rho sub [xy] (tau) #3 <= #3 1", Position:In Text]

for all values of  [EQUATION "tau", Position:In Text] .  As described by

Bendat and Piersol (1986), the function

[EQUATION "rho sub [xy] (tau)", Position:In Text]  measures the degree of

linear dependence between  [EQUATION "x(t)", Position:In Text]  and

[EQUATION "y(t)", Position:In Text]  for a displacement of

[EQUATION "tau", Position:In Text]  in  [EQUATION "y(t)", Position:In Text]

relative to  [EQUATION "x(t)", Position:In Text] .  Similarly, for linear

systems, the function  [EQUATION "gamma super 2 sub [xy] (f)", Position:In

Text]

measures the fractional portion of the mean square value at the output

[EQUATION "y(t)", Position:In Text]  that is contributed by the input

[EQUATION "x(t)", Position:In Text]  at frequency

[EQUATION "f", Position:In Text] .  On the other hand, the quantity

[EQUATION "#[ 1 #3 - #3 gamma super 2 sub [xy] (f) #]", Position:In Text]  is a

measure of the mean square value of  [EQUATION "y(t)", Position:In Text]  not

accounted for by  [EQUATION "x(t)", Position:In Text]  at frequency

[EQUATION "f", Position:In Text] .  If  [EQUATION "x(t)", Position:In Text]

and  [EQUATION "y(t)", Position:In Text]  are completely unrelated, the

coherence function will be zero.  Furthermore, Bendat and Piersol (1986)

indicate that if the coherence function is greater than zero but less than

unity, one or more of the following three possible physical situations exist.

1.   Extraneous noise is present in the measurements.

2.   The system relating

[EQUATION "x(t)", Position:In Text]  and

[EQUATION "y(t)", Position:In Text]  is not linear.

3.   [EQUATION "y(t)", Position:In Text]  is an output due

to an input  [EQUATION "x(t)", Position:In Text]  as

well as to other inputs.

The calculation of this function requires not only the computation of the cross

spectral density function but also the computation of power spectral densities

for each channel of data.  The code has been optimized to minimize the amount

of additional computational effort; however, in anticipation of the possibility

that a slow computer might be employed, the routine prompts the user for this

option at run-time.

4.3.4 Error Estimates

The definition of the coherence function makes possible at this point a

discussion of the error associated with estimates of the magnitude and phase of

the cross spectral density function and of the coherence function itself.  This

information comes from Bendat and Piersol (1986).  The normalized random error

of the estimate of the magnitude of the cross spectral density function is

given by

[EQUATION "center   | G sub [xy] (f) | #3 : #4#4 epsilon (f) #3#3 = #3#3 1 over

where  [EQUATION "| gamma sub [xy] (f) |", Position:In Text]  is the positive

square root of the coherence function evaluated at

[EQUATION "f", Position:In Text] , and  [EQUATION "n sub r", Position:In Text]

is the number of records in the input data file.  The error approaches that for

the power spectral density estimate (see equation 4.11) as

[EQUATION "| gamma sub [xy] (f) |", Position:In Text]  goes to one, and the

error is higher for values less than one.  Unlike the error estimate for the

power spectral density function, all of the error estimates in this section are

a function of frequency.

Since the phase angle may assume the value zero, a normalized random error

cannot be specified; instead the standard deviation (s.d.) of the phase angle

estimate is given by

[EQUATION "center theta sub [xy] (f) #3 : #4#4 s. #3 d. #1 (f) #3#3 = #3#3 [ #[

The standard deviation is seen to approach zero as

[EQUATION "| gamma sub [xy] (f) |", Position:In Text]  goes to one and does so

independent of  [EQUATION "n sub r", Position:In Text]  for

[EQUATION "n sub r #3 > #3 1", Position:In Text] .  Continuing, the normalized

random error of the coherence function itself can also be calculated and is

defined as

[EQUATION "center gamma super 2 sub [xy] (f) #3 : #4#4 epsilon (f) #3#3 = #3#3

[

where the estimated value of the coherence function is substituted as an

approximation to the unknown true values of

[EQUATION "gamma super 2 sub [xy] (f)", Position:In Text]  and

[EQUATION "| gamma sub [xy] (f) |", Position:In Text]  called for in equation

4.27.  Note that the normalized random error approaches zero as either

[EQUATION "n sub r #3 -> #3 infinity", Position:In Text]  or

[EQUATION "gamma super 2 sub [xy] #3 -> #3 1", Position:In Text] .  This

implies that coherence function estimates can be more accurate than either

autospectra or cross spectra estimates (which are used to calculate the

coherence function) when

[EQUATION "gamma super 2 sub [xy] (f)", Position:In Text]  is close to unity.

If the coherence function is calculated, the spectrum routine automatically

computes the three error estimates defined in equations 4.25 to 4.27 as a

function of frequency and saves the results in a separate output file.  The

file name and structure of this file will be discussed in the section on output

files.  Equations 4.25 to 4.27 are valid for the no taper and taper with

overlap cases.  If tapering is used without overlapping, then the error

estimates in equations 4.25 to 4.27 must each be increased by multiplying by

the square root of two, and this is automatically performed by the spectrum

routine.

It is perhaps useful to know the number of records required in the input

data file to attain a specified normalized random error or standard deviation

for each of the estimates mentioned above.  This has been calculated for

various values of  [EQUATION "gamma super 2 sub [xy] (f)", Position:In Text]

for the cases:

[EQUATION "epsilon #3 #[ #3 | G sub [xy] (f) | #3 #] #3 = #3 0.1", Position:In

T

,

[EQUATION "epsilon #3 #[ #3 gamma super 2 sub [xy] (f) #3 #] #3 = #3 0.1",

Posit

and

[EQUATION "s. #3 d. #3 #[ #3 theta sub [xy] (f) #3 #] #3 = #3 0.052 #3 rad #3

~~

, and the results are given in table 4.1 below.

[EQUATION "gamma super 2 sub [xy] (f)", Position:In

Text]

0.3    0.4    0.5    0.6    0.7    0.8    0.9

[EQUATION "epsilon #3 #[ #3 | G sub [xy] (f) | #3 #] #3 = #3 0.1",

Position:In Text]

334    250    200    167    143    125    112

[EQUATION "s. #3 d. #3 #[ #3 theta sub [xy] (f) #3 #] #3 ~~ #3 3

degree",Position:In Text]

426    274    183    122     79     46     21

[EQUATION "epsilon #3 #[ #3 gamma super 2 sub [xy] (f) #3 #] #3 = #3

0.1",Position:In Text]

327    180    100     54     26     10     2

Table 4.1  Number of records for specified error

4.4 Amplitude Spectral Density

The amplitude spectrum option was added to allow the user to examine the

Fourier coefficients directly.  The amplitude spectrum is defined as

[EQUATION "center F sub x (f) #3#3 = #3#3 lim below [T -> infinity] #3#3 E #3

#[

where  [EQUATION "| X (f,T) |", Position:In Text]  is the modulus of the

quantity calculated in equation 4.6.  If the input data file contains two

channels of data, denoted as  [EQUATION "x(t)", Position:In Text]  and

[EQUATION "y(t)", Position:In Text] , then selection of this option results in

the computation of  [EQUATION "F sub x (f)", Position:In Text]  and

[EQUATION "F sub y (f)", Position:In Text] .

A simple example taken from Stearns (1975) is used to demonstrate the

utility of this option.  Consider the following function which was

computer-generated and then saved as a set of digital values to simulate the

sampling process:

[EQUATION "center x(t) #3#3 = #3#3 e super [-t] #3 sin #3 t right #R(4.29)"]

A plot of a portion of this function is shown in figure 4.5.  The Fourier

transform of this function can be derived using the definition in equation 4.1

as

[EQUATION "center X(f) #3#3 = #3#3 1 over [[(i #3 2 pi f #3 + #3 1)] super [#3

2

and the modulus of this quantity becomes

[EQUATION "center |X(f)| #3#3 = #3#3 1 over [ #[ #3 [(2 pi f)] super [#3 4] #3

+

[PAGE BREAK]

[WHITESPACE 0.25 in, Place All on Next Page]

[PICTURE FIG45.CGM, Scale:0.800]

Figure 4.5  Plot of example input function

[WHITESPACE 0.25 in, Place All on Next Page]

[PICTURE FIG46.CGM, Scale:0.800]

Figure 4.6  Plot of amplitude spectrum calculation[PAGE BREAK]

This input function is deterministic, and therefore only one record of data

would normally be required; however, because an even number of records are

required by the spectrum routine for one channel of input data, two identical

records of this function were generated.

[EQUATION "N #3 = #3 2048", Position:In Text]  samples per record were created

with a time interval of  [EQUATION "Delta t #3 = #3 0.01", Position:In Text]

seconds between successive samples.  A portion of the results is shown in

figure 4.6 with the symbols representing the values computed by the spectrum

routine and the continuous line the values generated by equation 4.31.

[PAGE BREAK]

^SU' Output Data Files

For every input data file processed, the spectrum routine creates one (or

sometimes two) ASCII output files containing the results.  A second output file

is created only if the user processes two channels of input data and computes

the cross-spectral density and the coherence function; the second output file

then contains error estimates of cross-spectral magnitude and phase and of the

coherence function.

5.1 Naming Convention

The output file name will consist of nine characters.  The last four

characters specify the file extension; the default file extension, [.PRN], will

be used unless specified otherwise in the configuration file.  The first four

characters of the output file name will match the first four characters of the

corresponding input data file.  The fifth character is used to designate the

type of file that is being written.  The four possibilities are:

power spectrum                 P

cross spectrum                 C

amplitude spectrum             A

error estimates (if computed)  E

The output file will be written to the same location as the input data file;

this will, by default, be the location where the executable version of the

spectrum routine resides.  This location can be changed, however, in the

configuration file.

5.2 File Structure

These output data files consist of ASCII data as opposed to binary data.

They may be easily edited by any ASCII editor, imported into spreadsheet

programs for further manipulation or imported into plotting programs to be

graphed.  These files contain numbers only; no labels are included to describe

the various columns of data in the output.  While somewhat inconvenient for the

user, this was done to prevent the confusion that sometimes occurs when data

containing both numbers and labels are imported into other software programs.

In the sections that follow, a thorough description of the output to be

expected for each of the four types of output files is given, and this should

allow the user to interpret the results without any difficulty.

5.2.1 Power Spectrum Output Files

The first line of the output contains the mean and rms values computed as

described in equations 4.12 and 4.13 and then ensemble-averaged over all

records of the file.  Note that the mean and rms values reported here are

calculated subsequent to any cosmetic operations that may have been applied to

the data to improve the quality of the spectral density computation.  Removal

of the mean value, detrending, filtering and tapering operations may modify the

mean and rms so that they do not match the original values for the file which

were calculated and reported at run-time prior to the application of these

operations.  Continuing, the next line of the output file is blank.  The

calculated results begin on the third line and consist of four or six columns

of data depending on whether the input file contains one or two channels.  The

quantities are stored in the following order:

One channel:

[EQUATION "center #^ #R[mean] #4#4 #R[rms] #4#4#4#4#4#4#4#4#4#4#4#4#4 end

center

Two channels:

[EQUATION "center #^ #R[mean] #4#4 #R[rms] #4#4#4#4#4#4#4#4#4#4#4#4#4 end

center

In either case the output file contains

[EQUATION "N #1 / #1 2", Position:In Text]  rows of data after the blank line

with each row calculated for values of  [EQUATION "f", Position:In Text]

ranging from  [EQUATION "1 #1 / #1 N Delta t", Position:In Text]  to

[EQUATION "1 #1 / #1 2 Delta t", Position:In Text]  with a resolution of

[EQUATION "Delta f #3 = #3 1 #1 / #1 N Delta t", Position:In Text] .  This

structure is typical for all of the output files created by the spectrum

routine, and in the following descriptions of the output only the column labels

will be identified.

5.2.2 Cross Spectrum Output Files

A cross spectrum may only be calculated when the input file contains two

channels of data.  However, the output file is somewhat different depending

upon whether or not the coherence function is calculated.  In either case, the

file begins with the value of the cross correlation function evaluated at

[EQUATION "tau #3 = #3 0", Position:In Text]  and calculated according to

equation 4.18.  The next line is skipped, and the following record is the first

of  [EQUATION "N #1 / #1 2 ", Position:In Text]  rows of data arranged in five

or seven columns as described below.

Coherence function not calculated:

[EQUATION "center #^ R sub [xy] (0) #4#4#4#4#4#4#4#4#4#4#4#4#4#4#4#4 end center

Coherence function calculated:

[EQUATION "center #^ R sub [xy] (0) #4#4#4#4#4#4#4#4#4#4#4#4#4#4#4#4 end center

5.2.3 Amplitude Spectrum Output Files

When the amplitude spectrum option is chosen, the structure of the output

file is slightly different than for the other files.  No mean or rms value is

reported.  Instead, the file contains

[EQUATION "N #1 / #1 2 #3 + #3 1", Position:In Text]  rows of data arranged in

the columns shown below.  The first row consists of data computed for

[EQUATION "f #3 = #3 0", Position:In Text] ; data for this value of

[EQUATION "f", Position:In Text]  is not stored when other options are

selected, but is useful when examining the Fourier coefficients.  Actually, the

value  [EQUATION "f #3 = #3 epsilon", Position:In Text]  is used; this is

necessary since  [EQUATION "log #1 (0)", Position:In Text]  is undefined.  The

value of  [EQUATION "epsilon", Position:In Text] is defined in a PARAMETER

statement at the beginning of the code and is currently set at

[EQUATION "epsilon #3 = #3 1.0 #3 x #3 10 super [-20]", Position:In Text] .

One channel:

[EQUATION "center matrix [[ [#^ f] [log #3 f] [F sub [x] (f)] #4#4#4#4#4#4 ] [

[

[PAGE BREAK]

Two channels:

[EQUATION "center matrix [[ [#^ f] [log #3 f] [F sub [x] (f)] [F sub [y] (f)]

#4

5.2.4 Error Estimate Output Files

When both the cross-spectral density and the coherence function options

are selected, the spectrum routine calculates the errors associated with these

estimates as a function of frequency using equations 4.25 through 4.27.  These

error data are then stored in a separate file from the other results.  This

file contains  [EQUATION "N #1 / #1 2", Position:In Text]  rows of data

arranged in the columns identified below.  The columns containing normalized

random errors consist of dimensionless numbers, and the standard deviation of

the phase angle is expressed in degrees.

[EQUATION "center matrix [[ [#^ f] [log #3 f] [ epsilon #3 #[ #3 | G sub [xy]

(f

^Sm" Run-time Considerations

Two topics are discussed in this section which assume importance when the

program is executed.  The creation of a configuration file allows the user to

bend some of the rules regarding file name creation and the location of data

files as well as temporary files created by the spectrum routine.  The second

topic concerns the manner in which the size of the temporary files created by

the routine may be determined.

6.1 Configuration File

The presence of an ASCII file with the name SPECTRUM.CFG in the same

directory as the SPECTRUM.EXE file allows optional settings to take effect

which may have a favorable impact on the execution speed and overall ease of

use of the routine.  This ASCII file allows the user to specify three

parameters which will become default values when the program is executed.

1.      The first item is the drive and path information for the location of

input, output and coefficient data files.  If the user wishes to

specify that these files may be found in (and written to) the same

location as the SPECTRUM.EXE file, then the word ROOT should be entered

on a line by itself.  Otherwise, the desired drive and path information

should be entered in the following format:  C:\SPECTRUM\DATA\

Note that the last backslash is required.

2.      The second item in the file is the drive and path information which

specifies where temporary data files created by the spectrum routine

will be stored.  If a RAM disk is present on the user's computer and is

large enough to hold the temporary files, then the execution speed of

the routine will be greatly enhanced if these files are located on this

RAM disk.  A means for estimating the size of these temporary files is

discussed below.  Again, the user specifies ROOT or the desired drive

and path information using a format such as:  D:\TEMP\

3.      The last item allows the user to change the file extension for output

data files from its default value of [.PRN] to some other desired

value.  The user should place the extension on a line by itself in the

following format:  .OUT

Note that the leading period is required.

Comment lines may be included in the configuration file; these lines must start

with an asterisk character.  The routine is not case-sensitive -- the options

may be specified in upper or lower case.  The spectrum routine does not require

the presence of the configuration file; if this file is absent, then the

following default values are used:  1. root  2. root  3. [.prn]  If a

configuration file is used, then it must reside in the same directory as the

SPECTRUM.EXE file and all three options must be specified.  An example

configuration file follows.[PAGE BREAK]

*    This is a sample configuration file for Spectrum.  Comment lines

*    and blank lines must begin with an asterisk.

*

*    The first item in the file is the path where the input data

*    files are found and to which the output data will be written.

*    Acceptable values are: root, c:\ , g:\ , c:\test\bin\ , etc.  The

*    path must end with a backslash if root is not used.

root

*    The second item in the file is the drive on which the temporary

*    files are to be created.  Program execution speed will increase

*    dramatically if this is a RAM disk.

*    Acceptable values are: root, c:\ , g:\ , c:\test\bin\ , etc.  The

*    path must end with a backslash if root is not used.

d:\

*    The third item in the file is the default file extension to use

*    for the output data files.  If not specified in this file, then

*    the .PRN extension will be used in order that the data may be

*    imported into LOTUS for plotting.  The extension must consist of

*    a period followed by three letters.

.prn

6.2 Temporary Files

The input data files that are to be processed by the spectrum routine are

typically very large; therefore, the data is analyzed one record at a time.

Furthermore, the final result of the calculation is not arrived at in one pass;

instead, intermediate results are generated and stored in temporary files which

are then used to obtain the final results.  These temporary files are

automatically deleted upon completion; however, sufficient space must be

reserved on the hard disk or on a RAM disk for temporary use by these files.

The following discussion explains how these files are created and allows the

user to estimate their size based upon the size of the input data file.  Note

that if several input files are to be processed by the routine, only the

temporary files for the input data file currently being processed reside on the

hard disk at any one time; the user needs to allow sufficient space based upon

the size of the largest input data file.

Two temporary files are created for each channel of input data.  After a

record of the input data file is read in by the routine, the integer data is

sorted into one or two arrays depending on the number of input channels; each

channel is then transformed into floating point data (if necessary), the mean

value or a linear trend is removed and any desired filtering takes place.

These data are then stored to temporary files with the extensions : [.TRF] -

channel 1 and [.TR2] - channel 2 (if required).  This process is repeated until

all records of the input data file are read.  If the input data file consisted

of 2-byte integers, then the [.TRF] file will be twice the size of the input

[.DAT] file since the [.TRF] file contains 4-byte floating point numbers.  If

the input data file consists of two channels of integer data, then the [.TRF]

and [.TR2] files will each be the same size as the [.DAT] file (the [.TRF] and

[.TR2] files each contain half as many numbers as the input [.DAT] file, but

each number is a 4-byte value).  If the input data file consists of one channel

of 4-byte real numbers, the [.TRF] file will be the same size as the [.DAT]

file, and finally, for two channels of 4-byte values in the input file, the

[.TRF] and [.TR2] files will each be half the size of the [.DAT] file.  After

creating the [.TRF] and [.TR2] (if required) files, the spectrum routine

proceeds to the tapering operation.  If tapering is selected, the routine reads

a record of data at a time from the [.TRF] and [.TR2] files, creates two new

records for each channel (if overlapping is used) and stores the results in two

temporary files with the extensions [.TAP] and [.TP2].  The last step in the

process is to transform the data stored in the [.TAP] and [.TP2] files and

perform the required spectral calculation.  The final data is written to one

output file regardless of the number of channels of input data and the

extension [.PRN], or alternatively an extension specified in the configuration

file is used.  If tapering is not selected, then the [.TAP] and [.TP2] files

are not created, and storage space does not need to be allocated. If the [.TAP]

and [.TP2] files are created, each will be twice the size of the previous

[.TRF] or [.TR2] file, since two tapered records are written for every input

record read to implement the 50% overlap technique.  (If tapering is employed

but overlapping is not, then the [.TAP] and [.TP2] files will be the same size

as the [.TRF] or [.TR2] file.  The table belows gives the worst case scenario

and assumes overlapping is used when tapering is used.)  At the end of the

calculation, only the original [.DAT] file and the final [.PRN] file will

remain.  If no alternative is specified in the configuration file, the

temporary files will be created in the same directory as the input data files.

Several examples follow which show the sizes of the various files based upon an

input data file size of 1 Mb.

Two Channels ?      No        Yes         No        Yes         No

Integers ?       Yes        Yes         No         No        Yes

Tapering ?       Yes        Yes        Yes        Yes         No

[.DAT]         1 Mb       1 Mb       1 Mb       1 Mb       1 Mb

[.TRF]         2 Mb       1 Mb       1 Mb      500 kb      2 Mb

[.TR2]          --        1 Mb        --       500 kb       --

[.TAP]         4 Mb       2 Mb       2 Mb       1 Mb        --

[.TP2]          --        2 Mb        --        1 Mb        --

[.PRN]        64 kb      93 kb      48 kb      83 kb      64 kb

Total Temp

space needed      6 Mb       6 Mb       3 Mb       3 Mb       2 Mb

Table 4.2  Memory required for temporary files

^So' Examples

Three simple examples are considered that serve to illustrate various

features of the spectrum routine.  The example using colored noise demonstrates

the use of tapering and allows the calculated power spectrum to be compared

with a known power spectrum.  The computation of the amplitude spectrum of a

sinewave reveals the importance that sampling parameters can have on the

computed result, and the last example shows the effects of the application of a

variety of digital filters prior to the computation of power spectra.  A few

comments regarding the considerations necessary for the acquisition of

experimental data files are given at the end.

7.1 Colored Noise

A routine to produce bandwidth-limited white noise (colored noise) has

been written and used extensively during the testing phase of the development

of the spectrum routine.  This program prompts the user for a few parameters,

then generates the colored noise sequence and stores the output data in a

manner that simulates the sampling process.  This data may then be input into

the spectrum program, and the output power spectrum estimate may be compared

with the known power spectrum for colored noise.  This handy routine is a

useful utility program and has been included on the diskette with the name

COLOR.

The procedure for producing colored noise begins by generating random

numbers that vary from 0 to 1.  This sequence is uniformly distributed over the

interval from 0 to 1.  Let  [EQUATION "x sub n", Position:In Text]  be the nth

random number, then  [EQUATION "x sub n #3 - #3 1 #1 / #1 2", Position:In Text]

will be uniformly distributed over the interval from

[EQUATION "- 1 #1 / #1 2", Position:In Text]  to

[EQUATION "1 #1 / #1 2", Position:In Text] .  Now, recall that the mean and

variance of a uniformly distributed random variable is given by

[EQUATION "center mu sub x #3#3 = #3#3 [a #3 + #3 b] over 2 #4#4 #R[and] #4#4

si

where  [EQUATION "a", Position:In Text]  and  [EQUATION "b", Position:In Text]

are the endpoints of the interval.  Using these definitions, we find that the

mean and variance of the sequence

[EQUATION "x sub n #3 - #3 1 #1 / #1 2", Position:In Text]  is

[EQUATION "center mu sub x #3#3 = #3#3 0 #4#4 #R[and] #4#4 sigma sub x super 2

#

If a power spectrum for this sequence were calculated, then one would obtain

the mean value,  [EQUATION "mu sub x", Position:In Text] , the mean-square

value,  [EQUATION "psi sub x super 2", Position:In Text]  and the function

[EQUATION "G sub [xx] (f)", Position:In Text] .  Note that

[EQUATION "psi sub x super 2 #3 = #3 sigma sub x super 2 #3 + #3 mu sub x super

, so that for this particular sequence

[EQUATION "center psi sub x super 2 #3#3 = #3#3 sigma sub x super 2 #3#3 = #3#3

If these data are considered to be samples of a continuous signal

[EQUATION "x(t)", Position:In Text]  with sampling interval

[EQUATION "Delta t", Position:In Text] , then the power spectrum should look

like that shown in figure 7.1.

[PICTURE FIG71.CGM, Scale:0.500]

Figure 7.1  White noise spectrum with total power P

For convenience, it is useful to adjust the total power in the sequence in

order that the power density

[EQUATION "G sub [xx] (f) #3 = #3 1", Position:In Text]  for

[EQUATION "0 #3 <= #3 f #3 <= #3 f sub c", Position:In Text]  where

[EQUATION "f sub c #3 = #3 1 #1 / #1 2 Delta t", Position:In Text]  is the

Nyquist frequency.  To accomplish this, consider the sequence

[EQUATION "center y sub n #3#3 = #3#3 K #3 (x sub n #3#3 - #3#3 1 over 2) right

where  [EQUATION "x sub n", Position:In Text]  is a random number on the

interval from 0 to 1 as before and  [EQUATION "y sub n", Position:In Text]

ranges from  [EQUATION "- K #1 / #1 2", Position:In Text]  to

[EQUATION "K #1 / #1 2", Position:In Text] .  For this sequence, the mean and

variance are

[EQUATION "center mu sub x #3#3 = #3#3 0 #4#4 #R[and] #4#4 sigma sub x super 2

#

Now, choose  [EQUATION "P #3 = #3 1 #1 / #1 2 Delta t", Position:In Text], so

that

[EQUATION "center psi sub x super 2 #3#3 = #3#3 sigma sub x super 2 #3#3 = #3#3

From this, we find that

[EQUATION "K #3 = #3 [(6 over [Delta t]) super [1/2]]", Position:In Text] , and

for this value of  [EQUATION "K", Position:In Text] , the power density for the

sequence defined in equation 7.4 satisfies

[EQUATION "center G sub [yy] (f) #3#3 = #3#3 #( { stack left [[ 1 #4#4 0 <= f

<=

A plot of this power spectrum is shown in figure 7.2.

[PICTURE FIG72.CGM, Scale:0.500]

Figure 7.2  White noise spectrum with total power

[EQUATION "1 #1 / #1 2 Delta t", Position:In Text]

The final step is to create a new sequence

[EQUATION "z sub n", Position:In Text]  by passing the values

[EQUATION "y sub n", Position:In Text]  through a bandpass filter with transfer

function  [EQUATION "H(f)", Position:In Text] .  The power density for this new

sequence is given by

[EQUATION "center G sub [zz] (f) #3#3 = #3#3 | #1 H (f) #1 | super 2 #3#3 G sub

A recursive digital bandpass filter with

[EQUATION "| #1 H(f) #1 | #3 = #3 1", Position:In Text]  and described in

equation 3.15 has been used in the COLOR routine.  The user specifies the two

(-3 dB) points,  [EQUATION "f sub 1", Position:In Text]  and

[EQUATION "f sub 2", Position:In Text] , and the number of cascaded filter

stages to use.  The order of the filter is twice the number of cascaded

stages.  The output of the program is a sequence

[EQUATION "z sub n", Position:In Text]  with a power spectrum as shown in

figure 7.3.

[PICTURE FIG73.CGM, Scale:0.500]

Figure 7.3  Colored noise spectrum with total power

[EQUATION "(f sub 2 #3 - #3 f sub 1)", Position:In Text]

The color routine was used to create the sequence

[EQUATION "z sub n", Position:In Text]  (simulated sampling of the continuous

signal  [EQUATION "z(t)", Position:In Text] ) which was stored in a data file

that could be read by the spectrum routine.  The file contained 100 records

with 2048 data points per record.  The spacing between data points was 0.01

seconds.  The parameters for the bandpass filter were:

[EQUATION "f sub 1 #3 = #3 20 #3 Hz", Position:In Text] ,

[EQUATION "f sub 2 #3 = #3 30 #3 Hz", Position:In Text]  and five cascaded

stages for a tenth order filter.  A plot of a portion of the simulated time

series appears in figure 7.4.  The data file was processed by the spectrum

routine several times using various choices of tapering functions, and the

power spectra are plotted in figure 7.5.  When computing these spectra, no mean

removal or detrending was used, and no further filtering was performed.  As can

be seen,  [EQUATION "G sub [zz] (f) #3 = #3 1", Position:In Text]  in the

passband and rolls off to zero very rapidly elsewhere.  (Note that the base 10

logarithm of these numbers are plotted in order to magnify the rolloff

region.)  This plot clearly shows the advantages gained with the use of

tapering and overlapped processing as opposed to no tapering; however, in this

case, the choice of tapering function makes little difference.  Table 7.1 shows

the rms values reported for each case; all of these are within 0.6 % of the

theoretical value, and the minor variations are due to the fact that the

rolloff is not perfect.

[PICTURE FIG74.CGM, Scale:0.800]

Figure 7.4  Simulated time series  [EQUATION "z(t)", Position:In

Text]

Theory    No Taper   Hanning     Parzen     Tukey      Welch

rms values     3.16228    3.14463    3.14665    3.14617    3.14315

3.14429

error (%)        --        0.56       0.49       0.51       0.60       0.57

Table 7.1  RMS values reported by spectrum routine

[PAGE BREAK][WHITESPACE 0.25 in, Place All on Next Page]

[PICTURE FIG75.CGM, Scale:0.800]

Figure 7.5  Power spectra for colored noise; total power =

[EQUATION "root[10]", Position:In Text]

[PICTURE FIG76.CGM, Scale:0.800]

Figure 7.6  Effect of detrend on power spectra[PAGE BREAK]

For data generated by the computer, no linear trend in the data is to be

expected, and the detrending operation should not be used.  The indiscriminate

use of this operation can lead to added power at

[EQUATION "f #3 = #3 1 #1 / #1 N Delta t", Position:In Text]  since this

operation is applied record by record; this can only occur if the power at this

frequency is lower than the small amount of noise generated by the use of this

operation.  In almost every case where the random data originates from an

experiment (vis-a-vis computer generated perfect random data) this problem will

not occur, but the user should be aware of the possibility.  A plot of the

added power that occurs with the use of the detrending operation is shown in

figure 7.6.

The following is an example session showing the various prompts and

responses that were used with the color routine for this example.

color

Enter the low frequency cutoff : 20.0

(Defines passband)

Enter the high frequency cutoff : 30.0

Enter delta T seconds. (1.0/Sample Rate) : 0.01       (Sampling rate = 100 Hz)

Enter # of filter sections to cascade : 5             (Tenth order filter)

Enter # of points per record (N) : 2048               (Must be power of two)

Enter # of records (NUMREC) : 100                     (No limit on # records)

Enter a negative integer : -5                          (Required seed value

for)

(random number generator)

Enter output file name (4 chars) : COLR

C O L O R E D   N O I S E   U T I L I T Y

Creating COLR.DAT now.

Writing record 0100

Use voltage transformation factor = 20.0 / 4096.0

Program terminated successfully.

Next is an example session showing the prompts and responses needed to

process the COLR.DAT file using the spectrum routine.

spectrum colr

Transform into (V)oltages or into (O)ther quantity

or read (R)eal data which is already transformed : v

Enter VOFST 1) 10/4096, 2) 20/4096, 3) 2/4096 or 4) other : 2

Remove (M)ean value, (D)etrend or (N)either : n

Filter the data using (L)owpass, (H)ighpass

(B)andpass, Band(S)top or (N)one : n

How many channels of data (1 or 2) : 1

(A)mplitude, (P)ower or (C)ross spectrum : p

Taper the data (Y/N) : y

(H)anning, (P)arzen, (T)ukey or (W)elch window : h

Use overlap (Y/N) : y

Processing COLR.DAT now.

Record 0100  Ave = .15974E-03   Rms = 3.1035

File Totals : Ave = .26703E-05  Rms = 3.1446

Taper applied = Hanning.

50% overlap used.

0200 records will be written.

Tapering record 0200.

Transforming records 0199 and 0200.

Record 0200 was discarded.

Writing COLRP.PRN now.

mean         rms

2.18332E-06     3.1467

norm. random error = .10000

starting time :  09:15:01.71

ending time   :  09:16:06.68

elapsed time  :  00:01:04.97

Program terminated successfully.

Note that the maximum number of points per record is controlled by the

value of NMAX.  The value of NMAX is currently set to 16384.  (If the user

should ever desire to make a change to the value of NMAX, the change must be

made both in the COLOR routine and also in the SPECTRUM routine.  NMAX must be

the same number in both routines at all times.)

7.2 Sinusoidal Input

When developing the spectrum routine, it was felt that a simple test would

be to attempt to determine the Fourier coefficient (amplitude) of a sinewave by

computing its amplitude spectrum.  Although this can be done, it is not a

trivial procedure and is therefore repeated here as an example.  A sinewave

generation routine was written and is included on the diskette as SINE; this

routine creates a data file that can be read by the spectrum routine.  Each

record of this data file contains a portion of the time series

[EQUATION "center x(t,k) #3#3 = #3#3 A #3 sin #3 #[ 2 pi f sub 0 #1 t #3#3 +

#3#

where  [EQUATION "A", Position:In Text]  is the amplitude,

[EQUATION "f sub 0", Position:In Text]  is the frequency and

[EQUATION "theta sub k", Position:In Text]  is a phase angle which changes from

record to record and is a function therefore of the record number

[EQUATION "k", Position:In Text] .  The presence of a changing phase angle from

record to record is merely to simulate sampling of noncontiguous chunks of the

continuous function.  This phase angle will not alter the value of the

amplitude spectrum and is therefore left out of the following derivation for

simplicity.

The finite-range Fourier transform defined in equation 4.2 can be computed

for this function  [EQUATION "x(t)", Position:In Text]  and is given by

[EQUATION "center X(f,T) #3#3 = #3#3 [AT] over [2i] #3#3 [#[ #3 [sin #3 pi #3

(f

This function must now be evaluated at

[EQUATION "f #3 = #3 f sub 0", Position:In Text] , so taking the limit gives

[EQUATION "center lim below [f -> f sub 0] #3#3 X(f,T) #3#3 = #3#3 [AT] over

[2i

where the first term was evaluated using l'Hpital's rule.  The modulus of this

quantity is a function of the record length  [EQUATION "T", Position:In Text] ,

and is given by

[EQUATION "center | #3 lim below [f -> f sub 0] #3#3 X(f,T) #3 | #3#3 = #3#3

[AT

To further examine this dependence upon the record length, one can express

[EQUATION "T", Position:In Text]  in terms of  [EQUATION "n", Position:In Text]

, the number of cycles of the sinewave, using

[EQUATION "center T #3#3 = #3#3 N Delta t #3#3 = #3#3 n over [f sub 0] right

#R(

With this value for the record length, equation 7.12 becomes

[EQUATION "center | #3 lim below [f -> f sub 0] #3#3 X(f,T) #3 | #3#3 = #3#3

[AT

The function  [EQUATION "f #1 (#1 n)", Position:In Text]  is plotted in figure

7.7 and is shown to oscillate about the value one.  The extent of the

oscillation decreases with increasing  [EQUATION "n", Position:In Text] , and

in the limit as the record length goes to infinity, the function equals one.

[PICTURE FIG77.CGM, Scale:0.800]

Figure 7.7  Oscillatory function

[EQUATION "f #1 (#1 n)", Position:In Text]

Due to the finite nature of the signal, one can obtain the limiting value for

the Fourier coefficient only by paying careful attention to the choice of

record length.  Specifically, if the record length is chosen to contain an

integral number of cycles of the sinewave, then as can be seen from

figure 7.7,  [EQUATION "f #1 (#1 n) #3 = #3 1", Position:In Text]  for integer

[EQUATION "n", Position:In Text] .  Summarizing then, the limiting value of the

amplitude spectrum of the sinewave can be computed if the record length is

chosen to contain an integral number of cycles of the sinewave; when this

criterion is satisfied, then the modulus is given by

[EQUATION "center | #3 lim below [f -> f sub 0] #3#3 X(f,T) #3 | #3#3 = #3#3

[AT

Now, using the definition given in equation 4.28, the value of the amplitude

spectrum reported by the spectrum routine at the frequency

[EQUATION "f sub 0", Position:In Text]  will be the ensemble average over all

records of the data file of the modulus defined in equation 7.15.  Three

further constraints are placed upon the record length in addition to the

requirement of integral values of  [EQUATION "n", Position:In Text] :

[EQUATION "N", Position:In Text] must be a power of two,

[EQUATION "N", Position:In Text]  should be large for better accuracy and

[EQUATION "Delta t #3 <= #3 1 #1 / #1 [2 f sub 0]", Position:In Text]  which is

a statement of the fact that the frequency of the sinewave must be less than or

equal to the Nyquist frequency determined by the sampling interval.  These

requirements are satisfied if

[EQUATION "Delta t #3 = #3 1 #1 / #1 [2 super j #1 f sub 0]", Position:In Text]

,  [EQUATION "N #3 = #3 2 super k", Position:In Text]  and

[EQUATION "k >> j", Position:In Text]  for integer values of

[EQUATION "j", Position:In Text]  and  [EQUATION "k", Position:In Text] ; for

these choices  [EQUATION "n #3 = #3 2 super [#3 k-j]", Position:In Text] .

Generally, the best accuracy is obtained when

[EQUATION "n #3 = #3 N #1 / #1 4", Position:In Text]  (

[EQUATION "j #3 = #3 2", Position:In Text] ).

As an example of this technique, the sinewave

[EQUATION "center x(t,k) #3#3 = #3#3 1.25 #3#3 sin #3#3 #[ 2 pi #3 (128.0) #3 t

was created using the SINE routine.  The file contained ten records with 8192

points per record.  The sampling interval was chosen to be (using

[EQUATION "n #3 = #3 N #1 / #1 4", Position:In Text] )

[EQUATION "center Delta t #3#3 = #3#3 1 over [(4)(128)] #3#3 = #3#3 0.001953125

so that the record contained exactly 2048 cycles of the sinewave.  The spectrum

should have a peak at a frequency of 128 Hz where the amplitude is given by

[EQUATION "center F sub x (128) #3#3 = #3#3 lim below [T -> infinity] #3#3 E #3

The actual value reported by the spectrum routine was 9.9656 at a frequency of

128.0082 Hz which was the closest discrete frequency value.  A plot of the

amplitude spectrum is shown in figure 7.8.  When computing this spectrum, no

mean removal or detrending was used, and no filtering was performed.

Furthermore, application of a tapering function would significantly change the

expected amplitude spectrum and therefore no tapering was used.

[PICTURE FIG78.CGM, Scale:0.800]

Figure 7.8  Amplitude spectrum of sinewave

The required inputs to the SINE routine follow.

sine

Enter the amplitude of the sinewave : 1.25            (gives a simple answer)

Enter the frequency of the sinewave : 128.0           (can be any desired

number)

Enter the phase (in degrees) : 32.8                   (choose any initial

value)

Enter DC value : 0.0                                   (no DC for this example)

Enter delta T secs. (1.0/Sample Rate) : 0.001953125   (according to eq. 7.17)

Enter # of points per record (N) : 8192               (must be power of 2)

Enter # of records (ASIZE) : 10                       (OK for this example)

Enter output file name (4 chars) : SINE

S I N E W A V E   C R E A T I O N   U T I L I T Y

Creating SINE.DAT now.

Writing record 0010

Use voltage transformation factor = 20.0 / 4096.0

Program terminated successfully.

The required inputs to the spectrum routine to create the amplitude spectrum

given in figure 7.8 follow.

spectrum sine

Transform into (V)oltages or into (O)ther quantity

or read (R)eal data which is already transformed : v

Enter VOFST 1) 10/4096, 2) 20/4096, 3) 2/4096 or 4) other : 2

Remove (M)ean value, (D)etrend or (N)either : n

Filter the data using (L)owpass, (H)ighpass

(B)andpass, Band(S)top or (N)one : n

How many channels of data (1 or 2) : 1

(A)mplitude, (P)ower or (C)ross spectrum : a

Taper the data (Y/N) : n

Processing SINE.DAT now.

Record 0010 : Ave = .00000  Rms = .88208

File Totals   : Ave = .00000  Rms = .88090

No tapering applied.

0010 records will be used.

Transforming records 0009 and 0010.

Writing SINEA.PRN now.

starting time :  20:24:53.77

ending time   :  20:25:13.21

elapsed time  :  00:00:19.44

Program terminated successfully.

Note that the maximum number of points per record is controlled by the

value of NMAX.  The value of NMAX is currently set to 16384.  (If the user

should ever desire to make a change to the value of NMAX, the change must be

made both in the SINE routine and also in the SPECTRUM routine.  NMAX must be

the same number in both routines at all times.)[PAGE BREAK]

7.3 Experimental Noise Input

The input data file for this example was derived from an experiment

employing an electrodynamic shaker table.  This device is essentially a large

solenoid mated to an electronic feedback system which is used to accurately

control the oscillatory motion of the core.  The displacement amplitude and

frequency can be set to desired levels and then maintained very accurately.

This instrument was used in an experiment designed to test the frequency

response of an optical system which was built to measure displacement of a

surface at a point.  Prior to the actual frequency response test, the optical

system was used to measure the displacement of the core of the solenoid with

the shaker table energized but undergoing no motion.  The computation of a

power spectrum of this displacement noise can then be a useful diagnostic tool

for the identification of noise sources in subsequent displacement spectrum

measurements.

The signal was digitized using a 12-bit A/D converter with an input range

of -10 to 10 V.  The sampling rate was 2 kHz which resulted in a sampling

interval of  [EQUATION "Delta t #3 = #3 0.0005", Position:In Text]  seconds.

80 records of data were acquired with 2048 points per record.  The power

spectrum was computed for a range of frequencies up to the Nyquist frequency

which was 1 kHz; the resolution was given by

[EQUATION "Delta f #3 = #3 1 #1 / #1 N Delta t #3 = #3 0.97656 #3 Hz",

Position:

.  All power spectra shown in this section were computed by first detrending,

then tapering the input time records with the Welch taper and employing 50%

overlap.  The resulting power spectrum is shown in figure 7.9; logarithmic

scales are used for greater clarity.  The figure reveals that most of the power

in the signal lies below 100 Hz.  A 60 Hz contribution attributable to the

power supply may be identified as well as other resonances and their harmonics.

[PICTURE FIG79.CGM, Scale:0.800]

Figure 7.9  Power spectrum of displacement noise

This input file allows a useful demonstration of the filtering

capabilities of the spectrum routine.  Shown in Figure 7.10 are power spectra

of the same input file for which figure 7.9 was obtained; however, prior to the

spectral calculation, each of the four types of filters available for use were

applied to the data.  In each case five cascaded filter stages were used

providing a tenth order digital Butterworth filter.  As can be seen, the

rolloff is extremely rapid with power in the stopband 3 to 4 decades below that

in the passband.  This is an effective means for removing undesirable frequency

components from the data prior to spectral computation.  This can also be

useful for quickly estimating the amount of power contributed by specific peaks

or regions of the spectrum.  The rms value, calculated as the square root of

the area under the spectral density curve (according to eq. 4.13), is given in

table 7.2 for each of the filter cases shown in figure 7.10 and for the

no-filter case plotted in figure 7.9.

No filter   Lowpass    Highpass   Bandpass   Bandstop

rms values     0.014803   0.013319   0.009529   0.005894   0.010116

Table 7.2  RMS values reported after application of each filter type

The table indicates that about 10% of the contribution to the root-mean-square

value of the signal may be found above 100 Hz, about a third lies below 30 Hz

and just under half may be found between 50 and 100 Hz.  However, adding the

rms value for the bandpass case to the rms value for the bandstop case does not

equal the rms value for the no filter case.  This discrepancy is due to the

fact that some power is in the stopband for the bandpass and bandstop cases,

and this contributes a small amount to the rms values indicated.  Therefore,

this method should only be used for rough estimates; the most accurate method

is to integrate the spectral density function over frequency bands of interest.

[PAGE BREAK]

[WHITESPACE 4 in, Place All on Next Page]

Substitute Figure 7.10 for this page.[PAGE BREAK]

7.4 Sampling Considerations

A full discussion of sampling procedures will not be reiterated here; a

complete discussion of these techniques can be found in the references, and the

interested reader is urged to look there.  Instead, this section attempts to

present some of the considerations necessary when making choices among the

various sampling parameters, and the manner in which these choices affect the

spectral density results.

The most important parameter is probably the choice of sampling interval

[EQUATION "Delta t", Position:In Text] .  In general, one attempts to choose

this quantity to be of the order of the smallest time scale of interest in the

physical phenomenon under investigation.  The selection of

[EQUATION "Delta t", Position:In Text]  determines the largest frequency

(bandwidth) of the resulting spectral analysis.  This critical frequency

[EQUATION "f sub c", Position:In Text]  is known as the Nyquist frequency and

is given by

[EQUATION "center f sub c #3#3 = #3#3 1 over [2 Delta t] right #R(7.19)"]

Another quick way for determining the Nyquist frequency is to note that

[EQUATION "f sub c #3 = #3 f sub s #1 / #1 2", Position:In Text]  where

[EQUATION "f sub s #3 = #3 1 #1 / #1 Delta t", Position:In Text]  is the

sampling rate at which data are acquired.  If energy exists in the signal above

the Nyquist frequency determined by the desired sampling rate, then this energy

must be removed by means of an analog lowpass filter prior to A/D conversion.

This is necessary to prevent aliasing of the high frequency components in the

data.  Aliasing is essentially confusion among the high and low frequency

components in the data that occurs whenever the sampling rate is set too low.

A very good discussion of aliasing may be found in Bendat and Piersol (1986).

Another parameter of interest is the number of data points per record per

channel  [EQUATION "N", Position:In Text] .  This quantity combined with the

choice of  [EQUATION "Delta t", Position:In Text]  determines the record

length  [EQUATION "T #3 = #3 N Delta t", Position:In Text]  and the resolution

bandwidth of the spectral analysis,

[EQUATION "center Delta f #3#3 = #3#3 1 over [N Delta t] right #R(7.20)"]

A large value of  [EQUATION "N", Position:In Text]  for a given value of

[EQUATION "Delta t", Position:In Text]  gives a smaller

[EQUATION "Delta f", Position:In Text]  and can give a more detailed spectral

density estimate.  On the other hand, if the record length

[EQUATION "T", Position:In Text]  is very long, realizability problems may

occur: is the physical phenomenon stationary for these long time periods?  Is

there a limitation on maximum file size?  Can the resulting massive amounts of

data be adequately stored?

Another important parameter is the number of records

[EQUATION "n sub r", Position:In Text]  of length

[EQUATION "T", Position:In Text]  to sample for each data file.  These records

should be contiguous in time (in order to use overlapping) and constitute an

ensemble of statistically stationary records; the expected value operation

contained in the definitions of the spectral density estimates is approximated

when an ensemble average is calculated for this set.  Recall, that it is

[EQUATION "n sub r", Position:In Text]  (the number of records before

overlapping) which is used in the determination of the errors associated with

each of the spectral density estimates and not  [EQUATION "N", Position:In

Text]

or  [EQUATION "Delta t", Position:In Text] .  Increasing

[EQUATION "n sub r", Position:In Text]  rapidly decreases the normalized random

error  [EQUATION "epsilon", Position:In Text]  up to about

[EQUATION "n sub r #3 = #3 100", Position:In Text]  or so; for larger values

of  [EQUATION "n sub r", Position:In Text]  the error decreases much more

slowly.  There may be certain situations in which, for a given overall file

size, it is better to increase  [EQUATION "n sub r", Position:In Text]  and

decrease  [EQUATION "N", Position:In Text]  for a given

[EQUATION "Delta t", Position:In Text]  in order to increase accuracy at the

expense of resolution bandwidth  [EQUATION "Delta f", Position:In Text] .

^S" Utility Routines

Four utility programs are included on the diskette.  The SINE and COLOR

routines have been discussed in the previous section concerning examples.

MAKDAT is a routine designed to read a data file in ASCII format and then to

write the necessary file header and data to an unformatted file.  MAKCOEF is

similar; it reads a user-prepared ASCII format coefficient data file and then

writes the file header and coefficients to an unformatted file.  It is easiest,

of course, to add the necessary code to create the unformatted files directly

to the routine that is used to create (or collect) the data in the first place;

however, if the user is unfamiliar with unformatted FORTRAN data files or the

FORTRAN language in general, then these routines attempt to solve the problem.

8.1 MAKDAT routine

In order to use MAKDAT, the data to be analyzed by the spectrum routine

must be generated, or collected by an A/D converter, and then stored to a file

in ASCII format.  In this format the file may be viewed by any ASCII editor,

can be displayed on the screen and can be printed on a printer.  This is not an

optimum way to store data; these files tend to be extremely large.  In

anticipation of this fact other types of data files were explored for possible

use, and unformatted FORTRAN files appeared to be the simplest solution.

Unformatted files are typically a factor of five smaller in size than the

equivalent ASCII file.  An example will be given to illustrate the means for

transferring the data to unformatted files which can then be used by SPECTRUM.

Suppose that 100 records of one channel of data saved as 2-byte integers

and sampled at 10000 samples per second with 2048 data points per record have

been acquired.  These data should be placed in an ASCII file with the file

extension [.ASC] and with a file name conforming to the various naming

conventions discussed earlier.  The file must contain numbers only; the numbers

may be written one per line or many per line (but this does not shorten the

length of the file).  Blank lines may be added if desired.  The first number in

the file should be the first data point of the first record; the 2049th number

in the file should be the first data point of the second record, and the last

number in the file should be the 2048th data point of the 100th record.  A

session with MAKDAT to accomplish this transformation would proceed as follows.

makdat

(I)nteger (2-byte) or (F)loating-point (4-byte) data : i

Enter # of channels (1 or 2) : 1

One channel  : Total # points per record <= 16384.

Two channels : Total # points per record per channel <= 8192.

Enter # of points per record (power of two) : 2048

One channel  : Must be EVEN # of records.

Two channels : May be EVEN or ODD # of records.

Enter # of records in the data file : 100

One chan    : Delta t is spacing between data points.

Two chans : Delta t is spacing between data pts - SAME channel

Delta t divided by 2 is spacing between data pts

- different channels.

Enter sampling interval delta t (secs) : 0.0001

Enter ASCII input file name (4 chars) : test

D A T A   F I L E   C R E A T I O N   U T I L I T Y

Creating TEST.DAT now.

# channels = 1

record size = 4096 bytes

# of records = 100

delta t = 100 microseconds

Record 0100

Program terminated successfully.

The MAKDAT routine reads the user-prepared TEST.ASC file and calculates the

values needed for the file header.  The file header would consist of

[EQUATION "center 1 #4#4 4096 #4#4 100 #4#4 100 #4#4"]

The routine then transfers the data to the unformatted file with the name

TEST.DAT.  Finally, to process this file with the spectrum routine the user

would enter: spectrum test.

For two channels of data, it is necessary that the numbers be stored in

each record of the ASCII file in the following manner:

[EQUATION "center #R[one] #4 #R[channel] #^ #R[:] #4#4 x sub 0 #3 x sub 1 #3 x

s

using the notation defined earlier.  Therefore, the first number in the file

will be the first data point of the first record for channel 1; the second

number will be the first data point of the first record for channel 2; the

third number in the file will be the second number for the first record of

channel 1, and so on.  Also, care should be taken to enter correct responses to

the prompts when two channels of data are to be transferred.  In response to

the number of points per record per channel prompt, the user would enter 1024

points if the data file consisted of two channels of data with 2048 points per

record and therefore 1024 points for each channel in each record.  Note that

the maximum number of points per record is controlled by the value of NMAX; the

maximum number of points per record per channel is then NMAX / 2.  The value of

NMAX is currently set to 16384.  (If the user should ever desire to make a

change to the value of NMAX, the change must be made both in the MAKDAT routine

and also in the SPECTRUM routine.  NMAX must be the same number in both

routines at all times.)  Similarly, when responding to the prompt for the

sampling interval, the user would enter 0.0002 seconds for the sampling

interval between data points of the same channel for the case in which the data

were sampled at 10000 samples per second.  Although, 0.0001 seconds is the

sampling interval between adjacent data points in the record, the sampling

interval for data points of the same channel is a number twice as large.

8.2 MAKCOEF routine

The primary reason for using unformatted coefficient data files with the

spectrum routine is to allow the user a means for preparing all of the

coefficient data for the transformation of voltages into other dimensional

quantities for several files which will then be processed by SPECTRUM in a

batch.  The MAKCOEF utility allows the user to prepare this coefficient data in

an ASCII file; the utility will write the file header and then transfer the

data to an unformatted coefficient data file which can be read by the spectrum

routine.

Suppose that it was desired to process the data files D061.DAT thru

D086.DAT in a batch by the spectrum routine.  Each of these data files

consisted of two channels of 2-byte integers taken from an experiment in fluid

mechanics, say.  Furthermore, suppose that the quantities sampled were two

components of a fluid velocity, which were transduced into voltages by some

means and then sampled by an A/D converter.  The user knows the calibration

data for the transducer, and the required transformation from voltages back

into velocities can be accomplished for each velocity component by using a

polynomial of up to fourth degree in the form:

[EQUATION "center u sub n #3#3 = #3#3 b sub 0 #3 + #3 b sub 1 v sub n #3 + #3 b

where  [EQUATION "v sub n", Position:In Text]  is the nth data point expressed

as a voltage,

[EQUATION "b sub 0 #1 , #3 b sub 1 #1 , ... , b sub 4", Position:In Text]  are

the coefficients of the velocity transformation and

[EQUATION "u sub n", Position:In Text]  is the nth data point which has been

converted to a velocity.  Now, suppose further that the data in each of the

files were taken over a period of time in which the calibration coefficients

changed; thus, a different set of coefficients are required for each of the 26

files to be processed.  Following the instructions in the section on

coefficient data files, this scenario would require the creation of two

unformatted coefficient data files, DCON.DAT and DCON2.DAT, each containing the

26 sets of five coefficients

[EQUATION "b sub 0 #1 , #3 b sub 1 #1 , ... , b sub 4", Position:In Text]  for

the velocity transformation for channel 1 and channel 2, respectively.

The user should create each ASCII data file with a four character name

followed by the extension [.ASC].  The names should conform to the various

naming conventions specified earlier.  The ASCII file must contain numbers

only, although blank lines may be inserted into the file if desired.  The

numbers will be transferred to 4-byte floating point quantities in the

unformatted data file thereby limiting the coefficients to single precision (6

or 7 decimal places).  The coefficients may be stored one per line or many per

line in the ASCII file.  The first number in the file will be

[EQUATION "b sub 0", Position:In Text]  for channel 1 of data file D061.DAT;

the fifth number in the file should be  [EQUATION "b sub 4", Position:In Text]

for channel 1 of data file D061.DAT; the sixth number in the file will be

[EQUATION "b sub 0", Position:In Text]  for channel 1 of data file D062.DAT,

and so on.  The 130th and last number in the file should be

[EQUATION "b sub 4", Position:In Text]  for channel 1 of data file D086.DAT.

The user should then create a second ASCII file in a similar manner specifying

all of the coefficients for channel 2 of the 26 files to be processed.  The

utility MAKCOEF should then be executed twice, first transferring the contents

of the first ASCII file (for channel 1) to DCON.DAT and then for the second

ASCII file (for channel 2) to DCON2.DAT.  A session using MAKCOEF would proceed

as follows.

makcoef

Enter first letter of data file to

which these coefficients will be associated : d

Are these coefficients for channel (1 or 2) : 1

Enter # of sets of coefficients : 26

Enter starting file number : 61

Enter ASCII input file name (4 chars) : chn1

C O E F F I C I E N T   F I L E   C R E A T I O N   U T I L I T Y

Creating DCON.DAT now.

# sets of coeffs = 26

# coeffs in each set = 5

# of starting file = 61

Program terminated successfully.

The MAKCOEF routine will read the ASCII file CHN1.ASC and will write the

following file header to the unformatted file DCON.DAT:

[EQUATION "center 26 #4#4 61"]

The coefficients for channel 1 are then transferred to the unformatted file.

Note that 5 coefficients per set are required; therefore, this number is not

written to the file header.  If, at a later time, the user wishes to reprocess

the files D074.DAT through D080.DAT (perhaps specifying different options in

the spectrum routine), it is not necessary to re-create the two coefficient

data files; as long as the data files to be reprocessed are chosen from among

the 26 original members of the set, the spectrum routine will use only the

coefficients that it needs.  Similarly, if the user wishes to process the data

files in an arbitrarily numbered order (not consecutively), the coefficient

data files do not need to be changed as long as the data files come from the 26

original members of the set.  Thus, D061.DAT, D064.DAT, D069.DAT and D086.DAT

could be processed by SPECTRUM without changing the coefficient data files.

The converse is also true, however.  If the user desires at the outset to

process only the four arbitrarily ordered files above, two coefficient data

files must be created which contain not 4, but 26 sets of 5 coefficients for

the data files D061.DAT through D086.DAT.  The spectrum routine requires that

sets of coefficients in the coefficient data file be listed for consecutively

numbered files even if not all of the sets of coefficients are used.  With this

in mind, it is perhaps best to create the coefficient data files for an entire

set of input data files; then members of the set can be processed by the

spectrum routine either consecutively or in an arbitrarily numbered order at

any subsequent time.  Of course, the user could transform the voltages back

into dimensional quantities using his/her own code and store the resulting

floating-point numbers into ASCII data files (then use MAKDAT) or directly into

unformatted data files to be processed by SPECTRUM; then, all of this mess

concerning coefficient data files could be avoided altogether.

Note that the maximum number of points per record is controlled by the

value of NMAX.  The value of NMAX is currently set to 16384.  The maximum

number of files that may be processed by SPECTRUM in a batch is controlled by

the setting of IFMAX; this is currently set to 100.  (If the user should ever

desire to make a change to the value of NMAX or IFMAX, the change must be made

both in the MAKCOEF routine and also in the SPECTRUM routine.  NMAX and IFMAX

must be the same numbers in both routines at all times.)

^SReferences

[WHITESPACE 0.1 in, Place All on Next Page]

Bendat, J.S. and Piersol, A.G. 1986 Random Data, 2nd ed., John Wiley & Sons,

Inc., New York.

Hess, D.E. 1990 An experimental investigation of a compliant surface beneath a

turbulent boundary layer. Ph.D. thesis, Johns Hopkins University.

Microsoft Fortran Optimizing Compiler, Version 5.00, Microsoft Corporation,

Redmond, Washington, 1989.

Press, W.H., Flannery, B.P., Teukolsky, S.A. and Vetterling, W.T. 1986

Numerical Recipes, Cambridge University Press, New York.

Sirivat, A. 1985 Statistical Package for the Analysis of Data (SPAD), private

communication.

Stearns, S.D. 1975 Digital Signal Analysis, Hayden Book Co., Rochelle Park, New

Jersey.