___________________________________________________________________________

STAC FAX                                                           STAC FAX

Subject: Recovery From a Failed Stacker Installation on an IBM PS/1

STACKER NOTE TEC027 - 2/20/92                          STAC FAX Index #2505

___________________________________________________________________________

NOTE:  If Stacker has not yet been installed, see Tec020: Installing

Stacker on an IBM PS/1.

Background.

The PS/1 is a ROM-based machine.  It normally boots from its

"Built-In-DOS" on the ROM D: drive and  reads Config.sys and Autoexec.bat

from there as well.  The PS/1 can be reconfigured, using"CUSTOMIZE", to

read Config.sys and Autoexec.bat from C: so that the Stacker device

drivers may be loaded.  If the boot process is not modified using the

"CUSTOMIZE" program before loading Stacker, the data may appear to be

"missing" because the STACKER.COM device driver has not been loaded.

A "4 Quadrant" system menu is normally loaded from ROM upon boot. It

consists of:

1. Information

2. Microsoft Works

3. Your Software

4. IBM DOS

You can shell to a DOS prompt by pressing <shift F9>. The "4 Quadrant Menu"

is installed in Config.sys: "INSTALL=D:\SHELLSTB.COM".

"CUSTOMIZE How System Starts" is a program normally in the C:\DOS

directory, but is copied into the Stacker drive upon "STACKING... the

existing drive".  The program also should be on the DOS diskettes that

came with the machine.  In the Customize program, you can configure the

computer to read the system files, Config.sys, and Autoexec.bat  from C:

or D:

Note: THE CUSTOMIZE DEFAULTS CAN BE RESTORED BY POWERING DOWN THE SYSTEM,

HOLDING DOWN THE 2 MOUSE BUTTONS, AND POWERING BACK UP.

Recovery.

Note:  You MUST have the DOS program diskette containing CUSTOMIZ.EXE

or a bootable floppy disk to proceed.  Nothing further can be done

until you have one of these!

If you have the DOS program diskette.

1. Insert the DOS diskette in A: and Run the CUSTOMIZE program from the

Dos Quadrant.  (You can also press <shift> F9 to get to a prompt and run

A:CUSTOMIZ.)

If you do not have the DOS diskette but have a bootable system diskette.

1. Insert the system diskette in A: and from the system menu press

Alt+SysRq to force the system to boot from A: (the D: Rom drive

will not be accessible after this.)

2. Copy config.sys to A: and reboot

3. From the DOS directory on the Stacker drive (C:), run CUSTOMIZ.

Running CUSTOMIZE:

1. The third line from the bottom of the screen says "Read Config.sys:".

Click on "from Disk"

2. The next line down says " Read Autoexec.bat". Click on "From Disk"

3. The last line says "Disk to Read from". Click on C:

4. Press <ENTER> to save changes and exit.

5. Reboot the machine.

6. This will bring you to a C: prompt.  Edit the Config.sys on C:\ and

add the line: Install=D:\SHELLSTB.COM to restore the system Quadrant menu.

7. Restart the system for changes to take affect.

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712