Current Video and Printer Drivers

=================================

VIDEO DRIVERS: as of February, 1993

=============

This is a current listing of the video drivers included with CompuShow.

-  16 color 1024x768 requires a 512K VGA,

- 256 color  640x480 requires a 512K VGA,

- 256 color 1024x768 reguires a   1M VGA, and

- 256 color modes on an 8514/A adapter require the memory expansion.

Also, VGA adapters  that are built-in  to a  system board or  bundled with  a

computer (unless you're getting a specific name-brand board), are  frequently

low-end (cheap) adapters, and  probably don't have "Super-VGA"  capabilities.

Finally, remember that all  IBM standard   graphics modes are  'built-in'  to

the program. If your graphics adapter  doesn't go beyond the standard  modes,

then you don't need a video driver.

A2=ATI GraphicsSolution

320x200x16 640x200x16

A3=Ahead Systems

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

A4=Amstrad PC1512

640x200x16

A5=ATI VGAs

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

640x480x32k

A6=AT&T 6300

640x400x2 640x400x16

A7=AcuMos

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256

A8=AT&T DEB

640x400x16

C1=Chips & Technologies

800x600x16 640x400x256 640x480x256 800x600x256

C2=Compaq QVision

640x480x256 1024x768x256 640x480x32k 512x480x16m 640x400x16m

C3=Cirrus

640x400x16 720x540x16 800x600x16

D1=DGIS

[Available modes depend on the video adapter and DGIS interface]

D2=Diamond Speedstar 24

640x400x256 640x480x256 800x600x256 1024x768x256 640x480x32k 800x600x32k

640x480x16m

[DRIVERS.DOC] Page 1

E1=Everex VGAs

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

G1=Genoa 6400 GVGA

800x600x16 1024x768x16 512x512x256 640x350x256 640x400x256 640x480x256

800x600x256

H1=Hercules GraphxStatn

640x400x256 640x480x256 800x600x256 1024x768x256 1024x768x256 640x480x32k

512x480x16m

[The first 1024x768x256 is interlaced, the second is non-interlaced]

H2=Hercules InColor

720x348x16

I1=IBM (generic) CGA

160x100x16

I3=IBM PGC

640x480x256

I4=IBM 8514/A [single]

640x480x16 1024x768x16 640x480x256 1024x768x256

I5=IBM 8514/A [dual]

640x480x16 1024x768x16 640x480x256 1024x768x256

[Load the "adapter interface" (HDILOAD) in AUTOEXEC.BAT]

I7=IBM XGA

1024x768x16 640x480x256 1024x768x256 640x480x64k

N1=NCR

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

O1=Oak Technologies

800x600x16 1024x768x16 640x480x256 800x600x256 1024x768x256

[see "CAUTION", below]

P1=Paradise/WestrnDigtl

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256

P2=Plantronics

320x200x16

S1=STB EM16+

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

S2=STB Ergo+Sierra DAC

640x400x256 640x480x256 800x600x256 1024x768x256 320x200x32k 640x480x32k

800x600x32k

T1=Tseng 3000

800x600x16 1024x768x16 640x350x256 640x480x256 800x600x256

T2=Tandy 1000SL/TL

640x200x16

[DRIVERS.DOC] Page 2

T4=Tseng 4000

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

T5=Trident VGA

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

T6=Tseng4000+Sierra DAC

640x400x256 640x480x256 800x600x256 1024x768x256 640x400x32k 640x480x32k

800x600x32k

T7=TIGA

[Available modes depend on the video adapter and TIGA interface]

V1=Video 7

800x600x16 1024x768x16 640x400x256 640x480x256 800x600x256 1024x768x256

V2=VESA BIOS Extensions

800x600x16 1024x768x16 1280x1024x16 640x400x256 640x480x256 800x600x256

1024x768x256 1280x1024x256 640x480x32k 800x600x32k 1024x768x32k

1280x1024x32k 640x480x64k 800x600x64k 1024x768x64k 1280x1024x64k

640x480x16m 800x600x16m 1024x768x16m 1280x1024x16m

[Maximum of FIFTEEN of the above modes. NOTE: If there are fewer than 15

modes and your "VESA compliant" VGA should support mode(s) that don't appear

in CompuShow, contact the BOARD MANUFACTURER. The reason they're not shown

is because the video BIOS (the software part of the board) says it can't do

those mode(s), so the "fix" HAS TO COME FROM THE BOARD MANUFACTURER!]

CAUTION: Problems with Oak VGAs

-------------------------------

There seems to be a serious problem with the Oak BIOS, and if you try to use

CSHOW to display  an Oak mode  which isn't  available on your  VGA, it  will

still  attempt  to  set  the  graphics  mode,  and  throw  the  VGA  into  a

configuration where it will no longer  do ANY extended graphics modes  until

you re-boot! To work around this Oak BIOS bug:

- SETUP O2 K

- Use CSHOW to display a graphic in each video driver mode (the letters  K,

L, M, etc.) UNTIL YOU ENCOUNTER A MODE THAT DOESN'T WORK.

- Re-boot the computer and continue testing.

- Once you have a list of the modes  which DO WORK, do a setup using  those

letters: SETUP O2 KLMN

[DRIVERS.DOC] Page 3

Printer Drivers: as of February, 1993

----------------

Note that the <Q>uality  print mode of MOST  COLOR PRINTERS and the  LASERJET

and DESKJET at 300dpi  requires MORE THAN 640k  of memory, as CompuShow  must

expand, rescale and dither the entire image to the 'virtual screen'  (VScreen

on the Shift-F1 box), before it starts to print. This means if you don't have

EMS or XMS memory, you'll be limited to the <F>ast print mode.

The following printer drivers are included with the enhanced program:

Color Printer Drivers

=============================================================================

IBM Color Printer         Vertical dot spacing is 84 dpi.

Single Density: esc,'K',n1,n2         Prints at 84 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 168 dots/inch.

=============================================================================

Color 8 pin (EX-800)      GENERIC 8 pin color driver for 24/216" graphics

line spacing. Tested on a Star NX-1000 Rainbow and

Citizen MSP-50. Similar to EX-800, LX-800, FX-86e,

LX-86, LX-80.

Single Density: esc,'*',0,n1,n2       Prints at 60 dots/inch.

Dual Density:   esc,'*',1,n1,n2       Prints at 120 dots/inch.

=============================================================================

Color 8 pin (JX-80)       Identical to COLOR8A, except for 20/180" line

spacing. Similar to JX-80, LQ-1500, ALPS ALQ200.

Single Density: esc,'*',0,n1,n2       Prints at 60 dots/inch.

Dual Density:   esc,'*',1,n1,n2       Prints at 120 dots/inch.

=============================================================================

Color 24 pin (LQ-2550)    GENERIC 24 pin color printer for 24/216" graphics

line spacing. Similar to LQ-2550.

Single Density: esc,'*',0,n1,n2       Prints 8 pins at 60 dots/inch.

Dual Density:   esc,'*',33,n1,n2      Prints 24 pins at 120 dots/inch.

CRT III:        esc,'*',38,n1,n2      Prints 24 pins at 90 dots/inch.

=============================================================================

Okidata 290 series color  Tested on a model 292.

Single Density: esc,'K',n1,n2         Prints at 60 dots/inch.

Double density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

HPPAINT.DRV               The Hewlett Packard PaintJet.

(mono fast)     esc,'*t90R',esc,'*r1U'   90 dpi monochrome

(mono quality)  esc,'*t180R',esc,'*r1U' 180 dpi monochrome

(color fast)    esc,'*t90R',esc,'*r3U'   90 dpi  8 colors

(color quality) esc,'*t90R',esc,'*r4U'   90 dpi 16 colors

[DRIVERS.DOC] Page 4

Monochrome Printer Drivers

=============================================================================

IBM/EPSON 8pin (n/216")   a GENERIC driver for 8 pin printers that specify

line spacing in multiples of 1/216"

Single Density: esc,'K',n1,n2         Prints at  60 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

IBM/EPSON 8pin (n/180")   a GENERIC driver for 8 pin printers that specify

line spacing in multiples of 1/180"

Single Density: esc,'K',n1,n2         Prints at  60 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

Citizen MSP 10/15/20/25   a GENERIC driver for 8 pin printers that support 72

and 144 dpi.

One-to-one      esc,'*',5,n1,n2       Prints at  72 dots/inch.

Two-to-one      esc,'*',7,n1,n2       Prints at 144 dots/inch.

=============================================================================

KX-P2624 (24 pin: LQ1050) for 24 pin Epson compatible printers.

CRT II:         esc,'*',6,n1,n2       Prints 8 pins at 90 dots/inch.

Double Density: esc,'*',33,n1,n2      Prints 24 pins at 120 dots/inch.

=============================================================================

KX-P2624 (ProPrntr XL24E) for the IBM Pro Printer XL24E and compatibles.

Double Density: esc,'[g',n1,n2,1      Prints 8 pins at 120 dots/inch.

Double Density: esc,'[g',n1,n2,9      Prints 24 pins at 120 dots/inch.

=============================================================================

HP DeskJet 75/300 dpi     prints slightly darker than the LaserJet driver.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

300 dpi         esc,'*t300R'          Prints at 300 dots/inch.

=============================================================================

HP LaserJet 75/150 dpi    for Laser Jets with limited memory installed.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

150 dpi         esc,'*t150R'          Prints at 150 dots/inch.

=============================================================================

HP LaserJet 75/300 dpi    for Laser Jets with adequate memory installed.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

300 dpi         esc,'*t300R'          Prints at 300 dots/inch.

=============================================================================

HP Thinkjet [MDL 2225C]   supports 96 and 192 dpi.

Single Density: esc,'K',n1,n2         Prints at  96 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 192 dots/inch.

=============================================================================

IBM Compact               has one graphics mode, at 70 dpi

Single Density: esc,'L',n1,n2         Prints at 70 dots/inch.

=============================================================================

NEC 8023                  May also work with the C.Itoh Prowriter.

Bit-Image:      esc,'S',a1,a2,a3,a4   Prints at 80 dots/inch.

=============================================================================

Okidata ML92/ML93         prints 7 pin graphics.

10 pitch:       30,3                  Prints at 60 dots/inch.

12 pitch:       28,3                  Prints at 72 dots/inch.

[DRIVERS.DOC] Page 5