----------------------------------------

CADET -the Cad Text Editor

Version 4.0  User's Manual

----------------------------------------

Copyright 1990, 1991, 1992 Steve Waskow

ALL RIGHTS RESERVED

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

----------------------------------------

Wasco Technical Software

1211 Grand Junction Drive

Katy, TX  77450

----------------------------------------

Phone/Fax: (713) 392-1359     CompuServe: 72020,2031

------------------------------------------------------------------

DISCLAIMER AND LIMIT OF LIABILITY

------------------------------------------------------------------

STEVE WASKOW AND WASCO TECHNICAL SOFTWARE MAKES NO WARRANTY OF ANY

KIND, EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE, WITH RESPECT TO THIS SOFTWARE AND ACCOMPANYING

DOCUMENTATION.

IN NO EVENT SHALL STEVE WASKOW OR WASCO TECHNICAL SOFTWARE BE

LIABLE FOR ANY DAMAGES (INCLUDING DAMAGES FOR LOSS OF BUSINESS

PROFITS, BUSINESS INTERRUPTION, LOSS OF BUSINESS INFORMATION, OR

OTHER PECUNIARY LOSS) ARISING OUT OF THE USE OF OR INABILITY TO

USE THIS SOFTWARE, EVEN IF STEVE WASKOW OR WASCO TECHNICAL

SOFTWARE HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

------------------------------------------------------------------

COPYRIGHT NOTICE AND TRADEMARKS

------------------------------------------------------------------

CADET  and  CADET - the CAD Text Editor

are trademarks of Wasco Technical Software

The Cadet software programs, User's Manual and documentation are:

Copyright  1990, 1991, 1992 Steve Waskow

Wasco Technical Software.

Portions of the software are Copyright 1989, 1990 by Acropolis

Software, and are used and distributed under license to Wasco

Technical Software.

All rights to the Cadet software and the accompanying Cadet

documentation are reserved by Steve Waskow and Wasco Technical

Software.

AutoCAD & AutoLISP are registered trademarks of Autodesk, Inc.

Other brand and product names mentioned herein are trademarks or

registered trademarks of their respective holders.

CADET v4.0                     1                    User's Manual

------------------------------------------------------------------

TABLE OF CONTENTS

------------------------------------------------------------------

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . . .  4

INSTALLATION AND CONFIGURATION  . . . . . . . . . . . . . . . .  5

System Requirements . . . . . . . . . . . . . . . . . . . . .  5

Configuration Considerations  . . . . . . . . . . . . . . . .  5

Typematic Rate Accelerator  . . . . . . . . . . . . . . . .  5

Symbol Translation  . . . . . . . . . . . . . . . . . . . .  5

Dictionary Cache  . . . . . . . . . . . . . . . . . . . . .  6

Configuration Variables . . . . . . . . . . . . . . . . . . .  6

Directory Paths . . . . . . . . . . . . . . . . . . . . . .  6

COM Port Resets . . . . . . . . . . . . . . . . . . . . . .  6

AutoCAD 386/Extended AutoLISP . . . . . . . . . . . . . . .  6

Memory Management . . . . . . . . . . . . . . . . . . . . .  7

Sorting Coordinates . . . . . . . . . . . . . . . . . . . .  7

Text Pitch Factor . . . . . . . . . . . . . . . . . . . . .  8

Table Tolerance Factor  . . . . . . . . . . . . . . . . . .  8

Enable Text Restore . . . . . . . . . . . . . . . . . . . .  8

Automated Installation  . . . . . . . . . . . . . . . . . . .  9

Manual Installation (optional)  . . . . . . . . . . . . . . .  9

Changing Cadet Colors . . . . . . . . . . . . . . . . . . . . 10

Adding Cadet to AutoCAD's Support Path  . . . . . . . . . . . 10

Running Cadet on a Network  . . . . . . . . . . . . . . . . . 11

Running Cadet on a Ram Disk . . . . . . . . . . . . . . . . . 12

Running Transfer Files in a Ram Disk  . . . . . . . . . . . 12

Running CADET.DIC in a Ram Disk . . . . . . . . . . . . . . 13

Running Cadet Executables in a Ram Disk . . . . . . . . . . 13

AutoLISP Function Name Conflicts  . . . . . . . . . . . . . . 14

First Time Users  . . . . . . . . . . . . . . . . . . . . . . 15

LOADING CADET ROUTINES IN AUTOCAD . . . . . . . . . . . . . . . 15

CADET COMMAND LINE MENU . . . . . . . . . . . . . . . . . . . . 16

CADET from DOS and CADET-External File (CX) . . . . . . . . . . 18

CADET-AlignText (CA)  . . . . . . . . . . . . . . . . . . . . . 19

CADET-JustifyText (CJ)  . . . . . . . . . . . . . . . . . . . . 19

CADET-SetText (CS)  . . . . . . . . . . . . . . . . . . . . . . 20

CADET-PlusText (CP) . . . . . . . . . . . . . . . . . . . . . . 20

CADET-MultiText (CM)  . . . . . . . . . . . . . . . . . . . . . 21

CADET-Notes (CN)  . . . . . . . . . . . . . . . . . . . . . . . 23

CADET-Create Table (CC) . . . . . . . . . . . . . . . . . . . . 24

CADET-Import (CI) . . . . . . . . . . . . . . . . . . . . . . . 27

CADET-Edit (CE) . . . . . . . . . . . . . . . . . . . . . . . . 30

CADET-Edit Attributes (CEA) . . . . . . . . . . . . . . . . . . 30

CADET-Edit Global (CEG) . . . . . . . . . . . . . . . . . . . . 31

CADET-Edit Table (CET)  . . . . . . . . . . . . . . . . . . . . 31

CADET-Edit Notes (CEN)  . . . . . . . . . . . . . . . . . . . . 33

CADET-Restore (CR)  . . . . . . . . . . . . . . . . . . . . . . 34

CADET v4.0                     2                    User's Manual

USING THE CADET EDITOR  . . . . . . . . . . . . . . . . . . . . 35

Cursor Movement . . . . . . . . . . . . . . . . . . . . . . . 35

Mouse Support . . . . . . . . . . . . . . . . . . . . . . . . 36

Insert/Overwrite  . . . . . . . . . . . . . . . . . . . . . . 36

Editor Settings . . . . . . . . . . . . . . . . . . . . . . . 36

AutoIndent  . . . . . . . . . . . . . . . . . . . . . . . . 36

Type Speed  . . . . . . . . . . . . . . . . . . . . . . . . 37

Tab Mode  . . . . . . . . . . . . . . . . . . . . . . . . . 37

Tab Size  . . . . . . . . . . . . . . . . . . . . . . . . . 37

Squeeze . . . . . . . . . . . . . . . . . . . . . . . . . . 37

Status Lines  . . . . . . . . . . . . . . . . . . . . . . . . 38

Special Characters  . . . . . . . . . . . . . . . . . . . . . 38

Pull-Down Menus . . . . . . . . . . . . . . . . . . . . . . . 38

Pick Lists  . . . . . . . . . . . . . . . . . . . . . . . . . 40

Sorting Order . . . . . . . . . . . . . . . . . . . . . . . . 41

Deleting Text . . . . . . . . . . . . . . . . . . . . . . . . 42

Deleting Text Lines . . . . . . . . . . . . . . . . . . . . . 43

Inserting Text Lines  . . . . . . . . . . . . . . . . . . . . 45

Block Operations (Cut-and-Paste)  . . . . . . . . . . . . . . 47

Search-and-Replace, Find-Text . . . . . . . . . . . . . . . . 48

Changing Case, Height and Style . . . . . . . . . . . . . . . 49

Changing Line Spacing . . . . . . . . . . . . . . . . . . . . 50

Importing and Exporting Text  . . . . . . . . . . . . . . . . 50

Printing  . . . . . . . . . . . . . . . . . . . . . . . . . . 51

Spelling Checker  . . . . . . . . . . . . . . . . . . . . . . 51

Word-Wrap and Reformat  . . . . . . . . . . . . . . . . . . . 52

Zoom Toggle . . . . . . . . . . . . . . . . . . . . . . . . . 53

View Relative Text Positions  . . . . . . . . . . . . . . . . 53

Saving and Leaving Cadet  . . . . . . . . . . . . . . . . . . 53

CADET EDITOR COMMAND REFERENCE  . . . . . . . . . . . . . . . . 54

Cursor Movements  . . . . . . . . . . . . . . . . . . . . . . 54

Special Symbols and Characters  . . . . . . . . . . . . . . . 55

Editing Keys and Commands . . . . . . . . . . . . . . . . . . 55

PROBLEMS AND ERROR MESSAGES . . . . . . . . . . . . . . . . . . 59

AutoCAD Interfaces Do Not Load  . . . . . . . . . . . . . . . 59

AutoCAD Interfaces Load but Cadet Fails . . . . . . . . . . . 60

Did I Change That?  . . . . . . . . . . . . . . . . . . . . . 61

Mouse/Digitizer Problems  . . . . . . . . . . . . . . . . . . 62

INDEX . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 63

CADET v4.0                     3                    User's Manual

------------------------------------------------------------------

INTRODUCTION

------------------------------------------------------------------

Cadet is a professional software system which allows you to

create, edit and manipulate text in AutoCAD.  Cadet can create and

edit text tables and drawing notes, and edits attributes as easily

as text.  Although fundamentally an AutoCAD editor, Cadet can also

edit common ASCII text files from DOS or AutoCAD.  Other Cadet

features allow you to import and export drawing text, change text

justifications, align crooked text columns, add to existing text,

set text defaults by pick, and enter multiple lines of text at any

spacing.

The Cadet editor is a full-screen text editor with mouse support -

you can freely cursor about, editing in either insert or overwrite

mode.  Cadet's typematic rate accelerator allows fast cursor

movements.  You can edit up to 2000 lines of text and attributes

at once, each up to 255 characters in length.  The text can be

displayed sorted by columns or rows based on drawing position, or

in the order it was picked.  With a VGA/EGA monitor, Cadet can

zoom to 50/43 line display.

Access Cadet commands by menu or shortcut keys.  Cadet has full

mouse support, pull-down menus, point-and-shoot pick lists, and

on-line help.  Cadet's comprehensive status lines display cursor

position, speed, insert mode, text height, style, attribute data,

sort order, word wrap and line spacing.

Cadet's integrated spelling checker includes a 112,000+ word

dictionary - you can select from suggested spellings, edit, skip

or add words to the dictionary.  For text files and drawing notes,

Cadet has automatic indention, smart tabs, word wrap, and

paragraph and document reformat features.  Cadet also can change

the line spacing, and insert new lines of text into existing notes

and text tables.

Cadet's search-and-replace and find-text functions include case

sensitivity, whole word and global searches.  Use Cadet to change

text height, style and case on a line, block or global basis.  You

can cut-and-paste, insert, delete, copy, or move lines of text.

Delete text by character, word, line or block.

View, enter and edit AutoCAD's special characters for degrees,

plus or minus, and circle diameter symbols - without the %% codes.

Add and edit overscores and underscores as well.  Cadet's

emergency restore feature can allow you to recover Cadet text in

the event of power loss or failure.

CADET v4.0                     4                    User's Manual

------------------------------------------------------------------

INSTALLATION AND CONFIGURATION

------------------------------------------------------------------

This chapter contains important information, please read it

completely before beginning your Cadet installation.  Using the

supplied installation program is the simplest way to install Cadet

and configure your ACAD.PGP and ACAD.LSP files.  An alternate

method is outlined for installing Cadet manually.  First we will

discuss system requirements and a few of the configuration

considerations.

SYSTEM REQUIREMENTS

------------------------------------------------------------------

Cadet's requirements are basically the same as DOS versions of

AutoCAD.  Cadet requires an IBM AT, PS/2 or 286/386/486 compatible

with 640K RAM, hard disk, and MS-DOS 3.1 or higher.  An XT version

is available by special request.  Cadet works with all 286 and 386

DOS versions of AutoCAD R9 and up, using a minimum AutoCAD shell

space of 260000 bytes.  Optional support of 43/50 line text mode

is provided for EGA/VGA displays.

CONFIGURATION CONSIDERATIONS

------------------------------------------------------------------

TYPEMATIC RATE ACCELERATOR:

Cadet has a built-in typematic rate accelerator, which speeds up

the cursor movement on late model 286/386/486 class computers.

Cadet determines, to the extent possible, if your system BIOS

supports acceleration and enables it accordingly.  It is not

always possible to determine a computer type precisely, so the

Cadet installation allows you to override defaults.  You can also

set the default typematic speed, and leave the system accelerated

after exiting Cadet.

SYMBOL TRANSLATION:

The Cadet editor translates %%d, %%p and %%c character codes

(degree, plus/minus and circle diameter) into the true symbols.

AutoCAD's dimensioning feature produces a different set of codes -

ASCII codes 127, 128 and 129 - to draw the same symbols.  Cadet

can translate these ASCII codes as well, but translates them all

back as "%%" codes.  Translation of ASCII 127, 128 and 129 codes

to "%%" codes is not desirable with some third party fonts which

do not support AutoCAD's definitions, since incorrect symbols may

result.  This portion of the translation can be disabled during

installation.

DICTIONARY CACHE:

For speed, Cadet's spelling checker sets up a dictionary cache in

memory.  The default and recommended cache size is 511 blocks.  A

CADET v4.0                     5                    User's Manual

bigger cache can increase performance on slow hard drives by using

some of the memory otherwise reserved for text.  If memory for

Cadet's shell is limited, a smaller cache can increase the memory

available for text at some cost of performance.  The cache can be

set to any value between 0 and 1023.  Zero or any value less than

100 turns the cache off.

CONFIGURATION VARIABLES

------------------------------------------------------------------

Cadet uses eight global AutoLISP variables, set in your ACAD.LSP

and CADET.LSP files, to store configuration defaults.  These are

automatically set by the CADETIN installation/configuration

program.  You can make permanent changes by rerunning CADETIN or

by manually editing the files.  Temporary changes can be made in

AutoCAD by the Cadet command's Options menu, or by issuing

AutoLISP SETQ commands at the AutoCAD command prompt.

DIRECTORY PATHS:

Two global AutoLISP variables are used to specify Cadet

directories.  The path must be enclosed by quotes and use forward

slashes (not backslashes).  Directories must be terminated by a

slash.  The CADETL variable specifies the path of Cadet's AutoLISP

files.  The CADETD variable specifies the path for Cadet's

read/write operations.  Both are set to the Cadet directory by

default.  Typical examples would be:

(SETQ CADETL "C:/ACAD/CADET/")

(SETQ CADETD "C:/ACAD/CADET/")

Special uses for the CADETD variable and a companion DOS

environmental variable, CADET, are detailed in the RUNNING CADET

ON A NETWORK and RUNNING CADET ON A RAM DISK sections of this

manual.

COM PORT RESETS:

The variable CADETC controls whether or not Cadet resets the COM

ports after returning to AutoCAD from the editor.  See

MOUSE/DIGITIZER PROBLEMS for a discussion of when this may be

necessary.  Set CADETC to true or any value other than NIL to

enable COM port resets.

(SETQ CADETC T)

AUTOCAD 386/EXTENDED AUTOLISP:

Cadet employs two sets of AutoLISP files, depending on the amount

of AutoLISP memory in the system.  If AutoCAD 386 or AutoCAD R10+

with Extended AutoLISP is used, Cadet loads the file CADET40X.LSP

which contains the entire suite of Cadet functions.

CADET v4.0                     6                    User's Manual

If a 286 version of AutoCAD R9+ with Regular AutoLISP is used,

Cadet uses demand loading and unloading techniques with six files;

WTSC40.LSP, WTSE40.LSP, WTSJ40.LSP, WTSM40.LSP, WTSP40.LSP and

WTSR40.LSP.  This method gives somewhat slower access, but is

necessary in a limited memory environment.

The AutoLISP variable CADETB controls which files are used.  Set

CADETB to NIL for Regular AutoLISP.  Set CADETB to true or to any

value other than NIL for AutoCAD 386/Extended AutoLISP:

(SETQ CADETB T)

MEMORY MANAGEMENT:

If you are using AutoCAD 386 or Extended AutoLISP you can

disregard this section.  Cadet's memory management is applicable

only to 286 versions of AutoCAD using Regular AutoLISP.

In Regular AutoLISP, loading two or more of Cadet's AutoLISP

routines at once can degrade their performance or, depending on

what else is loaded, exceed AutoLISP's available memory.  Cadet's

memory management avoids this by using demand loading and

unloading techniques.  When you execute a Cadet routine, it

"unloads" any other active Cadet routine and loads itself into the

freed up memory.  All Cadet routines remain accessible, loading

automatically when called.

If you switch between Cadet applications often, have few other

AutoLISP applications and are willing to sacrifice performance to

avoid loading time, you can disable Cadet's memory management with

the AutoLISP variable CADETM.  Setting CADETM to true or to any

value other than NIL disables memory management.  For example:

(SETQ CADETM T)

SORTING COORDINATES:

Another Cadet configuration variable controls which text

coordinates are used for row and column sorting.  When editing

text, CE can use either the insert point of center, middle and

right justified text for the sorting coordinates, or the text's

lower left coordinates.  The AutoLISP variable CADETS controls

which point is used.  CE uses the default insert point when CADETS

is absent or set to NIL.  Set CADETS to true or to any value other

than NIL to use lower left coordinates.  For example:

(SETQ CADETS T)

CADET v4.0                     7                    User's Manual

TEXT PITCH FACTOR:

When creating new text with Cadet, the default vertical spacing

between lines is the text pitch factor times the text height.  A

1.619 factor is used by many of AutoCAD's furnished text fonts for

spacing lines with the DTEXT command, and is Cadet's default.

Cadet routines allow you to enter any pitch you desire at

execution time, but you can change the default to save keystrokes

if you consistently use a text pitch of different proportions.

Cadet uses a global AutoLISP variable CADETP to store the default

pitch factor.  If CADETP is absent or NIL, Cadet sets it to 1.619.

Use reasonable values.  Values of less than 1.0 overlap text, and

unless you desire a double-spaced look, you should not use a value

greater than 2.0.  For example:

(SETQ CADETP 1.5)

TABLE TOLERANCE FACTOR:

When editing existing text with Cadet, text items are sorted into

rows and columns based on their position in the AutoCAD drawing -

their X and Y coordinates.  Cadet uses a table tolerance factor to

allow a certain amount of misalignment in the row and column

positions.  This is especially important in the Cadet-Edit Table

interface, which expects reasonably aligned text and corrects

misalignments after editing.  The default tolerance factor is 1.0.

Cadet uses 0.5 times the tolerance factor times the average text

height for vertical tolerance, and 1.5 times the tolerance factor

times the average text height for horizontal tolerance.

You can tighten or relax the tolerance by setting Cadet's global

AutoLISP variable CADETT.  If CADETT is absent or NIL, Cadet sets

it to 1.0.  Cadet allows CADETT settings between 0.1 and 2.0.

Smaller values tighten the tolerance, larger values loosen it.

For instance, to tighten the tolerance to 0.33 times text height

vertically and 0.99 times text height horizontally, set CADETT to

0.66 (0.5 x 0.66 = 0.33 vertical, 1.5 x 0.66 = 0.99 horizontal):

(SETQ CADETT 0.66)

ENABLE TEXT RESTORE:

Cadet's emergency text restore feature (see CADET-RESTORE)

requires extra files to be written, adding to the overhead for all

Cadet-Edit interfaces.  To use this feature, you must enable it

with Cadet's global AutoLISP variable CADETR.  CADETR is set to

NIL by default, disabling restore files.  Set CADETR to true or

any value other than NIL to enable restore files:

(SETQ CADETR T)

CADET v4.0                     8                    User's Manual

AUTOMATED INSTALLATION

------------------------------------------------------------------

CADETIN.EXE is Cadet's installation/configuration program.  Log

onto the drive and directory which contains the Cadet programs and

enter "CADETIN" at the DOS prompt.  For new Cadet installations or

upgrades, choose INSTALLATION.  To change the defaults of an

existing Cadet installation, choose CONFIGURATION.  To continue,

just follow the on-screen instructions.

CADETIN updates or creates the file CADET.CFG and updates

CADET.LSP.  The CADET.CFG file must always be located in the same

directory as CADET.EXE.  The CADETIN program also automatically

locates all ACAD.LSP and ACAD.PGP files, and prompts for

confirmation before adding the Cadet interface to each.

MANUAL INSTALLATION (OPTIONAL)

------------------------------------------------------------------

Skip this section if you have used CADETIN for full installation.

This section describes how you can manually install Cadet if you

do not want the installation program to create directories or

modify your ACAD.PGP and ACAD.LSP files.  You will need an ASCII

text editor (Cadet itself can be used) to edit your ACAD.PGP and

ACAD.LSP files.

First, choose or make a directory for Cadet and use the DOS COPY

command to copy all Cadet files into it.  From the Cadet

directory, run the CADETIN installation/configuration program in

CONFIGURATION mode to set options in CADET.CFG and CADET.LSP.

Answer No to the prompts for ACAD.PGP and ACAD.LSP modification.

Next, edit your ACAD.PGP file, or, create one if none exists.  If

your system has more than one ACAD.PGP file, make the same

additions to all of them.  Add two lines to the bottom of ACAD.PGP

as shown.  These examples assume Cadet is in the C:\ACAD\CADET

directory - you will need to substitute the path of your Cadet

directory if different.  While Cadet works with as little as

260000 bytes, we recommend you use 300000 or more bytes in your

ACAD.PGP file's Cadet commands.

CADET!,C:\ACAD\CADET\CADET /A,300000,,0

CADETX,C:\ACAD\CADET\CADET /X,300000,,4

If you need a batch file to run digitizer utilities for mouse

emulation outside of AutoCAD, use CADET4 instead of CADET for the

program name.  See MOUSE/DIGITIZER PROBLEMS for details on the use

of the CADET4.BAT file.

Last, you must add a copy of the CADET.LSP file into your ACAD.LSP

file.  If you do not have an ACAD.LSP file, you must create one.

If your system has more than one ACAD.LSP file, make the same

CADET v4.0                     9                    User's Manual

additions to all of them.  Simply use your text editor to import

your CADET.LSP file into the bottom of ACAD.LSP.  No other changes

are necessary.  Do not attempt to edit protected ACAD.LSP files.

If you see the message "AutoCAD PROTECTED LISP file", do not

attempt to edit it - you must manually load Cadet as needed.

CHANGING CADET COLORS

------------------------------------------------------------------

A color configuration program, CADETCFG, allows you to

interactively modify Cadet's default colors.  To use CADETCFG,

change to the Cadet directory and enter "CADETCFG" at the DOS

prompt.

A sample Cadet screen is displayed with a list of items with

unique colors.  Use the [Up] and [Down] cursor keys to navigate

the list, and the [Right] and [Left] cursor keys (or [+] and [-]

keys) to modify the colors.  Color changes are immediately

reflected in the sample Cadet screen.  If you wish to restore

Cadet back to its default colors, press function key [F5].  To

abort the color configuration process, press [Esc].  Pressing

[F10] saves the new colors and exits the program.  The file

CADET.CFG is updated or created by the color configuration

process.  The CADET.CFG file must always be located in the same

directory as CADET.EXE.

ADDING CADET TO AUTOCAD'S SUPPORT PATH

------------------------------------------------------------------

It is not necessary to add Cadet to AutoCAD's support path if you

have followed procedures to install Cadet in your ACAD.LSP file.

If for some reason Cadet is not in your ACAD.LSP file, you will

have to manually load it each time you edit a drawing.  If you are

faced with this problem, using AutoCAD's support path will

simplify the loading process -  you do not need to remember or

type in Cadet's path when loading it.

AutoCAD uses a DOS environmental variable named ACAD to hold its

support path.  The ACAD variable must be placed in your

AUTOEXEC.BAT file or your AutoCAD start-up batch file, like so:

SET ACAD=C:\ACAD\CADET

AutoCAD R11 and later allows multiple directories, so you can

append the Cadet directory to a multiple ACAD path:

SET ACAD=C:\ACAD\SUPPORT;C:\ACAD\LISP;C:\ACAD\CADET

CADET v4.0                     10                    User's Manual

RUNNING CADET ON A NETWORK

------------------------------------------------------------------

Perhaps the simplest way to use Cadet in a networked environment

is to install Cadet on local drives of each individual

workstation.  Doing so avoids sharing conflicts and generally

gives faster access as well.

If preferred, Cadet can be installed and used in a shared network

directory, except for certain read/write files.  The spelling

dictionary CADET.DIC cannot be shared.  In addition, Cadet creates

small text transfer files which will conflict if written to a

shared directory.  The procedure for using Cadet on a shared

network directory requires that each individual workstation or

user have a copy of CADET.DIC, and an alternate path be set up and

used for it and the transfer files.  For best performance, the

alternate read/write path should be created on a local drive of

each workstation.  If this is not possible, a private network

directory for each individual can be used.

Create the read/write directories (unless existing directories are

to be used) and copy CADET.DIC into each.  Next, modify each

AUTOEXEC.BAT file or the batch files used to start AutoCAD.  Add a

SET command to assign the read/write path to the DOS environmental

variable CADET.  This variable is read by AutoLISP, so the

AutoLISP convention of using forward slashes rather than

backslashes must be followed.  Directories must be terminated by a

slash.  For example, if you are using C:\CADETRW as the read/write

directory, add:

SET CADET=C:/CADETRW/

If you are using AutoCAD R9, you must hard code the read/write

path into each ACAD.LSP and CADET.LSP file.  In each file, find

the line:

(SETQ CADETD "CadetPath")

The "CadetPath" is initially set to the Cadet directory or NIL.

Change it to the new Cadet read/write path, matching your batch

file's CADET environmental variable.  It too must use forward

slashes in lieu of backslashes, and must be enclosed by double

quotes.  For example:

(SETQ CADETD "C:/CADETRW/")

This step is unnecessary when using AutoCAD R10 or greater. In

such installations the DOS environmental variable CADET is

automatically read and assigned to CADETD.

CADET v4.0                     11                    User's Manual

RUNNING CADET ON A RAM DISK

------------------------------------------------------------------

Cadet transfers text data to and from AutoCAD drawings via small

disk files.  These transfer files are written to the Cadet

directory by default, but you can configure Cadet to use a

different path, such as a ram disk.

The reason to consider a ram disk is speed.  If you have a slow

hard drive, you can dramatically increase the performance of

Cadet's read/write operations by using a ram disk as the

read/write path.  Other Cadet files may also be located on the

user defined read/write path.  We show three options.

Two steps are necessary for using any of the following options.

One, Cadet must first be installed in the normal fashion.  Two,

you need to add a line in your AutoCAD start-up batch file or

AUTOEXEC.BAT file to set the environmental variable CADET to your

chosen read/write path.  This variable is read by AutoLISP, so the

AutoLISP convention of using forward slashes rather than

backslashes must be followed.  Directories must be terminated by a

slash.  For example, if the alternate path is E:\RW\, add:

SET CADETD=E:/RW/

RUNNING TRANSFER FILES IN A RAM DISK:

Running Cadet's transfer files in a ram disk yields the most

noticeable improvement of all options.  To run transfer files, the

ram disk will need 128,000 to 512,000 bytes free space, depending

on the amount of AutoCAD text normally edited.  The lower figure

is generally adequate for normal use.  Add the SET command in your

AUTOEXEC.BAT or AutoCAD batch file as previously described.

SET CADET=E:/RW/

If you are using AutoCAD R9 you must edit both your ACAD.LSP and

CADET.LSP files (this is unnecessary if you are using AutoCAD R10

or later).  In each file, find the line:

(SETQ CADETD "CadetPath")

The "CadetPath" is initially set to the Cadet directory or NIL.

Change it to your ram disk path.  The path string is enclosed in

double quotes, but otherwise must match your batch file's CADET

environmental variable:

(SETQ CADETD "E:/RW/")

CADET v4.0                     12                    User's Manual

RUNNING CADET.DIC IN A RAM DISK:

This option is only recommended if you have a slow hard drive and

want to speed up the spell checker.  The ram disk will need at

least 260,000 bytes free space for the dictionary.  Modify your

AUTOEXEC.BAT or AutoCAD batch file to add the SET command

previously described, and add a command to copy CADET.DIC to the

ram disk each time you boot up. For example:

SET CADET=E:/RW/

COPY C:\ACAD\CADET\CADET.DIC E:\RW

Warning:  If you add words to a Cadet dictionary in a ram disk,

you must copy the modified dictionary, CADET.DIC, back to the hard

disk before turning off or rebooting your machine.

RUNNING CADET EXECUTABLES IN A RAM DISK:

This option is generally not recommended, since its benefits are

minimal.  The ram disk will need at least 132,000 bytes of free

space.  First, modify your AUTOEXEC.BAT or AutoCAD batch file.

Add the SET command previously described, and lines to copy the

files CADET.EXE, CADET.OVR, and CADET.CFG to the ram disk each

time you boot up.  For example:

SET CADET=E:/RW/

COPY C:\ACAD\CADET\CADET.EXE E:\RW

COPY C:\ACAD\CADET\CADET.OVR E:\RW

COPY C:\ACAD\CADET\CADET.CFG E:\RW

Warning: If you use a batch file to load Cadet (see

MOUSE/DIGITIZER PROBLEMS), it must be modified to show Cadet's ram

disk path.  It may be copied to the ram disk or left in the Cadet

directory.

Last, edit the command lines in your ACAD.PGP file, and modify

Cadet's path to the new ram disk path, for example:

CADET!,E:\RW\CADET /A,300000,,0

CADETX,E:\RW\CADET /X,300000,,4

NOTE:  1. You can combine any or all of the above procedures.  Sum

the disk size requirements to determine the size

required for the combined configuration.

2. After modifications to AUTOEXEC.BAT, you must reboot

your computer for the change to become effective.

3. You should not run transfer files in a ram disk if you

plan on using Cadet's emergency restore feature.

CADET v4.0                     13                    User's Manual

AUTOLISP FUNCTION NAME CONFLICTS

------------------------------------------------------------------

To make the shortcut commands to Cadet routines easily accessible

from the keyboard, short two and three letter names have been

used: CE, CN, CC, CEN, etc.  These are AutoLISP functions.

Conflicts can occur if you have other functions or ACAD.PGP

commands with these names.

In general, ACAD.PGP defined commands dominate over AutoLISP

functions.  In the case of conflicting AutoLISP function names,

the last one loaded will dominate.

Name conflicts must be resolved by renaming one of the conflicting

functions or commands.  If you have other commands or functions

which match Cadet's, you must do some renaming.  If the conflict

is with an external command defined in your ACAD.PGP file, rename

the Cadet function.  If the conflict is with another AutoLISP

function, you may rename whichever function you wish.  Preferably,

leave the Cadet functions alone and rename the other function.

Never rename the Cadet functions on your original Cadet diskette.

Instead, first install Cadet as usual, then perform your

modifications on the Cadet files installed on your hard disk.

To rename functions, edit both your ACAD.LSP and CADET.LSP files.

Search for function names which conflict with Cadet's.  When you

find conflicts, carefully replace one of the function names with a

new name of your own choosing.

To rename Cadet's functions, look for the following lines:

(defun C:CC (...  ... ;Cadet-Create Table

(defun C:CN (...  ... ;Cadet-Notes

(defun C:CI (...  ... ;Cadet-Import

(defun C:CE (...  ... ;Cadet-Edit

(defun C:CET (...  ... ;Cadet-Edit Table

(defun C:CEN (...  ... ;Cadet-Edit Notes

(defun C:CEG (...  ... ;Cadet-Edit Global

(defun C:CEA (...  ... ;Cadet-Edit Attributes

(defun C:CS (...  ... ;Cadet-SetText

(defun C:CP (...  ... ;Cadet-PlusText

(defun C:CM (...  ... ;Cadet-MultiText

(defun C:CJ (...  ... ;Cadet-JustifyText

(defun C:CA (...  ... ;Cadet-AlignText

(defun C:CR (...  ... ;Cadet-Restore

(defun C:CX (...  ... ;Cadet-External File

CADET v4.0                     14                    User's Manual

Rename only the function name following the defun statement, do

not edit any other portion of the functions.  The new function

names must still include the C: prefix (the C: tells AutoCAD to

treat it as a command, it is not a drive designation).  For

example, to rename CN (Cadet-Notes) to NOTES, the edited line

would look like this:

(defun C:NOTES (...  ... ;Cadet-Notes

Once you rename any of these Cadet functions, you must manually

remove them prior to using Cadet's installation program to install

future upgrades.

FIRST TIME USERS

------------------------------------------------------------------

Although no trouble is anticipated, in keeping with the spirit of

"better safe than sorry", we suggest you save your drawing prior

to executing Cadet the first few times you try it.  This is

especially wise if you have a highly customized system.

If other programs work for you through AutoCAD's SHELL function,

you should have no trouble with Cadet, properly installed. Once it

becomes apparent to you that no problems exist, the act of saving

the drawing first is unnecessary.

------------------------------------------------------------------

LOADING CADET ROUTINES IN AUTOCAD

------------------------------------------------------------------

If you have followed the installation procedures to install Cadet

in your ACAD.LSP file, loading is automatic when you issue a CADET

command - no further steps are necessary.

If for some reason you did not install Cadet in your ACAD.LSP

file, you must load CADET.LSP at the AutoCAD command prompt.  This

is done with AutoLISP's LOAD command.  This example uses

C:\ACAD\CADET as the Cadet directory.  Note the use of forward

slashes instead of backslashes.

Command:(LOAD"C:/ACAD/CADET/CADET")

If you have added CADET's directory to AutoCAD's support path, you

do not need Cadet's path in the LOAD command:

Command:(LOAD"CADET")

CADET v4.0                     15                    User's Manual

------------------------------------------------------------------

CADET COMMAND LINE MENU

------------------------------------------------------------------

All Cadet routines are accessible from a simple command line menu.

This is the easiest way for novices to access the Cadet functions,

since you only need to remember one command.

Only brief descriptions of the individual Cadet functions will be

given here.  Full details are given in subsequent sections of the

manual.  To run the menu, enter "CADET" at the AutoCAD command

prompt.

Command:CADET

The top level of the Cadet command line menu shows the following

prompt:

Add/Manipulate/Edit/Restore text or Options/<EXit>?

Entering "A" or "Add" from the top level produces a second level

menu of text addition options:

Add/Manipulate/Edit/Restore text or Options/<EXit>? A

Plus/Multiple/Import/Notes/Create table/<Back>?

The ADD text menu options are:

P  Plus . . . . . Cadet-PlusText (shortcut: CP)

Add to existing text with matching

properties.

M  Multiple . . . Cadet-MultiText (shortcut: CM)

Enter multiple lines of text at a user

specified line spacing.

I  Import . . . . Cadet-Import (shortcut: CI)

Import an Ascii text file into the drawing.

N  Notes  . . . . Cadet-Notes (shortcut: CN)

Create notes and other single column text

with the Cadet editor.

C  Create table . Cadet-Create Table (shortcut: CC)

Create multiple column text tables with the

Cadet editor.

B  Back . . . . . Return to top menu level (default).

CADET v4.0                     16                    User's Manual

Entering "M" or "Manipulate" from the top level produces a second

level menu of text manipulation options:

Add/Manipulate/Edit/Restore text or Options/<EXit>? M

Align/Justify/Set/<Back>?

The MANIPULATE text menu options are:

A  Align. . . . . Cadet-AlignText (shortcut: CA)

Vertically straighten text columns.

J  Justify. . . . Cadet-JustifyText (shortcut: CJ)

Change from/to any text justification.

S  Set. . . . . . Cadet-SetText (shortcut: CS)

Set text defaults by picking an existing

text entity with the desired properties.

B  Back . . . . . Return to top menu level (default).

Entering "E" or "Edit" from the top level produces a second level

menu of editing options:

Add/Manipulate/Edit/Restore text or Options/<EXit>? E

Table/Notes/Attributes/Mixed/Global/File/<Back>?

The EDIT text menu options are:

T  Table. . . . . Cadet-Edit Table (shortcut: CET)

Edit multiple column text tables in the

Cadet editor.

N  Notes. . . . . Cadet-Edit Notes (shortcut: CEN)

Edit drawing notes in the Cadet editor.

A  Attributes . . Cadet-Edit Attributes (shortcut: CEA)

Edit block attributes in the Cadet editor.

M  Mixed. . . . . Cadet-Edit (shortcut: CE)

General-purpose editing of text and/or

attributes in the Cadet editor.

G  Global . . . . Cadet-Edit Global (shortcut: CEG)

Pulls all drawing text and attributes into

the Cadet editor.

F  File . . . . . Cadet-External File (shortcut: CX)

Edit external ASCII text files in the Cadet

editor, from within AutoCAD.

B  Back . . . . . Return to top menu level (default).

CADET v4.0                     17                    User's Manual

If Cadet's restore feature is enabled, entering "R" or "Restore"

from the top level executes the emergency Cadet-Restore (shortcut:

CR), used to restore Cadet text lost in a power outage.

Entering "O" or "Options" from the top level steps you through a

series of prompts to make temporary modifications to Cadet's

configuration variables:

CADETP, text pitch factor <1.6190>:

CADETT, table tolerance factor <1.0000>:

CADETR, enable restore files? Yes/No <No>:

CADETM, disable memory management? Yes/No <No>:

CADETS, sort by LLH coordinates? Yes/No <No>:

Enter "EX", "Exit" or press [Enter] from the top level to exit

back to the AutoCAD command prompt (default).

------------------------------------------------------------------

CADET from DOS and CADET-External File (CX)

------------------------------------------------------------------

Although fundamentally an AutoCAD editor, Cadet can also edit

ASCII text files from DOS or AutoCAD.  CADET is limited to a total

of 2000 lines of text, equivalent to about 35 typewritten pages.

To edit files from DOS, just enter "CADET" followed by the name of

the file.

C:/> CADET [FileName]

The file name on the command line is optional - if it is not

given, Cadet will prompt for one.  Enter the full path name if the

file is not in the current directory.  If the name of an existing

file is given, it is loaded into Cadet for editing, otherwise a

new file is created.  Files can also be selected from a pick list

by entering a directory and/or wildcards in lieu of a file name.

See USING THE CADET EDITOR, PICK LISTS for more detail.

To edit text files from within AutoCAD, enter "CADET" at the

command prompt, "E" for the Edit text menu, then "F" for File.

Optionally, you can edit external text files directly by entering

"CX" at the command prompt.

Command: CX

Cadet will then prompt you for a file name, as described above for

DOS.  The workings of the Cadet editor are detailed in a

subsequent section of the manual.

CADET v4.0                     18                    User's Manual

------------------------------------------------------------------

CADET-AlignText (CA)

------------------------------------------------------------------

Cadet-AlignText (shortcut: CA) is an AutoCAD utility designed as a

preprocessor for misaligned columns being edited by Cadet-Edit

Table.  Select a set of text entities, and CA moves them into

vertical alignment.  Text moves only in the horizontal direction,

and insertion points are used for alignment.

To run Cadet-AlignText from Cadet's command line menu, enter

"CADET" at the command prompt, "M" for the Manipulate text menu,

then "A" for Align.  Optionally, you can access Cadet-AlignText

directly by entering "CA" at the command prompt.  CA's first

prompt is for the alignment point.

Enter point to align text to:

Generally, you would use AutoCAD's INSert object snap and pick the

column's top line of text.  You can, however, enter any point -

with either your pointing device or the keyboard.  The next prompt

is to select the text.

Select objects:

This is the familiar AutoCAD interactive selection mechanism.  You

can use object snaps, Window, Crossing, Last, Previous, Remove and

Add, etc. to select the text.  Non-text entities will be ignored.

Once you have selected the text, press [Enter] and CA moves it

into alignment.

------------------------------------------------------------------

CADET-JustifyText (CJ)

------------------------------------------------------------------

Cadet-JustifyText (shortcut: CJ) is a utility used to change the

justification of existing AutoCAD text.  It can be used on single

entities or sets.

To run Cadet-JustifyText from Cadet's command line menu, enter

"CADET" at the command prompt, "M" for the Manipulate text menu,

then "J" for Justify.  Optionally, you can access

Cadet-JustifyText directly by entering "CJ" at the command prompt.

CJ's first prompt is to select the text.

Select objects:

This is the familiar AutoCAD interactive selection mechanism.  You

can use object snaps, Window, Crossing, Last, Previous, Remove and

Add, etc. to select the text.  Non-text entities will be ignored.

CADET v4.0                     19                    User's Manual

Once you have finished, Cadet-Justify prompts for a new

justification for the selected text.

Left/Center/Right/Middle/Fit/Align

In AutoCAD R11 and later, nine more options are displayed:

.../BL/BC/BR/ML/MC/MR/TL/TC/TR

Enter the new justification, and each text entity in the set is

changed.  If Fit or Align justification is chosen, you are

individually prompted for a new text endpoint for each entity in

the set.

------------------------------------------------------------------

CADET-SetText (CS)

------------------------------------------------------------------

Cadet-SetText (shortcut: CS) is a utility which allows you to set

new AutoCAD text defaults by picking a sample on the screen.  The

new text defaults - size, style, layer, angle, etc. - are

automatically set to match the chosen sample.

To run Cadet-SetText from Cadet's command line menu, enter "CADET"

at the command prompt, "M" for the Manipulate text menu, then "S"

for Set.  Optionally, you can access Cadet-SetText directly by

entering "CS" at the command prompt.  Cadet-SetText's only prompt

is to pick the sample text.

------------------------------------------------------------------

CADET-PlusText (CP)

------------------------------------------------------------------

Cadet-PlusText (shortcut: CP) is a Cadet utility which allows you

to continue to add text below any existing text without leaving

AutoCAD.  The new text's properties - size, style, layer, angle,

etc. - are automatically set to match the existing text.

To run Cadet-PlusText from Cadet's command line menu, enter

"CADET" at the command prompt, "A" for the Add text menu, then "P"

for Plus.  Optionally, you can access Cadet-PlusText directly by

entering "CP" at the command prompt.  Cadet-PlusText's first

prompt is to pick the existing text.

Pick text to add to:

The new text will be added below the existing text.  If you are

adding to a multiple line note, you should pick the bottom line.

CADET v4.0                     20                    User's Manual

Next, you are prompted for the line spacing.

Distance between rows <X.XXXX> or Auto:

This requests the vertical spacing between each text row or

entity.  The default <X.XXXX> equals the value of CADETP (see

CONFIGURATION VARIABLES) times the text height you've selected.

Press [Enter] to accept the default, type in a value, pick two

points, or enter "A" for automatic spacing.  In Cadet-PlusText,

the Auto spacing option gives you the same results as selecting

the default value.

Hint: If you are adding to a multiple line note, you can match its

spacing precisely by using AutoCAD's INSert object snap and

picking two adjacent lines.

You are then prompted for the text.  Type in as many lines as

necessary.  Like AutoCAD's DTEXT command, CP keeps prompting for

text until it gets a null response.

------------------------------------------------------------------

CADET-MultiText (CM)

------------------------------------------------------------------

Cadet-MultiText (shortcut: CM) is a Cadet utility which allows you

to create multiple lines of text without leaving AutoCAD.  Unlike

AutoCAD's DTEXT command, CM allows you to set the line spacing to

any desired value.  New text will be drawn in the current text

style.  If you need a text style other than the current one, set

the style with Cadet-SetText or AutoCAD's STYLE command prior to

running CM.

To run Cadet-MultiText from Cadet's command line menu, enter

"CADET" at the command prompt, "A" for the Add text menu, then "M"

for Multiple.  Optionally, you can access Cadet-MultiText directly

by entering "CM" at the command prompt.  Cadet-MultiText's first

prompt is to pick a justification and/or starting point for the

notes.

Starting point or Justification:

This is similar to AutoCAD's TEXT command prompt.  If you want

something other than left justification, type in the justification

prior to supplying the point.  CM supports all AutoCAD

justifications.

To view the full list of justification options, enter "J".

Left/Center/Right/Middle/Fit/Align

CADET v4.0                     21                    User's Manual

In AutoCAD R11 and later, nine more options are displayed:

.../BL/BC/BR/ML/MC/MR/TL/TC/TR

Use your pointing device to pick the point or enter the point in X

and Y coordinates.  The new text is placed at the current

elevation, so a Z coordinate is not necessary.

Next, if the current text style is not defined at a fixed height,

CM prompts you to supply a text height.

Text height <X.XXXX>:

The <X.XXXX> default is your default height for new text -

generally, the height last used to draw text.  Respond with the

height you need, or press [Enter] to accept the default.  Like

AutoCAD's TEXT command, you may use your pointing device to drag

the height from the point of origin. Next, if Fit or Align

justification is not being used, you are prompted for a rotation

angle.

Rotation angle <0>:

Enter the angle in decimal degrees or press [Enter] to accept the

default.  Following the rotation angle, you are prompted for the

line spacing.

Distance between rows <X.XXXX> or Auto:

This requests the vertical spacing between each text row or

entity.  The default <X.XXXX> equals the value of CADETP (see

CONFIGURATION VARIABLES) times the text height you've selected.

Press [Enter] to accept the default, type in a value, pick two

points, or enter "A" for automatic spacing.  In Cadet-MultiText,

the Auto spacing option gives you the same results as selecting

the default value.

You are then prompted for text.  Type in as many lines as

necessary.  Like AutoCAD's DTEXT command, CM keeps prompting for

text until it gets a null response.

CADET v4.0                     22                    User's Manual

------------------------------------------------------------------

CADET-NOTES (CN)

------------------------------------------------------------------

Cadet-Notes (shortcut: CN) is the Cadet editor interface of choice

for creating drawing notes.  CN is simple to employ, and accesses

the editor in a free-form style in which Cadet's advanced features

such as word wrap and paragraph reformat are available.

To run Cadet-Notes from Cadet's command line menu, enter "CADET"

at the command prompt, "A" for the Add text menu, then "N" for

Notes.  Optionally, you can access Cadet-Notes directly by

entering "CN" at the command prompt.  Cadet-Notes' first prompt is

to pick a justification and/or starting point for the notes.

Starting point or Justification:

This is similar to AutoCAD's TEXT command prompt.  If you want

something other than left justification, type in the justification

prior to supplying the point.  CN supports all AutoCAD

justifications except Align and adds two new ones - Bound and

Wrap.  Both ask for a second text endpoint, and use it to

establish the editor's word wrap position.  The Bound option is a

conditional Fit justification - it fits lines of text which are

too long, but left justifies shorter lines.  The Wrap option is

simple Left justification.  To view the full list of justification

options, enter "J".

Left/Center/Right/Middle/Fit/Bound/Wrap

In AutoCAD R11 and later, nine more options are displayed:

.../BL/BC/BR/ML/MC/MR/TL/TC/TR

Use your pointing device to pick the point or enter the point in X

and Y coordinates.  All new text is placed at the current

elevation, so a Z coordinate is not necessary.  CN always places

new text horizontally, regardless of your zero degree orientation

(you can rotate text after it has been placed).

Next, if the current text style is not defined at a fixed height,

CN prompts you to supply a text height.

Text height <X.XXXX>:

The <X.XXXX> default is your default height for new text -

generally, the height last used to draw text.  Respond with the

height you need, or press [Enter] to accept the default.  Like

AutoCAD's TEXT command, you may use your pointing device to drag

the height from the point of origin.

CADET v4.0                     23                    User's Manual

Last, you are prompted to supply the row spacing.

Distance between rows <X.XXXX> or Auto:

This requests the vertical spacing, or pitch, between each line of

text.  The default <X.XXXX> equals the value of CADETP (see

CONFIGURATION VARIABLES) times the text height you've selected.

To enter a constant value, either press the [Enter] key to accept

the default, type in a new distance, or use your pointing device

to define the distance by picking two points.

Enter "A" for automatic spacing.  Cadet can change the height of

each line of text individually.  To accommodate this, CN's Auto

spacing uses a function similar to AutoCAD's DTEXT command,

spacing each text line according to its height and style, to avoid

overlapping.

After the line spacing is given, the Cadet editor appears.  With

sufficient memory, the editor can accommodate up to 2000 lines of

text.  We cover the details of the Cadet text editor more fully in

the next section of this manual.  When you exit the editor, CN

returns to the AutoCAD graphics screen and draws the notes you

composed.

------------------------------------------------------------------

CADET-CREATE TABLE (CC)

------------------------------------------------------------------

Cadet-Create Table (shortcut: CC) is a specialized Cadet editor

interface for creating text tables such as schedules and bills of

materials.

To run Cadet-Create Table from Cadet's command line menu, enter

"CADET" at the command prompt, "A" for the Add text menu, then "C"

for Create table.  Optionally, you can access Cadet-Create Table

directly by entering "CC" at the command prompt.  Cadet-Create

Table's  first prompt is to pick a justification and/or starting

point of the first column (column A).  Cadet letters columns A, B,

C and so forth.

Starting point of column A or Justification:

This is similar to AutoCAD's TEXT command prompt.  If you want

something other than left justification, type in the justification

prior to supplying the point.  CC supports all AutoCAD

justifications except Align and adds a new one - Bound.  Bound

asks for a second text endpoint and uses it to establish the

editor's word wrap position.  The Bound option is a conditional

Fit justification - it fits lines of text which are too long, but

left justifies shorter lines.

CADET v4.0                     24                    User's Manual

To view the full list of justification options, enter "J".

Left/Center/Right/Middle/Fit/Bound

In AutoCAD R11 and later, nine more options are displayed:

.../BL/BC/BR/ML/MC/MR/TL/TC/TR

Use your pointing device to pick the point or enter the point in X

and Y coordinates.  All new text is placed at the current

elevation, so a Z coordinate is not necessary.  CC always places

new text horizontally, regardless of your zero degree orientation

(you can rotate text after it has been placed). The first

insertion point you enter also defines the vertical (Y coordinate)

origin of any subsequent columns, simplifying multiple column

definition.

Next, if the current text style is not defined at a fixed height,

CC prompts you to supply a text height.

Text height <X.XXXX>:

The <X.XXXX> default is your default height for new text -

generally, the height last used to draw text.  Type in a height or

press [Enter] to accept the default.  Like AutoCAD's TEXT command,

you can also use your pointing device to drag the height from the

point of origin.

In the next series of prompts CC will ask for column and row

information. You will be prompted for the number of columns first.

Multiple columns are optional.

Number of text columns (|||) <1>:

Enter the number of columns needed, or press [Enter] for the

default.  Cadet handles a maximum of 2000 entities, so the number

of columns you define will limit the number of rows you may

create.  You are prompted for the number of rows next.

Number of text rows (---) per column <max>:

The default <max> will give you the maximum number of text rows

per column - the quotient of 2000 divided by the number of

columns, memory permitting.  You can either enter the number of

rows you actually need, or press [Enter] to accept the default

maximum.  If you enter too large a value, CC defaults to the

maximum.  You may be composing text as you go and not know how

many rows you will need.  In such cases you should accept the

default maximum.  The number of rows defined just sets up space in

the editor, you do not have to fill each row.

CADET v4.0                     25                    User's Manual

If you have chosen more than a single row you will be prompted to

supply the row spacing.

Distance between rows <X.XXXX> or Auto:

This requests the vertical spacing between each text row or

entity.  The default <X.XXXX> equals the value of CADETP (see

CONFIGURATION VARIABLES) times the text height you've selected.

Entering a constant value assures that adjacent columns align.  To

enter a constant value, either press the [Enter] key to accept the

default, type in a new distance, or use your pointing device to

define the distance by picking two points.

Enter "A" for automatic spacing.  Cadet can change the height of

each line of text individually.  To accommodate this, CC's Auto

spacing uses a function similar to AutoCAD's DTEXT command,

spacing each text line according to its height and style, to avoid

overlapping.  The disadvantage to using Auto is that column

alignment is lost if the text height and vertical spacing varies.

Because of this, CC's Auto mode is only suited for single columns

of text.

The next prompt, if you have chosen two or more columns, requests

the origin and optionally, the text justification of the second

column (Column B).

Starting point of column B or Justification:

The actual wording may vary, since CC defaults to the

justification used for the previous column.  If you want to change

the justification, enter the new justification prior to supplying

the point.  Type in point coordinates (X, Y) or pick a point with

your pointing device.  Although you input a point, CC only uses

the horizontal (X coordinate) component.  Since CC automatically

aligns the column with column A, vertical accuracy of picked

points is not important.

If you have chosen three or more columns, you receive further

prompts.

Starting point of column C or Justification/Equal:

This prompt is essentially the same as the prompt for column B,

varying in wording according to the justification of the previous

column.  Added to this and following column prompts is the option

Equal.  Selecting Equal causes all remaining columns to be equally

spaced to the previous column offset, using the previous

justification.  Otherwise, similar prompts will repeat for each

remaining column.

CADET v4.0                     26                    User's Manual

After defining rows and columns, the Cadet editor appears,

configured for the specified table.  By changing Cadet's sort

order, you can enter text by either rows or columns.  Details of

the Cadet text editor are covered in a following section of this

manual.  When you exit the editor, CC returns to the AutoCAD

graphics screen and draws the text.

------------------------------------------------------------------

CADET-IMPORT (CI)

------------------------------------------------------------------

Cadet-Import (shortcut: CI) is a Cadet utility for importing ASCII

text files into your drawing in linear or tabular format.  CI runs

entirely from within AutoCAD.  An ASCII text file is one of plain

text, without the special formatting characters some word

processors create.  Most word processors can create an ASCII text

file however, with a special save or print-to-disk command.  To

check if a file is ASCII, print it to the screen with the DOS TYPE

command, for example: TYPE MyFile.  If the text is legible as it

prints on the screen, with no strange characters, it is ASCII.

To run Cadet-Import from Cadet's command line menu, enter "CADET"

at the command prompt, "A" for the Add text menu, then "I" for

Import.  Optionally, you can access Cadet-Import directly by

entering "CI" at the command prompt.  Cadet-Import's  first prompt

is for the file name.

File name of text file to import:

Enter the file name, including the drive and subdirectory if

different from the default, and file extension if any.  If CI

cannot find the file in the specified subdirectory, it shows an

error message and repeats a prompt for a file name.

From this point on, Cadet-Import's prompts are essentially

identical to those used by Cadet-Create Table.  You are prompted

to pick a justification and/or starting point of the first column

(column A).  Cadet letters columns A, B, C and so forth.

Starting point of column A or Justification:

This is similar to AutoCAD's TEXT command prompt. If you want

something other than left justification, type in the justification

prior to supplying the point.  CI supports all AutoCAD

justifications except Align and adds a new one - Bound.  Bound

asks for a second text endpoint and uses it to establish the

editor's word wrap position.  The Bound option is a conditional

Fit justification - it fits lines of text which are too long, but

left justifies shorter lines.

CADET v4.0                     27                    User's Manual

To view the full list of justification options, enter "J".

Left/Center/Right/Middle/Fit/Bound

In AutoCAD R11 and later, nine more options are displayed:

.../BL/BC/BR/ML/MC/MR/TL/TC/TR

Use your pointing device to pick the point or enter the point in X

and Y coordinates.  All new text is placed at the current

elevation, so a Z coordinate is not necessary.  CI always places

new text horizontally, regardless of your zero degree orientation

(you can rotate text after it has been placed).  The first

insertion point you enter also defines the vertical (Y coordinate)

origin of any subsequent columns, simplifying multiple column

definition.

Next, if the current text style is not defined at a fixed height,

CI prompts you to supply a text height.

Text height <X.XXXX>:

The <X.XXXX> default is your default height for new text -

generally, the height last used to draw text.  Type in a height or

press [Enter] to accept the default.  Like AutoCAD's TEXT command,

you can also use your pointing device to drag the height from the

point of origin.

In the next series of prompts CI will ask for column and row

information.  You will be prompted for the number of columns

first.  Multiple columns are optional.

Number of text columns (|||) <1>:

Enter the number of columns needed, or press [Enter] for the

default.  Cadet handles a maximum of 2000 entities, so the number

of columns you define will limit the number of rows you may

create.  You are prompted for the number of rows next.

Number of text rows (---) per column:

Enter the number of rows.  If you chose more than a single row you

will be prompted to supply the row spacing.

Distance between rows <X.XXXX> or Auto:

This requests the vertical spacing between each text row or

entity.  The default <X.XXXX> equals the value of CADETP (see

CONFIGURATION VARIABLES) times the text height you've selected.

Entering a constant value assures that adjacent columns align.  To

enter a constant value, either press the [Enter] key to accept the

CADET v4.0                     28                    User's Manual

default, type in a new distance, or use your pointing device to

define the distance by picking two points.  Enter "A" for

automatic spacing.  In Cadet-Import, the Auto spacing option gives

you the same results as selecting the default constant value.

The next prompt, if you have chosen two or more columns, requests

the origin and optionally, the text justification of the second

column (Column B).

Starting point of column B or Justification:

The actual wording may vary, since CI defaults to the

justification used for the previous column.  If you want to change

the justification, enter the new justification prior to supplying

the point.  Type in point coordinates (X, Y) or pick a point with

your pointing device.  Although you input a point, CI only uses

the horizontal (X coordinate) component.  Since CI automatically

aligns the column with column A, vertical accuracy of picked

points is not important.

If you have chosen three or more columns, you receive further

prompts.

Starting point of column C or Justification/Equal:

This prompt is essentially the same as the prompt for column B,

varying in wording according to the justification of the previous

column.  Added to this and following column prompts is the option

Equal.  Selecting Equal causes all remaining columns to be equally

spaced to the previous column offset, using the previous

justification.  Otherwise, similar prompts will repeat for each

remaining column.

After rows and columns are defined, CI draws the text from the

specified file into your drawing.

Cadet also has another method of importing files; use Cadet-Notes

or Cadet-Create Table, and import the file into the Cadet editor

with [Ctrl-XI].

CADET v4.0                     29                    User's Manual

------------------------------------------------------------------

CADET-EDIT (CE)

------------------------------------------------------------------

Cadet-Edit (shortcut: CE) is Cadet's general-purpose editor

interface.  It is used to edit attributes and text.  CE is well

suited for editing mixed selection sets with varying text

positions and properties.  For organized text such as notes and

tables, it is generally better to use another Cadet interface.

To run Cadet-Edit from Cadet's command line menu, enter "CADET" at

the command prompt, "E" for the Edit text menu, then "M" for

Mixed.  Optionally, you can access Cadet-Edit directly by entering

"CE" at the command prompt.  Cadet-Edit's  first prompt is to pick

the text and attributes.

Select objects:

This is the familiar AutoCAD interactive selection mechanism.  You

can use object snaps, Window, Crossing, Last, Previous, Remove and

Add, etc. to select the text.  You can select up to 2000 lines of

text or attributes.  Don't worry if non-text entities are

accidentally picked into the selection set.  CE safely filters

them out.  When you have picked the text, press [Enter] and CE

writes the text to the editor.

The text then appears in the Cadet editor, sorted by drawing

position.  See the INSTALLATION chapter for details on sorting

options.  Use the Cadet editor to make changes to the text as

necessary, or to export the text to a disk file.  After exiting

the editor, you are returned to the AutoCAD graphics screen where

CE automatically updates any modified text.

------------------------------------------------------------------

CADET-EDIT ATTRIBUTES (CEA)

------------------------------------------------------------------

Cadet-Edit Attributes (shortcut: CEA) is a specialized front-end

to the Cadet-Edit interface.  With CEA, ordinary text is filtered

from the selection set, leaving only block attributes.

To run Cadet-Edit Attributes from Cadet's command line menu, enter

"CADET" at the command prompt, "E" for the Edit text menu, then

"A" for Attributes.  Optionally, you can access Cadet-Edit

Attributes directly by entering "CEA" at the command prompt.

CADET v4.0                     30                    User's Manual

------------------------------------------------------------------

CADET-EDIT GLOBAL (CEG)

------------------------------------------------------------------

Cadet-Edit Global (shortcut: CEG) is another specialized front-end

to the general-purpose Cadet-Edit interface.  With CEG, all

drawing text and attributes are automatically selected and brought

into the editor.  CEG is ideal for global spelling checks.

To run Cadet-Edit Global from Cadet's command line menu, enter

"CADET" at the command prompt, "E" for the Edit text menu, then

"G" for Global.  Optionally, you can access Cadet-Edit Global

directly by entering "CEG" at the command prompt.

------------------------------------------------------------------

CADET-EDIT TABLE (CET)

------------------------------------------------------------------

Cadet-Edit Table (shortcut: CET) is a specialized Cadet editor

interface for editing organized multiple column text such as

tables and schedules.  Unlike the general-purpose CE interface,

attributes cannot be edited, but CET has other important

advantages.

Cadet orders CET's selected table into an array corresponding to

its placement in the drawing, automatically providing spaces for

empty table cells.  In addition to being able to fill empty

spaces, you can also insert complete new rows of text anywhere in

the table.  CET expects horizontally oriented text, and the text

selection must be fairly well aligned and equally spaced.   The

text selection may have single or multiple columns.

To run Cadet-Edit Table from Cadet's command line menu, enter

"CADET" at the command prompt, "E" for the Edit text menu, then

"T" for Table.  Optionally, you can access Cadet-Edit Table

directly by entering "CET" at the command prompt.  Cadet-Edit

Table's first prompt is to select the text table.

Select objects:

This is the familiar AutoCAD interactive selection mechanism.  You

can use object snaps, Window, Crossing, Last, Previous, Remove and

Add, etc. to select the text table.  Don't worry if non-text

entities are accidentally picked into the selection set.  CET

safely filters them out.  Press [Enter] and CET writes the text

table to the editor.

Cadet begins by ordering the selected text into rows and columns,

building a full tabular array.  If the array size (number of rows

times number of columns) exceeds 2000 or Cadet runs out of memory,

a warning is issued and you are returned to AutoCAD.  Arrays which

appear larger than they should, and arrays with broken rows or

CADET v4.0                     31                    User's Manual

columns are due to the text table being too misaligned or unevenly

spaced in the drawing.  Unevenly spaced tables can also throw off

the row spacing, but the spacing is easy to correct from within

the editor.

Cadet allows a tolerance for misalignment, stored in the CADETT

variable (see CONFIGURATION VARIABLES).  For extremely disorderly

tables the tolerance may not be enough.  In those cases you will

need to use AutoCAD to straighten up the table before using CET.

You may find the routines Cadet-AlignText and Cadet-JustifyText

helpful.

To allow for center and right justified columns, CET always sorts

on the basis of the text's insert point, regardless of the CADETS

configuration variable setting.  An organization problem occurs

when incompatible text justifications are mixed within a table -

the table may look neat enough, but the text insert points are a

jumble.  Text justifications should be consistent up and down each

column.  This may preclude selecting table headings in with the

body of the table if their justifications differ.  Also, baseline

compatible text justifications should be used throughout the

table.

For example, LEFT, FIT, CENTER and RIGHT justifications are

baseline compatible with each other, but not with MIDDLE

justifications.  Release 11 and later versions of AutoCAD have

additional families of compatible justifications for TOP, MIDDLE

and BOTTOM baselines.

The text table appears in the Cadet editor sorted by columns or

rows.  The sort order can be toggled back and forth -  also see

the INSTALLATION chapter for setting a default order.  Use the

Cadet editor to edit and add to the table as necessary.

After exiting Cadet, you are returned to the AutoCAD graphics

screen.  The text table is updated to reflect any changes, new

text entities (if any) are created, and any misfit rows and

columns are precisely realigned.

CADET v4.0                     32                    User's Manual

------------------------------------------------------------------

CADET-EDIT NOTES (CEN)

------------------------------------------------------------------

Cadet-Edit Notes (shortcut: CEN) is another specialized Cadet

editor interface.  CEN is used for editing notes and other single

columns of text.  Attributes cannot be edited, but CEN has other

important advantages over the general-purpose CE interface.

Cadet orders CEN's selection set corresponding to its placement in

the drawing, automatically providing spaces for skipped lines.

You can fill these empty lines and insert new lines anywhere in

the notes.  You can also use Cadet's word wrap and reformat

functions.  CEN expects horizontally oriented text, and the text

must be fairly well aligned and equally spaced.

To run Cadet-Edit Notes from Cadet's command line menu, enter

"CADET" at the command prompt, "E" for the Edit text menu, then

"N" for Notes.  Optionally, you can access Cadet-Edit Notes

directly by entering "CEN" at the command prompt.  Cadet-Edit

Notes' first prompt is to select the text.

Select objects:

This is the familiar AutoCAD interactive selection mechanism.  You

can use object snaps, Window, Crossing, Last, Previous, Remove and

Add, etc. to select text.  Don't worry if non-text entities are

accidentally picked into the selection set.  CEN safely filters

them out.  Press [Enter] and CEN writes the text to the editor.

Cadet begins by ordering the text lines into a list based on their

vertical drawing position.  If the list exceeds 2000 or Cadet runs

out of memory, a warning is issued and you are returned to

AutoCAD.  Notes which appear with extra or out of place blank

lines are due to the text being too unevenly spaced in the

drawing.  Notes which are unevenly spaced can also throw off the

line spacing, but the spacing is easy to correct from within the

editor.

For vertical alignment, CEN uses the tolerance stored in the

CADETT variable (see CONFIGURATION VARIABLES).  For extremely

disordered notes the tolerance may not be enough, and you will

need to use AutoCAD to make adjustments prior to using CEN.

CEN automatically horizontally aligns all text to the top line,

based on the text insert points.  The CADETS configuration

variable setting is ignored.  Organization problems can occur if

incompatible text justifications are mixed within the notes.  Only

LEFT and FIT justifications are compatible enough to be mixed in

notes.  The routines Cadet-AlignText and Cadet-JustifyText are

helpful in preparing or cleaning up disorganized notes.

CADET v4.0                     33                    User's Manual

Use the Cadet editor to edit and add to the notes as necessary.

After exiting Cadet, you are returned to the AutoCAD graphics

screen.  The notes are updated to reflect any changes, new

additions are created, and each text insert point is precisely

realigned.

------------------------------------------------------------------

CADET-RESTORE (CR)

------------------------------------------------------------------

Cadet-Restore (shortcut: CR) is the Cadet function you use to

restore edited text in the event of power loss or machine failure,

assuming the drawing file and Cadet restore files are still

intact.

Due to extra overhead, Cadet's restore feature is not implemented

by default.  For text to be recoverable, the following

requirements must be observed.  Cadet-Edit interfaces CE, CEA,

CEG, CEN and CET, must have the restore feature enabled and

AutoCAD R10 or later with Handles on.  Other Cadet interfaces do

not require Handles.  Cadet's restore feature is enabled with the

CADETR variable (see INSTALLATION and CONFIGURATION).  Temporary

changes can be made with the CADET command line menu.  AutoCAD's

Handles can be turned on by typing "HANDLES" at the AutoCAD

command line, then responding "ON".  The text must be saved by the

Cadet editor's [F2] Save command or [Ctrl-E] End command before

the loss occurs.

To restore text, first reenter the AutoCAD drawing.  To run

Cadet-Restore from Cadet's command line menu, enter "CADET" at the

command prompt, then "R" for Restore.  Optionally, you can access

Cadet-Restore directly by entering "CR" at the command prompt.

The restoration process will proceed automatically.  Cadet-Restore

works by looking for loose Cadet restoration files.  If a file

from a Cadet-Edit interface is found, the text in the file is

compared to that in the drawing and updates are made accordingly.

If Cadet-Restore finds a restoration file from a Cadet interface

which creates text, the text it contains is read directly into the

drawing.  Please note, if the text was not actually lost from the

drawing, you will end up with duplicate text over the original.

CADET v4.0                     34                    User's Manual

------------------------------------------------------------------

USING THE CADET EDITOR

------------------------------------------------------------------

The Cadet text editor is the heart of the Cadet system.  Cadet is

a full-screen text editor with mouse support.  Cadet supports a

dual level command interface.  At the expert level, direct

shortcut keys are available.  For novice and mouse users, there is

an easy to use pull-down menu system.

Cadet's shortcut keys are control (Ctrl) key combinations, entered

by holding down the control key while pressing the command letter.

Cadet also uses a few alternate (Alt) key combinations, entered in

a similar fashion.  This manual represents these keys in the

format; [Ctrl-A] and [Alt-A].  Most Cadet commands can also be

entered from the pull-down menu.  We show each command's menu

equivalent as the list of the menu choices required to reach it,

enclosed in curly braces, in the format; {Editing|erase All} and

{Display|About Cadet}.  When editing, you can press [F1]

{Display|Help screen} for a summary of Cadet's shortcut keys.

Separate text entities are referred to as text lines, and the body

of text in the editor as the document.  Several commands are

designed to work on either the current line (locally) or the

document (globally) at the user's option.

The behavior of the Cadet editor differs somewhat depending on

which Cadet DOS or AutoCAD interface accesses it.  In effect,

Cadet is four editors in one.  The editing modes and corresponding

interfaces are listed below.

MIXED   Cadet-Edit, Cadet-Edit Attributes, & Cadet-Edit Global

TABLE   Cadet-Create Table, Cadet-Edit Table

NOTES   Cadet-Notes, Cadet-Edit Notes

FILE    Cadet-External File, Cadet from DOS

The differences in Cadet's four editing modes are slight,

effecting only a few commands, but they are important.  These

differences are noted in the descriptions of Cadet commands and

features which follow.

CURSOR MOVEMENT

------------------------------------------------------------------

Mouse movements and the cursor keys are the basic method of moving

the cursor around the screen.  Cadet also has special keys to move

through text even more quickly.  The [Ctrl-Right] and [Ctrl-Left]

keys move the cursor a word at a time, right and left

respectively.  The [Tab] and [Shift-Tab] keys move right and left

according to the tab configuration.

CADET v4.0                     35                    User's Manual

The cursor can quickly be positioned to the beginning or end of

the text line with the [Home] and [End] keys.  [PgUp] and [PgDn]

moves vertically one screen depth, [Ctrl-PgUp] and [Ctrl-PgDn]

move to the head and end of the document, and in TABLE mode

[Alt-T] and [Alt-B] move to the top and bottom of the column or

row.

MOUSE SUPPORT

------------------------------------------------------------------

o  Mouse buttons always active Left = Pick, & Right+Left = [Esc].

During prompts and in menus and pick lists, Right = [Enter].

o  Editing screen.  Pick any point to reposition the cursor.

o  [F1], [F2], [F10].  Execute by picking from bottom of screen.

o  Prompts.  Reply by picking the answer from the prompt line.

o  Pull down menus.  Pick top status line to activate pull-downs.

o  Pick lists and menus.  Pick selections from the display.

o  List scroll bars. Pick a point along the bar relative to

desired list position.

o  List Up and Down arrows. Pick to navigate an item at a time.

o  Windows.  Pick the exit button in the top left corner of a

window to exit.

INSERT/OVERWRITE MODE

------------------------------------------------------------------

In overwrite (typeover) mode, characters overwrite existing text.

In insert mode, characters insert and displace existing text.

Pressing the [Ins] key toggles the insert mode.  The start-up

insert mode can be set with the CADETIN Installation/Configuration

program.

In most Cadet editing modes, pressing [Enter] when insert mode is

active will add a hard carriage return and create a new line (see

INSERTING TEXT LINES).  Pressing [Tab] when insert mode is active

will shift text right of the cursor to the next tab stop.

EDITOR SETTINGS

------------------------------------------------------------------

Cadet allows five of its default settings to be temporarily

modified via the [Ctrl-T] {Options} command.  Your choice of

defaults can be set with the CADETIN Installation/Configuration

program.

AUTOINDENT

When Autoindent is active, text which is word wrapped from an

indented line is placed at the same level of indentation as the

preceding line.  Likewise, when [Enter] is pressed, the cursor is

placed directly below the first character of the line above.  When

Autoindent is off, carriage returns and wrapped lines go to the

home position, regardless of previous indentations.

CADET v4.0                     36                    User's Manual

TYPE SPEED

Cadet's typematic rate accelerator can move the cursor up to three

times faster than normal.  You can choose Slow, Medium or Fast

speed.  Slow speed is the same as no acceleration.

TAB MODE

Cadet has two tab modes, Smart Tabs and Fixed Tabs.  In either

mode, pressing [Tab] when insert mode is active will push all text

to the right of the cursor to the tab stop.  This, alone or in

combination with word wrap, is useful for formatting indentations

in existing text.  When overwrite mode is active, only the cursor

is moved.

The default tab mode is Smart Tabs.  When [Tab] is pressed, the

cursor is positioned underneath the first character of the next

word in the line above.  The [Shift-Tab] key has no function in

Smart Tab mode.

The alternative to Smart Tabs is Fixed Tabs.  In Fixed Tab mode,

tab stops are set to fixed intervals, spaced according to the Tab

Size value (described below).  Pressing [Tab] moves the cursor

forward to the next tab stop, and [Shift-Tab] moves the cursor

back to the previous tab stop.

TAB SIZE

Cadet's Tab Size can be configured to any value between one and

twenty.  When reading files, Cadet automatically translates

incoming tabs (ASCII 9) to the specified number of spaces.  Cadet

saves files in translated form, without hard tabs.  Setting the

default tab size to one prevents tab translation.  The tab size

also determines the tab stop spacing in Fixed Tab mode.

SQUEEZE

The Squeeze mode works with Cadet's word wrap and reformat

command.  When active, Cadet removes all excess spaces in the line

of text before word wrapping.  Indented left margins are

unaffected.  Likewise, excess spaces are removed from paragraphs

when reformatting with the [Ctrl-G] command.

Caution:  Indented margins and tabbed spacings created in Cadet

will retain their proper format in AutoCAD only if LEFT

justification and a monospaced (non-proportional) font is used,

such as Autodesk's MONOTXT. Smoother and fancier monospaced fonts

are available from AutoCAD third party developers.  If you use a

proportional font, you may have to move indented text around to

get it to look right.  For tabular data, you can avoid the need

for monospaced fonts and tabs by using Cadet-Create Table.

CADET v4.0                     37                    User's Manual

STATUS LINES

------------------------------------------------------------------

Cadet has two status lines.  The upper status line shows the

editor status.

o  Cadet editing mode: Notes/Table/Mixed/File

o  Current editor sort order: Row/Col/Pick

o  Word wrap position and Squeeze mode.

o  Tab size and Tab mode.

o  Autoindent mode.

o  Typematic rate speed: Slow/Med/Fast

o  Line position of cursor.

o  Column position of cursor.

o  Editing insert mode: Ins/Ovr

In FILE editing mode, the lower status line only displays the name

of the file being edited.  In other modes, the lower status line

is the drawing status line.

o  Drawing name or file name.

o  Text height of text at cursor line.

o  Text font style of text at cursor line.

o  Line spacing, or in MIXED editing mode;

Block name, attribute tag and attribute prompt.

SPECIAL CHARACTERS

------------------------------------------------------------------

You may enter AutoCAD's special symbols directly into your text

without the "%%" codes.  Just press the appropriate Alt key

combination.

[Alt-O] Overscore toggle (%%o)

[Alt-U] Underscore toggle (%%u)

[Alt-D] Degrees symbol (%%d)

[Alt-P] Plus/minus symbol (%%p)

[Alt-C] Circle dia. symbol (%%c)

Cadet performs character code to symbol translation, to and from

AutoCAD.  Some translations can be disabled, see the INSTALLATION

chapter for details.

PULL-DOWN MENUS

------------------------------------------------------------------

Activate the menu by picking the top status line with a mouse or

pressing [F10].  The main menu bar is displayed at the top of the

screen, and consists of eight sub-menu headings.  The current

choice is marked with a highlight bar.  The highlighted letter in

each choice is its hot-key - capitalized below.

Change Editing Search Block Tools Display Options eXit

CADET v4.0                     38                    User's Manual

Select a sub-menu heading with a mouse pick, pressing its hot-key,

or moving the highlight bar atop it with the cursor and pressing

[Enter].  The bottom of the screen shows information on each

sub-menu as you cursor through the headings.  Once selected,

sub-menus drop and display available commands.

+--------+                   +---------+

| Change |                   | Editing |

+--------+-------------+     +---------+-----------------+

| Lower case  [Ctrl-L] |     | erase Word    [Ctrl-BkSp] |

| Upper case  [Ctrl-U] |     | erase All     [Ctrl-A]    |

| Juggle case [Ctrl-J] |     | erase to End  [Ctrl-End]  |

| Height      [Ctrl-H] |     | erase to Home [Ctrl-Home] |

| Style       [Ctrl-S] |     | Delete line   [Ctrl-D]    |

| liNe space  [Ctrl-N] |     | Insert line   [Ctrl-I]    |

+----------------------+     +---------------------------+

+--------+                        +-------+

| Search |                        | Block |

+--------+-----------------+      +-------+--------------+

| Find text    [Ctrl-F]    |      | Mark block  [Ctrl-M] |

| Replace text [Ctrl-R]    |      | Copy block  [Ctrl-C] |

| Head of doc  [Ctrl-PgUp] |      | write Block [Ctrl-B] |

| End of doc   [Ctrl-PgDn] |      +----------------------+

| col/row Top  [Alt-T]     |

| col/row Bot  [Alt-B]     |

+--------------------------+

+-------+                       +---------+

| Tools |                       | Display |

+-------+------------------+    +---------+--------------+

| sPell check     [Ctrl-P] |    | Zoom EGA/VGA  [Ctrl-Z] |

| eXternal files  [Ctrl-X] |    | Help screen   [F1]     |

| print hard copY [Ctrl-Y] |    | About CADET   [Alt-A]  |

| reformat Group  [Ctrl-G] |    | sort Order    [Ctrl-O] |

| Word wrap       [Ctrl-W] |    | View position [Ctrl-V] |

+--------------------------+    +------------------------+

+---------+                     +------+

| Options |                     | eXit |

+---------+------------+        +------+------------+

| Autoindent [Ctrl-TA] |        | End/save [Ctrl-E] |

| Type speed [Ctrl-TT] |        |_Quit_    [Ctrl-Q] |

| tab Mode   [Ctrl-TM] |        | Save     [F2]    ++----+

| tab Size   [Ctrl-TS] |        +------------------+ Yes |

| sQueeze    [Ctrl-TQ] |                           |_No_ |

+----------------------+                           +-----+

CADET v4.0                     39                    User's Manual

Choose a menu command with a mouse pick, pressing its hot-key, or

by cursoring to the choice and pressing [Enter].  As the cursor

moves through the selections, the bottom of the screen displays

help information.

Menu choices which are inactive in the current editing mode are

displayed in a dim color.  As illustrated on the Exit menu above,

many menu choices - such as {Exit|Quit} - will bring up a third

level menu of command options.  These command options are picked

in the same manner as commands.  To escape from a third level menu

back to the second level menu, press [Esc] or pick the second

level menu with a mouse.

To move to an adjacent second level sub-menu, cursor [Left] or

[Right] or pick its main menu heading with your mouse.  To escape

back through the menu levels, press [Esc].  To exit without

executing a command, press [Esc] from the top menu or pick any

point outside the menus with a mouse.

PICK LISTS

------------------------------------------------------------------

Cadet's pick lists are used to select files, text styles, and

spell checker suggestions.  A pick list is a list of selections in

a bordered window.  A highlight bar indicates the current

selection.  The [Up] and [Down] cursor keys traverse the list an

item at a time.  Longer lists will scroll as you traverse them.

[PgUp] and [PgDn] move up and down the list the length of the

window, and [Home] and [End] move to the respective ends of the

list.

Except for the spell checker list, typing the first few characters

of a word moves the highlight bar to the first matching selection.

Cadet will track and match the selections so long as you type

characters within two second intervals.  After a two second

interval, the tracking string clears.  You can force the tracking

string to clear at any time by pressing the spacebar.

The right hand side of the pick list window is a mouse sensitive

scroll bar.  The slider box on the scroll bar shows the position

of the current selection relative to the length of the list.  The

bold arrowheads at the top and bottom of the scroll bar are mouse

sensitive -  click the left mouse button on them to cursor up or

down the list.  To make bigger jumps along the file list, place

the mouse cursor over the scroll bar in the desired relative

position and click the left button.  The slider box and highlight

bar will be repositioned accordingly.

Make a selection by highlighting it and pressing [Enter] or by

picking any displayed selection with a mouse.  To exit the pick

list without making a selection, press [Esc], click the right and

CADET v4.0                     40                    User's Manual

left mouse buttons simultaneously, or use a mouse to pick the

upper left-hand window exit button.

The text style pick list pops up whenever you issue a change style

command. All styles previously defined in the AutoCAD drawing

appear in the list.  The spell checker pick list pops up when you

ask for suggested spellings.  It can contain anywhere from 1 to 18

suggestions.

The file pick list is more interactive.  It is available from the

file name prompt when importing files and at Cadet FILE editing

mode start-up (unless passed a file name on the command line).  To

display a pick list, enter a directory and/or a wildcard in lieu

of a file name.  The global "*.*" wildcard is assumed if no other

wildcard is given, and the current directory is assumed if no

directory is given.  Filenames are displayed in lower case,

subdirectories are capitalized and colored differently.  The

directory tree can be traversed by picking a subdirectory name

from the list.  To move towards the root, pick the parent

directory (..\).  To change drives, escape from the pick list and

enter the new path at the file name prompt.

SORTING ORDER

------------------------------------------------------------------

Cadet displays text in a linear list, with each entity occupying

one line of the editor.  In MIXED and TABLE editing modes, the

order in which the text is presented is important.  In these

modes, Cadet can display text in up to three orders; ROW (sorted

by rows), COL (sorted by columns), and - in MIXED mode only - PICK

(as picked).  The PICK order is unsorted, text entities are

displayed in the order you select them, and attributes in the

order they are encountered in the block definition.

Text sorting is performed on the basis of the entities' X and Y

coordinates.  Elevations in Z are ignored.  A tolerance is used in

row/column definition.  The default vertical tolerance is 1/2 the

average text height, the horizontal tolerance is 1-1/2 times the

average text height.  Tolerances can be adjusted with the CADETT

variable (see CONFIGURATION VARIABLES).

Cadet defaults to using the insert point for the sort coordinates,

rather than the text's lower left coordinates.  You can, however,

instruct Cadet to use the lower left coordinates for MIXED editing

mode sorting with the CADETS variable (see CONFIGURATION

VARIABLES).

At start-up, Cadet's default sort order is COL.  This default can

be changed with Cadet's Installation/Configuration program.  The

sort order can be changed in the editor by the [Ctrl-O]

{Display|Sort Order} command.  Depending on the block's design,

CADET v4.0                     41                    User's Manual

attributes are sometimes easier to edit when displayed in PICK

order.  Some text tables are easier to read in COL order, while

others are more suited to ROW order.

Consider the following examples, each has two columns, each word

is a left-justified text entity - a total of six separate entities

in each table:

Example #1  (in AutoCAD)   |   Example #2  (in AutoCAD)

|

PENCIL      TABLE        |     DOOR       WINDOW

PEN         CHAIR        |     CAT        DOG

PAPER       COUCH        |     SPOON      FORK

For identification, the editor letters columns and numbers rows

similar to a spreadsheet.  Example #1 is easier to read in column

order.  Example #2, however, is more suited to the editor's row

order.  Here are the examples shown as you would see them in

Cadet:

Example #1  (in Cadet)   |    Example #2  (in Cadet)

column sort order       |    row sort order

|

1a PENCIL                |    1a DOOR

2a PEN                   |    1b WINDOW

3a PAPER                 |    2a CAT

1b TABLE                 |    2b DOG

2b CHAIR                 |    3a SPOON

3b COUCH                 |    3b FORK

With the tables sorted according to the direction of associations,

it is fairly simple to follow the table in linear format.  The

[Ctrl-V] view position command can be used to display the relative

position of a portion of the table and gain perspective.

DELETING TEXT

------------------------------------------------------------------

Cadet has many ways to delete text besides the familiar [BkSp] and

[Del] keys.  Delete words with [Ctrl-BkSp] {Editing|erase Word}.

The [Ctrl-End] {Editing|erase End} and [Ctrl-Home] {Editing|erase

Home} commands delete to the end and beginning of the line

respectively.  Whole lines can be erased (blanked out) by pressing

[Ctrl-A] {Editing|erase All}, and marked blocks of text with the

[Ctrl-M] {Block|Mark block} command.

With the exception of blocks, these methods of deleting text are

all local commands - they only affect the current line.  If you

have not moved from the line or used a global command since

editing, pressing [Esc] will undo the erasure.

CADET v4.0                     42                    User's Manual

DELETING TEXT LINES

------------------------------------------------------------------

[Ctrl-D] {Editing|Delete line} is Cadet's delete line command.

Not only does it erase the line, the delete line command is

capable of pulling up the text below to fill the gap.  The

[Ctrl-BkSp] command and the [Del] key are also used in certain

situations to delete empty lines and carriage returns.  The delete

line command is global in nature - it affects more than just the

current line.  [Esc] cannot undo deleted lines.  Certain effects

of the delete line command vary depending on which editing mode is

in effect.

In NOTES and FILE editing modes, Cadet behaves similar to most

ordinary text editors.  Regardless of the cursor position,

[Ctrl-D] deletes the entire line.  The emptied space is filled as

all lines below pull up one position.  Pressing [Del] or

[Ctrl-BkSp] at the end of a line, or on an empty line, deletes the

carriage return and joins the line below to the cursor position.

Text positions are critical in MIXED editing mode - you rarely

want other text repositioned due to a line being deleted.

Generally, the [Ctrl-A] command should be used in lieu of [Ctrl-D]

- it erases lines without affecting others.  To protect the user

from against accidental text repositioning, delete line commands

use an "Are you sure?" prompt in MIXED mode.  In other respects,

these commands behave as they do in NOTES or FILE mode.  Please

note, however, that the flow of the text being pulled up is

dependant on the sort order in effect when the line is deleted.

You should always check the sort order first, changing it if

necessary, to ensure you get the desired results.

In TABLE editing mode, the delete line command is specialized for

editing multiple column tables.  In this mode, [Ctrl-D] gives you

the choice to delete a full row from the table, or to delete a

single line.  Deleting a table row removes the entire row in which

the line is a member, regardless of the number of columns.

Single lines can be deleted from either table rows or columns,

depending on the sort order.  When deleting a single line from a

table, its effect is limited to its member group (row or column).

Text lines below which belong to the group are pulled up, text

lines outside the group are unaffected. [Del] and [Ctrl-BkSp] do

not delete lines in TABLE mode.

To illustrate how the delete line command works in TABLE mode,

lets look at our column ordered example #1 and row ordered example

#2 once again.  In both examples the cursor is under the text on

line 1a, [Ctrl-D] is pressed, then Line is chosen.

CADET v4.0                     43                    User's Manual

This has the effect of deleting a line from the column in example

#1 and a line from the row in example #2.  The following results.

Example #1  (in Cadet)   |   Example #2  (in Cadet)

column sort order      |      row sort order

(before)     (after)     |   (before)     (after)

|

1a PENCIL    1a PEN      |   1a DOOR      1a WINDOW

2a PEN       2a PAPER    |   1b WINDOW    1b

3a PAPER     3a          |   2a CAT       2a CAT

1b TABLE     1b TABLE    |   2b DOG       2b DOG

2b CHAIR     2b CHAIR    |   3a SPOON     3a SPOON

3b COUCH     3b COUCH    |   3b FORK      3b FORK

Example #1           |        Example #2

(as redrawn in AutoCAD)   |   (as redrawn in AutoCAD)

|

PEN          TABLE       |   WINDOW

PAPER        CHAIR       |   CAT           DOG

COUCH       |   SPOON         FORK

Lets start with the original examples again, this time using the

delete row feature.  [Ctrl-D] is pressed, then Row is chosen to

delete the entire row.

Example #1  (in Cadet)   |   Example #2  (in Cadet)

column sort order      |      row sort order

(before)     (after)     |   (before)     (after)

|

1a PENCIL    1a PEN      |   1a DOOR      1a CAT

2a PEN       2a PAPER    |   1b WINDOW    1b DOG

3a PAPER     3a          |   2a CAT       2a SPOON

1b TABLE     1b CHAIR    |   2b DOG       2b FORK

2b CHAIR     2b COUCH    |   3a SPOON     3a

3b COUCH     3b          |   3b FORK      3b

(as redrawn in AutoCAD)  |  (as redrawn in AutoCAD)

|

PEN          CHAIR       |   CAT          DOG

PAPER        COUCH       |   SPOON        FORK

In both examples, the entire row #1 is deleted and everything else

moves up, emptying row #3.  In both cases, the integrity of the

remaining table is intact, the remaining entries remain in their

proper columns.

CADET v4.0                     44                    User's Manual

INSERTING TEXT LINES

------------------------------------------------------------------

The [Ctrl-I] {Editing|Insert line} command inserts a new line into

the group at the cursor position, pushing existing text down.

Only text above and to the right of the cursor is pushed down.  If

the cursor is someplace in the middle of the text, the text will

break at the cursor position.  The [Enter] key is also used in

certain modes to insert lines.  The [Ctrl-I] command is global in

nature as it affects more than just the current line.  [Esc]

cannot undo changes made by [Ctrl-I].  Like the delete line

command, certain effects of the [Ctrl-I] command vary depending on

which editing mode is in effect.  These variations are described

below.

In NOTES and FILE editing modes, Cadet behaves similar to most

ordinary text editors.  In these modes, pressing [Enter] when the

insert mode is on has the same effect as the [Ctrl-I] command

described above.  When overwrite mode is on, pressing [Enter] has

no effect other than moving the cursor down and home.

In MIXED editing mode, you rarely want text repositioned as a

result of editing.  The NOTES or TABLE interfaces should be used

for the kind of editing that involves reformatting text positions.

To avoid accidental text repositioning in MIXED mode, insert line

commands start with an "Are you sure?" prompt.  In addition, since

the MIXED editing interfaces are not capable of creating new text

entities, the [Ctrl-I] command requires that there be a blank line

at the bottom of the document.  A warning is printed if there is

no room to insert the line.  In other respects, the [Ctrl-I] and

[Enter] insert line commands behave as they do in NOTES or FILE

mode.

In the TABLE editing mode, the insert line command is specialized

for multiple column tables.  In this mode, [Ctrl-I] gives you the

choice to insert a full row into the table, or to insert a single

line.  When you insert a row, an entire new row of table cells is

inserted.  In TABLE mode, the [Enter] key does not insert lines,

regardless of the insert/overwrite mode.

When you insert a single line in TABLE mode, the line is inserted

in either the row or column, depending on the sort order.  To

preserve the integrity of the table, the effect of inserting a

single line is limited to its member group (row or column).  Text

lines below which belong to the group are pushed down, text lines

outside the group are unaffected.  This means that the last column

in the row must be empty - a warning is printed if there is no

room to insert the line into a particular group.  If you need to

create room, insert a new bottom row before inserting the line.

CADET v4.0                     45                    User's Manual

In TABLE mode, insert does not break lines at the cursor position.

New lines and rows are normally inserted above the current

position.  To make the new line insert below the current line,

position the cursor past the end of the text.  To make a new row

insert below the current row, move to the last column in the row

and position the cursor past the end of the text.

We will use the previously described examples to show how the

[Ctrl-I] command may be used in TABLE mode to insert rows.  This

time the cursor is moved past the right-hand end of the text on

the last line 3b.  [Ctrl-I] is pressed, then Row is chosen,

creating a new bottom row.

Example #1  (in Cadet)   |   Example #2  (in Cadet)

column sort order      |      row sort order

(before)    (after)      |   (before)    (after)

|

1a PENCIL   1a PENCIL    |   1a DOOR     1a DOOR

2a PEN      2a PEN       |   1b WINDOW   1b WINDOW

3a PAPER    3a PAPER     |   2a CAT      2a CAT

1b TABLE    4a           |   2b DOG      2b DOG

2b CHAIR    1b TABLE     |   3a SPOON    3a SPOON

3b COUCH    2b CHAIR     |   3b FORK     3b FORK

3b COUCH     |               4a

4b           |               4b

Lets continue with the above modified examples, this time using

the insert line option.  The cursor is moved back up under the

text in the first line 1a. [Ctrl-I] is pressed, then Line is

chosen to insert a single line.

Example #1  (in Cadet)   |   Example #2  (in Cadet)

column sort order      |      row sort order

(before)     (after)     |   (before)     (after)

|

1a PENCIL    1a          |   1a DOOR      1a DOOR

2a PEN       2a PENCIL   |   1b WINDOW    1b WINDOW

3a PAPER     3a PEN      |   2a CAT       2a CAT

4a           4a PAPER    |   2b DOG       2b DOG

1b TABLE     1b TABLE    |   3a SPOON     3a SPOON

2b CHAIR     2b CHAIR    |   3b FORK      3b FORK

3b COUCH     3b COUCH    |   4a           4a

4b           4b          |   4b           4b

In example #1, the new line pushes column #a entries down.

Example #2, sorted by rows, attempted to insert a line in row #1

but there was no room.

CADET v4.0                     46                    User's Manual

BLOCK OPERATIONS (CUT-and-PASTE)

------------------------------------------------------------------

Cadet can perform several block operations on partial or multiple

lines of text.

o  Cut and paste blocks of text from one position to another.

o  Copy blocks of text from one position to another.

o  Write blocks of text to a file.

o  Delete blocks of text.

o  Change the case, height or style of blocks of text.

The block marking mechanism is the [Ctrl-M] {Block|Mark block}

command.  First, move your cursor to the beginning of the text you

wish to block then press [Ctrl-M].  The character above the cursor

position will highlight.  You then use the cursor keys, mouse or

other text positioning commands to highlight the text you wish to

block.  Press [Enter] when the desired text is highlighted.  Any

amount of text can be marked, with one exception; as a protection

device in TABLE editing mode, you cannot mark across row/column

boundaries.  You are then given the following options, shown here

with the hot keys capitalized:

Copy/cuT/Delete block or

Uppercase/Lowercase/Juggle case/Height/Style?

The height and style options do not appear in FILE editing mode.

Select an action by pressing the highlighted hot key or picking

the choice with a mouse.  The copy and cut options store the block

into a buffer, the cut and delete options delete the marked block,

and the remaining options affect only the text properties of the

marked block.

The simplest block operation is deleting a block.  Press "D", and

it is gone.  Copy and cut-and-paste operations are straightforward

as well.  Press "C" to copy the marked block or "T" to cut out and

move the block.  The copy and cut operations use a temporary

holding place called the block buffer to store the text.  Any

prior contents of the block buffer is overwritten with new text

copied or cut.  The stored block can later be inserted or written

to file.

To complete a copy or cut-and-paste operation, move cursor to the

position you want to paste the block and press [Ctrl-C]

{Block|Copy block}.  The block will be inserted at the cursor

position, displacing any text to the right and below.  To write

the stored block to a file, press [Ctrl-B] {Block|Write block} and

enter a filename.  Optionally, you can add a drive and path name.

If you name an existing file, you will be asked if you want to

overwrite or append it.

CADET v4.0                     47                    User's Manual

Keep in mind that after pasting, the block still exists in the

block buffer.  It remains there until another block is cut or

copied.  This can be handy at times.  Suppose you are writing some

notes where your company name, International Computer Aided

Drafting Services, Inc. appears several times.  That's a lot of

typing.  Type in the company name once, mark it and copy it into

the block buffer.  Now, whenever you get to a place where the

company name is required, press [Ctrl-C] and the name is inserted.

Other block options are used to change properties of marked text

and do not affect the block buffer.  Press "U", "L" or "J" to

change the text to upper case, lower case or juggle the case (swap

upper/lower case).  The "H" and "S" options change the height and

style of AutoCAD text.  Unlike the other options, height and style

changes affect the block on a line basis, changing both partially

and fully marked lines.  AutoCAD does not allow two different

heights or styles to be mixed in a single text entity.

SEARCH-AND-REPLACE, FIND-TEXT

------------------------------------------------------------------

Search-and-replace replaces one text string with another.  The

text string can be a character, word, phrase or complete line of

text.  All text in the editor can be searched, and you can

selectively pick and choose the matches you wish to replace.

Most of the usual editing functions are available when entering

search strings.  When repeating searches, the strings last entered

appear as defaults.  Press [Enter] to accept the default, or type

in something different - the default string automatically clears

if the first key pressed is not a cursor key.

Press [Ctrl-R] {Search|Replace text} to start a search-and-replace

process.  You are prompted for the old text string, enter the

string you wish to replace.  Next, you are prompted for the new

text string.  If you simply want to delete all occurrences of the

old string, leave the new string blank.  The next prompt is for

three search options.

[ ] Ignore case    [ ] Whole words    [ ] Global search

The check boxes can be toggled on and off by picking it with a

mouse or pressing the hot key corresponding to the option.  Press

"I" to toggle ignore case/case sensitive searching.  Press "W" to

toggle whole word/embedded string searching.  Press "G" to toggle

global/forward searching.  Global searching scans the entire

document from the top, forward searching finds the first match

from the cursor position.

CADET v4.0                     48                    User's Manual

Now the search begins.  Matches are highlighted as the function

presents you with the following options.

Replace? <Yes>/No/Quit/All

Pressing "N" for no causes the old text to be skipped without

change, and global searches continued.  A "Y" response for yes (or

pressing [Enter]) causes the old text to be replaced by the new

text.  Again, global searches are resumed.  If you want to abort

the search, press "Q" for quit.  During global searches, to

replace all matches with no further questions, press "A" for all.

Use care when using the "A" option.  There is no undo function

available for replaces.

To simply find a word or phrase, use the [Ctrl-F] {Search|Find

text} command.  It works much like the search-and-replace

function.  Enter the string you're looking for and mark the

appropriate search options.  As matches are found in global

searches, you are presented the following prompt.

Find another? <Yes>/No

Press "N" to abort the search.  To find the next match, press "Y"

or [Enter].

CHANGING CASE, HEIGHT AND STYLE

------------------------------------------------------------------

You can quickly change your text case, text height, or text style

on a line-by-line or global basis.  These commands all work

similarly - they first ask if you want to perform the change to

the line or document.  If you choose Line, the change is made only

to the current line.  If you choose Document, the change is made

globally to all text in the editor.  You can undo a change to a

line by pressing [Esc], but not a global change to the document.

Changing the text case is simple, press [Ctrl-U] {Change|Upper

case} to change to upper case, [Ctrl-L] {Change|Lower case} to

change to lower case, or [Ctrl-J] {Change|Juggle case} to juggle

(switch) the case.

The text height of the line at the cursor position is displayed on

the status line.  To change text height, press [Ctrl-H]

{Change|Height} and respond to the prompt for line or document.

You then enter the new text height as a real (decimal) number (no

architectural units or fractions).  If you should enter an invalid

number, the entry is cleared and the prompt repeats.  Press

[Enter] when you've finished your entry.  To protect AutoCAD's

intent, you cannot change the height of fixed-height text styles.

CADET v4.0                     49                    User's Manual

To change the text font style, press [Ctrl-S] {Change|Style} and

respond to the prompt for line or document.  A pick list window of

all currently defined text styles appears, sorted by alphabetical

order.  Use the cursor keys to highlight the desired style and

press [Enter], or use a mouse to pick it.

CHANGING LINE SPACING

------------------------------------------------------------------

In TABLE and NOTES editing mode, you can change the text line

spacing globally with the [Ctrl-N] {Change|Line space} command.

When prompted to enter a new spacing, the current spacing is shown

as the default.  Enter the spacing as a real (decimal) number (no

architectural units or fractions).  If an invalid number is

entered, the prompt repeats.  [Ctrl-N] will not allow you to

change the spacing when using the Auto spacing option, since the

Auto feature scales the spacing on the basis of the text heights

of individual lines.

IMPORTING AND EXPORTING TEXT

------------------------------------------------------------------

You can import and export text to and from the editor with the

[Ctrl-X] {Tools|External files} command.  Select Import to read

text into the editor, or Export to write it out to file.  You are

then prompted to supply the name of the file to be read or

written.  Optionally, you may add a drive and path name.  In the

import command, you can select files from a pick list - see the

PICK LIST section for details.  If you name an existing file when

exporting, you will be asked if you want to overwrite it or append

it.

You can use import in any Cadet editing mode, being limited only

by the amount of space which is or can be made available.  The

imported text is inserted into the editor beginning at the current

line.  If you have multiple columns, the text is inserted either

row-by-row or column-by-column, depending on your current sort

order.  If there is insufficient room or memory, the imported text

is truncated and a warning is issued.

For normal text exports in TABLE and MIXED editing modes, the

contents of the editor are written out in the current sort order.

Be sure and sort the text as required with the editor's [Ctrl-O]

command before exporting.  In all editing modes except FILE,

AutoCAD  special characters are translated to the appropriate "%%"

codes as the text is written out to file.

In TABLE editing mode, you have an export option to write comma

delimited data files.  Each table cell is enclosed by double

quotes in a format suitable for importing into a spreadsheet or

data base.  Existing double quotes in the text are stripped out.

CADET v4.0                     50                    User's Manual

When you use the editor just for the purpose of exporting text

from AutoCAD, it is simplest to exit Cadet with the [Ctrl-Q]

command.  That way the Cadet interface will not have to inspect

each line of text for changes.

PRINTING

------------------------------------------------------------------

To print the document from Cadet, use the [Ctrl-Y] {Tools|Print

hard copy} command.  The default printer port, LPT1, can be

changed by the CADETIN Installation/Configuration program.  The

contents of the editor are sent to the printer as is, with no

additional formatting.  A form feed is issued at the end to eject

the last page.  You can press [Esc] at any time to stop printing.

SPELLING CHECKER

------------------------------------------------------------------

Cadet's integrated spelling checker has a 112,000+ word

dictionary.  To check spelling, use the [Ctrl-P] {Tools|Spell

check} command.  You have the option of checking the current word

or globally checking the document. Hint: To spell check entire

drawings, try the Cadet-Edit Global (CEG) interface.

When questionable spellings are found, you are presented with the

following prompt.

Unrecognized. <Suggest spell>/Edit word

/Add to dictionary/Ignore all/skip Once:

Press "E" to edit the word, the new spelling will be automatically

rechecked.  To add the word to the dictionary, press "A".  Be

careful with this option, verify spellings before you add them to

the dictionary.  Press "I" if you want to ignore the word and all

its future occurrences, or press "O" to skip and ignore the word

just once.

Press "S" or [Enter] to select from a list of suggested words.  A

pick list of up to eighteen words is presented.  Use the cursor

keys to highlight the correctly spelled word and press [Enter], or

use a mouse to pick it.  The options to edit, ignore or add the

word to the dictionary are still available.  If you do not see

your word in the list, use Edit to try a different spelling and

ask for suggestions again.  This usually brings up some new words

in the list.

Although the default settings are usually satisfactory, Cadet's

spell checker cache can be adjusted.  See the INSTALLATION chapter

for details.  Registered users receive WordMgr, a utility program

used to maintain the dictionary.  WordMgr can be used to remove

and add word lists in the dictionary.

CADET v4.0                     51                    User's Manual

WORD WRAP AND REFORMAT

------------------------------------------------------------------

Cadet's word wrap and paragraph reformat features are available in

both NOTES and FILE editing modes.  The word wrap default can be

set on or off with the CADETIN Installation/Configuration program.

Word wrap occurs at the length of the longest line read.  When

creating AutoCAD text with Cadet-Notes, you can set word wrap with

the Wrap justification option.  You can turn word wrap on and off

or change the word wrap position with the [Ctrl-W] {Tools|Word

wrap} command.  Enter the character position for word wrap's right

margin, or enter zero to turn word wrap off.  The word wrap

position can not be set less than 10 characters from the left

margin.

As you enter text with word warp on, the text automatically breaks

at the closest word within the designated margins, and wraps to

the line below.  If AutoIndent mode is on the line wraps to the

indentation level of the line above, otherwise the line wraps to

the home position.  If Squeeze mode is active, excess spaces are

removed from each line before wrapping.  See the EDITOR SETTINGS

section for more detail on AutoIndent and Squeeze. Automatic word

wrapping continues as long as you keep typing in new text.

To complete a paragraph, either skip a blank line or use the

[Enter] key to insert a hard carriage return.  The [Ctrl-I] insert

line command also produces hard returns.  Cadet uses these hard

returns as paragraph separators, and displays them on the screen

as solid rectangles.  These hard return marks are visual aids

only, and are not permanent.

Automatic word wrap is only active when typing new lines of text.

When editing previously entered text, word wrap is only partially

automatic.  Word wrap occurs only when the cursor passes the right

margin or the text line reaches its 255 character limit.  If the

margins are disfigured after editing, you can use the reformat

command to restore the original neatness.

Reformat text with the [Ctrl-G] {Tools|Reformat group} command.

You can choose one of three text groups to reformat; Paragraph,

Document, and Remaining paragraph.  If AutoIndent is inactive,

left margins are ignored.  When AutoIndent is active, the left

margin is established by the indentation level of the second line

of the paragraph - leaving the format of leading indentations,

bulleted and numbered paragraphs intact.  These rules are also

observed when documents are reformatted, paragraph boundaries are

honored.  The Remaining paragraph option only formats from the

current line to the end of the paragraph, and indents each line to

match the current one.

CADET v4.0                     52                    User's Manual

Squeeze mode also affects reformatting.  If Squeeze is active, all

excess spaces are removed during the reformat process.  Left

margins are unaffected by Squeeze.

If you have paragraphs with differing right margins, change the

word wrap position and format them individually, rather than by

document.  To merge paragraphs, remove the separators (hard

returns and blank lines) and reformat.  Hard returns can be

removed with the [Del] or [BkSp] keys, and blanks lines may be

removed with the [Ctrl-D] delete line command.

ZOOM TOGGLE

------------------------------------------------------------------

Cadet's zoom function is a condensed 43/50 line text mode for

VGA/EGA displays.  Normal text display is 80 columns by 25 lines.

VGA displays have an additional 50 line mode.  EGA displays have a

corresponding 43 line mode.  Use the [Ctrl-Z] {Display|Zoom

EGA/VGA} command to switch display modes.  This is a toggle

command, executing it again returns you to the previous display

mode.

VIEW RELATIVE TEXT POSITIONS

------------------------------------------------------------------

In Cadet's MIXED and TABLE multiple column editing modes, you can

view the position of text entities relative to one another with

the [Ctrl-V] {Display|View position} command.  While you cannot

edit the text in this viewer, it should quickly orient you on the

row and column sorting scheme.  Text may not be displayed in its

entirety - large tables and long text lines are truncated to fit

the screen.  More lines can be seen in Cadet's Zoom mode.  To exit

to normal editing mode, press [Esc].

SAVING AND LEAVING CADET

------------------------------------------------------------------

To save your work but continue editing, press [F2] {Exit|Save}.

When finished, the normal method of leaving is the [Ctrl-E]

{Exit|Save and end} command, which automatically saves your

changes before exiting.  Upon exiting, you are returned to the

environment from which you started - either DOS or AutoCAD.  When

using Cadet to create and edit AutoCAD text, the drawing is

updated upon return.

Another method of leaving the editor is the [Ctrl-Q] {Exit|Quit}

command.  Quit is an abort command, any changes since the last

save are discarded.  Similar to AutoCAD's quit command, you are

prompted for confirmation when you quit the editor.

CADET v4.0                     53                    User's Manual

------------------------------------------------------------------

CADET EDITOR COMMAND REFERENCE

------------------------------------------------------------------

CURSOR MOVEMENTS

------------------------------------------------------------------

[Left]  . . . Move left one space.

[Right] . . . Move right one space.

[Up]  . . . . Move up one line.

[Down]  . . . Move down one line.

[Enter] . . . Move down one line, inserting a new line and adding

a hard carriage return if insert mode is on.

Indents to same level as previous line if AutoIndent

is active, otherwise returns to the home position.

[Ctrl-Left] . Move left one word.

[Ctrl-Right]  Move right one word.

[Home]  . . . Move to the beginning of a text line, or top of a

list.

[End] . . . . Move to the end of a text line, or bottom of a list.

[Tab] . . . . Move right to the next tab stop (see Smart and Fixed

Tabs).  Pushes text right if insert mode is on.

[Shift-Tab] . In Fixed Tab mode, move left to the next tab stop.

[PgUp]  . . . Move up one page.

[PgDn]  . . . Move down one page.

[Ctrl-PgUp]   {Search|Head of doc}  Move to the top of the

document.

[Ctrl-PgDn]   {Search|End of doc}  Move to the bottom of the

document (last line with text).

[Alt-B] . . . {Search|Col/row bot}  In TABLE mode, move to the

bottom of the row or column.

CADET v4.0                     54                    User's Manual

[Alt-T] . . . {Search|Col/row top}  In TABLE mode, move to the top

of the row or column.

SPECIAL SYMBOLS AND CHARACTERS

------------------------------------------------------------------

[Alt-O] . . . AutoCAD %%o overscore toggle. represented on screen

as a bold upward pointing arrowhead.

[Alt-U] . . . AutoCAD %%u underscore toggle, represented on screen

as a bold downward pointing arrowhead.

[Alt-D] . . . AutoCAD %%d degrees symbol (�).

[Alt-P] . . . AutoCAD %%p plus/minus symbol (�).

[Alt-C] . . . AutoCAD %%c circle diameter symbol (�).

EDITING KEYS AND COMMANDS

------------------------------------------------------------------

[F1]  . . . . {Display|Help screen}  Display help.

[F2]  . . . . {Exit|Save}  Save text to file and continue editing.

[F10] . . . . Activate pull-down menus.

[Ins] . . . . Toggle insert/overwrite mode.  In insert mode,

characters are inserted at the cursor position, and

[Enter] adds hard carriage returns and inserts new

lines.  In overwrite mode, characters overwrite

existing characters.

[Esc] . . . . An UNDO command.  [Esc] is used to abort pending

commands.  [Esc] also can undo single line changes

before leaving the current line or using a global

command.

[Del] . . . . Delete the character above the cursor.

[BkSp]  . . . Delete the character to the immediate left of the

cursor.

[Alt-A] . . . {Display|About Cadet}  Displays CADET copyright

information screen.

CADET v4.0                     55                    User's Manual

[Ctrl-BkSp] . {Editing|Erase word}  Delete the Word to the right

of the cursor.  If the cursor is under a word, the

portion of the word above and to the right of the

cursor is deleted.  Also deletes hard carriage

returns.

[Ctrl-End]  . {Editing|Erase to end}  Deletes text from cursor

position to the end of line.

[Ctrl-Home] . {Editing|Erase to home}  Deletes text from the

cursor to the beginning of the line.

[Ctrl-A]  . . {Editing|Erase all}  Erase entire line of text.

[Ctrl-B]  . . {Block|Write block}  Write the block buffer to file.

[Ctrl-C]  . . {Block|Copy block}  Copy (paste) the block from the

block buffer.  The buffer must first be filled using

the [Ctrl-M] command.  Text to the right of the

cursor is displaced.

[Ctrl-D]  . . {Editing|Delete line}  Deletes text line or, in

TABLE mode, your choice of line or row.  Text below

is pulled up.

[Ctrl-E]  . . {Exit|End}  End, save changes.

[Ctrl-F]  . . {Search|Find text}  Find text string.  Choose Ignore

case, Whole words, and/or Global search.

[Ctrl-G]  . . {Tools|Reformat group}  Reformat group; Document,

Paragraph, or Remaining paragraph.  Available in

NOTES and FILE editing modes.

[Ctrl-H]  . . {Change|Height}  Change AutoCAD text height in line

or document.

[Ctrl-I]  . . {Editing|Insert line}  Inserts a new line or, in

TABLE mode, your choice of line or row.  Text below

is pushed down.

[Ctrl-J]  . . {Change|Juggle case}  Juggles upper case to lower

case and lower case to upper case, choose line or

document.

[Ctrl-L]  . . {Change|Lower case}  Change text to lower case,

choose line or document.

CADET v4.0                     56                    User's Manual

[Ctrl-M]  . . {Block|Mark block}  Mark a text block.  Cut and

paste, copy or delete the block, or change its case,

height or style.  Copied and cut blocks are moved

into a buffer.  The block can be inserted (pasted)

into another position with the [Ctrl-C] copy block

command or written out to a text file with the

[Ctrl-B] write block command.

[Ctrl-N]  . . {Change|Line space}  Change AutoCAD text line

spacing in NOTES and TABLE editing modes.

[Ctrl-O]  . . {Display|Sort order}  Change COL/ROW sort order in

TABLE editing mode, COL/ROW/PICK order in MIXED

editing mode.

[Ctrl-P]  . . {Tools|Spell check}  Check spelling of word or

document.  When questionable spellings are found,

you may choose to edit the word, add the word to the

dictionary, ignore all occurrences of the word,

ignore the word once, or select from suggested

spellings.

[Ctrl-Q]  . . {Exit|Quit}  Quit, abandon any changes since last

save.

[Ctrl-R]  . . {Search|Replace text}  Search-and-Replace text.

Choose Ignore case, Whole words, and/or Global

search.

[Ctrl-S]  . . {Change|Style}  Change AutoCAD text style of line or

document.  Choose from pick list of all defined

styles.

[Ctrl-TA] . . {Options|AutoIndent}  Turns AutoIndent mode on or

off.

[Ctrl-TM] . . {Options|Tab mode}  Select Smart Tabs or Fixed Tabs.

[Ctrl-TQ] . . {Options|Squeeze}  Set Squeeze word wrap mode on or

off.

[Ctrl-TS] . . {Options|Tab size}  Set tab size for Fixed Tab stops

and translation during file read.

[Ctrl-TT] . . {Options|Type speed}  Toggle the typematic rate

accelerator from Slow to Medium to Fast speed.

[Ctrl-U]  . . {Change|Upper case}  Change text line or document to

upper case.

CADET v4.0                     57                    User's Manual

[Ctrl-V]  . . {Display|View position}  View relative text

positions in TABLE and MIXED editing modes.  Press

[Esc] to exit.

[Ctrl-W]  . . {Tools|Word wrap}  Set Word-Wrap position in NOTES

and FILE editing modes.  Zero turns word-wrap off.

[Ctrl-XE] . . {Tools|External files|Export}  Export document to

file.  In TABLE editing mode you have an option to

write in comma delimited format.

[Ctrl-XI] . . {Tools|External files|Import}  Import an ASCII text

file into the editor.

[Ctrl-Y]  . . {Tools|Print hard copy}  Prints hard copy of

document.

[Ctrl-Z]  . . {Display|Zoom EGA/VGA}  Zoom in/out of VGA/EGA

condensed 8x8 font text mode.  VGA displays 50

lines, EGA displays 43 lines.

CADET v4.0                     58                    User's Manual

------------------------------------------------------------------

PROBLEMS AND ERROR MESSAGES

------------------------------------------------------------------

This section illustrates most of the problems which can occur when

initially setting up Cadet, or after making changes to your disk

directories or hardware.  You should begin by first reviewing the

INSTALLATION chapter of this manual, then trying some of the

solutions shown below before calling for technical support.

Chances are, the solution is simple, and in many cases can be

accomplished by merely reinstalling Cadet.

AUTOCAD INTERFACES DO NOT LOAD

--------------------------------------------

If you get something other than Cadet's AutoLISP interfaces after

entering one of the Cadet command names, you probably have a name

conflict with some other function or command.  To correct this,

see the section on AUTOLISP FUNCTION NAME CONFLICTS.

If you see the message "Insufficient memory -- AutoLISP disabled."

when you execute a 286 version of AutoCAD, you probably have too

many TSR (Terminate and stay resident) programs loaded.  You will

need to eliminate one or more of them if you wish to use AutoLISP

programs.

If you are running a 286 version of AutoCAD and getting messages

like "insufficient node space" or "insufficient string space" when

trying to execute the Cadet lisp routines, you have inadequate

memory set aside for the lisp programs you have loaded.  In most

cases this can be corrected by either:

1) using Cadet's memory management (see INSTALLATION),

2) adding a (vmon) command to the top of your ACAD.LSP file,

3) modifying your LISPHEAP and LISPSTACK settings,

4) using Extended AutoLISP (R10+ 286 versions), or

5) hardware permitting, upgrading to AutoCAD 386.

Check your AutoLISP Programmer's Reference and AutoCAD

Installation and Performance Guide for details on AutoCAD memory

management.

In cases, Cadet's demand loading routine for an AutoCAD interface

will say "Loading...", but then fall into a circular loop until it

is cancelled by the user or the AutoLISP stack is exhausted,

producing a "LISPSTACK overflow" message.  This results from

missing or improper settings of either the CADETL AutoLISP

directory variable or the CADETB AutoLISP version variable.  See

the CONFIGURATION VARIABLES section for more information.

CADET v4.0                     59                    User's Manual

AUTOCAD INTERFACES LOAD BUT CADET FAILS

------------------------------------------------------------------

The Cadet interfaces may start up all right, but after writing to

the editor a message "Unknown command. Type ? for list of

commands" is displayed.  First, make sure your ACAD.PGP file is

intact and includes the Cadet command lines described in the

installation chapter.  More often than not, this is an indication

that you have more than one ACAD.PGP file in your system, and you

installed Cadet into one AutoCAD is not using.  Check your drawing

directory and other support directories for ACAD.PGP duplicates.

The simplest fix is to use the CADETIN Installation/Configuration

program, which scans your hard drives to find all duplicates, and

install Cadet in each.

Similar to the above problem, after writing to the editor an error

message "bad command or file name" may be displayed.  This

indicates that the Cadet command lines in your ACAD.PGP file have

no path or the wrong path to CADET.  See the section titled MANUAL

INSTALLATION (OPTIONAL) for an example of how these command lines

should look.  Edit the command lines or use the CADETIN program to

correct the paths.

Another problem occurs when Cadet reports "Transfer file not

found."  This is a result of an improperly set CADETD variable,

CADET environmental variable, or a disk full problem.  See the

INSTALLATION and CONFIGURATION section, and review your directory

and path setup.

Memory problems surface when DOS reports "EXEC failure" or "Cannot

execute CADET.EXE", or when Cadet reports "Insufficient memory."

These messages indicate that you must allocate more memory for

Cadet to run.  Since Cadet runs in conventional memory, there are

two issues involved.  One, the amount of conventional memory which

is available before AutoCAD is run.  Two, the amount of that

memory which AutoCAD is releasing to shell applications such as

Cadet.  First, we discuss the available memory, then discuss

methods of getting AutoCAD to release more of it.

A significant amount of conventional memory is often consumed by

TSR programs.  Some are absolutely necessary; mouse drivers,

network drivers, and the like.  Others are used occasionally, if

at all, and are candidates for elimination or unloading.  Many

well behaved TSRs have the ability to unload themselves from

memory (if loaded last), these can be unloaded before starting

AutoCAD, then reloaded afterwards.  Another way to reclaim memory

consumed by TSRs, hardware permitting, is to use MS-DOS 5 and/or a

memory manager, such as QEMM, to load TSRs into high memory (above

640K).  MS-DOS 5 can also load DOS itself high for a significant

gain in conventional memory.

CADET v4.0                     60                    User's Manual

The methods of getting AutoCAD to release more memory to its shell

applications vary depending on which version of AutoCAD is being

used.  Some require a simple reconfiguration, others require

utility programs or updating AutoCAD.

If your installation uses a 286 version of AutoCAD, you can

increase the memory it releases by increasing the memory reserve

figures in the Cadet command lines in your ACAD.PGP file(s).  Try

raising the memory reserve 25000 bytes at a time, retesting after

each change.

CADET!,C:\CADET\CADET /A,325000,,0

CADETX,C:\CADET\CADET /X,325000,,4

With AutoCAD 386 you always get the maximum amount of memory for

shell programs regardless of the ACAD.PGP memory reserve.

Unfortunately, there is some memory which the DOS extender does

not release, as much as 200K in some cases.  The best solution for

AutoCAD 386 R11 is to use Autodesk's NEWDX utility to update the

Phar Lap Extender with a newer, smaller kernel version.  NEWDX is

shipped, but not installed, with R11 revision c2, and is available

from dealers and CompuServe's ACAD forum.

Another solution works with any version of AutoCAD 386 and can be

used in addition to NEWDX.  This involves using a shell utility

designed to recover memory from the DOS extender.  There are at

least two commercial applications which can do this, and one named

SHROOM (Shell Room) which AutoDESK ships with R11.  A shareware

version of SHROOM can be found in CompuServe's ACAD forum.  Oddly,

with one of the commercial utilities you actually need to decrease

the CADET! commands' memory reserve before it will function - try

a setting of about 64,000 bytes, you will get more.

DID I CHANGE THAT?

------------------------------------------------------------------

Occasionally, you may see Cadet report changing lines of text

which you pulled into the editor but did not edit.  This is due to

either minor realignments in text positioning, or Cadet's

translation of "%%" character codes.

When translating back to the drawing, lowercase is used for the

code letters (for example, %%u).  If you had originally entered

the code letter as uppercase, a change is reported.  Portions of

the Cadet translation feature can be disabled, see the chapters

INSTALLATION and USING THE CADET EDITOR for more information.

CADET v4.0                     61                    User's Manual

MOUSE/DIGITIZER PROBLEMS

------------------------------------------------------------------

To emulate a mouse when outside of AutoCAD, some digitizers

provide utility programs which must be executed before and/or

after a program run through AutoCAD's shell.  You can access these

for use with Cadet through the use of the CADET4.BAT batch file,

located in the Cadet directory, which is created during Cadet

installation.

The CADET4.BAT file has commands to execute and pass parameters to

Cadet, but you must edit it to add the utility commands specific

to your digitizer.  Command lines which execute your digitizer

utilities should include the full path name to the program.

An example CADET4.BAT is shown below.  A Summagraphics digitizer

is being used which requires that its MMRST program be run with a

/1 parameter after the editor is executed.  The Summagraphics

utilities are located in a directory named C:\SUMMA.  The first

two lines in the CADET4.BAT file already existed and are not

modified.

@ECHO OFF

C:\CADET\CADET %1

C:\SUMMA\MMRST /1

For your particular service, refer to your digitizer documentation

to see which included utilities must be executed before or after

an application.  Next, run the CADETIN Installation/Configuration

program and choose the batch file option, or modify your ACAD.PGP

files(s) to change the Cadet command lines to call the CADET4.BAT

file instead of CADET.EXE.  For example:

CADET!,C:\CADET\CADET4 /A,300000,,0

CADETX,C:\CADET\CADET4 /X,300000,,4

With certain digitizers, the AutoCAD cross-hairs may lock up after

using mouse emulation in an application run through AutoCAD's

shell such as Cadet.  In such cases, the COM ports must be reset

upon returning to AutoCAD.  Cadet can automatically reset the COM

ports in all its interfaces which call the Cadet editor.  This

option is controlled by the CADETC variable, which can be

configured by the CADETIN program.

If you are using Cadet's automated COM port reset and the Cadet

interfaces fail just as they return to AutoCAD, you need to rerun

the CADETIN program.  CADETIN needs to know if your AutoCAD

plotter driver is set to "None" or a plotter in order to configure

Cadet's COM port reset correctly.

CADET v4.0                     62                    User's Manual

------------------------------------------------------------------

INDEX

------------------------------------------------------------------

ACAD.LSP  . . . . . . . . . . . . .5, 6, 9, 10, 11, 12, 14, 15, 59

ACAD.PGP  . . . . . . . . . . . . . . . . . . 5, 9, 13, 14, 60, 61

ACAD environmental variable . . . . . . . . . . . . . . . . . . 10

accelerator . . . . . . . . . . . . . . . . . . . . . . .5, 37, 57

adding words to dictionary  . . . . . . . . . . . . . . 13, 51, 57

Alt-A (about Cadet) . . . . . . . . . . . . . . . . . . . . 35, 55

Alt-B (column/row bottom) . . . . . . . . . . . . . . . . . 36, 54

Alt-C (circle dia. symbol)  . . . . . . . . . . . . . . . . 38, 55

Alt-D (degrees symbol)  . . . . . . . . . . . . . . . . . . 38, 55

Alt-O (overscore symbol)  . . . . . . . . . . . . . . . . . 38, 55

Alt-P (plus/minus symbol) . . . . . . . . . . . . . . . . . 38, 55

Alt-T (column/row top)  . . . . . . . . . . . . . . . . . . 36, 55

Alt-U (underscore symbol) . . . . . . . . . . . . . . . . . 38, 55

attributes  . . . . . . . . . . . . . . . . 17, 30, 31, 35, 38, 41

AutoIndent  . . . . . . . . . . . . . . . . . . 36, 38, 52, 54, 57

AutoCAD versions  . . . . . . . . . . .5, 6, 7, 10, 11, 12, 59, 60

Backspace (BkSp) key  . . . . . . . . . . . . . . . . . 42, 53, 55

blank lines . . . . . . . . . . . . . . . . . . . . 33, 45, 52, 53

block, AutoCAD  . . . . . . . . . . . . . . . . . . 17, 30, 38, 41

block, text . . . . . . . . . . . . . . . . . . 42, 47, 48, 56, 57

block buffer  . . . . . . . . . . . . . . . . . . . 47, 48, 56, 57

cache . . . . . . . . . . . . . . . . . . . . . . . . . . . .6, 51

Cadet command line menu . . . . . . . . . . . . . . . . . . . . 16

Cadet editor  . . . . . . . . . . . . . . . . . . . . . .4, 35, 54

CADET.CFG . . . . . . . . . . . . . . . . . . . . . . . .9, 10, 13

CADET.LSP . . . . . . . . . . . . . . . . 6, 9, 10, 11, 12, 14, 15

Cadet-AlignText (CA)  . . . . . . . . . . . . . . . 17, 19, 32, 34

Cadet-Create Table (CC) . . . . . . . . . . . . 16, 24, 29, 35, 37

Cadet-Edit (CE) . . . . . . . . . . . . . . .8, 17, 30, 31, 34, 35

Cadet-Edit Attributes (CEA) . . . . . . . . . . . . . . 17, 30, 35

Cadet-Edit Global (CEG) . . . . . . . . . . . . . . 17, 31, 35, 51

Cadet-Edit Notes (CEN)  . . . . . . . . . . . . . . . . 17, 33, 35

Cadet-Edit Table (CET)  . . . . . . . . . . . . .8, 17, 19, 31, 35

Cadet-External File (CX)  . . . . . . . . . . . . . . . 17, 18, 35

Cadet-Import (CI) . . . . . . . . . . . . . . . . . . . . . 16, 27

Cadet-JustifyText (CJ)  . . . . . . . . . . . . . . 17, 19, 32, 34

Cadet-MultiText (CM)  . . . . . . . . . . . . . . . . . . . 16, 21

Cadet-Notes (CN)  . . . . . . . . . . . . . . . 16, 23, 29, 35, 52

Cadet-PlusText (CP) . . . . . . . . . . . . . . . . . . . . 16, 20

Cadet-Restore (CR)  . . . . . . . . . . . . . . . . .8, 13, 18, 34

Cadet-SetText (CS)  . . . . . . . . . . . . . . . . . . 17, 20, 21

CADETB variable . . . . . . . . . . . . . . . . . . . . . . .7, 59

CADETC variable . . . . . . . . . . . . . . . . . . . . . . .6, 62

CADETD variable . . . . . . . . . . . . . . . . . . . . .6, 11, 12

CADETL variable . . . . . . . . . . . . . . . . . . . . . . .6, 59

CADET v4.0                     63                    User's Manual

CADETM variable . . . . . . . . . . . . . . . . . . . . . . .7, 18

CADETP variable . . . . . . . . . . . . .8, 18, 21, 22, 24, 26, 29

CADETR variable . . . . . . . . . . . . . . . . . . . . .8, 18, 34

CADETS variable . . . . . . . . . . . . . . . . .7, 18, 32, 33, 41

CADETT variable . . . . . . . . . . . . . . . . .8, 18, 32, 33, 41

case, text  . . . . . . . . . . . . . . . . . . 47, 48, 49, 56, 57

change line spacing . . . . . . . . . . . . . . . . 21, 33, 50, 57

change sort order . . . . . . . . . . . . . . . . . . . 32, 41, 57

change text case  . . . . . . . . . . . . . . . 47, 48, 49, 56, 57

change text height  . . . . . . . . . . . . . . . . . . . . 49, 56

change text style . . . . . . . . . . . . . 20, 41, 47, 48, 49, 57

character codes . . . . . . . . . . . . . . . . .5, 38, 50, 55, 61

colors  . . . . . . . . . . . . . . . . . . . . . . . . 10, 40, 41

column  . 7, 8, 17, 19, 31, 36, 38, 41, 43, 45, 47, 50, 53, 54, 55

COM port reset  . . . . . . . . . . . . . . . . . . . . . . .6, 62

commands  . . . . . . . . . . . . . 14, 15, 16, 35, 36, 39, 40, 54

configuration variables . . . . . . . . . . . . . . . . . . .6, 18

conflicts . . . . . . . . . . . . . . . . . . . . . . . . . 11, 14

coordinates . . . . . . . . . . . . . . . . . . . . . 7, 8, 18, 41

copy block  . . . . . . . . . . . . . . . . . . . . . . 47, 56, 57

create text interfaces  . . . . . . . . . . . . . . 21, 23, 24, 27

Ctrl-A (erase all current line) . . . . . . . . . . 35, 42, 43, 56

Ctrl-B (write block to file)  . . . . . . . . . . . . . . . 47, 56

Ctrl-C (copy/paste block) . . . . . . . . . . . . . . . 47, 48, 56

Ctrl-D (delete line)  . . . . . . . . . . . . . . . . . 43, 53, 56

Ctrl-E (end/save) . . . . . . . . . . . . . . . . . . . 34, 53, 56

Ctrl-F (find text)  . . . . . . . . . . . . . . . . . . . . 49, 56

Ctrl-G (reformat group) . . . . . . . . . . . . . . . . 37, 52, 56

Ctrl-H (text height)  . . . . . . . . . . . . . . . . . . . 49, 56

Ctrl-I (insert line)  . . . . . . . . . . . . . . . . . 45, 52, 56

Ctrl-J (juggle case)  . . . . . . . . . . . . . . . . . . . 49, 56

Ctrl-L (lower case) . . . . . . . . . . . . . . . . . . . . 49, 56

Ctrl-M (mark block) . . . . . . . . . . . . . . . . 42, 47, 56, 57

Ctrl-N (line spacing) . . . . . . . . . . . . . . . . . . . 50, 57

Ctrl-O (sort order) . . . . . . . . . . . . . . . . . . 41, 50, 57

Ctrl-P (spell check)  . . . . . . . . . . . . . . . . . . . 51, 57

Ctrl-Q (quit) . . . . . . . . . . . . . . . . . . . . . 51, 53, 57

Ctrl-R (search-&-replace) . . . . . . . . . . . . . . . . . 48, 57

Ctrl-S (text style) . . . . . . . . . . . . . . . . . . . . 50, 57

Ctrl-T (set editor options) . . . . . . . . . . . . . . . . 36, 57

Ctrl-U (upper case) . . . . . . . . . . . . . . . . . . . . 49, 57

Ctrl-V (view relative position) . . . . . . . . . . . . 42, 53, 58

Ctrl-W (set word wrap)  . . . . . . . . . . . . . . . . . . 52, 58

Ctrl-X (external file import/export)  . . . . . . . . . 29, 50, 58

Ctrl-Y (print hard copy)  . . . . . . . . . . . . . . . . . 51, 58

Ctrl-Z (zoom) . . . . . . . . . . . . . . . . . . . . . . . 53, 58

Ctrl-BkSp (delete word) . . . . . . . . . . . . . . . . 42, 43, 56

CADET v4.0                     64                    User's Manual

Ctrl-End (delete to end of line)  . . . . . . . . . . . . . 42, 56

Ctrl-Home (delete to beginning of line) . . . . . . . . . . 42, 56

Ctrl-Left (word left) . . . . . . . . . . . . . . . . . . . 35, 54

Ctrl-PgDn (go to last line) . . . . . . . . . . . . . . . . 36, 54

Ctrl-PgUp (go to first line)  . . . . . . . . . . . . . . . 36, 54

Ctrl-Right (word right) . . . . . . . . . . . . . . . . . . 35, 54

cursor keys . . . . . . . . . . . . . . . . . . . . . . .5, 35, 54

cut & paste . . . . . . . . . . . . . . . . . . . . . . 47, 56, 57

degrees symbol  . . . . . . . . . . . . . . . . . . . . .5, 38, 55

delete block  . . . . . . . . . . . . . . . . . . . . . . . 47, 57

delete (Del) key  . . . . . . . . . . . . . . . . . 42, 43, 53, 55

delete line . . . . . . . . . . . . . . . . . . . . . . 43, 53, 56

delete word . . . . . . . . . . . . . . . . . . . . . . 42, 43, 56

diameter symbol . . . . . . . . . . . . . . . . . . . . .5, 38, 55

dictionary  . . . . . . . . . . . . . . . . . . .6, 11, 13, 51, 57

digitizer . . . . . . . . . . . . . . . . . . . . . . . .9, 13, 62

directory path  . . . . . 6, 9, 10, 11, 12, 15, 18, 41, 47, 50, 60

document  . . . . . . . . . . . . . 35, 36, 48, 49, 50, 51, 52, 54

edit text . . . . . . . . . . . . . . . . . . . . . 16, 17, 18, 35

elevation . . . . . . . . . . . . . . . . . . . 22, 23, 25, 28, 41

end command . . . . . . . . . . . . . . . . . . . . . . 34, 53, 56

End key . . . . . . . . . . . . . . . . . . . . . . . . 36, 40, 54

Enter key . . . . . . . . . . . . . 36, 39, 40, 45, 45, 47, 52, 54

entities  . . . . . . . . . . . . . . . . . . . . . . . . . 17, 35

erase . . . . . . . . . . . . . . . . . . . . . . . . . 42, 43, 56

Esc key . . . . . . . . . . . . . . 36, 40, 42, 43, 45, 49, 51, 55

exit  . . . . . . . . . . . . . . . 16, 36, 38, 39, 40, 53, 56, 57

export text . . . . . . . . . . . . . . . . . . . . . . . . 50, 58

extended AutoLISP . . . . . . . . . . . . . . . . . . . . 6, 7, 59

F1 function key (help)  . . . . . . . . . . . . . . . . 35, 36, 55

F2 function key (save)  . . . . . . . . . . . . . . 34, 36, 53, 55

F10 function key (pull-down menu) . . . . . . . . . . . 36, 38, 55

FILE editing mode . . . . . . . . . . . 35, 38, 41, 43, 45, 47, 52

find-text . . . . . . . . . . . . . . . . . . . . . . . 48, 49, 56

fixed height text style . . . . . . . . . . . . 22, 23, 25, 28, 49

global  . . . . .6, 17, 31, 35, 41, 42, 43, 45, 48, 49, 50, 51, 55

height, text  . . . . . . . .8, 21, 23, 25, 28, 38, 41, 49, 50, 56

help  . . . . . . . . . . . . . . . . . . . . . . . . . 35, 36, 55

Home key  . . . . . . . . . . . . . . . . . . . . . . . 36, 40, 54

hot-key . . . . . . . . . . . . . . . . . . . . 38, 39, 40, 47, 48

import text . . . . . . . . . . . . . . . . . . . . 16, 27, 50, 58

insert text block . . . . . . . . . . . . . . . . . . . 47, 48, 56

Insert key/insert mode  . . . . . . . . . . . . . . . . . . 36, 55

insert line . . . . . . . . . . . . . . . . . . . . . . 45, 52, 56

insert point  . . . . . . . . . . . . . . . . . .7, 32, 33, 34, 41

juggle case . . . . . . . . . . . . . . . . . . . . 47, 48, 49, 56

justification, text . . .7, 17, 19, 21, 23, 24, 27, 32, 33, 37, 52

CADET v4.0                     65                    User's Manual

line spacing  . . . . . . . . . . . . . 21, 22, 24, 33, 38, 50, 57

line, text  . . . . . . . . . . . . . . . . . . . . . . . . . . 35

loading Cadet . . . . . . . . . . . . . . . . . . . .7, 10, 15, 59

local . . . . . . . . . . . . . . . . . . . . . . . . . . . 35, 42

mark text . . . . . . . . . . . . . . . . . . . . . . . 42, 47, 57

match . . . . . . . . . . . . . . . . . . . . . . . . . . . 40, 48

memory management . . . . . . . . . . . . . . . . . . . .7, 18, 59

menu  . . . . . . . . . . . . . . . . . 16, 35, 36, 38, 39, 40, 55

MIXED editing mode  . . . . . . 17, 30, 35, 38, 41, 43, 45, 50, 53

mouse . . . . . . . . . . . . . .9, 13, 35, 36, 38, 40, 47, 48, 62

network . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

NOTES editing mode  . . . . . . 16, 23, 33, 35, 38, 43, 45, 50, 52

order . . . . . . . . . . . . . . . . . 32, 38, 41, 43, 45, 50, 57

overscore . . . . . . . . . . . . . . . . . . . . . . . . . 38, 55

overwrite mode  . . . . . . . . . . . . . . . . . . 36, 37, 45, 55

Page-Down (PgDn) key  . . . . . . . . . . . . . . . . . 36, 40, 54

Page-Up (PgUp) key  . . . . . . . . . . . . . . . . . . 36, 40, 54

paths . . . . . . . . . . 6, 9, 10, 11, 12, 15, 18, 41, 47, 50, 60

pick list . . . . . . . . . . . . . . . . . . . 18, 36, 40, 50, 51

PICK order  . . . . . . . . . . . . . . . . . . . . . . . . 41, 57

plus or minus symbol  . . . . . . . . . . . . . . . . . .5, 38, 55

print hard copy . . . . . . . . . . . . . . . . . . . . . . 38, 55

prompts . . . . . . . . . . . . . . . . . . . . . . . . 36, 38, 41

protected AutoLISP file . . . . . . . . . . . . . . . . . . . . 10

pull-down menus . . . . . . . . . . . . . . . . . . 35, 36, 38, 55

quit  . . . . . . . . . . . . . . . . . . . . . . . . . 49, 53, 57

ram disk  . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

reformat  . . . . . . . . . . . . . . . . . . . 23, 33, 37, 52, 56

replace . . . . . . . . . . . . . . . . . . . . . . . . . . 48, 57

restore . . . . . . . . . . . . . . . . . . . . . . .8, 13, 18, 34

row . . . . . . . 7, 8, 31, 36, 38, 41, 43, 45, 47, 50, 53, 54, 55

save  . . . . . . . . . . . . . . . . . . . . . 34, 37, 53, 55, 56

scroll  . . . . . . . . . . . . . . . . . . . . . . . . . . 36, 40

search-and-replace  . . . . . . . . . . . . . . . . . . . . 48, 57

settings  . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

shortcut commands . . . . . . . . . . . . . . . . . . . . . 14, 16

shortcut keys . . . . . . . . . . . . . . . . . . . . . . . 35, 54

sort order  . . . . . . . . . . . . . . 32, 38, 41, 43, 45, 50, 57

sorting coordinates . . . . . . . . . . . . . . . . . 7, 8, 18, 41

special characters  . . . . . . . . . . . . .5, 27, 38, 50, 55, 61

spell check . . . . . . . . . . . . .5, 11, 13, 31, 40, 41, 51, 57

Squeeze mode  . . . . . . . . . . . . . . . . . 37, 38, 52, 53, 57

status lines  . . . . . . . . . . . . . . . . . . . . . . . 36, 38

style, text . . . . . . . . 20, 21, 24, 26, 38, 41, 47, 48, 49, 57

support path  . . . . . . . . . . . . . . . . . . . . . . . 10, 15

symbols . . . . . . . . . . . . . . . . . . . . . . . . .5, 38, 55

system requirements . . . . . . . . . . . . . . . . . . . . . . .5

CADET v4.0                     66                    User's Manual

Tab keys  . . . . . . . . . . . . . . . . . . . . . 35, 36, 37, 54

tab mode  . . . . . . . . . . . . . . . . . . . . . . . 37, 38, 57

tab size  . . . . . . . . . . . . . . . . . . . . . . . 37, 38, 57

table . . . . . . . . . . . . . . . . . . . .8, 16, 17, 24, 27, 31

TABLE editing mode  . . . . . . 35, 36, 38, 41, 43, 45, 47, 50, 53

table tolerance factor, CADETT  . . . . . . . . .8, 18, 32, 33, 41

text files  . . . . . . . . . . . . . . . . . . . . 16, 17, 18, 27

text pitch factor, CADETP . . . . . . . .8, 18, 21, 22, 24, 26, 29

text restore  . . . . . . . . . . . . . . . . . . . .8, 13, 18, 34

transfer files  . . . . . . . . . . . . . . . . . . . . 11, 12, 60

translate . . . . . . . . . . . . . . . . . . . . . .5, 37, 50, 61

typematic rate accelerator  . . . . . . . . . . . . . . .5, 37, 57

underscore  . . . . . . . . . . . . . . . . . . . . . . . . 38, 55

undo  . . . . . . . . . . . . . . . . . . . . . 42, 43, 45, 49, 55

view  . . . . . . . . . . . . . . . . . . . . . . . . . 42, 53, 58

wildcards . . . . . . . . . . . . . . . . . . . . . . . . . 18, 41

word wrap . . . . . . . . . . . . . . . . . . . 23, 33, 37, 52, 56

zoom  . . . . . . . . . . . . . . . . . . . . . . . . . . . 53, 58

CADET v4.0                     67                    User's Manual