����� �����   ���������

����  ����   �����߱��                                 HS/Link

���   ���    ���    ��

���������    ���������                         High Speed Link

�����߱��      ���߱��

���   ���    ���   ���                  File Transfer Protocol

����� �����   ���������

����  ����    ��������

HS/Link

Version 1.1

Copyright (C) 1991, 1992 Samuel H. Smith

All Rights Reserved, Worldwide

PLEASE REFER TO HSLINK.NEW

FOR DETAILS OF RECENT PROGRAM CHANGES!

HS/Link is a high speed, full streaming, bidirectional, batch file

transfer protocol with advanced Full-Streaming-Error-Correction.  Each

side of the link is allowed to provide a list of files to be sent.  Files

will be sent in BOTH directions until both sides of the link are

satisfied.

HS/Link is not just another bidirectional protocol, it is also a very

fast protocol for normal downloading and uploading, incorporating some

new ideas (such as Full-Streaming-Error-Correction and Dynamic-Code-

Substitution) to make things happen a bit faster and with greater

reliability.

This program is not free!  You are hereby licensed to use this program

for a trial period of 30 days free of charge.  If, after 30 days, you

decide that you would like to continue using HSLINK, you must complete

the registration form (REGISTER.FRM) and return it to the author with the

required registration fee enclosed.

Registration removes the "UNREGISTERED" and "PLEASE REGISTER" messages,

as well as all time delays.  Registered copies broadcast their

registration number to the remote unit.

Please feel free to distribute HSLINK to your friends and local BBSes as

long as all of the files in this archive are included and unmodified.

See the LICENSE AND REGISTRATION section of this document for more

details.

HS/Link 1.1 User Documentation                                     - 1 -

Table of Contents

_________________________________________________________________

Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

Full Screen Status Display  . . . . . . . . . . . . . . . . . . . . .   5

Command Line Format . . . . . . . . . . . . . . . . . . . . . . . . .   6

Basic Command Line Options  . . . . . . . . . . . . . . . . . . . . .   6

HS/Link Configuration Manager . . . . . . . . . . . . . . . . . . . .   8

Advanced Command Line Options . . . . . . . . . . . . . . . . . . .    14

Command Line Examples . . . . . . . . . . . . . . . . . . . . . . .    20

Calling HS/Link from COMM programs  . . . . . . . . . . . . . . . .    21

Calling HS/Link from BBS programs . . . . . . . . . . . . . . . . .    22

Flow Control  . . . . . . . . . . . . . . . . . . . . . . . . . . .    24

Full Streaming Data Transmission  . . . . . . . . . . . . . . . . .    26

Full Streaming Error Recovery . . . . . . . . . . . . . . . . . . .    27

Dynamic Code Substitution . . . . . . . . . . . . . . . . . . . . .    28

Transmission of Minimal Blocks  . . . . . . . . . . . . . . . . . .    28

DSZLOG Statistic Logging  . . . . . . . . . . . . . . . . . . . . .    29

Network Compatibility . . . . . . . . . . . . . . . . . . . . . . .    30

Auto Downloading  . . . . . . . . . . . . . . . . . . . . . . . . .    31

Error Messages  . . . . . . . . . . . . . . . . . . . . . . . . . .    32

License and Registration  . . . . . . . . . . . . . . . . . . . . .    38

Support . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    39

Distributors  . . . . . . . . . . . . . . . . . . . . . . . . . . .    40

Index . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    42

HS/Link 1.1 User Documentation                                     - 2 -

Introduction

_________________________________________________________________

What is a file transfer protocol?

A file transfer protocol is a means of transferring information from one

computer to another.  The protocol has the responsibility of protecting

against errors in transit, and taking measures to correct any errors that

are detected.  The design and implementation of a protocol can make the

difference between slow and unreliable communication, and fast and

trouble free communication.

Not all protocols are created equal.  With today's high speed modems,

packet switched networks (such as PC Pursuit), and expensive long

distance charges, users demand more than ever that files get through

quickly and without corruption.

The HS/Link protocol is the result of years of communication experience,

and uses some very new and solid technology to see to it that you can

easily move files as quickly as possible, and with the least amount of

hassle.

The HSLINK program can be used from the DOS command line, from BATCH

files, or as an "external protocol" from most modern COMM programs and

BBS programs.  If your favorite BBS does not yet support the HS/Link

protocol, please ask your SYSOP to install it.

Recent advances in high speed modem technology have made it possible to

simultaneously transfer data at high speeds in both directions.  This

standardized technology is called "V.32"

or "V.32bis", and is available in many    ���������������������������Ŀ

new modems.  Until HS/Link, these         �Bidirectional features that�

advanced modems were left half idle, due  �really work...             �

to the single-direction nature of prior   �HS/Link can simultaneously �

file transfer protocols.  The             �send and receive files!    �

bidirectional capabilities of HS/Link     �����������������������������

take full advantage of these modems,

cutting online time as much as 50% when equal amounts of data are

transmitted and received.

HS/Link 1.1 User Documentation                                     - 3 -

Summary of Key Features

_________________________________________________________________

� Speed.

HS/Link operates at or very near peak efficiency, often reaching 98%

or more with pre-compressed files and non-buffered modems.  Even

higher speeds are possible with buffered or error correcting modems.

A number of features, such as fast 32 bit CRC protection, Full-

Streaming-Error-Recovery and Dynamic-Code-Substitution, combine to

give maximum performance and security.

� Simple interface for beginners.

In many cases, HS/Link requires only the COM port number and the

filename to be transmitted.  Configuration files are not required.

The program is easily installed in most COMM programs that support

external protocols.  The HSLINK program has multiple help screens,

providing a quick reference to the basic and advanced command line

options, including examples of how the program is used.

� Batch file transfers.

The protocol can handle any size batch of files, and can take file

specifications from either the command line or from a listing file.

� Bi-directional options that really work.

The program can simultaneously upload and download with a single

host request.  What's even better is how easy it is to use...

instead of a complex setup all you do is:

1) Tell the BBS the names of the files you want to download, and

then

2) Press PGUP (or whatever your COMM program uses to UPLOAD files)

and then tell your COMM program which files to upload

By virtue of the BBS being told that it is sending files and your

COMM program telling HS/Link that it too is sending files you then

have a bidirectional transfer.  There's nothing else to set up.  You

use the familiar BBS interface to request files to download and your

COMM program's familiar interface to tell it which files to upload.

It couldn't get easier!

� Aborted file crash recovery.

When desired, HS/Link can resume an aborted transfer, verifying all

existing data blocks to insure the resumed file completely matches

the file being transmitted.  This function can also update a file

that has only a small number of changed, added, or deleted blocks.

HS/Link 1.1 User Documentation                                     - 4 -

Full Screen Status Display

_________________________________________________________________

When HS/Link is operating, it produces a full screen status display,

divided into five windows.  For example:

����������� HS/Link v1.1 (5/11/92)  Copyright 1992 Samuel H. Smith ���������͸

� [UNREGISTERED] Courtesy of The Tool Shop BBS (818) 891-6780.               �

� Command line: -@tolap.cfg test\*.* -s2048 -w12 -a                          �

������������������������������������������������������������������������������

���������� 5 InComing Files, 710782 bytes ��������͸ ������� Settings ������͸

� \TMP\VIROSCAN.ZIP (79 blocks, 161003 bytes)      � �  COM2 open at: 115200 �

� File received OK, 2930 cps, 58 seconds.          � �Remote Serial#: 00001  �

� \TMP\PM.ZIP (37 blocks, 73745 bytes)             � �Remote Version: 1.10   �

� Block 14 (28672 bytes) 2622 cps                  � �        Window: NONE   �

� ������������۰��������������������� 18 seconds   � �    Block size: 2048   �

���������������������������������������������������� �      XON/XOFF: ON     �

��������� 9 OutGoing Files, 2297792 bytes ��������͸ � CTS handshake: OFF    �

� TEST\PHASE1.EXE (179 blocks, 365810 bytes)       � � RTS handshake: OFF    �

� File sent OK, 5907 cps, 01:01 minutes.           � �Slow handshake: ON     �

� TEST\PHASE2.EXE (204 blocks, 416884 bytes)       � �        Resume: OFF    �

� Block 20 (40960 bytes/w:20) 5570 cps             � �  Keep partial: ON     �

� ��۰������������������������������� 01:08 minutes� �     Overwrite: ON     �

���������������������������������������������������� �� ^X^X^X^X to abort! ���

�������������������������������� Batch Status ������������������������������͸

� Total 2047451 bytes (of 3008574) in 04:00 minutes, thruput 8509 cps.       �

� ���������������������������������۰���������������� 01:53 minutes remaining�

�������������������������� PLEASE REGISTER HS/LINK! ��������������������������

The first window contains the program identification and copyright.  When

you register your copy of HSLINK, this window will also show the

registered serial number.  Some warning messages may also appear in this

window.

The "Settings" window shows a summary of the various parameters that can

be altered with command line options.

The "InComing Files" window shows the status of files being received from

the remote system.

The "OutGoing Files" window shows the status of files being transmitted

to the remote system.

The "Batch Status" window summarizes overall session progress and

estimates the time remaining for the selected files.

To stop display bleedthrough under DesqView or DoubleDOS, use the -NV

option to disable direct video writing.

If desired, the full screen status display can be turned off.  To do

this, use the -NF command line option.  When running in -NF mode, a

simple line-oriented status display will scroll down the screen.  This

may be desired under some circumstances.

HS/Link 1.1 User Documentation                                     - 5 -

Command Line Format

_________________________________________________________________

To run the HSLINK program, a command line must be given.  The command

line has the following format:

HSLINK [options] [SENDFILE] [@DRIVE:\PATH\FILELIST]

SENDFILE  Indicates a list of files to be transmitted to remote computer.

The list may include the DOS wildcard characters '?' and '*'.

Specify ONLY files to be transmitted; received file names are

determined automatically.

@FILELIST Transmit list of files contained in FILELIST to remote

computer.  This is used when you want to make a long list of

files in advance, then transmit them all with a single command.

It is also used by BBS programs to produce long batches of

downloads.

�    Anything shown in brackets "[]" is optional and may not need to be

specified for proper operation of HS/Link.

�    The HSLINK command line is NOT case sensitive, and will work when

typed in upper, lower or mixed case.

�    Please make sure that HSLINK.EXE is in a search PATH directory.

Basic Command Line Options

_________________________________________________________________

The most important command line options are -P (set COM port), -U (set

download file directory), and -HS (enable slow handshake).

-Pport    This option specifies which COM port you are using.

Example:  -P2

-Udir     This option controls the destination directory for downloaded

files.  Without this option, all download files are placed into

the current directory.

Example:  -Uc:\bbsfiles

-HS       Handshake Slow (send XOFF and/or lower RTS during disk I/O).

This option may be required if your computer is unable to

simultaneously access the disk and the COM port.  If you see

frequent CRC error messages, even on clean phone lines, you

probably need to include -HS in your command line.

To abort an HS/Link transfer, press Control-X four or more times.

HS/Link 1.1 User Documentation                                     - 6 -

Configuration Files

_________________________________________________________________

Because of the large number of possible command line options, it is

possible to store frequently used command options in a configuration

file.  This eliminates the need to        ������������������������Ŀ

repeatedly type the options at a command  �HS/Link does NOT require�

line.  If you are calling HSLINK from     �configuration files.    �

multiple batch files, you can also use    ��������������������������

configuration files to store "common"

options, that can be changed in one place rather than separately in the

various batch files.

HSLINK will first search the current for

the HSLINK.CFG file.  If not found, it    ���������������������������Ŀ

will then search the directory where      �For best results, add your �

HSLINK.EXE is stored, followed by all     �HS/Link or COMM program    �

directories in the DOS PATH setting. If   �directory to your PATH     �

the HSLINK.CFG file is found, each file   �statement.                 �

line is loaded as a command line option.  �����������������������������

If you want to use an alternate configuration file, use the -@fname

command line option, which will cause 'fname' to be loaded.  For example:

HSLINK -@C:\WORK\TOLAP.CFG

will cause HSLINK to use C:\WORK\TOLAP.CFG as the configuration file.

If -@fname is not the first command line option, the default HSLINK.CFG

file will be searched for and loaded before the specified configuration

file.

Use -@ (without a filename) to prevent HSLINK from searching for a

configuration file, slightly speeding operation on systems that have a

long search PATH setting in effect.

To aid in creation and alteration of

configuration files, the HS/Link          ���������������������������Ŀ

Configuration Manager program, HSCONFIG,  �Use HSCONFIG to create and �

was written.                              �edit HS/Link option        �

�(configuration) files.     �

�����������������������������

HS/Link 1.1 User Documentation                                     - 7 -

HS/Link Configuration Manager

_________________________________________________________________

HSLINK includes the HS/Link Configuration Manager program, HSCONFIG.EXE.

With this program you can easily manipulate most of HS/Link's many

options.

The opening display for HSCONFIG looks like this:

Node-3 C:\HSL >hsconfig

HS/Link Configuration Manager 1.1 (C)1992 Samuel H. Smith; ALL RIGHTS RESERVED

������������������� New Configuration ��������������������Ŀ

�                                                          �

� The specified configuration file cannot be located.      �

�                                                          �

� This probably indicates that you are creating a new      �

� configuration for HS/Link.  All options will be set      �

� to default values.                                       �

�                                                          �

� Alternate configuration files can be specified on        �

� the command line to HSCONFIG.  The default configuration �

� file is HSLINK.CFG, and it may be located either in the  �

� current directory, or in any PATH directory.             �

�                                                          �

� Press <ENTER> or <ESC> to continue.                      �

�                                                          �

�  Index  <ESC>                                            �

������������������������������������������������������������

Can't find: C:\UTIL\HSLINK.CFG

HSCONFIG can be used to edit any configuration file, but the default is

to edit HSLINK.CFG in the current directory, or the same directory in

which HSLINK.EXE is found.  If you want to edit some other configuration

file, you must specify the filename on the HSCONFIG command line.  For

example:

HSCONFIG TOLAP.CFG

will create or edit TOLAP.CFG, which can later be used by HSLINK with a

command line like this:

HSLINK -@TOLAP.CFG

When no configuration filename is specified, and the default

configuration file HSLINK.CFG cannot be located, it will automatically be

created in the same directory that HSLINK.EXE is found in.  If you wish

to create the configuration file in a different directory, you must

explicitly specify the directory/filename to be used on the HSCONFIG

command line.

HS/Link 1.1 User Documentation                                     - 8 -

The primary screen in HSCONFIG is the "Basic Settings" screen, which

controls the most important settings for HS/Link operation.  The other

screens contain advanced settings that need not be changed until you

become more familiar with the program and begin to explore the advanced

capabilities.

������������������������� HS/Link: Basic Settings �����������������������͸

�                                                                         �

�    These are the most important settings for proper HS/Link operation   �

�                                                                         �

�  COM port number (1-8) . . . . . . . . . . . 1                          �

�                                                                         �

�  Directory for downloaded files. . . . . . . __________________________ �

�                                                                         �

�  Use "SLOW HANDSHAKE" during disk I/O. . . . N                          �

�                                                                         �

�������������������������������������������������������������������������;

��������������������� Welcome! ����������������������Ŀ

� Welcome to the HS/Link Configuration Manager.       �

� The easy way to handle HS/Link configuration files. �

�                                                     �

� Use the arrow keys to select entries on this page.  �

� Use the PgUp/PgDn keys to change pages.             �

� Press ESC to save changes when you are finished.    �

�                                                     �

� Use the F1 key to get HELP at any prompt.           �

�������������������������������������������������������

PgDn [next page]     PgUp [prior page]             F1 [help]     ESC [exit]

The HSCONFIG program contains context sensitive pop-up help for each and

every input prompt.  Simply press the F1 key to get an explanation of the

current entry.  For example:

������������������������������ -U �������������������������������Ŀ

� -Udir  > Download directory.                                    �

�                                                                 �

� This option controls the destination directory for downloaded   �

� files.  Without this option, all download files are placed into �

� the current directory.                                          �

�                                                                 �

�  Index  <ESC>                                                   �

�������������������������������������������������������������������

When in the online help system, press the SPACE and BACKSPACE keys to

select from the topics available at the bottom of the window, and press

ENTER to select a topic.  Press ESCAPE to return to the prior screen.

HS/Link 1.1 User Documentation                                     - 9 -

The "Advanced COM Settings" screen determines the settings of many

communications related options.  See the following section, "Advanced

Options" for a description of each of these options.

��������������������� HS/Link: Advanced COM Settings �������������������OVR͸

�                                                                           �

�         These settings control advanced communication features            �

�                                                                           �

� Non-Standard COM base (in HEX, 0 = standard COM port) . . . . . . 0000    �

�                                                                           �

� Non-Standard COM IRQ (1-15, 0 = standard COM port). . . . . . . . 0_      �

�                                                                           �

� COM port open speed (300-115200, 0 = automatic) . . . . . . . . . 0_____  �

�                                                                           �

� Effective modem-to-modem speed (0 = same as open speed) . . . . . 0_____  �

�                                                                           �

� Enable CTS hardware handshake . . . . . . . . . . . . . . . . . . Y       �

�                                                                           �

� Enable RTS hardware handshake . . . . . . . . . . . . . . . . . . Y       �

�                                                                           �

� Enable XON/XOFF software handshake. . . . . . . . . . . . . . . . Y       �

�                                                                           �

� Enable Carrier Detect checking. . . . . . . . . . . . . . . . . . Y       �

�                                                                           �

� Preserve original time/date stamp on received files . . . . . . . Y       �

�                                                                           �

���������������������������������������������������������������������������;

PgDn [next page]     PgUp [prior page]             F1 [help]     ESC [exit]

This screen manipulates the following options:

-PB (define non-standard COM base address),

-PI (define non-standard COM irq),

-B (COM port open speed),

-E (effective modem to modem speed),

-HC (disable CTS handshake),

-HR (disable RTS handshake),

-HX (disable XON/XOFF handshake),

-C (disable carrier detect monitoring) and

-NT (disable preservation of original date/time stamp)

These options will be described in detail in the next section.

HS/Link 1.1 User Documentation                                    - 10 -

The "Advanced Optional Features (part 1)" screen controls many advanced

protocol features.  See the following section, "Advanced Options" for a

description of each of these options.

�������������� HS/Link: Advanced Optional Features (part 1) ������������OVR͸

�                                                                           �

�            These settings control various optional features               �

�                                                                           �

� Attempt to Resume aborted transfers . . . . . . . . . . . . . . . N       �

�                                                                           �

� Keep partial files from aborted transfers . . . . . . . . . . . . N       �

�                                                                           �

� Allow received files to overwrite/modify existing files . . . . . N       �

�                                                                           �

� Enable file read/write buffering. . . . . . . . . . . . . . . . . Y       �

�                                                                           �

� Enable "Dynamic-Code-Substitution" logic. . . . . . . . . . . . . Y       �

�                                                                           �

� Size of transmitted data blocks (64-4096 bytes/block) . . . . . . 1024    �

�                                                                           �

� Number of blocks in transit before ACK is required (0-1000) . . . 8___    �

�                                                                           �

� Disable transmission of ACK for each block received . . . . . . . N       �

�                                                                           �

� Force remote unit to use local option settings. . . . . . . . . . N       �

�                                                                           �

���������������������������������������������������������������������������;

PgDn [next page]     PgUp [prior page]             F1 [help]     ESC [exit]

This screen manipulates the following options:

-R (resume aborted transfers),

-K (keep partial files),

-O (allow overwrite of receive files),

-NB (disable file buffering),

-NC (disable dynamic code substitution),

-S (data block size),

-W (acknowledge window size),

-A (disable receive acknowledge) and

-! (force remote to use local options)

These options will be described in detail in the next section.

HS/Link 1.1 User Documentation                                    - 11 -

The "Advanced Optional Features (part 2)" screen controls more advanced

protocol features.  See the following section, "Advanced Options" for a

description of each of these options.

�������������� HS/Link: Advanced Optional Features (part 2) ������������OVR͸

�                                                                           �

�              These settings control more optional features                �

�                                                                           �

� Idle method (0=none, 1=bios, 2=desqview, 3=ddos). . . . . . . . . 0       �

�                                                                           �

� Disable NS16550AN buffering logic . . . . . . . . . . . . . . . . N       �

�                                                                           �

� Enable "Minimal-Blocks" logic (requires MNP). . . . . . . . . . . N       �

�                                                                           �

� Incoming file duplication check list. . . . ____________________________  �

�                                                                           �

� DSZLOG status logging file. . . . . . . . . ____________________________  �

�                                                                           �

� Force CTS handshake even when CTS is initially missing. . . . . . N       �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

���������������������������������������������������������������������������;

PgDn [next page]     PgUp [prior page]             F1 [help]     ESC [exit]

This screen manipulates the following options:

-I (what to do during idle time),

-N5 (disable hardware buffering if NS16550 is present),

-NM (enable minimal blocks logic),

-NO (incoming file duplication checklist),

-FL (DSZLOG status logging file), and

-FC (force CTS handshake even when CTS is missing)

These options will be described in detail in the next section.

HS/Link 1.1 User Documentation                                    - 12 -

The "Screen Settings" screen allows you to control the format of the

status display screen, the colors to be used, and to enable or disable

direct-video screen updates.

������������������������� HS/Link: Screen Settings ��������������������OVR͸

�                                                                          �

�            These settings control the screen format and colors           �

�                                                                          �

�  Enable Full-Screen status display . . . . . . . . . . . . . . . . Y     �

�  Enable direct video writing (turn off in DesqView/DoubleDOS/etc). Y     �

�                                                                          �

�  Color of BORDER in full screen mode . . . . . . . . . . . . . . . 7__   �

�  Color of PROMPTS in full screen mode. . . . . . . . . . . . . . . 47_   �

�  Color of WINDOW TITLE in full screen mode . . . . . . . . . . . . 79_   �

�  Color of WINDOW CONTENTS in full screen mode. . . . . . . . . . . 31_   �

�  Color of PROGRESS GRAPHS in full screen mode. . . . . . . . . . . 14_   �

�                                                                          �

��������������������������������������������������������������������������;

��� Color Table ��������������������������������������������������Ŀ

�   0   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15  �

�  16  17  18  19  20  21  22  23  24  25  26  27  28  29  30  31  �

�  32  33  34  35  36  37  38  39  40  41  42  43  44  45  46  47  �

�  48  49  50  51  52  53  54  55  56  57  58  59  60  61  62  63  �

�  64  65  66  67  68  69  70  71  72  73  74  75  76  77  78  79  �

�  80  81  82  83  84  85  86  87  88  89  90  91  92  93  94  95  �

�  96  97  98  99 100 101 102 103 104 105 106 107 108 109 110 111  �

� 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127  �

��������������������������������������������������������������������

PgDn [next page]     PgUp [prior page]             F1 [help]     ESC [exit]

This screen manipulates the following options:

-NF (disable full screen status display),

-NV (disable direct video screen updates),

-CB (define border color),

-CM (define main/prompt color),

-CT (define title color),

-CW (define window contents color) and

-CG (define bargraph color)

These options will be described in detail in the next section.

HS/Link 1.1 User Documentation                                    - 13 -

Advanced Command Line Options

_________________________________________________________________

The HS/Link protocol program has a number of command line options, most

of which are rarely needed, or which are provided for advanced users to

obtain maximum performance and flexibility.

-@fname

Uses fname as an alternate configuration file.  Each line in the

specified file may contain a single command line option.  The

default is to search PATH directories for HSLINK.CFG.

-!

Take priority.  Locally specified -A, -HX, -S, and -W settings will

be used by the remote system.

-A

Disable transmission of ACK codes after each block.  This may

improve transfer speeds under some conditions.

-Bbaud

COM port opening baud rate 300-      ���������������������������Ŀ

115200 (default=current port OPEN    �Don't use the -B option if �

speed).                              �you don't need to!  In most�

This option controls the speed at    �cases HS/Link is able to   �

which the COM port will be opened.   �automatically determine the�

�proper port opening speed. �

Normally, the COM port speed can be  �����������������������������

determined from hardware and need

not be specified.

-C

Disable carrier detect checking.  Use this option when directly

connecting two machines via a null modem cable.

-CBcolor

Defines the border color in full screen mode.

-CMcolor

Defines the main screen color in full screen mode.

-CTcolor

Defines the window title color in full screen mode.

-CWcolor

Defines the window contents color in full screen mode.

-CGcolor

Defines the file and batch progress bargraph color in full screen

mode.

HS/Link 1.1 User Documentation                                    - 14 -

-Ebaud

Effective modem-to-modem baud rate.

This is used to estimate transfer times, and is included in the

DSZLOG output.  (default=current -B setting)

-FC

Force CTS hardware handshake.

This option prevents HSLINK from automatically disabling CTS

hardware handshake if CTS is not active initially.

-HC

Disables CTS hardware handshake.

CTS transmitter handshake is automatically enabled, but may conflict

with some unusual modem cables or serial ports.  If the CTS signal

is not active when HSLINK loads, it will automatically disable CTS

handshake for you.  Use the -HC command to manually disable CTS

handshake.

-HR

Disables RTS hardware handshake.

RTS receiver handshake is used when the slow-handshake option is

selected.  This form of handshake causes the RTS line to the modem

to be lowered whenever the disk is being accessed.  Some unusual

modem or cable arrangements may not tolerate the RTS line being

lowered.  In this case, the RTS handshake can be disabled with the -

HR option.

-HS

Handshake Slow (send XOFF and/or lower RTS during disk I/O).

The slow handshake option is available for systems with slow disk

access.  Use this if you get frequent CRC errors or COM Overrun

errors on otherwise clean lines.  The slow handshake option causes

XOFF to be sent and/or RTS to be lowered during any disk activity.

Even when -HS is active, the HS/Link protocol should transfer data

very nearly as fast as the slowest part of the link allows.

-HX

Disable XON/XOFF handshake.

XON/XOFF handshake is a software handshake scheme that should work

well on all combinations of modems and networks, and is enabled by

default.  If you prefer not to use XON/XOFF handshake, or you find

that it is not required for error-free transfer, you can disable it

with the -HX option.

Note that both the sender and the receiver must simultaneously

disable XON/XOFF handshake, otherwise it will remain in effect.

HS/Link 1.1 User Documentation                                    - 15 -

-Iidle

This option controls how idle time is given up under multitasking

environments.  The possible values are:

-I0  Default, do not give up idle time

-I1  Spend idle time checking keyboard status

-I2  Give up idle time under DesqView

-I3  Give up idle time under DoubleDOS

Some network server software, such as LanTastic, performs best when

the -I1 option is used.

-K

Keep partial files from aborted transfers.

-LFlogfile

Specify status log file.

This option allows you to override the DSZLOG environment variable

to specify the file to be used for status logging. Omit this option,

or leave blank to control this function through the DSZLOG

environment variable.

-N5

This option causes HS/Link to disable NS16550 buffering logic.  This

option may be needed with some early versions, or incorrect clone

versions of the NS16550 chip, which do not properly perform

buffering.

-NB

Disable buffering of receive and transmit files.  The default is to

divide approximately 24k of memory between InComing and OutGoing

files.

-NC

Disable Dynamic Code Substitution logic.

Certain byte codes cannot be transmitted directly over the COM link.

For instance, the codes for Control-X, Control-S and Control-Q must

be "escaped" to prevent a conflict with flow or program control.

The HS/Link protocol uses a new Dynamic Code Substitution system to

drastically reduce the number protocol overhead and "escape" codes

needed to transmit certain data streams.

Highly compressed data files (such as ZIP and GIF files) benefit the

most, since they often have many byte codes that cannot be

transmitted directly over the COM link.

-NF

Disable Full screen mode.  The standard status display takes over

the full screen (except the top 2 lines, which may contain a BBS

status line).

This option bypasses the full screen display and causes a line-

oriented output to be used.  Use this option if your environment's

screen management does not allow a full-screen display.

HS/Link 1.1 User Documentation                                    - 16 -

-NM

Enable Minimal Blocks logic.  Use    �������������������������Ŀ

with caution.  This option is        �HS/Link's Full Screen    �

recommended for use only on error-   �status display does not  �

corrected links, such as with MNP    �alter the first 2 display�

or V.42 connections.                 �lines.  The last display �

�line is also left alone. �

-NT                                       ���������������������������

Stamp current file time.   This

option stamps the current time/date on received files, instead of

retaining the original file time/date.

-NOdirlist

Incoming file duplication checklist.

This option allows you to check incoming files for duplication in a

list of directories.  This feature prevents incoming files from

duplicating files in directories other than the current -U setting.

The dirlist is a text file which contains one filename or directory

specification per line.  See MKDLIST documentation for special

instructions to speed CD-ROM duplication checking.

-NU

Block uploads.   This option is used by BBS programs that cannot

(yet) cope with bidirectional uploads.  It is used in place of the -

U option on the 'download' command line, and has the effect of

blocking any simultaneous upload requests from a user.

-NV

Disable direct Video for             �������������������������Ŀ

DesqView/DoubleDOS/etc.  Use this    �You may need to use -NV  �

option if you experience "bleeding"  �to stop display bleeding.�

of the status display from one       ���������������������������

partition to another.

-O

Allow receive files to overwrite existing files.

-Pport

Use COM port 1-8 (default=1).   Standard COM port definitions:

Channel   I/O Port  Interrupt

COM1      0x3F8     4

COM2      0x2F8     3

COM3      0x3E8     4

COM4      0x2E8     3

COM5      undefined

COM6      undefined

COM7      0x2E8     5

COM8      0x3E8     5

Use -PB and -PI instead of -P to define a non-standard COM port.

HS/Link 1.1 User Documentation                                    - 17 -

-PBbase

Set non-standard COM port base address.  Base addresses may be

specified in either decimal or hex.  Hex addresses must begin with $

or 0x.  For example, -PB1000, -PB$3E8 and -PB0x3E8 all specify the

same port base address.

-PIirq

Set non-standard COM port IRQ level to 1-15.  This option must be

used when -PB is used.  IRQ levels 8-15 are supported only on AT

class machines with cascaded interrupt controllers.

HS/Link 1.1 User Documentation                                    - 18 -

-R

Resume aborted transfer.  The        ������������������������Ŀ

receiver and transmitter will        �Use -R -K -O if you want�

verify the contents of an existing   �Download Crash Recovery �

file, and will begin transmission    ��������������������������

at the first block that does not

match.  This provides a reliable means of restarting an aborted

transfer, or of updating a modified file.  You must also use -O to

recover aborted downloads.

-Ssize

Sets transmit block size 2-4096 (default=1024) Larger block sizes

may result in higher transfer speeds under clean line conditions.

-Udir

Destination directory for received files (default=current directory)

-Wwindow

Number of blocks allowed without     ����������������������Ŀ

ACK 1-1000 (default=8)               �Use -W4 -S512 -! with �

Use -A to establish an "infinite"    �PC-Pursuit            �

window (i.e. no ACK required at      ������������������������

all).

The "ACK window" is the number of blocks that will be sent to the

modem before an ACKnowledge code is required.  If the specified

number of blocks have been sent and no ACK has yet been received,

output will be suspended until the ACK is received or the link times

out.  This provides a form of software flow control that works well

when parts of the link are very slow compared to other parts, such

as when PC Pursuit is in use.

The /w:XXX value displayed in the InComing Files window reports the

actual number of blocks that are being transmitted prior to

acknowledge.  Setting the -W value higher than the highest /w:XXX

value observed will have no further effect on transfer thruput.

HS/Link 1.1 User Documentation                                    - 19 -

Command Line Examples

_________________________________________________________________

HSLINK -?

Display a summary of HS/Link command line formats.

HSLINK -P1

Download from a BBS using HS/Link.  COM1 will be used at the current

port speed.  Received files will go into the current directory.

HSLINK -P2 -Uc:\bbsfiles -K

Download from a BBS using HS/Link.  COM2 will be used at the current

port speed.  Received files will go into the c:\bbsfiles directory.

Partial files from aborted transfers will be kept.

HSLINK -P1 -Uc:\bbsfiles -R -K -O

Continue an aborted download from a BBS using HS/Link.  COM1 will be

used at the current port speed.  Received files will go into the

c:\bbsfiles directory.

HSLINK -Uc:\bbsfiles c:\uploads\*.zip

Download from a BBS using HS/Link.  COM1 will be used at the current

port speed.  Received files will go into the c:\bbsfiles directory.

Simultaneously upload all *.ZIP files in the c:\uploads

subdirectory.

HSLINK file1.zip file2.zip file3.zip

Upload to a BBS using HS/Link.  COM1 will be used at the current

port speed.  Simultaneously downloads selected files from the BBS

into the current directory.

HSLINK -b115200 -p2 -c -hs -hc -a file1.zip file2.zip

Communicate files with a laptop computer using port COM2 and a three

wire null modem cable.

HSLINK -w4 -s512 -! -uc:\download

Download files through PC-Pursuit using a window of 4 blocks and a

block size of 512 bytes.  This setting stops most errors when PC-

Pursuit is heavily loaded.

HS/Link 1.1 User Documentation                                    - 20 -

Calling HS/Link from COMM programs

_________________________________________________________________

The HS/Link protocol can be easily installed in most modern COMM

programs.  Specific instructions have been prepared for many popular COMM

programs, and are available in separate files for download.

Communication program         Instructions posted in

_____________________         _______________________

BOYAN 4.01                    HS-BOY40.ZIP

BOYAN 5.0                     HS-BOY50.ZIP

{COMMO}                       HS-COMMO.ZIP

GT POWER 15.50                HS-GT15.ZIP

ProComm Plus and

ProComm Plus Test Drive       HS-PCPTD.ZIP

QMODEM 4.3                    HS-QMOD.ZIP

ROBO COMM 3.1                 HS-RC31.ZIP

TELEMATE 2.1                  HS-TMATE.ZIP

TELIX 3.1                     HS-TELIX.ZIP

Installation instructions for new COMM

programs are being written all the time,  ���������������������������Ŀ

so check with your local bulletin board   �Check The Tool Shop BBS for�

or on the Tool Shop for instructions for  �the latest version of      �

additional communication programs.        �HS-INST.ZIP.  New COMM and �

�BBS programs are frequently�

�added to the library of    �

�install instructions       �

�available.                 �

�����������������������������

HS/Link 1.1 User Documentation                                    - 21 -

Calling HS/Link from BBS programs

_________________________________________________________________

The HS/Link protocol can easily be installed in most modern BBS programs

as an external protocol.  Specific instructions have been prepared for

many popular BBS programs, and are available in separate files for

download.

BBS program                        Instructions posted in

___________________________        _______________________

Searchlight BBS                    HS-SLBBS.ZIP

Spitfire BBS version 3.0           HS-SF30.ZIP

UltraBBS version 2.08              HS-UB208.ZIP

Wildcat! Test Drive v. 2.15        HS-WC215.ZIP

Auntie 5.12                        HS-AU512.ZIP

GAP 5.1                            HS-GAP51.ZIP

MarkMail under Gap                 HS-MKML.ZIP

PCBOARD 14.5  (11/90)              HS-145.ZIP

PCBOARD 14.5a (10/30/91)           HS-145A.ZIP

PROBOARD 1.17                      HS-PB117.ZIP

PRODOOR 3.43 (10/30/91)            HS-PROD.ZIP

RemoteAccess 1.10                  HS-RA110.ZIP

RemoteAccess 1.11                  HS-RA111.ZIP

RemoteAccess                       HS-RA.ZIP

RBBS 17.3C (12/05/91)              HS-R173C.ZIP

Rybbs                              HS-RYBBS.ZIP

SuperBBS 1.15 Gamma4               HS-S1154.ZIP

Searchlight BBS                    HS-SLBBS.ZIP

Spitfire BBS version 3.0           HS-SF30.ZIP

UltraBBS version 2.08              HS-UB208.ZIP

Virtual BBS version 5.40           HS-VBB54.ZIP

Waffle                             HS-WAF.ZIP

Wildcat! Test Drive version 2.15   HS-WC215.ZIP

WWIV version 4.20                  HS-WWIV.ZIP

HS/Link 1.1 User Documentation                                    - 22 -

BBS Requirements

_________________________________________________________________

HS/Link's demands on a bulletin board program are extremely easy to fill.

The following is all that is necessary:

1)   The protocol accepts either a

filename to download or a list of    ������������������������Ŀ

files of the form                    �Send file lists are text�

@DRIVE:\PATH\DOWNLIST.LST.  The      �files that contain one  �

protocol will send only the          �path/filename per line. �

specified files if filenames         ��������������������������

(without leading @) are given, or

all of the files found in

C:\BBS\DOWNLIST.LST if @C:\BBS\DOWNLIST.LST is specified.

2)   The protocol will create a DSZLOG compatible log file indicating the

names of all files transferred as well as their status (refer to the

DSZ standard and the "DSZLOG Statistic Logging" section for the

specific contents of the file).

3)   Any files found in the DSZLOG file

which BBS did not tell the protocol  �������������������������Ŀ

to SEND are assumed to have instead  �HS/Link does NOT modify  �

been RECEIVED by the protocol.  The  �BBS upload listings!     �

BBS should then check for the        �The BBS program must     �

existence of such files and if       �detect uploads and do    �

found, ask for descriptions and      �it's own file management.�

post them in the upload directory.   ���������������������������

If the BBS program is not yet able to

cope with bidirectional uploads, the -NU  ����������������������Ŀ

option can be used on the 'download'      �Use -NU to block all  �

command line to prevent attempts to       �bidirectional uploads.�

perform a bidirectional upload.           ������������������������

HS/Link 1.1 User Documentation                                    - 23 -

Flow Control

_________________________________________________________________

Flow Control is required when the sending and receiving computers or

modems operate at different speeds, or when computers are unable to

simultaneously access disk drives and the COM port.  Flow Control is a

software and/or hardware means of slowing down the faster parts of the

connection to prevent overloading the slower parts of the system.

HS/Link uses both XON/XOFF (software) and RTS/CTS (hardware) handshake

when communicating with high speed modems or slow packet switched

networks.  The protocol also uses a window algorithm to suspend data

transmission if the receiver does not eventually respond to transmitted

data.

XON/XOFF handshake is a software handshake scheme that should work well

on all combinations of modems and networks, and is enabled by default.

If you prefer not to use XON/XOFF handshake, or you find that it is not

required for error-free transfer, you can disable it with the -HX option.

Note that both the sender and the receiver must simultaneously disable

XON/XOFF handshake, otherwise it will remain in effect.

The slow handshake option (-HS) is available for systems with slow disk

access.  Use this if you get frequent CRC errors or COM Overrun errors on

otherwise clean lines.  The slow handshake option causes XOFF to be sent

and/or RTS to be lowered during any disk

activity.  Even when -HS is active, the   ���������������������������Ŀ

HS/Link protocol should transfer data     �The -HS option stops most  �

very nearly as fast as the slowest part   �Overrun and CRC errors that�

of the link allows.                       �occur during or immediately�

�after disk I/O takes place.�

CTS transmitter handshake is              �����������������������������

automatically enabled, but may conflict

with some unusual modem cables or serial ports.  If the CTS signal is not

active when HSLINK loads, it will automatically disable CTS handshake for

you.  Use the -HC command to manually disable CTS handshake.

RTS receiver handshake is used when the slow-handshake option is

selected.  This form of handshake causes the RTS line to the modem to be

lowered whenever the disk is being accessed.  Some unusual modem or cable

arrangements may not tolerate the RTS line being lowered.  In this case,

the RTS handshake can be disabled with the -HR option.

If you have frequent CRC errors or COM Overrun errors, which persist in

spite of the handshake-slow option (-HS), you should consider upgrading

your serial port from the usual 8250 to a newer, buffered NS16550AN chip.

The buffered chips are highly recommended if you are operating at high

baud rates (>9600) or are accessing extended memory or are multi-tasking.

HS/Link 1.1 User Documentation                                    - 24 -

I have also had reports that use of large numbers of dos BUFFERS in

CONFIG.SYS can cause overrun errors.  Try reducing the BUFFERS setting to

10 or less, especially if you also use a DISK cache.

Some disk cache programs have also been reported to cause overrun errors.

Try turning the cache OFF before calling HSLINK, and turning it back ON

again after the transfer is completed.

Buffered ASYNC chips

_________________________________________________________________

The National NS16550AN is a pin compatible plug in replacement for 8250

and 16450 serial chips.  It contains hardware buffering to allow faster

operation in systems with high interrupt service latency.

If you have an NS16550AN buffered UART

chip, HS/Link will automatically enable   ���������������������������Ŀ

buffering for better performance with     �The NS16550AN chip stops   �

high speed modems and multi-tasking       �most Overrun and CRC errors�

environments.                             �encountered with COM port  �

�opening speeds of 9600 and �

The -N5 option may be needed with some    �higher.                    �

early versions, or incorrect clone        �����������������������������

versions of the NS16550 chip, which do

not properly perform buffering.

HS/Link 1.1 User Documentation                                    - 25 -

Full Streaming Data Transmission

_________________________________________________________________

The HS/Link protocol allows multiple data blocks to be transmitted before

an acknowledgement is required by use of block numbers combined with a

window algorithm.

The protocol also allows acknowledgements to be combined with data

flowing in the opposite direction, making it possible to achieve full

streaming simultaneously in both directions over a full duplex

communication link.

Even though the transmitters operate in full streaming mode, the

receivers normally will periodically acknowledge that data is being

received.  This acknowledgement can be disabled with the -A option.

Recent advances in high speed modem technology have made it possible to

simultaneously transfer data at high      ���������������������������Ŀ

speeds in both directions.  This          �The HST modem is not well  �

standardized technology is called "V.32"  �suited for bidirectional   �

or "V.32bis", and are available in many   �transfers.  Switch to V.32 �

new modems.  The bidirectional            �modes, if available, for   �

capabilities of HS/Link take full         �higher thru-put.           �

advantage of these modems, cutting        �����������������������������

online time as much as 50% when equal

amounts of data are transmitted and received.

Some older high speed modems, such as the HST and Hayes-V, do not

implement V.32 and are able to transfer quickly in only one direction,

while the other direction is relatively slow.  HS/Link will still perform

well as a single direction protocol with these modems, but bidirectional

thruput will be low due to the "ping

pong" effect of the modem switching the   ������������������������Ŀ

high speed channel back and forth.        �The "Ping Pong" effect  �

�slows bidirectional     �

�transfers on older 9600 �

�baud modems.            �

��������������������������

HS/Link 1.1 User Documentation                                    - 26 -

Full Streaming Error Recovery

_________________________________________________________________

The HS/Link protocol has the unique ability to recover from many

transmission errors without breaking out of full streaming mode.

This is important with buffered modems or packet switched networks with

long turnaround times.

Conventional protocols, such as Zmodem, simply reposition the transmitter

after an error, requiring a retransmission of not only the erroneous

data, but also the entire stream of data that was in transit at the time

the error was detected.

The HS protocol can correct bad blocks

without retransmitting intermixed good    ���������������������������Ŀ

blocks, resulting in excellent transfer   �Full-Streaming-Error-      �

speeds under moderate line noise          �Correction cannot stop flow�

conditions.                               �control failure-related    �

�errors.  See the section   �

�on Flow Control if you have�

�persistent errors.         �

�����������������������������

HS/Link 1.1 User Documentation                                    - 27 -

Dynamic Code Substitution

_________________________________________________________________

Certain byte codes cannot be transmitted directly over the COM link.  For

instance, the codes for Control-X, Control-S and Control-Q must be

"escaped" to prevent a conflict with flow or program control.

The HS/Link protocol uses a new Dynamic Code Substitution system to

drastically reduce number protocol overhead and "escape" codes needed to

transmit certain data streams.

Highly compressed data files (such as ZIP and GIF files) benefit the

most, since they often have many byte codes that cannot be transmitted

directly over the COM link.

The Dynamic Code Substitution feature can be disabled, if you wish, with

the -NC option.

Transmission of Minimal Blocks

_________________________________________________________________

The HS/Link protocol includes a Minimal

Blocks feature, which when activated,     ���������������������������Ŀ

causes the protocol to strip away         �Enable Minimal-Blocks with �

repeated information from the protocol    �the -NM option only if you �

frames and transmit only the minimal      �are operating with an error�

amount of information required for data   �free link.                 �

flow.                                     �����������������������������

This feature results in a savings of over 50% of the normal protocol

overhead characters in each block of data, while retaining full data

integrity.  All data and control blocks are fully protected by a 32 bit

CRC calculation, even when Minimal Blocks is in effect.

The final block in a file rarely ends on an exact transfer block

boundary.  The HS/Link protocol automatically shortens the final block,

eliminating extraneous data transmission at the end of file.

HS/Link 1.1 User Documentation                                    - 28 -

DSZLOG Statistic Logging

_________________________________________________________________

When you define the environment variable DSZLOG, the HSLINK program will

enable transfer logging to the specified filename.  The log file can also

be set with the -FL option.  For example, the AUTOEXEC statement:

SET DSZLOG=C:\LOGS\LOG.OUT

will cause HSLINK to log files transmitted to C:\LOGS\LOG.OUT.  This form

of logging provides compatibility with programs that are designed to

operate with DSZ or other DSZLOG-type protocols.

The log file consists of a single line of text for each file transferred.

Here are some sample log entries:

h 227596 10100 bps 1138 cps   0 errors   112 2316 V:\UPLOAD\ZLABP_13.ZIP 0

H 177901 10100 bps 1116 cps   1 errors     0  749 U:\PLAY1\ECOMM22.ZIP 0

h  16155 10100 bps 1061 cps   0 errors     4  795 W:\ULW\WINROACH.ZIP 0

L 136991 10100 bps    0 cps   0 errors    55 1823 W:\ULW\WINDE223.ZIP 11

E 152614 10100 bps    0 cps  14 errors    22   38 W:\ULW\CLPGR21C.ZIP 0 2

Each line contains the following fields:

Size      Description

----      -------------------------------------------

1         'H'=file received from remote

'h'=file sent to remote

'E'=file receive was aborted

'e'=file transmit was aborted

'L'=lost carrier during file receive

'l'=lost carrier during file transmit

7         The length of the file, in bytes.   If the transfer was

aborted, this field will contain the number of bytes

transferred before the transfer was aborted.

6         The effective modem-to-modem connect speed.

4         Literally ' bps'

5         The average transfer rate, in characters per second.

4         Literally ' cps'

4         The total number of blocks which required retransmission during

the transfer.

7         Literally ' errors'

6         The number of flow control restrictions.

5         The length of the last block transferred in the file.

?         The full pathname of the file which was transferred.

?         The Serial Number of the remote HSLINK, if registered.

Otherwise 0.

HS/Link 1.1 User Documentation                                    - 29 -

Network Compatibility

_________________________________________________________________

All file processing is fully network and

"share" compatible.  HSLINK can be        ������������������������Ŀ

easily installed in multi-user or         �HS/Link automatically   �

networked computers.  This works with     �enables file sharing in �

DOS version 3.0 and later.                �network environments.   �

��������������������������

When transmitting a file, the file is opened in "allow read, deny none"

mode.

When receiving a file, the newly created file is created in "allow read,

allow write, deny all" mode, which prevents other programs from altering

the file which is being created.

If a file sharing violation is detected when opening a file, the program

passes control to the system's current "critical error" handler.  To

automate recovery from file sharing violations, you should consider

installing my FATAL14.ZIP program, which will automatically retry after

sharing violations, and which will keep a detailed log of all critical

errors in a system since the time the program was loaded into memory.

All files transmitted or received with HS/Link retain their original

exact size and time/date stamp.  No extraneous data is added beyond EOF

as with some older protocols.  The current date/time will be stamped on

incoming files if the -NT option is used.  Otherwise, the original file's

time/date stamp will be retained.

When run under DOS 2.x, HSLINK automatically reverts to non-share file

modes.

HS/Link 1.1 User Documentation                                    - 30 -

Auto Downloading

_________________________________________________________________

It is possible for some terminal

programs to support HS/Link auto          ���������������������������Ŀ

downloading by means of "scripts" which   �With HS/Link Auto-Download �

wait for the HS/Link "ready" packet, and  �You don't have to hit any  �

then shell out to the HSLINK receiver     �additional keys to execute �

protocol.                                 �the HSLINK driver.         �

�����������������������������

This capability is currently active when HS/Link is installed in {COMMO}

and BOYAN 5.0, any may be available soon under many other terminal

programs as the respective authors add the capability to recognize

external protocol signon signatures.

The HS/Link "ready" packet is repeated at 10 second intervals when

waiting for the HSLINK receiver to be started.  The format of the "ready"

packet is:

"HS*" ^B "R"

or in hex,

0x02 0x52

When this sequence of characters is received during a terminal

session, the HSLINK protocol should be executed.

The HSLINK program sends a different "ready" packet when no files need to

be sent, but upload files are expected from the remote system.  This

"ready" packet signature can be used to implement HS/Link Automatic

Upload.  The format of the "upload ready" packet is:

"HS*" ^B "Q"

or in hex,

0x02 0x51

When this sequence of characters is received during a terminal session,

an HS/Link upload should be started.   The three character prefix "HS*"

was added with version 1.1 and is needed only if the shorter 2 character

signatures are being used for another purpose.

HS/Link 1.1 User Documentation                                    - 31 -

Error Messages

_________________________________________________________________

Bad ____ CRC

These messages indicate that an error was detected in a control

message between the sender and the receiver.  All control packets

are protected by a 32 bit CRC calculation to provide highly secure

and reliable communication.

If you repeatedly receive the "Bad ready packet CRC" message, it may

indicate that the internal protocol formats have been changed and

that you need to upgrade to a newer beta copy of HSLINK.

Bad data CRC (#)

Dropped bytes -# (#)

Noise burst # (#)

These messages indicate that an error was detected in a block of

data in the file being received.  "Dropped bytes" indicates that

some expected bytes did not arrive, and "Noise burst" indicates that

more bytes than expected arrived.  Error correction is automatic,

because of Full-Streaming-Error-Correction, may occur without

breaking the stream of data being received.

If this message occurs at regular intervals, it is a sign of flow

control failure.  In most cases, this can be corrected with the

addition of the -HS command line option.

COM#: Break!

COM#: Framing!

COM#: Overrun!

COM#: Parity!

These messages indicate that an error was detected in the serial

port receiver.  Framing and Parity errors often indicate a baud rate

mismatch.  Overrun errors usually indicate the need for flow control

or for an upgrade to a buffered serial chip.  Overrun errors can

often be reduced or eliminated with the use of the -HS command line

option.

Can't create file '___'

This indicates that the received file could not be created.  This

usually indicates that the directory specified in the -U command

line option is invalid, or that the disk is full.

Carrier lost!

This indicates that Carrier detect was dropped, aborting the HSLINK

session.  If your connection does not provide a carrier detect

signal, you can disable this check with the -C command line option.

This is often required when directly connecting two computers by

means of a "null modem" cable.

Control-X abort!

This indicates that the control-x character was received twice in

succession, either over the COM port or from the local keyboard.

HS/Link 1.1 User Documentation                                    - 32 -

Disk write failure!  (#)

This usually indicates that the disk is full and that further

downloads cannot be received.

Flow control released (#### t:# r:#)

If output flow has been suspended by either software or hardware

flow control for more than 15 seconds, the program will assume that

there has been a failure of flow control and will release the hold

so data can continue.  The codes in parenthesis indicate which flow

controls were active at the time (C=CTS was down, T=transmitter XOFF

was pending, R=receiver XOFF was pending, A=transmitter is currently

active) and the counts of bytes in the transmit and receive holding

queues.

FILE was incomplete!  (# #)

This message indicates that the transfer was aborted in some way

before the specified file was completely received.  Normally

incomplete files are erased from disk, but they will be retained of

the -K command line option is used.

Not allowed to overwrite existing file '___'

This indicates that a file was received but there was another file

with the same name already in the download file directory.  Either

erase the duplicate file, or use the -O option to allow existing

files to be overwritten.

Insufficient memory to load program.

Not enough memory!

There was not enough memory available to run HSLINK.  Free

additional memory by removing "TSR" software, device drivers,

ramdisks or disk caches.  If your COMM program has the option of

swapping to DISK or EMS, enable the option to free additional memory

for HSLINK's use.  Current beta copies of HSLINK require about 64k

of RAM, but the specific amount of memory required is subject to

change from beta to beta.  You can reduce the amount of memory

required if you use the -NB command line option to disable file

buffering.

Read failure at #

This indicates that there was a disk error reading a file being

transmitted.

Remote not responding!

This indicates that the remote computer did not execute HSLINK and

establish a connection within the two minute timeout period.

HS/Link 1.1 User Documentation                                    - 33 -

Information Messages

_________________________________________________________________

Receive timeout (#)

ACK timeout, resend #

This message indicates that it has been too long since the receiver

responded to data transmission.  The last data block is repeated in

an attempt to wake up the receiver.

Block size: #

This message indicates the actual block size being used for file

transfers.  The block size is controlled by the -W parameter, and

may be overridden by the remote computer if the remote uses the -!

(take priority) option.

COM#: # bps

This message indicates the actual COM port speed in use, either as

specified with the -B command line option, or as determined from the

current hardware settings.

HS/Link finished! (t:# r:#) Exit code = #

When HSLINK terminates, it sets the dos ERRORLEVEL to the specified

value.  This can be checked in batch files to detect error

conditions.  Under normal conditions, HSLINK exits with an

ERRORLEVEL of 0.  If, however, the session terminated due to errors,

or was aborted by the operator, the ERRORLEVEL will be non-0.  The

following ERRORLEVEL codes are possible:

ERRORLEVEL          Reason for link termination

----------          ----------------------------------------

0           One or more files transferred successfully

1           ^X typed at local keyboard

2           ^X received over COM port

3           Incompatible BETA versions

4           No files transmitted or received

5           Remote did not respond within 2 minutes

6           Too many framing errors, possible -B mismatch

7           Possible tampering or corruption in HSLINK.EXE

8           Possible tampering or corruption in HSLINK.EXE

9           COM port not responding, incorrect -P options

10           Carrier lost during initial handshake with remote

11           Link aborted due to CTS lockup

COMM and BBS programs should use the DSZLOG output file to determine

more specific details of transfer results.

The (t:# r:#) display indicates the number of files successfully

transmitted and received.

HS/Link 1.1 User Documentation                                    - 34 -

File received OK, # cps, # seconds.

The file was received correctly and is now closed.  The average

transfer speed is calculated based on the elapsed time between

receiving the file open packet and the last byte of the file.

Transfer speeds are calculated using the DOS timer tick, which is

not accurate for transfers taking less than 1 second to complete.

For very short transfers, a cps rating of 9999 will be returned.

File sent OK, # cps, # seconds.

The file was transmitted correctly and is now closed.  The average

transfer speed is calculated based on the elapsed time between

sending the first byte of the file and the last byte of the file.

Transfer speeds are calculated using the DOS timer tick, which is

not accurate for transfers taking less than 1 second to complete.

For very short transfers, a cps rating of 9999 will be returned.

File skipped!

The transmitter prints this message when the receiver requests the

current file be skipped.  This usually happens when the receiver

detects a duplicate filename and is not allowed to overwrite

existing files.

Handshake: CTS XON/XOFF SLOW

This message indicates the current combination of flow control

options that are in effect.  CTS indicates CTS hardware flow control

is active.  XON/XOFF indicates that XON/XOFF software flow control

is active.  SLOW indicates that flow will be restrained during disk

I/O to prevent overrun errors.

Out of sequence # (#)

This indicates that the received block was out of sequence,

indicating that a block was possibly missed or lost during

transmission.  The receiver will request the transmitter to

retransmit the missing block(s).

Received block # (# bytes) # cps

This displays the current status of the file being received.  The

current number of blocks and bytes received is displayed after each

block is received.  The estimated transfer speed in ch/sec is also

displayed.

Received block # (# bytes) E#

When "E#" is displayed following the received block # message, it

indicates that error recovery is in progress, and that the specified

block number was in error and is in transit from the transmitter.

When the erroneous block has been received correctly the E# display

will advance to the next erroneous block number, or will be erased

if no more erroneous blocks are pending.

Received block # (# bytes) R

When "R" is displayed following the received block # message, it

indicates that current block has been previously received correctly

and is therefore a "repeat" block.  Repeat blocks occur under severe

error conditions and are usually corrected automatically.

HS/Link 1.1 User Documentation                                    - 35 -

Receiving file: ___ (# blocks, # bytes)

This message indicates that a new file is being received from the

transmitter.  The file size is displayed in blocks and bytes.

Resending block #

This message indicates that the receiver has detected an erroneous

block and is requesting a retransmission to correct the data.  Data

flow continues where it left off prior to the resend, allowing error

recovery to take place without breaking out of full streaming mode

and without discarding data that is already in transit to the

receiver.  This is the basis of Full- Streaming-Error-Correction.

Restarting current file

This indicates that the receiver was unable to recover from errors

encountered in processing the current file being transmitted, and

has requested that the file be started again from scratch.

Seek from block # to #

Under the most severe error conditions, the receiver may request the

transmitter to discard data that is in transit to the receiver and

to rewind to an earlier position in the file and resume

transmission.

Sending block # (# bytes/w:#) # cps

This is the normal status display while a file is being transmitted.

The message indicates the latest block number to be transmitted, and

in parenthesis indicates the number of bytes transmitted and the

current number of blocks that have been transmitted without

acknowledgement, which represents the size of the transmit "window"

that is currently in use.  If this value hangs at the current -W

setting, you may benefit from a higher -W setting on the command

line.  The estimated transfer speed is also reported in ch/sec.

Sending file: ___ (# blocks, # bytes)

This message indicates that a new file is being transmitted to the

receiver.  The file size is displayed in blocks and bytes.

Transfer aborted!

This message indicates that the current file transfer has been

aborted for some reason.  The specific reason for the transfer abort

is usually also printed on the screen.

Waiting for remote.

This message indicates that HSLINK is waiting to handshake with the

remote computer.  HSLINK will wait for 60 seconds for the other

computer to come online; after that the transfer is aborted.

Warning: CTS signal missing!  -HC will be assumed.

The CTS hardware handshake is normally enabled by default.  If, on

initial loading, HSLINK finds that the CTS signal is down, it

assumes that the current hardware configuration is not using CTS

handshake and automatically disables further CTS handshake checks.

Use the -HC command line to manually disable CTS handshake.

HS/Link 1.1 User Documentation                                    - 36 -

Window: #

This indicates the actual number of blocks that are allowed to be in

transit without acknowledgement.  This is controlled by the -W

parameter, and may be overridden by the remote computer if the

remote uses the -!  (take priority) option.

Window: NONE

This indicates that the -A command line option has been specified,

and that the transmitter is required to continue indefinitely

without receiving any kind of acknowledgement from the receiver that

data is being received correctly.

HS/Link 1.1 User Documentation                                    - 37 -

License and Registration

_________________________________________________________________

This program is not free!  You are hereby licensed to use this program

for a trial period of 30 days free of charge.  If, after 30 days, you

decide that you would like to continue using HSLINK, you must complete

the registration form (REGISTER.FRM) and return it to the author with the

required registration fee enclosed.

Registration removes the "UNREGISTERED" messages as well as all time

delays.  Your registration number will be broadcast to the remote copy of

hslink.

Basic HSLINK registration costs $20 US, which includes a registration

certificate showing a Serial Number and Password.  For $30 US, a disk

containing the latest release of HSLINK, along with a registration

certificate can be obtained.  Printed and bound operation manuals are

also available for $15 each.  Each registration is valid for use on a

single CPU or on a single BBS system (multi-node systems require only a

single registration).

Please feel free to distribute unregistered HSLINK to your friends and

local BBSes as long as all of the files in this archive are included and

unmodified.

Send your registration payment to:

SAMUEL H.  SMITH

P. O. BOX 4808

PANORAMA CITY, CA 91412-4808

U.S.A.

HS/Link registration can also be ordered ON LINE by credit card through

any of the Tool Shop BBS phone lines.

LICENSE AGREEMENT FOR SHAREWARE DISTRIBUTORS

Shareware distributors may distribute this program as long as the author

has been notified, in writing, prior to ANY distribution.  The "per disk"

charge may not exceed $6 and the shareware distributor will not change

the above license fee for the end user of HSLINK.  End users are not

exempt from registering HSLINK with the author at the normal registration

fee.

HS/Link 1.1 User Documentation                                    - 38 -

Support

_________________________________________________________________

HS/Link was Written by Samuel H. Smith.   Contact me at:

The Tool Shop BBS

Phone number             Modem type

--------------      --------------------------------------------

(818) 891-6780      US Robotics 2400         (free line)

(818) 891-3772      US Robotics HST 9600     ($20/yr subscription)

(818) 891-1344      Supra v.32bis/v.42bis    ($20/yr subscription)

You will always find the latest release version of HS/Link on the Tool

Shop, as well as a variety of support files and programs.

HS/Link is also supported by a number of authorized distributors

throughout the world.  Please refer to DISTRIB.DOC for the latest

distributor list.

The HS/Link Support Conference is carried by UN'I-Net/US, SmartNet, RIME

and possibly a number of other echo networks.  You can get direct product

support through a local BBS that carries this echo conference.

DISCLAIMER

IN NO EVENT WILL I BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING

ANY LOST PROFITS, LOST SAVINGS OR OTHER INCIDENTAL OR

CONSEQUENTIAL DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO

USE THE PROGRAM, OR FOR ANY CLAIM BY ANY OTHER PARTY.

HS/Link 1.1 User Documentation                                    - 39 -

Distributors

_________________________________________________________________

The following individuals are valid distributors as of 04/23/92.

UNITED STATES

William G. Poissant

AMCOM

5530 State Road

Cleveland, Ohio 44134-2260

(216) 398-1331 (voice)

(216) 398-1330 (fax)

(216) 526-9480 AMCOM BBS 2400 baud   (8 lines)

(216) 526-9490 AMCOM BBS HST 14,400 v.32/V,32bis (3 lines)

(216) 526-9096 AMCOM BBS HST/DS 14,400 v32/v32bis/v42/v42bis

UNITED STATES

Gene De Libero

East Coast MIDI(tm) Information Systems

21 September Street,

Coram, New York 11727

(516) 928-4284 (voice)

(800) 736-0181 (voice - HS/Link orders ONLY!)

(516) 928-4986 BBS HST 14,400/v42bis (multi-node)

CANADA

Daniel Coulombe

Synapse BBS

22 Beloeil,

Gatineau, Quebec J8T 7G3

(819) 561-1697 (VOICE)

(819) 561-4321 (v.32)

(819) 243-0306 (Hayes)

(819) 561-0950 (HST)

EUROPE - Netherlands

Arthur A. Mol

The Owl's Nest

PO Box 227

3760 AE Soest

Netherlands

+31-2155-26527 (FAX)

+31-2155-12571 (BBS up to 14.4Kb HST/V32)

+31-2155-26527 (BBS node 2 shared w/fax)

HS/Link 1.1 User Documentation                                    - 40 -

EUROPE - Germany

Frank Maroke

BMS-Online / BMS-Systems GmbH

Waldstrasse 5-7

D(W)-6050 Offenbach/M. FRG/Germany

++49-69-881121 (BBS HST/DS)

++49-69-885042 (Voice)

EUROPE - Switzerland

Ernesto Guerber

PRO - DOS

Lenggenbachstr. 1

8951 Fahrweid

Switzerland

+748-27-75 (BBS HST/DS)

+748-33-32 (Voice)

+748-27-75 (FAX)

SOUTH AMERICA

Horacio Stolovitzky

SatLink Communications

Casilla de Correo 3618

(1000) Correo Central

Buenos Aires - ARGENTINA

For SOUTH AMERICA (Argentina, Chile, Brazil, Uruguay, Paraguay,

Bolivia, Peru, Ecuador, Colombia and

Venezuela)

+54-1-52-1057 (BBS)

+54-1-983-6740 (Voice)

INTERNET: hslink@satlink.org.ar

SINGAPORE

Anthony Chin

Artist On-Line Information System

Blk 458, Ang Mo Kio, Avenue 10, #14-1584

Singapore 2056

Republic of Singapore

+65-456-1100 (BBS HST/DS v.32bis)

AUSTRALIA

Peter Deegan

My Computer Company BBS

P.O. Box 99, Erskineville, NSW 2043

Australia

+61-2-565-1044 (BBS, v.32)

+61-2-550-4459 (FAX)

+61-2-565-1991 (Voice)

If you experience any problems with a particular distributor, please

contact me immediately so that we can get it resolved quickly.

HS/Link 1.1 User Documentation                                    - 41 -

Index

_________________________________________________________________

Aborting a transfer . . . .  6          -B . . . . . . . . . . 10

Auto-download . . . . . . . 31          -Bbaud . . . . . . . . 14

Auto-Upload . . . . . . . . 31          -C . . . . . . . . 10, 14

Basic settings  . . . . . .  9          -CB  . . . . . . . . . 13

Configuration files                     -CBcolor . . . . . . . 14

HSCONFIG . . . . . . .  7          -CG  . . . . . . . . . 13

specifying alternate               -CGcolor . . . . . . . 14

configuration                 -CM  . . . . . . . . . 13

files . . . . . .  7          -CMcolor . . . . . . . 14

Context sensitive help  . .  9          -CT  . . . . . . . . . 13

Crash recovery                          -CTcolor . . . . . . . 14

-K . . . . . . . . . . 11          -CW  . . . . . . . . . 13

-O . . . . . . . . . . 11          -CWcolor . . . . . . . 14

-R . . . . . . . . 11, 19          -E . . . . . . . . . . 10

DesqView                                -Ebaud . . . . . . . . 15

-I . . . . . . . . . . 12          -FC  . . . . . . . 12, 15

-I2  . . . . . . . . . 16          -FL  . . . . . . . . . 12

-NV  . . . . . . . . . 17          -HC  . . . . . . . 10, 15

Display bleeding  . . . . .  5          -HR  . . . . . . . 10, 15

Display colors  . . . . . . 13          -HS  . . . . . . 6, 9, 15

Distributors  . . . . . . . 40          -HX  . . . . . . . 10, 15

DoubleDOS                               -I . . . . . . . . . . 12

-I3  . . . . . . . . . 16          -Iidle . . . . . . . . 16

-NV  . . . . . . . . . 17          -K . . . . . . . . 11, 16

DSZLOG  . . . . . . . . . . 29          -LFlogfile . . . . . . 16

ERRORLEVEL  . . . . . . . . 34          -N5  . . . . . . . 12, 16

Errors                                  -NB  . . . . . . . 11, 16

-HS  . . . . . . . . .  6          -NC  . . . . . . . 11, 16

COM Overrun  . . . 15, 24          -NF  . . . . .  5, 13, 16

Error messages . . . . 32          -NM  . . . . . . . 12, 17

Exit codes  . . . . . . . . 34          -NO  . . . . . . . . . 12

File buffering  . . . . . . 11          -NOdirlist . . . . . . 17

File sharing  . . . . . . . 30          -NT  . . . . . . . 10, 17

Flow control  . . . . . . . 24          -NU  . . . . . . . . . 17

-HC  . . . . . . . . . 10          -NV  . . . . .  5, 13, 17

-HR  . . . . . . . . . 10          -O . . . . . . . . 11, 17

-HS  . . . . . . . . .  6          -P . . . . . . . . . .  9

-HX  . . . . . . . . . 10          -PBbase  . . . . . . . 18

HSCONFIG  . . . . . . . . .  7          -PI  . . . . . . . . . 10

HST . . . . . . . . . . . . 26          -PIirq . . . . . . . . 18

Installation                            -Pport . . . . . .  6, 17

In BBS programs  . . . 22          -R . . . . . . . . 11, 19

In COMM programs . . . 21          -S . . . . . . . . . . 11

Non-standard COM ports  10, 18          -Ssize . . . . . . . . 19

NS16550AN . . . . . . . . . 25          -U . . . . . . . . . .  9

Options . . . . . . . . . . 10          -Udir  . . . . . .  6, 19

-! . . . . . . . . 11, 14          -W . . . . . . . . . . 11

-@ . . . . . . . . . .  7          -Wwindow . . . . . . . 19

-@fname  . . . . . . . 14     PC-Pursuit

-A . . . . . . . . 11, 14          -W . . . . . . . . . . 19

HS/Link 1.1 User Documentation                                    - 42 -

Example  . . . . . . . 20

Registration  . . . . . 38, 40

Support . . . . . . . . . . 39

V.32  . . . . . . . . . . . 26

Zmodem  . . . . . . . . . . 27

HS/Link 1.1 User Documentation                                    - 43 -