Netwalker

Netwalker is a mailbox utility that allows a port to be used to collect

network connectivity information.  This is designed to be used on Netrom/

Thenet type nodes.

Installation of netwalker is thru Desqview.  It will require about a 38k

window.  The desqview parameters are the same for netwalker as they are

for the mailbox.  (See DV.DOC)

To run Netwalker, open the window in desqview, and enter the parmeters

for the node that you desire.

Example:  NW menu

N Opts Port File Timeout node_call [node_call]

Opts

I=Collects node identification info

N=Collect no extra data

P=Collect node parameters

U=Collect node version

Q - quit

cntl-f aborts

N P C n6mpw.nod 30 n6mpw-1 <cr>

What this means is 'Node, port C, collect node parameters, open file

n6mpw.nod to collect information, with 30 sec timeout-1 starting at node

n6mpw-1.  If you add the additional node_call it will log the information

from the perspective of the second node_call.

The screen output will be exactly what you will see in the file.  Netwalker

will connect to the local node and query it for its routing information.  It

will then disconnect and reconnect to connect to the next node in the list.

It will then collect its routing information. This will be repeated for as long

as there are nodes to connect to.  If a node that is in the routing list cannot

be connected to then the timer will disconnect the tnc from the local node

and go to the next node in the list.

The information collected in the file is information about the node and its

routing table.  At the bottom of the file there is a summary of Netwalkers

activity.

To stop Netwalker just enter cntl-f and it will disconnect from the local pad.

then enter q to quit and the window will be closed and the port returned to

normal bbs activity.