********************************************

HOW TO DEAL WITH MOTOR VEHICLE EMERGENCIES

********************************************

-----------------------------

BACKGROUND

-----------------------------

Imagine what would happen if you were driving your car and suddenly

the throttle stuck wide open.  Or what if you applied the brakes and

the car didn't slow down?  What would you do if your car's lights went

out when you were driving at night? Would you know what to do if the

steering locked so that you couldn't control the direction you were

going?  If a fire broke out under your hood, would you know what to do?

What if your engine began to overheat in a traffic jam?

At some time during their lifetime, most drivers encounter at least

one emergency situation involving a malfunction of their vehicle which,

if not handled promptly and correctly, can result in an accident,

injury, or death.

The purpose of this database is to describe some of the most common

emergency situations and how they can be handled, with accident

prevention and the safety of motorists and innocent pedestrians being

the prime concern.

The discussion that follows is intended primarily for those drivers

relatively inexperienced in coping with emergency malfunctions in their

vehicles. This database does not cover emergency situations requiring

major repairs and the use of a tow truck or wrecker to move a vehicle,

such as a blown engine, broken axle, or loss of a wheel.

--------------------------------

First Rule of Crash Avoidance

--------------------------------

The best way to avoid accidents caused by something going wrong with

your car is to keep it in safe running condition.

You should inspect your car regularly to make sure it is ready to be

driven safely.  Your car's Owner's Manual tells you what to check, how

often, and how to do it.  If you don't what to do this kind of thing

yourself, ask a service station attendent to do it for you. But, there

are certain things you should know and be able to do yourself.

For example, there are certain things you should have in your car for

emergencies:  a fire extinguisher (approved for vehicle use) that is

accessible and in good condition, a set of flares or other highway

warning devices, a flashlight that works, and some spare fuses. (Do you

know where your fuse panel is located?) You should also know how to use

a jack to change a tire.  And make sure your spare tire is fully

inflated (unless it is the type designed to be stored deflated and

inflated by use of a pressurized canister when needed) and in good

condition.

There are some things you should do virtually every day that you use

your car. Some things can be done when you get gasoline.  Other things

need be done only every few months or few thousand miles of travel.

--------------------------------

DAILY CHECKUPS

--------------------------------

The driver of a motor vehicle should perform a few safety checks

whenever the vehicle is driven.  This is particularly important with a

car that the driver doesn't own or drive regularly.  Many checks can

be done almost unconsciously by observing the lights and gauges on the

instrument panel, or by taking a quick look around the outside and

inside of the car.

A casual look at the tires before getting in the car may show one

that is dangerously underinflated or worn out, or with a bubble in the

sidewall. Inside the car, a casual glance at the gas gauge will tell

the driver whether he should fill the tank so as to avoid running out

of gas on the highway.  An oil pressure light slow to go out may

indicate a serious potential engine problem. Applying the brakes before

engaging a driving gear will give an indication of whether the brakes

are operating normally.  A check of the lights, including those on the

instrument panel, and a functional check of the windshield wipers and

washer are also good precautions.  Make sure your warning lights and

sensors are operating properly (oil pressure, brake failure, seat

belts, temperature, alternator/generator).  Make sure the window glass

is clean, both inside and outside.

Probably the most important safety check is whether you and your

passengers have their safety belts fastened properly. Despite the care

you take of your car, and the care you take in driving, you still may

be involved in a crash.  Your safety belts are the best insurance you

can have to minimize injuries or the chances of death in a collision.

Also remember, raising and adjusting the head restraints helps to

prevent whiplash if another vehicle runs into yours.

Finally, when the car begins to move, abnormal looseness in the

steering or a pulling to the right or left can be felt in the hands.

Problems such as these require immediate attention.

--------------------------------

WEEKLY/MONTHLY CHECKUPS

--------------------------------

About once a week, such as when you fill your gas tank, there are a

number of checks that should be done on a car. You can do these

yourself; particularly if you get gasoline at a self-serve pump, or

you can ask the service station attendant to help you.

Your car has many different fluids that should be checked regularly.

For example, you have oil in your engine and transmission that can

generally be checked by observing the fluid level on a dip stick

located under the hood. (Manual transmissions are more difficult to

check, but need be checked only when the car is lubricated and

serviced, or if the transmission is making strange noises or its

performanced feels odd.) Brake fluid, power steering fluid, radiator

coolant, and battery water should be checked at least once a month, or

as the Owner's Manual recommends.  Also be sure that the windshield

washer is filled.

Your tires are particularly important to your safety.  Check the air

pressure in your tires every other week, or after major changes in

temperature - especially warm to very cold. Buy your own tire gauge,

because many of the gauges at service facilities may be off by several

pounds per square inch. While checking your tires, examine the tread

depth remaining and look for bubbles, erratic treadwear, signs of

tread separation, or other abnormalities, and replace suspect tires.

Also look for stones or pieces of glass in the tire grooves and

carefully remove them.  If you notice a nail in the tire don't remove

it as the tire may go flat; instead, remove the tire, mount your

spare, and take the tire to a service facility for nail removal and

tire repair.  Be sure not to forget to check the spare tire.

Tire pressure should be checked when the tires are cold. Tires

should be inflated according to recommendations provided by the

vehicle manufacturer. Such recommendations are found on the door

pillar or in the glove compartment, and in your Owner's Manual.

Never let a tire salesman sell you tires not recommended in your

Owner's Manual.  If an alternate size tire may be used on your vehicle,

it will be indicated in your Owner's Manual.

Note:  For better fuel economy, you can increase cold inflation

pressures by UP TO four pounds over the recommended pressure.  But NEVER

exceed the recommended maximum pressure molded on the tire sidewall.

This little extra pressure will also let your tires run cooler.

Your lights are also important to your safety.  Turn on the lights

and walk around the car to make sure that all of them are working.

Have someone inside the car apply the brakes to be sure the brake

lights work, and try the turn signals in the right and left turn

positions to be sure they are working - front and rear.  The headlights

should be kep clean as well - dirty headlights give out much less light.

Similarly, the rearview mirrors (inside and side-mounted) should be

kept clean and properly adjusted, and periodically checked for looseness.

Check the car for unusual noises or other things that might signal

trouble. A squeal from under the hood, particularly when the engine is

raced, may mean a loose fan belt.  Drips under the car (in the area

where you usually park your car, for example) may mean that some vital

fluid is leaking out.  Do you smell or hear anything in the car that

might mean that the exhaust system is leaking? Does the handbrake

(parking brake) hold the car on a hill?  Are the steering or brakes

loose or spongy?  Does the engine run roughly or stall after it has been

run for a few minutes?

Any of these may be signs that the car needs immediate and proper

servicing.

--------------------------------

PERIODIC SERVICING

--------------------------------

You should take your car to a service establishment periodically (or

when signs of trouble develop), for regular maintenance and routine

safety checks.  Some people even like to do this work themselves. Your

car's Owner's Manual gives information on what needs to be done, how

often, and in many cases, how it should be done. Some of the more

complex services will require use of a shop manual applicable to your

vehicle.  If you are capable of taking on some of the more difficult

tasks yourself, and choose to do so, you can usually purchase a shop

manual from the manufacturer or at a bookstore.

Some of the things done in periodic maintenance involved checking:

condition of the belts and hoses under the hood; brake linings,

cyclinders, and hoses (for wear and leaks);  the exhaust system;

steering and suspension system components; wheel alignment (uneven

tire tread wear may be a sign of the need to realign the wheels); and

engine adjustments. Filters, oil, and some other components are usually

checked or replaced and the vehicle lubricated.

Despite the care you take of the car you own or drive, problems can

still develop while you are driving.  The rest of this database will

discuss the most common emergencies that can occur on the road and what

you can do if they happen to you.

--------------------------------

Stopping On The Highway

--------------------------------

Many of the emergency situations that will be described require

stopping on the highway.  Stopping on a highway for any reason is

dangerous.  If you must stop on a highway, observe the following

precautions:

* If the highway has paved shoulders, signal your intention to pull

off the highway, pull off at near traffic speed, then slow down.

If the shoulder is unpaved, signal a right turn and slow down to

a safe speed before pulling off the paved roadway.

* In dusk, darkness, or bad weather, leave your low-beam headlights

on, turn on your interior lights and your four-way flashers.

* If you have to stop in a risky location, (e.g., over the crest of a

hill or on a curve), get everyone out of the car and well away from

traffic.

* Place a flare or other warning device just behind the car and

another at least 300 feet farther back.  Check for these before

staring to drive.

* If you need help, raise the hood and tie a white cloth to the

antenna  or left door handle.

Note:  It should be pointed out that the hazard (emergency) lights

on most cars WILL NOT operate when the brakes are applied;  therefore,

once you are off the road (or if stranded on the road), you should

shut off the engine, put the car in "park," apply the parking brake,

AND TAKE YOUR FOOT OFF THE BRAKE PEDAL.

--------------------------------

Throttle Sticking

--------------------------------

Imagine that you're driving along a street or highway and for some

reason you must slow down.  You take your foot off the accelerator (which

controls the throttle), but the accelerator pedal does not return to its

normal position and the car continues to speed on.  Or, what if the

accelerator pedal suddenly goes down to the floor, and the car lurches

forward at a high rate of speed, even though you've taken your foot off

the accelerator.  What can you do?

Turn your IGNITION to OFF and APPLY the BRAKES.  But be sure you turn

the key to OFF, not LOCK.  On most cars you can't turn the steering

wheel when the key is in the lock position.

Note:  OFF means any key position that will turn off the engine BUT

NOT LOCK the steering.  Ignition systems on vehicles vary.  Make sure you

know which position will cut the engine without locking the steering.

Also NOTE, if you have power steering and power brakes, turning off

the ignition will require increased physical effort to steer and brake

the car as the car slows down. Don't pump the brakes as this will deplete

the vacuum in the booster, apply the brakes firmly and some power may be

retained.

As the car slows down, guide it off the roadway, if possible.

After you stop the car, look for the source of trouble.  The

accelerator pedal may be binding on the floor mat or the rug in your

car, and you can easily free the pedal by moving the rug or mat.

If this isn't the case, and if you know something about engines, look

in your engine compartment and check the accelerator linkage.  Some of

the parts may be stuck and binding, and a little oil (from the dipstick

used to check your oil) may solve the problem.

If you can't locate and remedy the problem (it may be caused by a

broken or missing accelerator return spring or by a broken motor mount),

DON'T DRIVE THE CAR.  Get help so the problem can be corrected.

If you think you've corrected the problem, make certain before driving

the car.  Apply the emergency (or parking) brake firmly, put the gear

selector in PARK or NEUTRAL, and start the car.  Exercise the accelerator

pedal a few times to make sure it returns to its normal position after

you remove your foot from the pedal.  Then put the car in gear and try

pressing the accelerator part-way down a few times before releasing the

parking brake and proceeding on your way.

--------------------------------

Brake Failure

--------------------------------

Newer cars have a split braking system designed to reduce the

possibility of total brake failure (such as the loss of brakes on all

four wheels). They have a warning light on the instrument panel that

lights up when your brakes are failing because of the loss of pressure

in the braking system.  If the BRAKE FAILURE light comes on, slow down,

pull off the road, and don't proceed until you have the problem corrected

or determine that you can drive safely to the nearest service facility.

Because of the split braking system, the chances are that you will have

some braking power left when the BRAKE FAILURE LIGHT comes on, but you

may have to apply more force to the brake pedal and will need a greater

distance to stop.  If half of the braking system remains, proceed

cautiously to the nearest service station or garage.

But, if you have a COMPLETE BRAKE FAILURE, what can you do to stop?

There are several things you can try, but you must act rapidly.

* First, get off the highway onto the shoulder or other clear area,

if possible.

* Try PUMPING your BRAKES rapidly to bring up your brake pressure.

* If pumping doesn't work, put the GEAR SELECTOR in a LOWER RANGE (D1

or extra low in cars with automatic transmissions or shift to a lower

gear in cars with manual transmissions) to obtain some braking power

from the engine, and APPLY the EMERGENCY OR PARKING BRAKE with

increased force.  On vehicles with no manual parking or emergency

brake release lever, use a modulated pressure on the pedal, as

necessary, to prevent total locking of brakes.

If your brakes fail on a hill or mountain grade and the above remedies

do not work, look for something to side swipe - a snowbank, a guardrail,

dirt mounds on the side of the road, or anything that will slow you down.

--------------------------------

Loss Of Steering

--------------------------------

Loss of steering can occur suddenly and without warning, Something in

the steering mechanism or its related components may break, fall off, or

jam, leaving the driver with no control of the car's direction.  (Partial

loss in power-assist steering can occur if a drive belt breaks; steering

control is not lost, but it requires additional effort.)

In such situations there is little you can do except to APPLY THE

BRAKES to come to a STOP as quickly as possible. While applying the

brakes, some WARNING to other motorists and pedestrians may be possible

by turning on your EMERGENCY FLASHERS, using your HEADLIGHTS, BLOWING

YOUR HORN, and using HAND SIGNALS.  (See previous note on EMERGENCY

FLASHERS.)

When your car finally comes to a halt, try to move it off the roadway.

Assistance from passing motorists or pedestrians may be sufficient to

turn the wheels by hand enough to permit pushing the vehicle off the

roadway.  Before such action is attempted, position your emergency warning

devices (flares or highway triangular warning signs) to alert other

motorists using the road.

Do not try to drive the car until you get the steering repaired or

until you get a wrecker to move the car to a repair facility.  Lift the

hood of the car and attach a white cloth to your radio antenna or door

handle to indicate to other motorists or police that you need assistance.

Steering problems are often preceded by such symptoms as extreme

tightness in the steering, that indicate something is binding or

obstructing the mechanism, or extreme looseness.  If your car develops

such symptoms, have the problem diagnosed and remedied at once.

--------------------------------

Loss of Power-Assisted Braking And Steering

--------------------------------

To those accustomed to power steering or power brakes, a malfunction

in the system providing the power may lead the driver to think his brakes

or steering have failed.  If your car is equipped with these power

features, and if suddenly find that steering is more difficult, or the

brakes will not respond when you touch the brake pedal, you can still

steer and brake. It takes considerably more effort, but it can be done.

Proceed with caution until you reach a safe parking spot and call a tow

truck.

--------------------------------

Fires

--------------------------------

Vehicle fires are generally caused by a fault in the electrical system

or by leakage in the fuel system (except for those caused by carelessness,

such as ash tray or upholstery fires caused by lighted cigarettes, cigars,

or pipe ashes.) Fires in the vicinity of the gas tank are potentially the

most hazardous.

If you see or smell smoke or see flames coming from your car,

immediately pull off the road to a safe spot, turn off the ignition, get

all passengers out of the car and remain at a great distance from it.

Warn motorists and passersby of the danger, and have someone call the

nearest fire department.

Every vehicle should have a fire extinguisher for emergencies, and it

should be a type recommended for use in a motor vehicle.

If you have a fire extinguisher, whether you should try to put out the

fire or not is a matter of judgement, depending on the severity of the

fire and the risk involved.  If the fire is severe or is a fuel-fed fire,

or the fire is in the vicinity of the gas tank, stand clear and get others

clear of the vehicle and wait for the fire department.

If you do attempt to put out an under-the-hood fire, be careful when

raising the hood. Use the rag to cover your hand when releasing the hood

latch so that you don't get burned.  Also turn your head aside as the

hood is released to prevent facial burns from flashing flames.

Finally, don't attempt to drive the car until the cause of the blaze

is determined and the problem corrected, including any damage caused by

the fire itself.

--------------------------------

Loss Of Lights

--------------------------------

A total loss of lights in conjunction with total electrical power loss

is usually caused by the burnout of a fuse, a fused wire, or by loosened

battery cables.

If only the headlamps go out, the problem may be in a circuit breaker

which has opened.  These are heat-actuated to open and close, so they may

come on and go off intermittently, giving you some light to help you to

safety.

A flashlight in the glove compartment is essential to fix most

electrical problems at night, so periodically check your flashlight to

make sure it is functioning properly.

If your lights go out, pull off the road and take care of the problem

before proceeding. Know where the fuse panel is in your car.  Your Owner's

Manual tells you where it is.  Also carry spare fuses of the type used in

your car's electrical system.  Find your fuse panel and identify the fuse

for your lights.  (The panel should be marked to identify the fuse for

the various systems.) Replace the blown fuse with a good one. If the new

fuse doesn't solve the problem, or if the new fuse blows, your problem

is more serious and requires repair by a competent automotive electrical

systems repairman.

If you don't have a spare fuse, DO NOT wrap a piece of tinfoil around

the old fuse. This defeats the prime purpose of the fuse and may result

in an electrical system fire.

Loss of just one light (e.g., a tail-light or one of the headlights)

is less serious than a total loss of lights.  This is probably the

result of a bulb burnout, and you'll still have enough visibility to

carefully drive to the nearest service facility to have the problem

corrected.

--------------------------------

Overheated Engines

--------------------------------

Engine overheating can be caused by a number of things - a leak in

the radiator or hoses permitting loss of coolant, a faulty water pump,

a bad thermostat, or a broken fan belt.  There may be nothing seriously

wrong.  Some engines have a tendency to get hot when being overworked

on hot days, e.g., moving slowly on a very hot day in a long line of

stop-and-go traffic (particularly air-conditioned vehicles.)

Become familiar with the normal position of the needle on your

termperature gauge when your car has reached normal operating

temperatures.  Knowing this, if you notice the needle moving

abnormally close to the "HOT" position it's time to do something.

For cars equipped only with a light that indicates when the engine is

hot, when this light comes on it's also time to do something.

If you notice steam or liquid being emitted from the front of your

car, this is usually one indication that it is overheating.

If your car has a tendency to heat up when moving slowly in dense

traffic in hot weather, there are several things you can do:

* If possible, pull off the road, turn off the engine, and let it cool

off before proceeding.

* If you can't pull off the road and if the engine has not reached

the critical overheat point (as noted by needle movement on the

gauge), turn off your air conditioner (if you have it on).  In

stop-and-go, slow-moving traffic, shift into NEUTRAL while stopped.

Both these actions reduce strain (load) on the engine and may bring

the temperature down. During long waits in traffic, a slight increase

in the engine's idling speed (press the accelerator lightly) can also

help reduce overheating through the action of the fan on the coolant

in the radiator.

* If none of the above work, turn on your heater (in addition to the

other actions mentioned) and roll the windows down.  This helps to

reduce the engine's temperature, even in cars not equipped with air

conditioners, by increasing the radiator capacity.

If the TEMPERATURE RISE is NOT NORMAL for your car, pull off the road

and turn off the engine.  Look for the source of trouble and let the

engine cool. DON'T REMOVE THE RADIATOR CAP. If the radiator cap is

removed before the engine has cooled, the STEAM or LIQUID is likely to

BE FORCIBILY RELEASED, AND CAN CAUSE SEVERE BURNS.  DO'S AND DON'TS FOR

OVERHEATED ENGINES:

* Remember, don't pour cold water into an overheated engine or you may

crack the engine block.  This mistake is often made.

* After an overheated engine has cooled, remove the radiator cap and

check the coolant level. If your car is equipped with a "surge tank,"

check the coolant level at that point.  The surge or radiator

"overflow" tanks contain markings that indicate when coolant is low,

normal, or hot;  they are designed to take the overflow liquid from

the radiator when the radiator gets hot, rather than let the coolant

be lost.  When the radiator cools, the overflow should return to

the radiator.

* Look for leaks or breaks in the radiator, surge tank, or in the

hoses, including heater as well as radiator hoses.

* Correct the problem if possible.  Friction tape or adhesive tape

can sometimes be used to TEMPORARILY stop or slow a leak in a hose

until you can get to a service facility to have it replaced.  If

the hose is split or ruptured near the end, sometimes you can remove

the hose end (using a screwdriver or pair of pliers, depending on the

type clamp used), cut off the damaged section (using a knife), and

replace the hose.  Note:  This often works for the smaller diameter

hoses (3/4 or 1 inch that are flexible and a little longer than they

have to be).  It may not be possible to do this with shorter hoses,

or hoses of larger diameter that are preshaped during manufacture.

* If a source of water is available, replace the lost fluid.

* If possible, after the engine temperature has dropped to normal

range after taking the remedial action indicated, drive to the nearest

service facility and have proper repairs done. DO NOT drive a car

with the engine overheated or with insufficient coolant.

If there is no source of water available to replace lost coolant, if

the car returns to an overheat condition, or a hose is ruptured too

badly, arrange for roadside repairs or a tow truck to correct your problem.

CAUTION:  If you are able to make temporary repairs so that you can

drive to the nearest service facility, don't move the car until the

engine has cooled down.  When you get to the service facility, WARN

THE ATTENDANT that the engine has been overheating so that he can take

precautions to make sure he is not burned or scalded.

Note:  Hoses should be inspected at least once a year for signs of

cracking, extreme softness, or other signs of deterioration.  Fan belts

should also be inspected for cracking and fraying.  When these items

are replaced, the old ones can be stored in the trunk and used in

emergencies.

---------------------------------

Loss Of Oil Pressure

---------------------------------

A sudden loss of oil pressure, if not promptly corrected, can result

in extensive damage to your car's engine as well as a highway breakdown.

Most cars have an OIL PRESSURE LIGHT on the instrument panel.  This

light comes on as soon as you turn on the ignition.  Shortly after the

engine starts, this light should go out.  If the light DOESN'T GO OUT

when the engine is running, or if it COMES ON WHILE you're DRIVING, you

have TROUBLE.  You may not have enough oil in your engine, or your oil

pump may be bad and not pumping oil through the engine.

If the OIL LIGHT COMES ON and stays lit, the FIRST thing to do after

pulling off the road is turn off the ignition and CHECK YOUR OIL.  If

the oil level is at or below the "ADD" mark on the dipstick, ADD OIL

BEFORE DRIVING THE CAR ANY FURTHER.  A spare can of oil carried in your

trunk is ideal for just such an emergency.  If this was the problem,

the oil light should go out when you restart the car.

Normally it is not advisable to operate the engine with the oil light

ON to protect the engine but, if there are no abnormal engine noises

and if your check shows you have enough oil and the LIGHT is STILL ON,

in an emergency you may drive cautiously a few miles to the NEAREST

service facility but NO FARTHER.

Get a good mechanic to check the car, because something else may be

wrong.

--------------------------------

Alternator/Generator Failure

--------------------------------

Most newer cars have an alternator.  Older cars have a generator.

Both serve the same purpose.  They make the electric current that keeps

your car's battery charged and operate the lights and anything else in

the car that needs electricity.

If the ALTERNATOR/GENERATOR LIGHT COMES ON and STAYS LIT, SOMETHING

MAY BE WRONG with the ELECTRICAL SYSTEM in your car. First check the

alternator/generator belt because if this belt is loose or broken, your

car may also be about to overheat.  Have a service facility correct

the problem as soon as you can.

If this happens out on the highway, you can often run the car using

the electrical current from your battery alone.  But you can do this

only for a short distance until the battery power is exhausted.  If

you must run your car for a short distance to get to a service facility,

turn off all electrical accessories (air conditioner, radio, or heater)

except the ignition.

If this happens at night, the extra drain on the battery will shorten

the distance you can drive, but you should be able to drive several miles

to a service facility.

--------------------------------

Windshield Wiper Failure

--------------------------------

Windshield wipers may fail when you need them most - when it's raining

or snowing.  To lower the odds of your wipers failing, periodically check

them or have a service facility do it for you.  Make sure you have good

blades and that they are properly adjusted to conform to your windshield.

Note:  It's a good idea to replace wiper blades once a year.

If you have the disappearing-type wipers, periodically check the

opening to the front of your windshield. Do this more frequently in the

fall and winter.  Remove leaves, twigs, snow or ice from the wiper

recesses and from around the wiper motor shaft and wiper arms.  Such

obstructions can place a strain on your wiper motor and result in wiper

failure.

If you have a failure on the highway (loose wiper, motor ceases to

turn wipers, blade flies off), get off the highway and see if you can

correct the problem.  Again, some obstruction may be hampering wiper

movement, or it may be possible to push the wiper arm on the spindle

more firmly or, in some cars, to tighten the screw or nut on the wiper

arm spindle.  Before you pull off the road, make certain it is safe to

do so.

Open the window and stick your head out to see, if necessary.

If you find that you can't fix the wipers yourself, wait until the

rain or snowstorm has let up, then proceed with caution to the nearest

service facility. If it's impractical to wait, you'll have to get help.

--------------------------------

Dropped Driveshaft

--------------------------------

One of the most dangerous problems to occur on the highway is a

dropped driveshaft.  This problem doesn't occur often, but when it

does, it happens suddenly.

In vehicles with front-mounted engines, the driveshaft is the long

tubular device underneath most real-wheel drive cars.  It is connected

on one end to the transmission and on the other to the differential in

the rear end.  Its purpose is to transfer power from the transmission

to the rear end to turn the rear wheels and propel the car.  The

driveshaft connections at the front and back are called universal joints.

If one of these fails, on some vehicles the driveshaft may drop to the

roadway.

Failure of the rear universal joint is not as serious, from a safety

standpoint, as failure of the one in front.  If the universal joint in

the rear fails, the rear end of the driveshaft may drop to the roadway.

Motive power will be lost, and the car cannot be driven until the

driveshaft is repaired.

If the front universal joint fails, a serious accident can result, and

the faster the car is moving the greater the danger.  In this case, the

front of the driveshaft may drop and can dig into the pavement.  The

forward motion of the car can then throw your car out of control.  Your

FIRST AND MOST IMPORTANT PRECAUTION is to slow down before pulling off

the road to prevent the drive train from digging into the rough berm.

To reduce the chance of this happening to your car, have your service

facility examine your car's universal joints periodically and check for

possible looseness in the joint assemblies or in the bearings. Universal

joints will be obviously noisy before they fall completely.  Generally,

this is accompanied by a severe vibration throughout the vehicle at all

speeds.

Further, if you ever hear a dragging noise under your car, slow down

and pull off the road, and look for the problem.

--------------------------------

Hood Pop-Up

--------------------------------

Failure of a hood latch (both primary and secondary) or improper

closure of hood and subsequent failure of the secondary latch, can

result in the hood popping open while you are driving.  When the hood

opens in this fashion, it will block your view of the road in front of

you.

This problem is not as prevalent as it was prior to 1969.  A Federal

safety standard, which became effective in January 1969, requires a

front-opening hood, which in any open position partially or completely

obstructs a driver's forward view through the windshield, to be provided

with a second latch position on the hood latch system or with a second

hood latch system. Despite this, hood latch failure can occur and you

should know what to do if your hood suddenly pops open while driving.

The first thing to remember is DON'T PANIC AND DON'T PANIC-STOP.  If

you apply your brakes suddenly and hard, you may be inviting a rear end

collision.  Instead, ease the car to the right or left (depending on the

lane of traffic you're in and the room you have on either side) and

depend on your limited view from left window for forward steering

reference (i.e., you may have to stick your head out the window to look).

Also, glance in your rear view mirror to see

how much room you have between you and the vehicle behind you.  Remove

your foot from the accelerator and apply your brakes slowly. Turn on

your emergency flashers and give a HAND SIGNAL to indicate that you are

going to STOP.  After you've signaled drivers to the rear, pull off the

road (to the left or right depending on the type highway and the lane

you're driving in) and try to remedy your problem.

If for some reason you cannot get the car off the highway (e.g.,

driving on a bridge, or guardrail to the right), do not leave your car.

After traffic has cleared behind you, proceed with caution to the nearest

point at which you can exit the highway.

CAUTION:  A frequent cause of this problem is the failure of a service

station attendant to close the hood properly after checking the oil,

radiator or battery.  You should get accustomed to the sound made by

your hood when it is closed firmly.  Thereafter, if you fail to hear

the customary "THUNK" when an attendant closes your hood, check the

hood yourself before proceeding.

--------------------------------

Submersion Of Your Car

--------------------------------

Emergencies of this nature are very rare and unpredictable; yet,

speedy and proper reactions by the driver and passengers are more

critical than in any other type of emergency situation. So, if your

car goes through a bridge railing or over an embankment into a deep

body of water, the following tips may help you survive.

* Cars with their windows and doors closed will float for a few

minutes.  Don't try to open a door to get out because the water

pressure will hold it shut.  Windows can be rolled down easily,

so open the window (windows in the case of passengers) and use the

opening as an escape route.

* If you have power windows, open them immediately before they short

out.  If they do short out and won't open, your only recourse is to

try to break them out with a very hard object, such as the heel of

your shoe.  The tempered glass in modern cars is hard to break.

* If you can't open or break a window and must open a door, remember -

cars with engines in the front will sink nose first. This action

will push some air to the rear of the car near the roof, helping to

equalize the pressure and making it easier to open a door.

Note:  None of these measures will help you if you are tossed about

and knocked out because you haven't worn your safety belt.  SO, WEAR

THAT SAFETY BELT.

--------------------------------

Loss Of Lug Nuts On The Wheels

--------------------------------

If you notice a WOBBLE IN A WHEEL or hear a CRACKING or RATTLING NOISE

coming from a wheel, especially at low speeds, the problem may be loose

lug nuts or a lug nut that has come off the wheel stud and is rattling

inside the hub cap or wheel cover.  This problem is often caused by

improper tightening of the nuts when a tire is replaced, or by faulty

lug bolt threads which will not retain the lug nuts tightly.

Take care of such a problem immediately before you lose a wheel.  Pull

off the road; display warning devices;  remove the hub caps or wheel

covers; and check the tightness of all the lug nuts.  Tighten all the

nuts that may be loose.  If all the lug nuts are tight, the sound could

be caused by a faulty or burned-out bearing. Caution should be taken in

driving to the nearest service facility for repairs.

If you've already lost more than one nut from a wheel (e.g., two of

the nuts have been lost), borrow one nut (no more) from another wheel

so that you can tighten the wheels adequately. Then, at your first

opportunity, replace all the missing lug nuts.

If two or more of the wheel studs are too badly stripped to permit

retightening of the nuts, leave the car beside the road and get help.

Have all faulty lug nuts or lugs replaced as necessary.

--------------------------------

Exhaust System Failures

--------------------------------

There's nothing you can do abut a blown muffler or broken tailpipe

when you're out on the highway except to get the problem taken care of

as soon as you can.

Sometimes a hanger holding your muffler or tailpipe in position will

break. The muffler or tailpipe may separate and drag along the pavement.

You may be able to HEAR the DRAGGING NOISE.  You'll also HEAR THE LOUD

ENGINE NOISE caused by a blown muffler or one that has separated and is

dragging.  When this happens, pull off the road and examine your tailpipe

and muffler. Often a temporary fix can be made by pushing the

muffler/tailpipe in place or raising it and holding it in place with a

piece of wire or a coat hanger.  CAUTION - WAIT for the EXHAUST SYSTEM

TO COOL DOWN before you touch it.  You can be severely burned if you

grasp a muffler or tailpipe while it is hot.  Until the exhaust system

is repaired, drive with windows open (as indicated in your Owner's Manual)

to ensure against carbon monoxide accumulation in the passenger

compartment.

--------------------------------

Flat Tires And Blow-Outs

--------------------------------

Keep good tires on your car and check them frequently.  This will do

much to reduce your chances of having a flat or blowout on the highway.

If you're not familiar with procedures for jacking your car and changing

wheels, practice these operations in the privacy of your garage or

driveway and under competent supervision.  BE ESPECIALLY CAREFUL of

the JACK and carefully FOLLOW INSTRUCTIONS ON ITS USE.  Such instructions

will usually be found in the trunk or spare tire well of your car.

If you have a flat tire or a blowout on the highway, get a firm grip

on the wheel and apply the brakes gently to slow down. Pull off the

road to a safe spot where you have enough room to park and get out of

the car without danger to yourself or without causing a traffic hazard

for other motorists.

CAUTION:  Don't SLAM ON THE BRAKES.  Sudden braking may throw your car

into a spin or out of control.

Note:  A blowout will be sudden.  You'll feel it in the wheel and you

may hear it.  You may also feel a part of the car dip.  Control of the

car may be difficult.  A flat tire will also be felt in the handling of

the car and steering may become unnatural, but this is more gradual than

a blowout.

If you can't pull off the road where you are, drive to a spot where

you can pull off and change the tire safely.  Do this even if you have

to drive on a flat or blown-out tire -- but drive slowly and PUT ON YOUR

EMERGENCY FLASHERS, DON'T try to CHANGE A TIRE ON UNEVEN OR HILLY GROUND.

It's better to risk ruining a tire or wheel by driving on a flat tire

than to risk having a jack slip when you're changing a tire on uneven

ground.

Note:  Before jacking up the car it's a good idea to loosen the lug

nuts slightly on the wheel to be changed so that the wheel can be removed

easily when the vehicle is raised on the jack.  If the lug nuts are not

loosened, the pressure necessary to remove them may be enough to cause

the vehicle to slip off the jack.

Some drivers may not be physically capable of changing a tire

themselves, or may not know how.  In such situations, follow the

procedures described earlier for requesting help, or request help over

your CB radio, if you have one.

--------------------------------

Driving Through Water Or In Heavy Rain Or Snow

--------------------------------

Driving under these conditions can result in three major problems -

engine drownout, brake failure, and possible loss of vehicle control.

Every year people are drowned trying to drive across flooded roads.

Before crossing a flooded road, know the depth of the water or don't

cross it. Another major problem resulting from heavy water on roadways

is hydroplaning, which results in loss of steering control.

If you drive through a deep puddle - especially if your car is moving

fast - water can be thrown up into your engine compartment and cause your

car to stall out due to moisture on your spark plug wires, coil, or

distributor.  A car moving in the other direction can cause you the same

problem if it's moving too fast and throws a lot of water (or road slush

in case of a snowstorm) onto your car.

If you encounter deep puddles or high water along the highway, DRIVE

THROUGH SLOWLY.

If your car stalls out, try to coast to the side of the road and wait

for the engine to dry out.  If you know the parts of your electrical

system, the drying out process can be quickened by taking a dry rag and

wiping the plugs, wires, and coil, and by drying the inside of the

distributor cap.

If your car stalls in the middle of a puddle and you can't move it,

and if you're in the vicinity of a stream that is overflowing onto the

highway, be alert to the possibility of a flash flood.  For your own

safety you may have to leave the car where it is and seek shelter until

the water recedes.

After moving through water your brakes may have lost their stopping

power due to submersion.  Apply your brakes lightly while driving to

dry out the linings and other components.

--------------------------------

Mini-Emergencies

--------------------------------

UNRULY CHILDREN:

Accidents have been caused due to lack of driver control when the

driver turns around to try to discipline unruly children.

Children should always be restrained in a child restraint (or child

seat) that meets Federal safety standards, or by a safety belt if they

are older or no child seat is available. This is primarily for their

safety if there should be a crash, but it also keeps children from

interfering with the driver.

If you must discipline young children in your car, DON'T LET GO OF THE

WHEEL AND TURN AROUND. Pull off the road, switch off the engine, and apply

any discipline necessary, and, make the children FASTEN their SAFETY BELTS

despite any objections.

INSECTS:

Accidents have been caused by drivers losing control of their cars due

to efforts to get rid of flies, bees, or other insects that have entered

the vehicle. Don't get frantic and try to kill an insect while driving.

Pulling off the road in a safe spot, turn off the engine, and then get

rid of the insect. Accept the possibility of a bee sting until you can

pull off the road safely; that's a lot better than risking a more serious

accident or injury.

If an insect or dirt flies into one of your eyes, close that ONE EYE

and pull off the road before you attempt to clear the foreign matter

from your eye.  DON'T TRY TO CLEAN YOUR EYE WHILE DRIVING.

If you get foreign matter in BOTH EYES, force yourself to keep your

eyes open, despite the discomfort.  Pull off the road safely and then

clear your eyes.

Another caution - animals loose in the front seat can also interfere

with attention, visibility, steering, and braking.

CLOTHING AND UPHOLSTERY FIRES:

Clothing and upholstery fires are usually smoker's problems. Ashes or

lighted particles may drop on your clothing or on the upholstery from

your cigarette, pipe, or cigar.  Don't try to put out the fire while

moving. Don't stop in traffic and get out of your car to brush off

burning ashes.  Put up with any pain or damage involved.  Retain control

of the car and pull off or to the side of the road before attempting to

put the fire out.

FIRE IN THE ASHTRAY:

Ashtrays are FOR ASHES, NOT CANDY AND GUM WRAPPERS.  If you have paper

in your ashtray, a cigarette or cigar that is not completely extinguished,

pipe ashes, or a match not completely out can cause a fire in your ashtray.

If you have such a fire, CLOSE THE ASHTRAY to diminish the flames or even

smother the fire. Then pull safely off the road and make sure the fire is

out.

To avoid such fires, carry a trash bag or trash receptacle in your

vehicle, and use it.

--------------------------------

First Aid Supplies For Your Car

--------------------------------

Many of the little problems that can disable a car along the roadway

can be taken care of by the driver - if he has some simple tool or some

little thing like a piece of tape or wire. You don't have to be a good

mechanic to fix many of the little things that can go wrong. Your car's

fan belt may become loose because the bolt holding the generator/alternator

in position became loose - this would be reflected in the warning light

on your dash indicating that the alternator/generator is not charging.

A wire may become disconnected, a tailpipe hanger may break, etc.  The

following is a list of some of the things you can carry in your trunk

or glove compartment that you may find useful from time to time:

NECESSARY ITEMS:

Glove Compartment:

* The name, address, and phone number of someone to call in an

emergency.

* Spare fuses for the electrical system.

* A good flashlight.

* An ice scraper for winter driving.

* A pocket knife.

Trunk:

* Spare tire (with air in it).

* Fire extinguisher.

* First aid kit.

* Jack and lug wrench for changing tires.

* Flares or reflective day/night devices.

Note:  Flares can be very dangerous in the hands of children, so store

them where children can't easily get them.

VERY USEFUL ITEMS:

* An EMPTY CAN that is fire-department approved to carry gasoline (if

you run out of gas).  Note:  NEVER CARRY GASOLINE in this can in

your trunk - this is VERY DANGEROUS.

* A box, tucked into the corner of your trunk and containing:

Pliers - useful for tightening clamps, small nuts that may work loose,

and twisting wires.

Screwdrivers - several sizes. Make sure one is a "Phillips" type.

Adjustable Wrench (or small set of open-end wrenches if you prefer)

to tighten nuts and bolts that may have worked loose.

Tape - to repair broken or frayed wires and to temporarily stop small

leaks in a hose until you can get to a service facility.

Wire - very useful to temporarily hold a muffler or tailpipe in place

if one of the hangers breaks or falls off.

Rags - can be used to dry up your distributor or wet wires if your

motor is drowned out in a heavy rain or from driving through high

water.

Battery Jumper Cables - handy to help get your car started if your

battery is weak, especially in the winter.  Consult your Owner's

Manual on how to use jumper cables.  If you don't have an Owner's

Manual, write for the free NHTSA Fact Sheet, "Battery Hazards".

Piece of Sandpaper - useful for cleaning dirty battery terminals when

the car won't start.

Can of Engine Oil - nice to have when the OIL light comes on and

you're far from a service station.  OTHER USEFUL ITEMS:

Tire Pump - for use when one of your tires develops a slow leak and

you'd rather drive to a service station to have it changed than do

it yourself. Can also be used to dry out a distributor that is wet.

Plastic Sheet - for use in changing a tire in the rain or if you have

to get under the car to check something.

FOR WINTER DRIVING:

Tire Chains - if you don't have snow tires.

A Small Shovel to help you get out of ruts and snowbanks.

A Small Bag of Sand to give you something to throw under the wheels

for better traction if you get stuck in ice or snow. To keep your

trunk clean, the sand should be stored in a plastic bag, which is

itself stored in a larger bag such as a gunnysack.

*****************************

HOW TO SAVE GAS AND MONEY

*****************************

------------------------------------

How To Save Gasoline... and Money

-------------------------------------

"You mean I could save up to 12 percent on my gasoline bills just by

keeping my car tuned?  "...or 5 percent just by keeping my tires

inflated to the tire manufacturer's recommended levels?  "...or between

3 and 10 percent by switching to radial tires?  "...and 20 percent on

the highway merely by driving carefully and never exceeding 55 miles per

hour?  "I'm interested. Tell me How."

"Gladly.  But first, let me tell you What and Why..."

--------------------------------

What It Costs To Drive

--------------------------------

If you're a typical driver, your car:

- travels over 11,000 miles per year

- consumes nearly 800 gallons of gasoline annualy (just over 2 gallons

per day)

- has an average fuel economy rating of 14 to 15 miles per gallon, and

- costs over $600 per year for gasoline alone (over 5 cents per mile).

Those figures assume that you're driving a private automobile in an

urban area.  Yet, even if you have a long commute, use your car for

business, operate a light truck, or for some other reason exceed those

averages, the relative impact of gasoline costs on the pocketbook

remains the same.

--------------------------------

Why It Pays To Save Gasoline

--------------------------------

By driving more efficiently you could easily: cut gasoline consumption

by 40 to 80 gallons per vehicle, probably without eliminating any

necessary travel and without inconvenience or sacrifice, and save $30 to

$60 or more per year.  The money saved is like getting a discount on the

gasoline you buy, amounting to 4 to 8 cents per gallon, perhaps a dollar

or more per tankful.  In fact, by planning your trips, caring for your

car properly, and taking ALL of the steps recommended in this database,

you can lower your gasoline bills considerably.

Gasoline is an essential form of energy, one we can no longer afford

to waste. It is refined from crude oil, a finite natural resource.

Approximately half of our oil supply is imported, and its price and

availability are largely determined by the oil exporting countries. By

saving gasoline, we can extend domestic oil supplies until alternate

sources of energy can be developed.  Moreover, the less you drive, the

more you cut costs and petty aggravation, air pollution, traffic

congestion, and accidents. And the more you help blunt the impact of

OPEC-dictated price increases. Passenger automobiles account for about

13 percent of all end-use energy consumption in the United States, and

about 31 percent of all the petroleum used.  If every driver of the more

than 100 million private automobiles, plus some 20 million light trucks,

reduced gasoline consumption just 5 percent, the the total savings would

be more than 5.5 billion gallons of gasoline per year. That's enough, in

savings a lone, to drive the average car between New York City and San

Fancisco about 25 million times - or for all of New York City's 8

million people to drive coast to coast 3 times.

--------------------------------

How To Drive More Efficiently

--------------------------------

1. Drive at a moderate speed. The most efficient range usually is 35 to

45 miles per hour.  On the highway, where you may need to maintain

a higher speed, stay at 55 mph.  Most automobiles get about 18

percent better mileage on the highway at 50 than at 65 mph, and 20

percent better mileage at 55 than at 70 mph. You may note that some

drivers abuse the speed limit on super-highways, but state police

are cracking down on speeders.

2. Drive at a smooth and steady pace.  Press lightly but steadily on

the accelerator so that you flow smoothly through traffic.  Avoid

tailgating, unnecessary accelerating, and braking.

3. Drive "defensively".  Anticipate traffic flow 10 to 12 seconds

ahead.  Maintain 2 or 3 seconds of buffer space between your car

and the others around you-calculated by noting when the car ahead

passes a fixed point, and counting the seconds until you reach that

point. This gives you the necessary time and space to drive safely

and use your energy efficiently.

4. Don't idle the motor for longer than 30 seconds.  Idling gets zero

miles to the gallon. Driving slowly for the first few minutes is a

much more efficient way to warm the motor than excessive idling.

Remember this rule when waiting in line, for instance, at a drive-

in window.

5. Keep windows closed when driving at highway speeds.  Open windows

increase wind resistance by creating drag.  If the outdoor

temperatures permit, use flow-through ventilation.  If heating or

cooling is needed, use a moderate setting.

-----------------------------

How To Plan Your Trips

-----------------------------

1. Share the ride.  About one third of all private automobile mileage

is for commuting.  When two people ride together, say to work, they

use about half the gasoline required if each drove alone.  Many

employers offer incentives such as free or reduced-rate parking for

carpools. Consider organizing or joining a vanpool;  the trend

towards vanpools is increasing.

2. Combine trips.  Plan your trips-to market, to the library, to

school, or to visit friends. You'll often find that one

neighborhood jaunt could serve two or three purposes, and you could

help your neighbors by offering to run errands for them. Gasoline

costs about 5 cents per mile, and much more for short trips.  Make

your shopping or optional trips when traffic is lightest, and

return home before the rush hour starts. Patronize your nearest

group of stores.

3. Eliminate unnecessary trips.  Find one trip a day which might be

better handled-or even eliminated-by telephoning, writing, or

combining with another trip. For instance, banking often can be

done more quickly and more efficiently by mail or telephone than in

person.

4. Take public transit if available.  On buses and trains, one can

read, daydream, sleep, or catch up on work.  When public transit

isn't available, groups of commuters sometimes charter buses, or

form vanpools, frequently with support from employers.

-----------------------------

How To Care For Your Car

------------------------------

1. Check tire pressure regularly.  Keep tires inflated to the

manufacturers highest recommended level.  Most such recommendations

are for cold pressure, so buy a good quality gauge and take readings

before starting out.  Become familiar with the difference between

your tires' cold and hot pressures, and adjust accordingly.  You can

improve mileage 1 percent for every two pounds of tire pressure

needed to bring them up to standard.

2. Replace all four tires with radials.  They will generally give 3

to 5 percent improvement in gas mileage in the city, and up to 10

percent on the highway.  When you buy a new car, specify radials.

They last longer, give better mileage, and often provide better

steering qualities.  But don't mix radials with conventional tires-

it can be dangerous.

3. Have wheels properly aligned Improper wheel alignment can increase

fuel use and cause unnecessary tire wear.  A regular alignment check

is a good idea.  Also check the alignment after the wheels have had

a jolt-from striking potholes, bumps, or curbs.

4. Are your brakes adjusted?  Brakes that drag or grab unevenly rob

you of gasoline, and can be dangerous. Be sure that the brakes both

grip and release properly.

5. Use gasoline of the proper octane rating The owners manual gives

the gasoline octane rating and gasoline type recommended by the auto

manufacturer.  What you want is octane just high enough to prevent

knocking.

6. Check and change oil and oil filter at recommended intervals.

Every time you add gasoline, check the oil.  Dirty oil or a low oil

level can cause friction and wear that rob you of gasoline mileage

and can seriously damage your engine.

7. Use a good quality SE multi-grade (multiviscosity) oil.  Look for

the API SAE rating on the container.  Multi-grade oils like 10W-30

and 10W-40 help reduce internal engine friction and give better

gasoline mileage than single-grade oils.  Do not use an oil of

higher viscosity than recommended in your owner's manual since

heavier oils tend to increase friction and decrease miles-per-gallon

efficiency.  A lower viscosity oil such as 5W-30 is recommended for

winter months in a cold climate.  Consider using the super oils;

they increase fuel economy and give better protection against wear

of motor parts. Always use oils that are marked SE.

8. Do the little things prudence requires. Don't overfill the gas

tank; eliminate any chance of spillage.  Don't carry unnecessary

weight.  For every additional 100 pounds, mileage goes down by as

much as four-tenths of a mile per gallon.  For safety's sake, never

carry a can of gasoline in the trunk.

9. Have you had an engine tune-up recently?  Do you need one?  If

your engine is hard to start, hesitates, sputters, idles roughly, or

the like, you are losing mileage and should check its condition

immediately. You may need to clean and adjust spark plugs and

ignition points, replacing if necessary;  check timing;  and replace

air and fuel filter elements.  How much you will save on gasoline

consumption will depend on the condition of your engine and how much

you drive in the course of a year, but savings could range up to 12

percent on your gasoline bills. Here are some other key points:

Remove foreign matter from the exhaust gas recirculatory valve and

hoses, and check the controls following the manufacturer's

specifications. Check for leaks in the fuel system.  Check, clean

or, if necessary, replace the positive crankcase ventilating valve

(PCV).  Check all electical ignition wiring and connections for

voltage drop and resistance; clean, tighten, and replace if

necessary. Inspect the choke for proper operation.  Set idle speed

to manufacturer's specification.

--------------------------------

How To Choose Your Next Car

--------------------------------

1. The greatest single step you can take to save gasoline is to

choose your next car carefully. Here's how:  If you are in the

market for a new car, consult the "Gas Mileage Guide for New Car

Buyers," available free from any dealer or by mail;  write Fuel

Economy, Pueblo, CO 81009. Study the tables carefully. They give the

most complete and accurate information available on the relative

fuel economy performance of all current model cars, station wagons,

and light trucks.  The estimates are in terms of miles per gallon

measured on the Environmental Protection Agency's standardized fuel

economy test.  The actual mileage you get will depend to a large

degree on how and where you drive - and on following the tips on

saving gasoline and money found on these pages. The Guide can be

very helpful.  Compare specifications and test results.

You can save hundreds of dollars a year at no sacrifice - simply by

choosing an economical car from the class in which you are

interested. If you expect to buy a used car, first consult the auto

magazines or buyers guides published by the consumer testing

organizations.  These should be available at the public library.

In buying a car, choose the size needed to satisfy your normal driving

patterns.  Consider such things as the car's weight, and select the

lightest car in the class. Also, choose the one with the lowest

practical number of cylinders. Choose manual transmission if most of

your driving is typically on the highway.  And avoid power option

extras.  If you need extra passenger or trunk room occasionally,

consider buying a compact and renting a station wagon once or twice

a year.

2. As said before, do the little things prudence requires. Select a

light-color car for a warm climate - to reflect the sun's rays;  you

might not even need air conditioning, or if you have it, you might

not use it much.  Tinted glass or a light colored interior also

helps cut down on heat inside the car.  If you don't need an

installed luggage rack on top, specify a model without one;  it adds

to wind resistance. Above all, don't let an over-zealous sales

manager pressure you into buying the "latest model" left on the

floor or lot.  You'll be driving - and paying for gasoline - for a

"lifetime", the lifetime of your ownership of the car.

-----------------------------

Why Not Also Consider...

-----------------------------

1. Local laws that can help you save energy.  Those that help reduce

automobile gasoline might include:  increasing availability of

public transportation;  express lanes for ridesharing vehicles and

buses during peak commuting hours;  one-way streets in heavy traffic

areas, synchronized traffic lights;  right turns on red;  banning

stopping, standing or parking on arterial streets during rush hours;

and providing overpasses at busy intersections.

2. Vacationing close to home.  Discover nearby attractions.  A nearby

hotel, resort, or campground might provide as satisfying a change

from routine as a similar one up to thousands of miles away.

3. Rediscovering the pleasures of walking, hiking, jogging, and

bicycling.  Does anyone have to be reminded that these modes of

transportation, relaxation, and exercise consume no gasoline?

4. Forms of motorized transport which compare favorably to automotive

efficiency.  A two-wheeled vehicle might meet your transportation

needs.

---------------------------------------

How To Set Your Own Gas-Saving Goal

---------------------------------------

To help achieve conservation objectives in the face of tight oil

supplies and rising prices, President Carter, in his April 5, 1979,

energy address to the nation, called on all of us to drive 15 fewer

miles per week. "At least once a week, take the bus, go by carpool -or,

if you work close to home, walk," he suggested.

"Conservation is our cheapest and cleanest energy source," the

President said. "It helps us to control inflation, and every barrel of

oil we save is a barrel we don't have to import." If every driver cut

normal driving by 15 miles a week, savings would add up about 450,000

barrels of oil per day - oil that would not have to be imported.  How

can you reduce your driving by 15 miles per week?  By putting into

practice the tips in the section here on "How to Plan Your Trips." How

can you reduce your gasoline bills 40 percent or more?  By putting into

practice ALL the tips listed here.  Would you like more tips? For a free

pamphlet "Tips for Energy Savers" and/or a half-hour, 16 mm color movie

"Running on Empty" suitable for driver education classes or other

groups, write ENERGY, BOX 62, OAK RIDGE, TN 37830.  Prepared by the:

U.S.  Department of Energy Office of Public Affairs Washington, D.C.

20585 DOE/OPA-0044 (5-79)

***************************************

COMMON SENSE IN BUYING A SAFE USED CAR

***************************************

-----------------------------

BACKGROUND

-----------------------------

Millions of used cars are sold each year. Most people who buy used

cars do so for reasons of economy. They either can't afford, or

unwilling to pay, the price of a new car. Many Motorists never buy a new

car -- They buy a succession of used cars on the premise that after a

new car has been used for a year or two the original owner will have

taken care of all the little, nagging problems, as well as absorbing the

heavy depreciation usual on new cars.

Buying a used car is an adventure that can result in presonal

satisfaction and considerable savings.  It can also turn into a

misadventure ending in dissatisfaction, frustration, monetary loss, and

a real threat to the safety of the car purchaser and to other motorists

and pedestrians.

Many of the problems experienced by the buyer of a used car are of his

own making. with proper care in the selection of a used car, most

Problems can be avoided. There are literally millions of used cars from

which to choose.  Many of these cars may contain uncorrected safety

defects or may be mechanically and structurally unsound and should be

removed from the highway until corrected. Yet It's not difficult to find

a good, safe, and dependable used car -- But it takes patience and

"Know-How."

This database provides some "common sense" guidance for selecting a

good used car.

---------------------------------------------

CONSUMER CONSIDERATIONS IN BUYING A USED CAR

---------------------------------------------

When a consumer goes shopping for a used car, generally his prime

considerations are price, gas mileage, style, appearance, power, and the

equipment he wants on the car -- and not necessarily in that order.  He

seldom considers the safety aspects of the car he is about to purchase.

More often than not he believes, without question, the statements

regarding the car's condition and performance made by a salesman or a

private individual whose main concern is to sell a car. Rarely does he

inspect the car closely to detect flaws. He may not even test drive it

except for a short, non-enlightening spin around te block.  Seldom will

he check the details of any warranty that may be offered, and he may not

even determine whether the dealer is equipped to repair the car when he

needs service. little wonder, then, that some consumers are disappointed

shortly after they buy a used car. Little wonder, too, that many buyers

question the wisdom of their purchase after it's too late. As a result,

many used car buyers find themselves saddled with cars that may be

unsafe and require costly repairs for the correction of defects that

should have been detected and corrected before they made a down payment

or signed any papers.

---------------------------------

PRIME RULE IN BUYING A USED CAR

---------------------------------

Next to buying a house, a car, even a late model used car, is usually

the most expensive single item an individual will buy. A home buyer will

inspect his prospective purchase thoroughly to determine whether the

house meets his needs, whether it contains flaws that should be

corrected before purchase.  The lending institution providing his

mortgage will also perform an inspection, and may call in heating,

plumbing, electrical, or structural experts if flaws are suspected. In

many parts of the country, a prospective home buyer wouldn't think of

bying a house until it is inspected for and declared free of termite

infestation.

A used car can have suspension, steering, running gear, and cooling

system problems, and hidden deterioration of its structural members and

body. However, the used car will seldom get the same, thorough

inspection as the used house. Many car buyers treat the transaction too

casually and purchase many avoidable problems along with their used car.

The first rule in buying a good used car, having determined your

requirements on vehicle size, economy, performance, and equipment, is to

inspect it as thoroughly as you would a house. when possible, having the

vehicle run through a diagnostic center or inspected by a competent

mechanic to determine it's overall general condition.

-----------------------------

SERVICE REQUIREMENTS

-----------------------------

In shopping for a good used car, be cautious and suspicious. Remember,

the previous owner had some reason for getting rid of it.  The reason

may have been that he wanted a new car, or it may have been that the car

was practically worn out and required major, costly repairs to make it

both safe and dependable.

Also remember that you seldom get more than you pay for, as far as a

used car purchase is concerned, and can often get less than you paid for

if you're not careful.  So, if you think you're getting a good used car

at a "dirt-cheap" price, you may be buying somebody's expensive troubles

instead.

Remember also that all cars require service from time to time, and,

the older the car, the better the chances that it will soon require

major repairs. Thus, before you decide to buy a specific used car, think

about where you will have trouble getting adequate service.  In this

connection:

* For some makes of cars, adequate service can be provided only by an

authorized factory dealership specializing in that make.  It's time-

consuming and sometimes expensive, in terms of time lost from work

and travel expenses, when the nearest service facility having the

required parts is a great distance away.

* As used cars become older, some repair parts become scarcer. This is

espcecially true of makes and models provided in limited quantities

at times, the only source for some parts may be a junkyard.

-----------------------------

WHAT TO BUY

-----------------------------

Generally, your best buy in a used car is one that is only two or

three years old.  Look for the year and model you like, and when you

find one, check the odometer to see how far it has been driven.  If cars

only a few years old have been given reasonable care by their previous

owners, they still have a lot of driving life left in them. Because of

their mechanical condition, they should also be safer than older

vehicles.  Such cars, though, are still fairly expensive, and may cost

more than you can afford, so, if you find several cars within your price

range, consider the one with the least mileage on the odometer and best

general overall condition.

-----------------------------

ODOMETER LAW

-----------------------------

A federal law passed in 1972 makes it illegal for anyone to do

anything that would cause a vehicle's odometer to show the wrong

mileage.  No person, not even the vehicle owner, is permitted to turn

back or disconnect the odometer (except to perform necessary repairs).

The federal law also requires that anyone selling a vehicle, or

transferring ownership in some way, must provide the buyer with a signed

statement indicating the mileage registered on the odometer the time of

transfer.  If the seller knows the registered mileage to be incorrect

because the odometer has been broken or previously tampered with, he

must include a statement to that effect on the mileage disclosure form.

When purchasing a vehicle, be certain you receive a mileage disclosure

statement before the transfer of title.

Anyone who illegally tampers with an odometer or who fails to provide

the required mileage disclosure statement may be sued in a private civil

action by the person wronged by the violation. If the suit is

successful, the person will recover $1,500 or three times the amount of

actual damages, whichever is greater.

A vehicle may look good and the odometer may read only 15,000 or

20,000 miles when, in fact, it has been driven 80,000 or 90,000 miles.

The best way to protect yourself from being victimized by a fraudulent

representation of a vehicle's mileage is to closely examine the

condition of the vehicle, contact a prior owner to inquire about the

mileage when he sold the vehicle, ask to see the mileage form given to

your seller by the prior owner, and make sure you receive a mileage

disclosure statement. You will probably be able to make a good

assessment of the reliability of the odometer's mileage based on the

information you receive.

-----------------------------

USED CAR WARRANTIES

-----------------------------

With a used car you may not get a warranty, especially if you buy it

from a private party or from an organization that has no facilities to

service the car it sells (e.g., a repossessed car purchased from a

finance company).  Even when provided, warranties differ from one dealer

to another.  You may get a complete warranty for parts and labor for

periods up to 30 days, 90-day warranty on parts only with the buyer

paying half the cost for parts and labor.

Make sure you understand the warranty before you purchase the car, and

make it work for you. If you suspect something is wrong with your used

car and it's still under warranty, don't wait.  Take it back right away.

Do this over and over until you're sure the dealer has corrected your

problems, and don't accept an excuse like "the shop is full and we have

a lot of appointments this week, bring your car back next week." Next

week may be too late -- Your warranty may have expired and the dealer or

his service manager may not honor any promise made orally. If possible,

obtain a written description of any work done during the warranty period

so that you will have a record if the same problem recurs after the

warranty expires.

-----------------------------

WHERE TO BUY

-----------------------------

For your driving safety as well as satisfaction with your purchase,

where you buy your used car is as important as what you buy.  Sources of

used cars include used car dealers, new car dealers, private owners, car

rental agencies or companies that buy new cars every year and sell their

older cars, and banks or loan companies that have repossessed cars.

A used car dealer who sells only used cars usually has many cars from

which to pick, but some of these dealers don't have a service facility

(garage) to repair the cars if something goes wrong.

So, if you need maintenance, you may have to take it to some other

service facility for required work.  Before buying a car from such a

source, make sure there is someplace where you can get it repaired,

particularly if you get a used car guarantee.  Some used car dealers who

don't have their own service facilities make arrangements with a garage

to repair the cars they have sold and guaranteed.

A new car dealer who also sells used cars taken in on trade will have

a service facility to do necessary repairs.  Normally, such dealers keep

the best of the used cars they take in on trade, and dispose of the

others through a wholesale used car outlet.

You can often buy a good used car at a lower price by watching the

classified ads in your newspaper and buying from a private owner. But,

you won't get a guarantee.  Be careful if you buy a car from a private

owner that you don't know. Inspect it carefully; better still, take it

to your local mechanic or a diagnostic station to have it inspected

before you finalize the purchase.  Also make sure you get a clear title-

-Sometimes buyers get stuck with a car that has been stolen, or one that

is about to be repossessed by a bank or finance company because the

owner didn't pay the loan payments.

You might also be able to buy a repossessed car from a bank or finance

company through the newspaper ads.  Such institutions often sell

repossessed cars for the outstanding balance due on them. Again, inspect

the car carefully before purchase -- If the previous owner was lax in

making payments, he may also have neglected to perform required

maintenance service.

You might also be able to pick up a good used car from a car rental

agency. Many of these agencies buy new cars every year and sell their

older cars after using them for only a year. Some big companies that

have a fleet of company cars do the same thing.  Remember, though, that

cars bought from rental agencies or fleets will generally have

accumulated a lot of mileage in a short time --However, in most

operations of this size such vehicles receive better scheduled

maintenance.

-----------------------------

AVOIDING USED CAR PROBLEMS

-----------------------------

There are a few basic rules that apply especially to the used car

buyer, because he is buying a vehicle someone else has used and perhaps

abused. Further, a used car buyer may often have to buy a car "AS IS,"

which means the seller guarantees nothing. Thus, if You want to avoid a

great many problems when buying a used car, Follow these rules:

1.  Don't buy at night, in the rain, or under any conditions that could

keep you from seeing everything and examining everything about the

car, inside and out.

* A muddy car lot or a cold rain, or a day when you don't have time to

be thorough are conditions that invite you not to crawl under the

car to inspect it thoroughly. Yet, if the frame is rusted out, if

the steering or suspension parts are dangerously worn, or if there

is brake fluid leaking down over curb-bruised tires, you might buy

these problems and worse by picking the wrong time and place to buy.

* Rain can make dents, patches, and repaints shine like mew. You

should check for rusted-out-doors, flooring, and tailgates, so don't

pick a rainy day to buy a used car. Likewise, don't pick one at

night.

* If you're too cold, too wet, or too busy to try every accessory,

examine the whole car, and take it for a thorough test drive, you're

just asking to be "taken."

2. Don't buy the car at all ...

* If you're refused a test drive with you at the wheel.

* If you're refused a request to let your own mechanic check the car

at his place of business.

* If any and all promises for brake and clutch adjustments, tune-ups,

parts replacement, etc. are not written down and signed by the man

in charge, before you buy.

* If you're refused a final test drive for final outside inspection

after the seller has completed all promised, pre-purchase repairs or

adjustments.

3. Remember -- Some vehicles on a used car lot may have unrepaired

safety defects -- defects for which the cars were recalled by the

manufacturer, but which the prior owners neglected to return for

repair. By calling the National Highway Traffic Safety Administration,

Office of Consumer Services (202) 426-0670 or 426-0671, giving the

make, model, year, and vehicle identification number (vin) of the car

in question, you can obtain a report of all defect-recalls for that

make/model.

-----------------------------

EXAMINE THE CAR

-----------------------------

Examine the outside of the car thoroughly. Look for rust, dents, and

evidence that portions of the car have rusted out and been repaired

using body putty.  Be especially careful in examining areas around the

bottom of doors and the rear fenders. Look under the car. Check for

breaks in the frame or signs that the frame has been welded. Look for

excessive rusting of the frame.  All these conditions can signal

weaknesses in the vehicle's structure.  Also, check the condition of the

muffler, tailpipe, and exhaust pipe.

Look for signs of oil or transmission fluid leakage on the ground or

pavement under the car.  Look for signs of fluid leaking from shock

absorbers. check the condition of the tires, including inside surfaces.

Look for signs of brake fluid leaking on the inside of the tires.

Walk around the car and check the condition of window glass. Check the

condition of the lenses in all vehicle lights -- front, back, and side.

Test all the lights to make sure they are functioning (headlights,

taillights, flashers, backup lights, brake lights, turn signals, running

lights in fenders).

Push down on the corners of the car, front and back, to check the

shock absorbers. If the car bounces up and down several times, the

shocks are worn.

Stand back some distance from the car and see if it is level. If one

corner is lower than another, one of the springs may be weak or it may

have a broken leaf. While standing at a distance, look for evidence that

the car has been in an accident (ripples in the fender, dents, or paint

that doesn't match).

Open the trunk and check the spare tire.  If it is worn unevenly or is

cupped, something may be wrong with the front end and a front tire was

put in the trunk as a spare to hide the problem. Make ure the car has a

jack and that it is in good condition.

Lift the hood of the car and check the condition of the belts and

hoses. Check the battery to see if it is cracked.  Pull out the oil

dipstick to see how dirty the oil is.  A check of the mileage on the

odometer and a comparison with mileage indicated on lubrication stickers

(affixed to doorposts or sometimes at some location under the hood will

give you an indication as to when the car had its last oil change and

lubrication, and whether the previous owner neglected such necessary

services.

(Note:  If the previous owner changed his oil and lubricated the car

himself, you won't find such evidence.)

Get inside the car. Examine the upholstery, safety belts, and

carpeting. If the car has seat covers, look under them.  Badly worn

carpeting or upholstery may be a sign of heavy vehicle usage. turn on

the ignition and check the lights, including the warning lights on the

dashboard. Take a friend along, and have him check the various lights

from the outside as you turn them on from the inside. Also check the

brake pedal for free play.

Take the car for a test drive over various types of roads. Test the

brakes for signs pulling.  Be alert to any vibrations in the steering

that may be signs of front-end trouble. If the car has a manual

transmission, be alert to excessive play, grabbing, or rattling of the

clutch.  Be alert to any vibrations or unusual noises from the engine,

transmission, rear end, or wheels that may signal trouble. push the

gearshift through its various positions and see how the car reacts. Let

the engine idle and be alert to noises or vibrations that may indicate a

badly tuned engine or one with bad valves. Be alert for any unusual

odors inside the car -- these should tell you that all is not well with

the car. while test driving the car, pay attention to how the

transmission shifts -- if the automatic transmission doesn't shift

smoothly or if there is hesitation when it shifts, the transmission may

need repair work. When it is safe to do so, step on the gas and look

into the rearview mirror for smoke from the exhaust -- if you see a lot

of white or bluish smoke, the engine may be bad and need an expensive

overhaul. Check to see how much power the car has on a hill -- if it has

little power, it may need an expensive valve job or an overhaul. Turn on

the heater, air conditioner, radio, and windshield wipers to make sure

all these systems work. And, if possible, before you buy the car have it

checked mechanically by your mechanic or a diagnostic center, and run it

through a car wash to see if it leaks.

-----------------------------

BE CAREFUL OF TRICKS

-----------------------------

Some used car dealers and even some private owners use various methods

to make a car look better than it actually is. Some of these are :

* Painting tires to make them look new. Always examine the tread for

treadwear and sidewalls for cracks, signs of tread separation, and

bubbles.

* Steam cleaning the engine and painting the valve covers and air

cleaner to make them look new.

* Installing seat covers to mask torn upholstery.

* Installing new rugs or mats to cover rusted-out parts of the floor.

* Spraying black paint on the hoses to make them look new.

* Removing an erratically worn tire from the front and placing it in

the trunk to mask a front-end problem.  Always check the tire in the

trunk.

* Putting a cheap repaint job on the car to hide body rust.

* Making makeshift repairs to the muffler and tailpipe. Examine these

for signs the holes or rusted-out portions have been repaired with

muffler patches or repair tape.

-----------------------------

FINAL REMINDERS

-----------------------------

The Point has already been made that you will rarely have anyone to

blame but yourself for the problems you fail to note when you buy your

used car.

* Be Patient. Don't be in a hurry to buy the first used car that looks

good.

* Be curious and observant. Inspect the car thoroughly and test drive

it to discover any mechanical faults, operational characteristics,

or unsatisfactory features that may make you regret buying the car.

* Be suspcious.  If you can get previous owner's name, call him to

verify the condition of the vehicle and its mileage when he got rid

of it.

* Be Choosy and uncompromising in your demand for signed promises and

claims if you expect to hold the seller responsible for later,

unsolvable problems.

IN A WORD, CAUTION BEFORE YOU buy is the best advice that can be

given. Remember, buying a good used car requires common sense.

********************************

CAR CARE AND SERVICE

********************************

This database is designed to help you, the average owner, keep your

car on the road and out of the repair shop.  It explains the functions

of the key parts and systems of your car, the regular service required,

and the more common symptoms of malfunction or wear when a prompt minor

service could prevent an eventual expensive repair.  It also offers

practical advice on obtaining quality service and repairs and on

appealing for help if you feel you have been cheated.

With regular, knowledgeable care and service, most cars provide years

of reliable driving.

--------------------------------

Accepting A New Car

--------------------------------

The first step in budget-wise care and service is buying a car in

perfect condition.  To be sure your new car has no built-in defects or

service headaches, and that you know how to operate it, make the

following checks before you leave the dealer's lot. OPERATIONAL CHECKOUT

Read the Owner's Manual carefully.  Sit in the driver's seat and,

before you start the car, have the salesman explain the starting

procedure, the dashboard gages and warning lights, and the operation of

all controls and accessories.

Read the warranty carefully and note the services that are free and

those for which you must pay.  No warranty, however, will cover normal

wear or abusive use.

INSPECTION

Examine the exterior for dents, irregularities, or paint scratches.

Check the interior for wrinkled or misfit floor mats; look at the

seats, door panels, and headlining for scuffs, tears, or crooked seams.

Open and shut the doors to see if they fit the frames and latch

properly.

Roll the windows up and down. Do they work smoothly and close all the

way?

Look in the trunk for the spare wheel and tire, the tire jack, and the

lug wrench.  Ask the salesman or service manager to show you how to

operate the jack.

Look in the engine compartment.  Is it clean and new looking?  Ask the

salesman to show you how to check the oil and coolant levels.

PERFORMANCE TEST

Start the engine and listen for any roughness or noise after it has

warmed up.

For an automatic transmission, set the emergency brake and then shift

to "drive."  The engine should continue to run smoothly and not stall.

Road test the car.  It should accelerate without jerks or hesitations,

the brakes should work evenly and effectively, and the steering should

be responsive and smooth.

Try the horn and the windshield wipers and washer. Have the salesman

try all the lights and the turn signals while you stand outside the car

and check their operation.

If the car fails to satisfy you on any of these points, insist that

the fault be corrected before you accept delivery.

In addition, be sure you understand the warranty provisions.  What

free repairs and services does it offer? When must you bring in the car

for checks and adjustments?

--------------------------------

The Break-In Period

--------------------------------

You can avoid a number of service and repair problems for your new car

by careful driving for the first 2,000 miles while the moving parts are

"seating in."  Follow the directions in your Owner's Manual and these

supplementary suggestions.

During the first 1,000 miles, drive at moderate speeds. Accelerate

gently, especially in the lower gears, and vary the speed frequently. Do

not pull a trailer.

During the next 1,000 miles, accelerate moderately, change speed

occasionally during highway driving, and don't pull a trailer.

Every few days, inspect the driveway or garage floor where you park.

If you find a spot, observe its color and location under the car so you

can pass the information along to the service technician.  Leak repair

should be free during the warranty period.

Continue occasional checks for leaks as long as you own the car.

----------------------------------------------

Regular Care and Preventive Service Signals

----------------------------------------------

Skipping routine care or skimping on adjustments is directly to blame

for many car repair costs and can permanently cripple your automobile.

So can missing or ignoring early clues to faulty operation.

To help you avoid such mistakes, the working parts and systems of

modern cars are outlined below together with the usual maintenance and

the more common signals for preventive service.  For the exact

procedures for your car, consult your Owner's Manual.

CARBURETOR

The carburetor, which delivers a mixture of fuel and air to the engine

cylinders, requires periodic checking.  Idle speed and automatic choke

adjustments should be made, if necessary, at each tune-up and are

particularly important in the spring and fall because of seasonal

changes in gasoline formulas.

Service Signals:  Hard steering, hesitation when you press the

accelerator, stalling at stop lights, idling too fast or too slow, a

sluggish feel at road speeds, a tendency to "run on" after you turn off

the ignition, or poor gasoline mileage.

CARBURETOR AIR CLEANER

If your carburetor has a dry air cleaner, the filter element should be

replaced periodically. Between replacements, check occasionally to see

the filter has no holes or other damage and tap it gently to dislodge

dirt. Never wash it or try to clean it with air under pressure.

If the carburetor has a wet (oil bath) filter, it should be cleaned

and the oil changed as the automobile manufacturer recommends.

Service Signals:  Hard starting, poor mileage, or loss of power.

EMISSION CONTROL DEVICES

Modifications of ignition and carburetion cut down air pollution from

exhaust gases while other devices decrease fumes from the gas tank,

carburetor, and crank-case.  The manufacturer's recommendations for

maintenance and adjustment should be followed precisely; otherwise,

operating problems may develop.

Service Signals:  Hard starting, stalling, hesitation, "running on"

after the ignition is turned off, or poor gasoline mileage.

ENGINE VALVES

The intake valves admit the fuel/air mixture from the carburetor into

the engine cylinders;  the exhaust valves permit the escape of waste

gases after the gasoline burns. Mechanical lifters to open and close the

valves should be adjusted at every tune-up or as the manufacturer

recommends. Hydraulic valve lifters do not normally require adjustment.

Service Signals:  Loss of power, a ragged or uneven feel to engine

performance, a light clattering noise, or hard starting - especially

when the engine is cold.

ELECTRICAL CHARGING SYSTEM

When you start your car, the source of electrical current is the

battery. After the car is running, the source is the alternator or

generator, and a voltage regulator protects the battery from overcharge.

Check the battery water level often and add water, preferably distilled,

as needed.  Battery cable connections should be kept clean and tight.

Service Signals:  Sluggish starting, dim lights when the engine isn't

running, or the battery frequently needs water.

IGNITION SYSTEM

Electricity from the battery, alternator, or generator flows to the

coil where it is stepped up to higher voltage.  From the coil, the

current travels to the distributor, which channels it to the spark plugs

in the engine cylinders.  At each plug, the current ignites the fuel/air

mixture.  Periodically, the ignition system should be checked and the

spark plugs and distributor points changed as needed.

Some recently built cars are equipped with an electronic ignition

system that creates a hotter spark, reduces or eliminates point wear,

lenghtens spark plug life, provides more precise ignition control, and,

in general, requires less frequent maintenance.

Service Signals:  Hard starting, uneven acceleration, loss of power at

high speed, rough idle, or a partial or complete "miss" in one or more

cylinders causing rough performance and a severe loss of power.

TIMING SYSTEM

For efficient engine operation, the spark plug must fire in each

cylinder at precisely the right moment. This firing is regulated by the

distributor, which can be adjusted to advance or retard the timing of

the spark.

Service Signals:  Hard starting, pinging, or a rough feel during

driving if the timing is too "fast;"  hard starting, sluggish

performance, excessive gasoline consumption, or overheating if the

timing is too "slow."

GASOLINE

Gasoline with too low an octane rating for your engine will burn

unevenly, or explode, inside the cylinders instead of smoothly producing

power.  The usual symptom is a light clattering or pinging during

acceleration or when the engine is pulling hard.  Continued use of the

wrong gasoline could result in serious damage.

Gasoline with an unnecessarily high rating, however, costs you extra

money and wastes energy. If your car accelerates without ping on the gas

you use now, try a tankful of the next lower octane.  After the car has

warmed up, accelerate hard from a full stop.  If the engine pings, go

back to the higher octane.  If it doesn't, repeat the test with gas with

a still lower rating.

ENGINE OIL, OIL FILTER AND LUBRICATION

Have the oil level checked at least every other time you fill up with

gasoline.  If the level is at or below the "add" mark on the dipstick,

put in oil of the type specified for your particular model of car.  Do

not use additives unless they are called for by the automobile

manufacturer.  Have the oil changed at recommended intervals.

Generally, the filter is changed every other time the oil is changed;

but if you live in a dusty climate or do a lot of stop-and-start

driving, your mechanic may suggest a filter change with every oil

change.

For lubrication, follow the manufacturer's recommendations.

Service Signals:  If the oil warning light shows red while the engine

is running at or above idle speed, turn the engine off immediately. Oil

is not reaching all the engine parts, and continued operation will

result in major engine damage.

TRANSMISSION FLUID

Check the fluid level at intervals in either a manual or an automatic

transmission, and replace the fluid at recommended mileages with the

type specified for your car model.  Learn the correct checking

procedures. For example, if you have an automatic transmission, the

engine must be running and, usually, the shift should be in the "park"

position.

Service Signals:  Whine during driving, uneven growling noises, or

jerky acceleration.

CLUTCH

In a car with a manual transmission, depressing the pedal disengages

the clutch, slows the transmission gear rotation, and permits you to

shift gears. Clutch linkage should be checked and adjusted at regular

intervals.

Service Signals:  A tendency for the gears to clash when you shift,

clutch slippage under startup or hard acceleration, or clutch "chatter"

at startup.

BRAKES

All late-model, U.S.-built automobiles using conventional drum and

shoe brakes have assemblies that adjust automatically for wear unless

mechanical troubles develop. Disc brakes, also, need no adjustment. All

brakes, however, should be inspected as recommended by the manufacturer,

and the brake linings or disc pads should be replaced as needed.

Occasionally, brake fluid must be replenished.

Service Signals:  A spongy feel when you press the brake pedal, longer

pedal movement, scraping noises, or the car pulls to ne side when

slowing or stopping.  Uneven or grabby brakes are especially dangerous

on slippery surfaces.

TIRE PRESSURE

Correct tire pressure is necessary for safe driving, good tire

mileage, and responsive steering.  Check at least monthly that your

tires (and the spare) are at the recommended pressure.  Buy a good tire

gage and learn to use it;  air towers at service stations are frequently

incorrect. Occasionally inspect the tire treads.  Wear on the edges

indicates too little pressure; too much causes excessive wear in the

center.  When the car is on a lift for an oil change, have the tires

inspected for cuts, bruises, or tread separations.

This precaution could prevent a blowout.

Service Signals:  Pulling to right or left, uneven braking, or

excessive squealing on corners.

TIRE BALANCE

Tires should be kept in balance for comfortable and safe driving and

for long tread life. For the best results, have the tires spin balanced

on the car whenever vibration becomes objectionable.  Follow the

recommended tire rotation for your automobile make and model and the

type of tire.

Service Signals:  Vibration, usually felt through the steering wheel

for the front tires or through the seat for the rear tires.

FRONT SUSPENSION

Front wheel suspensions should be properly aligned at all times for

safe handling and economical tire wear, but pot holes and other rough

road conditions may gradually throw wheels out of alignment.

Service Signals:  Abnormal or uneven tire wear, or the car pulls hard

to the right or left during driving even when tire pressure and balance

are correct.

LIGHTS

Headlamps must be set to provide maximum road illumination and minimum

glare for oncoming motorists.  Check periodically to be sure all lights

are in working order.

AIR ENGINE-COOLING SYSTEM

Air-cooled cars have a powerful fan, driven by a belt from the engine,

that forces a large volume of air over the hot engine.  The fan belt

should be regulary checked for correct tension and freedom from wear. If

the fan belt should break while you are driving, STOP THE ENGINE

IMMEDIATELY OR IT WILL OVERHEAT.

AIR CONDITIONER

Just before the operating season, have the drive belt checked for

correct tension and examine the liquid in the sight glass for bubbles

that could mean the system needs more refrigerant.  During the winter,

most conditioners will benefit from a 5-minute workout every 2 or 3

weeks as long as the temperature is above 40 degrees Fahrenheit. Check

with your mechanic to see if this is good advice for your particular

car.

Service Signals:  A drop in cooling power, unusual noises from the

fan, or rapid cycling of the compressor.  You can hear the compressor

click off and on or detect the added load on the engine.

LIQUID ENGINE-COOLING SYSTEM

All the car engines presently manufactured in the United States are

cooled by a liquid-circulating system assisted by a fan.  The system

includes a liquid coolant (water plus antifreeze), hoses, a water pump,

a thermostat, and a radiator.

Ask the service attendant to check the coolant level in the system

every other time you fill up with gasoline.  Whenever you have the car

in for routine maintenance, have the mechanic check the hoses for

soundness and the belt that drives the fan for correct tension.  Have

the belt replaced if it is frayed. Each spring and fall have the

concentration of the antifreeze-water mixture tested; if too little of

the antifreeze is present, you are inviting either a freezeup or a

boilover.

Service Signals:  The temperature gage often reads high, or you must

frequently add liquid to the system.

--------------------------------

Overheating Countermeasures

--------------------------------

If you are in stopped traffic and the temperature gage reads in the

danger zone, take these steps, one after the other.  You may not need to

take them all:

* Shift to neutral and press the accelerator one quarter down.  Be

sure the air conditioner is turned off, and stay at least a car

length behind the car ahead.

* Open the windows and turn on the heater.  This will make you

uncomfortable but may prevent a boilover.

* As soon as possible, pull over, open the hood, and turn the engine

off and let it cool. DO NOT TRY TO REMOVE THE RADIATOR CAP WHILE IT

IS HOT.

* After the engine cools, drive slowly to a service station to

identify the cause of overheating unless you have lost radiator

coolant by boilover. For lost coolant, add enough water to drive to

a service station for repairs and new coolant.

--------------------------------

Checking Gasoline Mileage

--------------------------------

One of the best signs that a car has a clean bill of health is good

gasoline mileage. Between tune-ups or other scheduled maintenance, you

can check your car's performance by the method outlined below.

* Have gas put in the tank until it is exactly full at the filler

neck.  At the same time, record the mileage reading on the dashboard

odometer.

* Drive until the tank is almost empty;  a calculation based on fewer

miles or gallons is less accurate.  Then, have the tank refilled to

the same level on the filler neck, and write down the gas pump

reading in gallons and the new odometer reading in miles.

* Subtract the original odometer reading from the new one to obtain

the miles traveled between gasoline fills.

* Divide the miles traveled by the gallons required to refill the

tank.  The answer is the miles the car traveled on each gallon of

gas, or the gasoline mileage.

If the result is poorer mileage than that listed in the Owner's Manual

for your model or markedly inferior to your last check, take the car to

a mechanic. Until its efficiency is restored, it will waste money and

gasoline and probably add pollutants to the air.

--------------------------------

How To Get The Best Service

--------------------------------

When your car is in the shop, you want an honest judgment of the

service needed plus skilled, careful workmanship.  This combination can

extend the life of your car, save you money and time, and enable you to

drive safely and confidently.  The following tips should help you obtain

such service.

OBSERVE THE WARRANTY

During the warranty period, have the car serviced exactly as

recommended. Routine maintenance may be performed at an independent

garage or service station without violating the warranty agreement; but

be sure to keep a receipt showing the mileage reading, the date, and the

nature of the service.

For repairs under warranty, you must take the car to an authorized

dealer.

USE BUSINESS SAFEGUARDS

When repair of service is necessary after the warranty period, be sure

to take the following steps:

* Inquire if the shop offers a warranty on service and replacement

parts and how long it is effective.

* Tell the service manager, the service advisor, or the mechanic ALL

the car's symptoms and problems, and be sure he understands what you

are saying. Master mechanics complain that they often have to "read

the customer's mind."

* Then, find out exactly what work is to be done and, if possible,

obtain a written estimate.  At least write out a list for yourself

and keep it. INSIST

THAT THE SHOP OBTAIN YOUR APPROVAL BEFORE DOING ANY ADDITIONAL WORK.

If you suspect the shop's estimate includes unnecessary repairs,

either obtain another estimate from different shop or take the car to a

diagnostic center. One that charges a fee for diagnosis but does no

repairs is the most apt to give an unbiased report.

When you pick up the car after repairs, check each item on the list or

estimate.  Do not accept the car unless everything is complete and

satisfactory.

If parts have been replaced, ask for the old ones to take with you.

If they are too large or messy to carry, ask to see them.

Road test the car before your drive home, and return it to the garage

immediately if it is not performing well.  Don't wait until tomorrow;

your claim for corrective service will be weakened.

CULTIVATE CAR FRIENDS

Try to use the same mechanic every time you have your car serviced.

This may not be possible if you deal with a large garage;  but you can

establish a relationship with the service manager or one particular

service advisor so that he remembers your car and its special needs.

When work is good, stop by and tell the dealer or the service manager,

and ask him to pass on your compliment to the mechanic. It will be time

well invested.

Unsatisfactory work over a period of time, however, is an indication

to change shops. Inquire in the neighborhood, and ask your friends to

recommend a reputable firm.

If you would prefer an independent garage or service center, you can

write to the Automotive Service Councils, Inc., 4001 Warren Boulevard,

Hillside, Illinois 60162, for information on their National Warranty

Program.  Participating shops give a customer a written guarantee

covering labor and parts.  A faulty repair may be corrected at any other

participating shop if the customer is not able to return to the original

one.

Or, for $1.75 you can purchase a copy of WHERE TO FIND A CERTIFIED

MECHANIC, a nationwide list of shops whose employees have passed special

tests on automotive repairs.  To order, send a check to Employers

Directory, National Institute for Automotive Service Excellence, Suite

515, 1825 K Street, N.W., Washington, D.C. 20006.

KNOW HOW TO COMPLAIN

At a dealership, talk to the owner about poor service or inadequate

repairs. If he refuses to give you satisfaction, consult your Owner's

Manual or the warranty for the address of the nearest district office of

the company, and write or telephone the customer service representative.

The next step, if you must take it, is to talk with or write to the vice

president for sales at the company headquarters. Keep copies of all

repair orders, receipts, and correspondence.

If these steps fail, you may wish to contact one of the pilot project

AUTOCAP's - or Automotive Consumer Action Panels - in which local or

State automotive trade associations and consumer representatives seek to

resolve customer difficulties.  To learn if an AUTOCAP has been

established in your locality and if it can handle your particular

problem, write to the National Automobile Dealers Association, 1640

Westpark Drive, McLean, Virginia 22101. AUTOCAP's were initiated at the

request of the U.S. Office of Consumer Affairs.

At a private garage or service center, also talk with the owner about

a complaint.  If the problem is not resolved, check to see if a

municipal, county, or State consumer agency can help you.  You can also

appeal to the local office of the Better Business Bureaus or write to

the U.S.  Office of Consumer Affairs, Washington, D.C. 20201.

As a last resort, pay the bill and then see a lawyer.