VMODE - USING THE EXTENDED-COLUMN MODES

The VGA/16 provides you with the capability to utilize extended column

modes with text applications. This means that your VGA/16 board, when

interfaced with appropriate color displays, can produce 132x44,

132x28, 132x25, 100x40, and 80x60 modes in addition to the standard

80x25 and 40x25 modes. With the appropriate software, the 132-column

display capability allows emulation of terminals that require 132

columns of text.

With the VGA/16 board, you can switch back and forth between the 80-

column display modes and the various extended-column display modes. To

use VMODE, follow these steps:

1. First, be sure that the VMODE.COM utility is present on the disk

you are using.

2. To switch to a different mode, type the mode you wish to use and

then press ENTER:

Type

VMODE 25      - to switch to 132x25 mode

VMODE 28      - to switch to 132x28 mode

VMODE 44      - to switch to 132x44 mode

VMODE 40      - to switch to 40x25 mode

VMODE 60      - to switch to 80x60 mode*

VMODE 80      - to switch to 80x25 mode

VMODE 100     - to switch to 100x40 mode*

* Note: Not all modes are supported by all monitors. Attempting to use

modes which your monitor does not support will produce unsatisfactory

results.

Three additional modes are provided to enable you to change the scan

line resolution. This is to accommodate some software that

specifically looks for a particular number of scan lines. The 400 scan

line mode provides the most pleasing text resolution.

VMODE 200 - to provide 200 scan lines in 40- or 80x25 text modes.

VMODE 350 - to provide 350 scan lines in 40- or 80x25 text modes.

VMODE 400 - to provide 400 scan lines in 40- or 80x25 text modes.

VMODE SCAN RATES

Additional modes are available to provide additional scan rates that

may improve synchronization with a variety of monitors. Normally, the

default scan rates are effective, but some monitors may require

different scan rates for the most satisfactory display results. By

setting these modes, frequencies are adjusted that affect displayed

graphic modes. The following modes are available via the VMODE

command.

example: VMODE x

VMODE     Modes       Vertical        Horizontal

x Value   affected    Refresh Rate    Frequency   Resolution

35K       29,2A,30    56Hz            35KHz       800x600

38K       29,2A,30    60Hz            38KHz       800x600

45M       37i,38i     86.5Hz          35.5KHz     1024x768 interlaced

65M       37n,38n     60.5Hz          49KHz       1024x76 non-

interlaced

72HZ Modes

The following graphic (not text) modes can be displayed after setting

the display the vertical refresh rate to 72HZ. The 72HZ mode will

provide a clearer display with less flicker on some monitors. Set the

VMODE parameter to VMODE 72HZ. After this is done you can use VMODE to

set the display to any of the graphic modes listed below.

VMODE           Vertical        Horizontal

x Value         Refresh Rate    Frequency       Resolution

11 [72HZ]       72.70Hz         38.70KHz        640x480*

12 [72HZ]       72.70Hz         38.70KHz        640x480*

25 [72HZ]       72.70Hz         38.70KHz        640x480*

**2E [72HZ]     72.70Hz         38.70KHz        640x480*

* monitor must be capable of 72.70Hz vertical and 38.70KHz horizontal

frequency.

72MHz Modes

The following graphic (not text) modes can be displayed after setting

the display the video clock speeds to 72MHz. The 72MHz mode will

provide a clearer display with less flicker on some monitors. Set the

VMODE parameter to VMODE 72m. After this is done you can use VMODE to

set the display to any of the graphic modes listed below.

VMODE           Vertical        Horizontal      Video

x Value         Refresh Rate    Frequency       Clock   Resolution

**37 [72m]      69.80           56.30           72.000  1024x768

***38 [72m]     69.80           56.30           72.000  1024x768

* monitor must be capable of 70Hz vertical and 56.3KHz horizontal

frequency.

** requires minimum of 512kb display memory; *** requires minimum of

1MB display memory.

Typing VMODE ? will display the full array of mode possibilities.

Users that are familiar with the characteristics of their monitors

will find these modes more useful.

NOTE:  Choose modes/resolutions that are compatible with your video

board configuration and monitor capabilities.

USING THE COMPATIBILITY MODES

Some programs are written to be run using specific modes or are

written according to particular video standards. For example, there

are programs that require a monochrome adapter or those that are

written exclusively for a Color Graphics Adapter. When these kinds of

programs are used, it becomes necessary to make your VGA/16 board

appear to be what the program requires. This is easily accomplished.

In order to select a mode that will change the "appearance" of the

video adapter to the software, you need only select the appropriate

VMODE parameter. When another mode is needed or desired, you can

simply select the mode using another VMODE parameter. In order to

return to the default mode (VGA), type VMODE VGA or power down (the

VGA mode will be in effect upon power-up). The following is a list of

modes and their uses. A complete listing of available modes can be

seen by typing VMODE ?.

VMODE CGA -     To set the adapter for compatibility with the Color

Graphics Adapter.

VMODE MDA -     To set the adapter for compatibility with the

Monochrome Display Adapter.

VMODE HERCULES -To set the adapter for compatibility with the Hercules

card.

VMODE EGA -     To set the adapter for compatibility with the Enhanced

Graphics Adapter.

VMODE VGA -     To set the adapter for compatibility with the Video

Graphics Array.

A typical use might be for using game software that requires CGA

resolution. Using VMODE CGA will put the VGA/16 board in CGA mode. If

the software needs to be booted in order to work, you simply place the

VGA in CGA-compatible mode with VMODE CGA, and warm-boot with your

game diskette in the A: drive.

You may switch modes in this manner as often as you wish. VMODE ? will

display a list of the available modes. Entering VMODE with no

parameter will display a menu screen from which to choose a text mode.