********************************************************************************

1. Add the following to config.c in the list of main level commands

********************************************************************************

#ifdef	PACKETWIN

"tsync_stat",	do_tsync_stat,	0, 0, NULLCHAR,

"tsydump",		do_tsync_dump,	0, 2, "tsydump <channel>",

#endif

********************************************************************************

2. Next add the following to config.c under the attach command list

********************************************************************************

#ifdef  PACKETWIN

/* Gracilis PackeTwin */

"tsync", tsync_attach, 0, 11,

"attach tsync <chan> <vec> <iobase> <hdx|fdx> <dma|ints> <rxdmachan> <name> <rxsize> <txsize> <speed> [txclk] [rxclk] [nrz|nrzi] [ipaddr] [#rxbufs]",

#endif

********************************************************************************

3. Add the following function declarations to commands.h

********************************************************************************

/* In twin_dr.c/twin_at.c  - Gracilis PackeTwin */

int tsync_attach __ARGS((int argc,char *argv[],void *p));

int do_tsync_stat __ARGS((int argc,char *argv[],void *p));

int do_tsync_dump __ARGS((int argc,char *argv[],void *p));

********************************************************************************

4. Add the following definition to the interfaces section of config.h

********************************************************************************

#define PACKETWIN	1	/* Gracilis PackeTwin driver */

********************************************************************************

5. Copy the following files to the build directory

********************************************************************************

# These are the PackeTwin driver itself

twin_dr.c twin_at.c twinvec.asm twin.h

# These are some definitions and support routines common to all

# Gracilis drivers.

gracilis.h gracilis.c

********************************************************************************

6. Create and add dependencies for the new driver sources to the makefile.

Currently, the dependencies are as follows:

********************************************************************************

# GRACILIS PackeTwin driver

gracilis.obj: gracilis.c global.h mbuf.h iface.h gracilis.h config.h lapb.h ax25.h proc.h

twin_dr.obj: twin_dr.c global.h config.h hardware.h mbuf.h iface.h ax25.h trace.h proc.h pktdrvr.h z8530.h gracilis.h twin.h

twin_at.obj: twin_at.c global.h config.h hardware.h mbuf.h iface.h ax25.h trace.h netuser.h proc.h pktdrvr.h devparams.h z8530.h gracilis.h twin.h

twinvec.obj: twinvec.asm

********************************************************************************

7. Add the following to the 'PC' section of the makefile

********************************************************************************

gracilis.obj twin_at.obj twin_dr.obj twinvec.obj

********************************************************************************

8. Also add them to the pc.tl file... thus:

********************************************************************************

+gracilis.obj &

+twin_dr.obj &

+twin_at.obj &

+twinvec.obj

********************************************************************************

9. Type "make" and wait till it's done...

********************************************************************************