�  �  �ķ  �ķ  ��ķ

�  �  ��з ��з ��ķ

ӽ   ��Ľ ��Ľ ��Ľ

�������������������������������������������������������������

*** VBBS MODEM DOCUMENTATION ***

�������������������������������������������������������������

Table of Contents

I.    VCONFIG/Channel Configuration

A.  COM Port

B.  Base Address

C.  IRQ

D.  Baud Rate

E.  Init String

F.  Locked Baud Rate

G.  Hardware (RTS/CTS) Handshake

H.  Minimum Baud Allowed

II.   UART Chips and Recommendations

A.  Reliability

B.  Multitasking

III.  Modem Initialization Settings

A.  Modem AT Command Settings

B.  High Speed Modem Initialization Strings

IV.   Discussion of Communications Standards

A.  Description of Standards

B.  Table of Modes

C.  Error Correction Standards

D.  Glossary of Terms

V.    VBBS FOSSIL Documentation

A.  Number of Simultaneous Open Ports

B.  2 Node System

C.  4 Node System

--------------------------------------------------------

VBBS Modem Documentation --  1

����������������������������������������������������������������ͻ

� I.  VCONFIG/Channel Configuration                              �

����������������������������������������������������������������ͼ

VBBS comes preconfigured to work correctly with most

2400 baud modems.  If you are setting up a 2400 baud system,

your modem configuration should be as simple as choosing the

correct modem port in VCONFIG's channel.cfg.  High speed

modems will probably require a more complex configuration.

Basic information is provided here for a sampler of those

modems.  Due to the variety of systems in use throughout the

network, indepth setup assistance for your specific hardware

is best obtained from the Modem Mania message base,

VirtualNET Sub #209.

VBBS allows multiple phone lines to be configured,

depending on the registration level chosen.  For each data

line in VBBS, a channel (phone line) configuration must be

set to define the Serial COM port that the individual line(s)

will be running from.  An example follows:

1.) COM Port :

2.) Base Address (Hex):

3.) IRQ (Hex) :

4.) Baud Rate :

5.) Init String :

6.) Locked Baud Rate :

7.) Hardware (RTS/CTS) Handshake :

8.) Minimum Baud allowed :

A. COM Port

�����������

Define a communications port for each channel in use.  COM

ports can be used more than once, but the repeated port must

have a different base address to prevent an IRQ conflict.

B. Base Address

���������������

The base address that DOS understands as the port assignment

for the channel. (example: COM1 is 3F8, COM2 is 2F8).

C. IRQ

������

Assign an IRQ (Interrupt ReQuest) for further definition of

the communications port.

D. Baud Rate

������������

Define your modem's baud rate speed, from 300 to 57600.

VBBS Modem Documentation --  2

E. Init String

��������������

The modem initialization string used by a particular channel

to define reset and operation of the modem.

F. Locked Baud Rate

�������������������

Toggle YES/NO for locked baud rate at high speeds.  This

should be set to NO for 2400-baud and slower modems.

Note: High-speed modems should use a locked baud rate to

indicate the maximum throughput achieved by the modem.  For

instance, a 14400-baud modem could be defined as 14400, 19200,

38400, or 57600, as the modem allows.

G. Hardware (RTS/CTS) Handshake

�������������������������������

Toggle "No, Yes(Normal), or Yes(Slow)".  This setting is for

high speed modems.  Yes(Normal) is the preferred choice.

H. Minimum Baud Allowed

�����������������������

Configure a minimum speed to tailor a high-speed line.

VBBS Modems Documentation --  3

����������������������������������������������������������������ͻ

� II.  UART Chips and Recommendations                            �

����������������������������������������������������������������ͼ

The NS16550AFN, or UART (Universal Asynchronous

Receiver/Transmitter) chip, is the heart of your asynchronous

communications board.  The advantage of the 16550 over the

16450 and 8250 UARTs is that it has a 16 byte buffer.

A.  Reliability

On the older chips, with their single-byte buffer,

you would lose data if a second byte came in from the modem

before the CPU had sufficient time to call the first byte.

The 16550, with its 16-byte buffer, gives the CPU up to 16

opportunities to access data before a character is lost.

B.  Multitasking

When VBBS is transmitting data, it has to stop the

CPU and fill the UART's transmit buffer.  That means that

the background caller will experience a system slowdown while

VBBS attempts to send data to another caller.  In addition,

the older 8250 chips were much less reliable, and were never

designed for the high speeds of current modems.  The

NS16550AFN, on the other hand, is designed with high bus

speeds and high modem speeds in mind.  When multitasking,

even at slower baud rates, the 16550 can be very helpful in

providing smooth operation for the entire system.

VBBS Modems Documentation --  4

����������������������������������������������������������������ͻ

� III.  Modem Initialization Settings                            �

����������������������������������������������������������������ͼ

Since high speed modems can utilize so many settings,

it is almost impossible to include them all in one initiali-

zation string.  Therefore, you may wish to use the NRAM fea-

ture to store the settings which are common to most applica-

tions and use the VBBS init string for VBBS-specific settings.

This collection of modem settings has been garnered

over time from VBBS sysops working with an unlimited variety

of hardware configurations.  If the following recommendations

provide unsatisfactory results, please refer to the VirtualNET

support subs for further assistance.

A.  Modem AT Command Settings

�����������������������������

Standard 2400 bps or 1200 bps Hayes Compatibles (NON-MNP):

Supports the extended "&" command set:

ATX4Q0E0V1&C1&D2S2=251S7=40

X4     Enables extended result codes

Q0     Modem returns result codes

E0     Modem does not echo commands back to the computer

V1     Modem returns verbal result codes

&C1    Modem asserts carrier detect based on connection

status

&D2    On DTR high-to-low transition, modem Hangs Up.

S2=251 Disable modem escape sequence

S7=40  Time modem waits for a carrier when initiating a new

connection

Does not support the extended "&" command set:

ATX4Q0E0V1S2=251S7=40

Modems which do not use the & command set usually have DIP

switches of some type to set their mode of operation. Verify

that your modem is set to assert CD (carrier detect) only

when connected to a remote modem and hang up on drop of DTR

(data terminal ready).

VBBS Modems Documentation -- 5

B.  High Speed Modem Initialization Strings

�������������������������������������������

�������������������������������������������������������������

Aceex 14.4

AT&F1S0=0M0E0V1X4&C1&D2&M5\N0S95=1S46=136S36=5

�������������������������������������������������������������

Boca 14.4 V32bis:

AT&F E0 V1 Q0 W2 S95=3 S7=50 S0=0

�������������������������������������������������������������

Compucom High Speed Modems:

AT&F2*H1%C0\N0S25=2M0QV1ES11=50&C1&D2H&W

Compucom STAR:

AT&F2*H2&D2S7=75&Y0&WX4M0L0S11=50

Recommendations:

Assign the above values to NRAM settings, and the

initialization string will actually read

ATZ0

To assign an NRAM setting, enter the above initialization

string, then type

AT&W0

To disable CCSP (Compucom proprietary protocol) type

AT*H0

Baud rate should be locked and set to 19200, or 38400 for

those with 16550 UARTs.

�������������������������������������������������������������

Cardinal 9600:

ATW2Q0V1E0&D2&C1S2=251M0&W2S95=1235

- or -

ATQ0V1E0&D2&C1S2=251S7=45M0

�������������������������������������������������������������

VBBS Modems Documentation --  6

�������������������������������������������������������������

Digicomm 14.4

AT&F&C1*S1E0

�������������������������������������������������������������

Lightning Communications:

AT&FE0&D2&C1&Q9S7=65W2M0

Locked Baud Rate:  38400

RTS/CTS         :  Slow

�������������������������������������������������������������

Motorola UDS 14.4 V32bis:

AT&F E0 V1 Q0 S0=0 %R2 \Q3 \V1 \M0 &C1 &D2 \N7 S7=50

�������������������������������������������������������������

SupraFAXmodem 14.4 V32bis:

AT&F2 E0 V1 Q0 W2 S95=3 S7=50

�������������������������������������������������������������

TwinCom V.32:

ATE0H0M0Q0V1X4S0=0S2=1W0S95=145

�������������������������������������������������������������

US Robotics Courier HST / Courier Dual Standard:

NRAM Settings Recommended:

X7     Return full result codes

&H1    Turns on hardware handshaking for output from DTE (CTS

line) *

&R2    Turns on hardware handshaking for input to DTE    (RTS

line) *

&B1    Fixes the DTE to DCE rate **

&N0    Negotiates highest link rate with fallback/fallforward

enabled

&M4    Enables ARQ if remote modem has ARQ

&C1    Modem asserts carrier detect based on connection

status

&D2    On DTR high-to-low transition, modem hangs up

&A3    Give FULL EXTENDED Result Codes

(Choose one of the 3 &K commands below based on your preference)

VBBS Modems Documentation --  7

&K1    Enables data compression if remote modem requests

compression

&K3    Enables V.42 bis data compression but not MNP5

&K0    Disables all compression

USRobotics Courier 14400 HST Dual Standard:

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B1  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=050  S12=050  S13=000  S14=000

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

- or -

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B0  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000  S14=001

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

VBBS Initialization String:

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&M4&N0&K1&A3

- or -

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&A3

�������������������������������������������������������������

VBBS Modems Documentation --  8

�������������������������������������������������������������

USRobotics Courier 9600 HST Settings

C=1  E=1  F=1  M=3  Q=0  V=1  X=7  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=TONE   ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=251  S03=013

S04=010  S05=008  S06=002  S07=025

S08=002  S09=006  S10=007  S11=040

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=005

S20=000  S21=010  S22=017  S23=019

DIAL=TONE   M=3  X=7  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=251  S03=013  S04=010  S05=008

S06=002  S07=025  S08=002  S09=006

S10=007  S11=040  S12=050  S13=000

S15=000  S19=005  S21=010  S22=017

S23=019

�������������������������������������������������������������

Additional USR Intitialization Strings:

USRobotics Dual Standard 14.4

�����������������������������

(Square LEDs -- not the 16.8 model):

AT&B1 B0 E0 V1 Q0 &N0 &M4 S7=50 &H1 &R2 &A3

(might work with newer 16.8 model also)

US Robotics HST 14.4

��������������������

(Round LEDs -- reports CONNECT 9600 even if 14400):

AT&B1 B1 E0 V1 Q0 &N0 &M4 S7=50 &H1 &R2 &A3

VBBS Modems Documentation --  9

Older US Robotics HST 9600

��������������������������

AT&B1 B1 E0 V1 Q0 &N0 &M4 S7=50 &H1 &R2

Also, another note: USRobotics modem users (Dual Std and

HST) can specify that a smaller transmit buffer should be

used during non-ARQ connections, so that when 2400/non-error

correcting callers call the board, the space-bar abort is

more responsive. The modem will auto-adjust the size of the

transmit buffer for each connection:

non-ARQ: 128 bytes

ARQ: 4k

To enable this modem feature, set S15=8.

�������������������������������������������������������������

Zoom 14400 Fax/Modem:

Locked Baud Rate:  38400

RTS/CTS         :  Slow

AT&C1&D2E0H0Q0V1X4W0S95=3\T2

�������������������������������������������������������������

Zoom 2400 V.42bis:

ATX4Q0V1E0&D2&C1S2=251S7=45

ACTIVE PROFILE:

B1 E1 L2 M1 N1 P Q0 V1 W1 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0

&S0 &T4 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002

S07:045 S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:005

S37:000 S38:020 S44:003

S46:138 S48:000 S49:008 S50:255

STORED PROFILE 0:

B1 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0

&S0 &T4 &X0

S00:000 S02:043 S06:002 S07:030 S08:002 S09:006 S10:014

S11:095 S12:050 S18:000

S25:005 S26:001 S36:005 S37:000 S38:020 S44:003 S46:138

S48:007 S49:008 S50:255

STORED PROFILE 1:

VBBS Modems Documentation -- 10

B1 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0

&S0 &T4 &X0

S00:000 S02:043 S06:002 S07:030 S08:002 S09:006 S10:014

S11:095 S12:050 S18:000

S25:005 S26:001 S36:005 S37:000 S38:020 S44:003 S46:138

S48:007 S49:008 S50:255

�������������������������������������������������������������

ZyXEL High Speed:

This is saved as Profile 0.  Init string is just ATZ0.

Be aware that register S20 sets the DTE speed, set to 19200,

so you may want to change this to S20=2 or S20=1.

CURRENT SETTING:

B0   E0   L4   M0   N5   Q0   V1   X6

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0  &R1

&S0  &X0  &Y0

*B0  *C0  *D0  *E0  *I0  *M0  *P9  *Q2  *S0

S00=000   S01=000   S02=251   S03=013   S04=010

S05=008   S06=003   S07=060   S08=002   S09=006

S10=007   S11=070   S12=000   S13=000   S14=002

S15=002   S16=000   S17=018   S18=000   S19=000

S20=003   S21=176   S22=000   S23=112   S24=133

S25=000   S26=000   S27=156   S28=064   S29=000

S30=000   S31=017   S32=019   S33=000   S34=030

S35=000   S36=000   S37=000   S38=016   S39=000

�������������������������������������������������������������

ZyXEL 14400 V.32 V.32bis V.42 V.42bis Send/Recieve Fax/Modem:

AT&FX6E0&C1&D2&G2H0M0|

Locked Baud: 19200

Hardware handshaking normal

�������������������������������������������������������������

VBBS Modems Documentation -- 11

����������������������������������������������������������������ͻ

� IV.   Discussion of Communications Standards                   �

����������������������������������������������������������������ͼ

A.  Description of Standards

����������������������������

CCITT v.21       300 bps.  (FSK).

Bell  103J

Bell  202        1200 bps, half duplex.

Leased lines.

CCITT v.22       1200 bps. (DPSK).

Bell  212A

CCITT v.22bis    2400 bps. (DPSK).

CCITT v.26ter    2400 bps with echo cancellation

Bell  208        4800 bps. Used on leased lines.

CCITT v.27ter    G3 FAX at 4800 bps with fall back to 2400

bps.

CCITT v.29       G3 FAX at 9600 bps with fall back to 7200

and 4800 bps.  Also used by Compucom

modems.

CCITT v.32       9600 bps with echo cancellation.  Fall back

to 4800 bps.  Both directions simultaneously.

(QAM).

CCITT v.32bis    14400 bps with echo cancellation.  Fall back

to 12000, 9600, and 7200 bps.  (TCM)  Both

directions simultaneously.

CCITT v.33       14400 with fall back to 12000 bps.

USR HST14400     US Robotics HST modulation.  14400 bps in

one direction and 450 bps in reverse

direction.  Fall back to 12000, 9600, and

4800 bps.

Trailblazer      Proprietary up to 18000 bps.

Hayes V-Series   Proprietary full-duplex 9600 baud.  Hayes'

CCITT LAP-B error correction and data

compression.

EIA Class2 FAX   G2 Fax standard.

CCITT v.FAST     19200 bps or slightly higher.  Uses echo

CCITT v.32bisbis.

ISDN             Integrated Services Digital Network on fiber

optic cable.

SMDS             Switched Multi-Megabit Data Service on fiber

optics.

VBBS Modems Documentation -- 12

HDSL             High-rate Digital Subscriber Link.  In

development.

B.  Table of Modes

������������������

MODE          BPS       BAUD      MODULATION      NOTES

-------------------------------------------------------------

v.33         14400      2400       128-TCM

v.33         12000      2400        64-TCM

v.32bisbis   19200      2400       512-TCM

v.32bis      14400      2400       128-TCM

v.32bis      12000      2400        64-TCM

v.32bis       9600      2400        32-TCM

v.32bis       7200      2400        16-TCM

v.32          9600      2400        32-TCM

v.32          9600      2400        16-QAM        Uncoded

v.32          4800      2400         4-DPSK

v.29          9600      2400        16-QAM

v.29          7200      2400         8-QAM

v.29          4800      2400         4-DPSK

v.27ter       4800      1600         8-PSK

v.27ter       2400      1200         4-DPSK

G3 FAX                                            v.29/.27ter

v.22bis       2400       600        16-QAM

v.22          1200       600         4-DPSK       BELL 212A

ISDN         65536     65536        FiberOptic    Digital

The baud rate over the phone line means that more than two

tones are used at the baud frequency to achieve the bps rate.

For instance, 4-DPSK means four tones.

C.  Error Correction Standards

������������������������������

NAME      DESC OF ERROR CORRECTION AND DATA COMPRESSION

-------------------------------------------------------------

MNP1      Hardware error correction.

MNP2      Hardware error correction.

MNP3      Hardware error correction.  Strips off start and

stop bits during transmission over phone line.

MNP4      Hardware error correction.  Fastest way to send

compressed files.  Strips off start and stop bits

during transmission.  Also adapts to line

conditions.

MNP5      Hardware error correction and data compression (up

to 2 time compression on TEXT).  Do NOT use on

compressed files!

MNP6      Additional Universal Link Negotiation and

Statistical Duplexing to MNP5 service

MNP7      Additional Enhanced Data Compression with MNP4.

MNP8      TBA

MNP9      Additional Enhanced Data Compression with v.32

modems.  Also adds Enhanced Universal Link

Negotiation, which allows connection to both MNP

and non-MNP modems at the highest performance

level.

MNP10     Hardware error correction and data compression.

VBBS Modems Documentation -- 13

v.42      CCITT error correction. LAPM.  Fastest way to send

compressed files.  Strips off start and stop bits

during transmission.

v.42bis   CCITT data compression (up to 4 time compression on

TEXT).  Uses a version of the Lempel-Ziv data

compression algorithm.

D.  Glossary of Terms

���������������������

baud        Actual rate of symbols trasmitted per second.

bps         Bits Per Second.

MNP         Microcom Networking Protocol.

CCITT       International Telephone and Telegraph

Consultative Committee.

FSK         Frequency Shift Keying.

DPSK        Phase Shift Keying.

QAM         Quadrature Amplitude Modulation.  Combination of

PSK and AM.

TCM         Trellis Coded Modulation.

LAPM        Link Access Procedure for Modems.

VBBS Modems Documentation -- 14

����������������������������������������������������������������ͻ

� V.    VBBS FOSSIL Documentation                                �

����������������������������������������������������������������ͼ

The "standard" serial driver which comes with VBBS is not

able to allow IRQ sharing between COM ports.  This is not so

much a limitation in the driver, as a limitation of the

underlying operating environment.  A FOSSIL driver, on the

other hand, can handle 2 or more serial ports using the same

IRQ line.  In fact, a FOSSIL can be used to control up to 8

ports.  It can be "programmed" to use almost any serial board

in existence.

VBBS has been extensively tested with both the BNU and X00

FOSSIL drivers. If you're new to FOSSIL drivers, they provide

a standard way for programs to communicate with serial ports.

The FOSSIL may load through either AUTOEXEC.BAT or CONFIG.SYS.

The advantages and disadvantages of each are discussed at length

within the individual FOSSIL documentation.

A.  Number of Simultaneous Open Ports

�������������������������������������

The basic CONFIG.SYS install for X00 is: DEVICE=X00.SYS

The basic AUTOEXEC.BAT install for BNU is: BNU

By default, a FOSSIL will assume you wish to have only one

communications port open at any given moment.  The FOSSIL

doesn't specify which port (COM1, COM2, COM3, etc) but is

merely concerned with the maximum number of simultaneous open

ports.  Each port you request is like a "data channel,"

corresponding to your VBBS data channels.  For more detailed

information on a driver, consult the specific FOSSIL

documentation.

B.  2-Node System

�����������������

Channel 1 to be on COM1

Channel 2 to be on COM2

�����������������������

X00: DEVICE=X00.SYS 0=COM1 1=COM2

BNU: BNU /P=2

VCONFIG: Set Channel 1 to COM1

Set Channel 2 to COM2

VBBS Modems Documentation -- 15

Channel 1 to be on COM2

Channel 2 to be on COM3

�����������������������

X00: DEVICE=X00.SYS 0=COM2 1=COM3

BNU: BNU /P=2

VCONFIG: Set Channel 1 to COM2

Set Channel 2 to COM3

C.  4-Node System

�����������������

Channel 1 to be on COM1

Channel 2 to be on COM2

Channel 3 to be on COM3

Channel 4 to be on COM4

�����������������������

X00: DEVICE=X00.SYS 0=COM1 1=COM2 2=COM3 3=COM4

BNU: BNU /P=4

VCONFIG: Set Channel 1 to COM1

Set Channel 2 to COM2

Set Channel 3 to COM3

Set Channel 4 to COM4