##1#[1z#[1z

The Amiga Contest Logging Programs V4.02

Copyright 1992,1993,1994 by Peter Hardie VE5VA

General Instructions (12-Jan-94)

New in V4.02

- If you type the DELETE character immediately after entering a DUPE QSO

the entire entry will be deleted rather than just the last field.

- The backspace character will backspace over a string of spaces instead

of one at a time.

- The program now uses a built-in form of the rexxhostlib package and so

the rexxhost.library is NOT required any more.

- The program now has the ability to colour  code the  DX spots arriving

from a packet cluster. Currently only the CQWW program supports this but

it  will  be  added  to  the  other  appropriate  programs  soon.  (Full

packetcluster support is not yet implemented).

- The SS program no longer requires the w-ve.list file and RAC no longer

needs the  ve.list  file.  Both  now  obtain  the  info  they  need from

dxcc.list.

New in V4.01

- The program  can be  forced to totally ignore a QSO by editing the log

(using ^E) and  putting  a  letter  Q  as  the  first  character  of the

callsign. This provides a way of deleting a QSO from the log, although

the entry is still there it will be ignored by the scoring and reporting

routines.

New in V4.00

- partial call  lookup has been added. Up to 20 calls will be displayed.

The partial call display shares the same area  on the  screen as  the CW

memories. See Section 3.2.

- Partial support for the Workbench has been added thanks to Ernie VE5CJ

who provided me with  all  the  icons  used  in  this  distribution. See

Section 3.4.

New in V3.05

- The log  file structure has changed slightly. I have added one comment

line at the front of the file. This line contains the callsign  used for

the  contest.  For  many  ops  this  callsign  will never change between

contests, but in Canada, for example,  we are  sometimes allowed  to use

special prefixes.  In this  case it  is convenient  to have the callsign

stored in the log file so that a program that generates QSL  labels, for

example, won't have to hunt around for the callsign. You also won't have

to try to remember,  possibly  months  after  the  event,  which special

prefix was used.

- there is  now a  driver for  Kenwood rigs. It should be able to handle

anything from the TS-440S upwards. It  only uses  a small  subset of the

available commands and all the rigs should be able to handle them.

#[1z

1. Introduction

These instructions  describe the  features that apply to all versions of

the contest logging programs (currently: CQWW,  WPX, RAC,  FD, SS, ARLX,

IARU, WAE, NAQP and QRP).

Chapter 8 describes instructions specific to each contest program.

You use  these programs  AT YOUR  OWN RISK  on an  AS-IS basis. I do NOT

guarantee that these programs will  perform  as  described,  nor  as you

expect them  to, and  I accept  no liability  whatever if these programs

lose information, score incorrectly or in any other way fail  to perform

correctly.

You MUST read Chapter 7.

If you  have used  these programs  before, then  at least read Chapter 6

"KNOWN BUGS and SHORTCOMINGS" to see if any  major bugs  have been fixed

or found.  Also look  at the file "changes" which documents changes made

to the programs as I go along.

In order to use the programs you MUST be able  to use  a text  editor to

set up some of the files that control the programs and at the moment the

programs can only be  started from  the shell.  I hope  to add workbench

support some time or other.

1.1 For the impatient

Those of  you who  don't read past the introduction of any documentation

will find that as long as you use the same directory structure as exists

on the disk, then you won't need to do much in the way of setting up the

programs other  than to  put a  setenv command  in your startup-sequence

which describes  your local timezone so that the program can compute UTC

for proper timestamping of  your  log  entries.  You  must  also  do the

following:

- If you  are going  to use  computer control of your rig you must start

up the  appropriate driver  program before  you start  up a contest

program.

- If you  use the  AREXX functions  (PacketCluster) then you must have a

copy of rexxhost.library in  your LIBS:  directory. The  library is

supplied with  this distribution. BUT there is currently NO program

to send the packetcluster information to  the logger  programs. You

can write one yourself. I hope to have one ready soon.

- You must  set up  a default  file for each program you wish to run. In

addition, you can put some of  the  common  default  commands  in a

global.defaults file in the lists directory.

#[1z

2. Hardware Requirements.

2.1 Computer

The logging  programs should  run on any Amiga which has at least 1Mb of

memory and they might work on a  512kb  machine  if  you  don't  use the

default 2000  QSOs (see  next paragraph). A hard drive is preferable for

storing the log during the contest as the I/O is  faster, but  it is not

essential. However, when running the program, your log file MUST be on a

disk. If you tell the computer to store the log in ram,  a power failure

will lose your entire log. The program has been tested on an A3000 using

Kickstart 1.3 and 2.04 and both work. It has also  been partially tested

on an A1000 with 2Mb using Kickstart 1.3 and it also works (although the

AREXX port hasn't been tested there).

The memory requirement will be the most critical  for those  of you with

only 512k.  When allocating the memory for the log, the programs ask for

it all in one big lump. This  way  you  will  know  before  you  start a

contest whether  or not you have enough memory. By default, the programs

ask for enough space to handle 2000 QSOs.  You can  specify a  larger or

smaller number  of QSOs by using the 'n' command in the default file for

the specific contest. But  you  must  ensure  that  whatever  number you

choose, it  will be big enough to handle the whole contest. If you don't

make the number big enough then  the program  will terminate  as soon as

the limit  has been  reached. In this case you can increase the QSO size

by editing the default file and then starting the contest program again.

The programs also allocate chunks of memory for the multiplier lists and

to do some types of report but their sizes cannot be altered.

2.2 Audio output

The computer sends an audible beep  out of  the audio  jacks and flashes

the monitor  screen when an error (e.g. a dupe or unrecognized callsign)

occurs. The old A1000 monitor has the audio built-in so  there's nothing

for you  to do  to use  it except  turn up the volume. Some of the newer

monitors (e.g. 1950) do not have audio output  so you  must hook  up the

Amiga's audio output to a suitable amplifier and speaker.

It takes  a noticeable amount of time to transmit the three beeps to the

audio device when a dupe or error occurs and so you have  the option, in

the default  file, of  turning the  audio off (if you use headphones you

probably won't hear it anyway!). The command "a  0" in  the default file

will turn the audio off.

The hardware described in the remaining sections is all optional.

2.3 Computer controlled radios

The  program  has  been  tested  with  an  ICOM  IC-735 using a homebrew

interface. It also works on a  FT-767GX using  their FIF-232C interface.

I also plan to have the program understand various other ICOM, Yaesu and

#[1z

Kenwood rigs. The diagram for my IC-735 interface, which uses two 2N2222

transistors is  included in  a separate file (iface.txt). If your rig is

not supported yet then send me a detailed spec of  the computer commands

and the interface required and I'll try to add it to the list.

All computer controlled radios use the serial port but some of them need

more of  the pins  wired up  than others.  The IC-735  requires that the

following pins be connected:

Pin 2   Tx

Pin 3   Rx

Pin 7   Gnd

My  interface  also  requires  +12V  to be connected and some commercial

interfaces may also need it:

Pin 23 (A1000 only) +12V

Pin 9  (all others) +12V

If you make  up  a  homebrew  interface  you  must  be  VERY  careful to

identify the  +12V pin  correctly or  you could  blow the 8520 chip. You

must also take great care when  plugging the  interface into  the serial

port's DB-25.  The port  uses a  male plug and if you use a DB-25 female

with a metal shell, then the shell could short the +12V to  ground. This

blows a  fuse in the computer and it costs a lot of money to replace it.

Therefore, whenever you plug in  the  serial  port  interface,  TURN THE

COMPUTER OFF  FIRST! The  only way  you can safely plug in the interface

without blowing the fuse while the computer is  switched on  is by using

an all-plastic DB-25.

Yaesu rigs appear to need the same pins connected as the ICOM.

Kenwood rigs also require:

Pin 4   CTS

Pin 5   RTS

Some rigs,  notably the Yaesu FT757GX can only receive commands from the

computer and cannot transmit  responses  at  all.  The  logging programs

allow for  control of  the radio  from the computer so that provided you

use only the program to control the rig,  it can  log your  contacts for

you. But it might be safer to get used to doing without.

2.4 Morse code output

WARNING -  as of V3.00 the morse code output has been moved from the RS-

232 Pin 20 to Pin 9 on the second joystick port.

All the contest programs have a  built-in keyer  with ten  memories. The

keyer speed  and the  content of  the memories can be initially set in a

configuration file and also  modified on-the-fly  from 10  to 50wpm. The

morse code  is transmitted out of pin 9 of the second joystick port. PTT

is output from Pin 5 of the second joystick port. You do not have to use

the PTT output - you can use your rig's VOX to key your rig if you wish.

#[1z

The cursor  UP and  DOWN keys can be used to change the keyer's CW speed

by plus or minus 2wpm each time the key is hit.

2.5 Morse code input

All of the contest programs also provide an electronic morse code keyer.

The keyer  can emulate  an iambic  paddle with dot and dash memory and a

choice of either Curtis  A or  B timing.  It will  also emulate  a semi-

automatic bug  with automatic  dots and  manual dash, or just a straight

key (which is just the key connected  to  the  dash  side  of  the semi-

automatic bug).

This  function  is  used  by  connecting your key to the second joystick

port. The pin connections are:

Pin 8 - Gnd

Pin 2 - Dot Input

Pin 4 - Dash Input (or straight key)

This function was provided more as an academic  exercise to  show that I

could do  it, but  it can  also be  useful if you need to send something

which is not covered by the content of the ten  memories and  it is more

convenient to have your paddle connected to the computer instead of your

keyer.

In the future, I may add code to read what you are transmitting with the

paddle so  that it  will read  the callsign  and save you having to type

that in.

#[1z

3. Software

When you use the contesting software you will  need one  or two programs

depending  upon  the  hardware  configuration.  If  you are going to use

computer control of your radio then you must first run a program (in the

drivers directory)  that corresponds  to your rig. For example, before I

use any contest program, I must first:

run drivers/ic735

This program handles the interface to  the  rig.  Once  this  program is

running you  do not  need to kill it after stopping the logging program.

You can leave it running in the background so that  if you  start up the

logging  program  again  later,  you  do  not  also have to start up the

driver.

There are currently driver programs for  the  ic735  (and  any  ICOM rig

using the same protocol), the Yaesu FT767GX II, and recent Kenwood rigs.

All drivers default to using  the  serial.device  unit  zero.  The ic735

defaults to  9600 baud  and the other two programs default to 4800 baud.

These can be changed using CLI arguments to the programs:

-s name   Use the device name specified by the "name".

-uN       Use the unit number N.

-bN       Use baud rate N.

-hN       For the ic735 program, set the rig's address to N  (default is

4)

-mN        For the ic735 program, tell the computer to use N as its

address instead of the default 1.

If you do not start up a driver program, the contest program will report

that there  is no  driver and will then use a default frequency and mode

which you can set  in one  of the  default files  (see below).  Also, if

there's no  driver, you  must tell the program when you change frequency

or mode on the  rig. There  is an  easy way  to do  this using  the band

stacking feature which is explained later.

The driver  program can  be terminated  by executing  the program called

stopdriver in the drivers sub-directory.

Once the driver is  running, you  can start  up the  appropriate logging

program, whether  it be  WPX, QRP or whatever. All contest programs need

the name of the logging file to be used. I use, for example,

qrp ve5va.qrp

If you do a lot of contesting it would probably help if the name  of the

logging file contained the date and mode: e.g.

cqww 1992-cw.cqww

Do NOT use a name that starts with the word "log". The programs output a

formatted log to a file whose name begins  with "log"  and it  is safest

not to use this so that the program does not destroy your log file.

#[1z

Some of  the functions  of the  logging programs  are common  to all and

others are specific to the particular program.

3.1 General functions

All of the programs can  be  terminated  by  typing  a  Control-D  or by

clicking the  left mouse button while pointing at the top left corner of

the window. The window does not have any gadgets on it but  you can also

make the  window go to the back of all the windows in the current screen

by clicking the left mouse button while pointing at the top right of the

window. You  can push the entire current screen to the back of all other

screens by typing right-amiga-B.

3.1.1 Default file

All of the logging programs read two default  files when  they start up.

These  allow  you  to  set  various options. All programs first read the

file called "global.defaults" which is  in  the  "lists"  directory (see

section 4  which describes  support files). This file can be used to set

options such as your  callsign which  will usually  be the  same for all

contests. Each  program then  reads a defaults file which is specific to

that contest program. The name of  this  second  file  depends  upon the

specific program being used and consists of the program name followed by

".defaults" and the file must be  in the  same directory  as the contest

program. For  example, the WPX program looks for the file "wpx.defaults"

and the QRP program  looks for  "qrp.defaults". This  file is  used, for

example, to  initialize the  content of  one or  more of  the morse code

memories which would change depending upon the contest  exchange. If you

are not  using computer  control of  your rig  you can specify a default

frequency and mode that the rig should use. Each sample default file has

documentation of the available commands but, briefly, they are:

!              Comment - the line is ignored.

c call         Set  your  callsign.  NOTE  that  this  is  normally your

standard callsign but if you use a special prefix, or are

portable in  a different  call area  or country, then you

should set this to the special callsign used  so that the

reports have the correct callsign on them.

This command MUST be present.

z N            Set your  zone to  N for  the CQWW  or IARU contests. You

don't  need  to  set  this  if  the  program  is  able to

determine  your   zone  uniquely  (e.g.  if  you  are  in

Britain). But if you are in  the U.S.,  for example, your

zone  canot   be  determined  with  certainty  from  your

callsign. When you start up a contest program requiring a

zone, make  sure that  the program  displays your correct

zone with your callsign near the top left of the screen.

F              Allow the program to  use Forbid()  and Permit()  while a

CW   memory   is   being   transmitted.   This   suspends

multitasking so that other processes won't interfere with

#[1z

the timing  of thetransmitted  code. It  is not foolproof

and sometimes something still interferes but I have tried

using Disable/Enable  and they  cause a wierd bug whereby

the program will send the same memory over and over again

forever.

n N            Set the  number of  QSOs in  the in-memory  log to N. The

program defaults to 2000. Each QSO is about  80 bytes, so

the default  requires at  least 160Kb of ram for the log.

If you aren't going to make  anywhere near  2000 QSOs you

can cut this down to a more reasonable number (e.g. 1000,

500 or whatever - but make sure you OVERestimate).

l N string     Set CW memory N to the string.

m S            Set morse code speed to S wpm. S is an integer between 10

and 50.

p N            Set the  Field Day or QRP ARCI power output multiplier to

N.  You  MUST  have  this  set  correctly  for  these two

contests because  it directly affects the scoring and the

report outputs.

P string       Set your output power  to  the  string  (no  more  than 5

chars). Not  used in  scoring the log, it is only printed

on QSLs when  I  get  around  to  writing  the  QSL label

program.

s F            Set the QRP ARCI power Source multiplier to F. F defaults

to 1 but if you use a battery you would use 1.25.

t Z            Set your  local  timezone  to  Z  (see  next  section for

format).

f freq Mode    Set the  default frequency  to 'freq' and mode to 'Mode'.

The frequency is specified in kilohertz.

k N            Set keyer type to N. 0=Iambic-B, 1=Iambic-A, 2=Bug

h N            Set QRP ARCI homebrew bonus points to N.

b              Backup  the  log  entries  to  the  printer  as  they are

entered. The  program always  writes each QSO to the disk

and this command makes it  write  it  to  the  printer as

well. If  you edit  the log and make any changes they are

written to the printer so that you can keep track  of the

changes. Serious contesters should always use this option

as a failsafe.

a n            Turn the audio on  (n=1) or  off (n=0).  If the  audio is

off  the  program  responds  to  dupes  faster because it

doesn't have to send three beeps to the audio device.

q n            Set the type of QRP contest. Currently 0  is QRP-ARCI and

#[1z

1  is   MI-QRP.  If  MI-QRP  is  chosen  then  the  power

multiplier (p command) is forced to one.

x N            Set the PTT delay to N*10 milliseconds ... i.e. the  PTT

delay is specified in units of 10 milliseconds.

e string       REQUIRED for  the FD  and SS  programs. Used  to set your

class and section so that they can be printed in the log.

ALSO REQUIRED  for Canadian ops (only) in the RAC contest

and must be used  to set  the two  letter abbreviation of

the province from which operation took place. Examples:

In FD     e 1e sk

In RAC    e sk

In SS     e a ve5va 82 sk

Note that  for SS  you put your whole exchange except for

the  serial   number   (i.e.   precedence,call,check  and

section)

It is  strongly recommended that you use the printer backup throughout a

contest so that a disk (or  program) failure  will not  lose your entire

log.

3.1.2 The date and time

All versions of the logging programs keep track of the date and time and

store these in each log entry. They also  show the  current time  at the

top left of the screen. Just before you start a contest you should first

ensure that your clock  is set  to the  correct time  (local or  UTC) by

synchronizing  it  to  one  of  the  time  services such as WWV. If your

computer time is local time and  not UTC,  then you  must also  tell the

computer about  your local  time zone.  This can  be done  in one of two

ways. Either you can specify the 't' command in the default file  or you

can put  a setenv  command in your s:startup-sequence file which defines

the name  TZ_CHU as  your local  time. In  either case  you specify your

local timezone as hours (and optionally minutes) west (positive) or east

(negative) of Greenwich. The program also handles daylight  savings time

conversion and  this must be specified in the same 't' command or setenv

call if it is applied in  your  area.  Finally,  if  your  timezone uses

daylight savings  time, then  the program also needs to know whether you

are in the northern or southern  hemisphere and  you encode  this bit of

information by  specifying the  DST indicator  in lower (North) or upper

(South) case. The general format of the time is:

[-]hh[:mm][D]

This means that the time is specified as a number of hours  which can be

preceded by an optional minus sign (for EAST) and optionally followed by

a number of minutes  (e.g. 3:30  for Newfoundland)  and all  of this can

have  an  optional  letter  after  it to specify that your timezone uses

daylight savings time.

The dates of the start and end of DST are different in  various parts of

the  world.  You  indicate  which  is  used  in  your zone by one of the

following letters. If you are in the northern  hemisphere then  you must

specify  the  letter  in  lower  case,  and  if  you are in the southern

#[1z

hemisphere then you must specify it in upper  case. The  letters and the

dates of the start and end of DST are listed below:

d or D:   North America. Start first Sunday in April and end last Sunday

in October.

u or U:   U.K. Start  last  Sunday  in  March  and  end  last  Sunday in

October (approximately!  - the precise dates are set by act of

parliament each year).

e or E:   Europe.  Start last  Sunday in  March and  end last  Sunday in

September.

c or C:   China(!). Start 12th April and end 12th September.

If you specify upper case for the southern hemisphere then the start and

end of DST are reversed. In all cases the start and end occur at 2am.

The  best   thing  to   do  is   to  add   a  setenv   command  in  your

s:startup-sequence file.  If you  use any  of my other programs, such as

the C_BBS packet program or the CHU program, then you  won't have  to do

anything extra  with them  if you use setenv TZ_CHU in your startup file

because they all use  the same  routine to  check for  your timezone and

adjust to UTC. I use the command:

setenv TZ_CHU 6

in my  s:startup-sequence file because I am in the Central Standard Time

zone (6  hours west  of Greenwich)  and we  do not  use daylight savings

time (to  be pedantically precise, we are actually on permanent Mountain

Daylight Savings Time but very few people understand  the difference, or

care!). In  Germany, which  is one  hour ahead  of UTC and uses European

daylight savings time you would use:

setenv TZ_CHU -1e

I am not sure what DST is used in Australia or New Zealand but if, as an

example, they  use the  same DST  as the U.K. (except that they start in

October and end in March) then an upper case 'U' is specified along with

the timezone correction.

(The odd name "TZ_CHU" is derived from a program I wrote which reads the

time from Canadian radio station CHU  and sets  the amiga  battery clock

fairly precisely).

3.1.3 QSO entry

All of  the programs  enter a  QSO on  the bottom line of the screen and

when it is complete, move it  up one  line. Above  that line  is a title

which  shows  the  name  of  each  field.  The  fields  whose  title  is

highlighted are the only fields that  you need  to enter  data into. The

others  will  be  filled  in  by  the  computer. For example, in the WPX

contest the title starts with the  callsign,  rst  and  QSO  number. The

#[1z

callsign and  QSO number  are highlighted  but the rst is not. Thus, you

enter a callsign and QSO number, but the computer will  fill in  the rst

for you.

Some  of  the  contest  programs  require  that  you type a space or tab

between each field (they ALL require a tab or space after the callsign).

However, there  are some exceptions. The SS program does not require any

tabs between the fields of the exchange because  it can  determine where

one ends  and another  begins so  you can type the entire SS exchange in

without typing a tab or space.

Some of the programs will fill in information for you  if they  can. For

example, CQWW will fill in the CQ zone of the callsign just typed if the

country is entirely contained within one CQ zone.  For example, entering

any G  call will cause CQWW to fill in 14 for the CQ zone. I have set up

the dxcc.list file so that it also splits some of the Canadian callsigns

into their  call areas  and CQWW  will fill  in those  zones that can be

identified from their prefix.

The SS program will fill in  some dummy  information for  an exchange if

you enter a DX call.

The ARLX  program will accept just the letter 'K' if a DX stations sends

you "KW" as their exchange. The program will change the 'K' to  'KW' for

you.

As you type in data on the QSO line you can use:

- the backspace key to delete the previous character

- the delete  key to  delete the  current field. If the cursor is at the

beginning of a field, then move  to the  beginning of  the previous

field.

- the cursor  left key  moves the  cursor one  place to the left without

deleting the character. Typing the cursor  left key  with the shift

key  down  makes  the  cursor  move  back to a question mark in the

callsign field if there is one,  or  to  the  first  field  that is

blank.

- the cursor  right key  moves the cursor non-destructively one place to

the right.

- a space or tab key causes the cursor to move to the next field  to the

right. If  the field just entered was the callsign then the call is

duped and the program will flash  the screen  and cause  an audible

beep (if  audio is  on) for  a dupe  (in which  case you can either

delete the call and start again, or enter  the QSO  anyway). If the

cursor is  at the  beginning of  a field  and nothing  has yet been

entered then the cursor will remain where it is. But  if the cursor

is at  the beginning of a field that already contains data then the

cursor will  skip to  the next  field without  changing the current

one.

- a carriage  return key  is only valid at the end of the last field and

then the line is entered, but ONLY if there is no question  mark in

the callsign  field. This  is the  ONLY way to get a QSO entered in

the log.

- typing the Alt key and carriage return makes the cursor go to  the end

#[1z

of the  entry line but does not enter it (you must hit CR again for

that).

- the escape key makes the cursor  move  to  the  right  until  it finds

either a  field whose  first character  is blank  or the end of the

data entry line, and then the  cursor  stops.  This  is  useful for

skipping over  previously entered  fields, or fields that have been

automatically filled in by the computer.

- Control I causes the program to  move the  cursor to  the beginning of

the  callsign  and  enter  Insert  mode. This allows you to enter a

callsign prefix when you  only had  a suffix  in the  field. Insert

mode is  terminated by typing a tab at which point the new callsign

is again duped. If it is not a dupe, then  the cursor  will skip to

the next empty field or the end of the entry line.

The first  field on the line is always the callsign of the station being

worked. When you hit a space or tab key, the callsign is  checked and if

it is a dupe under the contest rules, then the screen will flash and the

audio output will beep.  The program  will also  display the highlighted

string "*DUPE*"  above and to the right of the callsign - typing any key

makes this disappear. If you have a dupe, you can  enter the  delete key

to remove  the callsign  or you can continue with the exchange and enter

the QSO anyway. Dupes are indicated in the log by putting "?DUPE" in the

score field.  But, in  any case,  the QSO  will score  zero and will not

generate any multipliers. If you do  not hear  a complete  call, you can

enter a  question mark  (e.g. NJ7?)  and then enter the remainder of the

QSO. You can then easily return to the callsign to  fill in  the missing

piece by  typing the shift and cursor left keys. Then type escape to get

to the end of the field, followed by carriage return to enter the QSO.

If you enter an  invalid callsign  then the  display will  flash and the

message  "INVALID  CALLSIGN"  will  appear  above the title line. If the

call is valid but the program cannot locate the callsign  in the country

list, or it cannot find a valid prefix in WPX, the program will beep and

the multiplier field will have question  marks entered  into it.  If you

entered the  call incorrectly  then edit  the call and try again. But if

you are sure you have the call entered  correctly then  you should enter

the QSO anyway and proceed with the contest, although this contact won't

be scored at this time. The error will be caused  by a  problem with the

dxcc.list. Either  during the  contest (if you have time) or after it is

over, you  can examine  the callsign  and the  dxcc.list to  see why the

program could  not recognize  the call.  Then correct  the error, either

using the  editing  function  in  the  logging  program  to  correct the

callsign, or with a text editor to correct the dxcc.list and then simply

start up the logging program again  with  the  log  as  input.  When the

logging program is started, it always re-scores and re-dupes the current

log file so that once you  have corrected  an error  the logging program

will  correct   the  score  and  remove  the  question  marks  from  the

multiplier field. You can then proceed to generate  the required reports

using the %R command.

Some numeric  fields in  the contest  will allow  you to  type in a cut-

number instead of a numeral. The RST field  and the  serial number field

in WPX  and other contests allow you to do this. This is useful on CW so

that you can type exactly what you hear (if you  wish). For  example, in

#[1z

WPX you  may get  a serial  number transmitted  to you as "aont". If you

type this in, the program will correctly translate it into 1090.

The serial number field in WPX, the Nr/Power field in  QRP-ARCI and some

other fields  will also  be right  adjusted after entry. For example, if

you receive the serial number "45" in WPX, it will  be left  adjusted in

the serial  number field when you type it, but when you enter the QSO by

typing carriage return, the field will be right adjusted.

3.1.4 Frequency and mode

If you are using  computer controlled  frequency and  mode then  you can

change the  rig's frequency  and mode  by using  the mouse. Pointing the

mouse at the mode  display  and  clicking  the  left  button  causes the

computer to  change the  rig's mode  to the next in its list which is in

the order LSB, USB, AM, CW, RTTY (if the rig has it) and FM.

Pointing at any digit of the frequency display and holding down the left

mouse button  will cause  that digit  to be  incremented at  a rate of 7

times per second until you let go of  the button.  If you  hold down the

right  mouse  button  then  the  digit  will  be decremented 7 times per

second. In either case the  rig's  frequency  will  be  updated  as this

occurs IF  the rig  is capable of handling that frequency. The Yaesu and

Kenwood  protocols are rather slow  and  so  this  function  is  not too

impressive on those rigs, but on an ICOM IC-735 it works much better.

If  you  are  not  using  computer  control  of  a  rig (i.e. there's no

background driver) then the program initially displays the frequency and

mode that  are set in the default file and these can also be modified by

using the mouse buttons.

3.1.5 Morse code display

If the rig is in CW mode, the  morse code  speed is  displayed below the

list of the memory contents and you can change the speed of the keyer by

pointing the mouse at either of the digits and clicking  either the left

or right  mouse buttons. Each click of the left mouse button adds one to

the digit and each click of  the right  mouse button  subtracts one. The

keying speed is limited to between 10 and 50 WPM.

Beside the  CW speed is the name of the type of keyer - one of Iambic A,

Iambic B or Bug. Clicking the left mouse button on this field will cause

the program  to change  to the next type in the sequence. The keyer type

can be set in the default file.

The line  underneath the  CW speed  shows the  value set  for PTT delay.

This number is in units of 10 ms. Setting it to zero turns PTT off.

Each of  the 10  CW memories is associated with one of the corresponding

Function keys - thus, memory 1  is associated  with F1.  Typing just the

function key  causes that  memory to be transmitted in morse code out of

Pin 9 on the second joystick  port.  The  content  of  a  memory  can be

changed by  holding down the shift key and then typing the corresponding

function key. The cursor will move  to the  selected memory  and you can

then type  in what you want. Type a carriage return to finish typing the

#[1z

string. The display only  shows the  first 15  or so  characters of each

memory string, but each memory string can be up to 80 characters long.

There are some abbreviations which you can use when entering data into a

memory.

%c is replaced by the callsign  that is  currently stored  on the bottom

line of the display even if it is incomplete (e.g. NJ7?)

%C is replaced by your callsign which is specified in the default file.

%N is  replaced by  the current QSO number. A limited form of cut number

is employed. Leading zeroes are sent as the letter 'T'. Embedded or

trailing zeroes  are sent  as the  letter 'O'  and the  number 9 is

always sent as 'N'.

%n is the same as %N except that leading zeroes  are sent  as the letter

'T' but no other cut-numbers are used.

%R is replaced by the transmitted RST field.

%dD  Decrement the current code transmission speed by the number D which

must be a single numeric digit.

%iD  Increment the current code transmssion speed by the  number D which

must be a single numeric digit.

As an example, you might enter a CQWW exchange in a memory as:

%i95nn%d9 14 k

which will  increase the current code speed by 9 wpm, send 5nn, drop the

code speed back down by 9wpm and then send the zone (14) and the  'k' at

the original  speed. If  you wish to increase or decrease the code speed

by more than 9 wpm, you can string  two or  more increment  or decrement

commands together. Thus, %i9%i6 will increase the code speed by 15 wpm.

There is no way to change the speed to an absolute amount yet.

At  the  moment  none  of  the  memories  has  a specific meaning to the

program. However, I intend to make memory 10 a "CQ" memory such  that if

you type  the control key and F10 then the content of the memory will be

transmitted followed by a delay of  about 5  seconds and  if you  do not

type any key or touch the CW paddle, then the memory will be transmitted

again to a maximum of 20  times. I  may also  add another  feature which

will require that you use memory 1 as the exchange.

At a  future date  I also  hope to  change the  program so  that you can

specify a separate speed for each  memory and  also a  way to  chain the

memories together.

3.1.6 Band stacking

Holding down  the shift  key while pressing one of the up, down, left or

right arrows on the NUMERIC PAD (not the cursor control keys) causes one

of the  band stacking  functions. Shift-8  causes the current frequency,

mode and content of the callsign  field  to  be  stored  in  an internal

#[1z

stack. Pressing  shift-4 or  shift-6 causes  the program to move through

the stack returning the content of the stack to the  operational fields.

If a  computer controlled  rig is  used, its  current frequency and mode

will be set to the current stacked value. The stack  will hold  up to 50

entries. This  feature is  useful when  you hear  a station that is very

busy and you decide not to wait to bust the pileup. Type in the callsign

of the  station and then hit shift-8 to remember the station's callsign,

frequency and  mode. Once  a stack  entry has  been recalled,  it can be

deleted  from  the  stack  by  typing shift-2. The band stacking feature

works even if a computer controlled rig is not present and allows you to

store common  frequency/mode pairs  (the callsign  can be  blank) in the

stack. Switching  to  another  frequency  or  mode  is  easy  using this

feature.

The  program  stores  the  current  content  of  the  stack  in the file

"stack.dat" and if this file is present when the program  is started, it

is used  to initialize  the content  of the  stack. If you do not have a

computer controlled rig  you  can  initialize  the  stack  to  contain a

frequency/mode pair for each band and then use the left and right arrows

on the numeric pad to select the appropriate freq/mode.

3.1.7 Special Functions

If the callsign field on the bottom line of the  screen begins  with the

'%' symbol,  then the  remainder of the line is interpreted as a command

to  the  logging  program.  These  commands  perform  such  functions as

generating reports  and editing  the log. Each function can be called in

one of two ways. Either you can type a command of the form:

%C <arg>

where C is a single character  and <arg>  is an  optional argument. This

command is typed in place of a callsign.

Alternatively, the commands can be called with a control key combination

consisting of the control key and the command  letter. In  this case you

cannot specify  an argument. As an example, to do an all-band report you

can either type the command:

%R

in place of a  callsign,  or  you  can  just  type  the  key combination

control R (i.e. type the "r" key while holding down the control key).

3.1.7.1 %R Generating Reports

The %R  command generates various types of reports, logs and dupe sheets

which depend upon the particular contest. Two of the programs,  IARU and

FD,  also  produce  a  log  (called arrl.report) in ARRL Standard format

(although FD doesn't currently use this format).

#[1z

The command:

%R

produces an all-band summary report, logs for each band and  dupe sheets

for  each  band.  Some  contests,  such  as  Sweepstakes, do not require

separate band reports, in which case the program simply produces one log

and dupe file.

If you want to create a single band entry then specify the band with the

%R command:

%R 15

Will produce single band entry reports for the  15 meter  band. The file

name of the summary report (which should always be printed) has the form

report_MM.PGM where MM is the mode ('cw','sb' or if all-mode then 'all')

and PGM  is the  name of the logging program ('qrp', 'wpx' etc). The log

file names are of the form MM_BB.PGM where MM is the mode (cw or sb) and

BB is the band (either the numeric band for a single band entry or 'all'

for an all-band entry).  Similarly, the  dupe file  names have  the form

dupe_MMBB.PGM. All  the files  are produced in the current directory and

are spaced  ready for  printing (you  can use  the print  command in the

df0:c directory).

The QRP  contest also allows hi-band and lo-band entries and reports for

these entries can be generated with "%R H" or "%R L".

At the moment the contest reports are generated  in a  form suitable for

printing. I  hope to  add the ability to output reports in a format such

that, if you have  CROSSDOS(tm) or  the public  domain program  MSH, the

logging programs  will generate  the reports ready for copying to an MS-

DOS formatted disk. Currently, you can do this only with  those programs

which generate  an arrl.report  file which is in ARRL Suggested Standard

File Format. Copy this file to an MS-DOS  formatted disk  and change the

name  so  that  it  consists  of  your  callsign followed by ".log". For

example, using crossdos, I use the command:

copy arrl.report pc0:ve5va.log]

(The square bracket on the end of the filename tells CROSSDOS to convert

the file  into MS-DOS  format -  with a  CR/LF at  the end  of each line

instead of just a LF as is used on the Amiga). Note that the name on the

MS-DOS  disk  MUST  have  your  callsign  as  its name and ".log" as its

extension. You then write the details of the contest on  the disk label-

your callsign,  contest name,  entry class and the date of the contest.

If you do this, then the only paperwork you have to fill out and send in

is the summary report file for the contest.

#[1z

3.1.7.2 %E Edit the Log

The command:

%E

causes the program to enter screen editing mode. It shows QSOs in blocks

of 16 at a time on the screen and you can edit the QSOs and move forward

or backwards through the log. When the program first enters edit mode it

always shows the last block of QSOs and the cursor  will be  pointing at

the callsign  of the  last QSO  in the  log. This enables you to rapidly

edit the last QSO,  if necessary.  The four  cursor keys  in combination

with  the  shift,  control  and  alt  keys  can  be  used to perform the

following functions:

- Cursor up

Moves the cursor up to the previous QSO, maintaining its  current X

coordinate on  the screen.  If the  cursor is on the top QSO on the

screen then the previous block of 16 QSOs will be  displayed unless

the program  is displaying  the first  16 QSOs in the log, in which

case nothing will happen.

- Alt Cursor up

Moves cursor back to a previous QSO which  contains '?' (indicating

a questionable callsign or a DUPE).

- Shift cursor up

Move back one QSO block if there is one.

- Control cursor up

Displays the first block and moves the cursor to the first QSO.

- Cursor down

Similar to  cursor up  functions. Moves cursor down one line on the

screen if not at the last QSO in the log.  If at  the last  line on

the screen then show the next block if there is one.

- Alt cursor down

Move cursor down to next QSO containing '?' if there is one.

- Shift cursor down

Move forward one QSO block if there is one.

- Control cursor down

Displays the last block and moves the cursor to the last QSO.

- Cursor left

Move cursor  one place  left. If  at the  beginning of the line, do

nothing.

- Shift cursor left

Move cursor to beginning of current line.

- Cursor right

Move cursor one place right. If at the end of line, do nothing.

#[1z

- Shift cursor right

Move cursor to end of current line.

The backspace key deletes the character to its left (i.e.  changes it to

a space).

Typing alphanumeric  characters or  the characters '/' or '.' or a space

types those characters  over  the  top  of  what  was  there  -  i.e. it

replaces them in the log.

If  you  edit  any  log  entry  and  are also backing up your log to the

printer, the program will write each corrected entry out to  the printer

preceded by  a line which says "*EDIT". This allows you to keep track of

everything that has been done to the log.

When you edit a log entry  it can  change the  score. You  can change as

many QSOs  in the  log as  you wish  but when  you terminate the editing

session the program rescores the whole log.

Typing Control-D terminates the  editing  function  and  returns  you to

logging mode (after rescoring the log if any changes have been made).

If you  edit any  QSO, then when you terminate the program, it will also

do a backup function (see next section).

3.1.7.3 %B Backup the memory log

If no filename follows the %B  command  then  the  current  disk  log is

copied to the file:

/backup/backup.PPP

where PPP is the abbreviation for the contest program you are using such

as wpx or wrp. Then it writes the in-memory log  into the  original disk

log.  Note  that  this  requires  that  the directory called "backup" is

present as it is provided on  the distribution  disk. If  you cannot put

the backup  directory on  the same  disk as the contest programs then an

environment variable  LOGBACKUP  must be  defined before  the program is

started. It  is best  to put a setenv command in your s:startup-sequence

file so that it is defined every time you reboot your system.  It can be

defined to  be in ram: (not very wise though!), or a directory on a hard

or floppy disk (much wiser).

If you specify a filename e.g. "%B work:log.backup" then the disk log is

copied to  that file before the memory log is copied to the current disk

log.

The program does an automatic backup when you  terminate the  program if

you have edited any QSOs at all.

#[1z

3.1.7.4 %F Changing the Frequency and mode

When the computer is not being used with a computer controlled radio the

displayed frequency and mode can be changed using the mouse or the stack

as previously described. You can also use a command of the form:

%F FREQ MODE

This  command  changes  the  displayed  frequency  to FREQ which must be

specified in kilohertz and  must not  contain commas.  The MODE  must be

present and is one of:

cw lsb usb am fm rtty

3.1.7.5 %S Score the log

The command  %S causes  the program  to rescore  the log and display any

changes that result. You should normally not need to do this since a new

QSO is  scored upon entry and if you edit the log it is also rescored if

necessary when the edit function is terminated.

3.1.7.6 %X Screen dump

The command %X creates a file of the  form screen.PPP  where PPP  is the

name  of  the  program  (e.g.  wpx)  and  the file will contain an ASCII

representation of what is on the screen at  the time  the %S  command is

given. This is more a debugging command than a feature.

3.2 Partial Lookup

When you type a callsign and then a space or tab, one of two routines is

performed. If the string in the typed callsign field looks  like a valid

call then  the program  does a  normal dupe of the callsign. But if what

has been typed is not  apparently  a  valid  callsign  then  the program

performs  a  partial  callsign  lookup  instead.  If what has been typed

consists  only  of  alphabetic  characters,  then  the  program searches

through the  suffixes of  every call and makes a list of any matches. If

what has been typed contains alphabetic and numeric  characters then all

of each  callsign in  the log  is searched.  The program  will count and

report the number of matches and  show up  to 20  of those  matches. The

program  starts  at  the  most  recent  entry  in  the  log and searches

backwards so that it is more likely to find and display recent dupes.

The display of partial calls is done in the same area as  is occupied by

the CW  memory display.  The CW  memory display will be redisplayed over

the partial calls if you type a delete character or when a  QSO has been

entered.

#[1z

3.3 AREXX Support (PacketCluster)

I have  added an  AREXX port to the contest programs and included on the

distribution   disk   the   public    domain   AREXX    library   called

rexxhost.library.  All  contest  programs  start up the AREXX port named

VE5VA_LOGGER so that  they  will  be  able  to  send  and  receive AREXX

commands. In  order to  use the  AREXX functions you must have a copy of

rexxhost.library in your LIBS:  directory.  If  you  do  not  have AREXX

running, or  do not  have a  copy of rexxhost.library, it doesn't matter

because the programs will ignore it if it isn't there. At the moment the

AREXX port is used to do a simple demonstration of PacketCluster support

which will be expanded  in  the  future  to  give  full  support  from a

separate  program  (like  the  computer  interface drivers). The trivial

AREXX program test.lgx shows how to  send  DX  messages  to  the contest

program.

When a DX spot arrives, the program first dupes it. If it is a dupe then

the DX spot message is ignored.  If it  is not  a dupe  then the program

displays the new DX info in the bottom right of the screen at the top of

a list of DX spots. As new ones are added the old  ones are  pushed down

so that the one displayed at the top of the list is the most recent one.

The program displays the most recent five DX  spots it  has received but

internally it actually stores up to 50.

As  you  work  stations  and  add  them to the log, each contest program

rechecks each DX spot to see if  it  is  still  not  a  dupe  or  if its

multiplier status has changed.

At the  moment, only the CQWW program colour codes the DX spots. A plain

colour means the spot is not a multiplier (but also not a  dupe). If the

spot is displayed in yellow then it is a new country. If it is displayed

in red then it is a new zone  (which implicitly  means a  new country as

well).

You do  not have  to respond  immediately to an incoming DX spot. If you

are busy, the spot can be left on the DX queue as long as  you don't let

the queue  get too  big. However, there are three control characters you

can type which affect the DX spots. Typing ^K deletes the DX spot at the

top of  the list.  Typing ^P  pushes the  top DX spot onto the stack and

then deletes it from the list. Typing ^G means "Go there right  now" and

the program  will change  to the  frequency of the DX spot and will also

copy the callsign into the data entry area at the bottom of  the screen.

You can  type ^K  or ^P  while you  are entering a callsign or other QSO

data.

Although not useful yet, the programs can also send AREXX  messages. The

@ command  typed in  the callsign  field specifies an AREXX port address

(similar to the AREXX ADDRESS command). A string following the & command

is sent to that port. For example:

@FANCYDEMO

&HELLO THERE - THIS IS A TEST

will send  the string  "HELLO THERE  - THIS IS A TEST" to the AREXX port

whose name is "FANCYDEMO".  At the  moment the  program only  uses upper

#[1z

case and therefore it can't be used to send to ports whose AREXX address

contains lowercase or which expects lowercase commands. This  ability to

send to an AREXX port will be used to send DX spots to a PacketCluster.

The addition  of the  AREXX port  will allow  other programmers to write

separate programs which can interface to  the logger  programs. The only

thing we  would need to cooperate on is the structure and content of the

AREXX commands and all details of the internal workings of  the programs

will be irrelevant.

3.4 Workbench Support

I  do  not  use  the  workbench  and  so  the original versions of these

programs would only work with the CLI. I  have now  added some  icons to

the programs so that they can be started up from the workbench. However,

you must still be able to edit the  default files,  and will  still need

the CLI to be able to print out the reports after the contest.

If a contest program is started up by clicking on its icon, then it uses

the contest name followed by the string ".log"  as the  name of  the log

file to  be used.  For example,  clicking on the IARU contest icon means

that the iaru program will use the log file named "iaru.log".

A driver can be started up  by clicking  on its  icon. There  will be no

output  or  window  created  by  the  driver  and so it will appear that

nothing has happened but it will be in the background.

Clicking on the stopdriver  icon will  cause the  driver to  close. This

should only be done after the contest program has been terminated.

3.5 Morse code memories.

There are  some special  codes that  can be  put in  a morse code memory

string which will be interpreted and  replaced at  the time  the code is

transmitted.

%N   Transmit the QSO serial number. Leading zeroes are transmitted as a

't', other embedded or trailing zeroes are snet as the  letter 'o'

and the number nine is sent as the letter 'n'.

%n    Transmit the QSO serial number as for %N but only leading zeroes

are changed to the letter 't'. No other replacement is done.

%c   This is replaced by the callsign which is entered on the QSO line.

%C   This transmits your callsign (the one entered with the 'm' command

in the .defaults file).

%R   This is replaced by the transmitted RST entered on the QSO line. It

is usually used only in the QRP contest where you  actually send an

RST other than 599!

#[1z

4. Support Files

All of  the contest  programs use  some auxiliary files. For contests in

which a DXCC country is a  multiplier, the  file dxcc.list  is used. The

CQWW program  uses both the dxcc.list and the file called wae.list which

contains a list of countries that  are in  the WAE  list but  not in the

DXCC list  (e.g. Sicily).  The FD  and SS programs use files to describe

the ARRL sections.

All of these  files  must  reside  in  the  same  directory  and  on the

distribution disk  they are in the directory called "lists". If you keep

this structure then you do not need to do anything to tell  logger where

to find  this directory.  If, for  some reason,  you must move the lists

directory somewhere else  then  you  can  tell  logger  where  it  is by

defining an  environment variable called LOGLISTS. For example you could

use:

setenv LOGLIST work:logger/lists

in your startup sequence.

4.1 The DXCC files

The  dxcc.list  and  wae.list  files  have  the  same  format  and their

structure is described in the comments at the beginning of the files.

The CQWW program reads both the dxcc and wae files. Several of the other

programs, such as ARLX, use the dxcc file.

It is very difficult for me to  keep  these  files  up  to  date.  It is

therefore essential  that you  ensure that  these files are correct when

using a DX contesting program.

Wildcards are allowed to  specify  a  range  of  matching  callsigns and

currently the wildcards are:

# matches a string of numeric digits.

$ matches a single numeric digit

@ matches a string of alphabetic characters

$ matches a single alphabetic character

* matches an alphanumeric string only

Country  prefixes  can  also  be  specified  as  a  range  when  this is

appropriate. For  example,  Canada  is  allocated  the  entire  block of

callsigns from  VAA through to VGZ. This can be entered in the dxcc.list

file as VA-VG (but NOT VAA-VGZ).  However, wildcards  are more efficient

if they  can be  used because  they cause  only one  prefix entry in the

internal country list. Thus, 4U#UN will  cause  only  one  entry  in the

table but  will match a large number of possible 4U calls. Whereas VA-VG

will generate seven entries, one for each of VA, VB, ...., VG.

The DXCC files contain entries for  CQ  and  ITU  zones.  These  MUST be

entered correctly and if the country being entered covers two or more of

these zones then ALL the zones must be entered. This is because  if only

#[1z

one zone  is entered,  then the  CQWW contest,  for example, will assume

that the country is entirely within one zone and will automatically fill

the zone  information into  the QSO  line during the contest. So, if the

country (e.g. the U.S.) covers more than one  zone, these  zones must be

entered so  that CQWW  will not fill in a zone at all. It is possible to

split up a country  into callsign  areas if  this also  splits them into

individual CQ  and/or ITU  zones. For example, although Canada is spread

across several ITU and CQ zones, the province  of Saskatchewan  (VE5) is

entirely contained within CQ zone 4 and ITU zone 3. An entry can be made

in the dxcc.list file which informs the program of this so that the CQWW

program can  fill in  the zone. See the dxcc.list file for an example of

how this is done. It is  more difficult  to do  the same  thing for U.S.

calls  because  their  callsign  areas do not necessarily reflect actual

geographic location. Thus a W6 need not be in California. It is  best to

leave the U.S. entries as they are.

Currently the  latitude and longitude are read in by the program but are

not used in any way. I  plan to  add a  beam heading  calculation to the

programs  at  some  point  so  you  should  ensure that the latitude and

longitude entries are correct.

4.2 ARRL sections (FD and SS)

The ARRL sections are kept in the file  ss.list. The  format is somewhat

different than  the country  lists since zones and lat/long are omitted.

The first column specifies the correct abbreviation for the ARRL section

and the  second column  specifies the full name of the section. The last

column specifies any of the variations  that  you  might  receive  in an

actual  contest.  You  can  type  in  what you hear and the program will

replace it with the  correct abbreviation  when you  enter the  QSO. For

example, if  someone sends  you the section "MISS", you can type this in

and the program will change it to "MS" when you  enter the  QSO. On SSB,

whenever you  hear the  words "North",  "East", "South" or "West", enter

them as a single letter with a space  after them.  So if  you hear "EAST

BAY" enter  it as "E BAY". It would help if you look through the ss.list

file before a contest to familiarize yourself with what it  considers to

be acceptable  abbreviations. You can, of course, add more abbreviations

if you wish. The only wildcard used in these files is the asterisk which

matches the remainder of the string.

One word  of caution.  The ARRL abbreviation for the Los Angeles section

is "LAX", whereas "LA" is the state of Louisiana. If  you type  "LA" for

the  Los  Angeles  section,  the  program  has no way to know that it is

incorrect and will score it as a QSO with Louisiana. On SSB,  it is best

to just  enter "LOS" for a Los Angeles QSO because the ss.list will take

care of it and change it to LAX.

4.3 QRP contest

The QRP ARCI contest uses states/provinces/countries as multipliers, but

the only  file it  uses at  the moment is the dxcc.list. This means that

you MUST  enter the  correct state  abbreviation. I  will eventually add

another file  to this  program so  that it  will do for states/provinces

what the ss.list file does for ARRL sections.

#[1z

4.4 RAC contest

This contest uses  two  files.  The  first,  rac.list,  contains  a file

formatted like  ss.list which allows mapping of strings into the correct

province abbreviation. For example, "QUEBEC" or "QUE" will  be mapped to

the correct  "PQ". The  other file is the dxcc.list and the program will

extract only  the  information  it  needs  to  resolve  VE  CY0  and CY9

prefixes.

4.5 SS contest

The sweepstakes  contest program  uses the ss.list as described above in

Section 4.2. Like the RAC program,  the  SS  program  also  requires the

dxcc.list file  to resolve  all   entries for Canada, the U.S.A. and its

possessions. This allows the SS program to detect when a DX callsign has

been entered. It will log, but not score, a DX contact.

4.6 IARU contest

The IARU  contest program uses the cqww.list just like the CQWW program.

When you do a report in the IARU program it produces the usual reports.

#[1z

5. LOG FILE STRUCTURE

No matter which contest program you use, the log file always consists of

lines of  80 characters  each, the 80th character being the linefeed and

the 79th character indicates which  contest  program  produced  the log.

This allows  you to edit the log file with an editing program instead of

using the editing function in the contest programs.  BUT be  warned that

if you  modify a  line, it MUST still be exactly 80 characters long when

you are done. You should use a TEXT editor to edit the file, NOT  a word

processor. Word  processors add their own hidden control codes in a file

which will totally confuse the logger programs.

The codes used at the end of the line are:

F    Field day      (FD)

X    CQ WPX         (WPX)

Q    QRP ARCI       (QRP)

A    ARRL DX        (ARLX)

C    CQWW DX        (CQWW)

E    DARC WAE       (WAE)

S    Sweepstakes    (SS)

R    Canada Day     (RAC)

I    IARU HF        (IARU)

As of V3.03, the very first  line of  the log  file is  always a comment

line  which  can  contain  the  callsign  of the operator and their zone

(which is  optional). The  logging programs  automatically generate this

line when  a new log is opened. The line must begin with a semicolon and

is then followed by a 'C' to indicate a callsign follows after  a space.

If a  zone is included, it occurs after the callsign and is indicated by

the presence of the letter 'Z' followed by a zone  number. For  the IARU

contest, this  should be an ITU zone, and for the CQWW contest it should

be a CQ zone. This line  must also  be 80  characters long  and have the

contest indicator at the end.

As an example, the beginning of the line may look like this:

;C VE5QST Z3

This indicates that the callsign used for the contest log was VE5QST and

that they are in zone 3 (ITU).

For many areas of the world, the contest  programs can  figure out which

zone the  callsign is  in and so the zone can be omitted in these cases.

However, for  U.S.  ops  in  particular,  the  programs  cannot uniquely

determine either  the ITU  or CQ zone from the callsign. So the operator

must  provide  this  zone  information  in  their  default   file.  This

information is then written into the contest log file.

If you  look at  the log  file you will notice that all the fields up to

and including  the callsign  are always  the same  size and  in the same

columns.  What  follows  after  the  callsign  depends  upon the contest

exchange but the received exchange follows the callsign.  After that are

#[1z

columns which,  depending upon  the contest,  show the contest exchange,

new multipliers and, finally, the score for that  QSO. If  the QSO  is a

dupe, or  the program  could not  recognize the callsign, then the score

field contains "?DUPE" or "?ERROR". The SS program also uses  ?DX if the

QSO was a DX contact.

6. KNOWN BUGS and SHORTCOMINGS

- There  is  currently  no  way  to  change  the  transmitted QSO serial

number. It is directly tied to the number of QSOs in the log.

- The Field Day program is very limited in which bands it can  score. At

the moment  it will  only handle  the HF bands from 160 to 10m excluding

the two valid WARC bands. Although it shows PKT and SAT as  bands on the

score display, there's no way for you to set those bands yet.

#[1z

7. The AUTHOR

I have  had chronic  fatigue syndrome  since September  of 1984 and have

been on long term disability ever since. These  programs are  the result

of very slowly working on them a bit at a time, when I am able, over the

course of over two years (plus some prodding from Cyril VE5CB). There is

no timetable  associated with  enhancements to  the programs.  If I feel

well enough to tackle something I  will  do  it.  If  I  don't,  then it

doesn't get done.

I  have  made  these  programs  available  for anonymous FTP on the site

ftp.usask.ca in the directory  pub/amiga/hamradio.  If  you  want  to be

notified of a new release then send your Internet address to me:

ve5va.qrp@usask.ca

The programs  may be  freely distributed - with the accent on FREELY. NO

CHARGE MAY BE MADE FOR DISTRIBUTING THESE PROGRAMS (other than  the cost

of  the  disk  and  a  reasonable  charge for postage) AND NO COMMERCIAL

DISTRIBUTION,  INCLUDING  THAT  BY  COMPANIES  WHO  SELL  COLLECTIONS OF

PROGRAMS, IS PERMITTED.

THE SOURCE CODE WILL NOT BE MADE AVAILABLE.

I RESERVE  TO MYSELF  THE RIGHT  TO SEND THESE PROGRAMS TO FRED FISH FOR

DISTRIBUTION. DO NOT SEND THEM TO HIM.

IF I FIND THAT THE ABOVE CONDITIONS  ARE  NOT  ADHERED  TO  THEN  I WILL

EITHER STOP  DEVELOPMENT AND/OR  DISTRIBUTION ENTIRELY, OR I WILL MODIFY

THE PROGRAMS SO THAT VARIOUS PARTS  OF THE  PROGRAM ARE  ENCODED AND YOU

WILL ONLY BE ABLE TO GET THE UPDATES FROM ME.

Thanks   to    Olaf   Barthel   for   his   completely   public   domain

rexxhost.library. The complete package is on Fred Fish disk #682

My thanks also go  to Ernie  VE5CJ who  made up  the icons  used in this

distribution.

#[1z

8. Contest Programs.

This section  is incomplete  - sorry. It describes the idiosyncrasies of

each contest program. The major difference between each  one, of course,

is the exchange. But some programs also require that certain information

be provided by the default files.

All programs require that the user's  callsign  be  set  in  the default

files unless it is already present in the log file.

8.1 IARU

This program will correctly score the contest for ops anywhere.

8.1.1 Default requirements

This program  differs in  one respect from the other programs in the way

in which it displays the CW memories. All other programs  display the 10

memories down  the right hand side of the screen. But there's not enough

space on the screen to show  the 90  IARU zone  multipliers with  the CW

memories displayed  this way.  So IARU  displays the  CW memories as two

groups of 5 side-by-side.

The program will normally determine your  ITU zone  (to put  in the log)

from your  callsign. However,  if you are in an area where your callsign

could be in one of several zones, then you must  use the  'z' command in

the  iaru.default  file  to  specify  your  ITU  zone.  The  'z' command

overrides the  zone  determined  from  the  call,  so  that  if  you are

operating out  of zone, but this is not indicated in your call, then you

must also use the 'z' command to tell the program which ITU zone you are

in.

If you  are operating an IARU HQ station (very unlikely, but I have done

it - VE5QST in 1993) then  you  must  use  the  default  'e'  command to

specify your HQ abbreviation. e.g. I used  'e rac'.

8.1.2 The exchange

You  should  enter  the  callsign  followed  by  a  tab. There is then a

received rst field which the program  automatically fills  in to  be 599

and it then skips over to the zone field. If you receive something other

than 599 you can back up the cursor and edit the field.

If the callsign falls within only  one ITU  zone, then  the program will

fill in the zone number for you. Normally it will be correct and you can

proceed to the next QSO by hitting carriage return. If  it is  wrong, or

if the  other station  is an  HQ station,   you  can delete the zone and

enter the received zone or HQ name. The zone field is 5  characters wide

so that it can accept the Hungarian's MRASZ.

The program  always stores  your transmitted  rst as  599 and during the

contest the only way to change this is  to use  the edit  function after

the QSO has been entered.

#[1z

8.2 CQWW

This program  will score  correctly for ops anywhere. It determines your

zone and continent from your  callsign  (or  from  the  default  file if

necessary - e.g. for U.S. ops who MUST use the 'z' command).

8.2.1 Default requirements

Like the  IARU program,  if your  CQ zone  cannot be uniquely determined

from your callsign, you must use the 'z' default command to specify your

correct CQ  zone. Make  sure that the zone displayed beside your call at

the top left of the screen is correct before proceeding. If it is wrong,

then you  must either  edit the zone stored on the first line of the log

file or delete the log file, put your zone in the default file and start

the program up again.

8.2.2 The exchange

The  CQWW  exchange  is  RS(T)  and  CQ  zone number. If the program can

determine a unique zone for the given callsign, it will fill in the zone

number for  you. Like the IARU, received RST is filled in as 599 but can

be edited but  the  transmitted  RST  is  stored  as  599  and  can only

bechanged  by  editing  the  QSO  after  it  is  entered  (using  the ^E

function).

8.2.3 DX spot multipliers.

The  CQWW  program  colour  codes  DX  spot  information.  All  programs

automatically  throw  away  spots  that  you  have already worked on the

specified band. If an incoming spot is not a new  multiplier then  it is

displayed  in  white.  If  it  is  a new country then it is displayed in

yellow. If it is a new zone (and  therefore also  a new  country), it is

displayed in red.

8.3 FD

This is the ARRL Field Day. I don't know the rules for the RSGB/European

Field Day.

8.3.1 Default requirements

You must use the 'e' command to specify your class  and section  so that

they can  be entered  in the  log. For  example, I  use 'e  1e sk'. This

information is used to  generate  a  properly  formatted  ARRL  log file

called "arrl.report".

8.3.2 The exchange

As you  enter the  received exchange,  the FD program can tell where the

class field ends because it consists of a  number followed  by a letter.

So when entering the FD exchange, you do not need to type a space or tab

between the class and section fields.

When entering the section,  the  program  uses  the  ss.list  to convert

#[1z

received  exchanges  into  standard  ones.  For  example, if you receive

'SASK' as a section, the program will store this as 'SK' in the log.

8.4 QRP

8.4.1 Default requirements

The QRP program can handle the ARCI and  MI-QRP contests.  You must tell

the program which one you are entering by specifying the 'q' command.

'q 0'  specifies ARCI and 'q 1' specifies MI-QRP. For ARCI you must also

specify your power output multiplier using the 'p'  command. I  run less

than one watt so I use 'p 10'. For powers greater than 1 watt use 'p 7'.

If you are in the MI-QRP contest then 'p' is forced to one. In ARCI, you

must  also  specify  a  power  bonus  multiplier  if it applies. If, for

example, you use a battery which has been charged by  natural power then

you can claim a 1,25 multiplier bonus, in which case you would specify

s 1.25

Finally,  ARCI  has  bonus  points  for homebrew rigs and your number of

homebrew bonus points is specified with the 'h' command. e.g. 'h 5000'.

The above defaults all affect your  score and  the program  cannot score

your entry correctly if you leave any of these out.

8.4.2 The exchange

The  ARCI  exchange  is  RST,  state/province/country  and  either  ARCI

membership number or RF output power  (for non-members).  At the moment,

you must  enter the  correct state abbreviation because the program does

not check it against a list  of  standard  abbreviations.  This  will be

added in the future.

8.5 WAE

This program  is not  complete. It does not know anything about QTCs. It

will not send, receive or score them. I plan to  add this  feature later

but it may be quite some time before I get to it.

8.6 SS

The Sweepstakes program is similar to the Field Day program.

8.6.1 Default requirements

You must  specify the  'e' command  in the  ss.defaults file to describe

your SS exchange excluding the serial number. For example, I use

e q ve5va 82 sk

The program uses this  information so  that it  can generate  a properly

formatted ARRL  output file for you if you are going to send the results

#[1z

on disk. The file is called arrl.report.

8.6.2 The exchange.

Like the FD program,  SS can  tell which  part of  the exchange  you are

entering and so it is not necessary to type a space between each part.

8.7 RAC

This  program  handles  both  the  Canada Day and Canada Winter contests

sponsored by the Radio Amateurs of/du Canada. It works for both Canadian

and non-Canadian stations.

8.7.1 Default requirements

Canadian ops  must use  the 'e'  command to specify the abbreviation for

their province. e.g.  "e SK" or "e PQ".

8.7.2 The exchange

The exchange from a Canadian op is RST and province  and from  any other

station it is RST and serial number.

8.8 ARLX

This is  the ARRL  DX contest  program. Unfortunately,  I do not have it

working for DX.

#c