SCSIMounter 1.0

===============

(Document Revision 0.41)

Removable media partition mounter for Kickstart 2.0 or higher.

------------------------------------------------------------------------

Disclaimer

----------

SCSIMounter may be freely distributed for non-commercial purposes only.

The entire risk as to the quality and performance of this program is

with you.

The author assumes no responsibility or liability whatsoever with

respect to your use or inability to use of this software.

------------------------------------------------------------------------

Purpose

-------

Most owners of removable media SCSI drives are stuck with the problem

that their software doesn't handle disk changes correctly.

For example, the current Commodore implementation doesn't mount the

partitions on a removable media SCSI drive (e.g. SyQuest or Ricoh)

when there was no cartridge in the drive at boot time.

Even worse, after another cartridge with a different Rigid Disk Block

(i.e. a different partition table) has been inserted, the system

gets totally confused and reports read/write errors when trying

to access the device which can lead to loss of important data on

the cartridge.

Until now, the only possibility to work around this problem was to

reboot the system every time a new cartridge has been inserted.

SCSIMounter allows you to mount and unmount partitions without rebooting

the machine. It sports a nice 2.0-style point-and-click user interface

that makes handling removable media a piece of cake.

------------------------------------------------------------------------

Secondary use

-------------

SCSIMounter can also be used to mount partitions that have intentionally

not been mounted (e.g. the wb_1.3: partition on an A3000 when running

under 2.0). You don't have to create a mountlist for it!

------------------------------------------------------------------------

Requirements

------------

� Any Amiga with a SCSI host adaptor that supports the Commodore

RDB (Rigid Disk Block) and 'SCSI Direct' standards.

� Kickstart 2.0 or higher

The program has been tested on an A3000 with Commodore's scsi.device

and on an A2500 with a Microbotics HardFrame (HardFrame.device) but

I see no reason why it shouldn't work on any other host adaptor.

! Since there are so many different host adaptors with devices that behave

! totally different, I need to know your experiences. Please don't hesitate

! to contact me in case of a problem with your controller card. I'll be

! glad to fix the problem and send you a revised version of SCSIMounter.

The program may be of no use for owners of certain GVP controllers

which support removable media correctly (Though not all GVP software

might do so).

------------------------------------------------------------------------

Before you begin

----------------

First, determine the name of the device driver of your hard disk

controller card.

Examples of popular controllers and their devices:

manufacturer/controller           device

--------------------------------------------------

Commodore A590/A2091/A3000        scsi.device

Old GVP                           scsidev.device

Supra SupraDrive                  harddisk.device

Advanced Storage Systems Nexus    nexus.device

Microbotics HardFrame             HardFrame.device

ICD AdSCSI                        icddisk.device

IVS Trumpcard Professional        ivs_scsi.device

--------------------------------------------------

Click once one the SCSIMounter icon and select 'Information' from the

'Icons' menu.

Add or change the following tool type:

DEVICE=Name_of_your_scsi_device_here

for example if you use a HardFrame controller write:

DEVICE=HardFrame.device

Don't add path names here, only the name of the device is required.

Take care to capitalize the device name correctly (exec device names

are case sensitive!)

NOTE

SCSIMounter will not work with the Commodore A2090(A) controller card

because this product does not support Commodore's own 'SCSI direct' and

'RDB' standards properly.

------------------------------------------------------------------------

Start SCSIMounter

-----------------

SCSIMounter can be used either from Workbench or from the CLI.

* Workbench use

Double click on the SCSIMounter icon. After a short time, a window

with a listview and three buttons will appear. If the window doesn't

appear several things could have gone wrong:

� Do you use Kickstart 2.0 or higher?

� Did you specify and spell the name of your device driver correctly?

� Does your host adaptor support RDB?

� Is there any memory left on your system?

Requesters will try to tell you the cause if SCSIMounter fails to run.

* CLI use

Type 'SCSIMounter Name_of_your_scsi_device'

example:

1> SCSIMounter HardFrame.device

If no argument is given, SCSIMounter uses the default A3000/A2091/A590

device driver (scsi.device)

------------------------------------------------------------------------

Panels

------

1) Main panel

The main panels shows all devices connected to the SCSI port with

information about their capacity and current status. The currently

selected unit is higlighted.

UNIT - This is the unit number of a SCSI device.

Up to seven devices can be connected to the SCSI bus

DRIVE TYPE - Manufacturer and product specification

REV. - Revision of Hardware, Firmware and ROM of device

CAP. - Device capacity in Megabytes or Kilobytes

STATUS - Current Status of device. If the device is busy or

not accessable due to the lack of media (if it's a

removable media drive) the status will be set to

'not ready'

There are three 'action buttons' present:

QUIT - Quit SCSIMounter

UPDATE - Re-scan SCSI bus and update display

MOUNT - Mount partitions on selected drive

For every action buttons there's also a menu item with identical

function.

2) 'Partition Selector' panel

This panel shows all available partitions and allows you to

individually mount them.

CHECKGADGETS - Determine if the particular partition should

be mounted or not.

checked: mount corresponding partition

not checked: don't mount specified partition

Other partition Information on the partition selector -

Device name of partitions

Size of partitions

Dostype (FileSystem) of partition

Examples of dostypes:

DOS0 - Old filesystem partition

DOS1 - FastFileSystem partition

UNI1 - Unix boot partition

RESV - Reserved partition

CANCEL - Go back to main panel, don't mount anything

MOUNT - Unmount all currently mounted partitions on the selected

device and mount the new partitions (if any).

UNMOUNT ALL - If this gadget is checked, SCSIMounter will try

to unmount all partitions currently mounted on the

selected unit. If there are locks on this device

there will be a 'BUSY' icon on your Workbench screen.

------------------------------------------------------------------------

Operation

---------

SCSIMounter is pretty self-explanatory. Nevertheless, here are some

basic operation procedures.

* Mount a new cartridge

� Click on the listview entry of the desired device. (Make sure its

status is 'ready')

� Click on the 'Mount' button

The 'select partitions' panel will ask you which partitions to mount

� Chosse the partitions you would like to mount

� Make sure that the 'Unmount all' gadget is checked

� Click on 'Mount' button on the partition selector panel

SCSIMounter will automatically unmount all existing partitions

on the device and mount the new partitions.

* Add a drive

� Click on the 'Update' button

The new drive's name and other information  should appear in the

listview

� Click on the corresponding entry

� Click on the 'Mount' button

and so on...

WARNING !!!

Don't unmount your boot partition (where you system files live) or

you'll soon be forced to reboot you machine...

------------------------------------------------------------------------

Keyboard Equivalents

--------------------

* Requesters

OK             Rt. Amiga-O

Cancel         Rt. Amiga-C

* Main window & Project menu

About          Rt. Amiga-A

Mount          Rt. Amiga-M

Update         Rt. Amiga-U

Quit           Rt. Amiga-Q

* Partition Selector

Mount          Rt. Amiga-M

Cancel         Rt. Amiga-C

Select All     Rt. Amiga-A

Unselect All   Rt. Amiga-U

------------------------------------------------------------------------

Known Problems

--------------

� The program only removes the dos entries for the volumes and devices.

It doesn't remove the filesystem process and associated memory

allocations. Apparently the current FS doesn't support the ACTION_DIE

packet. I'm still looking for a legal way to solve this problem.

------------------------------------------------------------------------

Credits

-------

Author: Martin A. Blatter

Pfaff�cherstr. 59

CH-8913 Ottenbach

Switzerland

E-Mail: blatter@amiga.physik.unizh.ch

blatter%zethos@cbmswi.commodore.com

Phone:  ++41 1 761 20 02

Bug reports or suggestions are welcome. If you find this program useful,

feel free to send a donation to the above address.

This program uses ITools, the object-oriented user interface system

(c) 1991 by Christian A. Weber.

Special thanks to  � Christian A. Weber

� Peter Simeon for the SCSI inquiry code

� Michael Sinz for the 2.0 Workbench busy image :-)

� Eric Giguere

------------------------------------------------------------------------

Trademarks

----------

All trademarked product and company names used herein are used for

identification purposes only. No infringement of trademarks is intended.

Trademarks remain property of the trademark holder.