GRPERF v1.1

AutoCAD(tm) Video Graphics Performance Benchmark Suite

Copyright 1990 Off-Broadway Business Systems

(All Rights Reserved)

Portions Copyright 1990 A/E Automation Systems

AutoCAD and DXF are registered trademarks of Autodesk,

Inc.

GRPERF is a benchmark suite for measuring the relative performance

of graphics display hardware and drivers used in conjunction with

AutoCAD(tm), Release 11 and Release 10.

The tests performed by GRPERF are very detailed and are designed to

measure the draw performance of various types of geometry, and to

identify certain types of geometry which a driver or display board

is more capable of redrawing faster or slower than other types.

This benchmark was developed primarily for evaluating the "raw"

performance of hardware and video drivers, and for those involved

in developing same.  As a user, you may not find the results of

GRPERF to be very meaningful in terms of the kind of work you do,

but it will prove to be useful in evaluating the relative

performance of different display boards and display- list

processing ADI drivers.

Redraw tests.

There are nine tests performed by GRPERF.  Each test is composed of

a vector image that is exactly 1 x 1 drawing units in size.  The

redraw time of this image is measured at nine different display

magnifications, starting with a magnification that displays the 1

x 1 test image at 25 screen pixels, which doubles with each

successive magnification, ending with an image that is 256x larger

(1600 screen pixels) than the initial one.

At each of the nine display magnifications five redraws are

performed on the test image, and the time required to perform each

redraw is recorded.  Note that this time includes the time required

to first clear the display, as this operation is itself intrinsic

to the REDRAW operation.

For each of the five redraw times that are obtained at each

magnification, the highest and lowest times are discarded and the

three remaining times are then averaged together to produce a

single redraw time for each of the nine test magnifications.

Note that all times are only accurate to +/- 0.1 seconds. Once the

test is complete, the average redraw time of each of the nine

magnifications is then averaged together again, to produce a single

redraw time for the entire test.

Note that the test is designed to show variations in redraw time at

various different points, starting at one where the test image is

entirely within the view, and proceeding to a point where the view

contains only a portion of the image. You will probably note a

change in the redraw time at the point where the test image exceeds

the extents of the view, requiring the test image to be clipped to

the edge of the view.  This can give you a rough indication of the

performance of a display controller or an ADI driver's "trivial"

acceptance, rejection and line-clipping algorithms.

Benchmark performance times.

GRPERF contains built-in benchmark times for each

test/magnification as well as average times for all magnifications,

for all tests.  These times reflect the use of GRPERF with AutoCAD

Release 10 on a 25 Mhz. 386 PC with a standard VGA card running at

640 x 480, using the stock Autodesk-supplied VGA driver, with the

right-side SCREEN menu enabled, and where all redraw activity was

limited entirely to cached 32-bit system RAM without requiring any

disk I/O.

The benchmark reference times scale the performance of the system

on which GRPERF is run to provide a performance indicator or

"index" relative to the benchmark system performance as described

above.  This relative performance factor indicates the redraw

performance of the system being tested relative to the benchmark

system, where a value of 1.0 is equal to the performance of the

benchmark system, and 2.0 is twice as fast as the benchmark system.

Note that comparing a higher-resolution display configuration or

one with the AutoCAD SCREEN menu disabled (effectively increasing

the number of pixels in the view) may not be very meaningful in

terms comparing those results with other results using a different

display resolution.  In order to make a fair comparison between any

two test results, the dimensions of the view in screen pixels must

be exactly the same for all tests whose results are compared.

The redraw tests should be performed on a system with enough RAM to

maintain the test drawing's vector display list ENTIRELY in RAM,

without paging any portion of it to disk.  If a disk access is

required during the redraw test, then the test results should be

considered invalid and entirely meaningless.

Running GRPERF.

To execute GRPERF, place all files from the archives into a

directory in the, AutoCAD library search path, and REMOVE any

ACAD.LSP file so that it will not be loaded Automatically by

AutoCAD.  DO NOT attempt to run GRPERF with an any ACAD.LSP file

present in a path which permits it to be automatically loaded!

Then, at the DOS PROMPT:

C:\ACAD > GRPERF [<redraw alias>]

Where the optional parameter <redraw alias> is the command that

initiates an AutoCAD or ADI driver redraw.  If your ADI display

driver uses the AutoCAD REDRAW command, then there is no need to

specify this option.  You will then be placed in the AutoCAD

drawing editor.  At this point, you can select one of the nine

tests by pulling down the left-most item on the pull-down menu:

�����Ŀ

�Tests�  Results   Options

�����������������������Ŀ

� Orthogonal lines      �

� Orthogonal linetypes  �

� Orthogonal polylines  �

� Random lines          �

� Random polylines      �

� Orthogonal text       �

� Rotated text          �

� Solid flood           �

� Circles               �

� --------------------- �

� Run ALL tests         �

�������������������������

After you select a test, the drawing editor will be exited and re-

initialized and the test geometry will be generated.  If you have

not provided a hardware description for the test (a description of

the system or configuration being tested, in 30 characters or less,

which is included on the graph and tabular results), then you will

be prompted to specify this once. You can also change this

description by selecting the appropriate item from the "options"

pull-down menu.  The test description is stored in the file

GRCONFIG.TXT, and it will be used until you explicitly change it,

or erase the file.

After each test is complete, GRPERF displays the detailed results

in tabular form.  These results will also be posted in the file

GRPERF.LOG automatically for future reference.

Graphing test results.

After performing a test, GRPERF can draw a bar-graph representation

of the tabular results.  You can do this by selecting "graph test

results" from the second pulldown menu titled "results".  Once the

graph is generated you can then save it as either an AutoCAD

drawing file, or a slide, by selecting the appropriate item from

the "results" pulldown menu.

Graph drawings and slides are saved in filenames named "TESTn"

where 'n' is a number from 1 to 9 indicating the test number.  The

test numbers are the same as the order in which the corresponding

tests appear in the pulldown menu.

Running all tests in batch mode.

You can also have GRPERF run all nine tests sequentially and

automatically by selecting "run all tests" from the "tests" pull-

down menu.  When running all tests in batch mode, GRPERF

automatically generates the graph of each result and saves it as a

drawing file named in accordance with the conventions noted above.

It also posts tabular results from each test in the file GRPERF.LOG

When running all tests in batch mode GRPERF will also compute the

average redraw time and relative performance factor (VGA @ 25 Mhz

= 1.0) for each test and generate a bar graph of all test results,

and save the graph in the drawing file SUMMARY.DWG.

Altering test data.

Please note that the drawing and DXF(tm) files included with GRPERF

are not to be modified in any way.  These are the drawings which

are used to generate the test geometry.  They are named TEST1.DXF

thru TEST9.DXF and GRPERF.DWG, GR1.DWG, and GR2.DWG.  These .DWG

and .DXF files should NEVER BE MODIFIED for any reason whatsoever!

If they are, then GRPERF will not run, and no results that is

produced if it does run is valid.

A word about benchmarks.

All benchmarks can be somewhat deceptive, and accurate

representation of the true performance of any single aspect of two

dissimilar systems is extremely difficult to measure.  In addition,

results do not take into consideration a specific type of drawing

or discipline which can have a tremendous influence on the

effective performance that is achieved while using AutoCAD for

serious work, as opposed to running benchmarks.

GRPERF is primarily intended to help evaluate the 'raw' performance

aspects of graphics hardware and drivers, and is not designed to be

very meaningful as in indicator of performance as it may relate to

an end user and/or his or her specific application.  The tests

results can be easily misinterpreted due to the possibility of

their being influenced by many other factors other than video

performance which is almost impossible to compensate for.

In short, results should be never be considered as an accurate

indicator or reference standard by which the performance of any two

or more systems can be measured.  The use of this benchmark should

be limited to comparing AutoCAD graphics performance of different

graphics hardware or ADI drivers installed in the same system with

all other variables being precisely equal.

If you would like to compare the AutoCAD vector drawing performance

of two or more different video boards running at the same

resolution and with the same ADI display driver, then you should

swap the boards into the same system and run the benchmark on each

of them the same system, making sure that all other variables have

been eliminated.

Do not run the benchmark on different systems with different

graphics cards or drivers and expect the test results to be an

accurate indication of the raw performance of the hardware or the

drivers in each system.

Robert Schutz

Off Broadway

TEL 415/547-8801

FAX 415/547-2637

Compuserve  74435,16

Tony Tanzillo

A/E Automation Systems

Compuserve 71241,2067