1.0) Introduction

This file describes how to setup and run the classroom versions of

PSPICE and PROBE.  You are welcome to make as many copies as you

find convenient of all the files on the Classroom Version PSpice

diskette.  You may run the classroom version of PSpice on as

many different PC's as you wish.

This diskette came from a desire to respond to many requests we

have received for using PSPICE for teaching electronics courses.

Since the commercial version of PSPICE and PROBE are $950 and $450,

respectively, cost was a problem when trying to provide the program

to an entire class. From discussions we have had with professors it

appeared that for many situations a restriction on the circuit size of

10 transistors was acceptable.  That restriction allows us to offer

this version of PSPICE to all electronics professors in the United

States at no charge.  We hope that it will be useful to you.

2.0) System Configuration

PSPICE will run on an IBM PC, XT, or AT with 512kbytes of memory.

The floating-point co-processor (8087 or 80287) is optional.  If present,

the program will run at full speed.  Otherwise it will run 5-15 times

slower.  Either the monochrome or color display may be used.  Any

printer may be used.

PSPICE runs under MSDOS 2.0+ and requires the system to have been

booted with a CONFIG.SYS file which contains the statement:

FILES=10

It is also recommended that the CONFIG.SYS file contain the statement:

BUFFERS=10

Such a CONFIG.SYS file is included on Diskette 1.

For PROBE the only difference is that the floating-point processor

is not optional.  It is required.

3.0) Running PSPICE

PSPICE is 2 programs: PSPICE1.EXE (on Diskette 1) and PSPICE2.EXE (on

Diskette 2).  For systems with a fixed disk, simply copy the all the

files into a directory and then start PSPICE with the command:

PSPICE input-file output-file

For PC systems, you must run PSPICE1.EXE and then PSPICE2.EXE.  You can

either use one drive for the program diskettes, switching them between

PSPICE1.EXE and PSPICE2.EXE, or use 2 drives, putting one program

diskette in each.  In the 1st case, the input, output, and scratch

files would go on the 2nd drive.  In the 2nd case, the files would go

on the drive which has diskette PSPICE1.  For both cases regarding the PC,

the input and output files are specified when PSPICE1.EXE is run:

PSPICE1 input-file output-file

For all the above cases, the input-file and output-file may have

pathnames.  The program files, too, can be on any directory specified

by a previous DOS PATH command.  Note that PSPICE1.EXE does not call

or overlay or chain to PSPICE2.EXE.  PSPICE2.EXE must be explicitly

run.  See PSPICE.BAT for an example.

The input file must be specified, but its extension need not be.  Its

extension defaults to .CIR.  The output file is optional, its name

defaults to the input file's name and its extension defaults to .OUT.

If the input file is not specified you will be prompted for it.  The

output file may be a device, such as the printer.  This command would

run EXAMPLE1 to the printer:

PSPICE EXAMPLE1 PRN

assuming that you have the standard DOS reserved names.

PSPICE1.EXE creates 2 temporary files, PSPICEA.TMP and PSPICEB.TMP,

which are passed on to PSPICE2.EXE.  PSPICE2.EXE creates 2 more

temporary files, T3.TMP and T4.TMP.  At normal program termination of

PSPICE2.EXE all 4 temporary files are deleted.  For a long run their

combined size will be about 40kbytes.

4.0) Format of the Input and Output Files

The input and output files are ordinary text files and follow the

same rules as those for the UC Berkeley Spice program, version 2G.6,

with these omissions:

1) There is no distortion (.DISTO) analysis.  We recommend using the

.TRAN and .FOUR to calculate harmonic distortion.  This method

correctly accounts for clipping, which .DISTO does not.

2) There is no .ALTER command.

In addition, numerous enhancements to the Berkeley Spice have been

added including GaAs MESFET devices, group delay, and the ability to

reference device libraries in other files.  For more details on these

and other PSPICE enhancements, the PSpice User's Guide may be

purchased from MicroSim for $40.00.  Unfortunately, the user's guide

is not available in machine-readable format.  It is typeset and

contains both formulas and diagrams.

5.0) Restrictions for the classroom version

This version of PSpice will run in 512kbytes.  It will run with or

without the floating-point co-processor.  All the features of the

production PSpice as of May 1986 are included except that the circuit

size is restricted to a maximum of about 10 transistors.  The production

version of PSpice comes with a library of models for about 100 standard

devices (diodes, bipolar transistors such as the 2N2222, power MOSFET's,

opamps, and comparators).  This diskette includes a reduced version of

the standard parts libraries.

6.0) Running Probe

This diskette also contains Probe, the graphics post-processor for PSpice.

Probe is run by adding the statement:

.PROBE

to the input file to PSpice.  Then, PSpice stores all the node voltages

and device currents calculated during the simulation into a file called

PROBE.DAT.  The presence of the PROBE.DAT file will cause Probe to be

run automatically as soon as PSpice is finished.  In addition, once

the PROBE.DAT file is created, Probe can be run by itself (that is,

without re-running PSpice) by typing

PROBE

To run Probe the first time, we recommend adding a ".PROBE" statement

to EXAMPLE1.CIR with a text editor and then typing:

PSPICE EXAMPLE1

PSpice will simulate EXAMPLE1 and create a PROBE.DAT file.  After

PSpice is finished Probe will start up automatically.

Probe accepts commands through a menu displayed at the bottom of the

screen.  Most of these commands are self-explanatory.  Note that the

Add Trace command allows you to enter arithmetic expressions of voltages

and currents as well as the voltages and currents themselves.

The PROBE.DEV file contains a list of the devices attached to your

system.  The allowed devices for "Display = " are:

Text:          Non-graphics display

IBM:           IBM CGA (640x200 no color) adaptor

IBMEGA:        IBM EGA (640x350 with color) adaptor

Hercules:      Hercules graphics (720x348 no color) adaptor

FutureNet:     DASH2 graphics (640x350 no color) adaptor

The allowed devices for "Hard-copy = " are:

Text:          Non-graphics printer

Text132:       Non-graphics printer (132 columns)

Epson:         Epson FX-80 printer

Epson132:      Epson FX-100 printer

EpsonMX:       Epson RX and MX-80, Okidata IBM Compatible, Okidata

with Plug N' Play, and Epson-compatible printers

EpsonMX132:    Epson RX and MX-100 printers

Okidata:       Okidata ML92 printer

Okidata132:    Okidata ML93 printer

IBMClr:        IBM color printer (black only)

IBMClr132:     IBM color printer (black only, 132 columns)

IBMClrSlw:     IBM color printer (color)

IBMClrSlw132:  IBM color printer (color, 132 columns)

CItoh:         C. Itoh color printer (black only)

CItoh132:      C. Itoh color printer (black only, 132 columns)

CItohSlw:      C. Itoh color printer (color)

CItohSlw132:   C. Itoh color printer (color, 132 columns)

HP             Hewlett Packard 7470 and 7475 pen plotter

HI             Houston Instruments DMP pen plotter

HPLJ           Hewlett Packard LaserJet printer

We recommend that you use "Text" first before trying out your system's

graphic devices.  Note: this classroom version of Probe is for IBM and

IBM-compatible PC's only.  It will not run on the Texas Instruments

professional.

This version of Probe requires the 8087 floating-point co-processor.

Unlike PSpice it will not run without it.  This version of Probe has

all the capabilities of the production Probe as of May 1986.

7.0) Device Libraries

The classroom version of PSpice comes with models for some standard

parts.  These are found in the files with extension ".LIB".  These

are ascii, text files and we recommend that you print them out and

read them, starting with the file NOM.LIB.

Libraries may be referenced by using the statement:

.LIB <file name>

in the .CIR file.  See NOM.LIB for examples.  If <file name> is left

out then it defaults to "NOM.LIB".  If any device in the .CIR

file references a model or subcircuit that is not defined in the .CIR

file then PSPICE will search all files referenced by .LIB statements

and use .MODEL or .SUBCKT statements found in the library files.

8.0) More Information

Here are some commonly asked prices:

PSPICE (commercial version)                   $950 (quantity one)

PROBE  (commercial version)                   $450 (quantity one)

PARTS  (model parameter extractor)            $450 (quantity one)

User's Guide for PSPICE, PROBE, and PARTS      $40 (all quantities)

For more information please contact:

MicroSim Corporation

23175 La Cadena Dr.

Laguna Hills, CA  92653

(714) 770-3022

Our level of technical support for the commercial version of PSPICE is

quite high.  Unfortunately, we cannot provide the same kind of service

for the classroom version due to the large number of users and the fact

that it is provided at no charge.