RS232 TO MIDI CONVERTER           23-Jan-89

-----------------------

1. Introduction

The  following notes provide details on building an RS232 to MIDI

interface.  The  only  requirement is that the computer  have  an

RS232 port that can be programmed to operate at 31.25 Kbaud.

2. MIDI

Many  electronic  music  instruments are now fitted with  a  MIDI

(Musical   Instrument  Digital  Interface)  port.   This   allows

instruments  to control (or be controlled) by other  instruments.

A computer fitted with a MIDI port makes an excellent  instrument

controller.  Some  of the applications being - a MIDI multi-track

recorder/sequencer  or  a  voice  editing  program  for  a   MIDI

synthesizer.

MIDI is actually very similar to the RS232 serial  standard.  The

only differences being that MIDI operates at 31.25 Kbaud and uses

a 5 mA current loop rather than voltage levels.  It uses an 8 bit

word, 1 stop bit and no parity. Provided that your computer has a

serial interface that can be programmed to operate at 31.25 Kbaud

then the only other need is for a simple RS232 to MIDI interface.

3. MIDI Interface

The  interface features both MIDI IN and MIDI OUT ports and  uses

readily  available  components.  The  circuit has  been  designed

specifically  to  use  a  cheap  opto-isolator  rather  than  the

expensive  (and  often  difficult to obtain)  6N138  or  HCPL2730

usually  specified.  To simplify matters,  0v instead of -12v  is

used  as the negative RS232 output level.  This should not  be  a

problem  as most computers use the 1489 receiver chip which  will

operate  with 0v input.  Power requirements for the interface  is

modest.  Either  +12v or +5v (see circuit diagram) is  obtainable

in most instances from the host computer's RS232 port.

To  see  the circuit diagram of the interface,  simply  send  the

ascii file MIDI-CNV.CIR to your screen or printer.

4. Connections

After  building  the  interface,  check  the  wiring  thoroughly.

Incorrect  connections are likely damage either computer or  MIDI

instrument.

Connect  the  interface  between  a spare  serial  port  on  your

computer  (RS232  channel  B  in the case of  the  Ampro  Z80  or

Bondwell 14) and the MIDI instrument.  Some computers may require

the  connections going to pins 2 and 3 of the RS232 connector  be

reversed.

Computer                 Interface                MIDI Instrument

RS232              RS232          MIDI              MIDI

-----,             ,--------------------,          ,--------

|             |             OUT  4 | -------- | 4 IN

TX 2 | ----------- | 2                  |          |

|             |             OUT  5 | -------- | 5 IN

|             |                    |          |

|             |                    |          |

|             |              IN  4 | -------- | 4 OUT

RX 3 | ----------- | 3                  |          |

|             |              IN  5 | -------- | 5 OUT

|             |                    |          |

7 | ----------- | 7--,               |          '--------

|             |    |               |

-----'             '----|-----|---------'

|     |

gnd  +12v

Note - A MIDI cord consists of a two-wire cable with a 5 pin  180

degree  DIN audio connector on each end.  Pin 4 connects to pin 4

and pin 5 connects to pin 5. Two such cords will be required.

5. Testing

Once  you  have built the interface,  it can be tested  with  the

simple Basic program provided.  There are versions of the program

for the Ampro Z80 Little Board and the Bondwell 14.  If using the

Bondwell  you  will need to make a modification to  the  computer

first. See note elsewhere.

If  you  have a computer other than these then you will  have  to

modify  the  program  accordingly.  The main  requirement  is  to

correctly  initialise  your RS232 port for 31.25  Kbaud,  8  data

bits, 1 stop bit and no parity.

The  program itself does little more than send a note to the MIDI

instrument  when a key on the computer keyboard is  pressed.  The

pitch of the note varies depending on which key has been  struck.

Make  sure  that  your  MIDI instrument is set  receive  on  MIDI

channel 1 (there are 16 possible MIDI channels).

Well that's it. Once everything checks out you are ready to write

some  good MIDI software.  If you do,  it is a good idea to  keep

your hardware specific code (eg. serial port initialisation) in a

separate module.  This will allow others with different  machines

to get your code running with a minimum of trouble.

6. Bondwell 14 Modifications

The  following modification to the Bondwell allows Serial Port  B

to  be  used  for MIDI applications while leaving Serial  Port  A

unchanged  and  usable  for  normal  RS232  applications  (modem,

printer etc). The modification is simple and does not require the

cutting any tracks.

The  baud rate generator in the Bondwell uses a 1.8432 Mhz  clock

to  derive  all the standard RS232 baud rates.  To  generate  the

required  baud rate for MIDI (31.25 Kbaud) a clock  frequency  of

500  Khz  or  a multiple  thereof  is  needed.  The  modification

disconnects  the 1.8432 Mhz clock from 8253 timer 1 and  replaces

it  with  a 2 Mhz signal.  Timer 1 can then be programmed  for  a

division ratio of 4 giving the required 500 Khz.

a) Remove  top cover and locate 8253 timer chip.  Remove it  from

its socket.

b) Obtain a 24 pin IC socket which has flat pins.  Bend pin 15 of

the socket upwards. Plug the socket into the socket vacated by

the 8253.  Ensure that the bent pin is not in contact with pin

15 of the motherboard socket.

c) Obtain  a  length  of wire-wrap wire.  Solder one end  to  the

upturned socket pin. Solder the other end of the wire to pin 6

of IC39 (74LS04).

d) Insert 8253 into piggy-backed socket and replace cover.

---------------------

####################################################################################################