CPM3.DOC - Installing ConIX V22.12 under CP/M 3.0

-------------------------------------------------

The  ConIX Operating System provided on the distribution diskette

was  designed to function optimally under the  CP/M  version  2.2

operating  system.   If you are installing ConIX under CP/M  Plus

version 3.0, there are a few general steps you should follow.

INSTALL.COM  - This  program will install ConIX on  your  system.

CP/M  3  users will probably have to disable at least BDOS  error

vector  patching in order to get ConIX to work at all.   You  may

also  have  to  disable the ExpanDisk feature  or  BIOS  patching

entirely.   What  must be disabled varies from system to  system.

There  is  no  real scientific way of knowing what  can  be  left

enabled  and what should be disabled.   Only trial and error will

tell for sure.

CPM3CCPR.COM - This program generates a replacement CCP.COM which

you can install into your system.  This file is not ConIX itself,

but simply a loader which will insure that ConIX is automatically

loaded each time you cold boot your system.  We felt it necessary

to  provide this program since the prescribed  auto-boot  patches

often will not work under CP/M 3.

When   you  run  CPM3CCPR,   a  new  file  CPM3CCP.COM  will   be

automatically generated.   This new file is pre-relocated to load

at the top of TPA memory.    Therefore,  before running CPM3CCPR,

make  sure that system memory is organized as it will appear when

ConIX  itself  is  running  (namely,   all  TPA  region   patches

installed, if any).

To  install  the new CCP,  copy your current CCP  to  OLDCCP.COM,

erase CCP.COM, and then copy CPM3CCP.COM to CCP.COM.  Reboot your

system to have the new CCP load.

CPM3CCP  has  a  few nice features.   Firstly,  when  CPM3CCP  is

invoked,  it will look for the presence of a file A:NOBOOTCX.SYS.

If  found,  OLDCCP.COM  will be loaded and  executed  instead  of

CONIX.COM.  If ConIX is to be loaded, a message will be displayed

while it is being read off disk.   You may abort to OLDCCP.COM by

simply hitting CTRL-X.   Remember,  if you BYE out of ConIX,  you

will  get CPM3CCP.COM which will then throw you right back  where

you came from.  If you need the CP/M 3 CCP, be ready on the CTRL-

X key after running BYE.

CPM3FIX.XCC/COM  - Some  implementations  of CP/M 3  might  cause

trouble as far as BDOS error messages are concerned.  The symptom

is  flakey operation or a downright crash after a CP/M BDOS  disk

error message is printed.   If this happens,  you'll have to  run

CPM3FIX each time ConIX is cold loaded into your system.

The  best  way  to  do this is to use MENU to set  a  program  to

execute  automatically when PROFILE finishes setting  the  system

configurations.   The command [J] CPM3FIX should do fine.  If you

are already executing another Command Language  program,  execute

CPM3FIX from within it.

CP/M 3 RSX FEATURE - Since ConIX was written for CP/M 2.2,  there

is currently no support for Resident System Extension invocations

from  under ConIX.   Any RSX's which are to stay  permanently  in

memory should be loaded before ConIX is cold started.

#ll then throw you right back  where

you came from.  If you need the CP/M 3 CCP,