Super8

o  The Super8 is a new family of fast 8-bit microprocessors  from

Zilog.  Some of the features include:

o  325  internal registers.  272 are general purpose and  53  are

used  as  mode and control registers.  Two  "windows"  of  any

eight registers can be assigned as the "working registers"  at

any time.  These can be accessed very quickly using short form

instructions,  though  all registers can be  accessed  at  any

time.

o  Full  duplex UART with built in baud rate generator.  Supports

data speeds to 2.5 MB.

o  Up to 32 bit programmable I/O lines, w/ 2 handshake channels.

o  Interrupts from 27 possible internal and external sources map-

ped into 16 different vectors which can be assigned 8  differ-

ent priority levels.

o  Clocks  at 20 MHz; some instructions execute in as  little  as

600 nS (0.6 uS).

o  Special  fast interrupt mode will service an interrupt in  0.6

uS.   (This  mode also has the interesting  characteristic  of

executing different consecutive blocks of code each interrupt;

more conventional interrupt operation is also possible).

o  Addresses up to 128k of external memory in two 64k banks (data

and program) if desired.  Depending on the memory requirements

of  the system,  pins not needed as address lines can  be  as-

signed  as additional I/O lines.  (An 8k mask ROM  version  of

the chip is available for use in dedicated controller applica-

tions; however the external memory version is probably of more

interest to the hobbyist).

o  Hardware Multiply and Divide instructions.

o  Other unusual instructions including nybble swap instructions,

single  "jump on bit test" instructions, and special  "enter",

"exit" and "next" instructions.  These directly support higher

level threaded-code languages such as Forth, using a  register

called  the  "IP" (Instruction Pointer), a register  which  is

somewhat  analogous to the Program Counter in the  context  of

the high level language environment.

o  On chip oscillator directly drives crystal.

o  Two built-in counter/timers.

o  Built in DMA channel to allow high speed data transfers.

o  ==> Cost less than $5.00.

Zilog has been offering a development kit for $88 in  conjunction

with  a promotional contest they are running (1st prize for  best

design:  $5000). The kit comes complete with a set of  data  man-

uals, a cross assembler which will run on an IBM PC, and a devel-

opment board which you hook up to your PC's serial port.  On  the

board  is ZO880020 uP (48 pin DIP, 20 Mhz, no internal  ROM  ver-

sion),  an  EPROM with a debug monitor and RAM to  download  your

program to. Provision is made for a connector to pick up all  the

I/O  lines.  The only other chips needed are 3 TTL  "glue"  chips

and an RS-232 driver (the monitor makes use of the on board  UART

to perform communication with the host PC).

Not content with this, Jeff Wilson, an associate of mine, decided

to  write a CP/M (Z-80 only) cross assembler which will  run  far

faster  than the MS-DOS one supplied by Zilog.  Though the  final

product  is  to be commercial, Jeff has generously made  a  major

portion  of the assembler available for no charge  to  interested

parties.  He has also re-written the monitor, making  substantial

improvements. Included in this library is the following:

XS8.COM    The  CP/M  Z-80 --> Super8 cross-assembler  itself,  a

major subset of the full version.

XS8.DOC    Preliminary documentation for the assembler.

MON.S8     Super8 source code for the monitor; assembles w/ XS8.

SUPER8.DOC This text.  All files except this text were written by

Jeff D. Wilson.

T.S8       Every S8 source instruction in one file, can be assem-

bled  as a test of XS8.  Note that the  XS8  assembler

has  all  53  special register names,  as  defined  by

Zilog,  as  reserved words which are  predefined  with

their proper register value, eliminating the necessity

of  a large number of EQU's (Memorizing the names  and

functions of these 53 registers should take some time;

especially  as  many of them are bit mapped  in  their

functionality!)

FORTH.S8   S8 source defining enough Forth primitives to  perform

the "sieve benchmark". The words are placed in a list,

creating  a  Forth implementation  of  the  benchmark,

which runs extremely fast. See the documentation.

Additional  Notes:  The monitor code supplied would be  a  useful

development  tool  whether or not it is used with the  so  called

"contest  board" supplied in the Zilog Kit.  The  basic  hardware

requirements  are  the processor itself, 16k ROM  at  low  memory

(containing  the monitor code), and much development RAM  as  you

want starting at the top of memory. An RS-232 connector should be

connected  to the uP's UART pins for communicating with the  CP/M

host  machine  (only TX, RX, and GND req'd). Note that  the  UART

interrupts  are  routed indirectly through a jump in  RAM,  so  a

program  under development could intercept the UART  calls.  This

would allow the program under development to share the use of the

UART with the program being developed.

*****

"Super8" is a trademark of Zilog, Inc.  Neither Jeff Wilson nor I

have any affiliation with Zilog, Inc.  The XS8 assembler included

is  not a Zilog product, and is not supported by them (or  neces-

sarily  anyone  else,  for that  matter).   This  is  "Beta-test"

experimental material, and is offered "as is" for personal,  non-

profit use only. See documentation for additional information.

Comments, questions, bug reports, inquiries, or just for possible

updates-  call Jeff directly (see documentation) or leave a  mes-

sage at my data number.

- Steven Greenberg

21  March   1987

Data: (201) 447-6543

[24 hrs;  1200 baud]

######################################################################################################################