Documentation for Z80MR

Introduction

Z80MR is a Z80 macro assembler with syntax closely following RMAC and MAC.

It assembles standard Z80 mnemonics into an Intel Hex format. The resulting

file (which has a .HEX extension) can be translated to a .COM file with

LOAD.COM (on your CP/M disk that you received with your Kaypro) if it ORG's

at 100 (hex). If it ORG's elsewhere the .HEX file may be read into memory

and manipulated with DDT.COM.

Running Z80MR

The file to be assembled must have the extent *.AZM.  The syntax to

load the assembler is Z80MR <filename> without the extent.

Why Z80

The assembler you received with your Kaypro (ASM.COM) is an 8080 assembler.

The Kaypro actually runs a Z80 c.p.u. The reason this is possible is that

the Z80 actually runs all of the 8080 instructions but in addition there

are more instructions unknown to the 8080. The extra instructions were

designed for increased speed, easier programming, and more compact code.

For this reason it is to your best advantage to program in Z80 code for

the Kaypro.

Z80 Mnemonics

Z80 mnemonics are a great improvement to 8080. Thought was given to

logical, universal mnemonics that are much easier to remember and use. I

learned assembly language on the 8080 and resisted the change to Z80 at

first. But after using Z80 mnemonics for a short time I became very

unwilling to  do anything with 8080 code. Now I run almost every 8080

program that comes in through a 8080 to Z80 translating program

(XLATE2.COM). Even if you are writing programs for the 8080 it is still far

easier to  write in Z80 mnemonics. There is a special listing command that

flags Z80- only instructions for this very reason (described later).

Macros

Macros are a way of writing subroutines in assembly language and then

calling the subroutine by entering the 'macro name' into the source. The

macro may be called as many times as necessary anywhere in the program.

When the assembler is operated, the lines of source code that make up the

macro will be inserted into the file by the assembler. Note that using

a macro does not reduce the size of the object code that is produced since

all the lines of code that make up the macro definition are assembled into

the object file at assembly time. This is called expanding the macro. By

using the *MACLIST ON option, the lines of code produced by the expansion

of a macro are listed in the .PRN file. Then the code can be examined and

at times optimized in certain locations.

Assembler Syntax

Components and General Form of Assembly Language Programs

The structure of an assembly language program is more important to an

assembler than the actual instructions you write. A program that would run

beautifully can fail to assemble if the syntax is not correct. A program

with no errors at assembly time is not guaranteed to run correctly ( or as

expected ). The assemblers report of 0 errors means that it understood all

of the instructions you entered, not that your program is logically correct.

Fields

Assemblers are almost always field oriented some to a greater degree than

others. A field is a flexible position in the line of code with respect to

the right margin. This assembler recognizes 4 fields in an assembly language

source line.

label		operation	operand	 	comment

The assembler knows when it has reached the end of a field when it sees a

'field delimiter'. This can be a space or a tab for this assembler though

some require tabs so it is a good habit to always use tab characters as

delimiters.

Label Field

A symbol is a word used to represent a number. Symbols that refer to

addresses are called labels. The assignment of a number to a label can

either be defined as the lines below

TEN	EQU	10

START	EQU	100H

or calculated by the assembler as an address for branching instructions.

START:	JP	FINISH

NOP

NOP

FINISH:	JP	START

Also notice that the label is optional and is only for the programmers

conveniance.

Labels must appear in the label field. Some assemblers allow you to indent

labels but this one won't.

START	EQU	100H

START:	JP	FINISH

Will give you a problem. The EQU must be in the operation field and the

label in the label field.

Most assemblers require that the undefined labels be terminated in a

colon but this assembler does not require a colon for symbols in column 1.

START	JP	FINISH

will not generate an error but colons are another good habit and also

make your code more readable.

This assembler only examines the first six characters of any label or symbol

so that if the following labels were used in the same program

FINISH1	EQU	1000H

FINISH2	EQU	2000H

A 'D' error (duplicate symbols) would be generated.

Operation and Operand Fields

The operation field follows the label field and may either contain a Z80

op code mnemonic, an assembler directive (or pseudo op), or a macro call.

Assembler directives and macros are described later in this file. This

field will usally contain the mnemonic for a Z80 instruction. Some Z80

instructions only use this field while others contain an operand which

will be located in the operand field.

GOBACK:	OR	A

RET	Z

LD	A,0FFH

RET

The way Z80 mnemonics were designed, the number of nmenonics in the operation

portion of instructions is kept to a minimum since the operands really

distinguish the differences between similar instructions. The first line

above is a good example of this. The operation is an 'OR' operation on

the number in the accumulator (implied) with another register. It makes

sense that the operand should be the register containing the other number

in the 'OR' operation. In Z80 assembly language this is the case. The first

line OR's the accumulator with the accumualator (used to see if the

accumulator contains a 0). Notice that the second line uses the operand

field to contain the condition for a conditional jump (in this case the

zero flag). The third line uses the operand field to contain both the

target register for a load and the number to load. The last line is an

unconditional return which uses the same operator (RET) as the conditional

return but does not use the operand field because there are no conditons

to place there. This structure makes Z80 programs much more readable

than 8080 programs as well as making the instructions easier to remember.

The following is the same code written with 8080 mnemonics. Notice the

different philosophy on the use of the fields.

GOBACK:	ORA

RETZ

MVI	A,0FFH

RET

Also the LD command in the Z80 is used for all data moves while 8080 users

must remember a different mnemonic for different types of moves.

8080			Z80

MOV	H,A		LD	H,A

MVI	H,00		LD	H,00

LXI	H,0000		LD	HL,0000

The Comment Field

Comments are not limited to the comment field and can actually be the entire

line. All assemblers recognize the semicolon as the beginning of a comment

and most ignore the rest of the line. For compatability between assemblers

it is a good to begin comments with a semicolon. But for this assembler

the following methods of inserting comments are good syntax.

1. Beginning a line with an '*' in column one causes the assembler to ignore

therest of the line except if one of the assembler commands (described below)

immediately follows the asterisk (no embedded spaces).

2. A semicolon will cause the assembler to consider everything following it

to be considered a comment.

3. The first blank encountered following the beginning of the operand field

will cause the assembler to consider the rest of the line to be considered

a comment.

******************************************

;An adventure in Comments

* A short tale

START:	JP	FINISH		; finish this story

NOP			  ASM can't handle this

FINISH: RET	 Thats all folks

Would assemble with no errors. Comments do not appear in the object code.

Numbers and Bases

The assembler will accept numbers in HEX (base 16) BINARY (base 2) or

DECIMAL. Hex numbers must end with an H and binary numbers must end in

a B. Decimal numbers should have no suffix letter. When a HEX digit begins

with a letter, the letter should be preceded with a 0.

LD	A,0F3H

OR	01001000B

LD	HL,4000H+28

Commanding the Assembler

The primary responsibility of the assembler is to translate Z80 mnemonics

into object code. The assembler also recognizes certain commands and

directives that the programmer can use to manipulate the assembler's output.

These are often referred to as 'pseudo-ops'. This assembler requires these

pseudo-ops to be in upper case.  A description of these commands follows:

ORG	<expr>	; Sets the origin of the code or section of code. Actually

; it sets the reference number that the assembler uses to

; generate addresses for labels and instructions.

; <expr> could be a number or a previously defined symbol.

; e.g.	ORG	0

;       ORG	START

END	<symb>	; Determines the end of an assembly language program. <symb>

; if present describes the first executable instruction of

; the program.

DW	wordlist

DEFW	wordlist

; Both of these have identical meanings. In assembly language

; programs, 8 bit values are called bytes and 16 bit

; values are called words. Addresses are assembled with the

; most significant byte (MSB) following the least significant

; byte (LSB) because this is how the microprocessor handles

; these values. The DW pseudo-op allows us to describe these

; values in the way we are used to (MSB first) and still

; assemble correctly for the processor.

;	DW	8000H

;			 will assemble the same as

;	DB	00H

; 	DB	80H

;

; If more than one word is to follow a DW following values

; should be seperated by commas

; e.g.	DW	8000H,0F000H,0000H

; The wordlist can also be symbols

; e.g.	START:	JP	FINISH

;		DW	START,FINISH

;  	FINISH:	JP	START

DDB	wordlist

; This pseudo-op is a way of assembling 16 bit values with

; the MSB first (opposite of DW).

;	DDB	8000H

;			will assemble the same as

;	DB	80H

;	DB	00H

DB	bytelist

DEFB	bytelist

DEFM	bytelist

DATA	bytelist

; These four pseudo-ops have identical meanings. The bytelist

; can be one byte or multiple bytes seperated with commas.

; The bytes can be any mix of symbols, ascii characters in

; quotes, or numbers on the same line. This is familiar

; code in Kaypro programs:

;

;ESC	EQU	1BH

;CLRSCR	EQU	1AH

;CRLF	DDB	0D0AH

;

;	ORG	100H

;

;	LD	DE,MES

;	LD	C,9

;	CALL	5

;	RET

;

;MES:	DB	CLRSCR

;	DB   	ESC,'=',12+20H,12+20H

;	DB	'*Your Message Here *',CRLF

;	DB	'*Or Here*','$'

;

;	END

;

; If you've been waiting for an example to enter assemble

; and run, try this one out. Just enter it (with out the

; semicolons of course) assemble it and run it as described

; in AZM-COM.DOC.

;

; The program clears the screen, positions the cursor at

; row 12 column 12 and prints the message using the BDOS

; function 9 (print string).

;

; The symbol CLRSCR is defined by an EQU to the hex code to

; clear the screen on the Kaypro (^Z).

;

; The cursor positioning sequence on the Kaypro consists of

; the two lead-in characters (escape and an equals sign) and

; then the row+20H and the column+20H.

;

; Since the next bytes are just a carriage return, line feed

; pair the second part of the message will appear at the

; left side of the screen. We could include extra DB's to

; position the cursor anywhere on the screen if we like.

;

; BDOS function 9 (summoned by loading a 9 in the C register

; and calling 0005H) prints the characters it finds at the

; address in the DE registers until it sees a '$'.

DS	n

DEFS	n	; Reserve data space ( n bytes ). This is used to position

; allocate or label data storage space in a program. n is

; a number describing the number of bytes reserved.

;	DS	16

; Reserves 16 bytes. The next instruction will be located

; 16 bytes from the location counter when the DS was

; encountered.

label	EQU	<expr>

;

; The EQU sets the label equal to the expression. The

; label should not be terminated with a colon when used

; with an EQU pseudo-op. The label can be any symbol

; (byte or word) and the <expr> a number in any of the

; following forms:

;		SWEET	EQU	16	   ;decimal

;		SWEET	EQU	10H	   ;hex

;		SWEET	EQU	00010000B  ;binary

;

; With this assembler the EQU must be located in the

; operation field.

; A label defined with an EQU cannot be redefined later

; in the program.

label	DEFL	<expr>

;

; This assigns the value of the <expr> to the label like

; the EQU pseudo-op but a label defined with a DEFL can

; be redefined later in the program.

*INCLUDE	<filename>

*INCLUDE <filename>

; This pseudo-op causes the assembler to stop assembling

; lines in the file it is presently in and read in the

; file <filename>. It then begins assembling lines in this

; included file until it reaches the end of the file when

; it returns to the original file and resumes assembling

; lines in it once more. The <filename> can be any CPM

; filename.ext though if the extent is left off it looks

; for the given filename with an extent of .LIB. The asterisk

; must appear in column 1 with the word INCLUDE immediately

; following with no embedded spaces.

;

;*INCLUDE	DRIVER.AZM	; will begin assembly on

; the file DRIVER.AZM

;*INCLUDE Z80MACRO	; will begin assembly on

; the file Z80MACRO.LIB

;

Conditional Assembly Pseudo-Ops............................................

IF	<expr>

ELSE

ENDIF

Conditional assembly is a way of writing a single program so that

it can be assembled different ways or with different options by

only changing a couple of lines of codes. When the assembler

encounters an IF pseudo-op it evaluates the symbol <expr>. IF

<expr> is non-zero it assembles the following lines until it reaches

an ELSE or an ENDIF. If <expr> is 0 the lines are ignored until

the assembler encounters an ELSE or an ENDIF. If the ELSE is

encounter the assembler begins assembling lines again. The ENDIF

pseudo-op causes the assembler to resume assembling all lines.

You can not have an IF without an ENDIF.

Any of these pseudo-ops must appear in the operation field.

TRUE	EQU	0FFH

FALSE	EQU	0

KPRO2	EQU	TRUE

KPRO10	EQU	FALSE

IF	KPRO2

BITPRT	EQU	1CH

ELSE

BITPRT	EQU	14H

ENDIF

Operators..................................................................

Operators allow the programmer to make the assembler do arithmetic and

logical operations. They are usually used to manipulate operands or

generate symbols. Some of them are used to create tests for conditional

assembly. There should be no embedded spaces when using these operators as

the first blank encountered terminates the operand field. The operands

may be symbols or numbers in any of the bases. The operators supported

by this assembler are:

Arithmetic Operators

+		; arithmetic addition.

-		; arithmetic subtraction

*		; arithmetic multiplication

/		; arithmetic division (truncating the result)

Logical Operators (Bit Manipulation)

&

( or .AND. )	; logical AND operation

^

( or .OR. )	; logical OR operation

.XOR.		; logical exclusive OR operation

\

( or .NOT. )	; logical inversion

.SHR.		; shift left operand to right by right operand

.SHL.		; shift left operand to left by right operand

.HIGH.		; byte value is assigned the high byte of a

; 16 bit value

.LOW.		; byte value is assigned the low byte of a 16

; bit value

Conditional Assembly Operators ( return	TRUE or FALSE to IF )

=

( or .EQU. )	; logical equivalence

>

( or .GT. )	; greater than

.UGT.		; unsigned greater than

<

( or .LT. )	; less than

.ULT.		; unsigned less than

Listing Options Pseudo-Ops.................................................

There are a number of listing options. All of these options only effect

the print file (.PRN). The options include some for debugging as well as

some for the actual format of the file on the page. The .PRN file is the

basic tool assembly language programmers have for examining the output of

the assembler. The pseudo-ops beginning with an asterisk must begin in

column 1.

*EJECT

( or EJEC )	; The next line of the listing should be placed at the top

; of the next page.

*HEADING	; Place the text ( following this command ) on the top of

; each page. Usually used to date the listing file.

TITLE 'text'	; Place the text in the quotation marks (either double or

; single on the top of each page in the listing file.

SPAC n		; Leave n blank lines in the listing. Used to leave white

; space in the file with out using a page break.

*LIST ON

*LIST OFF	; Turn the listing on or off. This is usually used to omit

; long comments or certain sections from the .PRN file.

*MACLIST ON

*MACLIST OFF	; Turn the expansion of macros on or off. Seeing how the

; macros are being expanded is handy for optimizing code

; but can waste paper when that is no longer the area of

; interest.

LIST options

NLIST options	; These pseudo-ops allow you to turn any of the supported

; listing file options on (LIST) or off (NLIST) without

; changing the other options. Both of these pseudo-ops

; must be followed with one or more of the following option

; letters. If these pseudo-ops is used some options are

; on by default ( marked with (on) in the following list.

;

; A 	; List all bytes in DB, DW, DDB, etc. Otherwise

;	; only the bytes that can fit in one line are

;	; included in the listing ( others are implied ).

; B	; Place symbol table into object file.

; G	; Place system generated symbols into object files

; I (on); List lines of conditional code following a false

;	; conditional. If off only the code actually

;       ; assembled is listed.

; M (on); Expand macros in listing files

; O (on); Produce an object module. That is show the bytes

;       ; being generated by the assembler otherwise just

;       ; the source and (optionally) macro expansions.

; R     ; use absolute displacement for JR and DJNZ

; S (on); List source code in listing file

; T (on); List symbol table in listing file

; X 	; Generate and list cross references in listing file

; Z	; Generate an error for Z80 only opcodes. Allows you

;	; to write in Z80 mnemonics for an 8080 processor.

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

Error Reporting

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

When the assembler is unable to understand what you are instructing it to

do it generates an error message. These are almost always due to typo's or

bad form. It displays the error code below and the line the error was found

on to the console and also displays the error codes in the listing file.

D	; Duplicate symbol definition. You will see this error

; message if you do any of the following:

; 	Use the same symbol twice.

;		FORMATX

;		FORMATC

;		will generate an error (only 6 significant

;		characters).

;	Upper and lower case symbols with the same letters

;		FORMAT:

;		format:

;		are identical to the assembler.

;	Assigning a different value to a symbol that was

;		previously defined with a EQU pseudo-op.

;		If you are going to reassign use DEFL.

E	; Relocation error. I believe this occurs if the assembler

; cannot reassign an address as expected.

F	; Format Error. You will see this if you break any of the

; rules regarding field use and macro format.

K	; Keyword error. This means you tried to use one of the

; assemblers reserved words or pseudo-ops as a symbol.

;	ORG:	JP	END

;		NOP

;	END:	JP	ORG

; 	is in very bad taste.

L	; Label error. The attempt to assign a value to a lable was

; unsuccessful. Also remember that labels do not end in a

; colon when preceding EQU.

;	START:	EQU	100H	; is bad news

;	START	EQU	100H	; is perfect

M	; Missing label. The symbol you are using was never defined.

N	; Macro nesting error. Macros can be nested (that is a macro

; can call another macro) but if the nesting gets to deep

; the assembler will quit and give you one of these. Also,

; you can only call macros that were previously defined.

O	; Op code error. If you see this, look in the operation and

; operand fields. Consult the mnemonic table. People

; switching over from 8080 will see a few of these.

P	; Phase error. A 2 pass assembler builds a symbol table on

; the first pass and generates the object code on the second.

; If a number that it calculates for a symbol on the first

; pass does not agree with a number it generates in the

; pass this error is shown check the symbols in the line

; the error appeared.

Q	; Questionable operand. Actually theres no question about it

; it is a bad operand. Typo's give you these as well as

; blowing op code format. Usally easy to find your mistake.

S	; Syntax error. You broke one of the syntax rules described

; above.

T	; Symbol table full. Not much you can do with this except

; pare down the code.

U	; Undefined symbol. You used a symbol but forgot to define

; it in with an EQU.

V	; Value error. Usually means you are trying to do a 16

; bit operation with an 8 bit number or the other way

; around.

Macros

The macro is a powerful method of writing assembly language programs. It

makes it possible to write assembly language programs in a way that resembles

higher level languages. In fact by creating a library of macros you are

in essence creating your own language, and your own compiler.

Often times in assembly language (particularly writing for CP/M) each program

contains source lines that are used again and again in other programs. By

using macros the routines only need to be written once and then may be called

in any program. The best thing about a macro library is that only the macros

that are called produce object code. So there is no penalty in having a macro

library that is large and complete even if you are only going to call one

macro.

Macros have a form that is unique and must be followed closely for correct

results. The general form of a macro is

name	MACRO	#parameter1,#parameter2,....

instruction

instruction

instruction

.

.

.

ENDM

The name is the symbol that will be used to invoke the macro. MACRO is a

keyword that will indicate to the assembler that a macro is being defined.

The parameters always must begin with a '#' sign in macros and they are

seperated by commas. The instruction can be Z80 instructions, or any of the

assembler commands listed above incuding conditionals. The instruction can

also be another macro call (called nested macros) but only if the nested

macro has been already defined. The ENDM keyword tells the assembler that it

has reached the end of the code that must be assembled when this macro is

called. Do not use a colon behind the macro name.

The previous message program example can be rewritten to look like this

with macros.

ORG	100H

*INCLUDE Z80MACRO

BDOS	PRNSTR,MES

RET

MES:	DB	ESC,'=',12+20H,12+20H

DB	'*Your message here*'

DDB	CRLF

DB	'*Or here*'

END

With the following macro library called Z80MACRO.LIB

;Call Bdos function #FUNCT using paramater contained in #DE

ESC	EQU	1BH		; ascii escape

CRLF	EQU	0D0AH		; ascii	carriage return line feed

PRNSTR	EQU	9

BDOS	MACRO	#FUNCT,#DE

LD	C,#FUNCT	; FUNCTION NUMBER GOES TO C

LD	DE,#DE		; GET PARAMETER

CALL	5		; CALL BDOS

ENDM

We could also rewrite the cursor positioning sequence into a macro. Note

how just this small example can save us time in future programs. Also, the

macro library is a great place to keep frequently used symbols like ESC and

CRLF.

But what about using address symbols in macros? How can we avoid the 'D'

error if we call the macro more than once. The other keyword unique to

macros is LOCAL. This makes the assembler generate its own unique label

every time the macro is expanded in a program. Following the word LOCAL

( which must be on the second line of the macro ) are the symbols we want

the compiler to generate unique labels for. These symbols must also be

proceded with a '#' sign.

AJUMP	MACRO

LOCAL	#ADR_Z,#BACK

OR	A

JR	Z,#ADR_Z

LD	A,40H

JR	#BACK

#ADR_Z:	LD	A,04H

#BACK:	LD	DE,0

ENDM

The macro itself is not really useful but it is correct and shows the use

of local labels.

#A,04H

#BACK:	LD	DE,0

ENDM

The macro itself is not really useful but it is correc