Notes on Microbee RTC ( brief)

Part 1 Construction

If you have a Premium mother board type 8501-3-03C (inscribed in copper) and

8501-3-04CR ( silk-screened ) or later than the following parts can be fitted

directly to the mother board.

IC18  146818

IC15  74HC00

D1    1N4148 or similar diode

D2    1N4148 or similar diode

G1    32.768 KHz crystal

R1    22 ohm resistor

R2    1M ohm resistor

R3    150K ohm resistor

R4    4.7M ohm resistor

R7    1.8K ohm resistor

R14   1K ohm resistor

CV2   approx 6-60 pf trimmer capacitor ( adjust so clock keeps accurate time )

C1    10 mfd tantalum 16 volt

C6    .0047 mfd greencap capacitor

C7    33 pf capacitor

C8    33 pf capacitor

C15   .1 mfd monolithic capacitor

BB    3.6 volt Nicad Varta type 3/60DK (add this last)

24 pin socket   optional - use a good quality dual wipe type if used

14 pin socket     "         "  "  "     "      "    "    "   "   "

In addition three wire jumpers need to be added :-

1. W1 pads are near the colour connector. Add a jumper between W1-A2 and W1-B2.

Also check there is no connection between W1-A1 and W1-B1 or between W1-A3

and W1-B3 on either side on the board.

2. Connect a long insulated wire jumper between L1 and L1 pads. One is located

on the right side of the board to the right of IC71 a 74LS138 just above the

back-space key, the other is to the left of IC24 a 74LS32.

3. Connect a long insulated wire jumper between L2 and L2 pads. One is located

on the left side of the board just under the metal-work which supports the

keys almost directly underneath the top hole of the metal-work beside the

ESCAPE key. Again the other is near to IC24 a 74LS32, just to the right of

that IC.

N.B. Do not forget the little piece of wire around the crystal to hold it in

position.

Those with earlier Premiums or other types of Bee can get a kit and

instructions from MicroBee Systems for about $80 including an additional PC

board. These parts are also usable for those adding components directly to

the mother board.

--------========--------

Part 2 Chip details

The 146818 RTC chip consists of 50 general purpose registers, ten registers

which normally contain the time, calendar, and alarm data, and four control and

status registers. The 50 registers can be used as general purpose memory. All

are battery backed up. All 64 bytes are directly readable and writable by

the processor program except registers C and D which are read only.

The addresses on the MicroBee are as follows :-

Address		04

Write		06

Read		07

For example to read RTC register 2 the sequence would be :-

Write 02 to port     address 04

input data from port address 07

in basic this would be

10 OUT 4,2 : REM select register 2

20 A=IN(7) : REM the variable A will equal the contents of register 2

in assembler this would be

ld	a,002h		; load a with the required RTC register address

out	(004h),a	; output to RTC address register

in	a,(007h)	; read data from the required RTC register

; to Z80 register a

Similarly to write to RTC register 9 the sequence would be :-

Write 09 to port     address 04

write data to port   address 06

in basic this would be

10 OUT 4,9 : REM select register 9

20 OUT 6,B : REM the variable B will be written to register 9

in assembler this would be

ld	a,009h		; load a with the required RTC register address

out	(004h),a	; output to RTC address register

ld	a,data		; load Z80 register a with the required data

out	(data),a	; write data to RTC register 9

RTC Time, Calendar and Alarm register Functions

-------------------------------------------------------------------------------

Address  | Function   |Decimal| Range of RAM in HEX       |Example(Nums in Hex)

Location |            | Range | Binary Mode | BCD Mode    |Binary mode|Hex Mode

-------------------------------------------------------------------------------

0     | Seconds    | 0-59  |   X00-X3B   | X00-X59     |    15    |   21

-------------------------------------------------------------------------------

1     | Seconds    | 0-59  |   X00-X3B   | X00-X59     |    0F    |   15

| Alarm      |       |             |             |          |

-------------------------------------------------------------------------------

2     | Minutes    | 0-59  |   X00-X3B   | X00-X59     |    3A    |   58

-------------------------------------------------------------------------------

1     | Minutes    | 0-59  |   X00-X3B   | X00-X59     |    1E    |   30

| Alarm      |       |             |             |          |

-------------------------------------------------------------------------------

| Hours      | 1-12  | X01-X0C(AM) | X01-X12(AM) |    05    |   05

|(12 hr mode)|       | X81-X8C(PM) | X81-X92(PM) |          |

4     |            |       |             |             |          |

| Hours      | 0-23  |   X00-X17   |   X00-X23   |    05    |   05

|(24 hr mode)|       |             |             |          |

-------------------------------------------------------------------------------

| Hours Alarm| 1-12  | X01-X0C(AM) | X01-X12(AM) |    06    |   06

|(12 hr mode)|       | X81-X8C(PM) | X81-X92(PM) |          |

5     |            |       |             |             |          |

| Hours Alarm| 0-23  |   X00-X17   |   X00-X23   |    06    |   06

|(24 hr mode)|       |             |             |          |

-------------------------------------------------------------------------------

6     | Day of Week| 1-7   |  X01-X07    |  X01-X07    |    01    |   01

| Sunday=1   |       |             |             |          |

-------------------------------------------------------------------------------

7     |Day of Month| 1-31  |  X01-X1F    |  X01-X31    |    0B    |   11

-------------------------------------------------------------------------------

8     | Month      | 1-12  |  X01-X0C    |  X01-X12    |    02    |   02

-------------------------------------------------------------------------------

9     | Year       | 0-99  |  X00-X63    |  X00-X99    |    4F    |   79

-------------------------------------------------------------------------------

Example is for 5:58:21 Sunday February 11th 1979

Alarm set for 6:30:15

--------========--------

REGISTER A     Read/Write except bit UIP which is read only

-------------------------------------------------------------------------

|   b7   |   b6   |   b5   |   b4   |   b3   |   b2   |   b1   |   b0   |

-------------------------------------------------------------------------

|  UIP   |  DV2   |  DV1   |  DV0   |  RS3   |  RS2   |  RS1   |  RS0   |

-------------------------------------------------------------------------

UIP - This Update In Progress (UIP) bit is a staus flag that may be monitored

by the program. When UIP is a "1" the update cycle is in progress or will soon

begin. When UIP is a "0" the update cycle is not in progress and will not be

for at least 244 microsecs.

DV2,DV1,DV0 - Crystal frequency selection use 0,1,0 for the MicroBee

i.e. DV2=0 DV1=1 DV0=0

RS3,RS2,RS1,RS0 - Square-wave or Periodic Interrupt frequency select. On the

MicroBee only the Periodic Interrupt can be used (via IRQ to PIO B bit 7)

Chose the frequency / rate from the following chart

-----------------------------------------------------------

| RS3 | RS2 | RS1 | RS0 |    Rate       |    Frequency    |

-----------------------------------------------------------

|  0  |  0  |  0  |  0  |   None        |   None          |

-----------------------------------------------------------

|  0  |  0  |  0  |  1  |   3.90625 ms  |     256 Hz      |

-----------------------------------------------------------

|  0  |  0  |  1  |  0  |   7.81250 ms  |     128 HZ      |

-----------------------------------------------------------

|  0  |  0  |  1  |  1  |   122.070 us  |   8.192 kHz     |

-----------------------------------------------------------

|  0  |  1  |  0  |  0  |   244.141 us  |   4.096 kHz     |

-----------------------------------------------------------

|  0  |  1  |  0  |  1  |   488.281 us  |   2.048 kHz     |

-----------------------------------------------------------

|  0  |  1  |  1  |  0  |   976.562 us  |   1.024 kHz     |

-----------------------------------------------------------

|  0  |  1  |  1  |  1  |  1.953125 ms  |     512 Hz      |

-----------------------------------------------------------

|  1  |  0  |  0  |  0  |   3.90625 ms  |     256 Hz      |

-----------------------------------------------------------

|  1  |  0  |  0  |  1  |   7.81250 ms  |     128 Hz      |

-----------------------------------------------------------

|  1  |  0  |  1  |  0  |    15.625 ms  |      64 Hz      |

-----------------------------------------------------------

|  1  |  0  |  1  |  1  |    31.250 ms  |      32 Hz      |

-----------------------------------------------------------

|  1  |  1  |  0  |  0  |    62.50  ms  |      16 Hz      |

-----------------------------------------------------------

|  1  |  1  |  0  |  1  |      125  ms  |       8 Hz      |

-----------------------------------------------------------

|  1  |  1  |  1  |  0  |      250  ms  |       4 Hz      |

-----------------------------------------------------------

|  1  |  1  |  1  |  1  |      500  ms  |       2 Hz      |

-----------------------------------------------------------

--------========--------

REGISTER B     Read/Write Register

-------------------------------------------------------------------------

|   b7   |   b6   |   b5   |   b4   |   b3   |   b2   |   b1   |   b0   |

-------------------------------------------------------------------------

|  SET   |  PIE   |  AIE   |  UIE   |  SQWE  |  DM    |  24/12 |  DSE   |

-------------------------------------------------------------------------

SET - When set to a "1" the update cycle functions normally updating the

counts once per second. When set to a "0" the update cycle is inhibited. Use

of this bit allows the clock to be set correctly.

PIE - When set to a "1" the periodic interrupt flag will cause the IRQ pin to

become active. This rate is set by the RS3,RS2,RS1,RS0 bits. When set to a

"0" the IRQ pin is not activated.

AIE - When set to a "1" the Alarm flag will cause the IRQ pin to become active

when the alarm time matches the real time. When set to a "0" the IRQ pin is not

activated.

UIE - When set to a "1" the Update-Ended Interrupt flag will cause the IRQ bit

to become active. Since this happens once per second this is one way of getting

an interrupt once per second. When set to a "0" the IRQ pin is not activated.

SQWE - This function cannot be used on the MicroBee as the relevant pin on the

RTC chip is not connected. Leave as a "0".

DM - When set to a "1" the RTC chip uses binary data, when set to a "0" the RTC

chip uses BCD data. Use of BCD mode is generally easier.

24/12 - When set to a "1" the 24 hour mode is used. When set to a "0" the 12

hour mode is used.

DSE - (No it is NOT the Dick Smith Electronics bit!) It is the Daylight Savings

Enable bit. It was for the US, but recent changes to Daylight Saving dates in

the US mean this bit is now useless even there. Always set to a "0".

--------========--------

REGISTER C     Read Only Register

-------------------------------------------------------------------------

|   b7   |   b6   |   b5   |   b4   |   b3   |   b2   |   b1   |   b0   |

-------------------------------------------------------------------------

|  IRQF  |  PF    |  AF    |  UF    |   0    |   0    |   0    |   0    |

-------------------------------------------------------------------------

IRQF - Set to a one when any of the other flags are active i.e. PF, AF, or UF

and indicates the IRQ pin is active. A read of this register will clear all

flags so they should be saved by the user program if more than one flag is

being used for later examination.

PF - Periodic Interrupt Flag

AF - Alarm Flag

UF - Update-ended flag

--------========--------

REGISTER D     Read Only Register

-------------------------------------------------------------------------

|   b7   |   b6   |   b5   |   b4   |   b3   |   b2   |   b1   |   b0   |

-------------------------------------------------------------------------

|  VRT   |   0    |   0    |   0    |   0    |   0    |   0    |   0    |

-------------------------------------------------------------------------

VRT - When a "1" this indicates power sense pin is at a saisfactory level

and hence the registers should be valid. A "0" indicates an unsatisfactory

level and thus the register contents will be invalid.

--------========--------

Notes on setting the alarm

The alarm may be used in two ways :-

1. The 3 registers are set to the appropriate time and the alarm interrupt will

become active accordingly.

2. By setting any Hex code between XC0 and XFF in an alarm register, the alarm

compare will become active every time at that interval, i.e. setting the hour

alarm to XFF will mean the alarm interrupt becomes active each hour, setting

the hour and minute alarm registers to XFF will mean an alarm interrupt each

minute and setting all three alarm registers to XFF will mean an alarm

interrupt each second.

See the manufacturer's data books for more information.

--------========--------

Part 3 Software

1. See RTC.MWB for a demonstration in basic

2. SETCLOCK.MAC for am assembler program to set the clock

3. TIME.MAC for a simple program displaying the time

Thanks to Glenn Satchell for his code which saved me some time and effort.

The information here is correct to the best of my knowledge. Please leave a

message describing any omissions or needed corrections which will go into the

next version.

Tony Ellis VK2ZNH 02.05.86

#########################################################################