DOCUMENTATION FOR AMODEM VER.  4.2

TERMINAL PROGRAM.

This file will permit the ATARI

to access an RCPM BBS that runs

under the CP/M operating system.

Also SOME ATARI BB Systems.

And allow ATARI to ATARI transfer.

It features uploading and downloading

at 300 baud and is extremely user

friendly.

The file may be listed here with

command D.     AMODEM4  <Y/N>N

Then captured and saved to disk.

Then ENTERed with the interface

"booted" by an AUTORUN.SYS file.

When RUN, it will go into terminal

mode at which time the user should

dial the BBS. Some systems are of

the RING-BACK variety, so you should

be prepared to let the phone ring

ONCE. You will then hang up and

re-dial 10-15 seconds later.

When the system connects, type a

few <returns>s until the BBS responds.

Follow the HELP commands of the

system from here.

TRANSLATION - SELECT T will toggle

the translation mode between ATARI

(No translation) and ASCII (Light

translation). Use ATARI on ATARI

Systems and ASCII on RCPM Systems.

MENU - For ATARI Disk systems, you

may list your Menus with SELECT M

or SELECT 1 or 2 or 3 or 4.

To find files or space for new files.

DUPLEX - For ATARI to ATARI file

transfer, you will need Half Duplex

Since AMODEM sends no echo.

You may toggle it with SELECT P.

NOTE; Use ATARI Mode on each end.

---------------------------------

NOTE: the commands XMODEM S and

XMODEM R are for RCPM boards only

For other boards, follow the

directions for that system!

For AMIS boards do function D or U

then answer Y to XMODEM Protocol?

---------------------------------

DOWNLOADING - To download from the

BBS you must remember that it is

controlled by a file called XMODEM.

To have this file send a program to

your ATARI, you must press <SELECT>

key and type R to tell the ATARI to

get ready to receive from XMODEM.

You must then provide a FILESPEC

to which the ATARI will save the

program. This does not necessarily

have to be the same name as the BBS

has it saved as.

Once back in terminal mode, type the

command; XMODEM S <filename.ext>

and then RETURN at which time the

XMODEM controller will respond with

a statement that it has opened its

file and is prepared to transmit to

your ATARI. To begin the procedure,

press the START key and sit back

and relax. The XMODEM will send the

file, which can be of any type.

The screen will turn RED and when

the file has been downloaded, your

ATARI will save it to your filespec.

When the screen turns back to BLUE,

the process is complete.

TO UPLOAD - Reverse the procedure

by pressing the SELECT and S keys

to indicate you will Send to XMODEM

the ATARI <filespec><RETURN>.

Then type; XMODEM R <filename.ext>

and RETURN which will tell XMODEM to

prepare a filearea under that name

to store the program you are going

to send TO the BBS. To initiate

the upload, press the START key and

the computers will take over. Your

screen will turn GREEN until the

transfer is completed.

TO CAPTURE DATA - Press SELECT then

C and enter a filespec. To start

press the START or OPTION key.

All data that you see on the screen

will be saved to memory. You may

toggle the capture on and off with

the OPTION key. When you are done,

or the buffer is full, you can press;

SELECT and D. This will Dump the

memory to your filespec.

NOTE ABOUT ATARI <FILESPEC>

You may use any legal ATARI Device

and filename and type. Example;

D:MYFILE.BAS  or  D2:YOUR.PGM

C:  or  P:  or  R2:  or  E:

Remember that you can't send from

the Printer and you may send and

receive using the Editor (E:).

Please leave any questions on the

A.R.C.A.D.E. BBS at 313-978-8087

or Compuserve PEM-450: 70355,602

or write:  Jim Steinbrecher

37220 Tricia Drive

Sterling Hts, Mi 48077

************************************

NOTE-

There are many different versions of

the AMODEM terminal program in use.

The major versions <2.0, 3.0, 4.0>

were released by the author, while

the "sub-versions <3.1, 3.2, 4.1...>

were mostly prepared as personal

modifications by ATARI enthusiasts

from all over the USA. For the most

part, they added features such as

mmultiple disk drive access or duplex

toggles that were gathered together

and refined by Jim, and included as

standard features in the subsequent

major version.

One such sequence of evolution that

illustrates this best, is the <M>enu

option.

Version 3.0 had send/recieve/menu

access from D1: only. Ver. 3.1 added

all 4 drive availability to this.

When 4.0 was released in 10/82 this

was a standard feature. Ver. 4.1

modifies line 4000 and adds 2 other

lines that permits the same <M>enu

scroll of every drive as 4.0, but

now permits the user to type simply

a single number <1,2,3,4> to have

only that single menu shown.

SYSOP/Ron Luks

CEMSIG <PEM-450>

71645,253